# Approved arrangement

2.1 – non-agriculture products

Conditions

Version 5.0

© Commonwealth of Australia 2021

Ownership of intellectual property rights

Unless otherwise noted, copyright (and any other intellectual property rights, if any) in this publication is owned by the Commonwealth of Australia (referred to as the Commonwealth).

Creative Commons licence

All material in this publication is licensed under a Creative Commons Attribution 4.0 International Licence except content supplied by third parties, logos and the Commonwealth Coat of Arms.

Inquiries about the licence and any use of this document should be emailed to copyright@awe.gov.au.

Cataloguing data

This publication (and any material sourced from it) should be attributed as: Approved arrangements section, 2021, Approved arrangement non-agricultural products Department of Agriculture, Water and the Environment, Canberra, December 2021. CC BY 4.0.

ISBN 978-1-76003-214-2

This publication is available at awe.gov.au/import/arrival/arrangements/requirements.

Department of Agriculture, Water and the Environment

GPO Box 858 Canberra ACT 2601

Telephone 1800 900 090

Web awe.gov.au

The Australian Government acting through the Department of Agriculture, Water and the Environment has exercised due care and skill in preparing and compiling the information and data in this publication. Notwithstanding, the Department of Agriculture, Water and the Environment, its employees and advisers disclaim all liability, including liability for negligence and for any loss, damage, injury, expense, or cost incurred by any person as a result of accessing, using, or relying on any of the information or data in this publication to the maximum extent permitted by law.

## Version control

Updates to this document will occur automatically on the department’s website and the revision table below will list the amendments as they are approved.

| Date | Version | Amendments | Approved by |
| --- | --- | --- | --- |
| 9 May 2011 | 1.0 | Revised document | Co-regulation and Support Program |
| 30 Jun 2013 | 1.1 | Updated to reflect DAFF branding | Industry Arrangements Reform Program |
| 8 Feb 2016 | 2.0 | Updated template including nonconformity ratings | Approved Arrangements section |
| 2 May 2016 | 2.1 | Changes to criteria: - Fumigation - Wash bay - Waste water recycling - Separation/security | Approved Arrangements section |
| 16 Jun 2016 | 3.0 | Updated references to the department and the Biosecurity Act 2015. | Approved Arrangements section |
| May 2021 | 4.0 | Removal of reference to products associated with khapra beetle from the purpose statement in Table 1 Purpose | Approved Arrangements section |
| 15 December 2021 | 5.0 | Added - Information required for application - Key arrangement outcomes - Biosecurity risk information - Informative text - Conditions for waste water hypochlorite treatment - Condition to provide access to the department to install and monitor insect traps - Contact details for conditions requiring notification to the department - Conditions to ensure biosecurity waste is collected and identified to a department approved waste transporter. - Conditions to ensure departmental approval is obtained for any proposed changes. Updated - Web accessible template - Conditions for accredited persons - Maintaining records for two years instead of 18 months - Various conditions to clarify intent of wording Removed - Conditions for onsite fumigation | Approved Arrangements section |

Contents

Version control	3

Guide to using this document	5

Definitions	5

Other documents	5

Noncompliance guide	5

Key arrangement outcomes	6

Objective	7

Scope	7

Site location	8

Information required for application	8

Conditions	9

Table 1 Site personnel	9

Table 2 Security	10

Table 3 Isolation	11

Table 4 Construction	12

Table 5A Biosecurity area	13

Table 5B Biosecurity area	14

Table 6 Wash facilities	15

Table 7 Wash facilities – wash cabinets	15

Table 8 Wash facilities – wash bays	16

Table 9 Recycling waste water	17

Table 10 Site hygiene and monitoring	18

Table 11 Internal container inspections	19

Table 12 Facilities for departmental officers	19

Table 13 Waste	20

Table 14 Waste water hypochlorite treatment	21

Table 15A Records	22

Table 15B Records	23

Table 16 Ceasing operations	24

Table 17 Site maps	24

Table 18A Compliance	25

Table 18B Compliance	26

Table 18C Compliance	27

## Guide to using this document

This document sets out the conditions that must be met before the relevant Director will consider approval for the provision of biosecurity activities under section 406 of the Biosecurity Act 2015, otherwise known as an approved arrangement.

This document specifies the conditions to be met for the approval, operation and audit of this class of approved arrangement. Compliance with the conditions will be assessed by audit.

In the event of any inconsistency between these conditions and any Import Permit condition, the Import Permit condition applies. If the applicant chooses to use automatic language translation services in connection with this document, it is done so at the applicant’s risk.

Unless specified otherwise, any references to ‘the department’ or ‘departmental’ means the Department of Agriculture, Water and the Environment.

Further information on approved arrangements, department contact details and copies of relevant approved arrangement documentation is available on the department’s website: awe.gov.au.

### Definitions

Definitions that are not contained within the approved arrangements glossary can be found in the Biosecurity Act 2015, the relevant treatment methodology or the most recent edition of the Macquarie Dictionary.

Terms that have specific definitions contained within the approved arrangements glossary or Biosecurity Act 2015 are italicised where used.

### Other documents

The Approved Arrangements General Policies should be read in conjunction with these conditions. They will assist in understanding and complying with the obligations and conditions for the establishment and operation of an approved arrangement.

### Noncompliance guide

The noncompliance classification against each condition is provided as a guide only. If more than one noncompliance is listed against a condition, the actual noncompliance applied will correspond to the gravity of the issue. The noncompliance recorded against any conditions remains at the discretion of the biosecurity officer.

Noncompliance classifications are detailed in the Approved Arrangements General Policies.

The ‘reference: [number]’ detailed in the noncompliance guide is for internal (departmental) use only.

### Key arrangement outcomes

Key arrangement outcomes are high level outcomes the biosecurity industry participant is responsible for meeting under an approved arrangement.

Each class condition for an approved arrangement is assigned a key arrangement outcome.

Key arrangement outcomes are met by complying with the class conditions

Table 1 List of KAOs including their purpose and description

| KAO | Purpose |
| --- | --- |
| Containment | Goods subject to biosecurity control are contained in a way that prevents them, or any biosecurity risk material escaping into the environment. |
| Isolation | Goods subject to biosecurity control are isolated from other goods in a manner that prevents cross-contamination or cross-infestation. |
| Security | Controls are in place that prevent unauthorised access to goods subject to biosecurity control. |
| Identification | Goods subject to biosecurity control and areas in which biosecurity activities are carried out must be visually identifiable as such. |
| Traceability | Goods that are or were, subject to biosecurity control, are linked to records of the origin and movement of the goods and the biosecurity activities carried out in relation to the goods. |
| Hygiene | Approved arrangement sites are maintained in a state that minimises opportunity for, and susceptibility to pest, weed and disease establishment and/or infestation. |
| Movement | Goods subject to biosecurity control only move beyond the site in accordance with departmental conditions and any required departmental authorisation. |
| Release | Goods and their derivatives subject to biosecurity control are dealt with as such until they are formally released from biosecurity control, or they are exported or destroyed. |
| Awareness | People performing biosecurity activities involving goods subject to biosecurity control have the knowledge and capability to carry out those activities in accordance with the conditions of the approved arrangement. |
| Inspection | The site has the equipment, facilities and processes that enable inspection of goods subject to biosecurity control. |
| Treatment | The biosecurity industry participant has the processes and/or equipment and facilities to perform treatments of goods subject to biosecurity control in accordance with the conditions of the approved arrangement. |
| Arrangement compliance | The biosecurity industry participant is required to carry out biosecurity activities in accordance with the approved arrangement. |
| Notification | The department is advised of any event or circumstance for which it has specified that notification must be provided. |
| Supporting functions | Procedures, facilities, and equipment are in place for the biosecurity activities carried out under the approved arrangement. |

## Objective

Class 2.1 approved arrangements manages biosecurity risks associated with restricted cargo, vehicles, and machinery subject to biosecurity control.

### Scope

Class 2.1 approved arrangement sites are utilised for the receipt, deconsolidation, storage, inspection, and treatment of cargo, vehicles, and machinery – goods subject to biosecurity control which are imported:

- as containerised sea cargo or as air cargo,

- with compliant commodity documentation, where specified in biosecurity import conditions applicable to the goods, and

- with compliant non-commodity documentation.

Treatment performed under class 2.1 approved arrangement is limited to cleaning (washing, steam cleaning) within a wash bay or wash cabinet to remove any contamination identified through inspection by a biosecurity officer. Performance of any other treatment will require approval from the department e.g., for the relevant approved arrangement class.

Examples of goods subject to biosecurity control that may be handled at a class 2.1 approved arrangement site include:

- vehicles (including aircraft and parts)

- boats (including yachts, canoes and kayaks)

- new, used and/or reconditioned machinery

- new, used and/or reconditioned parts and equipment

- tyres

- metal waste and scrap

- used carpets

- empty bags previously containing material of biosecurity concern e.g., plant debris

Examples of biosecurity risks associated with goods subject to biosecurity control handled at a class 2.1 approved arrangement site include:

- infestation of insects in or on the goods, packaging material or containers e.g., timber pests (e.g., termites, ants and larvae), stored product pests (e.g., beetles and larvae), insects seeking refuge (e.g., ants, bees, wasps, moths and spiders), and hitchhiker pests (e.g., brown marmorated stink bug)

- animals such as rodents, reptiles, geckos, birds, and cats that can inadvertently travel in containers

- contamination of soil or plant material in or on goods, packaging material or containers e.g., soil can contain seeds of exotic plants or weeds, plant diseases, insects and insect eggs, and soil-borne animal diseases such as foot and mouth disease

- other contamination such as water can contain mosquito larvae and algae. Mosquitos are vectors for various diseases

- plant diseases associated with goods that made of or contain plant material e.g., Karnal bunt of wheat, citrus greening, leaf blight

- animal diseases associated with goods that are made of or contain animal material e.g., foot and mouth

- Information on biosecurity import conditions and biosecurity risks for imported goods, containers, other cargo and arriving vessels is available on the departments website.

### Site location

Class 2.1 approved arrangement sites must generally be located within the metropolitan area of a first point of entry for goods (from vessels) where a permanently based biosecurity officer is stationed.

### Information required for application

The following information is required by the department at time of application for a class 2.1 approved arrangement:

- A site map which complies with conditions for approved arrangement site plans prescribed for this arrangement

## Conditions

### Table 1 Site personnel

#### Information

- Accreditation training requirements are available on the department’s website.

- Capability includes accredited persons having the understanding/skill/ability to deal with the biosecurity risks associated with their role/function.

- Personnel includes employees and others working (paid or unpaid) for the biosecurity industry participant.

- Accredited person records can be:

- copies of training certificates

- register containing the information required in the reporting conditions table.

| KAO | Condition | Noncompliance guide |
| --- | --- | --- |
| Awareness | 1.1 The biosecurity industry participant must ensure that persons having physical access to goods subject to biosecurity control are aware that such items must only be handled by an accredited person or under the direct supervision of an accredited person. | Major Reference: 3008 |
| Awareness | 1.2 An accredited person must personally conduct or directly supervise biosecurity activities involving physical contact with, handling or treatment of goods subject to biosecurity control. | Major Reference: 3988 |
| Awareness | 1.3 Accredited persons must have successfully completed accreditation training for the relevant approved arrangement class as specified on the department’s website. | Major Reference: 3003 |
| Awareness | 1.4 Accredited persons must be able to demonstrate an understanding of conditions applicable to the biosecurity activities performed under this approved arrangement. | Major Reference: 3989 |
| Awareness | 1.5 Persons having physical access to goods subject to biosecurity control must be able to differentiate between goods subject to biosecurity control and goods that are not subject to biosecurity control. | Major Reference: 4347 |
| Awareness | 1.6 Arrangements must be in place to ensure persons handling goods subject to biosecurity control (including goods owned and handled for other parties) are aware of the biosecurity conditions that apply to that handling, as prescribed in the conditions for this arrangement, as well as conditions that may be prescribed in any import permit or biosecurity direction relevant to the goods. | Major Reference: 3532 |
| Traceability | 1.7 Records must be maintained of accredited persons. | Minor Reference: 3004 |

### Table 2 Security

Information

- Loss of goods subject to biosecurity control, in relation to security at the approved arrangement site includes:

- theft

- misplaced

- unauthorised removal

- unauthorised use.

- Security measures generally applies to the approved arrangement site boundary but may be at the boundary of biosecurity areas or buildings that house the biosecurity area.

- Security measures include both infrastructure (fences, locks, electronic monitoring) and procedural practices (training) to stop unauthorised people from accessing goods subject to biosecurity control.

- Unauthorised removal of goods subject to biosecurity control is also considered to be a containment issue.

- Information on reporting biosecurity incidents is available on the department’s website.

| KAO | Condition | Noncompliance guide |
| --- | --- | --- |
| Security | 2.1 Security measures must be in place to prevent access to and removal of goods subject to biosecurity control by unauthorised persons. | Major Reference: 2970 |
| Notification | 2.2 The department must be notified immediately following the detection that there is a loss of goods subject to biosecurity control. | Critical Reference: 4451 |
| Security | 2.3 Biosecurity areas must be in a secure lockable building or in an area surrounded by a lockable security fence. | Major or critical Reference: 3941 |

### Table 3 Isolation

Information

- Goods subject to biosecurity control includes biosecurity waste.

- Contamination may be physically removed from equipment that has come in contact with goods subject to biosecurity control using a brush for example, with residues collected and placed in a biosecurity waste container. Vacuums cannot be used for cleaning purposes unless specific approval or a biosecurity direction is given by the department.

- Equipment includes but is not limited to:

- trolleys, pallet jacks and other equipment used to transport goods subject to biosecurity control

- dustpan and brush and other equipment used to clean contamination from goods

- personal protective equipment e.g., footwear, gloves, coveralls

- receptacles used to hold, store goods

- other equipment used to treat or process goods subject to biosecurity control

- Disinfection of equipment following the removal of any contamination may be required by relevant approved arrangement conditions, import conditions and biosecurity directions.

| KAO | Condition | Noncompliance guide |
| --- | --- | --- |
| Isolation | 3.1 Goods subject to biosecurity control must be kept physically separated from other goods (including during transport), including: - imported goods that have been released from biosecurity control - domestic goods - goods for export - other goods subject to biosecurity control - the Australian environment. | Major or critical Reference: 2997 |
| Isolation | 3.2 Goods subject to biosecurity control must be stored in such a manner to ensure that a minimum of one pallet width or an impervious physical barrier is maintained between goods subject to biosecurity control and other goods. | Major Reference: 2943 |
| Treatment | 3.3 Goods which come into contact with goods subject to biosecurity control must be treated as subject to biosecurity control. | Major or critical Reference: 4452 |
| Identification | 3.4 If there is any doubt as to whether goods: - are subject to biosecurity control - remain subject to biosecurity control - become subject to biosecurity control then the goods must be handled in accordance with conditions for goods subject to biosecurity control. | Major Reference: 3011 |
| Treatment | 3.5 Any equipment detected with contamination from goods subject to biosecurity control, must not leave the biosecurity area until it has been: - cleaned to remove any contamination, or - disposed of in accordance with relevant approved arrangement conditions, import conditions and biosecurity directions. | Major or critical Reference: 2999 |

### Table 4 Construction

| KAO | Condition | Noncompliance guide |
| --- | --- | --- |
| Containment | 4.1 Buildings and structures where goods subject to biosecurity control are stored/handled, including wall and floor junctions must be weatherproof and sealed. | Major Reference: 322 |
| Containment | 4.2 Biosecurity areas must have floor surfaces finished with materials that are smooth, impervious, durable, and easy to clean. | Major Reference: 3900 |
| Containment | 4.3 Biosecurity area floors must be free from defects. | Major Reference: 3901 |

### Table 5A Biosecurity area

Information

- It is a condition of the arrangement that treatment of goods subject to biosecurity control must only be performed in the biosecurity area approved for that type of treatment e.g., goods directed for treatment by:

  - washing must only be washed in the onsite wash bay/cabinet, or

  - fumigation must only be treated in the onsite fumigation area/chamber approved under approved arrangement class 4.6 – fumigation.

- Goods subject to biosecurity control includes biosecurity waste.

- Biosecurity areas include areas used for treatment, inspection, and storage of goods subject to biosecurity control.

- These areas are established to prevent goods subject to biosecurity control and their contaminants from accidental or deliberate release or escape into the Australian environment.

- It must be immediately obvious that the goods subject to biosecurity control and the area the goods are held in are subject to biosecurity control.

- Identification of biosecurity areas and goods subject to biosecurity control can be through infrastructure (signs and labels) and procedural practices (colour coding and training) and must support the traceability of the goods. Biosecurity areas with more than one entry point may need more than one sign.

| KAO | Condition | Noncompliance guide |
| --- | --- | --- |
| Arrangement compliance | 5.1 Where goods subject to biosecurity control are directed by the department for treatment at the approved arrangement site, the: - approved arrangement for that site must be approved to carry out that type of treatment activity (e.g., irradiation, cleaning, fumigation, heat treatment, specific processing), and the - treatment must only be performed in the biosecurity area approved under the approved arrangement class for that type of treatment activity. | Major or critical Reference: 4457 |
| Isolation | 5.2 Goods subject to biosecurity control must be kept in the biosecurity area. | Major or critical Reference: 2038 |
| Isolation | 5.3 Biosecurity areas must not be used as a thoroughfare or access point to non-control areas or other non-related biosecurity areas. | Major Reference: 2426 |
| Identification | 5.4 Biosecurity areas must be clearly identified as biosecurity areas to all persons with physical access to the approved arrangement site. | Major Reference: 4234 |
| Isolation | 5.5 The boundaries of biosecurity areas must be: - clearly defined - highly visible - permanently constructed or affixed, and - durable. | Major Reference: 4278 |
| Identification | 5.6 Biosecurity areas must display a biosecurity sign/s | Major Reference: 3919 |
| Identification | 5.7 Biosecurity signs must: - be securely affixed - be durable - be visible and able to be clearly read by persons approaching the area - state 'Biosecurity Area - No unauthorised entry or removal of goods, or words to similar effect. - have black lettering on yellow background - have wording that is legible Note: signs produced/installed under former legislation that state “Quarantine Area - Authorised Persons Only’ will be accepted. New signs must comply with points a - f. | Minor Reference: 3776 |

### Table 5B Biosecurity area

| KAO | Condition | Noncompliance guide |
| --- | --- | --- |
| Supporting functions | 5.8 The biosecurity area must allow for biosecurity officers to easily perform their duties without work health and safety risks. | Major Reference: 830 |
| Containment | 5.9 Where non-containerised goods subject to biosecurity control are stored outside, the surfaces on which they are stored must be impermeable and drain directly to sewer. Alternatively, where this is not possible, these items must be tarped prior to inspection or treatment | Major or critical Reference: 3902 |
| Containment | 5.10 Goods subject to biosecurity control, not stored within a building, to be treated in the wash bay must be: - tarped, or - stored under cover, or - stored in a container, or - stored on an impervious surface that drains directly to sewer until such time as it can be moved to the wash bay for cleaning. | Major Reference: 2108 |
| Movement | 5.11 Goods subject to biosecurity control being moved off site for treatment (including fumigation, cleaning, heat, and irradiation) must be: - secured to prevent the potential escape and/or spread of biosecurity risk material - transported directly to an approved arrangement site, in accordance with the applicable biosecurity direction or approval from the department. | Major Reference: 3949 |
| Movement | 5.12 Goods subject to biosecurity control must not leave the biosecurity area of an approved arrangement site, inadvertently or deliberately, without prior biosecurity direction or approval from the department. | Critical Reference: 4236 |

### Table 6 Wash facilities

| KAO | Condition | Noncompliance guide |
| --- | --- | --- |
| Treatment | 6.1 Wash facilities must: - be of a size commensurate with the goods subject to biosecurity control being cleaned - be constructed of impermeable material and in a manner, which ensures that all residues/sediment and splash from cleaning operations are contained and flow directly into holding tanks or a municipal sewage system or department approved system - have sewage/drainage lines protected from physical damage. Note: Where municipal sewage connection is not available and tanks are used, the tanks must be fully enclosed. | Major Reference: 2975 |
| Treatment | 6.2 There must be adequate equipment available to carry out cleaning (steam/high pressure) and chemical disinfection spraying operations as directed by biosecurity officers. | Major Reference: 233 |
| Containment | 6.3 Soil traps must be installed in drains in locations where drainage inflow is likely to contain solids from goods subject to biosecurity control. | Major Reference: 2976 |
| Treatment | 6.4 Waste water must be either: - screened prior to discharge to municipal sewer or department approved treatment/holding tank - collected and transported off-site for discharge to sewer by a waste transport service operating under an appropriate approved arrangement with the department - collected and transported off-site for discharge to sewer by non-department approved waste transport service under department supervision - collected into a tank and hypochlorite treatment applied. | Major Reference: 1393 |

### Table 7 Wash facilities – wash cabinets

| KAO | Condition | Noncompliance guide |
| --- | --- | --- |
| Treatment | 7.1 Wash cabinets must be constructed in accordance with the following requirements, the: - cabinet must be constructed of sturdy, impermeable material - cabinet must have three fixed sides and a fixed top - sides of the cabinet must be attached to the base in a manner which prevents the escape of any wash water or cleaning residue - cabinet must incorporate, or be attached to, a soil trap - disposal system must include a grease trap which is connected to council approved sewer or another waste water system approved by the department. There must be no trade waste diversion valves in place. - drains and tanks must be covered, and sewerage/drainage lines must be protected from physical damage. | Major Reference: 763 |

### Table 8 Wash facilities – wash bays

| KAO | Condition | Noncompliance guide |
| --- | --- | --- |
| Isolation | 8.1 No unauthorised traffic or personnel (those not involved in the cleaning of goods subject to biosecurity control) or equipment are permitted on the wash pad, ramp or wash bay during treatments and cleaning. | Major Reference: 29 |
| Containment | 8.2 The wash bay must - have a floor impervious to liquid - have a floor drain leading into a soil trap which is connected directly to municipal sewer or to another waste water disposal system approved by the department. - ensure that any waste water storage, treatment and reticulation systems are enclosed. - ensure that any residues/sediment and splash from cleaning operations are contained and flow directly into holding tanks or a municipal sewerage system or department approved system - have sewage/drainage lines protected from physical damage. Note: where municipal sewage connection is not available and tanks are used, the tanks must be fully enclosed | Critical Reference: 2963 |
| Inspection | 8.3 A car hoist, inspection pit, ramp or stand must be made available, where required, for the inspection of the undercarriage of cars. | Major Reference: 433 |
| Containment | 8.4 The areas immediately adjacent to the wash bay must be impervious to liquid. | Major Reference: 2962 |
| Containment | 8.5 Waste water and residues from cleaning operations must be contained within the approved wash bay. | Major or critical Reference: 2957 |
| Treatment | 8.6 Waste water and residues from cleaning operations must only be discharged via the wash bay drain. | Major or critical Reference: 2958 |
| Treatment | 8.7 Soil residues from the wash bay soil trap or filter must be disposed of as biosecurity waste. | Critical Reference: 2961 |
| Treatment | 8.8 Equipment (including footwear and protective clothing) used in the wash bay must be cleaned prior to removal from the wash bay. | Critical Reference: 2960 |
| Treatment | 8.9 The wash bay must be washed down after cleaning operations each day. | Major Reference: 2959 |
| Treatment | 8.10 Where a trade water diversion is installed in the wash bay: - the wash bay must be cleaned after every wash - all residues and water from washing goods subject to biosecurity control and cleaning the wash bay after every wash must be discharged to sewer (and not to stormwater) - if the trade water diversion cannot be manually overridden to discharge to sewer, good subject to biosecurity control must not be washed during periods of high rainfall that could result in water being discharged to stormwater - if the trade water diversion can be manually overridden, good subject to biosecurity control can be washed at any time if all wash water must be discharged to sewer. | Critical Reference: 1394 |

### Table 9 Recycling waste water

Information

- Waste water recycling systems are typically a closed system, servicing only the wash bay or wash cabinet. Water used for washing activities will be collected and retained in a dedicated holding tank for reuse in further wash activities. These systems may be topped up with water from tap mains or other rain tanks.

| KAO | Condition | Noncompliance guide |
| --- | --- | --- |
| Treatment | 9.1 A recycling waste water system must incorporate screening the liquid waste through a 100-micron filter prior to use in wash bay operations with goods subject to biosecurity control, including final washes. Recycled waste water is not permitted to be used for domestic washes. | Major Reference: 2971 |
| Supporting function | 9.2 During the recycling process, the biosecurity industry participant must, when operational, undertake weekly inspection for leaks from: - pumps - valves - tanks (where applicable) - filter housing, pipes and connections where visible. | Major Reference: 2972 |
| Supporting function | 9.3 When leaks are detected they must be immediately repaired. | Major Reference: 2973 |
| Treatment | 9.4 Waste filter media and detritus/refuse captured by filter media or screens must be treated as biosecurity waste. | Major Reference 3281 |

### Table 10 Site hygiene and monitoring

#### Information

- Standard of hygiene will vary for each approved arrangement site dependant on the biosecurity activities that occur at that site.

- Site monitoring is used to identify animals, invertebrates and vegetation and minimise the potential for biosecurity risks to escape into the Australian environment.

- Information on reporting biosecurity incidents is available on the department’s website. Incidents that must be reported includes where ‘the goods are infested with a live pest (for example, an insect, invertebrate or other animal)’.

| KAO | Condition | Noncompliance guide |
| --- | --- | --- |
| Hygiene | 10.1 An effective pest and weed control system must be in place to ensure that goods subject to biosecurity control are isolated from environments in which pest and disease are likely to become established. The biosecurity industry participant must implement and keep associated records of a periodic inspection regime and ensure knockdown spray (i.e., standard household aerosol insecticide spray) is kept onsite. The onsite location of the knockdown spray, the pest and weed control system must include: - the use of insecticides, fumigation, rodenticides, periodic inspection, baits and/or traps - a site plan with numbered bait stations - if applicable, contract details. Note: the operations of adjacent facilities must be considered when determining any additional pest control measures to be implemented. | Minor or major Reference: 2102 |
| Hygiene | 10.2 The standard of hygiene at the approved arrangement site must be appropriate for the nature of the goods subject to biosecurity control. | Minor or major Reference: 2998 |
| Hygiene | 10.3 Buildings and biosecurity areas must be kept clean. Cargo and packaging residues, contaminants and spillages must be cleaned up and correctly disposed of as biosecurity waste without delay | Minor or major Reference: 1003 |
| Hygiene | 10.4 Cargo and packaging residues and spillages: - from goods subject to biosecurity control, or that are - detected in areas where goods subject to biosecurity control are, or have been handled must be cleaned up immediately and disposed of as biosecurity waste. | Major or critical Reference: 4467 |
| Notification | 10.5 The department must be notified immediately where cargo and packaging residues and spillages from goods subject to biosecurity control cannot be cleaned up immediately and/or disposed of as biosecurity waste. | Major Reference: 4468 |
| Arrangement compliance | 10.6 The biosecurity industry participant must provide the department access to the approved arrangement site for plant pest and disease surveillance purposes including the installation and monitoring of insect traps. | Major Reference: 4481 |
| Arrangement compliance | 10.7 Insect traps installed by the department must not be interfered with or damaged. | Major Reference: 4482 |

### Table 11 Internal container inspections

| KAO | Condition | Noncompliance guide |
| --- | --- | --- |
| Inspection | 11.1 Either an approved container inspection ramp or equipment to place the container safely and adequately on the ground for inspection must be available. If no container inspection ramp is provided, an area with an impervious surface must be available for conducting internal container inspections. | Major Reference: 158 |
| Supporting function | 11.2 Labour and equipment required to open the container doors must be supplied by the biosecurity industry participant. | Critical Reference: 385 |

### Table 12 Facilities for departmental officers

| KAO | Condition | Noncompliance guide |
| --- | --- | --- |
| Supporting functions | 12.1 The biosecurity industry participant must: - provide a first aid cabinet/kit which is fully stocked and meets the minimum commercial Australian Standard (AS2675-1983: Portable first aid kits for use by consumers) - provide vehicle parking for visiting biosecurity officers (Note: This may require department identified parking or providing a parking permit) - ensure adequate security for any departmental technical equipment left on the site - provide access and the availability of: - a desk, chair, and a telephone with direct outside call access - toilet facilities - handwashing facilities and a hygienic means of drying hands - clean amenities. | Minor or major Reference: 2098 |

### Table 13 Waste

#### Information

- Biosecurity waste generated from washing operations includes but is not limited to:

- residues removed from goods subject to biosecurity control

- water used for washing goods subject to biosecurity control and cleaning the wash bay or cabinet

- soil residues from soil traps in drains

- sludge, sediment from grease traps and other filters, separators

- other liquids such as fats, oil and grease filtered/separated from waste water

- waste filter media and detritus/refuse captured by filter media or screens

- For any dunnage, packing material and waste inspected and found to contain pests or disease agents, the department will direct the biosecurity risk be managed using a department approved treatment.

- Department approved disposal methods include:

| Solid biosecurity waste disposal | Liquid biosecurity waste disposal |
| --- | --- |
| Incineration (class 8.1) | Municipal sewer Hypochlorite treatment |
| Deep burial (class 8.2) |  |
| Autoclave (class 8.3) |  |
| As prescribed in the import permit or other biosecurity direction. |  |

- Disposal of liquids such as oil to municipal sewer may be restricted by local, state/territory, federal government regulations and relevant state Environmental Protection Agency (EPA) requirements. Methods approved for solid biosecurity waste disposal (above) may be utilised for liquid waste disposal, where permitted by local, state/territory, federal government regulations and relevant state EPA requirements.

| KAO | Condition | Noncompliance guide |
| --- | --- | --- |
| Containment | 13.1 Waste containers must be provided for loose items, contamination, residues, spillages, or material from goods subject to biosecurity control. | Major Reference: 4041 |
| Isolation | 13.2 The biosecurity waste container must: - be leak and pest-proof - have a secure lid. | Major or critical Reference: 3943 |
| Identification | 13.3 The biosecurity waste container must be labelled ‘Biosecurity Waste’. | Minor Reference: 4260 |
| Containment | 13.4 Lids of biosecurity waste containers must remain closed except when opened to add or remove biosecurity waste. | Major or critical Reference: 4509 |
| Treatment | 13.5 Loose goods of biosecurity concern that do not form part of the documented consignment (including soil, seeds, hessian bags, fruit cartons, plant, or animal contamination) as well as any spilled goods that are of biosecurity concern must be placed in the biosecurity waste container. | Major Reference: 1053 |
| Movement | 13.6 Solid biosecurity waste disposal must only be undertaken at an approved arrangement site that is approved for waste disposal. | Major or critical Reference: 4340 |
| Movement | 13.7 Biosecurity waste must be transported by a department approved waste transporter. | Major Reference: 2097 |
| Treatment | 13.8 Biosecurity waste must be identified as such to the waste disposal company and waste transporter. | Major or critical Reference: 1091 |

### Table 14 Waste water hypochlorite treatment

#### Information

- Hypochlorite treatment of liquid water is an alternative treatment option to discharging liquid waste to municipal sewer. It is typically used where there is no connection to sewer at the approved arrangement site.

- Once hypochlorite treatment is completed, the treated liquid waste can be discharged to any location, where permitted by local, state/territory, federal government regulations and relevant state EPA requirements.

| KAO | Condition | Noncompliance guide |
| --- | --- | --- |
| Treatment | 14.1 A hypochlorite treatment system must incorporate: - screening liquid waste through a 100-micron filter prior to treating with hypochlorite. - an enclosed liquid waste treatment tank with an attached mechanical agitation method for mixing hypochlorite and liquid waste. | Major Reference: 3523 |
| Treatment | 14.2 Hypochlorite treatment of waste water (e.g., liquid waste, imported fresh water) must be undertaken in accordance with the following process: - filtration through a 100-micron filter - liquid waste test to ensure a pH range between 5.0 and 7.0 (where the pH is not within this range, add acid or alkaline products and bring the liquid waste to within this range), then - add hypochlorite to achieve 200 parts per million (ppm) free chlorine (at the end of the 10 minutes agitation cycle), then - mechanically agitate in a retention vessel for 10 minutes, then - test (after agitation) to determine the free chlorine level is at least 200 ppm, then - retain the waste water in the treatment tank for 1 hour following confirmation of concentration at minimum 200 ppm, then - test that the concentration of the waste water is at least 5 ppm free chlorine at the conclusion of the 1-hour treatment period. | Major Reference: 3524 |
| Treatment | 14.3 Hypochlorite must be used within either: - an expiry time frame as specified by the manufacturer, or - used within two years of the manufacture date | Major Reference: 3347 |
| Supporting functions | 14.4 During the treatment of contaminated or potentially contaminated liquids the biosecurity industry participant must, when operational, undertake weekly inspection for leaks from: - pumps - valves - tanks - hypochlorite metering/dosing equipment (where applicable) - filter housing, pipes, and connections where visible. | Major Reference: 3526 |
| Containment | 14.5 When leaks are detected they must be immediately repaired. | Major Reference: 3527 |
| Treatment | 14.6 All waste filter media and detritus/refuse captured by filter media or screens must be treated as biosecurity waste. | Major Reference 3281 |

### Table 15A Records

#### Information

- Records and systems maintained by the biosecurity industry participant must be able to track goods subject to biosecurity control through the stages of the biosecurity goods pathway they are responsible for. This includes receipt, handling, treatment, disposal, and release.

- There must be two-way traceability, from the:

  - records to the physical goods

  - physical goods to the records.

| KAO | Condition | Noncompliance guide |
| --- | --- | --- |
| Traceability | 15.1 Goods subject to biosecurity control must be traceable in terms of (where applicable): - biosecurity direction number - import permit number - Air Waybill or Bill of Lading number - date of receipt - processing (including inspection, treatment, testing) details - release from biosecurity control - disposal details - storage location. | - Major - Major - Minor - Major - Major - Major - Major - Major Reference: 3517 |
| Traceability | 15.2 Records for each consignment of goods subject to biosecurity control must include: - biosecurity direction - Import Permit number (if applicable) - description of the goods subject to biosecurity control (including scientific names and quantities, if applicable) - date of receipt of goods and country of origin - details of any treatments - method and date of disposal/destruction of goods subject to biosecurity control and/or biosecurity waste - the date of movement and the department's permission for any movement of goods subject to biosecurity control - comprehensive details of any breaches of goods subject to biosecurity control from the approved arrangement site. | Minor, major or critical Reference: 2101 |
| Traceability | 15.3 Accredited persons records must include: - name of accredited person - date accreditation training completed - method of accreditation training (online or in-house) - copy of online training accreditation certificate – if applicable - copy of in-house training attendance record – if applicable. | Major Reference: 4275 |
| Traceability | 15.4 The biosecurity industry participant must ensure records are kept for a minimum of two years for goods subject to biosecurity control. | Minor or major Reference: 4004 |
| Traceability | 15.5 Records must be made available to the department upon request. | Minor or major Reference: 3944 |

### Table 15B Records

| KAO | Condition | Noncompliance guide |
| --- | --- | --- |
| Traceability | 15.6 Hypochlorite treatment records must include: - date and times of testing (e.g., times when testing of concentration is taken) - initial pH of liquid waste - pH adjustment (where required) i.e., initial pH and final pH (after addition of acid or alkali) - amount of hypochlorite added - concentration of free chlorine in treatment tank after agitation - amount of additional hypochlorite added (where required) - concentration of free chlorine in treatment tank after further agitation (when additional hypochlorite added) - concentration of free chlorine at conclusion of 1 hour treatment and time treatment completed - date of manufacture of hypochlorite. | Major Reference: 3529 |

### Table 16 Ceasing operations

Information

- The biosecurity industry participant is liable for all costs associated with closing and ceasing operations as an approved arrangement site.

| KAO | Condition | Noncompliance guide |
| --- | --- | --- |
| Notification | 16.1 The department must be notified, by email to aa.canberra@awe.gov.au, at least 15 business days prior to intended: - closure of a current approved arrangement site - relocation of the business, including the approved arrangement class function - ceasing of operation as an approved arrangement site. | Major Reference: 3699 |
| Treatment | 16.2 Any goods subject to biosecurity control that remain at the approved arrangement site must be: - treated or destroyed in accordance with a department approved method or - transferred to another approved arrangement site with prior approval from the department. | Major Reference: 4516 |

### Table 17 Site maps

| KAO | Condition | Noncompliance guide |
| --- | --- | --- |
| Identification | 17.1 Current site plans must be provided to the department. Site plans must be to scale and include biosecurity areas (storage, inspection, treatment/processing) and the location of parking for biosecurity officers. | Major Reference: 680 |
| Identification | 17.2 The location and size of the wash bay must be clearly marked on the site plan. | Minor Reference: 2977 |
| Identification | 17.3 The location and size of the wash cabinet must be clearly marked on the site plan. | Minor Reference: 2984 |

### Table 18A Compliance

Information

- Release from biosecurity control includes release by a biosecurity industry participant subject to s162 Biosecurity Act 2015 only if expressly authorised in the approved arrangement and in accordance with the conditions of the approved arrangement.

- Required treatments will be advised on import permits, directions, class conditions, non-standard conditions (variations), process management systems (PMS) and standard operating procedures (SOPs).

- Those events to be notified are advised on import permits, directions, class conditions, non-standard conditions (variations), PMS and SOPs.

- Information on reporting biosecurity incidents is available on the department’s website.

Note: SOPs are only required in those classes where there is a specific condition for a SOP to be in place.

| KAO | Condition | Noncompliance guide |
| --- | --- | --- |
| Arrangement compliance | 18.1 The biosecurity industry participant must: - carry out the biosecurity activities in accordance with the arrangement - comply with any conditions specified in the arrangement - comply with any requirements to which the arrangement is subject - comply with any biosecurity direction from the department - comply with any biosecurity control notice from the department - comply with any biosecurity control order from the department. | Major or critical Reference: 4316 |
| Arrangement compliance | 18.2 Goods subject to biosecurity control must be maintained and processed in accordance with the conditions of the relevant approved arrangement class. | Major or critical Reference: 2992 |
| Arrangement compliance | 18.3 Goods subject to biosecurity control must be maintained and processed at an approved arrangement site appropriate for the biosecurity risk associated with the goods. | Minor, major or critical Reference: 3535 |
| Arrangement compliance | 18.4 Goods subject to biosecurity control must be maintained and processed in accordance with import conditions specified in the department's Biosecurity Import Conditions Database (BICON). | Minor, major or critical Reference: 2993 |
| Arrangement compliance | 18.5 Goods subject to biosecurity control must be maintained and processed in accordance with any applicable import permit. | Major or critical Reference: 3536 |
| Arrangement compliance | 18.6 Goods subject to biosecurity control must be maintained and processed in accordance with any biosecurity direction from the department. | Minor, major or critical Reference: 2995 |
| Arrangement compliance | 18.7 Goods subject to biosecurity control must be handled and maintained and processed in accordance with the Biosecurity Act 2015 and subordinate legislation. | Minor, major or critical Reference: 2996 |

### Table 18B Compliance

| KAO | Condition | Noncompliance guide |
| --- | --- | --- |
| Notification | 18.8 The department must be notified immediately following detection of any of the following: - goods subject to biosecurity control have left the approved arrangement site, inadvertently or deliberately, without prior written direction or approval from the department - goods subject to biosecurity control were not maintained and processed in accordance with the conditions of the relevant approved arrangement class - goods subject to biosecurity control were not maintained processed in accordance with import conditions specified in the department's Biosecurity Import Conditions Database (BICON) and/or import permit - goods subject to biosecurity control were not maintained and processed in accordance with any other direction from the department - goods subject to biosecurity control were not maintained and processed in accordance with the Biosecurity Act 2015 and subordinate legislation. - the approved arrangement site is not appropriate to manage the biosecurity risk associated with the goods subject to biosecurity control. | Major or critical Reference: 4517 |
| Notification | 18.9 The department must be notified of any reportable biosecurity incident as soon as practicable, in accordance with the determination made by the Director of Biosecurity. | Major Reference: 3015 |
| Arrangement compliance | 18.10 Departmental officers and department approved auditors, must be provided access to the approved arrangement site to perform the functions and exercise the powers conferred on them by the Biosecurity Act or another law of the Commonwealth. | Critical Reference: 3013 |
| Arrangement compliance | 18.11 Biosecurity officers must be provided with all reasonable facilities, assistance, and information necessary for the effective performance of their functions or duties or the exercise of their powers under the Biosecurity Act. Note: Assistance includes but is not limited to moving, lifting and sampling goods subject to biosecurity control, as directed by a biosecurity officer to enable inspection or other biosecurity activities to be performed. | Major or critical Reference: 4518 |
| Arrangement compliance | 18.12 Information (verbal, electronic or hard copy) provided to the department must be accurate. Note: Civil, criminal, and regulatory penalties apply to giving false or misleading information. | Major or critical Reference: 4519 |
| Arrangement compliance | 18.13 Department approved auditors must be permitted to collect evidence of compliance and noncompliance with approved arrangement conditions through actions including the copying of documents and taking of photographs. | Major or critical Reference: 3016 |
| Arrangement compliance | 18.14 A contingency plan must be in place to manage unexpected events that threaten to compromise biosecurity integrity of the approved arrangement site. Unexpected events include: - appearance of pests or symptoms of disease - structural damage (due to storms etc.) - accident during transit - unauthorised removal of goods subject to biosecurity control - spillages of goods subject to biosecurity control - sudden unavailability of an accredited person. | - Major - Major - Major - Major - Major - Major Reference: 3009 |
| Notification | 18.15 The department must be notified, by email to aa.canberra@awe.gov.au, at least 15 business days prior to any modification to, or closure of, biosecurity areas where goods subject to biosecurity control are stored or treated/processed or otherwise dealt with. | Major or critical Reference: 4520 |

### Table 18C Compliance

| KAO | Condition | Noncompliance guide |
| --- | --- | --- |
| Arrangement compliance | 18.16 Departmental approval must be obtained prior to implementing changes to the location and/or construction of the approved arrangement site or biosecurity area boundaries. | Major Reference: 4024 |
| Notification | 18.17 The department must be notified by email to aa.canberra@awe.gov.au of any proposed changes to any of the following: - The location and/or construction of any part of the approved arrangement site boundary - The location and/or construction of any part of a biosecurity area boundary - The location and/or construction of any part of the boundary of any areas within the approved arrangement site boundary that are under the: - occupancy or control, or - shared occupancy or control of any other entities (note that this includes subleasing arrangements) - The identity of any other entities that: - occupy or control, or - share occupancy or control of any areas within the approved arrangement site boundary (note that this includes subleasing arrangements). | Major or critical Reference: 4521 |
| Notification | 18.18 The biosecurity industry participant must notify the department, by email to aa.canberra@awe.gov.au, within 15 business days of becoming aware of any change of status, not previously been notified to the department, of the biosecurity industry participant or their associates relevant to the operation of the approved arrangement in relation to any of the following matters: - conviction of an offence or order to pay a pecuniary penalty under the Biosecurity Act 2015, Quarantine Act 1908, Customs Act 1901, the Criminal Code, or the Crimes Act 1914 - debt to the Commonwealth that is more than 28 days overdue under the Biosecurity Act 2015, Quarantine Act 1908, Customs Act 1901, the Criminal Code, or the Crimes Act 1914 - refusal, involuntary suspension, involuntary revocation/cancelation or involuntary variation of an Import Permit, quarantine approved premises, compliance agreement or approved arrangement under the Quarantine Act 1908 or the Biosecurity Act 2015. | Critical Reference: 3012 |
| Notification | 18.19 The biosecurity industry participant must notify the department by email to aa.canberra@awe.gov.au, within 15 business days of any change in: - persons in positions responsible for controlling, directing, enforcing, or monitoring people performing biosecurity activities associated with the approved arrangement - biosecurity industry participant details, including: - entity name - Australian business number or Australian company number - postal address - email address - telephone number. | Major or critical Reference: 3836 |