February 2026

Methyl Bromide Fumigation Methodology v3.0

# Completing a Record of Fumigation (ROF)

This fact sheet explains the mandatory information required in a ROF for fumigations performed under the Methyl bromide fumigation methodology v3.0. Section 12.2 of the methodology defines all mandatory information.

A ROF must:

be completed for every treatment, including failed treatments

be completed at the fumigation site as the tasks are performed

be kept for 2 years

not contain false or misleading information.

## Record of Fumigation (ROF) Template

Use of the ROF template (Appendix 1 and Appendix 2) is not mandatory. It is provided as a resource to help treatment providers prepare their own ROF. The template may assist in meeting the required conditions; however, compliance is determined by whether the completed ROF includes all requirements outlined in Section 12 of the Methyl Bromide Fumigation Methodology v3.0.

Some sections of the template have been included based on feedback from industry to make the tool more practical and user-friendly.

Ensure documents or IT systems align with the requirements in Section 12. Do not rely on the template alone to determine compliance.

Tip: Using field notes

If field notes are used at the time of fumigation to record fumigation details and transcribe the information later, these field notes form part of the official record and must be kept with the fumigation documentation for two years.

The original field notes must be kept with the fumigation records. This helps show what happened during the treatment and reduces the risk of transcription errors. Without the original notes, it can be harder to confirm compliance or resolve discrepancies during audits.

For offshore treatment providers who are registered under AusTreat or AFAS and use the certificate portal, the field notes must be uploaded into the certificate portal with the original ROF. Field notes presented later may not be accepted as evidence.

### Section A: Fumigator in charge

Complete this section with the details of the fumigator in charge. The person named here must also complete the declaration in Section E of the form.

Fumigator in Charge: The licenced and/or accredited individual that is responsible for the conduct of the fumigation at the time specific fumigation activities are undertaken. Refer to: Clause 12.2.5 (t), and Glossary

Accreditation number: This is a number associated with an individual and relates to the fumigator-in-charge who will be signing the form. For example, fumigation licence number.  Not a mandatory field – Refer to relevant import/export policy for details.

### Section B: Job details

Complete this section with information about where the fumigation was carried out and who it was completed for.

Include site details, client information, and any references that link the fumigation to a specific consignment.

Treatment Provider ID: Number issued by regulatory authority.  E.g. AEI, AA number, RE number. Refer to: Clause 12.2.5 (a)

Job identification number: Number issued by regulatory authority.  Number that a company may use to identify a certain job. Not a mandatory field.

Location of fumigation: The location (site address) where the fumigation is performed. Refer to: Clause 12.2.5 (d)

Client name: The person or company that had requested the fumigation. Usually the person responsible for the goods. Refer to: Clause 12.2.5 (b)

Consignment identification: Container number(s), bill of lading, silo/shed number or other means to clearly identify the consignment. Refer to: Clause 12.2.5 (g)

Description of the consignment: Provide a clear description of what is being fumigated and the consignment contents. This helps confirm exactly what the treatment covered. For example: “Treating timber backing for seats in a sheeted stack” or “Fumigating empty sea container and dunnage” or “Treating rice in 25kg bags. Also, in container retail ready spices not requiring treatment”. This information ensures the treatment can be accurately assessed and verified. Refer to: Clause 12.2.5 (e)

Target of fumigation: The specific object or area that is intended to be treated through the fumigation process. The target of fumigation may be the commodity, packaging material, container, or conveyance or combination of these. Refer to: Clause 12.2.5 (f)

Important: Accurate recording of the target of fumigation helps provide a clear picture if a live pest is detected in a consignment.

For example: If the fumigator treats only the timber dunnage and a pest is later found within the goods, like rice, the fumigator may not be held accountable.

Tip: Description of the consignment for multiple sea containers in a sheeted enclosure

If fumigating multiple sea containers under a sheeted enclosure and the contents or treatment targets differ, each container may require a separate approach.

Variations in consignment description, treatment targets, and suitability mean that a general description may not ensure compliance for all containers.

Careful assessment of each consignment is essential to meet biosecurity requirements, the consignment and target of fumigation details for every sea container must be recorded.

### Section C: Fumigation Details

Complete this section with the specific details of the fumigation treatment being performed.

#### Specified treatment schedule

Specific treatment rates, exposure period and rules as imposed by the relevant authority – usually the importing jurisdiction. The treatment schedule will specify, dose rate, exposure period and temperature. Some treatment schedules have additional requirements including, but not limited to, load factor and minimum end point readings.

Dose Rate:  The prescribed concentration of methyl bromide to be used per unit of volume and the exposure period e.g. 80g/m3. Refer to: Glossary

Exposure Period: The amount of time, in one continuous block, that the consignment must be exposed to sufficient concentration levels of methyl bromide to be lethal to the targeted pests. Refer to: Section 8.6

Temperature: The temperature stated in the treatment schedule, for example, 10°C. Refer to: Section 12.3.3 (c).

An example of a treatment schedule

Dose rate: 24 g/m³ or above,

Exposure period: minimum of 12 hours (but less than 24 hours),

Temperature: at 10°C or above,

Additional requirement: a minimum start time concentration reading must be above 24 g/m3,

Additional requirement: a minimum end point reading of 12 g/m³.

#### Consignment suitability

Assess the suitability of the consignment as per the methodology and record result. Refer to: Section 3

Before a fumigation can proceed, the consignment must be made suitable in accordance with the methodology. If it cannot be made suitable, the treatment must not go ahead. You need to check things like:

packaging and wrappings: remove or perforate anything that blocks gas penetration

arrangement of goods: ensure gas can circulate to the target of fumigation; do not exceed 80% load for perishables

timber and coatings: surfaces must not be sealed or coated in a way that prevents gas access.

#### Fumigation Type

Record the type of fumigation performed. Fumigations can be either temperature-controlled fumigation or subject to the ambient temperature. The fumigation types are defined as:

Ambient temperature fumigation: the enclosure being fumigated is subject to environmental ambient temperatures or outdoors.

Controlled temperature fumigation: an artificial heat source is used to heat and maintain the temperature of an enclosure during a fumigation. Refer to: Clause 8.2.4

#### Enclosure type

Record the type of enclosure fumigation was performed in. Fumigations are conducted in a variety of different enclosures; different conditions apply to different enclosure types.

Common enclosure types are:

Sheeted enclosure: An enclosure created under a gas-proof sheet that is covering/enclosing the commodities to be fumigated. Includes sheeted sea containers and sheeted stacks. Section 4.2

Fumigation Chamber: A gas-tight fumigation enclosure with an inbuilt extraction system. All requirements for fumigation chambers specified in section 4.3 Fumigation chambers. Section 4.3

Un-sheeted container: Sea container suitable for fumigation. Section 4.1

If the enclosure that you use is not listed, for example silo or ship hold, provide these details under ‘other’, or you can change your ROF to suit your needs.

Refer to: Clause 12.2.5 (i) and Glossary

#### Enclosure volume

Record the enclosure dimensions and calculate enclosure volume. Refer to clauses 8.2.2, 8.2.3

#### Dose

In this section show your working for how the actual amount of fumigant applied was calculated.

Dose rate used: The rate used after temperature has been taken into consideration. This is the number used to calculate the dose.  Refer to: Section 6 and 8.1

Calculated dose: The weight of methyl bromide required to meet the treatment schedule. The calculation is: Dose rate used x Enclosure volume (account for chloropicrin if required). Refer to: clause 8.2

Chloropicrin: If the methyl bromide is mixed with another gas, compensation must be made to the dose amount, so the full amount of methyl bromide required is injected into the enclosure. Record the percentage of chloropicrin in the methyl bromide. Refer to: Clause 8.2.4

Amount CH3Br applied: This is the ‘dose’. The actual volume of methyl bromide injected into the enclosure, expressed as weight of methyl bromide. Once the dose is calculated, the amount must be rounded up to the next increment that can be accurately measured by the equipment used to dispense methyl bromide. If methyl bromide is supplied in cans, the dose must be rounded up to the next full can. Refer to: Clause 8.2.5, 12.2.5.

### Section D: Concentration readings

Complete this section with the concentration readings taken during all stages of the fumigation.

#### Serial numbers of monitoring device

Record the serial number of the gas concentration monitoring device(s) used. Record a minimum of the last 4 digits of the serial number for each device used. Refer to: Clause 12.2.5(r)

#### Concentration

Record all concentration readings taken. Refer to: Clause 12.2.5 (p).

The sections must be completed as follows:

Equilibrium: Record the results of the equilibrium calculations. If more than one calculation is required record all the results. Refer to: Section 8.5, 12.2.5 (q)

Concentration readings: Record readings from all monitoring locations exactly as shown by the device. For example, you must put 4.3 if your device reads 4.3 not 4 or “>4”.

Standard (g/m3): Record the standard concentration required for the time the concentration readings were taken. Refer to: Appendix 4: Methyl bromide monitoring tables, 12.2.5 (k)

Date/Time: Record the date and time the concentration readings are taken. Record the time after the readings are taken for concentration reading stage, not at the start. Refer to: 12.2.5 (p)

Fumigator’s initials: Record the initial or signature of the fumigator who took the readings at each concentration reading stage and TLV reading. This may be different for each stage and can be different to the fumigator in charge. Refer to: 12.2.5 (u)

Time and date: Enclosure ventilation start: record the time and date when enclosure ventilation starts.

Final TLV readings: Record the TLV with the date and time it was taken. If the first reading is below 5ppm, you only need to record that one. Extra space is provided if more readings are needed. Refer to: Clause 12.2.5 (s)

#### Top-up details

If a top-up is performed, record the information in this section. All top-ups must be recorded on the ROF.

Amount (g): this is the additional dose injected. Top-up amount (g) = Enclosure volume (m³) x (Maximum to allow top-up – Lowest concentration reading): Refer to: Clause 10.3.1, 12.2.10. The template in the methodology 3.0 has a typographic error. Amount should state ‘g’ not ‘g/m3’

Time: Time additional fumigant was injected. Refer to: Clauses 10.3.1, 12.2.10.

Concentration (g/m3): Concentration reading taken from the monitoring location that had the lowest reading to verify that the methyl bromide is at or above the standard concentration. Refer to Clause 10.3.4, 10.3.5. 12.2.10.

### Section E: Fumigation declaration

Complete this section with your declaration that the fumigation complied with all relevant conditions and the information on the ROF is true and correct. The fumigator in charge, as named in Section A must sign and date the ROF Refer to: Section 12.2.5 (t)

#### Fumigation results

You must indicate if the fumigation was a pass or a fail. ROF’s of failed fumigations must also be kept for 2-years. Refer to: Clause 12.2.3.

### More information

Learn more about Methodologies and documents for biosecurity treatments - DAFF.

Web agriculture.gov.au

Email offshoretreatments@aff.gov.au

Acknowledgement of Country

We acknowledge the continuous connection of First Nations Traditional Owners and Custodians to the lands, seas and waters of Australia. We recognise their care for and cultivation of Country. We pay respect to Elders past and present, and recognise their knowledge and contribution to the productivity, innovation and sustainability of Australia’s agriculture, fisheries and forestry industries.

© Commonwealth of Australia 2026

Unless otherwise noted, copyright (and any other intellectual property rights) in this publication is owned by the Commonwealth of Australia (referred to as the Commonwealth).

All material in this publication is licensed under a Creative Commons Attribution 4.0 International Licence except content supplied by third parties, logos and the Commonwealth Coat of Arms.

The Australian Government acting through the Department of Agriculture, Fisheries and Forestry has exercised due care and skill in preparing and compiling the information and data in this publication. Notwithstanding, the Department of Agriculture, Fisheries and Forestry, its employees and advisers disclaim all liability, including liability for negligence and for any loss, damage, injury, expense or cost incurred by any person as a result of accessing, using or relying on any of the information or data in this publication to the maximum extent permitted by law.