# National Pollutant Inventory

# Emission estimation technique manual

# for

# Fuel and organic liquid storage

# Version 3.3

# May 2012

ISBN: 0642 548064

 Commonwealth of Australia 2012

This manual may be reproduced in whole or part for study or training purposes subject to the inclusion of an acknowledgment of the source. It may be reproduced in whole or part by those involved in estimating the emissions of substances for the purpose of National Pollutant Inventory (NPI) reporting. The manual may be updated at any time. Reproduction for other purposes requires the written permission of the Department of the Environment, Water, Heritage and the Arts, GPO Box 787, Canberra, ACT 2601, e-mail: Unpi@environment.gov.auU, web: Uwww.npi.gov.au,U phone: 1800 657 945.

# Disclaimer

The manual was prepared in conjunction with Australian states and territories according to the National Environment Protection (National Pollutant Inventory) Measure.

While reasonable efforts have been made to ensure the contents of this manual are factually correct, the Australian Government does not accept responsibility for the accuracy or completeness of the contents and shall not be liable for any loss or damage that may be occasioned directly or indirectly through the use of, or reliance on, the contents of this manual.

# Emission Estimation Techniques

# For

# Fuel and Organic Liquid Storage

# Table of Contents

Disclaimer	i

1	Introduction	1

1.1	The process for NPI reporting	2

1.2	Information required to produce an annual NPI report	2

1.3	Additional reporting materials	2

2	Process description	3

2.1	Fuels and organic liquids	3

2.2.1 Bio-fuels	3

2.2	Tank types	4

3	Emission sources	5

3.1	Emissions to air	5

3.1.1	Fugitive emissions	5

3.1.2	Point source emissions	5

3.2	Emissions to water	5

3.3	Emissions to land	5

4	Threshold calculations	6

4.1	Fuel storage	6

4.1.1	Calculating usage (and thresholds) of stored fuels	8

4.2	Organic liquid storage	12

4.2.1	Calculating usage (and thresholds) of stored organic liquids	14

5	Emission calculations	17

5.1	Emission factors	18

5.2	Approved alternative	19

5.3	Emissions from fuel storage	19

5.4	Emissions from organic liquid storage	22

6	Transfers of NPI substances in waste	25

7	Next steps for reporting	26

8	References	27

Appendix A: Definitions and abbreviations	28

Appendix B:	29

Modifications to the fuel and organic liquid storage emission estimation technique manual (Version 3.2 April 2012)	29

Modifications to the fuel and organic liquid storage emission estimation technique manual (Version 3.1 May 2008)	29

Appendix C: Unit conversion factors for use with TANKS 4.09b	30

Appendix D: Tank types	31

D.1 Fixed roof tanks	31

D.1.1 Horizontal fixed roof	32

D.2 Floating roof tanks	32

D.2.1 Internal floating roof tanks	33

D.2.2 External floating roof	34

D.3 Other tank types	35

D.3.1 Pressure tanks	35

D.3.2 Variable vapour space tanks	36

Appendix F: Emission factors	38

Appendix F.1: Fuel storage- vertical fixed roof tank	39

Appendix F.2: Fuel storage- internal floating roof tank	43

Appendix F.3: Fuel storage- external floating roof tank	47

Appendix F.4: Fuel storage- above ground horizontal fixed roof tank	51

Appendix F.5: Fuel storage- underground horizontal fixed roof tank	55

Appendix F.6: Organic liquid storage- floating roof tank	59

Appendix F.7: Organic liquid storage- vertical roof tank	61

Appendix G: TANKS User Manual	63

# Fuel and Organic Liquid Storage

# List of Tables, Examples, Figures and Equations

Table 1: Minimum amount of fuel stored to trip the Category 1a (Total VOC) reporting threshold (25 tonnes)	6

Table 2: Minimum amount of individual substances in fuel stored to trip the Category 1 reporting threshold (10 tonnes)	7

Table 3: Minimum of pure organic liquid stored to exceed the Category 1 (10 tonnes) reporting threshold	12

Table 4: Minimum of pure organic liquid stored to exceed the Category 1a (25 tonnes) reporting threshold (Total VOC)	13

Example 1: Determine if a facility at an airport in Perth, WA needs to report to the NPI..	9

Example 2: Determine if a facility which is located in Zone 7 and stores diesel in an underground fixed roof tank and fuel oil in a vertical fixed roof tank needs to report to the NPI	10

Example 3: Determine if a facility that stores acetone in a floating roof tank in Victoria (Zone 11) needs to report to the NPI	15

Example 4: Determine which thresholds have been tripped for a facility that stores benzene and toluene separately in floating roof tanks in Zone 1	16

Example 5: Calculate the emissions from an airport in Perth, WA   ………………………...19

Example 6: Calculate the emissions for a facility which is located in Zone 7 and stores diesel in an underground fixed roof tank and fuel oil in a vertical fixed roof tank…………………21

Example 7: Calculate the emissions for a facility that stores acetone in a floating roof tank in Victoria (Zone 11)……………………………………………………………………………22

Example 8: Calculate the emissions for a facility that stores acetone in a floating roof tank in Victoria (Zone 11)……………………………………………………………………………24

UFigure 1: Vertical fixed roof tankU	32

UFigure 2: Internal floating roof tankU	34

UFigure 3: External floating roof tankU	35

UFigure 4: 12 climatic zones in AustraliaU	37

UEquation 1U	18

# Introduction

The purpose of all emission estimation technique (EET) manuals is to assist Australian manufacturing, industrial and service facilities to report emissions of listed substances to the National Pollutant Inventory (NPI). This manual describes the procedures and recommended approaches for estimating emissions from facilities engaged in the storage of liquid fuels, including petroleum and petroleum-based liquids, and organic liquids.

| EET MANUAL |  | Fuel and organic liquid storage |
| --- | --- | --- |
| ANZSIC (2006) CODE |  | 3321: Petroleum product wholesaling |
|  |  | Any industry sector where fuel or organic liquids are stored and used. |

Note that the ANZSIC code is part of NPI reporting requirements. The NPI Guide contains an explanation of ANZSIC codes.

This manual has been developed through a process of national consultation involving state and territory environmental agencies and key industry stakeholders. Particular thanks are due to Environmental Consultancy Services (ECS) for their assistance in developing this manual.

Facilities may use a variety of techniques to estimate their emissions as a result of fuel and organic liquid storage. Facilities with detailed information on their storage practices may use the USEPA TANKS software to estimate emissions from fuel and organic liquid storage. USEPA TANKS is available from the USEPA website: Uhttp://www.epa.gov/ttn/chief/software/tanks/

Additional instructions have been developed by KMH Environmental to assist users in using the USEAP TANKS software and can be found in Appendix G.

Please note that if you do use TANKS, emissions will be presented in imperial units. A conversion table to metric units of measure is given in Appendix C.

## The process for NPI reporting

The process for NPI reporting can be seen in the following flow chart:

## Information required to produce an annual NPI report

The following data will need to be collated for the reporting period to determine the emissions from fuel and organic liquid storage:

- type of fuel or organic liquid stored

- tank working volume (or storage capacity)

- throughput of the tank (e.g. how many times a week/month/year the tank is refilled)

- starting volume of liquid in the tank.

## Additional reporting materials

This manual is written to reflect the common processes employed in fuel and organic liquid storage. In many cases it will be necessary to refer to other EET manuals to ensure that a complete report of the emissions for the facility can be made.  Other applicable EET manuals may include, but are not limited to:

- Combustion in boilers

- Combustion in engines

- Fugitive emissions, and

- industry-specific emission estimation technique manuals.

# Process description

The first step in estimating emissions of NPI substances from your facility is creating a process flow diagram to highlight points where emissions may occur.

In the case of fuel and organic liquid storage this involves identifying the fuel and organic liquids stored on site and determining the type of tank used for storage.

## Fuels and organic liquids

The storage of the following fuels and organic liquids should be considered:

| Fuels |  |
| --- | --- |
| - Crude oil | - Diesel |
| - Fuel oil | - Leaded petrol (LP) |
| - Heating oil | - Unleaded petrol (ULP) |
| - Jet Kerosene | - Premium unleaded petrol (PULP) |
| - Avgas 100 | - RON 98 |
| - Avgas LL | - E10 |
| Organic liquids |  |
| - Acetaldehyde (ethanal) | - Ethylbenzene |
| - Acetic acid | - Ethylene oxide |
| - Acetone | - Formaldehyde |
| - Acetonitrile | - Glutaraldehyde |
| - Acrylamide | - n-Hexane |
| - Acrylic acid | - Methanol (methyl alcohol) |
| - Acrylonitrile | - 2-Methoxyethanol |
| - Aniline | - 2-Methoxyethanol acetate |
| - Benzene | - Methyl ethyl ketone |
| - Carbon disulfide | - Methyl isobutyl ketone |
| - Chloroform | - Methyl methacrylate |
| - Cumene (isopropyl benzene) | - Styrene |
| - Chloroethane (ethyl chloride) | - 1,1,2,2-Tetrachloroethane |
| - Cyclohexane | - Tetrachloroethylene |
| - 1,2-Dibromoethane | - Toluene (methyl benzene) |
| - 1,2-Dichloroethane | - 1,1,2-Trichloroethane |
| - Dichloromethane | - Trichloroethylene |
| - Ethanol (ethyl alcohol) | - Vinyl chloride monomer |
| - 2-Ethoxyethanol | - Xylene (-m) |
| - 2-Ethoxyethanol acetate | - Xylene (-o) |
| - Ethyl acetate | - Xylenes (mixture) |
| - Ethyl butyl ketone |  |

RON 98 corresponds to fuel with a Research Octane Number (RON) of 98, PULP is nominally rated at RON 95 and ULP is nominally rated at RON 91.

### 2.2.1 Bio-fuels

Ethanol replacement petrol (E10) is based on sugar-derived ethanol. Data in this review were obtained from several Australian manufacturers. The quoted lead content is based on a single manufacturer which offers an E10 leaded fuel on the Australian market.

Data on biodiesel (B20) blended fuels, based on vegetable oil-derived methyl ester, are not considered in this manual, due to a lack of current information from Australian sources. Emissions from storage of biodiesel must be considered if a threshold is exceeded. Reporters should attempt to estimate emissions from B20 using an alternative estimation technique where possible.

## Tank types

Fuels and pure organic liquids are generally quite volatile and, as such, the storage tanks used are designed to reduce their evaporation to air. The five basic tank configurations, and the liquids typically stored in them are:

- Fixed roof tanks - vertical (diesel and fuel oil), horizontal including underground (diesel, ULP, PULP)

- External floating roof tanks (domed and standard) – avgas, ULP, PULP

- Internal floating roof tanks – organic liquids

- Pressure tanks (high and low pressure) - LPG

- Variable vapour space tanks.

Please note that this manual does not consider emissions from the storage of fuels in a highly pressurised environment (such as LPG). If an emission occurs from pressurised storage, the emission must be reported to the NPI in the event a threshold is exceeded. Contact the relevant jurisdiction for additional assistance.

A brief description of these tank types can be found in Appendix D.

# Emission sources

General information regarding emission sources can be located in The NPI Guide. Emissions from fuel or organic liquid storage will generally be fugitive emissions to air unless there is a spill, in which case there may also be emissions to land and/or water.

## Emissions to air

Air emissions may be categorised as fugitive emissions or point source emissions.

### Fugitive emissions

Emissions from storage tanks can be categorised as working and standing losses.

Working losses are the combined loss from filling and emptying a tank. As the liquid level increases, the pressure inside the tank increases and vapours are expelled from the tank. A loss during emptying occurs when air drawn into the tank becomes saturated with organic vapour and expands, thus exceeding the capacity of the vapour space.

Standing losses occur through the expulsion of vapour from a tank due to the vapour expansion and contraction as a result of changes in temperature and barometric pressure. This loss occurs without any change in the liquid level in the tank.

### Point source emissions

Point source emissions are directed into a vent or stack and emitted through a single point source into the atmosphere.

## Emissions to water

Leaks from fuel and organic liquid storage facilities should not enter waterways.

If this does happen you are legally required to immediately contact your state or territory environment agency for guidance. Any such emissions are also reportable to the NPI.

## Emissions to land

Emissions of substances to land include slurries, spills and leaks, and such emissions may contain NPI listed substances. Emission sources can be categorised as:

- surface impoundments of liquids and slurries

- unintentional leaks and spills.

# Threshold calculations

For fuel and organic liquid storage, the relevant NPI thresholds are Category 1 and Category 1a. The usage of each of the substances listed as Category 1 and 1a under the NPI must be estimated. Usage is defined as the storage, handling, manufacture, import, processing, coincidental production or other uses of an NPI listed substance.

If a facility has ‘used’ a total of 10 tonnes or more of any Category 1 NPI substance or 25 tonnes or more of Total VOCs (NPI Category 1a substance) in a reporting year, you must report the emissions of each substance that has tripped the threshold. Note: in determining the emissions for a substance, all sources of the emissions from the operations/processes relating to the facility must be included, not just those related to fuel and organic liquid storage. For example: if your facility exceeds the reporting threshold for Total VOC, reporting is required for all emissions of Total VOC from all sources at the facility.

## Fuel storage

Tables 1 and 2 show the minimum amount of fuel stored in a year that would trip the reporting Category 1a (Total VOC) threshold, and the Category 1 threshold for individual substance in the fuel composition respectively.

This is to be used as guidance only and total usage figures should still be determined, especially when multiple fuels are stored on site.

Note: The data shown in Tables 1 and 2 was collected from a variety of sources including Australian refineries and material safety data sheets. It is important to note that the compositions represent averages.

Table 1: Minimum amount of fuel stored to trip the Category 1a (Total VOC) reporting threshold (25 tonnes)

| Product | Liquid density (kg/L) | Typical VOC composition (%) | Reporting threshold (L)2 |
| --- | --- | --- | --- |
| Crude oil | 0.848 | 31 | 983,000 |
| Fuel oil | 0.848 | 3 | 983,000 |
| Heating oil | 0.836 | 12 | 250,000 |
| Jet kerosene | 0.837 | 38 | 79,000 |
| Avgas 100 | 0.695 | 100 | 36,000 |
| Avgas LL | 0.718 | 100 | 35,000 |
| Diesel | 0.836 | 7.6 | 394,000 |
| Leaded petrol (LP) | 0.739 | 99 | 35,000 |
| Unleaded petrol (ULP) | 0.735 | 99 | 34,000 |
| Premium unleaded petrol (PULP) | 0.75 | 99 | 34,000 |
| RON 98 | 0.75 | 991 | 34,000 |
| E10 | 0.743 | 991 | 34,000 |

1. Approximated

2. Calculated by: 25 000 kg ÷ liquid density (kg/L) ÷ typical composition (%)

Rounded up to the nearest thousand

Table 2: Minimum amount of individual substances in fuel stored to trip the Category 1 reporting threshold (10 tonnes)

| Substance | Crude oil |  | Fuel oil |  | Heating oil |  | Jet kerosene |  | Avgas 100 |  | Avgas LL |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Typical composition (%) | Reporting threshold (L)1 | Typical composition (%) | Reporting threshold (L) | Typical composition (%) | Reporting threshold (L) | Typical composition (%) | Reporting threshold (L) | Typical composition (%) | Reporting threshold (L) | Typical composition (%) | Reporting threshold (L) |
| Benzene | 0.090 | 13,103,000 | 0.010 | 117,925,000 | - | ND | 0.367 | 3,256,000 | 1.300 | 1,107,000 | 4.000 | 349, 000 |
| Cumene | 0.040 | 29,482,000 | 0.215 | 5,485,000 | - | ND | 2.830 | 423,000 | 0.011 | 130,805,000 | 0.025 | 55,711,000 |
| Cyclohexane | 0.760 | 1,552,000 | - | ND | - | ND | 1.200 | 996,000 | 0.483 | 2,979,000 | 0.002 | 696,379,000 |
| Ethylbenzene | 0.160 | 7,371,000 | 0.010 | 117,925,000 | 0.070 | 17,089,000 | 0.517 | 2,311,000 | 0.175 | 8,222,000 | 6.500 | 215,000 |
| n-Hexane | 2.060 | 573,000 | 0.010 | 117,925,000 | - | ND | 4.650 | 257,000 | 0.900 | 1,599,000 | 2.525 | 552,000 |
| Lead | - | ND | - | ND | - | ND | - | ND | 0.350 | 4,111,000 | 0.267 | 5,217,000 |
| PAH 3 | 0.300 | Cat 2a / 2b | 0.820 | Cat 2a / 2b | 0.800 | Cat 2a / 2b | 0.985 | Cat 2a / 2b | 0.125 | Cat 2a / 2b | - | ND |
| Toluene | 0.410 | 2,877,000 | 0.030 | 39,309,000 | 0.080 | 14,953,000 | 0.180 | 6,638,000 | 2.123 | 678,000 | 14.250 | 98,000 |
| Xylenes | 0.710 | 1,661,000 | 0.100 | 11,793,000 | 0.330 | 3,625,000 | 1.880 | 636,000 | 0.955 | 1,507,000 | 6.750 | 207,000 |
| Ethanol | - | ND | - | ND | - | ND | - | ND | - | ND | - | ND |
| Other non-NPI listed VOCs | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND |

| Substance | Diesel |  | Leaded petrol |  | Unleaded petrol (ULP) |  | Premium unleaded petrol (PULP) |  | RON98 |  | Ethanol blend (E10) |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Typical composition (%) | Reporting threshold (L) | Typical composition (%) | Reporting threshold (L) | Typical composition (%) | Reporting threshold (L) | Typical composition (%) | Reporting threshold (L) | Typical composition (%) | Reporting threshold (L) | Typical composition (%) | Reporting threshold (L) |
| Benzene | 0.030 | 39,872,000 | 0.903 | 1,499,000 | 0.933 | 1,458,000 | 1.003 | 1,356,000 | 1.007 | 1,351,000 | 0.990 | 1,360,000 |
| Cumene | 0.975 | 1,227,000 | 0.110 | 12,302,000 | 0.100 | 13,605,000 | 0.120 | 11,338,000 | 0.170 | 8,003,000 | - | ND |
| Cyclohexane | 0.010 | 119,617,000 | 0.805 | 1,681,000 | 0.765 | 1,778,000 | 0.990 | 1,374,000 | 1.100 | 1,237,000 | - | ND |
| Ethylbenzene | 0.110 | 10,874,000 | 1.600 | 846,000 | 1.533 | 888,000 | 1.763 | 772,000 | 1.805 | 754,000 | 2.000 | 673,000 |
| n-Hexane | 0.010 | 119,617,000 | 2.600 | 520,000 | 1.830 | 743,000 | 1.520 | 895,000 | 2.025 | 672,000 | 2.000 | 673,000 |
| Lead | - | ND | 0.505 | ND | 0.001 | 1,360,544,000 | 0.000 | 13,605,442,000 | - | ND | 0.060 | 22,447,000 |
| PAH 3 | 0.360 | Cat 2a / 2b | 0.570 | Cat 2a / 2b | 0.610 | Cat 2a / 2b | 0.380 | Cat 2a / 2b | 0.480 | Cat 2a / 2b | - | ND |
| Toluene | 0.100 | 11,962,000 | 7.927 | 171,000 | 5.603 | 243,000 | 7.093 | 192,000 | 19.650 | 69,000 | 7.500 | 180,000 |
| Xylenes | 0.345 | 3,467,000 | 8.170 | 166,000 | 7.747 | 176,000 | 8.980 | 152,000 | 9.730 | 140,000 | 10.000 | 135,000 |
| Ethanol | - | ND | - | ND | - | ND | - | ND | - | ND | 10.000 | 135,000 |
| Other non-NPI listed VOCs | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND | ND |

- Calculated by: (10 000 kg ÷ liquid density (in Table 1) ÷ typical composition (%)

Rounded up to the nearest thousand.

- ND: Not Determined

- Polycyclic Aromatic Hydrocarbons are Category 2a / 2b substances

### Calculating usage (and thresholds) of stored fuels

To calculate the total usage of a stored fuel and determine if a threshold has been tripped:

- Determine the type and characteristics of tank you have.

Is your tank a:

- vertical fixed roof (usually used for diesel and fuel oil)

- internal floating roof (usually used for Avgas, ULP, PULP)

- external floating roof (usually used for Avgas, ULP, PULP)

- horizontal fixed roof (usually used for diesel, ULP, PULP), or

- an underground fixed roof (usually used for diesel, ULP, PULP).

- Determine the volume of fuel in the tank at the start of the reporting period.

- Determine fuel usage:

fuel use = starting volume (L) + (no. times tank filled × average volume filled (L));

or

fuel use = starting volume (L) + total volume filled (L)

- Determine if a reporting threshold has been exceeded for any NPI substances.

If one fuel stored on site: use volume required to trip threshold from Tables 1 and 2.
If multiple fuels stored on site: use ‘typical composition’ information from Tables 1 and 2.

Substance use	=	fuel use (1) x fuel density (1) x typical composition
		of substance in fuel (1)/100 + fuel use (2) x

fuel density (2) x typical composition of substance

in fuel (2)/100 etc.

If the threshold for any NPI substance has been exceeded the emissions of that substance resulting from all activities must be reported to the NPI.

Example 1: Determine if a facility at an airport in Perth, WA needs to report to the NPI. The site stores Avgas LL in an internal floating roof tank and the tank is refilled eight times a year with an average of 200 000L added each time. At the beginning of the reporting period, the gauge indicated that there was approximately 500 000L (500kL) in the tank.

Step 1: Determine the type and characteristics of the tank:

Tank type	Internal floating roof

Step 2: Determine the volume of fuel in the tank at the start of the reporting period

500,000 litres (500kL)

Step 3: Determine fuel usage

Annual use of Avgas LL = starting volume (L) + (no. times tank filled × average volume filled (L))

Annual use of Avgas LL	= 500,000 + (8 × 200,000)

= 2,100,000 L

Step 4: Establish if reporting thresholds have been exceeded

Only one fuel was stored onsite thus the volume of fuel used can be directly compared to the ‘volume required to trip threshold’ column of Tables 1 and 2.

From Table 1: 2 100,000 L is greater than the Category 1a reporting threshold for Avgas LL (35,000 L), therefore an emission of Total VOC is reportable.

From XTable 2X: 2,100,000 L is greater than the Category 1 reporting threshold for Avgas LL for the following substances (for which reporting is required):

- Benzene (threshold for Avgas LL usage =  349,000 L)

- Ethylbenzene (threshold for Avgas LL usage = 215,000 L)

- n-Hexane (threshold for Avgas LL usage = 552,000 L)

- Toluene (threshold for Avgas LL usage = 98,000 L)

- Xylenes (threshold for Avgas LL usage = 207,000L)

The thresholds for the following NPI substances have not been exceeded, and as a result emissions are not reportable (unless thresholds for these substances have been exceeded as a result of other operations at the facility, in which case, emissions must be calculated for these substances):

- Cumene (threshold for Avgas LL usage = 55,711,000)

- Cyclohexane (threshold for Avgas LL usage =  696,379,000)

Example 2: Determine if a facility which is located in Zone 7 and stores diesel in an underground fixed roof tank and fuel oil in a vertical fixed roof tank needs to report to the NPI. Engineering records indicate that the diesel tank contained approximately 72,000L (72kL) of diesel at the start of the reporting period and was refilled fifty-two times during the year with an average of 20,000L added each time. The fuel oil tank was empty at the beginning of the year and was filled once with a volume of 10,000L.

Step 1: Determine the type and characteristics of the tank

Tank type (diesel)	Horizontal fixed roof

Tank type (fuel oil)	Vertical fixed roof

Step 2: Determine the volume of fuel in the tank at the start of the reporting period

Diesel: 72,000L (72 kL)

Fuel Oil: 0L

Step 3: Determine fuel usage

Annual use of diesel = starting volume (L) + (no. times tank filled × average volume filled (L))

Annual use of diesel	= 72,000 + (52 × 20,000)

= 1,112,000 L

Annual use of fuel oil	=0 + (1 x 10,000)

=10,000L

Step 4: Establish if reporting thresholds have been exceeded

As two fuels were stored onsite, the typical composition information needs to be used to determine if thresholds were tripped.

Substance use	=	fuel use (1) x fuel density (1) typical composition of substance in
		fuel (1)/100 + fuel use (2) x fuel density (2)  typical composition of
		substance in fuel (2)/100

From Table 1 and 2 the typical composition of diesel and fuel oil is:

Diesel	Fuel oil

Total VOC	7.6%	3%

Benzene	0.030%	0.010%

Cumene	0.975%	0.215%

Cyclohexane	0.010%	-

Ethylbenzene	0.110%	0.010%

n-Hexane	0.010%	0.010%

Lead	-	-

PAH	0.360%	0.820%

Toluene	0.100%	0.030%

Xylenes	0.345%	0.100%

Ethanol	-	-

Table 1 also gives the density of the liquids:

Diesel: 0.836 kg/L

Fuel Oil: 0.848 kg/L

Total VOC usage	=	diesel use x 0.836kg/L x 7.6/100 + fuel oil use x 0.848kg/L x 
		    3/100
	      =	1 112 000L x 0.836 x 7.6/100 + 10 000L x 0.848kg/L x 3/100

=	70652kg + 254.4kg

=	70906.4kg (70.9 tonnes)

70.9 tonnes is greater than the Category 1a reporting threshold for Total VOC (25 tonnes), therefore reporting of Total VOC is required.

Repeating for the other substances gives the following usages:

Benzene	279.7kg

Cumene	9082.1kg

Cyclohexane	92.96kg

Ethylbenzene	1023.4kg

n-Hexane	93.81kg

Lead	0 kg

Toluene	932.2kg

Xylenes	3215.7kg

Ethanol	0kg

All of the above usages are less than the threshold for all Category 1 substances (10 tonnes) therefore no reporting for these substances is required.

## Organic liquid storage

Table 3 shows the minimum amount of pure organic liquid stored in a year that would trip the reporting threshold. If the organic liquid is mixed in solution you will need to adjust the figures accordingly.

Table 3: Minimum of pure organic liquid stored to exceed the Category 1 (10 tonnes) reporting threshold

| Substance | CAS No. | Liquid density at 20 °C (kg/L) | Reporting threshold (L) |
| --- | --- | --- | --- |
| Acetaldehyde (ethanal) | 75-07-0 | 0.788 | 12,700 |
| Acetic acid | 64-19-7 | 1.048 | 9,600 |
| Acetone | 67-64-1 | 0.792 | 12,700 |
| Acetonitrile | 75-05-8 | 0.784 | 12,800 |
| Acrylamide | 79-06-1 | 1.119 | 9,000 |
| Acrylic acid | 79-10-7 | 1.059 | 9,500 |
| Acrylonitrile | 107-13-1 | 0.808 | 12,400 |
| Aniline | 62-53-3 | 1.021 | 9,800 |
| Benzene | 71-43-2 | 0.879 | 11,400 |
| Carbon disulfide | 75-15-0 | 1.262 | 8,000 |
| Chloroethane( Ethyl chloride) | 75-00-3 | 0.918 | 10,900 |
| Chloroform | 67-66-3 | 1.489 | 6,800 |
| Cumene (isopropyl benzene) | 98-82-8 | 0.792 | 12,700 |
| Cyclohexane | 110-82-7 | 0.779 | 12,900 |
| 1,2-Dibromoethane | 106-93-4 | 1.255 | 8,000 |
| 1,2-Dichloroethane | 107-06-2 | 1.253 | 8,000 |
| Dichloromethane | 75-09-2 | 1.362 | 7,400 |
| Ethanol (ethyl alcohol) | 64-17-5 | 0.772 | 13,000 |
| 2-Ethoxyethanol | 110-80-5 | 0.930 | 10,800 |
| 2-Ethoxyethanol acetate | 111-15-9 | 0.975 | 10,300 |
| Ethyl acetate | 141-78-6 | 0.905 | 11,100 |
| Ethyl butyl ketone | 106-35-4 | 0.818 | 12,300 |
| Ethylbenzene | 100-41-4 | 0.864 | 11,600 |
| Ethylene oxide | 75-21-8 | 0.864 | 11,600 |
| Formaldehyde (37%, Formalin) | 50-00-0 | 1.090 | 9,200 |
| Glutaraldehyde (25%, Cidex) | 111-03-8 | 1.106 | 9,100 |
| n-Hexane | 110-54-3 | 0.658 | 15,200 |
| Methanol (methyl alcohol) | 67-56-1 | 0.792 | 12,700 |
| 2-Methoxyethanol | 109-86-4 | 0.965 | 10,400 |
| 2-Methoxyethanol acetate | 110-49-6 | 0.975 | 10,300 |
| Methyl ethyl ketone | 78-93-3 | 0.806 | 12,500 |
| Methyl isobutyl ketone | 108-10-1 | 0.798 | 12,600 |
| Methyl methacrylate | 80-62-6 | 0.945 | 10,600 |
| Styrene | 100-42-5 | 0.926 | 10,800 |
| 1,1,2,2-Tetrachloroethane | 79-34-5 | 1.593 | 6,300 |
| Tetrachloroethylene | 127-18-4 | 1.621 | 6,200 |
| Toluene (methyl benzene) | 108-88-3 | 0.867 | 11,600 |
| 1,1,2-Trichloroethane | 79-00-5 | 1.334 | 7,500 |
| Trichloroethylene | 79-01-6 | 1.463 | 6,900 |
| Vinyl chloride monomer | 75-01-4 | 0.911 | 11,000 |
| Xylenes (mixture of isomers) | 1330-20-7 | 0.860 | 11,700 |

- Calculated by: 10 000 kg ÷ liquid density, rounded up to the nearest hundred.

Table 4 shows the minimum amount of pure organic liquids (which are defined as volatile organic compounds under the NPI) stored to exceed the Category 1a threshold for reporting total volatile organic compounds (Total VOC). If both the Category 1 and Category 1a thresholds are exceeded for a substance, then an emission for both the substance and Total VOC will be reported. For example, if a facility uses 40 000 L of acetone in a year, an emission is required for both acetone and Total VOC. The emission is calculated by using the emission factor for acetone, and the results reported identically against both acetone and Total VOC.

If a facility uses multiple VOCs (in Table 4), then the usage of each should be added together to determine if the 25 tonnes per year threshold has been exceeded. In the event the threshold is exceeded, emissions for each individual substance should be calculated, and then added together for the emission of Total VOC. Emissions will be reported for both individual substances as well as Total VOC in this case.

Table 4: Minimum of pure organic liquid stored to exceed the Category 1a (25 tonnes) reporting threshold (Total VOC)

| Substance | CAS No. | Liquid density at 20 °C (kg/L) | Reporting threshold (L) |
| --- | --- | --- | --- |
| Acetaldehyde (ethanal) | 75-07-0 | 0.788 | 31,800 |
| Acetic acid | 64-19-7 | 1.048 | 23,900 |
| Acetone | 67-64-1 | 0.792 | 31,600 |
| Acetonitrile | 75-05-8 | 0.784 | 31,900 |
| Acrylic acid | 79-10-7 | 1.059 | 23,700 |
| Acrylonitrile | 107-13-1 | 0.808 | 31,000 |
| Aniline | 62-53-3 | 1.021 | 24,500 |
| Benzene | 71-43-2 | 0.879 | 28,500 |
| Carbon disulfide | 75-15-0 | 1.262 | 19,900 |
| Chloroform | 67-66-3 | 1.489 | 16,800 |
| Cumene (isopropyl benzene) | 98-82-8 | 0.792 | 31,600 |
| Cyclohexane | 110-82-7 | 0.779 | 32,100 |
| 1,2-Dibromoethane | 106-93-4 | 1.255 | 20,000 |
| 1,2-Dichloroethane | 107-06-2 | 1.253 | 20,000 |
| Dichloromethane | 75-09-2 | 1.362 | 18,400 |
| Ethanol (ethyl alcohol) | 64-17-5 | 0.772 | 32,400 |
| 2-Ethoxyethanol | 110-80-5 | 0.93 | 26,900 |
| 2-Ethoxyethanol acetate | 111-15-9 | 0.975 | 25,700 |
| Ethyl acetate | 141-78-6 | 0.905 | 27,700 |
| Ethylbenzene | 100-41-4 | 0.864 | 29,000 |
| Ethyl butyl ketone | 106-35-4 | 0.818 | 30,600 |
| Ethyl chloride (Chloroethane) | 75-00-3 | 0.918 | 27,300 |
| Ethylene oxide | 75-21-8 | 0.864 | 29,000 |
| Formaldehyde (37%, Formalin) | 50-00-0 | 1.09 | 23,000 |
| Glutaraldehyde (25%, Cidex) | 111-03-8 | 1.106 | 22,700 |
| n-Hexane | 110-54-3 | 0.658 | 38,000 |
| Methanol (methyl alcohol) | 67-56-1 | 0.792 | 31,600 |
| 2-Methoxyethanol | 109-86-4 | 0.965 | 26,000 |
| 2-Methoxyethanol acetate | 110-49-6 | 0.975 | 25,700 |
| Methyl ethyl ketone | 78-93-3 | 0.806 | 31,100 |
| Methyl isobutyl ketone | 108-10-1 | 0.798 | 31,400 |
| Methyl methacrylate | 80-62-6 | 0.945 | 26,500 |
| Styrene | 100-42-5 | 0.926 | 27,000 |
| 1,1,2,2-Tetrachloroethane | 79-34-5 | 1.593 | 15,700 |
| Tetrachloroethylene | 127-18-4 | 1.621 | 15,500 |
| Toluene (methyl benzene) | 108-88-3 | 0.867 | 28,900 |
| 1,1,2-Trichloroethane | 79-00-5 | 1.334 | 18,800 |
| Trichloroethylene | 79-01-6 | 1.463 | 17,100 |
| Vinyl chloride monomer | 75-01-4 | 0.911 | 27,500 |
| Xylenes (mixture of isomers) | 1330-20-7 | 0.86 | 29,100 |

- Calculated by: 25 000 kg ÷ liquid density

Rounded up to the nearest hundred.

### Calculating usage (and thresholds) of stored organic liquids

To calculate the total emission of a stored organic liquid:

- Determine the type and characteristics of the storage tank you have.

- Determine the volume of organic liquid in the tank at the start of the reporting period.

- Determine organic liquid usage:

use = starting volume (L) + (no. times tank filled × average volume filled (L));

or

use = starting volume (L) + total volume filled (L).

- Determine if a reporting threshold has been exceeded (for both the individual substance and Total VOC)

Note: if multiple liquids are used, you will need to convert L of organic liquid to kgs to determine if the Total VOC threshold has been tripped

Usage (kg) = Usage (L) x density (kg/L).

If the quantity of any individual organic liquid used exceeds 10 tonnes (10,000 kg) or 25 tonnes for Total VOC you will need to report the emissions of the substance, and any other emissions of the substance resulting from other activities on your facility to the NPI.

Example 3: Determine if a facility that stores acetone in a floating roof tank in Victoria (Zone 11) needs to report to the NPI. The tank is refilled monthly (12 times a year) with an average of 2,000L added each time. Records indicate that there was approximately 5,000 L in the tank at the start of the reporting period.

Step 1: Determine the type and characteristics of the tank

Tank type		Floating roof

Step 2: Determine the volume of organic liquid in the tank at the start of the reporting period

5,000 L

Step 3: Determine organic liquid usage

Annual use of acetone = starting volume (L) + (no. times tank filled × average volume filled (L))

Annual use of acetone		= 5,000 + (12 × 2,000)

= 29,000 L

Step 4: Establish if reporting thresholds have been exceeded

From Table 3: 29 000 L is greater than the Category 1 reporting threshold for acetone (12 700 L), therefore reporting of emissions for this substance is required.

From Table 4: 29,000 L is less than the threshold for reporting of Total VOC

(31,600 L) from acetone, therefore emissions of Total VOC are not reportable in this example.

Example 4: Determine which thresholds have been tripped for a facility that stores benzene and toluene separately in floating roof tanks in Zone 1. The benzene tank is filled monthly (12 times a year) with an average 2,000L added each time. The toluene tank is filled fortnightly (26 times a year) with an average of 2,000L added each time. Records indicate that there was approximately 1,000L in both tanks at the start of the reporting period.

Step 1: Determine the type and characteristics of the tank

Tank type (benzene)		Floating roof

Tank type (toluene)		Floating roof

Step 2: Determine the volume of fuel in the tank at the start of the reporting period

Benzene: 1,000L

Toluene: 1,000L

Step 3: Determine organic liquid usage

Annual use of benzene	=	starting volume (L) + (no. times tank filled × average
		volume filled (L))

Annual use of benzene	=	1,000 + (12 × 2,000)

=	25,000 L

Annual use of toluene	=	starting volume (L) + (no. times tank filled × average
		volume filled (L))

Annual use of toluene	=	1,000 + (26 × 2,000)

=	53,000 L

Step 4: Establish if reporting thresholds have been exceeded

Benzene: from Table 3: 25,000 L is greater than the Category 1 reporting threshold for benzene (11 400 L), therefore reporting of emissions for this substance is required.

Toluene: from Table 3: 53,000 L is greater than the Category 1 reporting threshold for toluene (11 600 L), therefore reporting of emissions for this substance is required.

Total VOC: As multiple organic liquids are stored, need to determine the total usage in kg. Thus need the densities of the liquids from Table 1.

Liquid density (kg/L)	0.879 (benzene) and 0.867 (toluene)

Annual use of

Total VOC (kg)	=	benzene use (kg) + toluene use (kg)

=	benzene use (L) x benzene density (kg/L) + toluene
		use (L) x toluene density (kg/L)

=	25,000L x 0.879kg/L + 53,000L x 0.867kg/L

=	67,926kg (67.9 tonnes)

67.9 tonnes is greater than the Category 1a reporting threshold for Total VOC (25 tonnes), therefore reporting of Total VOC is required.

# Emission calculations

The emissions to air, land and water for every NPI substance that trips a threshold must be reported from all point and fugitive sources on the facility. The NPI Guide outlines detailed information on thresholds and identifying emission sources.

There are five emission estimation techniques (EETs) that may be used to calculate emissions from your facility. These are:

- sampling data or direct measurement

- mass balance

- fuel analysis or engineering calculations

- emission factors, and,

- an approved alternative.

Using one of these techniques will result in your data being displayed on the NPI database as being of ‘acceptable reliability’.

Generally, facilities with fuel and organic liquid storage report emissions for relevant substances using the emission factor method however mass balance or fuel analysis may also be used. Emission factors, mass balance and fuel analysis (engineering calculations) methodologies are described in this section, but other emission estimation techniques can be found in The NPI Guide.

This manual seeks to provide the most effective emission estimation techniques for the NPI substances relevant to the storage of fuels and organic liquids. However, the absence of an EET for a substance in the manual does not imply that an emission should not be reported to the NPI. Reporters should make the best attempt to estimate emissions using an alternative estimation technique in these situations.

You should note that the EETs presented in this manual relate principally to average process emissions. Emissions resulting from non routine events are rarely discussed in the literature, and there is a general lack of EETs for such events. However, it is important to recognise that emissions resulting from significant operating excursions and/or accidental situations (e.g. spills) will also need to be estimated. Emissions to land, air and water from spills must be estimated and added to process emissions when calculating total emissions for reporting purposes. The emission resulting from a spill is the net emission, i.e. the quantity of the NPI reportable substance spilled, less the quantity recovered or consumed immediately (within 24 hours) during clean up operations.

## Emission factors

An emission factor is a tool that is used to estimate emissions to the environment. In this manual, it relates the quantity of substances emitted from a source to some common activity associated with those emissions. Emission factors are usually expressed as the mass of a substance emitted multiplied by the unit mass, volume, distance or duration of the activity emitting the substance (e.g. kilograms of benzene per tonne of unleaded petrol stored).

When using emission factors, you should be aware of the associated emission factor rating (EFR) code and what the rating implies. An A or B rating indicates a greater degree of certainty than a D or E rating. The main criterion affecting the uncertainty of an emission factor remains the degree of similarity between the equipment/process selected in applying the factor and the target equipment/process from which the factor was derived.

The EFR system is:

A		Excellent

B		Above average

C		Average

D		Below average

E		Poor

U		Unrated

Emission factors are used to estimate a facility’s emissions by the general equation:

Equation 1

Ekpy, i	=	(A × OpHrs) × EFi × [1 – (CEi ÷ 100)]

Where:

Ekpy, I	=	emission rate of pollutant i, kg/yr

A	=	activity rate, t/hr

OpHrs	=	operating hours, hr/yr

EFi	=	uncontrolled emission factor for pollutant i, kg/t

CEi	=	overall control efficiency of pollutant i, %

Emission factors applicable to this manual are listed in Appendix F.

You must ensure that you estimate emissions for all substances relevant to your process, including activities not related to fuel or organic liquid storage.

Emission factors developed from measurements for a specific process may sometimes by used to estimate emissions at other sites. For example, a company may have several units of similar model and size. If emissions were measured from one facility, an emission factor could be developed and applied to similar sources. If you wish to use a site-specific emission factor, you should be mindful of the climate conditions of each facility. You should also seek approval from your state or territory environment agency before its use for estimating emissions.

## Approved alternative

You are permitted to use emission estimation techniques that are not outlined in this document. However, you must seek the consent of your state or territory environmental agency. For example, if your company has developed site-specific emission factors, you may only use these if they have been approved by your local environmental agency.

## Emissions from fuel storage

To calculate the total emission of a stored fuel:

- Determine the fuel usage (previously done through the threshold calculations).

- Convert litres to kilograms

Mass = use × liquid density.

- Convert kilograms to tonnes

Tonnes = use (kg) divided by 1000.

- Determine the zone for the facility (Appendix E).

- Get the emission factors (Appendix F).

- Calculate the emissions of the stored fuel.

Emission = Mass × emission factor for the tank type and substance.

Note: If a threshold has been exceeded for a substance for which no emission factor is noted in Appendix F (for example PAH), the reporter is advised to consider using one of the other emission estimation techniques described in this manual. For further advice the reporter should contact the contact the NPI in the state or territory in which the facility is based.

Example 5 (continued from example 1): Calculate the emissions from an airport in Perth, WA. The site stores Avgas LL in an internal floating roof tank and the tank is refilled eight times a year with an average of 200 000L added each time. At the beginning of the reporting period, the gauge indicated that there was approximately 500 000L (500kL) in the tank.

Step 1: Determine the fuel usage

From Example 1 the annual use of Avgas LL is 2,100,000L

Step 2: Convert litres to kilograms

From Table 1: Liquid density (kg/L)	0.718

Use (kg)	=	Use (L) x density

=	2,100,000 x 0.718

=	1,507,800 kg

Step 3: Convert kilograms to tonnes

=	1,507,800 kg/1000

=	1,508 tonnes

Step 4: Determine the zone for the facility

Perth, WA is located in Zone 2

Step 5: Get the emission factors

Tank type (internal floating roof) and Zone 2

Emission factors (kg/tonne) Appendix F:

Benzene		0.00263

Cumene		0.00001

Cyclohexane	<0.00001

Ethylbenzene	0.00284

n-Hexane		0.00204

Lead		0.00011

Toluene		0.00690

Xylenes		0.00299

Total VOC		0.06915

Step 6: Estimate emissions for the substances identified as tripping the threshold (See Example 1).

Emissions(substance) kg = Use (tonnes) × emission factor

Emissions(Benzene) = Use × emission factor = 1 508 × 0.00263

Emissions(Benzene) = 3.97 kg

Emissions(Ethylbenzene) = Use × emission factor = 1 508 × 0.00284

Emissions(Ethylbenzene) = 4 .28 kg

Emissions(n-Hexane) = Use × emission factor = 1 508 × 0.00204

Emissions(n-Hexane) = 3.07 kg

Emissions(Toluene) = Use × emission factor = 1 508 × 0.00690

Emissions(Toluene) = 10.4 kg

Emissions(Xylenes) = Use × emission factor = 1 508 × 0.00299

Emissions(Xylenes) = 4.509 kg

Emissions(Total VOC) = Use × emission factor = 1 508 × 0.06915

Emissions(Total VOC) = 104.28 kg

The above substances and their emissions in kg must be reported to the NPI.

Example 6 (continued from Example 2): Calculate the emissions for a facility which is located in Zone 7 and stores diesel in an underground fixed roof tank and fuel oil in a vertical fixed roof tank. Engineering records indicate that the diesel tank contained approximately 72,000L (72kL) of diesel at the start of the reporting period and was refilled fifty-two times during the year with an average of 20,000L added each time. The fuel oil tank was empty at the beginning of the year and was filled once with a volume of 10,000L.

Step 1: Determine the fuel usage

From example 2 the annual use of diesel is 1 112 000L and the annual use of fuel oil is 10 000L.

Step 2: Convert litres to kilograms

From Table 1: Liquid density (kg/L)	0.836 (diesel) and 0.848 (fuel oil)

Diesel use (kg)	=	Use (L) x density

=	1,112,000 x 0.836

=	929,632 kg

Fuel oil use (kg)	=	Use (L) x density

=	10,000 x 0.848

=	8480 kg

Step 3: Convert kilograms to tonnes

Diesel use		      =     929,632 kg/1000

=     929.63 tonnes

Fuel oil use	      =    8,480 kg/1000

=    8.48 tonnes

Step 4: Determine the zone for the facility

Zone 7

Step 5: Get the emission factors

Zone 7 and tank type: horizontal fixed roof (diesel) and vertical fixed roof (fuel oil)

Diesel emission factors (kg/tonne):

Total VOC		0.73810

Fuel oil emission factors (kg/tonne):

Total VOC		0.13333

Step 5: Estimate emissions for the substances identified as tripping the threshold (See Example 2)

Emissions(Total VOC)	=	Use × emission factor (for each fuel type)

=	929.63 × 0.73810 + 8.48 x 0.13333

Emissions(Total VOC) = 	687.29 kg

An emission for Total VOC of 687.29 kg must be reported to the NPI.

## Emissions from organic liquid storage

To calculate the total emission of a stored organic liquid:

- Determine the organic liquid usage (previously done through the threshold calculations).

- Convert litres to kilograms

Mass = use × liquid density

- Convert kilograms to tonnes

Tonnes = use(kg) divided by 1000.

- Determine the zone for the facility (Appendix E).

- Get the emission factors (Appendix F).

- Calculate the emissions of the stored organic liquid.

Emission = Mass × emission factor for the tank type and substance

Note: If a threshold has been exceeded for a substance for which no emission factor is noted in Appendix F, the reporter is advised to consider using one of the other emission estimation techniques described in this manual. For advice, the reporter should contact the contact the NPI in the state or territory in which the facility is based.

Example 7 (continued from Example 3): Calculate the emissions for a facility that stores acetone in a floating roof tank in Victoria (Zone 11). The tank is refilled monthly (12 times a year) with an average of 2,000L added each time. Records indicate that there was approximately 5,000 L in the tank at the start of the reporting period.

Step 1: Determine the organic liquid usage

Acetone usage: 29,000 L

Step 2: Convert litres to kilograms

Use (kg) 	= Use (L) × density

= 29,000 × 0.792

= 22,968 kg

Step 3: Convert kilograms to tonnes

Acetone use	= 22 968 kg/1000

= 22.968 tonnes

Step 4: Determine the zone for the facility

Zone 11

Step 5: Get the emission factors

Zone 11, Floating roof tank

Emission factor (kg/tonne):	0.253

Step 6: Calculate the emissions for acetone

Emissions(acetone)	=	Use × emission factor

=	22.968 × 0.253

Emissions(acetone) 	=	5.81 kg

An emission for acetone of 5.81 kg must be reported to the NPI.

Example 8 (continued from Example 4): Calculate the emissions for a facility that stores benzene and toluene separately in floating roof tanks in Zone 1. The benzene tank is filled monthly (12 times a year) with an average 2,000L added each time. The toluene tank is filled fortnightly (26 times a year) with an average of 2,000L added each time. Records indicate that there was approximately 1,000L in both tanks at the start of the reporting period.

Step 1: Determine the organic liquid usage

Annual use of benzene – 25,000L

Annual use of toluene – 53,000L

Step 2: Convert litres to kilograms

Liquid density (kg/L)	0.879 (benzene) 0.867 (toluene)

Benzene use (kg)	= Benzene (L) × density

= 25,000 × 0.879

= 21,975 kg

Toluene use (kg)	= Toluene (L) × density

= 53,000 × 0.867

= 45,951 kg

Step 3: Convert kilograms to tonnes

Benzene use		= 21,975 kg/1000

= 21.975 tonnes

Toluene use 		= 45,951 kg/1000

= 45.951 tonnes

Step 4: Determine the zone for the facility

Zone 1

Step 5: Get the emission factors

Zone 1, floating roof tank

Emission factor (kg/tonne)	0.037 (benzene) 0.171 (toluene)

Step 6: Calculate the emissions for each substance

Emissions (benzene)	= Use × emission factor

= 21.975 × 0.037

= 0.81308 kg

Emissions (toluene)	= Use × emission factor

= 45.951 × 0.171

= 7.858 kg

As Total VOC emissions are reportable as a result of the combined usage of both substances, and individual use of Total VOC, the Total VOC emission is estimated as follows:

Emissions (Total VOC) 	= Emissions (benzene) +Emissions (toluene)

= 0.81308 kg + 7.858 kg

= 8.67 kg

The emissions for benzene, toluene and Total VOC must be reported to the NPI.

# Transfers of NPI substances in waste

The NPI requires the mandatory reporting of NPI substances that are transferred as waste to a final destination. Transfers are required to be reported if a Category 1, Category 1b or Category 3 reporting threshold is exceeded. For example, if the threshold has been exceeded for the Category 1 substance benzene as a result of use of this substance on site, transfers to final destination of benzene as well as emissions are reportable.

There is no requirement to report transfers of substances that are exclusively Category 2a or 2b in the event that they have been tripped only by the fuel and energy use threshold (i.e. there is no requirement to report transfers of oxides of nitrogen, particulate matter ≤10 µm, particulate matter ≤ 2.5 µm, polychlorinated dioxins and furans, or polycyclic aromatic hydrocarbons).

Both emissions and transfers are reportable in kilograms.

In the specific context of fuel and organic liquid storage operations, the quantities of NPI substances contained in waste moved onsite or offsite to landfill or other final destination will need to be reported as a transfer. These also include waste containing NPI substances that are also discharged to the sewerage system as part of an industrial disposal system.

Currently there are no generic transfer factors that are available for estimation of transfers from fuel and organic liquid storage operations. Reporters are advised to estimate transfers based on monitoring, licensing arrangements (such as those for discharge to sewer), engineering calculations, or an appropriate alternative technique for the operation.

The transfer of NPI substances to a destination for reuse, recycling, reprocessing, purification, partial purification, immobilisation, remediation or energy recovery can be reported voluntarily. This is an opportune way for facilities to promote good news stories to their local community.

Further information regarding transfers of waste can be located in The NPI Guide.

# Next steps for reporting

This manual has been written to reflect the common processes employed in fuel and organic liquid storage. To ensure a complete report of the emissions for your facility, it may be necessary to refer to other EET manuals. These include:

- Combustion in boilers

- Combustion in engines

- Fugitive emissions, and

- industry-specific emission estimation technique manuals.

When you have a complete record of substance emissions from your facility, report these emissions according to the instructions in The NPI Guide.

The NPI website (www.npi.gov.au) may have additional reporting materials, including calculation tools, for fuel and organic liquid storage.

# References

Department of the Environment and Heritage. National Pollutant Inventory. 21 October 2004. Emissions Estimation Technique Manual for Fuel and Organic Liquid Storage. Version 2.5.

Department of the Environment and Water Resources, National Pollutant Inventory, Fuel Storage Review, Report Number RA07017, May 2007, Environmental Consultancy Services (ECS).

Department of the Environment and Water Resources, National Pollutant Inventory, Organic Liquid Storage Review, Report Number RA07016, May 2007, Environmental Consultancy Services (ECS).

Environment Canada, National Pollutant Release Inventory, Data and Reports, Guide 2004.

Merck and Co. 2006, Merck Index, 14th Edition, USA

Standards Australia. Australian Standard AS 3570 – 1998. Automotive diesel fuel.

United States Environmental Protection Agency. September 2006. Emission Factor Documentation for AP-42. Section 7.1 Organic Liquid Storage Tanks. Final Report. United States Environmental Protection Agency, Office of Air Quality Planning and Standards Emission Factor and Inventory Group. Research Triangle Park, NC, USA.

# Appendix A: Definitions and abbreviations

|  | Definition |
| --- | --- |
| °C | degrees Celsius |
| API | American Petroleum Institute |
| Avgas | Avgas is a high-octane fuel used for aircraft. Avgas is an abbreviation for aviation gasoline, and is only used in aircraft that use piston engines. Jet aircraft and turboprops use kerosene jet fuel. |
| Avgas 100 | Avgas 100 is a high octane grade aviation gasoline, containing a maximum of 1.12 grams of lead/litre. |
| Avgas LL | Avgas low lead |
| CAS | Chemical Abstracts Service |
| E10 | Ethanol replacement petrol is based on sugar-derived ethanol. Data in this review are based on several Australian manufacturers. The quoted lead content is based on a single manufacturer which offers an E10 leaded fuel on the Australian market. Reports indicate that although E10 is marketed as a clean fuel, above 16 °C it possessed a higher Reid Vapour Pressure than standard US gasoline and increase acetaldehyde emissions. |
| EET | Emission Estimation Technique |
| Jet kerosene | Also known as HAvturH, HJet-AH, HJet-A1H, HJet-BH, HJP-4H, HJP-5H, HJP-7H or HJP-8H |
| kg/L | kilograms (of substance) per litre |
| LP | leaded petrol |
| m | metres |
| m3 | cubic metre = 1000 litres |
| MSDS | Material Safety Data Sheet |
| p | The ratio of the circumference to the diameter of a circle; approximately equal to 3.14 |
| psi | pounds per square inch |
| PULP | Premium unleaded petrol is a special blend of petrol designed to bring high octane, and hence high engine power, as well as knock- free performance to unleaded cars with a high-octane requirement. |
| RON 98 | Also known as 98 RON, it is a high-octane unleaded fuel. |
| SI Units | System International Units, also referred to as ‘metric units’. |
| tonnes | = 1000 kilograms |
| ULP | Unleaded petrol |

# Appendix B:

# Modifications to the fuel and organic liquid storage emission estimation technique manual (Version 3.3 May 2012)

| Page | Outline of alteration |
| --- | --- |
| Appendix G | TANKS User Manual |

# Modifications to the fuel and organic liquid storage emission estimation technique manual (Version 3.2 February 2010)

| Page | Outline of alteration |
| --- | --- |
| Appendix F | Corrected emission factors for Total VOC |
| 11 | Updated fuel densities based for ULP, PULP, RON98 and E10 |
| 22 | Corrected example 7, based on updates to Appendix F |

# Modifications to the fuel and organic liquid storage emission estimation technique manual (Version 3.1 May 2008)

| Page | Outline of alteration |
| --- | --- |
| Throughout | Removed specific guidance or instructions relating to the use of TANKS 4.09. |
| 4 | Included discussion on biofuels |
| 6-16 | Updated threshold calculations for determining fuel storage |
| 17-23 | Updated emission estimation techniques relevant to fuel and organic liquid storage |
| 24 | Included section on next steps for reporting. |
| 28 | Appendix C: Transferred unit conversion factors |
| 29 | Appendix D: Reviewed tank types |
| 36 | Appendix F: New emission factors for fuel types and organic liquid types for different tank types These include obtaining emission factors for the NPI-listed volatile organic compounds. |
| Appendix F | Correct emission factors for the substance Total VOC to incorporate non-NPI listed VOC components for the fuels LP, ULP, PULP, RON98 and E10. |
| Throughout | References to “Kerosene” corrected to “Jet Kerosene”. |

# Appendix C: Unit conversion factors for use with TANKS 4.09b

| COLUMN 1 To convert from | COLUMN 2 To convert to | COLUMN 3 Multiply by |
| --- | --- | --- |
| Mass |  |  |
| Kilogram (kg) | Pound (lb) | 2.204 |
| Pound (lb) | Kilogram (kg) | 0.454 |
| Tonne (metric) | Ton (US~2 000lb) | 1.102 |
| Ton (US~2 000lb) | Tonne (metric) | 0.907 |
| Tonnes (t) | Kilograms (kg) | 1000 |
| Kilograms (kg) | Tonnes (t) | 0.001 |
| Volume |  |  |
| US gallon3 | Cubic metres (m3) | 0.003 785 |
| Cubic metres (m3) | US gallon | 264.200 |
| US gallons | Litres (L) | 3.785 |
| Litres (L) | US gallons | 0.2642 |
| Litres (L) | Cubic foot (ft3) | 0.0353 |
| Cubic foot (ft3) | Litres (L) | 28.33 |
| Length |  |  |
| Metres (m) | Foot (ft) | 3.280 |
| Foot (ft) | Metres (m) | 0.304 |
| Temperature |  |  |
| Celsius (oC) | Fahrenheit (oF) | 9/5 × oC + 32 |
| Fahrenheit (oF) | Celsius (oC) | (oF – 32) × 5/9 |
| Pressure |  |  |
| lb per square inch (psi) | Hectopascal (hPa) | 69 |
| Hectopascal (hPa) | lb per square inch (psi) | 0.01450 |
| Millibar | Hectopascal | 1 |
| Note: - The unit psi is either psig (pound per square inch gauge) or psia (pounds per square inch absolute). Gauge is in reference to 1 atmosphere pressure (14.7 psia or 101.3 kPa) - Note US gallons are UnotU the same as UK gallons. - In relation to any work involving TANKS the unit gallon or gal refers to US gallons. |  |  |

# Appendix D: Tank types

### D.1 Fixed roof tanks

Fixed roof tanks consist of a cylindrical shell capped with a permanently fixed roof that may be flat, conical or dome-shaped. The tank is generally allowed to vent freely to the atmosphere, or will have pressure/vacuum vents (or a breather valve) installed. The vents allow the formation of internal working pressure to minimise losses caused by changes in temperature, pressure and liquid level.

The major differences between horizontal and vertical tanks are the orientation, and the use of the tank as either an above ground or submerged (below ground) tank. Vertical tanks are only constructed above ground. As with vertical tanks, horizontal tanks are equipped with pressure/vacuum vents. The general rule of thumb in designing horizontal tanks is to ensure that the length of the shell is no more than six times its diameter. This design reduces the chance of structural failure; however this restricts horizontal tanks to smaller capacities.

Several methods are used to control emissions from fixed-roof tanks:

- installation of an internal floating roof and adjusting seals to minimize evaporation of the product being stored

- vapour balancing, where vapours expelled are directed to the emptying petroleum tanker truck. The truck then transports the vapours to a central station where a vapour recovery or control system is used to control emissions, or

- vapour-recovery systems, where emissions are collected from storage vessels and converted to the liquid product. Several vapour-recovery procedures may be used, including vapour/liquid absorption, vapour compression, vapour cooling, vapour/solid adsorption, or a combination of these.

Figure 1: Vertical fixed roof tank

(Source: USEPA, September 2006)

#### D.1.1 Horizontal fixed roof

Horizontal fixed roof tanks are constructed for both above-ground and underground service and are usually built of steel, steel with a fibreglass overlay or fibreglass-reinforced polyester. They are usually equipped with pressure-vacuum vents, gauge hatches, sample wells and access points. Underground tanks may be protected to prevent corrosion of the tank shell. Capacity is generally less than 150 000 litres.

Emission sources for above-ground fixed roof tanks are the same as those for vertical fixed roof tanks.

Emissions from underground fixed roof storage tanks are associated mainly with changes in the liquid level in the tank. Losses caused by changes in temperature or barometric pressure are minimised for underground tanks because the surrounding earth limits diurnal temperature change; changes in barometric pressure result in only small losses.

### D.2 Floating roof tanks

There are two distinct types of floating roof tank designs utilised for storage of fuel and organic liquids. These are internal and external floating roof tanks, both of which can have variations in their final design.

Emissions from floating roof tanks may be considered as the sum of standing storage losses and withdrawal losses. Withdrawal losses occur as the liquid level, and thus the floating roof, is lowered. Some liquid remains on the inner tank wall surface and evaporates. For an internal floating roof tank that has a column supported fixed roof, some liquid also clings to the columns and evaporates. Evaporative loss occurs until the tank is filled and the exposed surfaces are again covered. Standing storage losses from floating roof tanks include rim seal deck fitting losses, and deck seam losses for constructions other than welded decks. Other potential standing storage loss mechanisms include breathing losses as a result of temperature and pressure changes.

#### D.2.1 Internal floating roof tanks

An internal floating roof tank has a permanent fixed roof on the tank and a floating roof on top of the stored liquid. The floating roof can either rest directly on the liquid surface (a contact deck) or on pontoons, resting several centimetres above the liquid surface (a non contact deck).

Non contact decks are the most common type currently in use. Typical non contact decks are constructed of an aluminium deck and an aluminium grid framework supported above the liquid surface by tubular aluminium pontoons or some other buoyant structure.

Contact type decks include: aluminium sandwich panels with a honeycombed aluminium core floating in contact with the liquid; resin-coated, fibreglass-reinforced polyester (FRP), buoyant panels floating in contact with the liquid; and pan-type steel roofs, floating in contact with the liquid with our without the aid of pontoons. Most contact internal floating decks are pan-type steel or aluminium sandwich panel type. FRP deck types are less common.

Evaporative losses from floating roofs may come from deck fittings, non welded deck seams and the annular space between the deck and tank wall. In addition, these tanks are freely vented by circulation vents at the top of the fixed roof. The vents minimise the possibility of organic vapour accumulation (a “no vapour” zone) in the tank vapour space in concentrations approaching the flammable range.

Figure 2: Internal floating roof tank

(Source: USEPA, September 2006)

#### D.2.2 External floating roof

A typical external floating roof tank consists of an open-topped cylindrical steel shell equipped with a roof that floats on the surface of the stored liquid. The floating roof consists of a deck, fittings and rim seal system. The floating decks are usually constructed of welded steel plate and are of two general types – pontoon and double-deck. With all types of external floating roof tanks, the roof rises and falls with the liquid level in the tank. External floating decks are equipped with a rim seal system attached to the deck perimeter and in contact with the tank wall. The purpose of the floating roof and rim seal system is to reduce evaporative loss of the stored liquid. Some annular space remains between the seal system and the tank wall. The seal system slides against the tank wall as the roof is raised and lowered. The floating deck is also equipped with fittings that penetrate the deck and serve operational functions. The external floating roof design is such that evaporative losses from the stored liquid are limited to losses from the rim seal system and deck fittings (standing storage loss) and any exposed liquid on the tank walls (withdrawal loss).

Figure 3: External floating roof tank

(Source: USEPA, September 2006)

Domed external floating roof tanks have the heavier type of deck used in external floating roof tanks, as well as a fixed roof at the top of the shell, as for the internal floating roof tank. The function of the fixed roof is as a weather barrier; however the tanks are freely vented by circulation vents at the top of the fixed roof. Deck fittings and rim seals are basically identical to those on external roof tanks.

### D.3 Other tank types

#### D.3.1 Pressure tanks

Two classes of pressure tanks exist, low pressure and high pressure. These tanks are generally used for storing liquids with high vapour pressures. Pressure tanks are fitted with a pressure/vacuum vent that is set to prevent venting loss from boiling and breathing loss from daily temperature or barometric pressure changes. Under ideal operating conditions, high pressure tanks can be used with little or no working losses, whereas low pressure tanks exhibit working losses from atmospheric venting.

#### D.3.2 Variable vapour space tanks

Variable vapour space tanks are equipped with expandable vapour reservoirs to accommodate vapour volume fluctuations resulting from temperature and pressure changes. These tanks are usually connected to the vapour spaces of fixed roof tanks. The two most common types of variable vapour space tanks are lifter roof tanks and flexible diaphragm tanks.

Lifter roof tanks have a telescoping roof that fits loosely around the tank wall. The space between the roof and the wall is closed by a wet seal, a trough filled with liquid, or a dry seal, using a flexible coated fabric.

Flexible diaphragm tanks use flexible membranes to provide expandable volume. These may either be separate gasholder units or integral units mounted on top of fixed roof tanks.

Losses from variable vapour space tanks occur during tank filling when the vapour is displaced by the liquid. Loss of vapour occurs only when the tank’s vapour storage capacity is exceeded.

Appendix E: Climatic zones

The storage of fuels and organic liquids in differing Australian climatic zones will contribute to variance in emissions from storage tanks. Default meteorological data has been developed using averaged data for various locations within each of 12 regions identified in the Australian Standard for automotive diesel fuel. These data can be used in the absence of specific meteorological data for a given location.

The 12 regions outlined on the map are South Australia South (1), Western Australia South (2), Western Australia Central (3), Australia North (4), Australia Central (5), Queensland Far North East (6), Queensland Central North (7), Queensland Central (8), Queensland South (9), New South Wales (10), Victoria (11), and Tasmania (12).

Figure 4: 12 climatic zones in Australia

# Appendix F: Emission factors

The emission factors in this manual were developed through the modelling of raw data, assumptions and averages for Australian facilities, using the USEPA TANKS software application. It is assumed that the storage tanks are in excellent condition, well maintained, and that best practice is followed in filling and extracting. Moreover, the emission factors assume that there is throughput through the tank during the year. In the event you exceed a threshold as a result of storage (use), but the tank does not have any throughput, there will still be an emission as a result of venting. In these circumstances you should consider using an alternative emission estimation technique, or a comprehensive tool such as the USEPA TANKS application.

The emission factors are developed based on typical operations that occur within Australia. You may wish to adjust your emissions to reflect additional controls that are in use at your facility. For example, if your facility employs a vapour recovery mechanism on a venting tank, you should adjust the calculated emission by the efficiency of the device.

If the storage tanks operated by your facility do not fall within these parameters you may wish to use the US EPA TANKS application or an alternative approach to determine the most appropriate emissions estimates.

All emission factors are rated U as they are based on models using assumptions and averages, rather than actual measured emissions data.

## Appendix F.1: Fuel storage- vertical fixed roof tank

|  | Zone (kg/tonne) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
|  | South SA | South WA | Central WA | Northern Australia | Central Australia | Cape York | Central North Qld | Central Qld | South Qld | NSW | Vic | Tas |
| Crude oil |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00103 | 0.00115 | 0.00178 | 0.00174 | 0.00151 | 0.00164 | 0.00160 | 0.00156 | 0.00133 | 0.00114 | 0.00099 | 0.00078 |
| Cumene | 0.00002 | 0.00002 | 0.00004 | 0.00004 | 0.00003 | 0.00004 | 0.00004 | 0.00004 | 0.00003 | 0.00002 | 0.00002 | 0.00001 |
| Cyclohexane | 0.00896 | 0.01002 | 0.01541 | 0.01507 | 0.01312 | 0.01421 | 0.01385 | 0.01354 | 0.01160 | 0.00991 | 0.00865 | 0.00685 |
| Ethylbenzene | 0.00017 | 0.00020 | 0.00033 | 0.00034 | 0.00027 | 0.00031 | 0.00030 | 0.00029 | 0.00024 | 0.00020 | 0.00017 | 0.00012 |
| n-Hexane | 0.03823 | 0.04257 | 0.06471 | 0.06304 | 0.05543 | 0.05953 | 0.05817 | 0.05702 | 0.04906 | 0.04213 | 0.03697 | 0.02946 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00133 | 0.00151 | 0.00244 | 0.00242 | 0.00203 | 0.00227 | 0.00219 | 0.00212 | 0.00178 | 0.00150 | 0.00128 | 0.00099 |
| Xylenes | 0.00064 | 0.00074 | 0.00125 | 0.00101 | 0.00111 | 0.00107 | 0.00073 | 0.00061 | 0.00124 | 0.00116 | 0.00088 | 0.00046 |
| Total VOC | 0.05038 | 0.05621 | 0.08596 | 0.08366 | 0.07350 | 0.07907 | 0.07687 | 0.07517 | 0.06528 | 0.05606 | 0.04896 | 0.03867 |
| Fuel oil |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00026 | 0.00029 | 0.00043 | 0.00043 | 0.00037 | 0.00041 | 0.00039 | 0.00038 | 0.00033 | 0.00028 | 0.00025 | 0.00020 |
| Cumene | 0.00025 | 0.00029 | 0.00048 | 0.00050 | 0.00039 | 0.00047 | 0.00044 | 0.00042 | 0.00035 | 0.00029 | 0.00024 | 0.00018 |
| Cyclohexane | - | - | - | - | - | - | - | - | - | - | - | - |
| Ethylbenzene | 0.00002 | 0.00003 | 0.00005 | 0.00005 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00003 | 0.00003 | 0.00002 | 0.00002 |
| n-Hexane | 0.00042 | 0.00047 | 0.00068 | 0.00067 | 0.00059 | 0.00065 | 0.00063 | 0.00061 | 0.00053 | 0.00046 | 0.00041 | 0.00033 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00022 | 0.00025 | 0.00039 | 0.00039 | 0.00032 | 0.00037 | 0.00035 | 0.00034 | 0.00029 | 0.00025 | 0.00021 | 0.00017 |
| Xylenes | 0.00021 | 0.00023 | 0.00038 | 0.00039 | 0.00031 | 0.00037 | 0.00035 | 0.00033 | 0.00028 | 0.00023 | 0.00020 | 0.00015 |
| Total VOC | 0.07576 | 0.08788 | 0.14545 | 0.15152 | 0.11818 | 0.14242 | 0.13333 | 0.12727 | 0.10606 | 0.08788 | 0.07273 | 0.05455 |
| Heating oil |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | - | - | - | - | - | - | - | - | - | - | - | - |
| Cumene | - | - | - | - | - | - | - | - | - | - | - | - |
| Cyclohexane | - | - | - | - | - | - | - | - | - | - | - | - |
| Ethylbenzene | 0.00009 | 0.00010 | 0.00016 | 0.00016 | 0.00013 | 0.00015 | 0.00014 | 0.00014 | 0.00012 | 0.00010 | 0.00008 | 0.00006 |
| n-Hexane | - | - | - | - | - | - | - | - | - | - | - | - |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00029 | 0.00033 | 0.00051 | 0.00052 | 0.00043 | 0.00049 | 0.00047 | 0.00045 | 0.00039 | 0.00033 | 0.00028 | 0.00022 |
| Xylenes | 0.00034 | 0.00038 | 0.00062 | 0.00063 | 0.00051 | 0.00060 | 0.00057 | 0.00054 | 0.00046 | 0.00038 | 0.00032 | 0.00024 |
| Total VOC | 0.34940 | 0.39759 | 0.61446 | 0.62651 | 0.51807 | 0.59036 | 0.56627 | 0.54217 | 0.46988 | 0.39759 | 0.33735 | 0.26506 |
| Kerosene |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00475 | 0.00528 | 0.00787 | 0.00784 | 0.00674 | 0.00748 | 0.00721 | 0.00704 | 0.00608 | 0.00522 | 0.00457 | 0.00365 |
| Cumene | 0.00166 | 0.00191 | 0.00317 | 0.00326 | 0.00257 | 0.00307 | 0.00289 | 0.00276 | 0.00229 | 0.00188 | 0.00157 | 0.00118 |
| Cyclohexane | 0.01607 | 0.01782 | 0.02640 | 0.02624 | 0.02268 | 0.02505 | 0.02419 | 0.02365 | 0.02048 | 0.01762 | 0.01549 | 0.01243 |
| Ethylbenzene | 0.00064 | 0.00073 | 0.00117 | 0.00119 | 0.00096 | 0.00113 | 0.00107 | 0.00103 | 0.00086 | 0.00072 | 0.00061 | 0.00046 |
| n-Hexane | 0.09805 | 0.10826 | 0.15852 | 0.15695 | 0.13698 | 0.15009 | 0.14532 | 0.14244 | 0.12384 | 0.10709 | 0.09466 | 0.07646 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00067 | 0.00075 | 0.00116 | 0.00117 | 0.00097 | 0.00111 | 0.00106 | 0.00103 | 0.00088 | 0.00074 | 0.00064 | 0.00050 |
| Xylenes | 0.00193 | 0.00220 | 0.00357 | 0.00365 | 0.00293 | 0.00344 | 0.00326 | 0.00312 | 0.00262 | 0.00218 | 0.00183 | 0.00140 |
| Total VOC | 0.60909 | 0.68182 | 1.05455 | 1.06364 | 0.88182 | 1.00909 | 0.96364 | 0.93636 | 0.80000 | 0.67273 | 0.58182 | 0.45455 |
| Avgas 100 |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.01296 | 0.01429 | 0.02162 | 0.02189 | 0.01796 | 0.01626 | 0.02068 | 0.01972 | 0.01895 | 0.01415 | 0.01254 | 0.01018 |
| Cumene | <0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | <0.00001 | <0.00001 |
| Cyclohexane | 0.00498 | 0.00548 | 0.00824 | 0.00832 | 0.00787 | 0.00543 | 0.00687 | 0.00751 | 0.00723 | 0.00622 | 0.00483 | 0.00394 |
| Ethylbenzene | 0.00017 | 0.00019 | 0.00031 | 0.00032 | 0.00030 | 0.00026 | 0.00019 | 0.00025 | 0.00028 | 0.00022 | 0.00016 | 0.00012 |
| n-Hexane | 0.01462 | 0.01602 | 0.02379 | 0.02394 | 0.02267 | 0.02094 | 0.01587 | 0.01418 | 0.01994 | 0.02171 | 0.01809 | 0.01164 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00605 | 0.00675 | 0.01061 | 0.01087 | 0.01023 | 0.00921 | 0.00669 | 0.00582 | 0.00865 | 0.00967 | 0.00780 | 0.00463 |
| Xylenes | 0.00075 | 0.00085 | 0.00140 | 0.00146 | 0.00137 | 0.00121 | 0.00085 | 0.00072 | 0.00112 | 0.00128 | 0.00100 | 0.00056 |
| Total VOC | 1.32909 | 1.45636 | 2.16273 | 2.17636 | 2.06091 | 1.90364 | 1.44273 | 1.28909 | 1.81273 | 1.97364 | 1.64455 | 1.05818 |
| Avgas LL |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.03859 | 0.04256 | 0.06440 | 0.06519 | 0.05348 | 0.06161 | 0.05873 | 0.05643 | 0.04843 | 0.04215 | 0.03734 | 0.03032 |
| Cumene | 0.00001 | 0.00001 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00001 | 0.00001 | 0.00001 | 0.00001 |
| Cyclohexane | 0.00002 | 0.00002 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00002 | 0.00002 | 0.00002 | 0.00002 |
| Ethylbenzene | 0.00596 | 0.00675 | 0.01104 | 0.01146 | 0.00881 | 0.01072 | 0.01005 | 0.00949 | 0.00792 | 0.00668 | 0.00570 | 0.00443 |
| n-Hexane | 0.03969 | 0.04350 | 0.06460 | 0.06500 | 0.05415 | 0.06157 | 0.05894 | 0.05687 | 0.04914 | 0.04308 | 0.03851 | 0.03161 |
| Lead | 0.00001 | 0.00001 | 0.00002 | 0.00002 | 0.00001 | 0.00002 | 0.00002 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | <0.00001 |
| Toluene | 0.03930 | 0.04388 | 0.06897 | 0.07065 | 0.05618 | 0.06646 | 0.06282 | 0.05985 | 0.05068 | 0.04344 | 0.03781 | 0.03005 |
| Xylenes | 0.00516 | 0.00585 | 0.00961 | 0.00998 | 0.00765 | 0.00934 | 0.00874 | 0.00825 | 0.00687 | 0.00579 | 0.00494 | 0.00383 |
| Total VOC | 1.345 | 1.475 | 2.190 | 2.203 | 1.836 | 2.087 | 1.998 | 1.928 | 1.666 | 1.460 | 1.305 | 1.072 |
| Diesel |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00038 | 0.00043 | 0.00063 | 0.00063 | 0.00054 | 0.00060 | 0.00058 | 0.00057 | 0.00049 | 0.00042 | 0.00037 | 0.00029 |
| Cumene | 0.00056 | 0.00065 | 0.00107 | 0.00111 | 0.00087 | 0.00104 | 0.00098 | 0.00094 | 0.00078 | 0.00064 | 0.00053 | 0.00040 |
| Cyclohexane | 0.00013 | 0.00015 | 0.00022 | 0.00022 | 0.00019 | 0.00021 | 0.00020 | 0.00019 | 0.00017 | 0.00014 | 0.00013 | 0.00010 |
| Ethylbenzene | 0.00013 | 0.00015 | 0.00025 | 0.00025 | 0.00020 | 0.00024 | 0.00022 | 0.00022 | 0.00018 | 0.00015 | 0.00013 | 0.00010 |
| n-Hexane | 0.00021 | 0.00023 | 0.00034 | 0.00033 | 0.00029 | 0.00032 | 0.00031 | 0.00030 | 0.00026 | 0.00023 | 0.00020 | 0.00016 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00036 | 0.00041 | 0.00063 | 0.00064 | 0.00053 | 0.00061 | 0.00058 | 0.00056 | 0.00048 | 0.00041 | 0.00035 | 0.00027 |
| Xylenes | 0.00035 | 0.00040 | 0.00064 | 0.00066 | 0.00053 | 0.00062 | 0.00059 | 0.00057 | 0.00047 | 0.00039 | 0.00033 | 0.00025 |
| Total VOC | 0.03231 | 0.03538 | 0.05231 | 0.05077 | 0.04462 | 0.04923 | 0.02000 | 0.04615 | 0.04000 | 0.03538 | 0.03077 | 0.02462 |
| LP |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00704 | 0.008 | 0.01311 | 0.01308 | 0.01076 | 0.01209 | 0.01168 | 0.0112 | 0.00942 | 0.00793 | 0.00677 | 0.00529 |
| Cumene | 0.00004 | 0.00005 | 0.00008 | 0.00009 | 0.00006 | 0.00008 | 0.00007 | 0.00007 | 0.00006 | 0.00005 | 0.00004 | 0.00003 |
| Cyclohexane | 0.0065 | 0.00737 | 0.01198 | 0.01193 | 0.00988 | 0.01104 | 0.01068 | 0.01026 | 0.00865 | 0.0073 | 0.00625 | 0.00491 |
| Ethylbenzene | 0.00119 | 0.00138 | 0.00245 | 0.00251 | 0.00193 | 0.0023 | 0.00218 | 0.00205 | 0.00168 | 0.00137 | 0.00113 | 0.00084 |
| n-Hexane | 0.03304 | 0.0373 | 0.05996 | 0.05947 | 0.04972 | 0.05512 | 0.05348 | 0.05148 | 0.04358 | 0.03699 | 0.03185 | 0.02516 |
| Lead | 0.00001 | 0.00001 | 0.00003 | 0.00003 | 0.00002 | 0.00003 | 0.00003 | 0.00002 | 0.00002 | 0.00001 | 0.00001 | 0.00001 |
| Toluene | 0.01767 | 0.02033 | 0.03459 | 0.03492 | 0.02786 | 0.03214 | 0.03079 | 0.02927 | 0.02428 | 0.02015 | 0.01689 | 0.01292 |
| Xylenes | 0.00505 | 0.0059 | 0.01049 | 0.01074 | 0.00826 | 0.00983 | 0.00932 | 0.00878 | 0.00716 | 0.00584 | 0.0048 | 0.00358 |
| Total VOC | 2.54154 | 2.86923 | 4.61231 | 4.57462 | 3.82462 | 4.24000 | 4.11385 | 3.96000 | 3.35231 | 2.84538 | 2.45000 | 1.93538 |
| ULP |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00804 | 0.00895 | 0.01248 | 0.01327 | 0.01073 | 0.01264 | 0.01203 | 0.01133 | 0.0101 | 0.00894 | 0.00773 | 0.00643 |
| Cumene | 0.00004 | 0.00004 | 0.00007 | 0.00008 | 0.00006 | 0.00007 | 0.00007 | 0.00006 | 0.00005 | 0.00004 | 0.00004 | 0.00003 |
| Cyclohexane | 0.00853 | 0.00682 | 0.00758 | 0.01049 | 0.01114 | 0.00906 | 0.01061 | 0.01012 | 0.00954 | 0.00757 | 0.00657 | 0.00549 |
| Ethylbenzene | 0.00167 | 0.00126 | 0.00143 | 0.00216 | 0.00236 | 0.00179 | 0.00222 | 0.00208 | 0.00193 | 0.00143 | 0.00119 | 0.00095 |
| n-Hexane | 0.03183 | 0.02569 | 0.02842 | 0.03889 | 0.04112 | 0.03377 | 0.03924 | 0.03751 | 0.03548 | 0.0284 | 0.02479 | 0.02084 |
| Lead | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 |
| Toluene | 0.0138 | 0.01555 | 0.02252 | 0.02425 | 0.019 | 0.02298 | 0.02169 | 0.02026 | 0.01781 | 0.01554 | 0.0132 | 0.01075 |
| Xylenes | 0.00529 | 0.00605 | 0.00916 | 0.01 | 0.00756 | 0.00943 | 0.00881 | 0.00815 | 0.00705 | 0.00604 | 0.00503 | 0.00399 |
| Total VOC | 2.44846 | 1.97615 | 2.18615 | 2.99154 | 3.16308 | 2.59769 | 3.01846 | 2.88538 | 2.72923 | 2.18462 | 1.90692 | 1.60308 |
| PULP |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00804 | 0.00895 | 0.01248 | 0.01327 | 0.01073 | 0.01264 | 0.01203 | 0.01133 | 0.0101 | 0.00894 | 0.00773 | 0.00643 |
| Cumene | 0.00004 | 0.00004 | 0.00007 | 0.00008 | 0.00006 | 0.00007 | 0.00007 | 0.00006 | 0.00005 | 0.00004 | 0.00004 | 0.00003 |
| Cyclohexane | 0.00682 | 0.00758 | 0.01049 | 0.01114 | 0.00906 | 0.01061 | 0.01012 | 0.00954 | 0.00853 | 0.00757 | 0.00657 | 0.00549 |
| Ethylbenzene | 0.00126 | 0.00143 | 0.00216 | 0.00236 | 0.00179 | 0.00222 | 0.00208 | 0.00193 | 0.00167 | 0.00143 | 0.00119 | 0.00095 |
| n-Hexane | 0.02569 | 0.02842 | 0.03889 | 0.04112 | 0.03377 | 0.03924 | 0.03751 | 0.03548 | 0.03183 | 0.0284 | 0.02479 | 0.02084 |
| Lead | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 |
| Toluene | 0.0138 | 0.01555 | 0.02252 | 0.02425 | 0.019 | 0.02298 | 0.02169 | 0.02026 | 0.01781 | 0.01554 | 0.0132 | 0.01075 |
| Xylenes | 0.00529 | 0.00605 | 0.00916 | 0.01 | 0.00756 | 0.00943 | 0.00881 | 0.00815 | 0.00705 | 0.00604 | 0.00503 | 0.00399 |
| Total VOC | 1.97615 | 2.18615 | 2.99154 | 3.16308 | 2.59769 | 3.01846 | 2.88538 | 2.72923 | 2.44846 | 2.18462 | 1.90692 | 1.60308 |
| RON98 |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00868 | 0.00966 | 0.01347 | 0.01433 | 0.01158 | 0.01364 | 0.01298 | 0.01223 | 0.0109 | 0.00965 | 0.00835 | 0.00694 |
| Cumene | 0.00007 | 0.00008 | 0.00012 | 0.00013 | 0.0001 | 0.00012 | 0.00011 | 0.0001 | 0.00009 | 0.00008 | 0.00006 | 0.00005 |
| Cyclohexane | 0.00981 | 0.0109 | 0.01509 | 0.01601 | 0.01302 | 0.01526 | 0.01455 | 0.01372 | 0.01226 | 0.01089 | 0.00945 | 0.00789 |
| Ethylbenzene | 0.00148 | 0.00169 | 0.00255 | 0.00278 | 0.00211 | 0.00262 | 0.00245 | 0.00227 | 0.00196 | 0.00169 | 0.00141 | 0.00112 |
| n-Hexane | 0.02843 | 0.03145 | 0.04303 | 0.0455 | 0.03737 | 0.04342 | 0.04151 | 0.03927 | 0.03522 | 0.03143 | 0.02743 | 0.02306 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.04839 | 0.05454 | 0.079 | 0.08504 | 0.06665 | 0.08059 | 0.07606 | 0.07105 | 0.06245 | 0.05448 | 0.0463 | 0.0377 |
| Xylenes | 0.00664 | 0.0076 | 0.01151 | 0.01256 | 0.00949 | 0.01184 | 0.01106 | 0.01023 | 0.00885 | 0.00759 | 0.00632 | 0.00502 |
| Total VOC | 2.18692 | 2.41923 | 3.31000 | 3.50000 | 2.87462 | 3.34000 | 3.19308 | 3.02077 | 2.70923 | 2.41769 | 2.11000 | 1.77385 |
| E10 |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00853 | 0.0095 | 0.01324 | 0.01408 | 0.01139 | 0.01341 | 0.01276 | 0.01202 | 0.01071 | 0.00949 | 0.00821 | 0.00683 |
| Ethylbenzene | 0.00164 | 0.00187 | 0.00282 | 0.00308 | 0.00233 | 0.0029 | 0.00271 | 0.00251 | 0.00218 | 0.00187 | 0.00156 | 0.00124 |
| n-Hexane | 0.02808 | 0.03106 | 0.0425 | 0.04494 | 0.03691 | 0.04288 | 0.04099 | 0.03878 | 0.03479 | 0.03104 | 0.02709 | 0.02278 |
| Lead | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 |
| Toluene | 0.01847 | 0.02082 | 0.03015 | 0.03246 | 0.02544 | 0.03076 | 0.02903 | 0.02712 | 0.02384 | 0.0208 | 0.01767 | 0.01439 |
| Xylenes | 0.00683 | 0.00781 | 0.01183 | 0.01291 | 0.00976 | 0.01217 | 0.01137 | 0.01052 | 0.0091 | 0.0078 | 0.00649 | 0.00516 |
| Ethanol | 0.04872 | 0.05567 | 0.08409 | 0.09173 | 0.06945 | 0.08649 | 0.08085 | 0.07482 | 0.06479 | 0.0556 | 0.04635 | 0.03688 |
| Total VOC | 2.16000 | 2.38923 | 3.26923 | 3.45692 | 2.83923 | 3.29846 | 3.15308 | 2.98308 | 2.67615 | 2.38769 | 2.08385 | 1.75231 |

## Appendix F.2: Fuel storage- internal floating roof tank

|  | Zone (kg/tonne) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
|  | South SA | South WA | Central WA | Northern Australia | Central Australia | Cape York | Central North Qld | Central Qld | South Qld | NSW | Vic | Tas |
| Crude oil |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00015 | 0.00015 | 0.00016 | 0.00016 | 0.00015 | 0.00016 | 0.00016 | 0.00016 | 0.00015 | 0.00015 | 0.00015 | 0.00015 |
| Cumene | 0.00006 | 0.00006 | 0.00006 | 0.00006 | 0.00006 | 0.00006 | 0.00006 | 0.00006 | 0.00006 | 0.00006 | 0.00006 | 0.00006 |
| Cyclohexane | 0.00127 | 0.00129 | 0.00133 | 0.00135 | 0.00131 | 0.00134 | 0.00133 | 0.00132 | 0.00130 | 0.00129 | 0.00127 | 0.00125 |
| Ethylbenzene | 0.00025 | 0.00025 | 0.00025 | 0.00025 | 0.00025 | 0.00025 | 0.00025 | 0.00025 | 0.00025 | 0.00025 | 0.00025 | 0.00025 |
| n-Hexane | 0.00361 | 0.00366 | 0.00386 | 0.00392 | 0.00376 | 0.00388 | 0.00384 | 0.00379 | 0.00373 | 0.00367 | 0.00359 | 0.00353 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00065 | 0.00065 | 0.00066 | 0.00066 | 0.00065 | 0.00066 | 0.00066 | 0.00066 | 0.00065 | 0.00065 | 0.00065 | 0.00065 |
| Xylenes | 0.00110 | 0.00111 | 0.00111 | 0.00111 | 0.00111 | 0.00111 | 0.00111 | 0.00111 | 0.00111 | 0.00111 | 0.00110 | 0.00110 |
| Total VOC | 0.00710 | 0.00717 | 0.00743 | 0.00752 | 0.00730 | 0.00746 | 0.00741 | 0.00734 | 0.00725 | 0.00718 | 0.00708 | 0.00699 |
| Fuel oil |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 |
| Cumene | 0.00010 | 0.00010 | 0.00010 | 0.00010 | 0.00010 | 0.00010 | 0.00010 | 0.00010 | 0.00010 | 0.00010 | 0.00010 | 0.00010 |
| Cyclohexane | - | - | - | - | - | - | - | - | - | - | - | - |
| Ethylbenzene | <0.00001 | <0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | <0.00001 | <0.00001 | <0.00001 |
| n-Hexane | 0.00001 | 0.00001 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00001 | 0.00001 | 0.00001 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 |
| Xylenes | 0.04308 | 0.04308 | 0.04308 | 0.04308 | 0.04308 | 0.04308 | 0.04308 | 0.04308 | 0.04308 | 0.04308 | 0.04308 | 0.04308 |
| Total VOC | 0.03030 | 0.03030 | 0.03030 | 0.03030 | 0.03030 | 0.03030 | 0.03030 | 0.03030 | 0.03030 | 0.03030 | 0.03030 | 0.03030 |
| Heating oil |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | - | - | - | - | - | - | - | - | - | - | - | - |
| Cumene | - | - | - | - | - | - | - | - | - | - | - | - |
| Cyclohexane | - | - | - | - | - | - | - | - | - | - | - | - |
| Ethylbenzene | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00003 |
| n-Hexane | - | - | - | - | - | - | - | - | - | - | - | - |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 |
| Xylenes | 0.00014 | 0.00014 | 0.00014 | 0.00014 | 0.00014 | 0.00014 | 0.00014 | 0.00014 | 0.00014 | 0.00014 | 0.00014 | 0.00013 |
| Total VOC | 0.04819 | 0.04819 | 0.04819 | 0.04819 | 0.04819 | 0.04819 | 0.04819 | 0.04819 | 0.04819 | 0.04819 | 0.04819 | 0.04819 |
| Kerosene |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00027 | 0.00028 | 0.00032 | 0.00034 | 0.00030 | 0.00033 | 0.00032 | 0.00031 | 0.00029 | 0.00028 | 0.00026 | 0.00025 |
| Cumene | 0.00134 | 0.00135 | 0.00137 | 0.00138 | 0.00136 | 0.00137 | 0.00137 | 0.00136 | 0.00136 | 0.00135 | 0.00134 | 0.00133 |
| Cyclohexane | 0.00089 | 0.00093 | 0.00107 | 0.00111 | 0.00100 | 0.00108 | 0.00106 | 0.00102 | 0.00097 | 0.00093 | 0.00088 | 0.00082 |
| Ethylbenzene | 0.00025 | 0.00025 | 0.00026 | 0.00026 | 0.00026 | 0.00026 | 0.00026 | 0.00026 | 0.00026 | 0.00025 | 0.00025 | 0.00025 |
| n-Hexane | 0.00418 | 0.00442 | 0.00522 | 0.00549 | 0.00481 | 0.00530 | 0.00516 | 0.00494 | 0.00468 | 0.00443 | 0.00411 | 0.00381 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00010 | 0.00010 | 0.00011 | 0.00011 | 0.00010 | 0.00011 | 0.00011 | 0.00010 | 0.00010 | 0.00010 | 0.00010 | 0.00009 |
| Xylenes | 0.00091 | 0.00092 | 0.00094 | 0.00095 | 0.00093 | 0.00094 | 0.00094 | 0.00093 | 0.00092 | 0.00092 | 0.00091 | 0.00090 |
| Total VOC | 0.09091 | 0.09091 | 0.10000 | 0.10000 | 0.09091 | 0.10000 | 0.10000 | 0.09091 | 0.09091 | 0.09091 | 0.09091 | 0.08182 |
| Avgas 100 |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00085 | 0.00088 | 0.00101 | 0.00105 | 0.00094 | 0.00102 | 0.00100 | 0.00096 | 0.00092 | 0.00088 | 0.00084 | 0.00079 |
| Cumene | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 |
| Cyclohexane | 0.00032 | 0.00033 | 0.00038 | 0.00039 | 0.00035 | 0.00038 | 0.00037 | 0.00036 | 0.00035 | 0.00033 | 0.00031 | 0.00030 |
| Ethylbenzene | 0.00008 | 0.00008 | 0.00008 | 0.00008 | 0.00008 | 0.00008 | 0.00008 | 0.00008 | 0.00008 | 0.00008 | 0.00008 | 0.00008 |
| n-Hexane | 0.00071 | 0.00075 | 0.00088 | 0.00092 | 0.00081 | 0.00089 | 0.00087 | 0.00083 | 0.00079 | 0.00075 | 0.00070 | 0.00065 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00104 | 0.00106 | 0.00113 | 0.00115 | 0.00109 | 0.00114 | 0.00112 | 0.00111 | 0.00108 | 0.00106 | 0.00104 | 0.00101 |
| Xylenes | 0.00043 | 0.00043 | 0.00044 | 0.00044 | 0.00043 | 0.00044 | 0.00044 | 0.00043 | 0.00043 | 0.00043 | 0.00042 | 0.00042 |
| Total VOC | 0.06455 | 0.06818 | 0.08000 | 0.08364 | 0.07364 | 0.08091 | 0.07909 | 0.07545 | 0.07182 | 0.06818 | 0.06364 | 0.05909 |
| Avgas LL |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00252 | 0.00263 | 0.00300 | 0.00312 | 0.00281 | 0.00303 | 0.00297 | 0.00287 | 0.00275 | 0.00263 | 0.00249 | 0.00235 |
| Cumene | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 |
| Cyclohexane | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 |
| Ethylbenzene | 0.00282 | 0.00284 | 0.00292 | 0.00295 | 0.00288 | 0.00293 | 0.00291 | 0.00289 | 0.00287 | 0.00285 | 0.00282 | 0.00279 |
| n-Hexane | 0.00194 | 0.00204 | 0.00239 | 0.00250 | 0.00221 | 0.00242 | 0.00236 | 0.00227 | 0.00215 | 0.00204 | 0.00190 | 0.00177 |
| Lead | 0.00011 | 0.00011 | 0.00011 | 0.00011 | 0.00011 | 0.00011 | 0.00011 | 0.00011 | 0.00011 | 0.00011 | 0.00011 | 0.00011 |
| Toluene | 0.00678 | 0.00690 | 0.00733 | 0.00748 | 0.00711 | 0.00738 | 0.00730 | 0.00718 | 0.00704 | 0.00691 | 0.00674 | 0.00659 |
| Xylenes | 0.00291 | 0.00299 | 0.00302 | 0.00296 | 0.00300 | 0.00299 | 0.00297 | 0.00295 | 0.00293 | 0.00293 | 0.00290 | 0.00288 |
| Total VOC | 0.06576 | 0.06915 | 0.08102 | 0.08475 | 0.07492 | 0.08203 | 0.08000 | 0.07695 | 0.07288 | 0.06915 | 0.06441 | 0.06000 |
| Diesel |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00002 | 0.00002 | 0.00002 | 0.00003 | 0.00002 | 0.00003 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 |
| Cumene | 0.00040 | 0.00040 | 0.00041 | 0.00041 | 0.00040 | 0.00041 | 0.00041 | 0.00040 | 0.00040 | 0.00040 | 0.00040 | 0.00039 |
| Cyclohexane | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 |
| Ethylbenzene | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 |
| n-Hexane | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 |
| Xylenes | 0.00014 | 0.00014 | 0.00015 | 0.00015 | 0.00015 | 0.00015 | 0.00015 | 0.00015 | 0.00015 | 0.00014 | 0.00014 | 0.00014 |
| Total VOC | 0.00154 | 0.00154 | 0.00154 | 0.00154 | 0.00154 | 0.00154 | 0.00154 | 0.00154 | 0.00154 | 0.00154 | 0.00154 | 0.00154 |
| LP |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00049 | 0.00051 | 0.00059 | 0.00061 | 0.00055 | 0.00059 | 0.00058 | 0.00056 | 0.00054 | 0.00051 | 0.00048 | 0.00046 |
| Cumene | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 |
| Cyclohexane | 0.00044 | 0.00046 | 0.00053 | 0.00055 | 0.0005 | 0.00053 | 0.00052 | 0.00051 | 0.00048 | 0.00046 | 0.00044 | 0.00041 |
| Ethylbenzene | 0.00059 | 0.0006 | 0.00061 | 0.00062 | 0.0006 | 0.00062 | 0.00061 | 0.00061 | 0.0006 | 0.0006 | 0.00059 | 0.00059 |
| n-Hexane | 0.00173 | 0.00182 | 0.00215 | 0.00225 | 0.00199 | 0.00217 | 0.00212 | 0.00203 | 0.00193 | 0.00183 | 0.0017 | 0.00158 |
| Lead | 0.00018 | 0.00018 | 0.00018 | 0.00018 | 0.00018 | 0.00018 | 0.00018 | 0.00018 | 0.00018 | 0.00018 | 0.00018 | 0.00018 |
| Toluene | 0.00322 | 0.00328 | 0.0035 | 0.00357 | 0.00339 | 0.00352 | 0.00348 | 0.00342 | 0.00335 | 0.00329 | 0.00321 | 0.00313 |
| Xylenes | 0.003 | 0.00302 | 0.00309 | 0.00312 | 0.00305 | 0.0031 | 0.00308 | 0.00306 | 0.00304 | 0.00302 | 0.00299 | 0.00297 |
| Total VOC | 0.13308 | 0.14000 | 0.16538 | 0.17308 | 0.15308 | 0.16692 | 0.16308 | 0.15615 | 0.14846 | 0.14077 | 0.13077 | 0.12154 |
| ULP |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00056 | 0.00059 | 0.00067 | 0.0007 | 0.00063 | 0.00068 | 0.00066 | 0.00064 | 0.00061 | 0.00059 | 0.00055 | 0.00052 |
| Cumene | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 |
| Cyclohexane | 0.00047 | 0.00049 | 0.00056 | 0.00058 | 0.00052 | 0.00056 | 0.00055 | 0.00053 | 0.00051 | 0.00049 | 0.00046 | 0.00043 |
| Ethylbenzene | 0.00063 | 0.00063 | 0.00065 | 0.00066 | 0.00064 | 0.00065 | 0.00065 | 0.00064 | 0.00064 | 0.00063 | 0.00063 | 0.00062 |
| n-Hexane | 0.00134 | 0.00142 | 0.00167 | 0.00175 | 0.00154 | 0.00169 | 0.00165 | 0.00158 | 0.0015 | 0.00142 | 0.00132 | 0.00123 |
| Lead | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 |
| Toluene | 0.00252 | 0.00256 | 0.00273 | 0.00279 | 0.00265 | 0.00275 | 0.00272 | 0.00267 | 0.00262 | 0.00257 | 0.0025 | 0.00244 |
| Xylenes | 0.00314 | 0.00316 | 0.00324 | 0.00326 | 0.0032 | 0.00325 | 0.00323 | 0.00321 | 0.00319 | 0.00316 | 0.00314 | 0.00311 |
| Total VOC | 0.10308 | 0.10923 | 0.12846 | 0.13462 | 0.11846 | 0.13000 | 0.12692 | 0.12154 | 0.11538 | 0.10923 | 0.10154 | 0.09462 |
| PULP |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.0006 | 0.00063 | 0.00072 | 0.00075 | 0.00067 | 0.00073 | 0.00071 | 0.00069 | 0.00066 | 0.00063 | 0.00059 | 0.00056 |
| Cumene | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 |
| Cyclohexane | 0.0006 | 0.00063 | 0.00072 | 0.00075 | 0.00067 | 0.00073 | 0.00071 | 0.00069 | 0.00066 | 0.00063 | 0.00059 | 0.00056 |
| Ethylbenzene | 0.00072 | 0.00073 | 0.00075 | 0.00075 | 0.00074 | 0.00075 | 0.00074 | 0.00074 | 0.00073 | 0.00073 | 0.00072 | 0.00071 |
| n-Hexane | 0.00112 | 0.00118 | 0.00139 | 0.00145 | 0.00128 | 0.0014 | 0.00137 | 0.00131 | 0.00124 | 0.00118 | 0.0011 | 0.00102 |
| Lead | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 |
| Toluene | 0.00319 | 0.00325 | 0.00346 | 0.00353 | 0.00335 | 0.00348 | 0.00344 | 0.00338 | 0.00331 | 0.00325 | 0.00317 | 0.00309 |
| Xylenes | 0.00364 | 0.00366 | 0.00375 | 0.00378 | 0.00371 | 0.00376 | 0.00375 | 0.00372 | 0.00369 | 0.00367 | 0.00363 | 0.00361 |
| Total VOC | 0.08615 | 0.09077 | 0.10692 | 0.11154 | 0.09846 | 0.10769 | 0.10538 | 0.10077 | 0.09538 | 0.09077 | 0.08462 | 0.07846 |
| RON98 |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.0006 | 0.00063 | 0.00072 | 0.00075 | 0.00068 | 0.00073 | 0.00072 | 0.00069 | 0.00066 | 0.00063 | 0.0006 | 0.00056 |
| Cumene | 0.00007 | 0.00007 | 0.00007 | 0.00007 | 0.00007 | 0.00007 | 0.00007 | 0.00007 | 0.00007 | 0.00007 | 0.00007 | 0.00007 |
| Cyclohexane | 0.00067 | 0.0007 | 0.0008 | 0.00083 | 0.00075 | 0.00081 | 0.00079 | 0.00076 | 0.00073 | 0.0007 | 0.00066 | 0.00062 |
| Ethylbenzene | 0.00074 | 0.00074 | 0.00076 | 0.00077 | 0.00075 | 0.00077 | 0.00076 | 0.00076 | 0.00075 | 0.00074 | 0.00074 | 0.00073 |
| n-Hexane | 0.00149 | 0.00157 | 0.00185 | 0.00193 | 0.00171 | 0.00187 | 0.00182 | 0.00175 | 0.00166 | 0.00158 | 0.00146 | 0.00136 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00883 | 0.009 | 0.00959 | 0.00978 | 0.00928 | 0.00964 | 0.00954 | 0.00937 | 0.00918 | 0.009 | 0.00878 | 0.00857 |
| Xylenes | 0.00395 | 0.00397 | 0.00407 | 0.0041 | 0.00402 | 0.00408 | 0.00406 | 0.00403 | 0.004 | 0.00397 | 0.00394 | 0.00391 |
| Total VOC | 0.11462 | 0.12077 | 0.14231 | 0.14846 | 0.13154 | 0.14385 | 0.14000 | 0.13462 | 0.12769 | 0.12154 | 0.11231 | 0.10462 |
| E10 |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00059 | 0.00062 | 0.00071 | 0.00074 | 0.00067 | 0.00072 | 0.0007 | 0.00068 | 0.00065 | 0.00062 | 0.00059 | 0.00055 |
| Cumene | - | - | - | - | - | - | - | - | - | - | - | - |
| Cyclohexane | - | - | - | - | - | - | - | - | - | - | - | - |
| Ethylbenzene | 0.00082 | 0.00082 | 0.00085 | 0.00085 | 0.00083 | 0.00085 | 0.00085 | 0.00084 | 0.00083 | 0.00082 | 0.00082 | 0.00081 |
| n-Hexane | 0.00147 | 0.00155 | 0.00183 | 0.00191 | 0.00169 | 0.00184 | 0.0018 | 0.00173 | 0.00164 | 0.00156 | 0.00145 | 0.00132 |
| Lead | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 |
| Toluene | 0.00337 | 0.00343 | 0.00366 | 0.00373 | 0.00354 | 0.00368 | 0.00364 | 0.00358 | 0.0035 | 0.00344 | 0.00335 | 0.00326 |
| Xylenes | 0.00405 | 0.00408 | 0.00418 | 0.00421 | 0.00413 | 0.00419 | 0.00417 | 0.00414 | 0.00411 | 0.00408 | 0.00405 | 0.00401 |
| Ethanol | 0.00508 | 0.00527 | 0.00596 | 0.0062 | 0.0056 | 0.00603 | 0.00591 | 0.00572 | 0.00549 | 0.00528 | 0.00503 | 0.00477 |
| Total VOC | 0.11308 | 0.11923 | 0.14077 | 0.14692 | 0.13000 | 0.14154 | 0.13846 | 0.13308 | 0.12615 | 0.12000 | 0.11154 | 0.10154 |

## Appendix F.3: Fuel storage- external floating roof tank

|  | Zone (kg/tonne) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
|  | South SA | South WA | Central WA | Northern Australia | Central Australia | Cape York | Central North Qld | Central Qld | South Qld | NSW | Vic | Tas |
| Crude oil |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00019 | 0.00020 | 0.00022 | 0.00023 | 0.00019 | 0.00019 | 0.00019 | 0.00018 | 0.00018 | 0.00020 | 0.00018 | 0.00017 |
| Cumene | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 |
| Cyclohexane | 0.00184 | 0.00167 | 0.00164 | 0.00164 | 0.00173 | 0.00194 | 0.00162 | 0.00157 | 0.00171 | 0.00150 | 0.00155 | 0.00144 |
| Ethylbenzene | 0.00022 | 0.00021 | 0.00021 | 0.00021 | 0.00021 | 0.00022 | 0.00021 | 0.00021 | 0.00021 | 0.00021 | 0.00021 | 0.00020 |
| n-Hexane | 0.00636 | 0.00563 | 0.00549 | 0.00556 | 0.00492 | 0.00517 | 0.00593 | 0.00676 | 0.00542 | 0.00520 | 0.00583 | 0.00472 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00061 | 0.00059 | 0.00059 | 0.00062 | 0.00065 | 0.00066 | 0.00061 | 0.00061 | 0.00061 | 0.00060 | 0.00062 | 0.00057 |
| Xylenes | 0.00092 | 0.00091 | 0.00091 | 0.00093 | 0.00094 | 0.00095 | 0.00092 | 0.00092 | 0.00092 | 0.00092 | 0.00092 | 0.00090 |
| Total VOC | 0.01018 | 0.00926 | 0.00911 | 0.00924 | 0.00870 | 0.00919 | 0.00954 | 0.01030 | 0.00910 | 0.00868 | 0.00936 | 0.00806 |
| Fuel oil |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00004 | 0.00005 | 0.00005 | 0.00006 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00003 | 0.00005 | 0.00004 | 0.00003 |
| Cumene | 0.00011 | 0.00013 | 0.00012 | 0.00012 | 0.00014 | 0.00012 | 0.00012 | 0.00011 | 0.00011 | 0.00012 | 0.00011 | 0.00010 |
| Cyclohexane | - | - | - | - | - | - | - | - | - | - | - | - |
| Ethylbenzene | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 |
| n-Hexane | 0.00008 | 0.00007 | 0.00006 | 0.00007 | 0.00008 | 0.00009 | 0.00006 | 0.00006 | 0.00007 | 0.00005 | 0.00006 | 0.00005 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00005 | 0.00005 | 0.00004 | 0.00004 | 0.00005 | 0.00006 | 0.00004 | 0.00004 | 0.00004 | 0.00005 | 0.00004 | 0.00003 |
| Xylenes | 0.00008 | 0.00007 | 0.00007 | 0.00008 | 0.00007 | 0.00006 | 0.00007 | 0.00007 | 0.00007 | 0.00006 | 0.00006 | 0.00006 |
| Total VOC | 0.03333 | 0.03939 | 0.03636 | 0.03636 | 0.04242 | 0.03636 | 0.03636 | 0.03333 | 0.03333 | 0.03636 | 0.03333 | 0.03030 |
| Heating oil |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | - | - | - | - | - | - | - | - | - | - | - | - |
| Cumene | - | - | - | - | - | - | - | - | - | - | - | - |
| Cyclohexane | - | - | - | - | - | - | - | - | - | - | - | - |
| Ethylbenzene | 0.00003 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00003 | 0.00003 | 0.00004 | 0.00003 | 0.00003 |
| n-Hexane | - | - | - | - | - | - | - | - | - | - | - | - |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00007 | 0.00008 | 0.00008 | 0.00009 | 0.00007 | 0.00007 | 0.00007 | 0.00007 | 0.00006 | 0.00007 | 0.00006 | 0.00006 |
| Xylenes | 0.00015 | 0.00016 | 0.00017 | 0.00018 | 0.00016 | 0.00016 | 0.00016 | 0.00015 | 0.00015 | 0.00016 | 0.00015 | 0.00014 |
| Total VOC | 0.08434 | 0.09639 | 0.09639 | 0.10843 | 0.08434 | 0.08434 | 0.08434 | 0.08434 | 0.07229 | 0.08434 | 0.07229 | 0.07229 |
| Kerosene |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00069 | 0.00076 | 0.00083 | 0.00091 | 0.00070 | 0.00066 | 0.00067 | 0.00062 | 0.00057 | 0.00074 | 0.00062 | 0.00053 |
| Cumene | 0.00106 | 0.00109 | 0.00115 | 0.00119 | 0.00108 | 0.00108 | 0.00108 | 0.00106 | 0.00103 | 0.00109 | 0.00103 | 0.00100 |
| Cyclohexane | 0.00234 | 0.00257 | 0.00279 | 0.00302 | 0.00235 | 0.00220 | 0.00225 | 0.00208 | 0.00191 | 0.00250 | 0.00209 | 0.00180 |
| Ethylbenzene | 0.00023 | 0.00025 | 0.00026 | 0.00028 | 0.00024 | 0.00024 | 0.00024 | 0.00023 | 0.00022 | 0.00024 | 0.00022 | 0.00021 |
| n-Hexane | 0.01345 | 0.01478 | 0.01597 | 0.01732 | 0.01341 | 0.01240 | 0.01273 | 0.01172 | 0.01074 | 0.01438 | 0.01195 | 0.01027 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00014 | 0.00015 | 0.00016 | 0.00017 | 0.00014 | 0.00014 | 0.00014 | 0.00013 | 0.00012 | 0.00014 | 0.00013 | 0.00011 |
| Xylenes | 0.00081 | 0.00084 | 0.00090 | 0.00094 | 0.00083 | 0.00082 | 0.00082 | 0.00080 | 0.00077 | 0.00083 | 0.00077 | 0.00073 |
| Total VOC | 0.12727 | 0.13636 | 0.14545 | 0.15455 | 0.12727 | 0.12727 | 0.12727 | 0.11818 | 0.10909 | 0.12727 | 0.11818 | 0.10000 |
| Avgas 100 |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00252 | 0.00276 | 0.00301 | 0.00326 | 0.00254 | 0.00238 | 0.00243 | 0.00225 | 0.00207 | 0.00269 | 0.00225 | 0.00195 |
| Cumene | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 |
| Cyclohexane | 0.00096 | 0.00105 | 0.00114 | 0.00123 | 0.00097 | 0.00090 | 0.00092 | 0.00085 | 0.00079 | 0.00103 | 0.00086 | 0.00075 |
| Ethylbenzene | 0.00009 | 0.00009 | 0.00010 | 0.00010 | 0.00009 | 0.00009 | 0.00009 | 0.00008 | 0.00008 | 0.00009 | 0.00008 | 0.00008 |
| n-Hexane | 0.00265 | 0.00290 | 0.00313 | 0.00339 | 0.00264 | 0.00243 | 0.00250 | 0.00230 | 0.00212 | 0.00283 | 0.00236 | 0.00203 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00169 | 0.00181 | 0.00198 | 0.00212 | 0.00173 | 0.00168 | 0.00170 | 0.00160 | 0.00150 | 0.00178 | 0.00156 | 0.00140 |
| Xylenes | 0.00044 | 0.00046 | 0.00049 | 0.00051 | 0.00045 | 0.00045 | 0.00045 | 0.00044 | 0.00042 | 0.00046 | 0.00043 | 0.00041 |
| Total VOC | 0.24091 | 0.26364 | 0.28455 | 0.30818 | 0.24000 | 0.22091 | 0.22727 | 0.20909 | 0.19273 | 0.25727 | 0.21455 | 0.18455 |
| Avgas LL |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00749 | 0.00822 | 0.00895 | 0.00970 | 0.00756 | 0.00709 | 0.00725 | 0.00670 | 0.00616 | 0.00801 | 0.00671 | 0.00580 |
| Cumene | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 |
| Cyclohexane | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 |
| Ethylbenzene | 0.00308 | 0.00322 | 0.00344 | 0.00360 | 0.00316 | 0.00313 | 0.00314 | 0.00303 | 0.00292 | 0.00319 | 0.00295 | 0.00278 |
| n-Hexane | 0.00719 | 0.00789 | 0.00849 | 0.00919 | 0.00716 | 0.00660 | 0.00679 | 0.00626 | 0.00575 | 0.00768 | 0.00640 | 0.00551 |
| Lead | 0.00009 | 0.00009 | 0.00009 | 0.00009 | 0.00009 | 0.00009 | 0.00009 | 0.00009 | 0.00009 | 0.00009 | 0.00009 | 0.00009 |
| Toluene | 0.01096 | 0.01178 | 0.01284 | 0.01375 | 0.01123 | 0.01089 | 0.01101 | 0.01037 | 0.00973 | 0.01157 | 0.01013 | 0.00912 |
| Xylenes | 0.00303 | 0.00316 | 0.00335 | 0.00349 | 0.00310 | 0.00308 | 0.00309 | 0.00299 | 0.00290 | 0.00313 | 0.00292 | 0.00277 |
| Total VOC | 0.24373 | 0.26746 | 0.28780 | 0.31153 | 0.24271 | 0.22373 | 0.23017 | 0.21220 | 0.19492 | 0.26034 | 0.21695 | 0.18678 |
| Diesel |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00007 | 0.00007 | 0.00009 | 0.00008 | 0.00007 | 0.00006 | 0.00007 | 0.00006 | 0.00006 | 0.00007 | 0.00006 | 0.00005 |
| Cumene | 0.00039 | 0.00043 | 0.00040 | 0.00040 | 0.00044 | 0.00040 | 0.00040 | 0.00039 | 0.00038 | 0.00040 | 0.00038 | 0.00036 |
| Cyclohexane | 0.00002 | 0.00003 | 0.00002 | 0.00003 | 0.00003 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00003 | 0.00002 | 0.00002 |
| Ethylbenzene | 0.00005 | 0.00006 | 0.00006 | 0.00006 | 0.00007 | 0.00006 | 0.00006 | 0.00005 | 0.00005 | 0.00006 | 0.00005 | 0.00005 |
| n-Hexane | 0.00004 | 0.00004 | 0.00003 | 0.00004 | 0.00005 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00004 | 0.00003 | 0.00003 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00009 | 0.00010 | 0.00009 | 0.00009 | 0.00011 | 0.00009 | 0.00009 | 0.00008 | 0.00008 | 0.00009 | 0.00008 | 0.00007 |
| Xylenes | 0.00016 | 0.00018 | 0.00017 | 0.00017 | 0.00019 | 0.00016 | 0.00016 | 0.00016 | 0.00015 | 0.00017 | 0.00015 | 0.00014 |
| Total VOC | 0.00615 | 0.00615 | 0.00462 | 0.00615 | 0.00769 | 0.00462 | 0.00462 | 0.00462 | 0.00462 | 0.00615 | 0.00462 | 0.00462 |
| LP |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00149 | 0.00164 | 0.00179 | 0.00194 | 0.00151 | 0.00141 | 0.00144 | 0.00133 | 0.00122 | 0.0016 | 0.00133 | 0.00115 |
| Cumene | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 |
| Cyclohexane | 0.00137 | 0.0015 | 0.00163 | 0.00176 | 0.00138 | 0.00128 | 0.00131 | 0.00121 | 0.00112 | 0.00146 | 0.00122 | 0.00106 |
| Ethylbenzene | 0.00065 | 0.00068 | 0.00073 | 0.00077 | 0.00067 | 0.00066 | 0.00067 | 0.00064 | 0.00062 | 0.00068 | 0.00063 | 0.00059 |
| n-Hexane | 0.00653 | 0.00719 | 0.00776 | 0.00838 | 0.00654 | 0.00599 | 0.00618 | 0.00568 | 0.00522 | 0.007 | 0.00581 | 0.005 |
| Lead | 0.00014 | 0.00014 | 0.00014 | 0.00014 | 0.00014 | 0.00014 | 0.00014 | 0.00014 | 0.00014 | 0.00014 | 0.00014 | 0.00014 |
| Toluene | 0.00531 | 0.00573 | 0.00626 | 0.0067 | 0.00546 | 0.00527 | 0.00535 | 0.00502 | 0.00471 | 0.00562 | 0.0049 | 0.0044 |
| Xylenes | 0.00316 | 0.00329 | 0.0035 | 0.00365 | 0.00324 | 0.00321 | 0.00322 | 0.00312 | 0.00301 | 0.00326 | 0.00304 | 0.00288 |
| Total VOC | 0.50231 | 0.55308 | 0.59692 | 0.64462 | 0.50308 | 0.46077 | 0.47538 | 0.43692 | 0.40154 | 0.53846 | 0.44692 | 0.38462 |
| ULP |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.0017 | 0.00187 | 0.00204 | 0.00221 | 0.00172 | 0.0016 | 0.00165 | 0.00152 | 0.0014 | 0.00182 | 0.00152 | 0.00131 |
| Cumene | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 | 0.00004 |
| Cyclohexane | 0.00143 | 0.00157 | 0.00171 | 0.00185 | 0.00145 | 0.00134 | 0.00138 | 0.00127 | 0.00117 | 0.00154 | 0.00128 | 0.00111 |
| Ethylbenzene | 0.00069 | 0.00072 | 0.00078 | 0.00081 | 0.00071 | 0.0007 | 0.00071 | 0.00068 | 0.00065 | 0.00072 | 0.00066 | 0.00062 |
| n-Hexane | 0.00507 | 0.00559 | 0.00603 | 0.00652 | 0.00508 | 0.00465 | 0.0048 | 0.00442 | 0.00406 | 0.00544 | 0.00452 | 0.00389 |
| Lead | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 |
| Toluene | 0.00414 | 0.00447 | 0.00489 | 0.00523 | 0.00427 | 0.00412 | 0.00417 | 0.00392 | 0.00368 | 0.00439 | 0.00383 | 0.00344 |
| Xylenes | 0.00331 | 0.00345 | 0.00367 | 0.00383 | 0.00339 | 0.00336 | 0.00338 | 0.00326 | 0.00315 | 0.00342 | 0.00318 | 0.00302 |
| Total VOC | 0.39000 | 0.43000 | 0.46385 | 0.50154 | 0.39077 | 0.35769 | 0.36923 | 0.34000 | 0.31231 | 0.41846 | 0.34769 | 0.29923 |
| PULP |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00183 | 0.00201 | 0.00219 | 0.00237 | 0.00185 | 0.00172 | 0.00177 | 0.00163 | 0.0015 | 0.00196 | 0.00164 | 0.00141 |
| Cumene | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00005 | 0.00004 | 0.00004 | 0.00005 | 0.00004 | 0.00004 |
| Cyclohexane | 0.00185 | 0.00204 | 0.00221 | 0.00239 | 0.00187 | 0.00174 | 0.00178 | 0.00165 | 0.00152 | 0.00199 | 0.00166 | 0.00144 |
| Ethylbenzene | 0.0008 | 0.00083 | 0.00089 | 0.00093 | 0.00082 | 0.00081 | 0.00081 | 0.00078 | 0.00075 | 0.00082 | 0.00076 | 0.00072 |
| n-Hexane | 0.00421 | 0.00464 | 0.00501 | 0.00541 | 0.00422 | 0.00387 | 0.00399 | 0.00367 | 0.00337 | 0.00452 | 0.00375 | 0.00323 |
| Lead | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 |
| Toluene | 0.00525 | 0.00566 | 0.00619 | 0.00662 | 0.0054 | 0.00521 | 0.00528 | 0.00497 | 0.00466 | 0.00556 | 0.00485 | 0.00435 |
| Xylenes | 0.00383 | 0.004 | 0.00425 | 0.00444 | 0.00393 | 0.0039 | 0.00391 | 0.00378 | 0.00366 | 0.00396 | 0.00369 | 0.0035 |
| Total VOC | 0.32385 | 0.35692 | 0.38538 | 0.41615 | 0.32462 | 0.29769 | 0.30692 | 0.28231 | 0.25923 | 0.34769 | 0.28846 | 0.24846 |
| RON98 |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00183 | 0.00202 | 0.0022 | 0.00238 | 0.00186 | 0.00173 | 0.00178 | 0.00164 | 0.00151 | 0.00197 | 0.00164 | 0.00142 |
| Cumene | 0.00006 | 0.00007 | 0.00007 | 0.00007 | 0.00007 | 0.00006 | 0.00007 | 0.00006 | 0.00006 | 0.00007 | 0.00006 | 0.00006 |
| Cyclohexane | 0.00206 | 0.00226 | 0.00246 | 0.00266 | 0.00208 | 0.00193 | 0.00198 | 0.00183 | 0.00168 | 0.00221 | 0.00185 | 0.0016 |
| Ethylbenzene | 0.00081 | 0.00085 | 0.00091 | 0.00096 | 0.00084 | 0.00083 | 0.00083 | 0.0008 | 0.00077 | 0.00084 | 0.00078 | 0.00073 |
| n-Hexane | 0.00561 | 0.00618 | 0.00667 | 0.00721 | 0.00563 | 0.00515 | 0.00532 | 0.00489 | 0.00449 | 0.00603 | 0.005 | 0.0043 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.01453 | 0.01568 | 0.01714 | 0.01835 | 0.01496 | 0.01444 | 0.01464 | 0.01376 | 0.0129 | 0.0154 | 0.01343 | 0.01205 |
| Xylenes | 0.00416 | 0.00433 | 0.00461 | 0.00481 | 0.00426 | 0.00422 | 0.00424 | 0.0041 | 0.00396 | 0.00429 | 0.004 | 0.00379 |
| Total VOC | 0.43154 | 0.47538 | 0.51308 | 0.55462 | 0.43308 | 0.39615 | 0.40923 | 0.37615 | 0.34538 | 0.46385 | 0.38462 | 0.33077 |
| E10 |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.0018 | 0.00198 | 0.00217 | 0.00234 | 0.00183 | 0.0017 | 0.00175 | 0.00161 | 0.00148 | 0.00194 | 0.00161 | 0.00139 |
| Cumene | - | - | - | - | - | - | - | - | - | - | - | - |
| Cyclohexane | - | - | - | - | - | - | - | - | - | - | - | - |
| Ethylbenzene | 0.0009 | 0.00095 | 0.00101 | 0.00106 | 0.00093 | 0.00092 | 0.00092 | 0.00089 | 0.00085 | 0.00094 | 0.00086 | 0.00081 |
| n-Hexane | 0.00555 | 0.00611 | 0.00659 | 0.00712 | 0.00556 | 0.00509 | 0.00525 | 0.00483 | 0.00444 | 0.00595 | 0.00494 | 0.00425 |
| Lead | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 |
| Toluene | 0.00555 | 0.00598 | 0.00654 | 0.007 | 0.00571 | 0.00551 | 0.00559 | 0.00525 | 0.00492 | 0.00588 | 0.00513 | 0.0046 |
| Xylenes | 0.00427 | 0.00445 | 0.00473 | 0.00494 | 0.00438 | 0.00434 | 0.00436 | 0.00421 | 0.00407 | 0.00441 | 0.00411 | 0.00389 |
| Ethanol | 0.01163 | 0.01292 | 0.0149 | 0.01635 | 0.01237 | 0.01209 | 0.01222 | 0.0112 | 0.01019 | 0.01263 | 0.01047 | 0.00896 |
| Total VOC | 0.42692 | 0.47000 | 0.50692 | 0.54769 | 0.42769 | 0.39154 | 0.40385 | 0.37154 | 0.34154 | 0.45769 | 0.38000 | 0.32692 |

## Appendix F.4: Fuel storage- above ground horizontal fixed roof tank

|  | Zone (kg/tonne) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
|  | South SA | South WA | Central WA | Northern Australia | Central Australia | Cape York | Central North Qld | Central Qld | South Qld | NSW | Vic | Tas |
| Crude oil |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00061 | 0.00068 | 0.00102 | 0.00104 | 0.00086 | 0.00098 | 0.00094 | 0.00091 | 0.00078 | 0.00067 | 0.00058 | 0.00047 |
| Cumene | 0.00001 | 0.00001 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00001 | 0.00001 | 0.00001 |
| Cyclohexane | 0.00529 | 0.00591 | 0.00885 | 0.00897 | 0.00751 | 0.00851 | 0.00817 | 0.00787 | 0.00681 | 0.00587 | 0.00509 | 0.00410 |
| Ethylbenzene | 0.00010 | 0.00012 | 0.00019 | 0.00020 | 0.00016 | 0.00019 | 0.00018 | 0.00017 | 0.00014 | 0.00012 | 0.00010 | 0.00007 |
| n-Hexane | 0.02258 | 0.02513 | 0.03716 | 0.03755 | 0.03172 | 0.03565 | 0.03433 | 0.03314 | 0.02880 | 0.02493 | 0.02175 | 0.01763 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00079 | 0.00089 | 0.00140 | 0.00144 | 0.00116 | 0.00136 | 0.00129 | 0.00123 | 0.00105 | 0.00089 | 0.00075 | 0.00059 |
| Xylenes | 0.00038 | 0.00044 | 0.00071 | 0.00074 | 0.00058 | 0.00070 | 0.00066 | 0.00062 | 0.00052 | 0.00043 | 0.00036 | 0.00028 |
| Total VOC | 0.02977 | 0.03318 | 0.04935 | 0.04997 | 0.04200 | 0.04741 | 0.04559 | 0.04395 | 0.03812 | 0.03292 | 0.02865 | 0.02314 |
| Fuel oil |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00015 | 0.00017 | 0.00024 | 0.00025 | 0.00021 | 0.00024 | 0.00023 | 0.00022 | 0.00019 | 0.00017 | 0.00015 | 0.00012 |
| Cumene | 0.00015 | 0.00017 | 0.00027 | 0.00029 | 0.00022 | 0.00028 | 0.00026 | 0.00024 | 0.00020 | 0.00017 | 0.00014 | 0.00011 |
| Cyclohexane | - | - | - | - | - | - | - | - | - | - | - | - |
| Ethylbenzene | 0.00001 | 0.00002 | 0.00003 | 0.00003 | 0.00002 | 0.00003 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00001 | 0.00001 |
| n-Hexane | 0.00025 | 0.00027 | 0.00038 | 0.00040 | 0.00033 | 0.00038 | 0.00036 | 0.00035 | 0.00031 | 0.00027 | 0.00024 | 0.00020 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00013 | 0.00015 | 0.00022 | 0.00023 | 0.00018 | 0.00022 | 0.00021 | 0.00019 | 0.00017 | 0.00014 | 0.00012 | 0.00010 |
| Xylenes | 0.00012 | 0.00014 | 0.00021 | 0.00023 | 0.00017 | 0.00022 | 0.00020 | 0.00019 | 0.00016 | 0.00014 | 0.00011 | 0.00009 |
| Total VOC | 0.04545 | 0.05152 | 0.08182 | 0.08788 | 0.06667 | 0.08485 | 0.07879 | 0.07273 | 0.06061 | 0.05152 | 0.04242 | 0.03333 |
| Heating oil |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | - | - | - | - | - | - | - | - | - | - | - | - |
| Cumene | - | - | - | - | - | - | - | - | - | - | - | - |
| Cyclohexane | - | - | - | - | - | - | - | - | - | - | - | - |
| Ethylbenzene | 0.00005 | 0.00006 | 0.00009 | 0.00009 | 0.00007 | 0.00009 | 0.00008 | 0.00008 | 0.00007 | 0.00006 | 0.00005 | 0.00004 |
| n-Hexane | - | - | - | - | - | - | - | - | - | - | - | - |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00017 | 0.00019 | 0.00029 | 0.00030 | 0.00024 | 0.00029 | 0.00027 | 0.00026 | 0.00022 | 0.00019 | 0.00016 | 0.00013 |
| Xylenes | 0.00020 | 0.00022 | 0.00035 | 0.00037 | 0.00029 | 0.00035 | 0.00033 | 0.00031 | 0.00026 | 0.00022 | 0.00019 | 0.00015 |
| Total VOC | 0.20482 | 0.22892 | 0.34940 | 0.36145 | 0.28916 | 0.34940 | 0.32530 | 0.31325 | 0.26506 | 0.22892 | 0.19277 | 0.15663 |
| Kerosene |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00223 | 0.00247 | 0.00356 | 0.00371 | 0.00304 | 0.00355 | 0.00337 | 0.00322 | 0.00283 | 0.00246 | 0.00214 | 0.00175 |
| Cumene | 0.00078 | 0.00089 | 0.00143 | 0.00154 | 0.00116 | 0.00146 | 0.00135 | 0.00126 | 0.00106 | 0.00089 | 0.00073 | 0.00056 |
| Cyclohexane | 0.00754 | 0.00835 | 0.01192 | 0.01240 | 0.01022 | 0.01189 | 0.01130 | 0.01083 | 0.00951 | 0.00829 | 0.00724 | 0.00594 |
| Ethylbenzene | 0.00030 | 0.00034 | 0.00053 | 0.00056 | 0.00043 | 0.00054 | 0.00050 | 0.00047 | 0.00040 | 0.00034 | 0.00028 | 0.00022 |
| n-Hexane | 0.04599 | 0.05074 | 0.07159 | 0.07417 | 0.06172 | 0.07121 | 0.06788 | 0.06519 | 0.05753 | 0.05039 | 0.04425 | 0.03656 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00031 | 0.00035 | 0.00052 | 0.00055 | 0.00044 | 0.00053 | 0.00050 | 0.00047 | 0.00041 | 0.00035 | 0.00030 | 0.00024 |
| Xylenes | 0.00090 | 0.00103 | 0.00161 | 0.00172 | 0.00132 | 0.00163 | 0.00152 | 0.00143 | 0.00122 | 0.00102 | 0.00086 | 0.00067 |
| Total VOC | 0.28182 | 0.31818 | 0.47273 | 0.50000 | 0.40000 | 0.48182 | 0.45455 | 0.42727 | 0.37273 | 0.31818 | 0.27273 | 0.21818 |
| Avgas 100 |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00782 | 0.00868 | 0.01222 | 0.01294 | 0.01047 | 0.01241 | 0.01174 | 0.01114 | 0.00985 | 0.00864 | 0.00751 | 0.00620 |
| Cumene | <0.00001 | <0.00001 | 0.00001 | 0.00001 | <0.00001 | 0.00001 | 0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 |
| Cyclohexane | 0.00301 | 0.00333 | 0.00466 | 0.00492 | 0.00400 | 0.00472 | 0.00447 | 0.00425 | 0.00377 | 0.00331 | 0.00289 | 0.00240 |
| Ethylbenzene | 0.00010 | 0.00011 | 0.00017 | 0.00019 | 0.00014 | 0.00018 | 0.00017 | 0.00016 | 0.00013 | 0.00011 | 0.00009 | 0.00008 |
| n-Hexane | 0.00883 | 0.00973 | 0.01344 | 0.01415 | 0.01163 | 0.01360 | 0.01292 | 0.01232 | 0.01096 | 0.00968 | 0.00849 | 0.00709 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00365 | 0.00410 | 0.00600 | 0.00643 | 0.00504 | 0.00614 | 0.00576 | 0.00542 | 0.00473 | 0.00408 | 0.00348 | 0.00282 |
| Xylenes | 0.00046 | 0.00052 | 0.00079 | 0.00086 | 0.00065 | 0.00082 | 0.00076 | 0.00071 | 0.00061 | 0.00052 | 0.00043 | 0.00034 |
| Total VOC | 0.80273 | 0.88455 | 1.22182 | 1.28636 | 1.05727 | 1.23636 | 1.17455 | 1.12000 | 0.99636 | 0.88000 | 0.77182 | 0.64455 |
| Avgas LL |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.02330 | 0.02584 | 0.03640 | 0.03855 | 0.03118 | 0.03696 | 0.03496 | 0.03319 | 0.02934 | 0.02572 | 0.02236 | 0.01847 |
| Cumene | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | <0.00001 |
| Cyclohexane | 0.00001 | 0.00001 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00002 | 0.00001 | 0.00001 | 0.00001 |
| Ethylbenzene | 0.00360 | 0.00410 | 0.00624 | 0.00677 | 0.00514 | 0.00643 | 0.00598 | 0.00558 | 0.00480 | 0.00408 | 0.00342 | 0.00270 |
| n-Hexane | 0.02397 | 0.02641 | 0.03651 | 0.03844 | 0.03157 | 0.03693 | 0.03509 | 0.03344 | 0.02977 | 0.02629 | 0.02306 | 0.01925 |
| Lead | <0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | <0.00001 | <0.00001 |
| Toluene | 0.02373 | 0.02664 | 0.03898 | 0.04177 | 0.03275 | 0.03986 | 0.03739 | 0.03520 | 0.03070 | 0.02651 | 0.02264 | 0.01831 |
| Xylenes | 0.00312 | 0.00355 | 0.00543 | 0.00590 | 0.00446 | 0.00560 | 0.00520 | 0.00485 | 0.00416 | 0.00353 | 0.00295 | 0.00233 |
| Total VOC | 0.81254 | 0.89525 | 1.23763 | 1.30305 | 1.07017 | 1.25186 | 1.18949 | 1.13356 | 1.00915 | 0.89119 | 0.78169 | 0.65254 |
| Diesel |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00022 | 0.00025 | 0.00036 | 0.00037 | 0.00030 | 0.00036 | 0.00034 | 0.00032 | 0.00028 | 0.00025 | 0.00021 | 0.00018 |
| Cumene | 0.00033 | 0.00038 | 0.00060 | 0.00065 | 0.00049 | 0.00062 | 0.00057 | 0.00053 | 0.00045 | 0.00038 | 0.00031 | 0.00024 |
| Cyclohexane | 0.00008 | 0.00009 | 0.00012 | 0.00013 | 0.00010 | 0.00012 | 0.00012 | 0.00011 | 0.00010 | 0.00008 | 0.00007 | 0.00006 |
| Ethylbenzene | 0.00008 | 0.00009 | 0.00014 | 0.00015 | 0.00011 | 0.00014 | 0.00013 | 0.00012 | 0.00010 | 0.00009 | 0.00007 | 0.00006 |
| n-Hexane | 0.00012 | 0.00013 | 0.00019 | 0.00020 | 0.00016 | 0.00019 | 0.00018 | 0.00017 | 0.00015 | 0.00013 | 0.00012 | 0.00010 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00021 | 0.00024 | 0.00036 | 0.00038 | 0.00030 | 0.00036 | 0.00034 | 0.00032 | 0.00028 | 0.00024 | 0.00020 | 0.00016 |
| Xylenes | 0.00020 | 0.00023 | 0.00036 | 0.00039 | 0.00030 | 0.00037 | 0.00034 | 0.00032 | 0.00027 | 0.00023 | 0.00019 | 0.00015 |
| Total VOC | 0.01846 | 0.02000 | 0.02923 | 0.03077 | 0.02462 | 0.02923 | 0.02769 | 0.02615 | 0.02308 | 0.02000 | 0.01846 | 0.01538 |
| LP |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00433 | 0.00481 | 0.00666 | 0.00716 | 0.00572 | 0.00684 | 0.00647 | 0.00608 | 0.00542 | 0.0048 | 0.00415 | 0.00347 |
| Cumene | 0.00002 | 0.00003 | 0.00004 | 0.00005 | 0.00003 | 0.00004 | 0.00004 | 0.00004 | 0.00003 | 0.00003 | 0.00002 | 0.00002 |
| Cyclohexane | 0.00399 | 0.00443 | 0.00609 | 0.00653 | 0.00525 | 0.00624 | 0.00592 | 0.00557 | 0.00498 | 0.00442 | 0.00384 | 0.00322 |
| Ethylbenzene | 0.00073 | 0.00083 | 0.00125 | 0.00137 | 0.00103 | 0.0013 | 0.00121 | 0.00111 | 0.00097 | 0.00083 | 0.00069 | 0.00055 |
| n-Hexane | 0.02029 | 0.02241 | 0.03047 | 0.03255 | 0.02641 | 0.03117 | 0.02961 | 0.02794 | 0.0251 | 0.02239 | 0.01954 | 0.01648 |
| Lead | 0.00001 | 0.00001 | 0.00001 | 0.00002 | 0.00001 | 0.00002 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | <0.00001 |
| Toluene | 0.01085 | 0.01221 | 0.01757 | 0.01911 | 0.0148 | 0.01817 | 0.01705 | 0.01588 | 0.01398 | 0.0122 | 0.01037 | 0.00847 |
| Xylenes | 0.0031 | 0.00354 | 0.00533 | 0.00588 | 0.00439 | 0.00556 | 0.00516 | 0.00476 | 0.00413 | 0.00354 | 0.00294 | 0.00235 |
| Total VOC | 1.56077 | 1.72385 | 2.34385 | 2.50385 | 2.03154 | 2.39769 | 2.27769 | 2.14923 | 1.93077 | 1.72231 | 1.50308 | 1.26769 |
| ULP |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00494 | 0.00549 | 0.0076 | 0.00817 | 0.00653 | 0.0078 | 0.00738 | 0.00694 | 0.00619 | 0.00548 | 0.00474 | 0.00396 |
| Cumene | 0.00002 | 0.00003 | 0.00004 | 0.00005 | 0.00003 | 0.00004 | 0.00004 | 0.00004 | 0.00003 | 0.00003 | 0.00002 | 0.00002 |
| Cyclohexane | 0.00419 | 0.00465 | 0.00639 | 0.00685 | 0.00551 | 0.00655 | 0.00621 | 0.00584 | 0.00523 | 0.00464 | 0.00403 | 0.00338 |
| Ethylbenzene | 0.00077 | 0.00088 | 0.00132 | 0.00145 | 0.00109 | 0.00137 | 0.00128 | 0.00118 | 0.00102 | 0.00088 | 0.00073 | 0.00058 |
| n-Hexane | 0.01578 | 0.01743 | 0.02369 | 0.02531 | 0.02053 | 0.02423 | 0.02302 | 0.02172 | 0.01951 | 0.01741 | 0.0152 | 0.01282 |
| Lead | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 |
| Toluene | 0.00847 | 0.00954 | 0.01372 | 0.01492 | 0.01156 | 0.01419 | 0.01331 | 0.0124 | 0.01092 | 0.00952 | 0.00809 | 0.00661 |
| Xylenes | 0.00325 | 0.00371 | 0.00558 | 0.00615 | 0.0046 | 0.00582 | 0.00541 | 0.00499 | 0.00432 | 0.0037 | 0.00308 | 0.00246 |
| Total VOC | 1.21385 | 1.34077 | 1.82231 | 1.94692 | 1.57923 | 1.86385 | 1.77077 | 1.67077 | 1.50077 | 1.33923 | 1.16923 | 0.98615 |
| PULP |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00531 | 0.0059 | 0.00817 | 0.00878 | 0.00702 | 0.00839 | 0.00794 | 0.00746 | 0.00665 | 0.00589 | 0.0051 | 0.00425 |
| Cumene | 0.00003 | 0.00003 | 0.00005 | 0.00006 | 0.00004 | 0.00005 | 0.00005 | 0.00005 | 0.00004 | 0.00003 | 0.00003 | 0.00002 |
| Cyclohexane | 0.00542 | 0.00601 | 0.00827 | 0.00887 | 0.00713 | 0.00848 | 0.00804 | 0.00756 | 0.00676 | 0.00601 | 0.00521 | 0.00437 |
| Ethylbenzene | 0.00089 | 0.00101 | 0.00152 | 0.00167 | 0.00125 | 0.00158 | 0.00147 | 0.00136 | 0.00118 | 0.00101 | 0.00084 | 0.00067 |
| n-Hexane | 0.0131 | 0.01447 | 0.01967 | 0.02102 | 0.01705 | 0.02013 | 0.01912 | 0.01804 | 0.01621 | 0.01446 | 0.01262 | 0.01065 |
| Lead | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 |
| Toluene | 0.01073 | 0.01207 | 0.01737 | 0.01889 | 0.01463 | 0.01796 | 0.01685 | 0.0157 | 0.01382 | 0.01205 | 0.01025 | 0.00837 |
| Xylenes | 0.00377 | 0.0043 | 0.00647 | 0.00713 | 0.00533 | 0.00675 | 0.00627 | 0.00578 | 0.00501 | 0.00429 | 0.00357 | 0.00285 |
| Total VOC | 1.00769 | 1.11308 | 1.51308 | 1.61692 | 1.31154 | 1.54846 | 1.47077 | 1.38769 | 1.24692 | 1.11231 | 0.97077 | 0.81923 |
| RON98 |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00533 | 0.00592 | 0.0082 | 0.00882 | 0.00704 | 0.00842 | 0.00797 | 0.00749 | 0.00668 | 0.00592 | 0.00512 | 0.00427 |
| Cumene | 0.00004 | 0.00005 | 0.00007 | 0.00008 | 0.00006 | 0.00008 | 0.00007 | 0.00006 | 0.00006 | 0.00005 | 0.00004 | 0.00003 |
| Cyclohexane | 0.00602 | 0.00668 | 0.00919 | 0.00985 | 0.00792 | 0.00942 | 0.00893 | 0.0084 | 0.00752 | 0.00667 | 0.00579 | 0.00485 |
| Ethylbenzene | 0.00091 | 0.00104 | 0.00155 | 0.00171 | 0.00128 | 0.00162 | 0.0015 | 0.00139 | 0.0012 | 0.00103 | 0.00086 | 0.00069 |
| n-Hexane | 0.01746 | 0.01928 | 0.02621 | 0.028 | 0.02272 | 0.02681 | 0.02548 | 0.02404 | 0.02159 | 0.01926 | 0.01681 | 0.01418 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.02972 | 0.03344 | 0.04812 | 0.05233 | 0.04053 | 0.04977 | 0.04669 | 0.04349 | 0.03829 | 0.0334 | 0.02839 | 0.02319 |
| Xylenes | 0.00408 | 0.00466 | 0.00701 | 0.00773 | 0.00577 | 0.00731 | 0.00679 | 0.00626 | 0.00543 | 0.00465 | 0.00387 | 0.00309 |
| Total VOC | 1.34308 | 1.48308 | 2.01615 | 2.15385 | 1.74769 | 2.06231 | 1.96000 | 1.84923 | 1.66077 | 1.48154 | 1.29308 | 1.09077 |
| E10 |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00524 | 0.00582 | 0.00806 | 0.00867 | 0.00692 | 0.00828 | 0.00783 | 0.00736 | 0.00657 | 0.00582 | 0.00503 | 0.0042 |
| Cumene | - | - | - | - | - | - | - | - | - | - | - | - |
| Cyclohexane | - | - | - | - | - | - | - | - | - | - | - | - |
| Ethylbenzene | 0.00101 | 0.00115 | 0.00172 | 0.00189 | 0.00142 | 0.00179 | 0.00167 | 0.00154 | 0.00133 | 0.00115 | 0.00096 | 0.00076 |
| n-Hexane | 0.01724 | 0.01904 | 0.02589 | 0.02766 | 0.02244 | 0.02648 | 0.02516 | 0.02374 | 0.02133 | 0.01902 | 0.01661 | 0.01401 |
| Lead | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 |
| Toluene | 0.01134 | 0.01276 | 0.01837 | 0.01997 | 0.01547 | 0.01899 | 0.01782 | 0.0166 | 0.01461 | 0.01275 | 0.01083 | 0.00885 |
| Xylenes | 0.00419 | 0.00479 | 0.0072 | 0.00794 | 0.00593 | 0.00751 | 0.00698 | 0.00644 | 0.00558 | 0.00478 | 0.00398 | 0.00317 |
| Ethanol | 0.02992 | 0.03414 | 0.05122 | 0.05645 | 0.04223 | 0.05341 | 0.04963 | 0.0458 | 0.03972 | 0.03408 | 0.02841 | 0.02268 |
| Total VOC | 1.32615 | 1.46462 | 1.99154 | 2.12769 | 1.72615 | 2.03692 | 1.93538 | 1.82615 | 1.64077 | 1.46308 | 1.27769 | 1.07769 |

## Appendix F.5: Fuel storage- underground horizontal fixed roof tank

|  | Zone (kg/tonne) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
|  | South SA | South WA | Central WA | Northern Australia | Central Australia | Cape York | Central North Qld | Central Qld | South Qld | NSW | Vic | Tas |
| Crude oil |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00039 | 0.00043 | 0.00059 | 0.00065 | 0.00050 | 0.00063 | 0.00058 | 0.00054 | 0.00049 | 0.00043 | 0.00038 | 0.00032 |
| Cumene | 0.00001 | 0.00001 | 0.00001 | 0.00002 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 |
| Cyclohexane | 0.00344 | 0.00380 | 0.00509 | 0.00562 | 0.00438 | 0.00543 | 0.00506 | 0.00474 | 0.00426 | 0.00380 | 0.00330 | 0.00280 |
| Ethylbenzene | 0.00007 | 0.00007 | 0.00011 | 0.00012 | 0.00009 | 0.00012 | 0.00011 | 0.00010 | 0.00009 | 0.00007 | 0.00006 | 0.00005 |
| n-Hexane | 0.01473 | 0.01618 | 0.02146 | 0.02360 | 0.01854 | 0.02282 | 0.02133 | 0.02001 | 0.01806 | 0.01618 | 0.01415 | 0.01209 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00051 | 0.00057 | 0.00080 | 0.00089 | 0.00067 | 0.00086 | 0.00079 | 0.00073 | 0.00065 | 0.00057 | 0.00048 | 0.00040 |
| Xylenes | 0.00024 | 0.00027 | 0.00040 | 0.00046 | 0.00033 | 0.00044 | 0.00040 | 0.00036 | 0.00032 | 0.00027 | 0.00023 | 0.00018 |
| Total VOC | 0.01939 | 0.02133 | 0.02846 | 0.03135 | 0.02452 | 0.03030 | 0.02828 | 0.02649 | 0.02387 | 0.02134 | 0.01861 | 0.01586 |
| Fuel oil |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00011 | 0.00013 | 0.00017 | 0.00019 | 0.00015 | 0.00018 | 0.00017 | 0.00016 | 0.00014 | 0.00013 | 0.00011 | 0.00009 |
| Cumene | 0.00011 | 0.00012 | 0.00019 | 0.00021 | 0.00015 | 0.00020 | 0.00018 | 0.00017 | 0.00014 | 0.00012 | 0.00010 | 0.00008 |
| Cyclohexane | - | - | - | - | - | - | - | - | - | - | - | - |
| Ethylbenzene | 0.00001 | 0.00001 | 0.00002 | 0.00002 | 0.00001 | 0.00002 | 0.00002 | 0.00002 | 0.00001 | 0.00001 | 0.00001 | 0.00001 |
| n-Hexane | 0.00019 | 0.00020 | 0.00027 | 0.00030 | 0.00023 | 0.00029 | 0.00027 | 0.00025 | 0.00023 | 0.00020 | 0.00018 | 0.00015 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00010 | 0.00011 | 0.00015 | 0.00017 | 0.00013 | 0.00016 | 0.00015 | 0.00014 | 0.00012 | 0.00011 | 0.00009 | 0.00008 |
| Xylenes | 0.00009 | 0.00010 | 0.00015 | 0.00017 | 0.00012 | 0.00016 | 0.00015 | 0.00013 | 0.00012 | 0.00010 | 0.00008 | 0.00007 |
| Total VOC | 0.03333 | 0.03636 | 0.05758 | 0.06364 | 0.04545 | 0.06061 | 0.05455 | 0.05152 | 0.04242 | 0.03636 | 0.03030 | 0.02424 |
| Heating oil |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | - | - | - | - | - | - | - | - | - | - | - | - |
| Cumene | - | - | - | - | - | - | - | - | - | - | - | - |
| Cyclohexane | - | - | - | - | - | - | - | - | - | - | - | - |
| Ethylbenzene | 0.00004 | 0.00004 | 0.00006 | 0.00007 | 0.00005 | 0.00007 | 0.00006 | 0.00006 | 0.00005 | 0.00004 | 0.00003 | 0.00003 |
| n-Hexane | - | - | - | - | - | - | - | - | - | - | - | - |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00013 | 0.00014 | 0.00020 | 0.00022 | 0.00017 | 0.00022 | 0.00020 | 0.00018 | 0.00016 | 0.00014 | 0.00012 | 0.00010 |
| Xylenes | 0.00014 | 0.00016 | 0.00024 | 0.00027 | 0.00020 | 0.00026 | 0.00024 | 0.00022 | 0.00019 | 0.00016 | 0.00014 | 0.00011 |
| Total VOC | 0.15663 | 0.16867 | 0.24096 | 0.26506 | 0.20482 | 0.26506 | 0.24096 | 0.21687 | 0.19277 | 0.16867 | 0.14458 | 0.12048 |
| Kerosene |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00166 | 0.00183 | 0.00247 | 0.00274 | 0.00212 | 0.00264 | 0.00246 | 0.00230 | 0.00206 | 0.00183 | 0.00159 | 0.00134 |
| Cumene | 0.00056 | 0.00064 | 0.00097 | 0.00111 | 0.00078 | 0.00106 | 0.00096 | 0.00087 | 0.00075 | 0.00064 | 0.00053 | 0.00042 |
| Cyclohexane | 0.00562 | 0.00619 | 0.00831 | 0.00917 | 0.00714 | 0.00886 | 0.00826 | 0.00772 | 0.00694 | 0.00619 | 0.00539 | 0.00457 |
| Ethylbenzene | 0.00022 | 0.00025 | 0.00036 | 0.00041 | 0.00030 | 0.00039 | 0.00036 | 0.00033 | 0.00029 | 0.00025 | 0.00021 | 0.00017 |
| n-Hexane | 0.03436 | 0.03772 | 0.05005 | 0.05503 | 0.04324 | 0.05322 | 0.04974 | 0.04666 | 0.04212 | 0.03774 | 0.03300 | 0.02820 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00023 | 0.00026 | 0.00036 | 0.00040 | 0.00030 | 0.00039 | 0.00036 | 0.00033 | 0.00029 | 0.00026 | 0.00022 | 0.00018 |
| Xylenes | 0.00066 | 0.00075 | 0.00110 | 0.00125 | 0.00090 | 0.00119 | 0.00109 | 0.00100 | 0.00087 | 0.00075 | 0.00062 | 0.00051 |
| Total VOC | 0.20909 | 0.23636 | 0.32727 | 0.36364 | 0.27273 | 0.35455 | 0.32727 | 0.30000 | 0.26364 | 0.23636 | 0.20000 | 0.16364 |
| Avgas 100 |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00565 | 0.00624 | 0.00844 | 0.00934 | 0.00722 | 0.00901 | 0.00838 | 0.00783 | 0.00702 | 0.00625 | 0.00541 | 0.00458 |
| Cumene | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 |
| Cyclohexane | 0.00218 | 0.00240 | 0.00322 | 0.00356 | 0.00277 | 0.00343 | 0.00320 | 0.00299 | 0.00269 | 0.00240 | 0.00209 | 0.00177 |
| Ethylbenzene | 0.00007 | 0.00008 | 0.00012 | 0.00013 | 0.00010 | 0.00013 | 0.00012 | 0.00011 | 0.00009 | 0.00008 | 0.00007 | 0.00005 |
| n-Hexane | 0.00641 | 0.00703 | 0.00933 | 0.01026 | 0.00806 | 0.00992 | 0.00927 | 0.00870 | 0.00785 | 0.00704 | 0.00615 | 0.00526 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00261 | 0.00292 | 0.00410 | 0.00459 | 0.00344 | 0.00441 | 0.00407 | 0.00377 | 0.00334 | 0.00293 | 0.00249 | 0.00206 |
| Xylenes | 0.00032 | 0.00037 | 0.00054 | 0.00061 | 0.00044 | 0.00058 | 0.00053 | 0.00049 | 0.00042 | 0.00037 | 0.00031 | 0.00025 |
| Total VOC | 0.58273 | 0.63909 | 0.84818 | 0.93273 | 0.73273 | 0.90182 | 0.84273 | 0.79091 | 0.71364 | 0.64000 | 0.55909 | 0.47818 |
| Avgas LL |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.01684 | 0.01860 | 0.02513 | 0.02781 | 0.02151 | 0.02683 | 0.02497 | 0.02333 | 0.02092 | 0.01861 | 0.01613 | 0.01364 |
| Cumene | <0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | <0.00001 | <0.00001 |
| Cyclohexane | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 |
| Ethylbenzene | 0.00255 | 0.00289 | 0.00422 | 0.00479 | 0.00347 | 0.00458 | 0.00419 | 0.00384 | 0.00335 | 0.00289 | 0.00242 | 0.00196 |
| n-Hexane | 0.01739 | 0.01910 | 0.02534 | 0.02786 | 0.02189 | 0.02694 | 0.02519 | 0.02362 | 0.02133 | 0.01911 | 0.01671 | 0.01428 |
| Lead | <0.00001 | <0.00001 | 0.00001 | 0.00001 | <0.00001 | <0.00001 | 0.00001 | 0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 |
| Toluene | 0.01699 | 0.01900 | 0.02664 | 0.02984 | 0.02237 | 0.02867 | 0.02645 | 0.02450 | 0.02168 | 0.01901 | 0.01619 | 0.01341 |
| Xylenes | 0.00221 | 0.00251 | 0.00367 | 0.00417 | 0.00301 | 0.00398 | 0.00364 | 0.00334 | 0.00291 | 0.00251 | 0.00209 | 0.00169 |
| Total VOC | 0.58949 | 0.64746 | 0.85898 | 0.94441 | 0.74203 | 0.91322 | 0.85390 | 0.80068 | 0.72305 | 0.64780 | 0.56644 | 0.48407 |
| Diesel |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00017 | 0.00019 | 0.00025 | 0.00028 | 0.00021 | 0.00027 | 0.00025 | 0.00023 | 0.00021 | 0.00019 | 0.00016 | 0.00014 |
| Cumene | 0.00024 | 0.00028 | 0.00041 | 0.00047 | 0.00034 | 0.00045 | 0.00041 | 0.00037 | 0.00032 | 0.00028 | 0.00023 | 0.00018 |
| Cyclohexane | 0.00006 | 0.00006 | 0.00009 | 0.00009 | 0.00007 | 0.00009 | 0.00009 | 0.00008 | 0.00007 | 0.00006 | 0.00006 | 0.00005 |
| Ethylbenzene | 0.00006 | 0.00007 | 0.00010 | 0.00011 | 0.00008 | 0.00010 | 0.00009 | 0.00009 | 0.00008 | 0.00007 | 0.00005 | 0.00004 |
| n-Hexane | 0.00009 | 0.00010 | 0.00013 | 0.00015 | 0.00012 | 0.00014 | 0.00013 | 0.00012 | 0.00011 | 0.00010 | 0.00009 | 0.00008 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.00016 | 0.00018 | 0.00025 | 0.00028 | 0.00021 | 0.00027 | 0.00025 | 0.00023 | 0.00020 | 0.00018 | 0.00015 | 0.00013 |
| Xylenes | 0.00015 | 0.00017 | 0.00025 | 0.00028 | 0.00020 | 0.00027 | 0.00025 | 0.00023 | 0.00020 | 0.00017 | 0.00014 | 0.00012 |
| Total VOC | 0.01385 | 0.01538 | 0.02000 | 0.02308 | 0.01846 | 0.02154 | 0.02000 | 0.01846 | 0.01692 | 0.01538 | 0.01385 | 0.01231 |
| LP |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00283 | 0.00313 | 0.00423 | 0.00468 | 0.00362 | 0.00451 | 0.0042 | 0.00392 | 0.00352 | 0.00313 | 0.00271 | 0.00229 |
| Cumene | 0.00002 | 0.00002 | 0.00003 | 0.00003 | 0.00002 | 0.00003 | 0.00003 | 0.00002 | 0.00002 | 0.00002 | 0.00001 | 0.00001 |
| Cyclohexane | 0.00262 | 0.00288 | 0.00387 | 0.00427 | 0.00332 | 0.00413 | 0.00385 | 0.0036 | 0.00324 | 0.00289 | 0.00251 | 0.00213 |
| Ethylbenzene | 0.00047 | 0.00053 | 0.00077 | 0.00088 | 0.00064 | 0.00084 | 0.00077 | 0.0007 | 0.00061 | 0.00053 | 0.00044 | 0.00036 |
| n-Hexane | 0.01334 | 0.01465 | 0.01943 | 0.02137 | 0.01679 | 0.02067 | 0.01932 | 0.01812 | 0.01636 | 0.01466 | 0.01281 | 0.01095 |
| Lead | <0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | 0.00001 | <0.00001 | <0.00001 |
| Toluene | 0.00704 | 0.00787 | 0.01104 | 0.01236 | 0.00927 | 0.01188 | 0.01096 | 0.01015 | 0.00898 | 0.00788 | 0.00671 | 0.00556 |
| Xylenes | 0.00199 | 0.00226 | 0.00331 | 0.00376 | 0.00272 | 0.00359 | 0.00328 | 0.00301 | 0.00262 | 0.00226 | 0.00189 | 0.00152 |
| Total VOC | 1.02615 | 1.12692 | 1.49462 | 1.64385 | 1.29154 | 1.59000 | 1.48615 | 1.39385 | 1.25846 | 1.12769 | 0.98538 | 0.84231 |
| ULP |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00323 | 0.00357 | 0.00482 | 0.00534 | 0.00413 | 0.00515 | 0.00479 | 0.00448 | 0.00401 | 0.00357 | 0.0031 | 0.00262 |
| Cumene | 0.00002 | 0.00002 | 0.00003 | 0.00003 | 0.00002 | 0.00003 | 0.00003 | 0.00002 | 0.00002 | 0.00002 | 0.00001 | 0.00001 |
| Cyclohexane | 0.00275 | 0.00303 | 0.00406 | 0.00449 | 0.00349 | 0.00433 | 0.00404 | 0.00378 | 0.0034 | 0.00303 | 0.00263 | 0.00224 |
| Ethylbenzene | 0.0005 | 0.00056 | 0.00082 | 0.00093 | 0.00067 | 0.00089 | 0.00081 | 0.00075 | 0.00065 | 0.00056 | 0.00047 | 0.00038 |
| n-Hexane | 0.01037 | 0.01139 | 0.01511 | 0.01661 | 0.01306 | 0.01607 | 0.01502 | 0.01409 | 0.01272 | 0.01139 | 0.00996 | 0.00852 |
| Lead | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 |
| Toluene | 0.0055 | 0.00615 | 0.00862 | 0.00965 | 0.00724 | 0.00928 | 0.00856 | 0.00793 | 0.00701 | 0.00615 | 0.00524 | 0.00434 |
| Xylenes | 0.00209 | 0.00237 | 0.00346 | 0.00394 | 0.00284 | 0.00376 | 0.00344 | 0.00315 | 0.00275 | 0.00237 | 0.00197 | 0.0016 |
| Total VOC | 0.79769 | 0.87615 | 1.16231 | 1.27769 | 1.00462 | 1.23615 | 1.15538 | 1.08385 | 0.97846 | 0.87615 | 0.76615 | 0.65538 |
| PULP |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00347 | 0.00384 | 0.00519 | 0.00574 | 0.00444 | 0.00554 | 0.00515 | 0.00481 | 0.00432 | 0.00384 | 0.00333 | 0.00281 |
| Cumene | 0.00002 | 0.00002 | 0.00003 | 0.00004 | 0.00003 | 0.00003 | 0.00003 | 0.00003 | 0.00002 | 0.00002 | 0.00002 | 0.00001 |
| Cyclohexane | 0.00356 | 0.00392 | 0.00526 | 0.0058 | 0.00452 | 0.00561 | 0.00523 | 0.00489 | 0.0044 | 0.00392 | 0.00341 | 0.00289 |
| Ethylbenzene | 0.00057 | 0.00065 | 0.00094 | 0.00107 | 0.00077 | 0.00102 | 0.00093 | 0.00086 | 0.00075 | 0.00065 | 0.00054 | 0.00044 |
| n-Hexane | 0.00862 | 0.00946 | 0.01255 | 0.0138 | 0.01084 | 0.01335 | 0.01247 | 0.0117 | 0.01056 | 0.00946 | 0.00828 | 0.00707 |
| Lead | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 |
| Toluene | 0.00696 | 0.00778 | 0.01091 | 0.01222 | 0.00916 | 0.01174 | 0.01083 | 0.01003 | 0.00888 | 0.00779 | 0.00663 | 0.00549 |
| Xylenes | 0.00242 | 0.00274 | 0.00402 | 0.00456 | 0.0033 | 0.00436 | 0.00398 | 0.00365 | 0.00318 | 0.00274 | 0.00229 | 0.00185 |
| Total VOC | 0.66308 | 0.72769 | 0.96538 | 1.06154 | 0.83385 | 1.02692 | 0.95923 | 0.90000 | 0.81231 | 0.72769 | 0.63692 | 0.54385 |
| RON98 |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00349 | 0.00385 | 0.00521 | 0.00576 | 0.00446 | 0.00556 | 0.00517 | 0.00483 | 0.00433 | 0.00385 | 0.00334 | 0.00283 |
| Cumene | 0.00003 | 0.00003 | 0.00004 | 0.00005 | 0.00004 | 0.00005 | 0.00004 | 0.00004 | 0.00003 | 0.00003 | 0.00002 | 0.00002 |
| Cyclohexane | 0.00395 | 0.00435 | 0.00584 | 0.00645 | 0.00502 | 0.00623 | 0.00581 | 0.00543 | 0.00488 | 0.00436 | 0.00379 | 0.00322 |
| Ethylbenzene | 0.00058 | 0.00066 | 0.00096 | 0.00109 | 0.00079 | 0.00105 | 0.00096 | 0.00088 | 0.00077 | 0.00066 | 0.00055 | 0.00045 |
| n-Hexane | 0.01148 | 0.0126 | 0.01672 | 0.01838 | 0.01445 | 0.01778 | 0.01662 | 0.01559 | 0.01407 | 0.01261 | 0.01102 | 0.00942 |
| Lead | - | - | - | - | - | - | - | - | - | - | - | - |
| Toluene | 0.01928 | 0.02156 | 0.03023 | 0.03386 | 0.02538 | 0.03253 | 0.03001 | 0.0278 | 0.0246 | 0.02157 | 0.01837 | 0.01522 |
| Xylenes | 0.00262 | 0.00297 | 0.00435 | 0.00494 | 0.00357 | 0.00473 | 0.00432 | 0.00396 | 0.00345 | 0.00297 | 0.00248 | 0.00201 |
| Total VOC | 0.88308 | 0.96923 | 1.28615 | 1.41385 | 1.11154 | 1.36769 | 1.27846 | 1.19923 | 1.08231 | 0.97000 | 0.84769 | 0.72462 |
| E10 |  |  |  |  |  |  |  |  |  |  |  |  |
| Benzene | 0.00343 | 0.00379 | 0.00512 | 0.00566 | 0.00438 | 0.00546 | 0.00509 | 0.00475 | 0.00426 | 0.00379 | 0.00328 | 0.00278 |
| Cumene | - | - | - | - | - | - | - | - | - | - | - | - |
| Cyclohexane | - | - | - | - | - | - | - | - | - | - | - | - |
| Ethylbenzene | 0.00065 | 0.00073 | 0.00107 | 0.00121 | 0.00088 | 0.00116 | 0.00106 | 0.00097 | 0.00085 | 0.00073 | 0.00061 | 0.0005 |
| n-Hexane | 0.01134 | 0.01245 | 0.01651 | 0.01816 | 0.01427 | 0.01756 | 0.01641 | 0.0154 | 0.0139 | 0.01245 | 0.01089 | 0.00931 |
| Lead | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 | <0.00001 |
| Toluene | 0.00736 | 0.00823 | 0.01154 | 0.01292 | 0.00969 | 0.01242 | 0.01145 | 0.01061 | 0.00939 | 0.00823 | 0.00701 | 0.00581 |
| Xylenes | 0.00269 | 0.00305 | 0.00447 | 0.00508 | 0.00367 | 0.00486 | 0.00444 | 0.00407 | 0.00354 | 0.00306 | 0.00255 | 0.00206 |
| Ethanol | 0.01922 | 0.02178 | 0.03181 | 0.03612 | 0.02615 | 0.03454 | 0.03155 | 0.02896 | 0.02525 | 0.0218 | 0.01821 | 0.01476 |
| Total VOC | 0.87231 | 0.95769 | 1.27000 | 1.39692 | 1.09769 | 1.35077 | 1.26231 | 1.18462 | 1.06923 | 0.95769 | 0.83769 | 0.71615 |

## Appendix F.6: Organic liquid storage- floating roof tank

| Substance | CAS No. | Liquid density at 20 °C (kg/L) | Zone (kg/tonne) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
|  |  |  | South SA | South WA | Central WA | Northern Australia | Central Australia | Cape York | Central North Qld | Central Qld | South Qld | NSW | Vic | Tas |
| Acetaldehyde (ethanal) | 75-07-0 | 0.788 | - | - | - | - | - | - | - | - | - | - | 0.747 | 0.583 |
| Acetic acid | 64-19-7 | 1.048 | 0.160 | 0.161 | 0.163 | 0.164 | 0.162 | 0.163 | 0.163 | 0.162 | 0.161 | 0.161 | 0.160 | 0.159 |
| Acetone | 67-64-1 | 0.792 | 0.257 | 0.270 | 0.319 | 0.324 | 0.336 | 0.293 | 0.315 | 0.301 | 0.285 | 0.271 | 0.253 | 0.237 |
| Acetonitrile | 75-05-8 | 0.784 | 0.182 | 0.185 | 0.197 | 0.198 | 0.201 | 0.191 | 0.196 | 0.193 | 0.189 | 0.185 | 0.180 | 0.176 |
| Acrylamide | 79-06-1 | 1.119 | 0.155 | 0.155 | 0.155 | 0.155 | 0.155 | 0.155 | 0.155 | 0.155 | 0.155 | 0.155 | 0.155 | 0.155 |
| Acrylic acid | 79-10-7 | 1.059 | 0.157 | 0.157 | 0.158 | 0.158 | 0.158 | 0.157 | 0.158 | 0.158 | 0.157 | 0.157 | 0.157 | 0.156 |
| Acrylonitrile | 107-13-1 | 0.808 | 0.196 | 0.202 | 0.221 | 0.223 | 0.228 | 0.211 | 0.220 | 0.214 | 0.208 | 0.202 | 0.195 | 0.188 |
| Aniline | 62-53-3 | 1.021 | 0.070 | 0.072 | 0.079 | 0.080 | 0.082 | 0.076 | 0.079 | 0.077 | 0.075 | 0.072 | 0.070 | 0.068 |
| Benzene | 71-43-2 | 0.879 | 0.037 | 0.038 | 0.042 | 0.042 | 0.043 | 0.040 | 0.041 | 0.041 | 0.039 | 0.038 | 0.037 | 0.036 |
| Carbon disulfide | 75-15-0 | 1.262 | 0.304 | 0.323 | 0.395 | 0.401 | 0.420 | 0.357 | 0.388 | 0.368 | 0.345 | 0.324 | 0.298 | 0.276 |
| Chloroform | 67-66-3 | 1.489 | 0.246 | 0.258 | 0.301 | 0.305 | 0.316 | 0.279 | 0.298 | 0.286 | 0.272 | 0.259 | 0.243 | 0.228 |
| Cumene (isopropyl benzene) | 98-82-8 | 0.792 | 0.172 | 0.173 | 0.175 | 0.175 | 0.176 | 0.174 | 0.175 | 0.174 | 0.174 | 0.173 | 0.172 | 0.172 |
| Cyclohexane | 110-82-7 | 0.779 | 0.213 | 0.221 | 0.246 | 0.255 | 0.233 | 0.249 | 0.244 | 0.237 | 0.229 | 0.221 | 0.211 | 0.202 |
| Dibromoethane (1,2) | 106-93-4 | 1.255 | 0.281 | 0.281 | 0.283 | 0.283 | 0.282 | 0.282 | 0.282 | 0.282 | 0.282 | 0.282 | 0.281 | 0.280 |
| Dichloroethane (1,2) | 107-06-2 | 1.253 | 0.188 | 0.193 | 0.208 | 0.213 | 0.200 | 0.210 | 0.207 | 0.203 | 0.198 | 0.193 | 0.187 | 0.182 |
| Dichloromethane | 75-09-2 | 1.362 | 0.340 | 0.367 | 0.474 | 0.515 | 0.416 | 0.484 | 0.464 | 0.433 | 0.398 | 0.368 | 0.332 | 0.301 |
| 2-Ethoxyethanol | 110-80-5 | 0.930 | 0.157 | 0.157 | 0.158 | 0.159 | 0.158 | 0.158 | 0.158 | 0.158 | 0.158 | 0.157 | 0.157 | 0.156 |
| 2-Ethoxyethanol acetate | 111-15-9 | 0.975 | 0.156 | 0.157 | 0.157 | 0.157 | 0.157 | 0.157 | 0.157 | 0.157 | 0.157 | 0.157 | 0.156 | 0.156 |
| Ethyl acetate | 141-78-6 | 0.905 | 0.204 | 0.210 | 0.234 | 0.243 | 0.222 | 0.237 | 0.233 | 0.226 | 0.218 | 0.211 | 0.202 | 0.194 |
| Ethyl alcohol (ethanol) | 64-17-5 | 0.772 | 0.176 | 0.179 | 0.189 | 0.193 | 0.183 | 0.190 | 0.188 | 0.185 | 0.182 | 0.179 | 0.175 | 0.172 |
| Ethylbenzene | 100-41-4 | 0.864 | 0.161 | 0.162 | 0.165 | 0.167 | 0.164 | 0.166 | 0.165 | 0.164 | 0.163 | 0.162 | 0.161 | 0.160 |
| Ethyl butyl ketone | 106-35-4 | 0.818 | 0.156 | 0.157 | 0.157 | 0.157 | 0.157 | 0.157 | 0.157 | 0.157 | 0.157 | 0.157 | 0.156 | 0.156 |
| Formaldehyde (37%) | 50-00-0 | 1.090 | 0.159 | 0.159 | 0.161 | 0.162 | 0.160 | 0.161 | 0.161 | 0.160 | 0.160 | 0.159 | 0.158 | 0.158 |
| Glutaraldehyde (25%) | 111-03-8 | 1.106 | 0.165 | 0.166 | 0.170 | 0.171 | 0.168 | 0.170 | 0.170 | 0.169 | 0.167 | 0.166 | 0.165 | 0.164 |
| Hexane (-n) | 110-54-3 | 0.658 | 0.269 | 0.284 | 0.334 | 0.351 | 0.308 | 0.339 | 0.330 | 0.316 | 0.300 | 0.284 | 0.265 | 0.247 |
| 2-Methoxyethanol | 109-86-4 | 0.965 | 0.158 | 0.159 | 0.162 | 0.163 | 0.160 | 0.162 | 0.162 | 0.161 | 0.160 | 0.159 | 0.158 | 0.157 |
| 2-Methoxyethanol acetate | 110-49-6 | 0.975 | 0.157 | 0.157 | 0.161 | 0.165 | 0.159 | 0.163 | 0.161 | 0.160 | 0.158 | 0.157 | 0.156 | 0.156 |
| Methyl alcohol (methanol) | 67-56-1 | 0.792 | 0.182 | 0.186 | 0.200 | 0.202 | 0.206 | 0.193 | 0.199 | 0.195 | 0.190 | 0.186 | 0.181 | 0.176 |
| Methyl ethyl ketone | 78-93-3 | 0.806 | 0.198 | 0.204 | 0.225 | 0.227 | 0.232 | 0.214 | 0.223 | 0.218 | 0.211 | 0.204 | 0.197 | 0.190 |
| Methyl isobutyl ketone | 108-10-1 | 0.798 | 0.167 | 0.169 | 0.175 | 0.176 | 0.178 | 0.172 | 0.175 | 0.173 | 0.171 | 0.169 | 0.166 | 0.164 |
| Methyl methacrylate | 80-62-6 | 0.945 | 0.175 | 0.177 | 0.187 | 0.189 | 0.191 | 0.182 | 0.187 | 0.184 | 0.181 | 0.178 | 0.174 | 0.171 |
| Styrene | 100-42-5 | 0.926 | 0.155 | 0.156 | 0.158 | 0.158 | 0.157 | 0.158 | 0.158 | 0.157 | 0.156 | 0.155 | 0.155 | 0.154 |
| Tetrachloroethane (1,1,2,2) | 79-34-5 | 1.593 | 0.162 | 0.162 | 0.166 | 0.167 | 0.164 | 0.166 | 0.166 | 0.165 | 0.164 | 0.162 | 0.161 | 0.160 |
| Tetrachloroethylene | 127-18-4 | 1.621 | 0.164 | 0.166 | 0.171 | 0.172 | 0.168 | 0.171 | 0.170 | 0.169 | 0.167 | 0.165 | 0.164 | 0.162 |
| Toluene (methyl benzene) | 108-88-3 | 0.867 | 0.171 | 0.173 | 0.180 | 0.183 | 0.176 | 0.181 | 0.180 | 0.178 | 0.175 | 0.172 | 0.170 | 0.167 |
| Trichloroethane (1,1,2) | 79-00-5 | 1.334 | 0.166 | 0.168 | 0.173 | 0.175 | 0.170 | 0.174 | 0.173 | 0.171 | 0.169 | 0.167 | 0.166 | 0.164 |
| Trichloroethylene | 79-01-6 | 1.463 | 0.188 | 0.192 | 0.208 | 0.214 | 0.200 | 0.210 | 0.207 | 0.203 | 0.198 | 0.190 | 0.187 | 0.181 |
| Xylene (mixture of isomers) | 1330-20-7 | 0.860 | 0.161 | 0.162 | 0.165 | 0.165 | 0.163 | 0.165 | 0.164 | 0.164 | 0.163 | 0.162 | 0.161 | 0.160 |

## Appendix F.7: Organic liquid storage- vertical roof tank

| Substance | CAS No. | Liquid density at 20 °C (kg/L) | Zone (kg/tonne) |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | 1 | 2 | 3 | 4 | 5 | 6 | 7 | 8 | 9 | 10 | 11 | 12 |
|  |  |  | South SA | South WA | Central WA | Northern Australia | Central Australia | Cape York | Central North Qld | Central Qld | South Qld | NSW | Vic | Tas |
| Acetaldehyde (ethanal) | 75-07-0 | 0.788 | - | - | - | - | - | - | - | - | - | - | 38.024 | 19.487 |
| Acetic acid | 64-19-7 | 1.048 | 0.197 | 0.225 | 0.365 | 0.372 | 0.300 | 0.352 | 0.333 | 0.319 | 0.268 | 0.222 | 0.188 | 0.143 |
| Acetone | 67-64-1 | 0.792 | 4.582 | 5.124 | 7.912 | 7.793 | 6.697 | 7.321 | 7.119 | 6.923 | 5.929 | 5.076 | 4.424 | 3.523 |
| Acetonitrile | 75-05-8 | 0.784 | 1.212 | 1.354 | 2.059 | 2.045 | 1.751 | 1.938 | 1.873 | 1.823 | 1.567 | 1.340 | 1.167 | 0.926 |
| Acrylamide | 79-06-1 | 1.119 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 | 0.000 |
| Acrylic acid | 79-10-7 | 1.059 | 0.062 | 0.071 | 0.119 | 0.123 | 0.096 | 0.115 | 0.109 | 0.103 | 0.086 | 0.071 | 0.059 | 0.044 |
| Acrylonitrile | 107-13-1 | 0.808 | 1.903 | 2.129 | 3.250 | 3.224 | 2.761 | 3.052 | 2.952 | 2.872 | 2.465 | 2.107 | 1.833 | 1.453 |
| Aniline | 62-53-3 | 1.021 | 0.012 | 0.014 | 0.025 | 0.027 | 0.020 | 0.025 | 0.023 | 0.021 | 0.017 | 0.014 | 0.011 | 0.008 |
| Benzene | 71-43-2 | 0.879 | 2.126 | 2.374 | 3.600 | 3.573 | 3.067 | 3.388 | 3.277 | 3.191 | 2.745 | 2.350 | 2.048 | 1.627 |
| Carbon disulfide | 75-15-0 | 1.262 | 6.351 | 7.102 | 11.209 | 10.992 | 9.390 | 10.185 | 9.950 | 9.644 | 8.217 | 7.041 | 6.152 | 4.939 |
| Chloroform | 67-66-3 | 1.489 | 4.133 | 4.619 | 7.070 | 6.966 | 6.010 | 6.568 | 6.385 | 6.218 | 5.339 | 4.575 | 3.988 | 3.171 |
| Cumene (isopropyl benzene) | 98-82-8 | 0.792 | 0.158 | 0.182 | 0.303 | 0.313 | 0.246 | 0.294 | 0.277 | 0.264 | 0.219 | 0.180 | 0.150 | 0.113 |
| Cyclohexane | 110-82-7 | 0.779 | 2.669 | 2.973 | 4.471 | 4.428 | 3.823 | 4.203 | 4.072 | 3.971 | 3.426 | 2.943 | 2.574 | 2.054 |
| Dibromoethane (1,2) | 106-93-4 | 1.255 | 0.524 | 0.539 | 0.635 | 0.592 | 0.611 | 0.581 | 0.589 | 0.604 | 0.567 | 0.534 | 0.522 | 0.477 |
| Dichloroethane (1,2) | 107-06-2 | 1.253 | 1.530 | 1.713 | 2.620 | 2.609 | 2.221 | 2.473 | 2.385 | 2.319 | 1.988 | 1.695 | 1.471 | 1.163 |
| Dichloromethane | 75-09-2 | 1.362 | 8.194 | 9.319 | 16.094 | 16.100 | 12.864 | 14.503 | 14.012 | 13.332 | 11.032 | 9.240 | 7.905 | 6.209 |
| 2-Ethoxyethanol | 110-80-5 | 0.930 | 0.067 | 0.085 | 0.143 | 0.148 | 0.119 | 0.139 | 0.131 | 0.127 | 0.106 | 0.084 | 0.060 | 0.043 |
| 2-Ethoxyethanol acetate | 111-15-9 | 0.975 | 0.049 | 0.054 | 0.080 | 0.080 | 0.068 | 0.076 | 0.073 | 0.072 | 0.062 | 0.053 | 0.047 | 0.038 |
| Ethyl acetate | 141-78-6 | 0.905 | 2.289 | 2.571 | 3.980 | 3.967 | 3.357 | 3.751 | 3.615 | 3.508 | 2.995 | 2.545 | 2.199 | 1.728 |
| Ethyl alcohol (ethanol) | 64-17-5 | 0.772 | 0.816 | 0.936 | 1.552 | 1.578 | 1.264 | 1.481 | 1.405 | 1.345 | 1.120 | 0.926 | 0.776 | 0.587 |
| Ethylbenzene | 100-41-4 | 0.864 | 0.271 | 0.309 | 0.500 | 0.510 | 0.411 | 0.482 | 0.457 | 0.438 | 0.367 | 0.305 | 0.258 | 0.197 |
| Ethyl butyl ketone | 106-35-4 | 0.818 | 0.045 | 0.050 | 0.076 | 0.077 | 0.064 | 0.073 | 0.070 | 0.068 | 0.058 | 0.049 | 0.043 | 0.034 |
| Formaldehyde (37%) | 50-00-0 | 1.090 | 0.146 | 0.167 | 0.281 | 0.292 | 0.226 | 0.272 | 0.256 | 0.245 | 0.200 | 0.165 | 0.138 | 0.106 |
| Glutaraldehyde (25%) | 111-03-8 | 1.106 | 0.449 | 0.493 | 0.713 | 0.707 | 0.618 | 0.675 | 0.654 | 0.643 | 0.559 | 0.487 | 0.435 | 0.358 |
| Hexane (-n) | 110-54-3 | 0.658 | 5.172 | 5.754 | 8.652 | 8.531 | 7.408 | 8.078 | 7.852 | 7.664 | 6.618 | 5.698 | 4.995 | 4.002 |
| 2-Methoxyethanol | 109-86-4 | 0.965 | 0.136 | 0.164 | 0.328 | 0.363 | 0.242 | 0.330 | 0.297 | 0.275 | 0.210 | 0.162 | 0.125 | 0.088 |
| 2-Methoxyethanol acetate | 110-49-6 | 0.975 | 0.055 | 0.080 | 0.326 | 0.490 | 0.173 | 0.390 | 0.289 | 0.238 | 0.135 | 0.079 | 0.045 | 0.022 |
| Methyl alcohol (methanol) | 67-56-1 | 0.792 | 1.256 | 1.429 | 2.312 | 2.319 | 1.910 | 2.177 | 2.086 | 2.009 | 1.689 | 1.413 | 1.201 | 0.925 |
| Methyl ethyl ketone | 78-93-3 | 0.806 | 2.009 | 2.253 | 3.464 | 3.449 | 2.931 | 3.265 | 3.150 | 3.059 | 2.619 | 2.229 | 1.931 | 1.522 |
| Methyl isobutyl ketone | 108-10-1 | 0.798 | 0.545 | 0.622 | 1.008 | 1.025 | 0.828 | 0.968 | 0.919 | 0.882 | 0.740 | 0.615 | 0.519 | 0.395 |
| Methyl methacrylate | 80-62-6 | 0.945 | 0.907 | 1.024 | 1.613 | 1.626 | 1.345 | 1.538 | 1.470 | 1.419 | 1.203 | 1.013 | 0.868 | 0.676 |
| Styrene | 100-42-5 | 0.926 | 0.164 | 0.187 | 0.303 | 0.310 | 0.249 | 0.293 | 0.277 | 0.265 | 0.222 | 0.185 | 0.156 | 0.119 |
| Tetrachloroethane (1,1,2,2) | 79-34-5 | 1.593 | 0.283 | 0.323 | 0.520 | 0.530 | 0.428 | 0.501 | 0.475 | 0.456 | 0.383 | 0.319 | 0.270 | 0.206 |
| Tetrachloroethylene | 127-18-4 | 1.621 | 0.429 | 0.486 | 0.770 | 0.780 | 0.640 | 0.738 | 0.703 | 0.678 | 0.573 | 0.481 | 0.410 | 0.317 |
| Toluene (methyl benzene) | 108-88-3 | 0.867 | 0.711 | 0.801 | 1.250 | 1.258 | 1.047 | 1.192 | 1.141 | 1.104 | 0.938 | 0.792 | 0.680 | 0.531 |
| Trichloroethane (1,1,2) | 79-00-5 | 1.334 | 0.499 | 0.567 | 0.906 | 0.918 | 0.750 | 0.868 | 0.826 | 0.795 | 0.671 | 0.561 | 0.476 | 0.366 |
| Trichloroethylene | 79-01-6 | 1.463 | 1.516 | 1.702 | 2.639 | 2.636 | 2.225 | 2.496 | 2.402 | 2.330 | 1.989 | 1.687 | 1.454 | 1.139 |
| Xylene (mixture of isomers) | 1330-20-7 | 0.860 | 0.201 | 0.230 | 0.375 | 0.384 | 0.307 | 0.363 | 0.343 | 0.328 | 0.275 | 0.228 | 0.191 | 0.146 |

# Appendix G: TANKS User Manual