Knowledge report

Ecological and hydrogeological survey of the Great Artesian Basin springs - Springsure, Eulo, Bourke and Bogan River supergroups

Volume 1: history, ecology and hydrogeology

This report was commissioned by the Department of the Environment on the advice of the Interim Independent Expert Scientific Committee on Coal Seam Gas and Coal Mining. The review was prepared by UniQuest and revised by the Department of the Environment following peer review.

September 2014

Copyright

© Copyright Commonwealth of Australia, 2014.

Ecological and hydrogeological survey of the Great Artesian Basin springs - Springsure, Eulo, Bourke and Bogan River supergroups, Volume 1: history, ecology and hydrogeology, Knowledge report is licensed by the Commonwealth of Australia for use under a Creative Commons By Attribution 3.0 Australia licence with the exception of the Coat of Arms of the Commonwealth of Australia, the logo of the agency responsible for publishing the report, content supplied by third parties, and any images depicting people. For licence conditions see: 84TUhttp://creativecommons.org/licenses/by/3.0/au/U84T

This report should be attributed as ‘Commonwealth of Australia 2014, Ecological and hydrogeological survey of the Great Artesian Basin springs - Springsure, Eulo, Bourke and Bogan River supergroups. Volume 1: history, ecology and hydrogeology, Knowledge report, prepared by UniQuest for the Department of the Environment, Commonwealth of Australia’.

The Commonwealth of Australia has made all reasonable efforts to identify content supplied by third parties using the following format ‘© Copyright, [name of third party] ’.

Enquiries concerning reproduction and rights should be addressed to:

Department of the Environment, Public Affairs

GPO Box 787 Canberra ACT 2601

Or by email to:  public.affairs@environment.gov.au

This publication can be accessed at: www.iesc.environment.gov.au

Acknowledgements

This knowledge report was commissioned by the Department of the Environment on the advice of the Interim Independent Expert Scientific Committee on Coal Seam Gas and Coal Mining. The report was prepared by UniQuest with input from:

A/Prof Rod Fensham – School of Biological Sciences, University of Queensland

Prof Joan Esterle – School of Earth Sciences, University of Queensland

Dr Sue Vink – Centre for Water in the Minerals Industry, University of Queensland

Ms Alexandra Wolhuter – Centre for Water in the Minerals Industry, University of Queensland

Ms Jennifer Silcock – School of Biological Sciences, University of Queensland

Ms Kimberley Hines – School of Earth Sciences, University of Queensland

Mr Jeremy Drimer – QLD Department of Science, Information Technology, Innovation & the Arts

It was revised by the Department of the Environment following peer review by Dr MA (Rien) Habermehl (Consultant Hydrogeologist) and relevant Queensland, New South Wales and South Australian Government departments.

Disclaimer

The views and opinions expressed in this publication are those of the authors and do not necessarily reflect those of the Australian Government or the Minister for the Environment or the Interim Independent Expert Scientific Committee on Coal Seam Gas and Coal Mining or the statutory Independent Expert Scientific Committee on Coal Seam Gas and Large Coal Mining Development (IESC).

While reasonable efforts have been made to ensure that the contents of this publication are factually correct, the Commonwealth and IESC do not accept responsibility for the accuracy or completeness of the contents, and shall not be liable for any loss or damage that may be occasioned directly or indirectly through the use of, or reliance on, the contents of this publication.

Contents

Copyright	iii

Acknowledgements	iii

Disclaimer	iv

Contents	v

Summary	vii

Abbreviations	x

Glossary	xi

Part 1: Cultural history and ecological values of Great Artesian Basin springs in the Springsure, Eulo, Bourke and Bogan River supergroups	1

1	Introduction	2

1.1	Great Artesian Basin springs—background	2

1.2	Cultural history	4

1.3	Overview of ecological values	8

1.4	Outline of report	10

1.5	Methods	10

2	Springsure supergroup	13

2.1	Historical record	13

2.2	Project surveys and reassessment of spring status	16

2.3	Cultural history	19

2.4	Biological values	19

3	Eulo supergroup	22

3.1	Historical record	22

3.2	Project surveys and reassessment of spring status	25

3.3	Cultural history	29

3.4	Biological values	37

4	Bourke supergroup	47

4.1	Historical record	47

4.2	Project survey and reassessment of status of springs	48

4.3	Cultural history	55

4.4	Biological values	62

5	Bogan River supergroup	68

5.1	Historical record	68

5.2	Project survey and reassessment of spring status	68

5.3	Cultural history	71

5.4	Biological values	72

6	Conclusions and recommendations	74

6.1	Acknowledgements	76

Part 2—Hydrogeological survey of the Great Artesian Basin springs in the Springsure, Eulo, Bourke and Bogan River supergroups	77

7	Introduction to the springs of the Great Artesian Basin	78

7.1	Project aim and scope	81

7.2	Methodology	81

8	Hydrogeological overview of the Great Artesian Basin	84

8.1	Geological basins	85

8.2	Aquifers and aquitards of the Great Artesian Basin	86

9	Hydrochemistry of the Great Artesian Basin	89

9.1	Salinity	89

9.2	Total dissolved solids	90

9.3	Temperature and pH	90

9.4	Ion composition	91

9.5	Alkalinity	91

9.6	Environmental isotopes data	91

10	Springsure supergroup springs, Queensland	93

10.1	Regional geology	97

10.2	Regional aquifers and aquitards	97

10.3	Hydrogeological summary	98

11	Eulo supergroup springs, Queensland	100

11.1	Regional geology	100

11.2	Regional aquifers	103

11.3	Hydrogeological summary	104

12	Bourke supergroup springs, New South Wales	107

12.1	Regional geology	107

12.2	Regional aquifers	109

12.3	Hydrogeological summary	109

13	Bogan River supergroup springs, New South Wales	111

13.1	Regional geology	111

13.2	Regional aquifers	114

13.3	Hydrogeological summary	114

14	Risk assessment	116

15	Concluding remarks	124

16	References	126

Appendix A:	Metadata for Queensland Springs Database	133

Appendix B:	Example spring report, Wiggera Spring, Eulo supergroup	145

Appendix C:	Survey status; Springsure, Eulo and Bourke supergroups	150

Appendix D:	Nomination form and guidelines for changing the conservation status of a species under the Nature Conservation Act 1992 (Qld)	209

Appendix E:	Conceptual diagrams of Great Artesian Basin spring types	217

Summary

This project report describes the surveys of 848 springs in four Great Artesian Basin (GAB) supergroups: 252 in Springsure, 436 in Eulo, 145 in Bourke and 7 in Bogan River. The surveys included all of the likely Environment Protection and Biodiversity Conservation Act 1999 (Cwlth)– (EPBC Act–) listed springs. A further eight spring locations in the Eulo, Bourke and Bogan River supergroups, and 105 in the Springsure supergroup, have not yet been surveyed but are considered unlikely to be EPBC Act related.

The project team surveyed 503 springs in 94 spring complexes that had not previously been surveyed; and extended the current knowledge base of other previously surveyed springs.

This report is presented as two volumes:

Ecological and hydrogeological survey of the Great Artesian Basin springs - Springsure, Eulo, Bourke and Bogan River supergroups. Volume 1: history, ecology and hydrogeology, Knowledge report (this report), which is divided into two main sections:

Part 1: Cultural history and ecological values of Great Artesian Basin springs in the Springsure, Eulo, Bourke and Bogan River supergroups—provides information on the history and ecology of the spring supergroups.

Part 2: Hydrogeological survey of the Great Artesian Basin springs in the Springsure, Eulo, Bourke and Bogan River supergroups—provides information on the hydrogeology of the spring supergroups, including identification of source aquifers and an analysis of the potential impacts of coal seam gas development on the springs.

Ecological and hydrogeological survey of the Great Artesian Basin springs - Springsure, Eulo, Bourke and Bogan River supergroups. Volume 2: hydrogeological profiles, Knowledge report (CoA 2014), which includes a database of GAB springs and hydrogeological profiles for springs with both a high-conservation ranking and EPBC Act listing in the targeted supergroups.

Springs have been classified into discharge (EPBC Act–listed community), and recharge or watercourse (not EPBC Act–listed community) springs. Fifteen GAB spring-specific conceptual models were developed to describe the hydrogeological setting of springs and to clarify the definition of discharge springs that identifies EPBC-listed springs. Nine of these conceptual models describe EPBC Act–listed springs (Appendix E, types C to K).

All 92 EPBC Act–listed discharge springs in the Springsure supergroup have been surveyed. The unsurveyed springs are elevated in the landscape and unlikely to be affected by coal seam gas activities. Habitat for EPBC Act–listed species Eriocaulon carsonii is present in 23 springs in the Springsure supergroup, habitat for Arthraxon hispidus in 24 springs and habitat for Thelypteris confluens in two springs. Species listed under the Nature Conservation Act 1992 (Qld) (NCA Act) occur in 56 springs of the Springsure supergroup.

In the Eulo supergroup, there are 177 active EPBC Act–listed discharge springs with four springs at one location providing habitat for the EPBC Act–listed species Eriocaulon carsonii. A further two springs in the Eulo supergroup contain species listed under the NCA Act, and 22 springs contain species endemic to spring wetlands.

In the Bourke supergroup, there are 51 active EPBC Act–listed discharge springs with six springs at one location providing habitat for the EPBC Act–listed species Eriocaulon carsonii, which is also listed under the Threatened Species Conservation Act 1995 (NSW). No other springs in the Bourke supergroup support endemic species. There are no active springs in the Bogan River supergroup.

The importance of springs to Indigenous people for the past thousands of years is evident in the archaeological record, although stories relating to individual springs in the regions studied in this project have mostly not been told.

The Eulo and New South Wales springs were similarly vital to early pastoralists as the only reliable sources of water across large semi-arid expanses. However, as bores were drilled since the late 1800s and the extent of the GAB became known, springs became redundant. Many springs dried up as artesian aquifer pressures were drawn down by flowing artesian bores, and spring locations were eventually forgotten.

Due to the loss of springs, and the local impacts of excavation, piping, and feral and domestic herbivores, many populations of endemic species were lost, and it is likely that some spring-dependent species became extinct.

The ability to identify source aquifers for springs is dependent on the quantity and quality of data for nearby waterbores. The lack of bore screen data at the Eulo supergroup, and interpreted stratigraphy for New South Wales bores and springs, limited our ability to determine source aquifers for springs of the Eulo, Bourke and Bogan River supergroups with certainty.

Springs in the Springsure group are most likely to be impacted by current coal seam gas development. Of particular concern are the Lucky Last and Scotts Creek springs, with a predicted (modelled) artesian pressure drawdown of more than 0.2 metres, due to hydraulic connectivity between targeted coal seams and their likely source aquifer. These two discharge spring clusters (hence, EPBC Act–listed communities), provide habitat for the EPBC Act–listed species Eriocaulon carsonii and they have high-conservation value. Other springs that may be impacted, although with a predicted (modelled) drawdown of less than 0.2 metres, include Spring Rock Creek, Abyss and possibly Dawson River 8. It should be noted that the drawdown risk threshold of 0.2 metres—as set in s. 379 (3) of the Water Act 2000 (Qld), and used to assess and rank springs likely to be impacted by coal seam gas—is more a reflection of the predictive capability of the groundwater model than the level of drawdown that would have an impact on a spring’s ecological values.

Coal seam gas development and extraction is currently remote from active springs in the Eulo, Bourke and Bogan River supergroups, and is of less concern than large free-flowing artesian pastoral bores in proximity to springs that have not as yet been rehabilitated and regulated.

Springs supporting endemic species must be the focus of ongoing conservation efforts. Particularly urgent is the rehabilitation of remaining high-flow artesian bores that occur within 50 kilometres of high-value spring groups. Fencing, secure tenure agreements and strategic ongoing pest control will also be important.

Groundwater data collection and modelling capabilities need to be improved to more accurately predict the impact of coal seam gas extraction on aquifers that source springs in the Springsure supergroup.

The database presented as part of the project report and any updated version, need to be extended to include all springs throughout the Basin. This will assist the priority setting for conservation efforts and inform regulatory decision-making in the regions of coal seam gas and large coal mining activities, and allow consideration of their impacts on surface and groundwater resources.

This report should be read in conjunction with complementary work recently completed by the Queensland Water Commission (QWC 2012b, Underground water impact report for the Surat Cumulative Management Area) and by the National Water Commission (NWC 2013, Allocating water and maintaining springs in the Great Artesian Basin).

Further background information can be found in work completed by the Queensland Herbarium (2012 Ecological and botanical survey of springs in the Surat Cumulative Management Area) and by the Queensland Water Commission (QWC 2012a, Hydrogeological attributes associated with springs in the Surat Cumulative Management Area).

Abbreviations

| General abbreviations | Description |
| --- | --- |
| DERM | Queensland Government Department of Environment and Resource Management (ceased operations in 2012) |
| DNR | Queensland Department of Natural Resources |
| EPBC Act | Environment Protection and Biodiversity Conservation Act 1999 |
| GAB | Great Artesian Basin |
| GWDB | Groundwater Database |
| L/s | litres per second |

Glossary

| Term | Description |
| --- | --- |
| Alkalinity | The quantitative capacity of aqueous media to react with hydroxyl ions. The equivalent sum of the bases that are titratable with strong acid. Alkalinity is a capacity factor that represents the acid-neutralising capacity of an aqueous system. |
| Anthropogenic | Relating to, or resulting from, the influence of human beings on nature. |
| Anticline | In structural geology, an anticline is a fold that is convex up and has its oldest beds at its core. |
| Aquifer | Rock or sediment in formation, group of formations or part of a formation, that is saturated and sufficiently permeable to transmit quantities of water to wells and springs. |
| Aquifer connectivity | The degree to which groundwater can transfer between two adjacent aquifers or to the surface. |
| Aquifer discharge | Water leaving an aquifer. |
| Aquifer recharge | The amount of water replenishing an aquifer over a given time period. |
| Aquitard | A saturated geological unit that is less permeable than an aquifer and incapable of transmitting useful quantities of water. Aquitards often form a confining layer over an artesian aquifer. |
| Artesian | Pertaining to a confined aquifer in which the groundwater is under positive pressure (i.e. a bore screened into the aquifer will have its water level above ground). |
| Aquatic ecosystem | Any watery environment from small to large, from pond to ocean, in which plants and animals interact with the chemical and physical features of the environment. |
| Bore/borehole | A narrow, artificially constructed hole or cavity used to intercept, collect or store water from an aquifer, or to passively observe or collect groundwater information. Also known as a borehole, well or piezometer. |
| Casing | A tube used as a temporary or permanent lining for a bore. Surface casing: the pipe initially inserted into the top of the hole to prevent washouts and the erosion of softer materials during subsequent drilling. Surface casing is usually grouted in and composed of either steel, PVC-U or composite materials. Production casing: a continuous string of pipe casings that are inserted into or immediately above the chosen aquifer and back up to the surface through which water and/or gas are extracted/injected. |
| Coal seam | Sedimentary layers consisting primarily of coal. Coal seams store both groundwater and gas and generally contain saltier groundwater than aquifers that are used for drinking water or agriculture. |
| Coal seam gas | A form of natural gas (generally 95–97 per cent pure methane, CHR4R) typically extracted from permeable coal seams at depths of 300–1000 m. |
| Confined aquifer | An aquifer bounded above and below by confining units of distinctly lower permeability than that of the aquifer itself. Pressure in confined aquifers is generally greater than atmospheric pressure. |
| Cretaceous period | A period of geologic time, 145 million to 66 million years ago. |
| Depressurisation | The lowering of static groundwater levels through the partial extraction of available groundwater, usually by means of pumping from one or several groundwater bores. |
| Dewatering | The lowering of static groundwater levels through complete extraction of all readily available groundwater, usually by means of pumping from one or several groundwater bores. |
| Diffusion | Process whereby ionic or molecular constituents move under the influence of their kinetic activity in the direction of their concentration gradient. |
| Drawdown | The reduction in groundwater pressure caused by extraction of groundwater from a confined formation, or the lowering of the watertable in an unconfined aquifer. |
| Fault | A planar fracture or discontinuity in a volume of rock, across which there has been significant displacement along the fractures as a result of earth movement. |
| Fracture | The separation of an object or material into two or more pieces under the action of stress. |
| Geologic stratum | A layer of sedimentary rock or soil with internally consistent characteristics that distinguish it from other layers. The ‘stratum’ is the fundamental unit in a stratigraphic column and forms the basis of the study of stratigraphy. |
| Geological layer | A layer of a given sample. An example is Earth itself. The crust is made up of many different geological layers which are made up of many different minerals/substances. The layers contain important information about the history of the planet. |
| Groundwater | Water occurring naturally below ground level (whether in an aquifer or other low-permeability material), or water occurring at a place below ground that has been pumped, diverted or released to that place for storage. This does not include water held in underground tanks, pipes or other works. |
| Hydraulic conductivity | The rate at which a fluid passes through a permeable medium. |
| Hydraulic fracturing | Also known as ‘fracking’, ‘fraccing’ or ‘fracture simulation’, is the process by which hydrocarbon (oil and gas) bearing geological formations are ‘stimulated’ to enhance the flow of hydrocarbons and other fluids towards the well. The process involves the injection of fluids, gas, proppant and other additives under high pressure into a geological formation to create a network of small fractures radiating outwards from the well through which the gas, and any associated water, can flow. |
| Hydraulic gradient | The change in hydraulic head between different locations within or between aquifers or other formations, as indicated by bores constructed in those formations. |
| Hydraulic head | The potential energy contained within groundwater as a result of elevation and pressure. It is indicated by the level to which water will rise within a bore constructed at a particular location and depth. For an unconfined aquifer, it will be largely subject to the elevation of the watertable at that location. For a confined aquifer, it is a reflection of the pressure that the groundwater is subject to and will typically manifest in a bore as a water level above the top of the confined aquifer, and in some cases above ground level. |
| Hydraulic pressure | The total pressure that water exerts on the materials comprising the aquifer. Also known as pore pressure. |
| Hydrogeology | The area of geology that deals with the distribution and movement of groundwater in the soil and rocks of Earth's crust (commonly in aquifers). |
| Hydrology | The study of the movement, distribution and quality of water on Earth and other planets, including the hydrologic cycle, water resources and environmental watershed sustainability. |
| Jurassic period | The period of geologic time 201.3 million to 145 million years ago. |
| Lineaments | Linear surface expressions of subsurface fracture zones, faults and geological contacts. |
| Permeability | The measure of the ability of a rock, soil or sediment to yield or transmit a fluid. The magnitude of permeability depends largely on the porosity and the interconnectivity of pores and spaces in the ground. |
| Physicochemical parameters | Relating to both physical and chemical characteristics. |
| Pore-fluid pressure/pore pressure | See Hydraulic pressure. |
| Porosity | The proportion of the volume of rock consisting of pores, usually expressed as a percentage of the total rock or soil mass. |
| Potentiometric surface | An imaginary surface representing the static head of groundwater and defined by the level to which water will rise in a tightly cased well. |
| Production well | A well drilled to produce oil or gas. |
| Quaternary | The period of geologic time 2.5 million to zero million years ago. |
| Saturated zone | That part of Earth's crust beneath the regional watertable in which all voids, large and small, are filled with water under pressure greater than atmospheric. |
| Screen | The intake portion of a bore, which contains an open area to permit the inflow of groundwater at a particular depth interval, while preventing sediment from entering with the water. |
| Sediment | A naturally occurring material that is broken down by processes of weathering and erosion, and is subsequently transported by the action of wind, water or ice, and/or by the force of gravity acting on the particle itself. |
| Stratigraphy | A branch of geology that studies rock layers (strata) and layering (stratification). |
| Stratification | The formation of density layers (either temperature or salinity derived) in a water body through lack of mixing. It can create favourable conditions for algal blooms and lower dissolved oxygen levels in the bottom layers with the associated release of nutrients, metals and other substances. |
| Tertiary | A geologic period (from 66 million to 2.588 million years ago) that is no longer recognised as a formal unit by the International Commission on Stratigraphy, but is still widely used. |
| Tilt | The change in the slope of the ground as a result of differential subsidence. It is calculated as the change in subsidence between two points divided by the distance between those points. Tilt is usually expressed in units of millimetres per metre (mm/m), or as a ratio of rise to run (mm:mm). A tilt of 1 mm/m is equivalent to a change in grade of 0.1 per cent. |
| Triassic | The period of geologic time 248 million to 206 million years ago. |
| Unconfined aquifer | An aquifer that has the upper surface connected to the atmosphere. |
| Vadose zone | The ‘unsaturated’ zone, extending from the top of the ground surface to the watertable. In the vadose zone, the water in the soil’s pores is at atmospheric pressure. |
| Water quality | The physical, chemical and biological attributes of water that affect its ability to sustain environmental values. |
| Watertable | The upper surface of a body of groundwater occurring in an unconfined aquifer. At the watertable, pore water pressure equals atmospheric pressure. |
| Well | A human-made hole in the ground, generally created by drilling, to obtain water. See also Bore |
| Yield | The rate at which water (or other resources) can be extracted from a pumping well, typically measured in litres per second (L/s) or megalitres per day (ML/d). |

# Part 1: Cultural history and ecological values of Great Artesian Basin springs in the Springsure, Eulo, Bourke and Bogan River supergroups

Jen SilcockP1P, Owen PowellP1P, Jeremy DrimerP1P, Rod FenshamP1,2

P1 PSchool of Biological Sciences, University of Queensland

P2P Queensland Herbarium, Department of Science, Information Technology, Innovation & the Arts

# Introduction

‘If it could be wished that any superstition should remain among us, it is that which attached a peculiar sacredness to the pure spring.’

© Copyright, Anon. (1858, p. 352)

Springs occur where water flows naturally to the earth’s surface from underground. They take many forms, depending on underlying geology and hydrology. Some feed great rivers, while others create tiny desert pools. The maxim nullus enim fons mon sacer—‘there is in fact no spring that is not sacred’—has been attributed to the ancient Roman King Servius, and indeed it is likely that for every spring there is a custodian and a story.

This maxim reflects the universal sanctity of water as a primordial, life-giving element (Strang 2004). However, there is something unique—astounding, even—about clear pure water rising out of earth or rock. Groundwater is by definition invisible and mysterious; we can only experience it directly once it has left its realm (Schwalbaum 1997). Perhaps then it is not surprising that springs, the surface expression of this vast and hidden world, are revered as sources of mystery and magic, healing and renewal, wisdom and transcendence; and are beloved of poets, romantics and mystics (Chapelle 1997; Dowden 2000). At the same time, terrors and passages to the underworld can lurk beneath their calm surfaces (Bowman 1998), stirring an undercurrent of ambivalence in human relations to groundwater.

In ancient times, imagination was virtually all people had to draw on when trying to explain the source and behaviour of springs, resulting in the development of elaborate mythologies that remain embedded in our associations with groundwater today (Chapelle 1997). Springs assume a special aura in desert environments due to their necessity for life and the sharp contrast with surrounding ‘inhospitable wastes’. The recurring paradisal vision in the Bible is of a desert made fruitful, blooming into a garden oasis, through the blessing of divine waters (Smith 1912; Dickson 1987). The miraculous nature of desert springs is encapsulated in Isaiah’s prophesy:

‘Then shall the lame man leap as a deer, and the tongue of the dumb shall sing: for in the wilderness shall waters break out and stream into the desert. And the glowing land shall become a pool, and the thirsty ground springs of water.’

© Copyright, (35:6)

Australia is the driest vegetated continent and has the smallest area of land surface covered by wetland of any continent. Inland Australia has been described as the ‘dead heart’, a ‘wide brown land’ and the ‘vast central desert’ (Finlayson 1935; Madigan 1946). When people imagine the interior, it is often as a dry, harsh and dusty place—the ‘great grey plain’ of Henry Lawson, a land of ‘ever-receding horizons, lonely graves’ (Durack 1959) and ‘pitiless skies’ (Duncan-Kemp 1934). In the midst of this aridity, springs provide vital physical as well as psychological succour. Their tiny surface area relative to the immensity of the surrounding landscape belies their immense ecological, cultural, economic and social significance.

## Great Artesian Basin springs—background

Although there are springs throughout inland Australia, the largest clusters are those emanating from the Great Artesian Basin (GAB). Underlying one-fifth of mainland Australia, it comprises numerous aquifers. Springs occur where the potentiometric surface of the aquifer from which the water issues is above the ground surface (Habermehl 1982). Major regional clusters of spring complexes with some consistent hydrogeological characteristics form 12 ‘supergroups’ in the Basin (Figure 1).

Great Artesian Basin springs can be divided into two categories: recharge and discharge (Fensham & Fairfax 2003). Recharge springs occur within outcrop areas of sandstone formations on the eastern margins of the Basin, before the aquifer rocks dip underground. Water drains out of the rocks under gravity or through the intersection of the ground surface with a saturated aquifer, rather than welling upwards under artesian pressure. The water has a relatively short residence time in the aquifer, and tends to be neutral or slightly acidic with low levels of dissolved solids. Spring flows in recharge areas exhibit dynamism related to recent rainfall history.

Discharge springs emanate where the confining bed or aquitard is thin or weakened by faults or folds, and where aquifers abut against impervious basement rocks, mainly in relatively arid areas remote from the recharge zones. Spring flows are not related to recent rainfall, although the size of a spring wetland may fluctuate depending on seasonal conditions. Waters are generally alkaline with high levels of dissolved solids, reflecting the long residence time of the groundwater within the aquifer and because the groundwater gains salts during the upward movement from the artesian aquifer through the overlying, predominantly clayey aquitards. There are three broad types of discharge springs: peat, mud and water; the distinction between the two latter types is considered in detail in Chapter 3.

© Copyright, Habermehl 1982; Ponder 1986; Fensham & Fairfax 2003.

Figure 1 Great Artesian Basin, showing spring supergroups and recharge areas (shaded). The supergroups surveyed for this project are Springsure, Bogan River, Bourke and Eulo.

## Cultural history

### Aboriginal

Great Artesian Basin springs have been intimately known, used and cared for by Aboriginal people for millennia. The Aboriginal names of some springs have been preserved across the Basin—Pigeongah, Mommedah, Muk Quibunya, Bookera and Allawonga, to name a few of the more poetic—but the stories and significance of most individual springs have not been revealed in Queensland. Indigenous belief systems have parallels with spring mythologies worldwide, including the presence of supernatural beings and oracles, and the healing power of the waters. Rainbow Serpents of Aboriginal Australia remain an ongoing, living presence in some springs, keeping the water pure and punishing anyone who transgresses laws or rites (McDonald et al. 2005). In Western Australia, springs, soaks and wells are all viewed as being connected to the underlying groundwater, which is referred to as kurtany—literally, ‘mother’ (Yu 2002).

Dalhousie Springs in South Australia are collectively known as Irrwanyere, or ‘the healing spring’ (Ah Chee 2002), whereas an oracle dwelt in a purportedly bottomless spring near the Mulligan River in far western Queensland. This ‘big fellow masser’ was consulted in a peculiar manner, as recounted by surveyor Twisden Bedford:

‘ …one Aboriginal taking a big stone in each hand, dived head first into the bubbling water. A second Aboriginal jumped in immediately afterwards, and catching hold of his predecessor’s legs, which appeared above the surface of the water, forced him further down. A third Aboriginal then jumped in and forced the second down, all remaining under the water for as long a time as they could hold their breath in abeyance. They then all came to the surface, when the leading native gravely announced that he had interviewed the big fellow masser, and that big fellow flood come up along a one-fellow moon. And what is more, the flood did come in another month as predicted.’

© Copyright, Bedford (1886)

As well as being intrinsically sacred, springs were of vital importance in an arid land, as camp sites and nodes in continent-wide trade and communication routes (Boyd 1990; Harris 2002). Archaeological remains associated with one camp site at Dalhousie extend for almost three kilometres (Bayly 1999), whereas the scalded plain surrounding Youleen Springs in south-western Queensland has a rich and complex surface archaeology dating back at least 13 000 years (Robins 1998). In May 1876, explorer William Hodgkinson met a group of 30–40 people west of the Diamantina River near a group of permanent springs, possibly Pathungra Springs. The country was showing signs of drought and people were congregating around the few remaining permanent waters (Hodgkinson 1877). Stone tools are abundant across the claypans surrounding Pathungra Springs and other springs along the Hamilton River. Mysteriously, artefacts are in very low densities at other springs, including a large group in waterless country north of Aramac in central Queensland.

### European

Pastoral settlement of inland Australia was profoundly limited by the availability of water. Pioneering squatters concentrated their runs and grazing activities on permanent waterholes along major rivers and the sparse network of artesian springs. Beyond these tiny nodes, however, vast tracts of land remained ‘untrodden by hoof’ (Jack 1900). The GAB springs to the west and south of Lake Eyre played a vital role in ‘opening up’ the country (Harris 2002). Explorer John McDouall Stuart was the first to describe the extent and importance of the springs in 1859. He perceived them as important stepping stones to the interior and used them on his six expeditions. His eventual triumphant transcontinental crossing in 1861–62 was only successful because of these springs (Bailey 2006). The reliable water supply provided by the springs, coupled with favourable reports of grazing land, soon enticed pastoralists into the region. Later, two significant infrastructure developments, the Overland Telegraph Line (started in 1870, completed in 1872) and the Central Australian Railway (commenced in 1878, completed in 1929) followed the arc of mound springs from the south to north-west of Lake Eyre (Gibbs 2006).

Similarly, many roads and tracks in Queensland and New South Wales follow lines of springs. Although not as numerous or widespread as riverine waterholes (Silcock 2009), where springs do occur, they were considered a vital pastoral resource, especially during drought. One correspondent described a ‘fine spring’ in the vicinity of the Hamilton River: ‘one of many on the run that would ensure that in a drought stock would be watered’ (Nolan 2003). When taking stock from the Channel Country to the 61TGulf61T113T of Carpentaria113T, droving parties used Parker [Elizabeth] Springs as a vital stopover between the Diamantina and Georgina rivers, and a man was engaged for the specific duty of maintaining the groundwater supply and its delivery through a network of drains (Nolan 2003).

Water was not merely a vital resource; it was also a blessed relief from the inland heat and the silence, holding out ‘the sweet, cool promise that things could be otherwise’ (Cathcart 2009). Nevertheless, the utilitarian focus on springs almost completely nullified the sacred. Rather than ‘building altars and offering sacrifices’ as counselled by Roman philosopher Seneca, springs were drilled, drained and dug out in attempts to ‘improve’ them, although many were also fenced to stop stock becoming bogged. Although the honest yeomanry of the frontier were not by nature given to superstition, their interferences with springs did sometimes seem to awaken dark spirits of the earth. In a letter to the Sydney Morning Herald in 1882, JE Kelley provided a vivid description of his attempts at ‘improving’ a mud spring through drilling into a mound about 60 centimetres below its base:

‘All went well till the outside crust and sound ground were tunnelled through; but at the first dive of the bar into the “pudden” the whole mound groaned and surged like a large hole in agony, and the first thing I saw was my man floating out the mouth of the drive at the rate of 10 knots on a small sea of cold, white, watery lava. The whole hill continued to groan and shake as it appeared to gather vent, and after every groan it discharged ton upon ton of this white batter from the drive. A superstitious person would have believed that the thing was alive.’

© Copyright, Kelley (1882)

Springs also provided tantalising clues as to the existence of a vast supply of water below the parched inland, and played a wider role in the discovery, development and scientific understanding of the Basin. They were in effect tacit participants in their own demise. The 1884 New South Wales Royal Commission on water conservation collected information on springs to examine the potential extent of groundwater supplies (Gilliat 1885). Initially, shallow bores were sunk near the springs where success was assured—the first of these was in 1879 at Wee Wattah Spring north of the Darling River and others soon followed (Percy 1906). Although clean flowing water from these early bores could be conveniently directed into drains and troughs, this was only a modest improvement on springs, and failed to capture public imagination or provide adequate proof for groundwater theorists.

More adventurous drilling to greater depths and at further distances from the springs through the 1880s gradually revealed a vast underground basin stretching from Richmond in Queensland to Oodnadatta in South Australia and Moree in New South Wales. Water was obtained at Thurulgoona Station south of Cunnamulla in 1886 and this success was quickly mirrored in other areas (Powell 2012). This seemingly providential and boundless aquifer would come to be known as the Great Artesian Basin and delivered precious water from the bowels of the earth at enormous pressure.

Sinking of bores increased rapidly following severe stock losses in the Federation Drought (1898– 1902) and it was a condition of pastoral leases by 1918 that artesian bores must have been sunk. However, bore drilling remained an expensive and imprecise operation, and it was not until the 1950s that the combination of favourable seasons, high wool prices, improved technology and government subsidies gave pastoralists the opportunity to invest extensively in bores (Noble et al. 1998). By 1960, 18 000 (flowing artesian, and nonflowing artesian or subartesian) bores had been drilled across the Basin, resulting in a drastic loss of pressure in the aquifer. Total recorded discharge from free-flowing artesian bores in the Basin was more than 2000 megalitres per day from 1700 bores in 1916, but by 1998, less than 1000 megalitres per day were discharging from 3400 bores (GABCC 2000).

Spring flows across, and in the discharge areas of, the Basin declined similarly. Data indicate that some 40 per cent of discharge spring groups in Queensland have become completely inactive since pastoral settlement, whereas some springs within another 14 per cent of groups are inactive (Fairfax & Fensham 2002; Fensham et al. 2004). The loss of springs in Queensland, as a result of aquifer drawdown has been most severe in the Flinders River, Bourke, Springvale, Barcaldine and Eulo supergroups (Figure 1).

Though the springs continued to be used by pastoralists, the seemingly inexhaustible supplies obtained from flowing and nonflowing artesian bores meant that springs played an increasingly peripheral role. Bores were celebrated in both folklore and the annals of the colonial scientific literature. The vision of towering rigs (Figure 3) became the stuff of legends, as powerfully encapsulated in Banjo Paterson’s poem Song of the artesian water (Paterson 1896). The miracle of groundwater was easier to appreciate in the form of clear, gushing fountains as opposed to muddy, wobbling quagmires. Springs, once the lifeblood of early settlement, faded into obscurity, a mere footnote in the historical development of one of the world’s largest artesian basins.

© Copyright, State Library of New South Wales, item 03632.

Figure 2 Drilling rig located along the Bourke–Hungerford road, circa 1895.

89T© Copyright, no. 196121, John Oxley Library, Brisbane.

Figure 3 89TCharleville town bore, circa 1902. Caption reads that the bore depth is 1371 feet (418 m), yielding three million gallons (13 638 mP3P) per day. Temperature is recorded at 102 °F (38.9 °C) with water pressure at 95 pounds per square inch (655 002 Pa). Turned on at the bore head, the water reaches 120 feet (36.6 m).

In some respects, mirroring the global resurgence of spring pilgrimages, well dressings and health spas, Australia’s GAB springs have re-entered the public and scientific consciousness during the past three decades. Desert springs in South Australia, notably Blanche Cup, The Bubbler and Dalhousie Springs (Schmiechen 2004) have become popular tourist destinations, whereas elsewhere artesian spas (Moree, New South Wales), artesian mud baths and nature drives have been established. Traditional owners and pastoralists continue to tell stories about the springs, and there are even (unconfirmed) nymph sightings from time to time. Springs have also become the focus of biological surveys, which have revealed an astounding diversity of specialised and strange life forms.

## Overview of ecological values

Globally, wetlands tend to have low levels of endemism, and favour plants and animals that are mobile or readily dispersed (Junk 2006; Horwitz et al. 2009), because they are relatively ephemeral ecosystems. Wetlands that have persisted through glacial periods of aridity provide habitat for relictual species that were more widespread in wetter times (De Deckker 1986; Davis et al. 1993; Murphy et al. 2010). GAB discharge springs are distinctive wetlands that have (Fensham et al. 2011b):

permanent and relatively constant water levels

an unusual water chemistry

great antiquity

isolation from other wetlands.

These factors have combined to create a remarkable concentration of specialised endemics from this habitat, which have persisted despite the extinction of many springs and more recent local disturbances from excavation, invasive species and introduced herbivores. GAB discharge spring wetlands are listed as a nationally endangered ecological community under the Environment Protection and Biodiversity Conservation Act 1999 (Cwlth).

Thirteen vascular plant species are endemic to GAB discharge spring wetlands. Some are widespread across numerous supergroups (e.g. Eriocaulon carsonii, Myriophyllum artesium), and others are restricted to a single spring or spring complex (e.g. Eriocaulon giganteum and E. aloensis in the Pelican Creek complex) (Davies et al. 2007). As permanently wet habitats in an arid land, discharge springs also support extremely disjunct populations of some plant species, including Gahnia trifida and Baumea juncea in South Australia, and Cenchrus purparascens (formerly Pennisetum alopecuroides) in south-west Queensland (Fensham et al. 2010).

Sodic and salty groundwater scalds are often associated with discharge springs, and these areas have a specialised non-aquatic flora. Three species endemic to this habitat are highly restricted in the Barcaldine supergroup: Gunniopsis species (RJ Fensham 5094) is known from three populations totalling just 80 plants within a two-square-kilometre area on Edgbaston Reserve. Chloris species (Edgbaston RJ Fensham 5694) and Sphaeromorphaea major have small populations around the springs at Edgbaston and Doongmabulla. The near threatened Sporobolus partimpatens is known from groundwater scalds across five supergroups, and the Eulo and Barcaldine supergroups are each home to endemic, undescribed Calocephalus species.

Eight fish species are endemic to the GAB: five are recorded only from Dalhousie in South Australia (Glover 1989), two are restricted to a single spring complex in the Barcaldine supergroup and one to Elizabeth Springs in the Springvale supergroup (Wager 1995; Fairfax et al. 2007; Fensham et al. 2012). The radiation of molluscs is especially remarkable, with 38 species endemic to GAB discharge springs, including 26 that are restricted to a single spring group (Ponder et al. 1989; Ponder & Clark 1990; Ponder 1995; Worthington Wilmer et al. 2008; Fensham et al. 2010). Discharge springs are also centres for other specialised biota, including spiders, beetles, leeches, crustaceans and flatworms, many of which have very restricted distributions (Zeidler 1997; Fensham et al. 2010). Bacteria-like organisms occur in GAB aquifers (Kanso & Patel 2003); however, spring microorganisms remain extremely poorly characterised.

In contrast, the neutral or acidic water chemistry, and greater temporal fluctuations of GAB recharge springs seem to have limited the evolution of specialised endemics. These springs tend to emanate from sandstone gullies and are dominated by generalist wetland species from this habitat, including a variety of ferns and sedges (Fensham et al. 2011a). They do, however, provide habitat for very isolated populations of species otherwise restricted to much wetter climates, but some in the Springsure supergroup contain endemic species as discussed in Chapter 2.

## Outline of report

This project focused on 4 of the 12 GAB supergroups: Springsure, Eulo, Bourke and Bogan River in Queensland and New South Wales (Figure 1). The Springsure group is very different from the others, consisting mostly of recharge springs located outside the semi-arid zone. The other groups comprise mostly discharge springs occurring in areas receiving more than 500 millimetres of average annual rainfall, although the source aquifers of some of the New South Wales spring groups remains uncertain. For each supergroup, the historical record is reviewed and the results of our surveys presented. The cultural significance and natural values are discussed based on a review of the historical and ecological literature, and field survey results.

## Methods

For each supergroup, this project aimed to:

assess the location and status of all springs mentioned in historical sources

record the history and cultural values of springs

assess their contemporary biological values

determine any signs of recovery with bore capping and potential for habitat restoration.

Firstly, historical records of springs were systematically analysed. Several sets of historical maps were examined, including original pastoral run surveys, (1860–1880), historical map sheets (1880–1940) as well as more recent topographic maps (1964–2008). Historical literature relevant to the spring areas was reviewed, including explorer journals, newspaper articles, reports of government geologists and land/water inspectors, official government reports and books. People with long-term historical knowledge of these areas were interviewed where possible.

This historical analysis formed the basis of a comprehensive on-ground survey of springs in the Eulo, Bourke and Bogan River supergroups. A further survey of priority springs in the Springsure group was carried out, which added to information gathered through a recent Queensland Water Commission project (Fensham et al. 2012). Once a spring had been located in the historical record, we used a combination of computer-based mapping software (ArcGIS 10.1) and Google Earth satellite imagery to interpret the spring’s location in the landscape and obtain its approximate coordinates. In New South Wales, John Pickard’s report on New South Wales’ artesian springs (Pickard 1992) was invaluable for many springs, whereas the Eulo surveys built on the 1999–2000 surveys initiated by Rod Fensham and Russell Fairfax (Fairfax & Fensham 2003; Fensham & Fairfax 2003).

During the field survey, the combination of the assigned coordinates and the local knowledge of landholders was usually sufficient to identify the spring’s location or its possible remnants. Springs can often be detected through a series of visual cues, including:

soil dampness (indicating the spring is potentially active)

mounding

scalded terrain (due to carbonate and salt deposits)

sunken depressions.

Evidence of both Aboriginal (stone flakes and cores, hearths, grindstones) and European occupation (e.g. old fences or yards, ruins of huts, bottles, tin) in an otherwise waterless area may also indicate the presence of a now-inactive spring.

At all sites, the landscape position and surrounding vegetation were described and photos taken. Each vent location in a spring group was determined with a handheld global positioning system (GPS) unit and its activity status recorded. The elevation of the main vent/s was recorded using an OmniSTAR differential GPS. This gave an ellipsoidal height reading, which was converted to actual height (Australian Height Datum) using a geoid–ellipsoid separation (N value) calculated from the Geoscience Australia website (www.ga.gov.au/ausgeoid/nvalcomp.jsp).

For active springs, soak and wetland area (defined as more than 50 per cent cover of wetland vegetation), excavation damage (wells, pipes, bores, direct excavation), and impacts of stock and feral animals were recorded. Where a spring remained active only through excavation below the aquifer surface (e.g. Nulty, Kullyna and Toulby Springs), it was classified as inactive. All plant species present in the spring wetland were recorded, and the wetland was surveyed for fish, molluscs and other invertebrates. Unidentified and significant plant species were vouchered and these collections have been lodged at the Queensland Herbarium. For rare and threatened species (Silcock et al. 2011), detailed assessments of habitat, population size, extent and threats were made (Keith 2000). In springs with free water, pH, conductivity and temperature measurements were taken. If the spring wetland area was more than 10 metres in length, readings were taken at the vent and then every 10 metres to the edge of the wetland.

In addition to indicating the differences in water chemistry between springs, this also helped interpret whether the water was artesian or of run-on origin. Artesian water tends to be basic with high conductivity, while rainwater tends to be neutral or slightly acidic with lower conductivity. Nevertheless, the presence of run-on water from recent rainfall or flooding sometimes made it difficult to differentiate between active spring vents and inactive vents, or nonspring depressions such as gilgais and hollows. In most instances, landholder interviews provided sufficient information to make this distinction. Where this local knowledge was not available, assessments were made based on the field characteristics of the site. The problem of identifying permanent springs was particularly pronounced in the Springsure supergroup, where recent above-average rainfall years meant that ephemeral springs were abundant across the sandstone slopes visited during the survey.

All historical and ecological data collected for the four supergroups were entered into a Microsoft Access database (refer to metadata in Appendix A for full details). The database also incorporates the existing SPRLOC database compiled and maintained by the Queensland Herbarium, and new ecological and botanical data on springs in the Surat Cumulative Management area (Fensham et al. 2012). Separate water chemistry and vegetation species databases were also created, which are linked to the main database. Fauna collections were made, but are mostly retained as unidentified samples. Likely endemic mollusc species are identified. Other endemic fauna have been identified, but only where taxonomic advice has been available. Photos are contained in a linked photo library. A report was compiled for each spring group and these are located in an ‘Additional information’ library, along with available historical information and hydrogeological reports. An example of a spring report is provided in Appendix B. Springs that are considered to be non-GAB in origin remain in the database; however, their records can be filtered out using the field ‘GAB’ (true or false).

In the Access database each record (row) is referred to as a ‘spring’ and refers to an individual vent. In some cases, multiple vents feed an individual wetland, and this is captured by a separate ‘wetland’ field.

The field surveys conducted of the three southern supergroups were almost comprehensive and six additional priority springs in the Springsure supergroup were visited. The springs not visited were either inaccessible at the time of survey or were unable to be located with any degree of precision from historical records (refer to Appendix C). All databases and spring reports have been compiled and submitted with the present report. The historical record, cultural and biological values for each supergroup are discussed in Chapters 2–5.

The report refers to individual springs and these are not identified on the figures. The database needs to be used in conjunction with this report to identify individual localities.

# Springsure supergroup

The Springsure supergroup is the most easterly in the Great Artesian Basin (GAB) (Figure 1 in Chapter 1) and comprises mostly recharge springs along the Great Dividing, Expedition, Dawson and associated mountain ranges. More than 250 individual recharge vents have been recorded across 83 spring groups, and 10 groups encompassing 93 documented vents are classified as discharge springs. A section of the Springsure supergroup was the subject of a recent project managed through the Queensland Water Commission in the Surat Basin (Fensham et al. 2012). Given this recent work, and the fact that only a handful of priority springs were visited for this project, the cultural significance and biological values are only briefly summarised here.

## Historical record

More than 40 survey run plans and historical maps drafted between 1863 and 1936 were inspected. In contrast to the New South Wales and Eulo supergroups, most springs are not marked on any plans or maps, probably because their occurrence in a wetter climate with more surface water meant that they were not as critical to early pastoralists. Springs are marked on pastoral run plans L42217 (1889) and L42207 (1900) in the Taroom, Boggomoss and Mt Rose area, where there are numerous springs across a small area and none are referred to individually. Pastoral Run Plan L42217 (1889) depicts springs in the Salvator Rosa area (Figure 4). Top and Belinda Springs are named, whereas numerous unnamed springs are marked along Louisa Creek, and between Mt Flat Top and Fred’s Hill. Belinda Springs is also marked on the four-mile series 6d map (1921).

Figure 4 Pastoral Run Plan L42217 (1889) showing Belinda Springs and unnamed springs along Louisa Creek. This is one of the few run surveys to show springs in the Springsure supergroup.

Major Thomas Mitchell provided the first written record of the Springsure springs, compiled as he advanced north in search of a route to the Gulf of Carpentaria. On 27th June 1846, he found ‘ ... an extensive valley … bounded by the fine trap range of Hope’s Table Land … [which] contained springs, in various ravines along its sides’ (Mitchell 1847). Geo-referencing of Mitchell’s route suggests that Mitchell was referring to GAB recharge springs on present-day Babbiloora in the Carnarvon Ranges north-east of Augathella. Three days later, Mitchell recorded that Mr Stephenson found a ‘copious spring’ in a valley east of Mt Hopeless. Bevan Spring is eight kilometres to the east of Mt Hopeless, and Mitchell may have been referring to this or another spring on Farraday or Curnalong creeks, or their tributaries. Visits to springs in this area and comparison of features with Mitchell’s descriptions will be required to ascertain which springs his party visited.

However, a more momentous discovery was the line of springs along Louisa Creek in what is now Salvator Rosa National Park. Mitchell first encountered these springs on 2 July 1846 while scouting ahead of his party to secure both water and a suitable route forward:

‘Returning with two men and Yuranigh to the valley where I had been yesterday, I followed it downwards, and soon found that it widened very much, and contained dry ponds, with the traces of a deep current of water at some seasons…Further on, I perceived reeds in the hollow of the valley, and Yuranigh said there must be a spring, upon which he walked in amongst them, but still found the earth dry. The reeds at length covered an extensive flat, so green, that I sent Corporal Graham to examine that point. He emerged from the reeds with a face that, at a distance, made Douglas, my other man, say, “He has found water”. He had found a running stream, to which he had been guided by its own music, and taking a tin pot, he brought me some of it. The water was clear and sparkling, tasting strongly of sulphur, and Yuranigh said that this was the head of a river that never dried up. In this land of picturesque beauty and pastoral abundance, within eighty miles of the tropics, we had discovered the first running stream seen on this journey.’

© Copyright, Mitchell (1847).

What is now known as Major Mitchell’s Spring consists of an extensive peat deposit confined by the valley margins (Figure 5). There are numerous springs along Louisa Creek, which runs into the Nogoa River. Having discovered a secure water supply, Mitchell brought the expedition forward. By 5 July 1846, they encamped on the right bank of the ‘reedy rivulet’ near the ‘Pyramids’ where he established a depot marked XLIV (Figure 6); from here Mitchell explored north. Travelling down the bank of the stream on 6 July, Mitchell observed that it was:

‘Flowing, and full of sparkling water to the margin. The reeds had disappeared, and we could only account for the supply of such a current, in such a country, at such a season, by the support of many springs. We made sure of water now for the rest of our journey; and that we might say of the river “Labitur et labetur in omne volubilis aevum”.’

© Copyright, Mitchell (1847).

Mitchell’s lyrical optimism for this stream that seemed to ‘glide on … rolling forever’ was destroyed by his realisation that the Nogoa, and then the Belyando, were taking his party towards the east coast, rather than the Gulf of Carpentaria. He returned to his ‘swamp with the spring’ on 5 September 1846 before heading west in search of the north-westerly flowing stream of his dreams. Mitchell took four men on this western journey, while the remainder of the party stayed at a depot at the springs. The water from the spring-fed swamp ‘had a sulphureous taste, and nausea and weak-stomach were complained of by some of the men’ (Mitchell 1847).

Figure 5 Major Mitchell spring, a singular example of a recharge spring that forms a large bed of peat in a sandstone gully.

Figure 6 Mitchell’s route through the Salvator Rosa area, showing Pyramids Depot (Camp XLIV) at Major Mitchell’s Spring, south of Lake Salvator.

Few other historical references to the Springsure springs were found. There is a reference to a spring on Crystalbrook in the Chesterton Range north of Mitchell in the Queensland Government Mining Journal in 1918. Government geologist Lionel C Ball described the spring as approximately 100 metres in maximum diameter rising three metres above a black soil flat. If this is correct, the mound has shrunk considerably since then. In 1999, the mound was described as just 30 metres long and 0.4 metres high. According to Ball (1918), the substance of the mound was an extremely fine black mud and bright ‘sulphur yellow’ lichen was growing on the surface. When dry, the material could be easily ignited. Before taking over the property, the manager claimed that the mound had been on fire ‘during a period of eighteen months’. The daily flow of the spring did not exceed 90 800 litres per day. The water was described as being quite clear, odourless and free from gas bubbles, although it had a slightly ‘astringent’ taste and a faint small of sulphur dioxide could be detected downwind. Locally it was regarded as poisonous, highly corrosive and avoided by stock, though ‘birds were observed by us drinking it’. Jensen (1926) also noted the presence of ‘sulphur springs’ on Crystalbrook.

## Project surveys and reassessment of spring status

This project targeted springs within the Springsure supergroup that were identified as having potentially high biological values, low to medium elevation in the landscape, proximity to petroleum tenure, and relatively easy access and good return for effort. We visited an additional seven springs meeting these criteria in the Expedition Range and Blackdown Tableland areas between Taroom and Blackwater in September 2012.

In the Expedition Tableland, Turtle Creek spring on Coorada (Figure 7 a) and Bracklyn Spring on Stonecroft are permanent springs emanating from creek beds, the latter in a steep gorge near the head of Roundstone Creek. They run Turtle and Roundstone creeks, respectively, for several kilometres. An almost-permanent spring on Spring Creek was surveyed (Figure 7 b); however, it went dry during the drought in 2009. All other springs on Glen Elgin, including those documented along Stockyard, Peach and Black Gin creeks, also went dry in 2009. Many of the springs are dynamic and appear in different sections of the creeks in different seasons.

Three springs on Blackdown Tableland were visited: Rainbow Falls (Figure 7 c), Ballamoo Cliffs and Cleanskin Paddock (Figure 7 d). At the two former sites, preceding wet seasons meant that water was gushing from many places along the creek lines, rendering it impossible to locate the permanent springs. Cleanskin Paddock spring, near the south-eastern boundary of Blackdown Tableland National Park, has one of two inland populations of the endangered swamp orchid Phaius australis, as discussed in Section 2.4.

Most of the remaining unvisited springs in the Springsure supergroup are not regarded as high priorities for survey, because they are high in the landscape and are probably not discharging under artesian pressure (type A and B springs, as well as non-GAB types L–M in the hydrogeological report). Thus, they are unlikely to be affected by coal seam gas activities (Figure 8; also see supporting spreadsheet, Appendix C).

(b)

(a)

(d)

(c)

Figure 7 (a) Source of the permanent Turtle Creek spring on Coorada, (b) Spring Creek spring, the most permanent spring on Glen Elgin, (c) Jeremy Drimer and Carlin Burns near the source of Rainbow Falls; however, the amount of water flowing down these creeks in September 2012 made it difficult to locate the permanent springs, and (d) Cleanskin Paddock spring, a one-hectare swampy area dominated by ferns and sedges, and supporting a highly disjunct population of Phaius australis.

GAB = Great Artesian Basin.

Figure 8 Map showing visited and unvisited springs in Springsure supergroup. Most unvisited springs are high in the landscape and not likely to be affected by coal seam gas activities.

## Cultural history

The mountain ranges within which the Springsure springs lie have a long and rich Aboriginal history, as evidenced by the many habitation, burial and rock art sites (Mulvaney & Joyce 1965; Godwin 2001). Numerous rock art shelters, including on Carnarvon National Park, Mt Moffatt and Dooloogarah, occur near recharge springs, which would have provided vital water during dry times. During this project, rock art sites were documented on an overhang along Roundstone Creek downstream of the spring vent and above Rockland Spring (unvisited) on Blackdown Tableland.

Early pastoral settlement was not intimately tied to the Springsure springs, as it was in more arid areas. Nevertheless, most springs in more accessible areas were, and in many cases continue to be, used as stock water. Many have been excavated or piped, and yards have been built adjacent to some springs. A small number of springs, including one on Carnarvon Station, are known to have supplied Chinaman’s gardens in the early days of pastoral settlement.

## Biological values

Discharge springs of the Springsure supergroup contain populations of two spring endemics, Eriocaulon carsonii (recorded from 15 spring wetlands) and Myriophyllum artesium (recorded from 5 spring wetlands). An undescribed Dimeria species is recorded only from one recharge spring in Salvator Rosa National Park. The boggomoss snail (Adclarkia dawsonensis) is listed as Critically Endangered under the Environment Protection and Biodiversity Conservation Act 1999 (EPBC Act) and occurs adjacent to one ‘boggomoss’ spring, but is not an aquatic species (Stanisic pers. comm. 2011). The endemic hydrobiid snail Jardinella carnarvonensis is widespread in the springs and associated perennial streams within Carnarvon Gorge. In addition, 40 species are known to have isolated disjunct distributions at GAB springs (defined as being more than 250 kilometres from the nearest nonspring record) (Fensham et al. 2012).

Disjunct populations of the endangered swamp orchid Phaius australis occur at springs in Carnarvon Gorge and Blackdown Tableland National Park, while three other listed species are recorded from GAB recharge springs (Table 1). The Phaius population surveyed at Blackdown for this project covered an area of 100 metres × 50 metres on a spongy seep adjacent to a running creek in a steep valley. It occurred patchily amongst ferns and sedges with scattered shrubs and trees, including Schoenoplectus mucronatus, Baumea rubiginosa, Cyperus lucidus, Lygodium microphyllum, Blechnum indicum, Pteridium esculentum, Melastoma malabathricum subsp. malabathricum, Sacciolepis indica and Viola betonicifolia subsp. betonicifolia. Total population size was estimated to be at least 500 plants. All were healthy and about to burst into flower in September 2012 (Figure 9). No other endemic, disjunct or listed species were recorded at springs surveyed for this project.

Table 1 Spring endemic and listed species in the Springsure supergroup, under the Environment Protection and Biodiversity Conservation Act 1999 (Cwlth) and the Nature Conservation Act 1992 (Qld).

| Species | R/D | Spring endemic? | EPBC Act | NCA | Notes on occurrence in supergroup |
| --- | --- | --- | --- | --- | --- |
| Plants |  |  |  |  |  |
| Eriocaulon carsonii | D | Y | E | E | 15 spring wetlands |
| Myriophyllum artesium | D | Y | – | E | 5 spring wetlands |
| Phaius australis | R | N | E | E | 1 spring wetland |
| Arthraxon hispidus | R | N | V | V | 17 spring wetlands |
| Thelypteris confluens | R | N | – | V | 2 spring wetlands |
| Livistona nitida | R | N | – | NT | 7 spring wetlands |
| Dimeria sp. (Salvator Rosa RJ Fensham 3643) | R | Y | – | – | 1 spring wetland |
| Molluscs |  |  |  |  |  |
| Jardinella carnarvonensis | R | N | – | – | Widespread in Carnarvon Gorge |

– = not listed, D = discharge, E = endangered, EPBC Act = Environment Protection and Biodiversity Conservation Act 1999 (Cwlth), N = no, NCA = Nature Conservation Act 1992 (Qld), NT = near threatened, R = recharge, V = vulnerable, Y = yes.

Figure 9 Phaius australis at Cleanskin Paddock spring, Blackdown Tableland, September 2012.

The impact of coal seam gas extraction on springs is reviewed in the hydrogeological section of this report. Using modelling, there are six spring complexes that are predicted to suffer a greater than 0.2-metre drawdown with cumulative prospective development. Of these, individual wetlands in the Lucky Last (Figure 10a) and the Scotts Creek complex (Figure 10b) have high biological values (

Table 2). They are also discharge springs and are thus protected under the EPBC Act. The other spring complexes predicted to be affected do not have substantial biological values, mainly because they occur in watercourses that are regularly scoured by floodwaters.

(b)

(a)

Figure 10 Discharge springs in the Springsure supergroup with high biological values predicted to suffer drawdown due to coal seam gas activities: (a) Lucky Last and (b) Scotts Creek.

Table 2 Biological values of spring complexes predicted to suffer a more than 0.2-metre drawdown with cumulative prospective coal seam gas developments.

| Spring complex | Listed species | Disjunct populationsPa |
| --- | --- | --- |
| Lucky Last | Eriocaulon carsonii (6) | Cenchrus purpurascens (3), Isachne globosa (6) |
| Scott’s Creek | Eriocaulon carsonii (4) | Cenchrus purpurascens (2), Cyperus laevigatus (4), Eriocaulon carsonii, Schoenus falcatus (1) |
| 311 | None | Ampelopteris prolifera (1), Isachne globosa (2) |
| Barton | None | None |
| Spring Rock Creek | None | None |
| Yebna Boggomoss | None | None |

a Number of populations of listed and disjunct populations (> 500 km from the nearest known population) in each complex is shown in brackets.

# Eulo supergroup

Springs comprising the Eulo supergroup occur across a 150 kilometre × 100 kilometre area roughly bounded by the towns of Eulo, Hungerford and Thargomindah in south-western Queensland. The Eulo and Bourke supergroups lie within the Great Artesian Basin (GAB) Warrego management zone, in which average pressure has decreased by 39 metres (0–120 metres)—the greatest loss of pressure in the GAB. The Eulo springs as a group currently discharge about one-third of their early 20th century output (Fairfax & Fensham 2003). The springs were first surveyed by bore inspectors Ogilvie and Edwards in 1911–12. This historical record was examined by Fairfax & Fensham (2003), and Fensham et al. (2004) surveyed the active springs. Four spring groups are located south of Hungerford in New South Wales and were surveyed by Pickard (1992).

## Historical record

Half of the 111 spring groups documented in the Eulo supergroup are marked on the four-mile series drafted between 1916 and 1931. Only 25 are marked on survey plans drawn between 1870 and 1964. Some large spring groups are not marked on any maps or plans, including the Dead Sea (although the block is called ‘The Springs’) and Tunga. Nine springs, including eight not marked on other maps, are marked on 1:250 000 geology maps (Table 3). A map dated circa 1900 of Tinneburra Resumed Area drawn by FH Biden shows Garrawin and Pitherty mud springs, while a railway plan from 1886 depicts Bingara Old Station and Dewalla Springs.

Table 3 Springs marked on historical maps and plans in the Eulo supergroup.

| Map/Plan inspected | Springs marked |
| --- | --- |
| Original survey of runs W51.59 (c. 1870) | Carpet Springs, Roses Paddock |
| Original survey of runs W51.40 (c. 1870) | Bingara Springs, Dead Sea springs (as ‘The Springs’ block), Wiggera, Wirrarah |
| Perambulator survey of runs on Bundilla, Acklands, Bingara and Dewalla Creeks W.51.41 (1876) | Bingara, Dewalla, Granite, Tunca, Wirrarah, Wiggera, Wombula |
| Warrego District Plan showing survey of runs between Lower Paroo and Warrego rivers to 29th Parallel W51.52 (1880) | None |
| Perambulator survey of runs on Lower Paroo and tributaries W.51.51 (c. 1880) | Curracunya Springs |
| Original survey of runs W51.62 (1886) | Baroona Springs |
| Original survey plan B144.198, B144.155 (c. 1890) | Dewalla |
| Survey plan NL.5 (c. 1900) | Ningaling, Thorlindah Outstation |
| Survey plan NL.8 (1913) | Jubilee (Corina) Springs, Muntha Napper (Bull) |
| Railway survey plan, Cunnamulla–Thargomindah (1896) | Dewalla and Bingara Old Station |
| Tinnenburra Resumed Area (FH Biden, c. 1900) | Garrawin mud spring, Pitherty well and mud spring |
| Pastoral run of Brindingabba (1886) | Jacombe, Picnic Sandhill, Tynghynia, |
| County of Irrara (1964) | Tynghynia |
| NSW pastoral holding index map (1888) | Warroo |
| 4-mile maps: Series 1 sheets 2D (1928), 3A (1916), 3C (1931); Series 2 sheets 6 (1964), 7 (1959), 15 (1961), 16 (1940) | Barb Tank, Baroona, Basin, Bingara Bore, Bingara Old Station (and mounds), Bush, Bokeen, Boorara Woolshed, Bullenbilla, Burtanya, Dewalla, Cainawarra Bore, Carpet, Curracunyah, Goonerah, Gooringara, Granite, Fish, Jubilee, Kopanyu, Little Kopanyu/Kapingee, Masseys (Horseshoe), Minyeburra, Muntha Napper, Myrton Bore, Nowanee, Ooliman, Oonunga, Parally Mill and Spring, PetherPaP, RileyPaP, Taleroo, Tarko (Fish), Thorlindah Outstation, Tilberry Outstation, Tunca, Tunkata, Twomanee, Umatcha, Waihora, Wanco, Wandilla, Wiggera, Wirrarah, Wombula, WooregymPaP, Youleen, Yowah Creek mud springs and hot spring |
| Geology maps | Boundary, Pitherty Road, Police Paddock, Farnham North, Farnham South, Town Common C, Springvale mounds, Tunkata West, Wombula |

c. = circa. NSW = New South Wales.
a Pether and Riley springs both marked on series 2, but not series 1 maps; Wooregym marked on series 1 but not series 2.

Between October 1911 and April 1912, the Queensland Government’s bore inspectors Ogilvie and Edwards inspected artesian water supplies in the Eulo area, recording about 60 spring groups. These records are contained in unpublished artesian reports, volumes VIII, IX and VI, held by the Queensland State Archives (reference numbers RSI 3037-1-8, 9 and 16). Ten were deemed unimportant and not visited, and it can be assumed that their flow rates were negligible. Oligive and Edwards’ descriptions of six groups of dry mounds suggest that they may have been mostly inactive before the first artesian bores being sunk in the district in the 1880s. They described most of the springs visited, sometimes including vegetation and wetland area, and measured flow rates for 22 groups. As well as these descriptions and some maps of individual spring groups (see spring reports), Ogilvie gave the following overview:

‘The important springs are almost all in the valley of Boorara Creek, hemmed in on either side by sandstone ridges about 250 ft high, which seems to support the view that the water is of local origin (as the lower beds of this series are very porous) … On the other hand, the temperature of the springs (where the actual outlet is accessible) varies between 79 ° and 105 °F, which is higher than one would expect from local supplies. There is also generally a small alkaline deposit in the vicinity of springs similar in appearance to that found on the bore drains of the district … At Youleen one of the springs possesses a very slight odour of sulphur. It and the taste of the springs of this district resemble that of the local bore water. In most of the springs the evaporative effect is very marked, and especially at the main Horseshoe Spring … ’

© Copyright, Ogilvie (1912).

Wiggera and Youleen Springs had the largest flows in the district, and were considered the principal springs. From the descriptions of Ogilvie and Edwards, a further 19 groups had considerable flows and often supported large wetlands, while 25 were visited but seem to have been small and were regarded as ‘unimportant’ (Table 4).

Table 4 Springs in the Eulo group visited or mentioned, but not visited by Ogilvie and Edwards, in 1911–12.

| Important | Unimportant |  |
| --- | --- | --- |
|  | Visited | Unvisited |
| Baroona | Springs 13 kilometres north of Currawinya (Basin Bore) | BoomerangPa |
| Bingara Old Station | Barb Tank | BurtanyaPa |
| Bullenbilla | Bingara Bore | Springs 8 kilometres north-west of Colanya (Corina house springs) |
| Bush | Bingara Old Station mounds | North-west corner of The Springs block (Dead Sea group) |
| Corina (Jubilee) | Bokeen | GourminyaPa |
| Dewalla | Boorara Woolshed | Kapingee |
| Gooning | Boundary | Nowanee |
| Granite | Cainawarro Bore | Between Tunca and Boundary |
| Horseshoe (Masseys) | Caiwarro mud springs (Bodalla and Mooning mud springs) | Taleroo Bore |
| Kopanyu | Colanya Bore | Wonko (Wanco) |
| Ooliman | Curracunya |  |
| Oonunga | Currawinya Station |  |
| Tarko (Fish) | Fish |  |
| Tilberry | Goonerah |  |
| Tunga | Kungie |  |
| Tunkata | Little Granite |  |
| Twomanee | McNichol’s (Apex mud springs) |  |
| Wiggera | Minyeburra |  |
| Wirrarah | Myrton Bore |  |
| Wooregym | Pitherty mud springs |  |
| Youleen | Tareen Outstation |  |
|  | Thorlindah Oustation |  |
|  | Tunca |  |
|  | Tunkata South |  |
|  | Umatcha |  |

a Not located on the ground in 2012.
Note: Where Oligive and Edwards referred to sites by a different name, the current name is given in brackets.

## Project surveys and reassessment of spring status

From previous surveys (Fairfax & Fensham 2003; Fensham et al. 2004), historical sources, geology maps and local knowledge, 111 spring groups (defined as vents situated more than one kilometre apart or in a different landscape setting) were compiled. A total of 105 springs (encompassing 436 separate vents) were surveyed between October and December 2012. This included 43 spring groups that had not been visited in previous surveys. All springs emanate from the Basin with the probable exception of Kungie Lake spring. Situated approximately 100 metres west of Kungie No. 1 bore, Ogilvie described it as being marked by a small boggy area in 1912. A small rocky area on the dry lake bed may be the old spring (Andrew Campbell 2012, pers. comm.; Figure 11a), but the area does not have the appearance of a GAB spring and is not considered in this report.

Currawinya Station Spring is somewhat ambiguous, because it does not have scalded ground around it and now lies beneath a bore-fed pool (Figure 11b). However, there are other small springs in similar landscape settings (e.g. Parally mill and the spring on Boorara), and Edwards’ description (23/2/1912) is consistent with an artesian source:

‘Situated at Currawinya Station and 2 ½ chains S of the creek. A well 4 feet × 4 feet has been sunk to a depth of 12 feet and timbered. The water is of excellent quality and cold. The spring just overflows at the surface keeping a hole (10 gallon capacity) filled. The water is used for domestic purposes at the station and the supply is reckoned at 600 gpd  …. As the supply kept up throughout the drought and the well water rises above this level in the creek there appears to be a source of supply other than soakage.’

© Copyright, Edwards (2012).

(b)

(a)

Figure 11 (a) Site of Kungie Lake Spring, west of Tinnenburra No. 1 bore, but long inactive and probably not emanating from the Great Artesian Basin, and (b) Currawinya Station Spring, ambiguous but assumed to be a small, now-inactive Basin spring.

Springs in the Eulo supergroup are loosely clustered in three lines aligned south or south-west, and a south-eastern line running into New South Wales, and can be classified as either mud or water springs (Figure 12). Mud springs form where the groundwater has mixed with fine sediment to form a slurry emanating from the spring vent. The slurry may not flow to the surface for long periods and dry surface layers can form a ‘skin’ across the soupy interior material. In contrast, water springs have clear water and their permanent soak areas often support wetland vegetation. The water emanates from a discrete vent or vents, and saturates the surface across an extent determined by flow and evaporation. Within the vegetation, there may be pools of free water formed where the vegetation or sediment has created an obstruction to flow, forming a dam. All springs east of the Paroo River from Farnham Plains to Pitherty are mud springs. Mud and water springs are interspersed throughout the central and western lines, which run from the Dead Sea to Karto and Bingara to Wombula, respectively. Two isolated groups of inactive water springs occur west of the Warrego River on Baroona, which are here considered eastern outliers of the Eulo supergroup. At least some springs in 63 groups remain active.

Figure 12 Eulo supergroup, showing active and inactive mud and water springs. ‘Both’ means that active and inactive springs occur together.

Thirty-six spring groups are comprised solely of mud springs, and 75% of these contain at least some active springs (Table 5). Most mud springs are mounded and can occur as single isolated mounds, in small clusters or cover large areas up to 10-square kilometres containing hundreds of vents (Figure 13 a–c). The largest mound spring areas occur in a line from Currawinya to Boorara west of Hoods Range, and at Jubilee, Corina and Tunkata springs near the Boorara–Corina springs boundary. Many mounds were probably inactive before European settlement, based on the historical record, local knowledge and the consolidated, pebbly nature of the mounds, some with large trees growing out of them. Of the nine completely inactive groups, only three—Waihora, Garrawin and Tunga mound—show signs of recent activity and may have ceased flowing due to aquifer drawdown. Five mud spring groups comprise flat, unvegetated muddy slurries or puddles; all of these remain active, although the flow of Bokeen Spring has probably declined since Edwards visited in 1912. Seven groups contain a mixture of active and inactive depressions and mounds (Table 5).

Whether flat or mounded, mud springs seem to be inherently dynamic. The Wandilla Springs on the Paroo floodplain can fill with water overnight without any rainfall and dry back in a similarly inexplicable fashion (Carmel Meurant 2012, pers. comm.). Mounds in the Eulo area have popped up within the past decade (Mick Beresford & Ian Pike 2012, pers. comm.), whereas others ‘spew’ out mud sporadically, but may lie dormant for years (Figure 14). There are reports of mud springs ‘exploding’ with ‘loud bangs’ in the Eulo area and on Currawinya (Anon. 1904; Pickard 1992; Deveson 2004).

(b)

(a)

(c)

Figure 13 (a) Active mounded mud spring on the Paroo River floodplain, Farnham South group, (b) Big Bog mud spring, in a small scalded depression in the Dead Sea, Penaroo, and (c) mound spring landscape at Jubilee Springs from the air; this cluster covers 2 kilometres × 1.5 kilometres along the eastern side of Boorara Creek and comprises hundreds of vents, most of which were apparently inactive before European settlement.

(a)

(b)

Figure 14 Mud spring on a claypan west of Springvale house, (a) with plumes flowing to the west of the mound in February 1999, and (b) active but with no surface mud in December 2012.

At some sites, mud and water springs occur in close proximity. Within a large basin known as the Dead Sea north-west of Eulo, the southern and northern spring groups are comprised of water springs, while the central group contains only mud springs. Bingara Bore, Bullenbilla, Springvale Claypan, Corina House, Fish (Ogilvie and Edwards’ ‘Tarko’), Jubilee, Tunca, Wirrarah and Yarraman springs all have mud and water springs situated on the same claypan.

Of the 71 water springs documented, 36 remain active (Table 5). The two largest springs of the district, Wiggera and Youleen, are completely inactive (Figure 15). Where there are historical descriptions from Ogilvie and Edwards, a decline in flow and wetland area is often evident even when springs remain active, for example at Tunga, Myrton Bore and Gooning Springs. Water springs in the Eulo group can either be vegetated and form the ‘carpet springs’ as described by Ogilvie and Edwards, or be unvegetated pools (Figure 16). Although it can be difficult to tell which category an inactive spring would have fallen into, based on field characteristics and the historical record, we consider that 51 water springs would have been vegetated and 18 unvegetated.

(b)

(a)

Figure 15 Described in 1912 as the two largest springs in the Eulo supergroup, both Wiggera and Youleen are now extinct. (a) Wiggera stopped flowing in the 1950s, while water was being pumped out of Youleen Springs via a windmill until the 1960s, and (b) the five-hectare formerly swampy area created by springs south of Youlaingee Creek.

(b)

(a)

Figure 16 (a) Poached Egg Spring in Basin Bore group, Currawinya National Park, a typical unvegetated water spring, and (b) Massey Spring wetland, showing carpets of spring vegetation interspersed with shallow pools of free water.

Table 5 Active and inactive springs by spring type, Eulo supergroup.

| Spring type | Active | Inactive | Total | Proportion active (%) |
| --- | --- | --- | --- | --- |
| Mud | 25 | 9Pa | 36 | 69 |
| Mounds | 12 | 9 | 21 | 57 |
| Depressions | 5 | 0 | 5 | 100 |
| Mounds and depressions | 10 | 0 | 10 | 100 |
| Water | 36 | 35 | 71 | 51 |
| Carpet | 29 | 24 | 53 | 55 |
| Unvegetated | 7 | 11 | 18 | 39 |
| Total | 61 | 44 | 107 | 57 |

a Includes six that were probably inactive before pastoral settlement.
Note: ‘Active’ includes spring groups with at least some active springs. The character of three unsurveyed inactive springs (Cainawarro bore, Tunkata West and Gourminya) is unknown and they are not included in the table.

The dynamism of mud springs and their relative immunity to aquifer drawdown stands in stark contrast to the water springs, which have been greatly affected by loss of pressure in the aquifer. This suggests that mud and water springs may result from different hydrogeological processes; however, much remains to be understood about spring formation and behaviour.

## Cultural history

The Kunja, Budgiti, Kullilli and Mardigan people have lived in the Eulo–Hungerford area for at least 14 000 years (McKellar 1984; Robins 1999). Many of the springs in the Eulo supergroup have retained Aboriginal names, including Gourminya, Tunkata, Curracunyah, Goonamurra, Minyeburra, Muntha Napper, Oonunga and Wirrarah. Although the stories associated with these places have mostly been lost, Aboriginal artefacts were recorded at all except 2 of the 105 spring groups surveyed, testament to the long and rich association between people and springs.

Robins (1993) investigated the archaeology of the Currawinya Lakes area and documented a high density of artefacts at Youleen (Youlain), Bokeen, Kopanyu (Kaponyee) and Riley Springs. Excavations at Youleen revealed an especially rich and complex archaeology dating back at least 13 000 years, including silcrete tools (cores, flakes and retouched flakes), a piece of ochre, hearths and bone fragments—the only ones observed in association with archaeological material in the area (Robins 1998). Youleen Springs, formerly one of the largest in the district and just five kilometres west of the huge salt Lake Wyara, has by far the highest density and variety of archaeological material recorded at any spring in the Eulo supergroup. The two main camp sites correspond to Robins’ western and eastern sites, and each cover about 10 hectares on the scalded flats adjacent to Youlaingee Creek (Figure 17).

(b)

(a)

Figure 17 (a) Robins’ western site, showing high density of stone artefacts across claypan with intact hearth in foreground, and (b) an axe measuring 15 centimetres across.

An additional 27 springs visited for this project had high densities of artefacts, similar to Bokeen and Kopanyu as recorded by Robins (1997). These sites typically include a range of silcrete flakes, reworked flakes, cores, grindstone fragments, hammerstones and/or hearths, suggesting more intensive and consistent usage (Veth 1993, Witter 2004). In general, sites south-west of Eulo have higher artefact densities than the eastern line, probably reflecting the supply of fresh water provided by these larger spring groups, as well as their proximity to the Currawinya Lakes and permanent waterholes of the Paroo River. Distribution of artefacts is also dependent upon distance to sources of siliceous rock, mostly sourced from hard mulga country and dissected residuals (Robins 1997; Holdaway et al. 2000). Most springs south-west of Eulo occur close to granite outcrops and/or stony hills that include silcrete, so are close to both water and stone sources. The recorded density of artefacts on claypans at least partially reflects the ease of visibility and their exposure through erosive processes (Robins & Swanson 2006).

Stone tools and hearths are abundant on claypans throughout the five spring groups along the Currawinya–Boorara road, probably reflecting their position between a large quarry site in Hoods Range to the east and the almost-permanent freshwater Lake Numalla to the west. The largest and still-active mud spring of the Barb Tank group was itself a quarry site, with boulders and larger rocks thrown up by the mud spring used as core stones (Figure 18a). The artefact scatter at Warroo Springs includes a high density of stone tools and intact stone hearths (Figure 18b), and extends at least one kilometre. Even relatively small and isolated springs such as Burtanya, Colanya, Goonerah, Tareen and Waihora have high densities of stone tools, cores and hearths on their surrounding claypans. Conversely, the large water springs along Yowah Creek are characterised by a mysteriously low density of artefacts, although a thorough exploration of the area could reveal sites situated away from the main wetlands.

(b)

(a)

Figure 18 (a) Large active mud spring in Barb Tank group, Currawinya National Park, measuring 30 metres × 20 metres × 2 metres; larger rocks thrown up by the spring were used as core stones for making tools, and many flakes remain strewn across the mound, and (b) intact stone hearth at Warroo Springs south-east of Hungerford.

Aside from the springs, there are only three permanent natural water bodies (i.e. have not been dry since pastoral settlement) across the 150 kilometres × 100 kilometres area encompassing the Eulo supergroup—all waterholes on the lower Paroo on Currawinya National Park and Ningaling (Silcock, unpublished data). Semipermanent waterholes are scattered along the channels of the Paroo and Yowah Creek. However, with the notable exception of the almost-permanent freshwater Lake Numalla, GAB springs are the only lasting surface water between the Paroo and Bulloo rivers, a distance of almost 200 kilometres. Not surprisingly, the water springs and some mud springs were used extensively by early settlers, as traveller stopovers, sites for huts and stations, and vital watering points for livestock.

The line of springs from Fords Bridge to Hungerford formed critical stepping stones for the pioneer pastoralists, as discussed in the Bourke supergroup chapter. The main road still follows this line of springs, and old cart tracks are visible beside the main road on a hard patch of red earth 10 kilometres west of Warroo Springs (Figure 19). In 1865, John CostelloeP0FP and his family followed these springs through the dry country north-west of Bourke with 200 cattle and 15 horses. They set up a depot at the most north-westerly spring, 16 kilometres south-east of present-day Hungerford, which they named ‘Warroo Springs’. Mary Durack describes their arrival at the springs in Kings in grass castles:

‘Feeling their way up the inconsistent stream of the Warrego and across the Cuttaburra , they moved on to the sandy, timeless plains of the Paroo into an area of seemingly permanent springs, which they judged to be fair horse country but of little use for the cattle.’

© Copyright, Durack (1959, p. 88)

However, by 1867, these ‘permanent’ springs had ‘almost given out’ (Durack 1959; p. 102–103). When the Costelloe’s relatives, the Duracks, arrived at Warroo Springs, the party moved to a good waterhole John Costelloe had found on Moble Creek some 450 kilometres to the north. By 1875, the Costelloe–Durack pastoral empire covered nearly all the land between Kyabra Creek and the Diamantina River in Queensland’s Channel Country, and they later owned stations across northern Australia. Thus, Warroo Springs played a key role in the pastoral history of inland Australia. The first generation of the Dunk family to take up the Brindingabba lease also initially lived at the springs. The remains of an old hut lie on a sandy rise near the bore, above the now-inactive spring.

Figure 19 Wagon tracks remain beside the current road, a sign of early travellers who followed the line of springs.

People travelling between Eulo and Thargomindah used the Dewalla and Old Bingara Springs until at least 1920. A drover travelling west in search of work in March 1881 described the gardens at Bingara as ‘well laid-out, with small tanks for watering, and a large tank outside for the horses, all supplied from the springs’ (Nameerf 1881). Edwards (1911) recorded that ‘a single mud spring at [Old Bingara] was the main supply for the old station and is now used at the accommodation house for coach travellers.’ The garden edging remains at Old Bingara, along with pipes, tanks, concrete slab, posts and piles of bottles. There are three graves of early settlers nearby: Fitzherbert Brooke (died 1887), Dolly Martyr (aged 17) and an unmarked grave (Figure 20 a–b).

Curracunya Town Reserve and Hotel are marked on the four-mile map of 1916, and were established to serve thirsty travellers on the stock route between Hungerford and Thargomindah (Figure 21). The ruins and the old road remain on a stony rise above the now-dry springs (Figure 20c). Gooning Spring was ‘the only good supply for travelling stock between Dynevor and Boorara’ (Edwards 1912), while Tarko (now known as Fish) Springs provided a drink for travellers between Granite Springs and Boorara. The few main roads through the area now take the most direct route between towns, and these old tracks have become disused and mostly impassable, fading relicts of a time when the springs dictated movements of people as they had for millennia.

(c)

(a)

(b)

Figure 20 (a) Grave of Fitzherbert Brooke, who died in 1887, (b) undated grave of Dolly Martyr, aged 17 years, near Old Bingara Station, and (c) ruins of Curracunya Hotel on a stony ridge above the basin with now-extinct springs.

Figure 21 Four-mile series 1 sheet 3C (1931), showing Curracunya Springs Hotel and Well, an important stopover on the stock route between Hungerford and Thargomindah.

Newspaper reports from travellers in the 1880s to early 1900s refer to the ‘celebrated’ or ‘famous’ mud springs on the Eulo–Thargomindah Road west of Yowah Creek, south of the present-day Merimo homestead. They seem to have been a minor tourist attraction, as well as providing water for a town reserve, hut and stock. A drover looking for work travelled from the Balonne to the Bulloo in 1881 and recounted his travels to The Queenslander, including a detailed description of emerging from mulga ridges onto:

‘ … a fine open plain, on the opposite extremity of which are seen what appear to be a collection of huts. These are the Springs. What seemed to be a field of green maise is the main spring, fenced to keep off cattle or horses, which if they once get bogged could not be very easily extricated. There are numerous springs, which a Mr Riordan, who is contractor with Mr Bradley for fencing, opened, and having made channels running into the lower creek, has succeeded in obtaining an unfailing supply of splendid water, quite sufficient to water his horses and bullocks and for household purposes. Riordan has a good hut at the Springs, which he makes his headquarters. Leaving the Springs, I got onto Bingara … ’

© Copyright, Nameerf (1881)

Three years later, The Brisbane Courier’s ‘special representative’ to the ‘never-never country’ went on a coach journey between Eulo and Thargomindah, writing:

‘An hour or so, however, saw the coach over [Yowah Creek] and into mulga country with here and there a small sand ridge. The celebrated mud springs were then come to, and were observed with interest. From these volcano shaped mounds, some of them 20 feet high, came trickling streams of clear water, filling the numerous billabongs in their vicinity … ’

© Copyright, Anon. (1894)

Correspondent ‘HFO’, despite being glad to have left behind the ‘dirty looking and dirty smelling’ township of Eulo with its ‘evil reputation as a place where a man could be “lambed down” as quickly as in any part of Queensland’ and ‘where no one who could possibly help it would remain an hour longer than necessary’, was less impressed by the springs:

‘Distant twenty miles from Eulo is the Springs Station … Here I got my first experience of the mud springs of Australia. They are conical excrescences upon the surface of the earth, looking like piles of dug up loose soil. It is better to inspect—admire them you cannot—from a short distance as a too close acquaintance is likely to lead to unpleasant if not dangerous consequences.’

© Copyright, Anon. (1889)

Fourteen huts, three of which became stations, were built on springs. Springs, or wells dug on or near springs, provided the only surface water for boundary riders at Bokeen, Myrton Bore, Oonunga and Tynghynia, and for outstations at Corina Springs, Goonerah (Figure 22), Granite Springs, Tareen (Figure 23), Tilberry and Wooregym. Many of these springs would have had much greater flow rates to support these now-abandoned habitations. The ruins of an extensive homestead—including concrete foundations, elaborate piping and plumbing, tanks, animal cages, garden edging and timber yards—lie beside Goonamurra spring and waterhole north of Eulo, while the remains of a small hut, apparently occupied by a band of outlaw horse-duffers, are nestled amongst the granite boulders behind Twomanee Spring (Figure 24a). The location of Oonunga Springs and hut on Bingara, described in detail by Ogilvie and Edwards, remains a mystery after extensive searching.

A cook’s galley and shearers hut were located on a sandhill above the Baroona Springs north of Cunnamulla. The woolshed and hut as described by Edwards were relocated to a nearby bore that was sunk in 1907 and, ultimately, to their present site near the Baroona house (Bora Schmidt 2012, pers. comm.). An old mud oven remains on a low dune above the now-inactive springs under a huge coolabah. A silted-up well marks the site of a mud spring at the Boorara woolshed, which was cleaned out each year to provide water at shearing time (Figure 24b).

Figure 22 Two boxed wells mark the site of the inactive spring at Goonerah on Boorara, which once supplied the outstation. A rusty bucket still with the wire attached lies near the well (left) and was probably last used by Ruby and Jack McKellar, who lived there until the 1950s. The outstation ruins are on a low dune overlooking the claypan and spring (right).

Figure 23 Well built on small spring (left) at the abandoned Tareen Outstation, in the northern portion of Boorara (right).

(a)

(b)

Figure 24 (a) The remains of a hidden hut among granite boulders behind Twomanee Spring, Granite Springs, and (b) silted-up well marking the site of old mud spring, with Boorara shearers quarters in the background.

The Eulo springs were highly valued and used for stock water by early pastoralists. Ogilvie and Edwards several times quoted the number of stock able to be watered at a spring as a measure of its importance. Wiggera Spring was discharging a precious 180 kilolitres per day in 1911, and had been the subject of litigation between the owners of Dynevor and Boorara. To the south, Gooning Springs provided the main watering place for a 1300 hectare paddock and was viewed as a ‘valuable asset [to Dynevor] owing to its proximity to Mitchell grass plains south of the spring’ (Edwards 1912). A correspondent to the South Australian Register reported that improved springs provided ‘splendid, never-failing reservoirs of water … and by this means mostly all the stock on the Springs Station [now Carpet Springs/Merimo] and Dynevor Downs are watered’ (Anon. 1889, p. 332).

Most water springs and some mud springs have wells, usually silted up and dry, sunk in or near them (Figure 25a). The supply obtained at a long-dry well near Pitherty mud spring (Figure 25b) south of Eulo was especially large: in 1912, Ogilvie (1945) described how ‘after baling for 1¼ hours the level had been lowered by only 2 feet, 6 inches, whereas in an ordinary well about 10 feet would have been taken out’. Many water springs have been excavated to provide better access for stock. Excavating mud springs tends to be perilous and unsuccessful, so they were often simply fenced or, in the case of Jacombe Spring on Warroo, lined with timber corduroy to prevent stock becoming bogged. The remains of old yards lie near some springs, including the impressive beefwood yards at Minyeburra Springs on Tarko (Figure 26a). In Taleroo Paddock, on Karto north-west of Hungerford, a partially buried sheep yard near a spring marked on the 1916 four-mile map provides the only clue that there was once permanent water in the vicinity (Figure 26b).

(b)

(a)

Figure 25 (a) One of two timbered dry wells, marking the site of an inactive spring at Bush Springs, Wombula, and (b) Pitherty Well, 400 metres south of an active mud spring, has been dry for at least 50 years, but once had a huge flow rate.

(b)

(a)

Figure 26 (a) Beefwood yards near Minyeburra springs and hand bore, Tarko, and (b) half-buried low sheep netting fence in vicinity of spring marked on a four-mile map, Karto.

Today, many active springs continue to be used for stock water. Recent years have seen the establishment of some ‘groundwater tourism’, including the Eulo mud baths, suitably paired with a date winery, and a tourist drive and interpretative signage at the Springvale Mound Springs Nature Refuge. The name Picnic Sandhill Mud Spring hints at some private recreational use and enjoyment of springs by landholders and visitors. Numerous spring groups now fall under nature refuges and national parks, in recognition of their outstanding biological values.

## Biological values

The Eulo supergroup is probably the most biologically significant of the four spring groups considered for this project, despite the substantial losses with aquifer drawdown and the fact that only nine spring groups still support endemic species. Spring endemics include six plants, three crustaceans and seven molluscs, many with very restricted distributions (Table 5). In particular, seven molluscs are endemic to single spring groups (four to Yowah Creek, and one each to Granite, Tunga and Paroo River springs). Two crustaceans, Ponderella ecomanufactia and P. bundoona are known only from the Paroo River and Yowah Creek springs, while the latter complex contains the only known populations of an undescribed crustacean (Austrochiltonia sp. AMS P68160), spider (Mamersella sp. AMSKS85341) and flatworm (Weissius capaciductus). Eragrostis fenshamii is known from four Eulo spring groups, Hydrocotyle dipleura from five, Sporobolus pamelae and Eriocaulon carsonii from two each, and Isotoma sp. (Myross RJ Fensham 3883) from Yowah Creek only. Myriophyllum artesium occurs at seven spring groups and is also known from bore drains in the area.

None of the previously unsurveyed springs surveyed in 2012 contained spring endemic species; however, an additional population of the vulnerable Hydrocotyle dipleura was found at Little Granite Springs. This species is known only from eight spring complexes in the Eulo and Barcaldine supergroups. The Granite Springs group were very heavily grazed in the 1999 and 2004; however, their wetlands had recovered with reduced grazing pressure in 2012 (Figure 27). The species assemblages of other spring wetlands have generally remained constant between 1999–2000 and 2012. However, Tunga Spring was very heavily grazed in 2012, and there was no sign of its endemic snail Jardinella sp. AMS C.156780 or the tiny annual forb Triglochin nana (Figure 28). Massey Spring and the recently fenced Fish Springs show that spring wetlands can recover rapidly once grazing pressure is relieved. However, this does not mean that some species have not been lost, as discussed later in this section.

Figure 27 Massey Spring in the Granites Spring group, heavily grazed in 2004 (left), and spring wetland vegetation recovered in 2012 (right).

Figure 28 Tunga Spring in August 2000 with dense wetland vegetation (left), and spring wetland much diminished by goats in October 2012 (right).

Yowah Creek is the biological jewel of the Eulo supergroup with 15 spring endemics, including 4 molluscs and 1 crustacean known only from this group, and numerous disjunct (Table 6). It is third only to the Pelican Creek complex north of Aramac and Dalhousie in South Australia in its concentration of endemics. There are five springs, most containing multiple vents, in a narrow band running south-west across the Yowah Creek channels. Site 223 has the largest wetland area, and supports all endemic and disjunct species recorded, including the only population of Isotoma sp. (Myross RJ Fensham 3883) in the Eulo supergroup (Figure 29). This species is known only from eight springs in the Pelican Creek complex and this spring, where it is patchily abundant with the total population numbering more than 500 plants.

Table 6 Spring groups containing endemic species and/or disjunct populations in the Eulo supergoup.

| Spring group | Endemic species only known from spring complex | Species endemic to GAB spring wetlands | Disjunct populations |
| --- | --- | --- | --- |
| Yowah Creek | Crustacea: Austrochiltonia sp. AMS P68160; Molluscs: Jardinella sp. AMS C.400131, Jardinella sp. AMS C.400130, Jardinella sp. AMS C.400133 , Jardinella sp. AMS C.400132 | Plants: Eragrostis fenshamiii, Eriocaulon carsonii, Hydrocotyle dipleura, Isotoma sp. (Myross RJ Fensham 3883), Myriophyllum artesium, Sporobolus pamelae Crustacea: Ponderella ecomanufactia, Ponderella bundoona Other invertebrates: Mamersella sp. AMS KS85341; Weissius capaciductus | Plants: Cenchrus purpurascens, Phragmites australis, Schoenus falcatus, Spirodela punctata, Utricularia caerulea |
| Paroo River | Molluscs: Jardinella sp. AMS C.410721 | Plants: Eragrostis fenshamiii, Myriophyllum artesium Crustacea: Ponderella ecomanufactia, Ponderella bundoona Other invertebrates: Weissius capaciductus | None |
| Masseys (Horseshoe) | Molluscs: Jardinella eulo | Plants: Eragrostis fenshamiii, Myriophyllum artesium | Plants: Utricularia caerulea |
| Tunga | Molluscs: Jardinella sp. AMS C.156780 | Plants: Myriophyllum artesium | Plants: Schoenus falcatus, Triglochin nana, Utricularia dichotoma |
| Dead Sea (including Dead Sea South and Little Bundoona) | None | Plants: Eragrostis fenshamii, Hydrocotyle dipleura, Sporobolus pamelae | None |
| Little Granite | None | Plants: Hydrocotyle dipleura, Myriophyllum artesium | Plants: Utricularia caerulea |
| Merimo (Site 238) | None | Plants: Eragrostis fenshamii, Myriophyllum artesium | Plants: Phragmites australis |
| Wooregym | None | Plants: Hydrocotyle dipleura, Myriophyllum artesium | Plants: Triglochin nana |
| Tungalla | None | Plants: Myriophyllum artesium | None |

GAB = Great Artesian Basin.

Figure 29 Spring wetland of Site 223, the biggest spring of the Yowah Creek group and containing all endemic species recorded from the group, including the extremely restricted aquatic forb Isotoma sp. (Myross RJ Fensham 3883).

Based on the descriptions of Ogilvie and Edwards, and knowledge of the species assemblages of active springs, it is possible to gain a picture of some spring wetlands before their extinction. Most of the descriptions refer to ‘clumps of tall rushes’, ‘green feed’, ‘grassy bogs’ or ‘carpet springs’ (Table 7). Cyperus laevigatus dominates many active carpet springs in the supergoup, and was probably a common component of the extinct spring wetlands. Long-term resident Ian Pike recognised Eriocaulon carsonii from Wiggera Spring, which suggests that it was also present in the other large spring groups where Ogilvie and Edwards describe ‘grassy bogs’ and ‘carpet springs’, including Baroona, Youleen and Dewalla, and possibly Gooning, Ooliman and Myrton Bore. These larger spring wetlands are also most likely to have contained the other endemic species, although Hydrocotyle dipleura, Eragrostis fenshamii and Sporobolus pamelae can grow on small soaks (e.g. at the Dead Sea and Little Granite springs). The mat-forming aquatic forb Myriophyllum artesium seems most likely to be the ‘vegetable growth’ giving the water a ‘weedy taste’ at Dewalla, and was probably present in many of the larger springs with free water.

Baroona Springs seems to have had an especially diverse wetland assemblage, with ‘the whole surface thickly coated with duckweed, mosses, rushes and other vegetation [growing] so thickly … that the cattle do not bog’ (Edwards 1912). The descriptions of the large, grassy bogs at numerous springs are especially tantalising, and raise the possibility that localised endemic plants and fauna from this supergroup are now extinct. Given the extremely restricted distribution of most snail species (Ponder 2004), it seems certain that some were lost as springs became extinct. Figure 30 shows the inferred distribution of spring endemics, based on the historical record, species composition of active springs and characteristics of inactive springs surveyed. It appears that the biggest losses would have been in the western springs from Dewalla to Youleen, as well as at Baroona.

Table 7 Vegetation descriptions of Ogilvie and Edwards, 1911–1912.

| Spring | Description |
| --- | --- |
| Baroona West | ‘There are several small springs not flowing, but within 100 yards of the main spring their presence being noticeable on account of the clumps of tall rushes.’ |
| Baroona East | ‘ … the whole surface is thickly coated with duckweed, mosses, rushes and other vegetation. So thickly are these growing that the cattle do not bog … ’ |
| Bullenbilla Group 1 | ‘About 6 of the [mound] springs overflow slightly and form small patches of green feed … ’ |
| Bullenbilla Group 2 | ‘ … 5 very large mounds and 2 small carpet springs in the bed of a watercourse … ’ |
| Bush Springs | ‘ … the whole [spring, well and trough] being enclosed in a very dense clump of reeds.’ |
| Curracunyah | ‘The springs do not overflow to any extent and merely render a few acres of the flat almost impassable.’ |
| Dewalla | ‘The discharge of the main spring is probably about 500 g.p.d…it is absorbed in a grassy bog about 1 chain wide. In large springs decaying vegetable growths have given the water a weedy taste … ’ |
| GooningPa | ‘The quality of the water is excellent being quite free from odour or taste in spite of a dense growth of reeds around the spring.’ |
| OolimanP aP | ‘The main spring of this group is situated 1¼ chains south west of the bore and is surrounded by a grassy bog about 20 foot wide.’ |
| Fish (Tarko)P a | ‘The main spring has at some time been fenced in and the remains of some troughing are alongside, but at present no use is made of the spring, which has become overgrown with reeds.’ |
| Myrton BoreP a | ‘The surplus forms a small swamp in the bed of the claypan and this is overgrown with the reed peculiar to the spring area.’ |
| TunkataP aP | ‘ … and the water [in the spring/well] was protected by growths of tall reeds.’ |
| Wiggera | ‘A few reeds flourish on the edge of the pool, and good feed is growing in the watercourse, which, however, is rather boggy.’ |
| Wiggera (carpet) | ‘A few small carpet springs occur at the main spring and 4 or 5 about 15 chains west.’ |
| Youleen | ‘The water here spreads over an area of a few acres causing swamps. A growth of green grass has attracted the cattle with the result that the whole area is now a bog and consequently a proper estimate of the flow is difficult to arrive at.’ |

© Copyright, Edwards 2012.
a Denotes active springs in 2012.

Figure 30 Likely distribution of endemic species, as inferred from historical record and field surveys.

Although lacking specific references to vegetation, the descriptions of Ogilvie and Edwards indicate that Bingara Bore, Curracunya, Kopanyu/Kaponyee, Minyeburra and central Wirrarah springs were also water springs with sizeable wetlands and possibly endemic species. It is also likely that many still-active springs have lost endemic species through drawdown, excavation, and/or grazing and trampling. For example, the ‘grassy bog about 20 foot wide’ (Ogilvie 1912) at Ooliman Spring has been completely obliterated by goats and pigs (Figure 31a), while the ‘small swamp in the bed of the claypan … overgrown with the reed peculiar to the spring area’ (Edwards 1912) at Myrton Bore is non-existent. The description of Kopanyu as ‘the best spring on [Currawinya] and second only to Youleen and Wiggera in the district’ (Ogilvie 1912) seems incongruous with its current local name of the ‘stinking spring’—a single pool of black water with a patch of Cyperus laevigatus on one side (Figure 31b). This pool is surrounded by scalds, mounds and depressions, and it seems that it was a bigger complex supporting extensive wetlands before bore drilling. Ogilvie also states that the flow, and presumably wetland area, of Tunga Spring was much larger before the bore was sunk (Oligvie 1912).

The centre of large springs is often inaccessible to herbivores, and the shrinking of spring wetlands with aquifer drawdown would have not only reduced habitat, but also exposed the wetland to grazing and trampling. The fleshy endemic Eriocaulon carsonii is especially vulnerable to destruction by pigs, which dig up its roots in their feeding frenzies. Triglochin nana, currently known in Queensland from just three springs, was collected at Goonamurra by Queensland Government botanist Selwyn Everist in 1938 (Queensland Herbarium database). In December 2012, this spring was active, but very heavily grazed and trampled by cattle, and Triglochin sp. were not found.

Many springs, including Tungalla (Figure 31c), Fish (Ogilvie and Edwards’ ‘Tarko’), Double Well, Tunkata and Wanco, have been excavated, sometimes repeatedly, to provide better access to water for stock. These disturbed wetlands are usually dominated by Cyperus laevigatus, sometimes with Myriophyllum artesium, and it is possible that the original spring assemblages may have been richer, including, for example, Eragrostis fenshami, Sporobolus pamelae and/or Eriocaulon carsonii. In other cases, excavation has completely obliterated spring wetlands—for example, at Rose and Bullock Paddock springs in the Dead Sea group.

Despite considerable progress of the 61TGreat Artesian Basin Sustainability Initiative61T, including rehabilitation and regulation of 27 bores with flows of more than 0.5 litres/second within 50 kilometres of high-value springs, there remain flowing bores (Figure 31d) within close proximity of high-value springs, some of which have large flows (Table 8; Figure 32).

Table 8 High-flow boresPa Pin close proximity to high-value springs in the Eulo supergroup that have not been capped and should be a priority for rehabilitation.

| Property (run) | Latitude | Longitude | Flow (L/sec) | Distance to (km) and name of high-value spring |
| --- | --- | --- | --- | --- |
| Tilbooroo (RN4538) | –27.76330 | 144.94310 | 38.00 | 20 (Dead Sea) |
| Jandell (RN4902) | –27.81610 | 144.49140 | 35.32 | 31 (Yowah Creek) |
| Bingara (RN5006) | –28.11110 | 144.61690 | 13.28 | 14 (Tunga) |
| Strathlea (RN2273) | –28.24010 | 145.48810 | 5.09 | 45 (Paroo River) |
| Yowah town (RN4976) | –27.97310 | 144.63610 | 3.36 | 13 (Yowah Creek) |
| Moonjaree (RN4547) | –28.05440 | 145.47470 | 1.92 | 46 (Paroo River) |
| Gumahah (RN4561) | –28.47110 | 145.33330 | 1.68 | 45 (Paroo River) |
| Tarko (RN6750) | –28.34140 | 144.74310 | 1.50 | 16 (Tunga) |
| Werai Park (RN5329) | –28.56640 | 144.94530 | 1.39 | 40 (Wooregym) |
| Wandilla (RN5409) | –28.31640 | 145.07030 | 1.19 | 18 (Paroo River) |
| Werai Park (RN1821) | –28.60440 | 144.92390 | 0.97 | 43 (Wooregym) |
| Currawinya National Park (RN7408) | –28.75860 | 144.41600 | 0.81 | 49 (Little Granite) |
| Gumahah (RN4552) | –28.50390 | 145.24690 | 0.76 | 43 (Paroo River) |
| Moonjaree (RN4541) | –27.99530 | 145.33680 | 0.56 | 37 (Paroo River) |
| Springvale (RN6574) | –28.20530 | 144.91110 | 0.56 | 7 (Merimo) |
| Jandell (RN4901) | –27.73560 | 144.41030 | 0.54 | 42 (Yowah Creek) |
| Carpet Springs (RN5538) | –28.16810 | 144.86890 | 0.50 | 6 (Merimo) |
| Springvale (RN2546)Pb | –28.18690 | 144.98860 | 0.47 | 5 (Paroo River) |
| Goonamurra (RN5047)PbP | –28.04420 | 144.87500 | 0.44 | 13 (Dead Sea) |
| Wittenburra (RN6207)Pb | –28.39500 | 144.67970 | 0.25 | 17 (Wooregym) |
| Turn Turn (RN9094)Pb | –28.30390 | 144.82170 | 0.20 | 9 (Wooregym) |
| Springvale (RN12202)Pb | –28.18940 | 144.97750 | 0.20 | 7 (Paroo River) |
| Paddabilla (RN4546)Pb | –28.20220 | 145.09140 | 0.17 | 7 (Paroo River) |

a High flow is defined as more than 0.5 megalitres (ML)/day.
b These bores have a flow of <0.5 ML/day, but are <20 kilometres from high-value springs.
Source: Unpublished Queensland GABSI records.

(b)

(d)

(c)

(a)

Figure 31 Spring wetland destruction. (a) Ooliman Spring, Ogilvie’s ‘grassy bog’ completely obliterated by goats and pigs, (b) Kopanyu Spring pool, now referred to as the ‘stinking spring’, but formerly regarded as the third-best spring in the district, (c) Tungalla Spring wetland, excavated in the 1950s and 1970s, possibly supported a more diverse wetland assemblage, and (d) Werewilka bore, situated in the western Eulo supergroup, 13 kilometres west of the high-value Granite Springs group, which has now been capped under the 61TGreat Artesian Basin Sustainability Initiative61T.

Figure 32 Status of bores within 50 kilometres of high-value springs; high-flow bores that should be a focus of rehabilitation are represented by red and orange circles, enclosed by squares. Uncapped bores are free-flowing without rehabilitation, partially capped bores have been rehabilitated but are still free flowing.

A comprehensive survey of Calocephalus sp. (Eulo ME Ballingall MEB2590), a daisy endemic to groundwater scalds and the sides of mound springs in the Eulo supergroup, was also conducted between October and December 2012. This species is listed as near threatened, but this survey has resulted in it being recommended for listing as vulnerable under the Nature Conservation Act 1992 (Qld) (Appendix D). Eighteen populations were found across a 7000-square-kilometre area (Figure 33a), but with an area of occupancy of just 3.2 square kilometresP P(Keith 2000). The total population size was estimated to be 15 000 individuals. The largest population, at Basin Bore on Currawinya National Park, contained more than 5000 plants, while some populations contained only 50 individuals. However, population size is known to vary substantially with seasonal conditions, with episodes of mass death and germination.

The species is highly palatable, and 13 of the 18 populations were heavily grazed, usually by goats and/or cattle. Plants were typically cropped to ground level, although most still had a few flowers. Ungrazed plants (usually in fenced areas) have a very different growth form to grazed plants, spreading to 40 centimetes tall and up to 80 centimetres across, and being laden with flowers (Figure 33b). It seems likely that the species was more abundant before the introduction of feral and domestic herbivores, and continued grazing pressure is likely to result in further declines in the abundance and fecundity of the species. However, plants can survive and produce seedlings, even under very heavy grazing pressure, and ongoing monitoring of fenced and unfenced sites at Granite Springs and Currawinya will shed further light on the long-term effects of grazing.

Other species associated with groundwater scalds and representing highly disjunct populations were also recorded, including Sporobolus partimpatens (recorded at 7 sites), Trianthema sp. (Coorabulka RW Purdie 1404; 23 sites) and Xerochloa barbata (8 sites). Aquifer drawdown does not affect the habitat of these species, as scalds remain present even after the extinction of the springs.

(b)

(a)

Figure 33 (a) Distribution of Calocephalus sp. (Eulo ME Ballingall MEB2590) based on 2012 surveys (green stars = species present, red crosses = species absent), and (b) an ungrazed specimen inside a fenced area at Basin Bore, Currawinya National Park.

The Eulo springs are also valuable for their fossil and archaeological deposits. Megafauna fossils, including some of the best-preserved Diprotodon skeletons in Australia were recently excavated by the Queensland Museum at old springs between Rose and Bullock Paddock springs on Bundoona. Crocodile and Diprotodon teeth were found at Riley Spring (Robins 1999), and bone fragments excavated at Youleen Spring included:

a brushtail possum (Trichosurus vulpecula) mandible

a phalanx form a pademelon-sized animal

the jaw of the now-extinct crescent nail-tail wallaby (Onychogalea lunata), dated securely to the Pleistocene

a right maxillary fragment from a small rock-wallaby sized wallaby

a dentary from spectacled hare-wallaby (Largorchestes conspicillatus)

a mandible from a southern brown bandicoot (Isoodon obseulus) (Robins 1998).

# Bourke supergroup

Rade (1954) noted the association of springs with faults or very shallow basement rock in western New South Wales, especially where an impervious bed blocks an artesian aquifer on the side of the fault opposite to the prevailing direction of water flow. Habermehl (1982) described the Bourke supergroup as extending from White Cliffs in south-west New South Wales and extending across northern New South Wales to Culgoa Floodplain National Park just across the Queensland border. Pickard (1992) documented about 45 sets of springs occurring in the Western Division of New South Wales, mostly within the Bourke supergroup, and visited 29 of these.

## Historical record

Most springs in New South Wales are marked on county map sheets and/or pastoral run surveys drafted between 1860 and 1964 (Table 9). Eight springs identified for this project were not marked on historical maps; however, Hawkes Spring Bore is marked on modern 1:250 000 topographic maps. Vincent Dowling’s 1860s sketch map from his diary and reproduced in Bourke and District Historical Society (1969), shows the approximate location of many springs (spelling as per Dowling): Yantabulla, Cullawillalee (Mother Nosey), Wapweela, Boongunyarra, Thooro, Pullamonga, Coonbilly, Youngarinnia, Yarranongany, Kullyna, Native Dog, Bunnavinyah, Gurrera, Old Morton Plains, Gooroomere, Sandy, Tooloomi, Cumborah and Thully. References to certain springs also appear in various newspaper articles and government reports; these are considered in Section 4.3. Queensland Government bore inspectors Ogilvie and Edwards visited Tego and Towry springs in 1911, and provided descriptions and a ‘mud map’.

Table 9 Springs marked on historical maps and plans, western New South Wales.

| Map/Plan inspected | Springs marked |
| --- | --- |
| Four-mile series, 1920 | Deadman, Log, Tego, Towry, Tin |
| County of Gunderbooka (1905) | Kullyna, Native Dog, Sledgehammer (WR31), Thully, Yarrongany |
| Corella Pastoral Run (c. 1886) | Thully |
| County of Irrara (1903) | Boongunyarrah, Coonbilly, Mother Nosey, Tanawanta, Thooro Mud, Thooro, Yantabulla, Youngerina |
| County of Irrara (1964) | Lake Eliza, Mascot, Tanawanta, Thooro, Yantabulla |
| Lissington pastoral run no. 90 (c. 1870) | Bunnavinyah, Colless, Nulty, Old Gerara |
| Pirillie pastoral run no. 53 (c. 1870) | Pullamonga |
| Weilmoringle pastoral run no. 3 (c. 1870) | Gooroomero, Sandy, Tooloomi, Yotomi |
| Toorale pastoral run no. 257 (c. 1870) | Goonery |
| County of Barrona (1913) | Goonery |
| Maranoa pastoral run no. 62 (c. 1870) | Yantabulla |
| County map of Narran (1914) | Coorigul |
| Pastoral holdings index NSW (1886) | Cuddie, Cumborah |
| Llanillo pastoral run no. 242 (c. 1860) | Cumborah |
| Wyabray pastoral run (1859) | Cuddie |
| Momba pastoral run no. 55 (c. 1870) | Bathing, Peery Lake Springs (western and eastern groups) |
| Tarella pastoral run (1878) | Paralna, Yoorltoo |
| County of Killara (1912) | Mulyeo, Wee Wattah |
| Dunlop pastoral run no. 240 (c. 1870) | Mulyeo |
| Lila Springs pastoral run no. 247 (c. 1870) | Lila |
| Multagoona pastoral run no. 228 (c. 1870) | Sweetwater |
| County of Yantara (1906) | Bingewilpa |
| County of Culgoa (1914) | Old Gerara, Bunnavinyah, Gooroomero |

c. = circa, NSW = New South Wales.

## Project survey and reassessment of status of springs

Of the 50 spring groups collated in our database, 26 had been visited in previous surveys (21 by Pickard in 1992, and six by Fensham and Fairfax in 2000) and were revisited. Three sites visited by Pickard and assigned as springs—Youngerina (Little Bore), Boongunyarrah and Gooroomero—were not located despite extensive searching. Seven sites documented but not visited by Pickard were surveyed in 2012, whereas eight previously or recently unknown springs were located from the historical record and/or local knowledge. We searched for but could not locate three springs marked on historical maps (Paralna, Sandy and Yotomi), and five springs were not searched for in the field. The locations of two of these (Old Morton Plains and Brenda Spring) remain obscure, whereas Yantabangee on the Paroo Overflow was underwater at the time of survey in 2012.

We consider that many of the springs in the line from Culgoa Floodplain to Lila Springs are unlikely to emanate from the Great Artesian Basin (GAB) (Figure 37). Nulty, Scrubber and Colless Springs are associated with the Tertiary land surface, which outcrops east of Enngonia and are assumed to be type O (non-GAB) springs (see conceptual diagrams in Appendix E). All seem to be permanent, although Nulty is the largest and its excavated pool maintains a constant water level (Brian Bambrick 2012, pers. comm.; Figure 34b). Colless and Scrubber are soaks emanating from rocky ground, and both fluctuate with recent rainfall. In November 2012, Colless was a tiny soak in a rocky creekline, while Scrubber had contracted in the past month but still covered an area of about 20 metres × 15 metres on the upper slope of a high rocky ridgeline (Figure 34b). Deadman Spring on Culgoa National Park is also associated with the Tertiary surface but was probably never permanent (Andy Coward 2012, pers. comm.).

(b)

(a)

Figure 34 (a) Nulty Spring feeds an excavated pool, and (b) scrubber emanates from the upper slopes of a rocky ridge. Both appear to be associated with the Tertiary land surface rather than the underlying Great Artesian Basin.

The remainder of this line—Tin, Log, Bernard’s, Gooroomero, Toolami, Bunnavinyah and Old Gerara, and probably Sandy and Yotomi, which were not located during the field survey—appear to be associated with a watertable close to the surface. Further interpretation is required to determine the association of these springs with the GAB or Tertiary aquifers. However, based on their character (in particular, a lack of groundwater scalds), the long-standing excavations at many sites and the historical description of WE Abbott, we have assumed that they were mostly small and did not support permanent wetlands. Abbott travelled the western districts to appraise pastoral land on behalf of the New South Wales Government in 1880 and wrote:

‘Gerrarra, or Gerrarra Springs, belongs to a Mr. Shearer (who is a very old resident in that part of the country), and is only remarkable for the springs from which the run takes its name. There are two springs situated within about a mile and a half of each other in a line running about north-east and south-west, and Mr. Shearer told me they form part of a line of springs extending in the same general direction, which were known to him for a distance of 150 miles, the springs being in some cases like those at Gerrara, close together and in others separated by long intervals of waterless country. Some of the springs in this line overflow and some, as those at Gerrara, only rise to within a short distance of the surface. Mr. Shearer has made an excavation in the rock at the mouth of each of the springs of which he is the owner, and draws the water out by horse power for his stock … and as the springs do not supply water as fast as it is drawn out, I had an opportunity of examining the formation both above and below where the water is coming out of the rock. I found that near the surface there was about 4 feet of a soft coarse red sandstone, unlike anything I had seen anywhere else on the Darling, with apparently a slight dip to the south-west, and under this about 4 feet of a coarse light coloured sand cemented slightly together, out of which the water was coming. The cemented sand was resting on a very hard conglomerate, composed chiefly of quartz pebbles; in fact, the same conglomerate which has already been described in this paper under the name of the murrillo conglomerate.’

© Copyright, Abbott (1881)

Abbott describes the ‘murrillo conglomerate’ as ‘[showing] in the banks of the Grawin, and … lying on a white rather soft rock, almost as light as meerschaum … In some places, this underlying rock is so soft as to be easily cut with a knife, and in others as hard as flint’ (Abbott 1881). Mr Bagot, a long-term resident on the Moonie River near the Queensland–New South Wales border, informed Abbott that:

‘ …all the springs about the Narran that are known to him are found in the murrillo conglomerate; some of them rising above the surface and some, like Gerarra, only coming to the surface. This seems to confirm the opinion which I have already formed, that the murrillo conglomerate, where it shows in the north-western part of the Colony, is very much older than the western plains, being part of an older continent, and from the Barwon River to beyond the Warrego forms a sort of bar which obstructs the flow of the underground water to the south-west and so causes it to rise up to or near the surface in many places, by flowing along the surface of the conglomerate whenever there is a slight deposit of sand between the clay and the conglomerate, as at Gerarra.’

© Copyright, Abbott (1881)

From these descriptions, it seems that water never rose to the surface as a natural spring at Old Gerara and Bunnavinyah; rather, excavations were made to reach a shallow watertable (Figure 35 a–b). The first cartographic reference to these ‘springs’ appears on the County of Culgoa map of 1914, by which time there was indeed water at the ground surface. The area around Gooroomero Spring is dotted with wells dug into a porous, friable rock layer, and water can be obtained in other places by digging. Water flows out of these rocks during rain periods, so that shallow puddles retain water for a long time, and the ground has a hollow sound at times (JD Davies 2012, pers. comm.). The three small springs on Culgoa National Park—Log, Tin and Bernard’s—are situated between the larger GAB spring groups Towry and Tego. However, they do not have extensive scalds and are more similar in appearance to the springs in the Gooroomero area. Although their origin is uncertain, they do not appear to have ever been permanent springs.

Toulby and Lila springs (both now inactive, although Toulby maintains an excavated hole; Figure 36) are the only springs in this line that appear to have been permanent GAB springs, and are characterised by large scalded areas created by groundwater precipitates. The standing water level of bore RN 004579 near Toulby has declined about 15 metres since 1950, suggesting that the spring once had a much larger flow. The three Tertiary springs discussed above are assumed to be the ‘overflowing’ springs described to Abbott in 1881.

(b)

(a)

Figure 35 Excavations into a shallow watertable at (a) Old Gerara and (b) Bunnavinyah.

(b)

(a)

Figure 36 (a) Lila Spring, showing extensive groundwater scald, and (b) excavated spring at Toulby house.

Northwest of Bourke, we were unable to find any trace of Boongunyarrah Spring, which Pickard (1992) photographed as a dry, shallow ditch on a stony rise near Mother Nosey Spring. A stony rise is an unlikely location for a GAB spring. Even before deep artesian drilling, this ‘spring’ had been dug out, and seems to have been a well tapping the watertable close to Mother Nosey Spring. According to Gilliat:

‘All indications of the old mud spring, if there was one, have disappeared. The present basin has at some time been dug out, and apparently has been poached by the trampling of cattle that the overflow I noticed four years ago has ceased, and the water stands about 6 inches below the edge of the basin, an occasional bubble indicating the presence of water from below.’

© Copyright, Gilliat (1885)

Our interpretation leaves 34 GAB spring groups (Figure 37; Table 10). Fourteen groups are comprised of single vents. Peery is by far the biggest group, with at least 130 documented vents. Tego, Towry, Native Dog, Thooro Mud Springs, Coonbilly and Mooroonowa South all have more than seven vents. Sixty-five per cent of GAB spring groups are now inactive, leaving only 12 groups with some active springs (Table 10), although the flow of at least some of these is greatly diminished (Figure 38 a). Some vents documented may have been inactive before European settlement, particularly in the Native Dog, Thooro Mud and Mooroonowa South groups (Figure 38 b).

GAB = Great Artesian Basin.

Figure 37 Springs formerly classified as part of the Bourke supergroup, north-central New South Wales. Non-Great Artesian Basin (GAB) springs include Tertiary springs and those in the line east of Enngonia.

Table 10 Great Artesian Basin spring vents and those identified as active in the Bourke supergroup.

| Spring | Total vents identified | Active vents | Notes |
| --- | --- | --- | --- |
| Coonbilly | 11 | 0 | Large spring complex, probably mostly water |
| Goonery | 1 | 0 | Mud springs |
| Hawkes | 1 | 0 | Now site of bore |
| Kullyna | 2 | 0 | One spring excavated plus inactive mud spring area |
| Lake Eliza | 7 | 7 | Small soaks across claypan |
| Lila | 2 | 0 | Excavated ditch plus mound spring area |
| Mascot | 3 | 2 | Active flat mud springs on claypan; possibly other inactive vents on edges of claypan |
| Mooroonowa North | 5 | 2 | Two active mud springs plus at least 3 inactive mud springs |
| Mooroonowa South | ~11 | 2 | Two small active mud springs; other scalded depressions across claypan—possibly pre-European extinct? |
| Mother Nosey | 1 | 1 | Active mud spring |
| Mulyeo | 1 | 0 | Now site of bore |
| Native Dog | ~16 | 0 | Mounds, craters and depressions |
| Peery Lakes | ~130 | ~130 | Many vents, mostly underwater in 2012; data from Jowett & Christie (2008) |
| Pullamonga | 1 | 0 | Active mud spring |
| Sledgehammer | 1 | 0 | Likely to have been a tiny, isolated spring |
| Sweetwater | 1 | 1 | Flow and wetland area probably much reduced |
| Tanawanta | 1 | 1 | Active mud spring; water is close to surface on other claypans in area |
| Tego | ~20 | ~20 | Six main vents, others dynamic |
| Thooro | 1 | 0 | On claypan; underwater in 2012 |
| Thooro Mud | 21 | 12 | Mud springs; some inactive mounds probably inactive pre-European settlement |
| Thully | 2 | 2 | Mud springs on claypan |
| Toulby | 1 | 0 | Excavated spring |
| Towry | 7 | 6 | Four main vents, one excavated plus numerous small vents |
| Wapweelah | 1 | 0 | Now site of bore; impossible to inspect old spring area |
| Wee Wattah | 1 | 0 | Spring was located on scalds 50 m west of bore; water spring with Eriocaulon carsonii present |
| Yantabangee | 1 | 0 | Mound spring surveyed by Pickard; underwater in 2012 |
| Yantabulla | >10 | 0 | Number of vents based on 1887 town plan |
| Yarrongany | >2 | 0 | ‘Springs’ marked on historic maps |
| Yoorltoo | 1 | 0 | Inactive depression |
| Youngerina | 3 | 0 | Site of bore; apparently one of largest springs in NSW |
| Youngerina (Little Bore) | 2 | 0 | Not located in 2012 |

~ = approximately, m = metre, NSW = New South Wales.
Note: Springs not located by Pickard (1992) or in 2012 surveys (Paralna, Bingewilpa, Bathing) are not included.

A cluster of springs north-west of Fords Bridge are mostly active mud springs and some inactive water springs, and are associated with fault lines intersecting with an underlying granite basement. Springs in the Yantabulla area occur along the eastern margin of a granitic basement horst, and small faults connect Kullyna – Native Dog and Coonbilly–Youngerina springs (Rade 1954). Goonery, Sledgehammer and Thully are single isolated springs; only Thully is still active.

A separate cluster occurs on the Paroo Overflow near the south-western edge of the GAB, all of which are extinct except the Peery Lake springs. Three previously undocumented springs were located on survey plans and county maps west of Peery, representing the most westerly springs in New South Wales (Figure 39). The far-western outlier, Bingewilpa, was not searched for during this project, but its location based on David (1891) and the County of Yantara map (1906) seems fairly certain to be Bingewilpa bore.

(b)

(a)

Figure 38 (a) Diminished but still active spring and wetland at Sweetwater Spring south of Enngonia, and (b) an example of a pebbly mound that was probably extinct before pastoral settlement, Native Dog Springs.

Figure 39 Section of Tarella pastoral run map (1878) showing Yourltoo and Paralna springs.

## Cultural history

Many springs in western New South Wales have Indigenous names, including Yantaboollaboola (Yantabulla), Bunnavinyah, Yanaranah (Youngerina), Culla–Willalee (Mother Nosey), Boongunyarrah, Goonarah (Goonery), as well as the impressive but mysterious Widgegelahgambone, which was mentioned by Kelley (1882) but was not located on any survey plans. The name Yantabulla apparently comes from an Aboriginal word meaning ‘stones around a spring’ (Geographic Names Board of NSW 2012). Unfortunately the original pronunciation, translation and stories attached to these places have mostly been lost. However, the surface archaeological record hints at the long and rich Aboriginal association with these springs. Stone tools were found at all except two of the spring groups surveyed, and were in high densities and spread across large areas at 14 sites. These sites typically include a range of reworked silcrete flakes, cores, grindstone fragments and/or hearths, suggesting more intensive and consistent usage (Veth 1993; Witter 2004).

Aboriginal artefacts are abundant at nearly all spring areas west of Bourke, while most sites to the north-east of Bourke have only scattered stone flakes. The notable exception is Tego Springs in Culgoa Floodplain National Park, where there is a high density of artefacts on a sandy ridge about 500 metres north of the springs. A grindstone and numerous stone flakes have also been found at the nearby Log Spring (Andy Coward 2012, pers. comm.). Artefact scatters are typically associated with scalded areas and claypans adjacent to springs, which partly reflects visibility in these landscapes (Robins & Swanson 2006). Especially rich and dense surface archaeology was noted on the groundwater scalds surrounding Coonbilly, Goonery (Figure 40 a), Hawkes, Kullyna, Lake Eliza (Figure 40 b) and Native Dog Springs.

At Youngerina, stone tools are scattered across the spring area but become abundant on bare areas in stony mulga immediately to the south. Stone flakes, cores and grindstones are abundant in patches along the western shore of Peery Lake, but the major concentration of artefacts, including numerous large grindstones, occurs at the base of a lunette on the eastern shore (Figure 41). This reflects proximity to the ephemeral freshwater lake, where people would congregate when the lake was filled from Paroo River floods. However, the relatively tiny but permanent springs would have provided a reliable drought refuge.

(b)

(a)

Figure 40 (a) Claypans to the east25T of Goonery Spring, typical of sites with high density of artefacts, and (b) high density of stone tools25T and intact stone hearths near springs on Lake Eliza.

Figure 41 Large grindstone on the eastern shore of Peery Lake close to Peery East spring, which was underwater in October 2012.

Aboriginal artefacts often lie alongside relics of more recent European occupation. An eroding shrubby sandhill 300 metres west of the inactive Wee Wattah spring seems to have been the focus for both Aboriginal and early European activity. Stone flakes, cores and grindstone fragments are intermingled with old wheels, wire, crockery, bottles and a half-buried cart (Figure 42-a). A similar situation exists at Yantabulla, where stone tools and European detritus including (mostly) bottles, wire and tin are strewn across the claypans on the eastern outskirts of the now-abandoned village (Figure 42b).

(b)

(a)

Figure 42 Aboriginal and European history occur together (a) on a sandhill west of Wee Wattah Spring, and (b) on the outskirts of Yantabulla.

Lack of dependable surface water was the major impediment to pastoral settlement of north-western New South Wales. Most goods travelling to Queensland arrived from Bourke, via a tenuous passage easily broken by floods or drought. During the 1860s drought, the Maitland Mercury rued that:

‘ … the only obstacle which throws Bourke from the sure line of rapid progress is the almost total absence of water northwards … stations [on the Warrego] are now turning east to Roma and other places for supplies and if the rain doesn’t come trade to the Warrego will stop.’

© Copyright, Maitland Mercury (1865)

South of Cunnamulla, the Warrego River dwindles to a mostly dry sandy channel, and there are only a couple of reliable waterholes along Cuttaburra Creek and the Paroo River. In this waterless country, the small but permanent artesian springs assumed enormous importance and were routinely declared public water reserves, even if they remained too far off of tracks to ever be used as such. Historical maps explicitly reveal the close associations between the early pastoral economy and springs. The Bourke to Hungerford route, trod by historical luminaries such as the Duracks and Henry Lawson, as well as countless unknown shearers and swaggies, follows Youngerina, Mother Nosey, Lake Eliza, Yantabulla and Warroo springs (Figure 39). With the exception of Mother Nosey and Lake Eliza, these springs are no longer active and certainly not modern tourist attractions. However, they would have been critical stopovers for thirsty stock, bullock drivers and travellers. A secondary track from Youngerina to Pullamonga and Coonbilly Springs is also depicted on the 1903 County of Irrara map (Figure 43).

Figure 43 County of Irrara sheet map (1903), depicting section of the Bourke to Hungerford route heading north-west, linking Youngarignia (Youngerina), Boongunyarrah/Culla Willalee (Mother Nosey) and Yantabulla Springs, all marked as square water reserves. The dotted line heading east from Youngarignia marks a secondary track connecting Coonbilly and Pullamonga Springs, then heading north to Thoroo Springs and the mud springs to the north-east.

Similarly, Queensland’s four-mile map series from 1913 shows the original Brewarrina–Cunnamulla road clearly following Towry, Log and Tego springs. This road no longer exists, but old wagon tracks are still visible in places. Tego Springs, the most northerly in the line, provided water for the township of Tego. A well was sunk adjacent to the main spring (Ogilvie 1912) and this water supplied a Cobb & Co station and a hotel. Although no buildings remain, the location of the main street has been marked with steel pickets based on the old town plan (Andy Coward 2012, pers. comm.). Further west, Cobb & Co stations and the requisite drinking establishments were also located at Lila and Native Dog springs.

In 1861, pioneer squatter Vincent Dowling established a station at Yantabulla, 100 metres from the most westerly spring, which he used as a base for his explorations into Queensland’s Paroo and Bulloo River country. Dowling’s homestead later became the site of the Yantabulla Hotel, and the town a centre of commerce with telegraph station, post office and Cobb & Co change station, and even a lemonade factory. The 1887 town plan shows numerous spring vents, and a Chinese garden near a large vent (Figure 44). In his evidence to the 1885 Royal Commission on Water Conservation, HA Gilliat described how about 300 cubic metres had been excavated from this vent. During this process, the blue mud issuing forth had forced the ‘old Chinaman’ out several times until he carted in a load of stones. According to the local publican, 9 or 10 shafts had been put down around town, all of which yielded salt water with the exception of a well sunk to 12 metres on a spring vent (Gilliat 1885).

Yantabulla has been abandoned since its last residents, Keith and Nora Roberts, left in the mid-1990s. A few dilapidated buildings and diffuse piles of bottles and other detritus of habitation remain (Figure 45a). The springs have not flowed since at least the 1930s (Pickard 1992). There are no obvious signs of springs along the depression running north-west to south-east as depicted in the 1887 plan, although some coolibah-lined depressions may be old springs. A shallow (two metre) abandoned well was found on a sandy rise east of town (Figure 45b). The springs that fed the Chinese garden are dry depressions with scalded rims dotted across a scalded coolibah swamp just north of town (John Stephenson 2012, pers. comm.; Figure 45c).

Figure 44 Yantabulla town plan 1887, showing streetscape and location of several springs and a Chinese garden.

| (a) | (b) |
| --- | --- |
| (c) |  |

Figure 45 (a) The last occupied buildings of the abandoned town of Yantabulla, (b) abandoned well shaft on the eastern outskirts of the town, and (c) large inactive spring vent on the western edge of a coolibah claypan north of town, which once supported a Chinese garden.

Huts and homesteads were often built near springs. Ruins in various states of disrepair were present at one-quarter of springs surveyed in the Bourke supergroup. The ruins of a cypress pine hut lie near the Tertiary Colless Spring and well east of Enngonia (Figure 46a). It is uncertain who lived in the hut or what it was used for, but there were apparently rifle slots in the walls, giving rise to its local name of the ‘bushranger’s hut’ (Brian Bambrick 2012, pers. comm.). The stone ruins of Youldoo homestead and abandoned well sit on a barren rise above the inactive and confusingly named Yourltoo Spring (Yourltoo is another abandoned homestead to the north; Figure 46b). West of Weilmoringle, stone foundations remain adjcent to a dry excavated depression known as the site of Tooloomi Spring (Figure 46c). Other homesteads, such as those at Toulby and Nulty springs, have only been abandoned in the past decade.

Woolsheds were built at Old Gerara (Figure 46d) and Pullamonga springs, and there are remains of old yards, fences and troughs at many springs. Even where such structures are absent, old bottles, wire, tin, assorted contraptions and, at Wee Wattah, a half-buried cart, testify to an early pastoral presence. Even at Bernard’s Spring, a small soak in a mulga gully on Culgoa National Park that is not marked on any historical maps and was ‘discovered’ in the 1950s, a poplar box stump has been cut with an axe and there are old horse yards nearby. Where springs are no longer active, relics of European industry remain isolated discords in an arid landscape, and a reminder of past human activity once nourished by groundwater.

(b)

(d)

(a)

(c)

Figure 46 (a) Ruins of the ‘bushranger’s hut’ at Colless Spring, Stanbert, (b) Yourltoo Spring with homestead ruins in the background, (c) stone foundations and lone kurrajong near Tooloomi Spring, Glenora, and (d) hand shears on a stump at Old Gerara, near site of old woolshed.

The Bourke springs played a central role in the discovery and development of the Basin. Springs along stock and trade routes were targeted as sites of early drilling. According to WB Henderson, Superintendent of Diamond Drills, the bore sunk at Goonery Springs on the otherwise dry Bourke–Wanaaring track eventually yielded a supply of 4550 litres a day at 24 metres. The shaft was sunk roughly 400 metres from the springs that ‘were not giving out much water when we started the bore’ (Gilliat 1885). A large flowing bore was also sunk at Native Dog Springs, a public watering place north of Bourke (Figure 47). Simultaneously, private interest sought to increase stock carrying capacities and, in 1879, drilling at Wee Wattah Spring on the Killara Station purportedly led to the first artesian bore (Percy 1906), closely followed by bores at the nearby Mulyeo Spring.

© Copyright, Kerry and Co, Sydney.

Figure 47 Glass negative, imperial plate, ‘Artesian Bore, Native Dog, Barringun District’, circa 1884–1917.

In a pattern repeated across the GAB, bores ultimately rendered springs redundant and many were completely forgotten as they dried up. Today, Peery Springs are protected as part of the Paroo–Darling National Park and recognised for their natural and cultural values, as are Towry and Tego in Culgoa Floodplain National Park in Queensland. Other springs lie mostly dry and completely forgotten, except in some cases by pastoralists and local townsfolk.

## Biological values

Only Towry, Tego and Peery springs have vents with measurable flows and support substantial wetlands. Both Tego and Towry consist of numerous vents dominated by Cyperus laevigatus on scalded plains (Figure 48a–b). One spring at Tego has a dense mound of Phragmites australis, representing a disjunct population of this mostly more mesic species, and providing habitat for echidnas and small birds. Scald endemic Sporobolus partimpatens was abundant at one vent at Tego in September 2003 and March 2004, but grasses were very dry and not seeding in October 2012. Towry Flat contains a disjunct population of groundwater specialist Trianthema sp. (Coorabulka RW Purdie 1404). Both groups of springs are underwater during floods, and the introduced mosquito fish Gambusia holbrooki has invaded the springs. Snails have been collected from the vent with Phragmites by Craig Eddie, but only old shells were observed in October 2012. Tego Springs were fenced by Queensland Parks and Wildlife in 2003, and were heavily impacted by cattle, sheep, goats and pigs before this. Towry Bore, situated about 500 metres from the springs, was controlled in 2006, but no effect on the springs has been observed (Andy Coward 2012, pers. comm.).

(b)

(a)

Figure 48 (a) Spring wetland at Tego Springs; dark green clumps are Cyperus laevigatus, and (b) one of three main vents on Towry Flat, dominated by Cyperus laevigatus with a central shallow pool.

Peery Lake contains the most biologically significant springs in the Bourke supergroup. This ephemeral freshwater lake was full in 2012, and all except 7 of the 130 vents documented by Jowett and Christie (2007) along the north-western shore were underwater (Figure 49a). Cyperus laevigatus dominates most spring wetlands, although large areas had died at exposed vents in 2012 due to pelicans roosting on these ‘islands’ (Figure 49b). In 1992, Pickard found the endangered spring endemic Eriocaulon carsonii on one mound, but by 2003 it had spread to adjacent mounds (NSW NPWS 2003). Jowett and Christie (2007) recorded E. carsonii on six mounds; we found mostly just one clump, with large populations at two springs. All six Eriocaulon springs were underwater in October 2012 and none was found at vents surveyed. The observations at the Peery Springs indicate that E. carsonii can survive prolonged flooding, and that its distribution may be dynamic within the spring group across decadal scales. Utricularia dichotoma was recorded in 18 spring wetlands (Jowett & Christie 2007) and Schoenoplectus pungens can be locally common (Bowen & Presey 1993), both representing highly disjunct western populations.

(b)

(a)

Figure 49 (a) Small Cyperus laevigatus island in Peery Lake in October 2012; a cluster of three springs with Eriocaulon carsonii recorded in 2007 are more than 50 metres to the west of this spring (towards the shore) but were submerged in October 2012, and (b) mounded vent, showing dead Cyperus laevigatus, apparently due to pelicans using the island as a roosting site.

Cyperus laevigatus is also present at Sweetwater Spring, Wapweelah bore and, unexpectedly, Scrubber Spring on a stony hill. Although this species can be locally common in bore drains in western New South Wales, it occurs in isolated disjunct populations dependent on groundwater-fed wetlands. The only other wetland species recorded at springs were the common grass Leptochloa fusca and the introduced grass Cynodon dactylon. No snails or fish were observed in any springs. With the exception of Sporobolus partimpatens at Tego and Trianthema sp. (Coorabulka RW Purdie 1404) at four springs, no scald specialists were found.

What were the biological losses due to spring extinction in western New South Wales? The type specimen of Eriocaulon carsonii (Figure 50a) was collected at Wee Wattah Spring (Figure 50b) in 1888 (NSW NPWS 2003), and is the only documented local extinction. At least 10 spring groups are or were comprised solely of mud springs, which by their nature do not support vegetated wetlands. Of the remaining 25 groups, Mulyeo and Hakes Spring bore are located within 50 kilometres of Wee Wattah in similar landscape settings, and it seems likely that their spring wetlands once supported endemic species such as Eriocaulon carsonii. Two spring groups that remain partially active—Lake Eliza and Sweetwater—would once have supported larger spring wetlands, possibly with endemic species. The large inactive spring areas of Native Dog (Figure 50c), Coonbilly (Figure 50d), Yantabulla, Youngerina (including Little Bore) and Goonery also seem likely to have supported substantial spring wetlands.

(b)

(a)

(d)

(c)

Figure 50 (a) Salt pipewort, Eriocaulon carsonii, (b) site of the now-inactive Wee Wattah spring where the type specimen was collected in 1888, (c–d) old springs in the inactive Native Dog (c) and Coonbilly (d) spring complexes, which probably supported substantial spring wetlands and possibly endemic species.

As for the Eulo group, there remain flowing bores within close proximity of high-value springs, some of which have large flows (Table 11; Figure 51). However, the current status of most bores is not known. Data on the status of bores and their flow rates need to be collected, and bores prioritised for rehabilitation accordingly.

Table 11 Flowing bores within 50 kilometres of Peery Springs; all were flowing at some stage but their current status is unknown.

| Work number (name) | Latitude | Longitude | Flow notes | GABSI status | Flow (L/sec) (year measured) |
| --- | --- | --- | --- | --- | --- |
| GW004350 | –30.88319 | 143.59597 | Flowing at some stage | Not specified | – |
| GW004354 (Momba no. 4) | –30.65958 | 143.82513 | Flowing at some stage | Not specified | 0.20 (1965) |
| GW004581 (Tongo) | –30.34736 | 143.83984 | Flowing at some stage | Not specified | 0.20 (1964) |
| GW004733 (Kallara no. 22) | –30.55763 | 144.03818 | Flowing at some stage | Not specified | – |
| GW010384 | –30.63486 | 143.92096 | Flowing at some stage | Not specified | – |
| GW010490 (Mackeon’s) | –30.52625 | 143.72179 | Flowing at some stage | Expression of interest received for funding, but project unlikely to go ahead | 1.30 (1956) |
| GW010579 (Klondyke) | –30.65152 | 143.71568 | Flowing at some stage | Expression of interest received for funding, but project unlikely to go ahead | 0.52 (1987) |
| GW010580 (Jock’s) | –30.59736 | 143.79318 | Flowing at some stage | Not specified | 0.88 (1987) |
| GW010582 (Mac’s) | –30.61625 | 143.73290 | Flowing at time of drilling | Expression of interest received for funding, but project unlikely to go ahead | 0.61 (1987) |
| GW010800 (North Tongo) | –30.43652 | 143.82790 | Flowing at some stage | Not specified | 0.10 (1961) |
| GW010804 (Middle) | –30.59986 | 143.83374 | Flowing at some stage | Not specified | 1.00 (1975) |
| GW010869 (Mt Jack) | –30.81264 | 143.66958 | Flowing at the time of drilling | Not specified | – |
| GW011198 (Palooka) | –30.66375 | 143.68402 | Flowing at the time of drilling | Expression of interest received for funding, but project unlikely to go ahead | 1.53 (1987) |
| GW013049 (Ram) | –30.62375 | 143.68624 | Flowing at some stage | Expression of interest received for funding, but project unlikely to go ahead | 0.40 (1987) |
| GW014316 (Yantabangee) | –30.53791 | 143.75124 | Flowing at some stage | Expression of interest received for funding, but project unlikely to go ahead | 2.40 (1987) |
| GW014329 (Klondyke no. 2) | –30.65652 | 143.80068 | Flowing at some stage | Not specified | – |
| GW014330 (Klondyke no. 3) | –30.66069 | 143.85652 | Flowing at some stage | Not specified | 0.10 (1960) |
| GW014331 | –30.68819 | 143.70596 | Flowing at the time of drilling | Not specified | – |
| GW014458 (Kiwi) | –30.63180 | 143.78040 | Flowing at some stage | Expression of interest received for funding, but project unlikely to go ahead | 0.12 (1987) |
| GW014705 (Coorpooka) | –30.73319 | 143.65596 | Flowing at some stage | Not specified | 0.21 (2004) |
| GW016685 | –30.56597 | 143.70652 | Flowing at the time of drilling | Not specified | – |
| GW016954 (Glasson’s) | –30.49041 | 143.74429 | Flowing at some stage | Not specified | 0.3 (1975) |
| GW019373 (Peery) | –30.67653 | 143.62430 | Flowing at some stage | Not specified | 1.60 (2008) |
| GW019977 (Klondyke no. 4) | –30.62514 | 143.70985 | Flowing at some stage | Expression of interest received for funding, but project unlikely to go ahead | 0.52 (1987) |
| GW020164 | –30.49541 | 143.93929 | Flowing at the time of drilling | Not specified | – |
| GW022750 (Top Flow) | –30.73652 | 143.66680 | Flowing at some stage | Not specified | 0.26 (1987) |
| GW022751 (Bottom Flow) | –30.84041 | 143.62346 | Flowing at some stage | Not specified | – |

– = unknown, GABSI = Great113T Artesian Basin Sustainability Initiative.

Figure 51 Flowing Great Artesian Basin (GAB) bores within 50 kilometres of Peery Springs; all were flowing for some period after drilling but their current status is unknown. Any that are still flowing should be a priority for rehabilitation.

# Bogan River supergroup

Rade (1955) considered these springs to be associated with the eastern margin of a peninsula-like basement prominence that forms the western margin of the Coonamble Basin or Embayment. Habermehl (1982) identified the Bogan River supergroup as comprising three widely spaced springs along a 150 kilometre line from Coolabah to Walgett. Pickard (1992) visited and documented Cumborah Spring, but did not visit Coolabah and Cuddie. Cuddie Spring is famous as a site of Pleistocene megafauna fossils, which were discovered when a well was sunk in the 1870s. Some of these were sent to Sydney and analysed by WB Clark, and extensive archaeological investigations commenced in 1933 (see Field 2006 for an overview of the site and research conducted). Coorigul is marked on 1:250 000 topographic maps; however, it had not been documented in previous surveys.

## Historical record

All springs except Coolabah are located on historical maps: Coorigul on the county map of Narran (1914); Cumborah on the Llanillo pastoral run survey 242 and pastoral holdings index of New South Wales map (1886); and Cuddie on the Wyabray pastoral run survey (1859) and pastoral holdings index of New South Wales map (1886). Although Coolabah Spring is not shown on maps or plans, the Mayfield property used to be called ‘The Springs’ and The Springs Tank is marked to the east of the spring on 1:250 000 topographic maps. Maps included in the reports of the Interstate Conferences on Artesian Water (1913, 1914, 1922, 1924 and 1925) and Interstate Conference on Water Conservation and Irrigation (1939) show most of these springs.

Major Thomas Mitchell passed through the area in March 1847 en route to the Balonne, and was taken to some springs on a plain south-south-east of Narran Lakes. His journal entry for 5 March reads:

‘Our guides brought us at length to some waterholes, amongst some verdant grass on a plain, where no stranger would have looked for water; and here we encamped fifteen good miles from the Barwan. The ponds were called “Carawy,” and were vitally important to us, enabling us to pass on towards the Narran, which was still, as we had been informed, twenty-five miles off. As we approached these springs, I saw some natives running off, and I sent one of the guides after them to say we should do them no harm, and beg them to stop, but he could not overtake them.’

© Copyright, Mitchell (1847)

There are no springs marked on plans or maps, no likely signatures on Google Earth, and the landholders we spoke to had no knowledge of springs in this area. It is possible that Mitchell’s use of the term ‘springs’ is loose and they were simply pools of rain or flood water, or small ephemeral soaks, or that the springs are now extinct and forgotten.

## Project survey and reassessment of spring status

Four springs were surveyed in November 2012 (Figure 52). Coorigul Spring on the Queensland – New South Wales border is an intermittent Tertiary spring, similar in appearance to Tertiary springs near Dirranbandi to the north-east. It is situated in a hollow in low stony hills where the local watertable is very close to the surface (Dick & Kate Bucknell 2012, pers. comm.; Figure 53a). These factors, as well as the water-quality data (pH 6.43, 28.0 °C, 468 microsiemens per cm, compared with a pH of 11 at the house bore water, which comes from 900 metres), suggest that this spring is emanating from a local Tertiary aquifer rather than the Great Artesian Basin (GAB).

Cumborah Springs are situated at the junction of Tertiary sandstone and the Griman Creek Formation, which is both a Basin aquifer and aquitard. Habermehl (1982) considered that they might be related to the Tertiary sediments and thus not part of the GAB. Based on geological evidence and the appearance of the springs, we interpret them as non-GAB type O springs, with downward flow through the Tertiary material discharging at the junction of the Griman Creek Formation. Water present in March 1992 and November 2012 is probably run-on water, and the vegetation is dominated by generalist wetland species (Figure 53b).

During the drought of 1877, a well was sunk to 8.4 metres in the centre of the lake at Cuddie Spring (Figure 53c), and water flowed up at a rate of 11 360 litres per day (Abbott 1881). There is broad consensus among geomorphologists that Cuddie was never a true spring with free water on the surface, at least not in recent history. This is based on the fact that the upper 3 metres of the sediments are lacustrine clays and there are no precipitate salts (Judith Field 2012, pers. comm.). However, the historical record suggests otherwise. Abbott recorded that:

‘When the country was first taken up, the cuddie was simply a bog-hole, which never dried up—always, even in the greatest droughts, affording water to the aboriginals when they scooped it out, but forming a dangerous trap for cattle … ’

© Copyright, Abbott (1881)

The district surveyor provided evidence about the site to the 1885 Royal Commission of Water Conservation:

‘It was originally a mud spring. It is in a hollow 100 yards across or more, and in the hollow a well has been sunk to a depth of 28 feet. Great difficulty was experienced in sinking the well, owing to the way the sides fell in, but timber was used to keep up the sides. The water now trickles over the top of the timber, which is several feet above the surface of the ground.’

© Copyright, Gilliat (1885)

The Wyabray pastoral run survey (1859) and pastoral holdings index of New South Wales map (1886) both show ‘Cuddie Spring’. Anderson and Fletcher (1934) reported that the spring had been inactive ever since a nearby bore had been sunk at the turn of the century. Based on this historical evidence, we consider Cuddie to have been a single isolated mud spring emanating from the Basin. Anderson and Fletcher (1934) also found bullock and horse remains 0.6 metres deep, an indication that this thickness of sediments was deposited during the past 200 years according to Watkins and Meakin (1996). These authors also refer to a stratigraphic hole drilled by the Geological Survey of New South Wales at Cuddie Springs in 1990, with Rolling Downs Group sediments encountered at 41 metres and overlain by Quaternary sands. Although Cuddie Springs and its immediate surroundings do not give the appearance of a true artesian basin spring, there is the (slight) possibility that groundwater was derived from the Pleistocene units (Trangie Formation). Alternatively, it could have been derived from the Lower Cretaceous – Jurassic Keelindi beds, the latter being the equivalent of the Mooga Sandstone, or even from the Pilliga Sandstone. This artesian groundwater could have risen to the surface along one of the many lineaments, or interpreted or intersecting faults derived from geophysical data and Landsat imagery (Figures 18–20, 22 and 23) shown in Watkins and Meakin (1996), and on the side map and geological cross-section at the Walgett SH 55-11 Geological Series map sheet 1:250 000.

The site of Coolabah Spring was marked with a steel picket by the previous lessee, who described it as a small permanent seep (Pickard 1992). In November 2012, the shallow depression on a flood-out of Turners Creek was filled with rainwater (Figure 53d). The site did not have the appearance of a GAB spring; however, scalds of extinct springs on floodplains can be easily obscured over time. The area surrounding ‘The Springs tank’ marked on topographic maps one kilometre to the east of the marked spring was also inspected, and had numerous depressions and scalds. The presence of The Springs tank and the fact that Mayfield used to be called ‘The Springs’ indicates that there were springs present in the area, either on the floodplain at the marked site or at the tank. Like Cuddie Spring, the site is situated along the margin of the Coonamble Embayment, and we consider it most likely to be an extinct type K GAB spring (see conceptual diagrams in Appendix E).

Figure 52 Springs in the Bogan River district, New South Wales. Cuddie and Coolabah are considered to be Great Artesian Basin springs based on available historical and hydrogeological evidence.

(b)

(a)

(d)

(c)

Figure 53 The four springs in the Bogan River ‘supergroup’. (a) Coorigul, a small pool in a hollow in low stony hills, (b) Cumborah, a damp depression in a hollow at the junction of Tertiary sandstone and the Griman Creek Formation, (c) Cuddie, the site of an old well sunk on an ephemeral lakebed; pickets and disturbance in the centre of the lake are from archaeological excavations, and (d) Coolabah, a muddy puddle on Turners Creek floodplain.

## Cultural history

The archaeological record at Cuddie Springs shows the presence of people in the area from about 35 000 years ago (Field et al. 2002, Trueman et al. 2005). Cuddie is one of the few springs in the Basin where an Aboriginal creation myth has survived in the historical record. According to WE Abbott, who visited in 1881, Cuddie meant ‘bad water’, although he conceded that this term may have been commonly used by Aboriginal people to describe a number of locations. While reporting on archaeological investigation at Cuddie Springs the early 1930s, Anderson and Fletcher (1934) recounted an Aboriginal legend that purportedly explained the remains of so many skeletons, as well as perhaps the connotation of ‘bad water’:

‘Long ago there was an immense gum tree, miles in height, which grew on the bank of the Geerah waterhole on the Barwon River, eleven miles north of Cuddie Springs. Near it was an Aboriginal camp, and in its branches a pair of huge eagles had built their nest, whither they used to carry the little black babies to feed their young. At last the blacks decided to get rid of these troublesome neighbours by cutting down the tree. When it was done the trunk was found to be hollow, and, in defiance of the laws of hydrostatics, the water ran along this huge pipe, emerging from the top end, where it fell at Cuddie Springs, the escaping water making a deep hollow in which were deposited the bones of the animals which had formed the food of the eaglets. So, according to the legend, did Cuddie Springs come into being, and this is the origin of the bones found there.’

© Copyright, Anderson and Fletcher (1934)

Coorigul Spring also features in Aboriginal mythology, as the start of a dreaming track following the Bogan River. This story has been recorded by anthropologists and is painted as a mural at a Lightning Ridge service station. There is a stone arrangement on the slope above the spring. It was obscured by tall grass at the time of our visit, but Kate Bucknell described it as consisting of large stones arranged in wav39Ty39T lines (pers. comm., 2012). There are stone flakes scattered throughout the area. Traditional owners continue to visit the site. Stone flakes are also scattered in the hollow containing Cumborah Springs, but no artefacts were seen on the cracking clay floodplain around Coolabah Spring.

Mitchell’s description of the mysterious Caraway Springs on 6 March 1847 suggests it was a major habitation site for the local people, and sounds a prescient warning about the fate that was to befall inland springs:

‘I found near the ponds, several huts made of fresh branches of trees and the remains of fires, doubtless the deserted home of the fugitives of yesterday. At these fires I found the roasted pods of the acacia already mentioned (Munumula) [Acacia stenophylla]. The water was surrounded by fresh herbage, and such was the simple fare of those Aborigines, such the home whence they fled. As I looked at it in the presence of my sable guides, I could not but reflect that the white man’s cattle would soon trample these holes into a quagmire of mud, and destroy the surrounding verdure and pleasant freshness for ever. I feared that my good-natured but acute guides thought as much, and I blushed inwardly for our pallid race.’

© Copyright, Mitchell (1847)

Cumborah Well is situated adjacent to the springs and continues to provide water for the village of Cumborah. The ruins of an old cottage, possibly of a boundary rider, lie near Coorigul Spring. The old well at Cuddie Spring was not located. Based on Abbott’s description, it was situated in the middle of the lake close to where the most recent archaeological excavations have been conducted. About 100 metres away, on the edge of the claypan, stand the remains of an old timber tankstand and a line of 20-centimetre-high timber posts running towards the centre of the claypan for about 50 metres. This probably represents the remnants of old troughing and piping that carried water for stock to firmer ground (Abbott 1881).

## Biological values

No spring endemics or listed species were recorded at the four springs visited. Plant species present were generalist wetland or terrestrial species, including Alternanthera angustifolia, Bothriochloa decipiens, Centipeda crateriformis, Eleocharis plana, Fuirena ciliaris, Glossostigma diandrum, Juncus aridicola, Juncus polyanthemos, Malvastrum americanum, Nymphoides crenata and Schoenus apogon; and the introduced species Anagallis arvensis, Aster subulatus, Cynodon dactylon, Echinochloa colona, Juncus bufonius, Soliva anthemifolia and Xanthium occidentale. No fish or snails were found in any springs.

# Conclusions and recommendations

All except eight known Great Artesian Basin (GAB) spring groups in the Eulo, Bourke and Bogan River supergroups, and nearly all priority springs in the Springsure group, have now been surveyed and historical and biological values documented. Cuddie and Coolabah springs now stand alone in the Bogan River supergroup. They lie only 120 kilometres south-east of the nearest GAB spring (Thully) in the Bourke supergroup and could be regarded as outliers of this group. The springs on the Paroo Overflow are almost 100 kilometres south of Goonery and separated by a large expanse of Quaternary dunefields (Figure 34 in Section 4.2). A case could be made for placing them in a new supergroup. Conversely, the Eulo springs and the line from north-west of Fords Bridge are close to being one supergroup, especially as both are related to granitic basement structures. Nevertheless, any groupings in southern Queensland and northern NSW will remain somewhat arbitrary, and we have persisted with the supergroup domains defined by Habermehl (1982).

Although many wetlands, populations and most likely some species have been lost as a result of artesian aquifer pressure drawdown and local disturbances, especially in the Eulo supergroup, the active springs continue to provide habitat for endemic species and disjunct populations. It is critically important to preserve the springs that remain. Preservation and conservation effort must be directed to the nine spring groups south-west of Eulo that still support endemic species (Table 7 in Section 3.4) and Peery Lake in New South Wales. The remaining active springs may be candidates for rehabilitation in the future, but are not a priority for action until the high-value springs have been secured.

The Great Artesian Basin Sustainability Initiative artesian bore rehabilitation program has made considerable progress, but massive flowing bores remain very close to high-value springs, including the Yowah Creek group (Table 8 in Section 3.4). There is an urgent imperative to rehabilitate the bores that may be critical 39Tf39Tor the preservation of the last remaining springs.

Other strategies must be site specific, but will include fencing, feral animal control and tenured protection agreements (Table 12). Priority sites for fencing are Masseys and Little Granite on Granite Springs, Wooregym and Tunga on Bingara Springs, and the southern group in the Dead Sea on Penaroo Springs. Other sites with endemics are situated on floodplains where fencing is not practical or are not heavily impacted by herbivores. The desirability and practicalities of fencing need to be negotiated with landholders for individual sites. In most cases, fencing to exclude both stock and feral animals is desirable despite the extra cost and maintenance required, given the high densities of goats and pigs. Fatchen (2000) has reviewed the effects of fencing on the natural values of springs and clearly demonstrates results peculiar to individual sites. The consequences of fencing should be monitored to determine the effects. A gate in a fence provides flexibility for future management.

Table 12 Conservation recommendations for high-value springs.

| Spring group | Fencing (feral and stock-proof) | Nature refuge | Feral animal control |
| --- | --- | --- | --- |
| Yowah Creek | Impractical | High priority | Pigs |
| Paroo River | Impractical, as in creek channels | On Town common | Stock are more of an issue than ferals |
| Masseys (Horseshoe) | High priority | Priority |  |
| Tunga | High priority, but would be challenging due to adjacent creekline and run-off from surrounding hills | Priority | Goats and pigs |
| Dead Sea (including Dead Sea South and Little Bundoona) | High priority, but would be a very large fence | Existing | Goats and pigs |
| Little Granite | High priority | Priority | Goats and pigs |
| Merimo (Site 238) | Impracticable, as in creek channels | Priority | Pigs |
| Wooregym | High priority | Priority | Goats and pigs |
| Tungalla | Lower priority, as has been excavated in past and is not as heavily impacted by herbivores | Lower priority, as excavated in past | Pigs |

Notes:
Bore rehabilitation applies to all springs. 
Yowah Creek, Paroo River, Masseys and Tunga springs all contain species not known from any other locations; all other springs contain spring endemics.

The high-value springs Yowah Creek, Paroo River, Masseys and Tunga are located on four properties or amalgamations, currently owned by three landholders. All value the springs and are sympathetic to conservation efforts. However, secure tenure agreements will protect against future destruction should the properties change hands. In Queensland, Nature Refuge Conservation Agreements are signed between landholders and the state. These agreements may allow for continued production and land management activities such as sustainable grazing and water use, but prohibit further excavation, the introduction of exotic species to the springs and groundwater extraction that will impact on spring flows. It is possible for extension officers to undertake property assessments, negotiation of the conservation agreement, and provide follow-up advice and assistance with management. The Dead Sea Springs on Penaroo already are included under a Nature Refuge Conservation Agreement.

A bore swap initiative that allowed for a 100 per cent government subsidy for bore rehabilitation and regulation on properties with high-value springs in exchange for secured agreements on the springs has been partially successful. Similar initiatives could be developed to complete bore rehabilitation and regulation in the vicinity of high-value springs.

There is a responsibility to ensure that demands for groundwater, whether it be for its own sake or as a by-product of another resource, do not cause further degradation to the already-debilitated GAB springs. Rather, there are opportunities through coordinated and targeted restoration to enhance the cultural and biological values of the last remaining springs. Priorities for future action are:

ensure restoration of artesian aquifer pressures in areas containing high-value artesian springs, particularly in the Eulo and Bourke supergroups

enhance groundwater modelling capabilities for the prediction of impacts of coal seam gas extraction on artesian springs, including timing, extent, duration and possible eventual recovery in the Springsure supergroup

complete the database for springs throughout the GAB to further refine priority action

develop tenure-secure agreements to implement best-practice management at important spring locations.

## Acknowledgements

The hospitality and assistance of all landholders, managers and rangers in Queensland and New South Wales is gratefully acknowledged: Brian Bambrick (Stanbert), Kerry and Cally Hayes (Ellavale), Patrick Gain (Mascot), JD and Kaz Davies (Glenora), Andy Coward (Culgoa Floodplain National Park), Shirley Meyer (New South Wales National Parks, White Cliffs), Dick and Kate Bucknell (Calooma/The Springs), Paul and Adele Friend (Waratah), Glen Knight (Bundy), Bully Hayes (Box Vale), Justin and Julie McClure (Killara), John Dunk and John Stephenson (Warroo), Lorraine Lewis (Springvale/Lochangar), Dan Munster (Congarrara), Bruce and Christine Sharpe (Comeroo/Maranoa), Mark Pritchard (Meulleyarra/Thoura), Leon Zanker (Laurel Vale), Jan Currey and the Johnstones (Cuddie Spring), Graeme Pfitzner (Werewilka/Granite Springs), Dick O’Connell (Wombula), James Hatch (Wombah), Shane, Peta, Gordon and Tim Warner (Merimo/Bingara), Reb and Matt Price and family (Wittenburra), Ken Looker (Karto), David and Carmel Meurant (Wandilla), Rupert and Nikki Schmidt (Mooning), Andrew and Jane Campbell (Pitherty), Greg and Tony Sherwin (Kilcowera), Mick Beresford (Farnham Plains), John and Cherry Gardiner (Garrawin), Ian and Kerry Canning (Ningaling), Jake Berghofer (Springvale), Ross Campbell (Waihora), Bora and Sally Schmidt (Baroona), Randall and Cheryl Newsham (Bundoona), Carly Atkins (Coorada), Kelvin Hahn (Stonecroft), Mark Driscoll (Glen Elgin) and Carlin Burns (Blackdown Tableland National Park).

Eric and Kate Beresford, Ian Pike and Dan McKellar shared their knowledge of springs in the Eulo area. Dan and Shiralee McKellar, and Moc and Sheree Parker are thanked for their interest and hospitality during the Eulo surveys. Steve Peck from Charleville National Parks, accompanied by Joel d’Souza, assisted with surveys and vehicle recovery at some springs in the Eulo group. John Pickard’s work on the New South Wales springs was invaluable and is gratefully acknowledged. Tracy Wattz provided photos of Poached Egg Spring and its eponymous daisy, and patiently assisted in trip preparation for the Eulo springs.

# Part 2—Hydrogeological survey of the Great Artesian Basin springs in the Springsure, Eulo, Bourke and Bogan River supergroups

Alexandra WolhuterP1P, Kimberly HinesP2P, Sarah RobbinsP3P, Sue VinkP1P and Joan EsterleP2

P1 PCentre for Water in the Minerals Industry, Sustainable Minerals Institute, University of Queensland

P2 PSchool of Earth Sciences, University of Queensland

P3 PSchool of Civil Engineering, University of Queensland

# Introduction to the springs of the Great Artesian Basin

Great Artesian Basin (GAB) springs have been divided into 12 regional supergroups (Habermehl 1982, Fairfax & Fensham 2003). They are generally present along the margins of the GAB, and occur in Queensland, New South Wales and South Australia (Figure 54).

The Basin’s springs can be classified as either recharge or discharge springs. Recharge springs are supplied by groundwater from an aquifer or aquifers that are unconfined in the spring region, and occur in the recharge areas of the GAB. Water drains out of the aquifer either by the saturated aquifer intersecting with the surface or through the force of gravity. Recharge springs occur along the eastern margin of the Basin, where the aquifers are outcropping.

Discharge springs are fed by groundwater welling upwards under artesian pressure through thin confining beds or along faults. These springs occur near the western and southern margins of the GAB, but some are also present in and near the eastern margin, such as some in the Springsure supergroup. Artesian groundwater residence times for discharge springs are much longer than for recharge springs and, as a result of a longer time in contact with aquifer sediments, the water has high levels of dissolved solids and is more alkaline. Discharge springs form either water or mud springs. Mud springs are formed where artesian groundwater collects and mixes with fine sediments from the overlying aquitards, so that a mud slurry comes out of the spring vent, creating a mound on selected springs. Clear water emanates from the vents of water springs, sometimes creating pools of water that can support vegetation (Habermehl 1982).

These two basic spring types (discharge and recharge) can be further refined into 15 different conceptual models (Appendix E), which describe the mechanism by which the groundwater reaches the surface. Appendix E sets out diagrams depicting each of these conceptual models. All of the 15 conceptual models are represented in the GAB. Springs types shown within nine of the conceptual models (C–K) are discharge springs protected under the Environment Protection and Biodiversity Conservation Act 1999 (Cwlth) (EPBC Act) and are descriptive of the springs investigated in this report, with the exception of the Coorigul and Cumborah supergroups.

Conceptual models D–F describe artesian discharge springs related to a fault or faults (E and F), or anticline or monocline (D), allowing artesian groundwater from a confined aquifer or aquifers to reach the ground surface. Models C, G and H describe situations where artesian water from a confined aquifer is able to flow to the surface through confining beds that are relatively thin since deposition or have been thinned by erosion (C); or are thin as the sedimentary units, including the aquifers and aquitards that covered basement rocks, were compacted (G and H). Springs emanating from the contact between outcropping basement sediments and onlapping sediments from the aquifer or multiple aquifers are described in conceptual models I and J. Model I shows an artesian spring fed from the top aquifer; model J shows an artesian spring being fed from a mixture of artesian groundwater from the lower and upper confined aquifers, or along the contact with the basement rocks. Lastly, conceptual model K describes the situation where spring water emanates from an artesian discharge spring located at the limit of the confining bed at the western or southern edges of the GAB, though most artesian springs in these areas are associated with faults and basement abutments.

The volume of water discharged by GAB recharge springs is highly variable. Springs located next to one another, with the same likely source aquifer can have very different flow rates, probably depending on the ease of or resistance to vertical throughflow of artesian groundwater through the confining beds. Most springs produce less than 10 litres per second (L/s); however, some springs have been recorded to have flow rates of up to 85 L/s and and as large as 150 L/s (Habermehl 1982). In 1982, the estimated combined discharge from 600 Basin springs amounted to 1500 L/s, about 5 per cent of modelled GAB recharge (Habermehl 1982).

Historical exploitation of GAB aquifers has, however, greatly reduced spring discharge and many springs have become extinct. For example, springs in the Eulo supergroup currently discharge one-third of their early 20th century volumes, and 62 per cent of springs have become inactive. A regime of rehabilitation of bores and regulating flow, such as capping flowing bores and improvements in the efficiency of water usage, has been implemented through the Great Artesian Basin Sustainability Initiative to address the dramatic decline in spring flow caused by the drawdown of source artesian aquifers (Fairfax & Fensham 2003).

Potential new threats to springs may, however, be posed by the coal seam gas industry operating in the Basin. This report investigates the hydrogeology of the GAB springs to assess whether the coal seam gas industry and coalmining pose threats to some of the GAB springs, and what those threats may be.

This report provides an overview for high-value springs in the Springsure supergroup—they have been investigated in great depth by the Queensland Water Commission (QWC 2012a). This part 2, in conjunction with part 1 of the Ecological and hydrogeological survey of the Great Artesian Basin springs—Springsure, Eulo, Bourke and Bogan River supergroups, Volume 1: history, ecology and hydrogeology provides the most in-depth investigation for the Springsure, Bourke, Eulo and Bogan River supergroups to date.

Figure 54 Sedimentary basins and artesian springs of the Springsure, Eulo, Bourke and Bogan River supergroups.

## Project aim and scope

Through the collation and interpretation of regional geological, hydrogeological and water-quality information, this report aims to:

identify the most likely source aquifers for springs in the Springsure, Eulo, Bourke and Bogan River supergroups that are listed in the EPBC Act

classify the investigated springs into conceptual spring types

assess the potential threats to the springs posed by coal seam gas development.

To limit the scope of this project, we only investigated the springs that are EPBC Act–listed and were ranked with a high conservation status in an earlier survey (Fensham et al. 2010) or by the survey conducted in Part 1 (Table 13). For the Springsure and Eulo supergroups, springs with a conservation ranking of 1a or 1b as determined according to Fensham et al. (2010) were investigated. As there were few springs with a high-value ranking in Bourke, springs with a conservation ranking of 1a—4a were investigated. All Bogan River springs are included in this report. For each spring complex investigated, a hydrogeological profile of the area has been compiled and is detailed in Volume 2 of this report. A summary of the major hydrogeological findings for each supergroup is provided in chapters 10–13.

Table 13 Spring complexes investigated in this report.

| Supergroup | Spring complex |
| --- | --- |
| Bogan River | Coorigul, Cumborah, Cuddie and Coolabah |
| Bourke | Peery Lakes, Sweetwater, Thooro Mud, Tego and Towry |
| Eulo | Yowah Creek, Dead Sea, Carpet, Eulo Town, Merimo, Granite, Tunga and Wooregym |
| Springsure | Lucky Last and Abyss, Scott’s Creek, Cockatoo Creek, Boggomoss, Dawson River 6, Dawson River 2, Dawson River 8 |

## Methodology

To determine the source aquifer for the springs, geological, hydrogeological and hydrochemical methods were used, as defined in Parsons Brinkerhoff Australia’s report for the Queensland Department of Environmental and Resource Management (DERM 2010):

Geological methods. Develop an understanding of the geology in the area using surface geological maps, subsurface geometry of formations, geological structure information, bore locations and bore logs (including driller’s logs, geologists’ interpretations and lithology logs), and stratigraphic elevations.

Hydrogeological methods. Use hydrogeological information to improve the understanding of the geology, including hydrogeological maps, groundwater and potentiometric level for waterbores that had been adequately identified, spring elevation, spring flow information, historic rainfall records, and temperature of spring and aquifer waters.

Hydrochemical methods. Use hydrochemical information to identify the source aquifer of the spring. These data include physiochemical parameters, major ion chemistry, minor elements chemistry, isotope geochemistry, as well as geothermometer data and anthropogenic substances if required and where available.

The above methods defined by Parsons Brinkerhoff build upon work undertaken to identify source springs by Environmental Hydrology Associates (EHA 2009), where the source aquifers were assessed for a limited selection of spring complexes within the Springsure and Eulo supergroups.

### Geological methods

Geological data for the area surrounding each spring complex were collated, including information on:

Surface lithology. For example, the 1:250 000 map sheet series and regional geological features, such as mapped faults and other geological boundaries obtained from Queensland Interactive Resources and Tenure Maps, and the New South Wales Department of Trade & Investment and Geoscience Australia. Literature on the geology of the Basin was also used.

Groundwater bore locations and stratigraphic logs obtained from the DERM Groundwater Bore Database (GWDB) for Queensland bores (DERM 2012) and PINNEENA for New South Wales bores (NSW OoW 2010), as well as geophysical logs of waterbores by Habermehl (2001).

Stratigraphic boreholes obtained from Queensland Interactive Resources and Tenure Maps.

Spring vent locations obtained during field surveys of the springs as reported in Part 1.

This information was also used to map each spring complex using ArcGIS 10.1. The understanding of the regional geology was applied to identify aquifers within the region of the target spring complex.

### Hydrogeological methods

Published literature on regional potentiometric elevations, for potential source aquifers and hydrogeology of the GAB, was then used to refine the understanding of the hydrogeology, identify potential source aquifers for the investigated spring complex and classify it into a conceptual spring type.

Information on potentiometric elevations for GAB aquifers was obtained from Quarantotto (1989) and the Queensland Department of Natural Resources (DNR 1996). These data were only available for the Springsure and Eulo region. Current and historical waterbore standing water levels and flow rates from the DERM GWDB or PINNEENA were used to identify artesian aquifers where potentiometric information was not available, or as an additional source of information.

### Hydrochemical methods

The physicochemical parameters of water samples taken from the springs were compared with waterbore hydrochemical data available in the DERM GWDB and from QWC (2012a). Major ion composition of spring water and waterbores were also compared. This information was combined with casing information for waterbores to try and identify source aquifers.

Spring water chemistry data for the Springsure supergroup was obtained from the QWC (2012a), fieldwork conducted in 2011, and DNR (1996). Data from EHA 2009 and the GAB Springs Database was used for the Eulo springs. Only basic physicochemical information was available for springs in the Bourke and Bogan River supergroups and waterbores in New South Wales. This was collected during the spring surveys undertaken in 2012 for Part 1 of this report, which were conducted at the same time as the desktop assessments. As a result, the process for identifying potential source aquifers for the Bourke and Bogan River supergroups was largely limited to geological and hydrogeological methods.

Minor element chemistry and isotope geochemistry data were only occasionally available for springs and waterbores.

### Synthesis of methods

The results and outputs from the geological, hydrogeological and hydrochemical methods were combined into a report for each spring complex (see Volume 2). The results were then compared with those obtained by EHA (2009) and the information provided by the QWC (2012a) report to identify any inconsistencies, and ensure the work would be compatible with and add to previous work on the spring complexes.

By combining the results from each method, it was possible to identify potential source aquifers for each of the spring complexes. Data gaps and ambiguities could be identified that impact on the ability of this methodology to accurately identify a spring’s source aquifer.

Unfortunately, some key data on waterbores were missing for bores at the Eulo, Bourke and Bogan River supergroups. At the Eulo supergroup, there are little data on bore screening. For example, for many of the bores within 10 kilometres of the spring complexes investigated in this report, only information on steel piping is provided, with no data on where or whether there is screening at different depths, or whether the bore taps a single aquifer at the bottom of the pipe. In addition, often no information on which aquifers are contributing to the bore is provided. There are also a number of bores that source water from multiple aquifers. This made it difficult to determine which aquifer/s were the source of the spring, as one could not presume that bores without screening information were only tapping one aquifer. For the Bourke and Bogan River supergroups, the ability to identify a source aquifer was limited, due to the lack of interpreted stratigraphy and lack of bore water chemistry data. Where information was lacking, best estimates were made using the available data, and uncertainties were flagged.

# Hydrogeological overview of the Great Artesian Basin

The Great Artesian Basin (GAB) is an extensive and complex groundwater system underlying about one-fifth of Australia. A long-accepted view of the GAB is that it is a single, large, contiguous groundwater flow system in which aquifers are considered to be laterally continuous across the extent of the entire GAB. However, this view does not adequately reflect what is now understood about the hydrogeological complexity that governs groundwater movement in the GAB. The findings of the GAB water resources assessment (Smerdon et al. 2012) confirm that the GAB is not a simple, laterally continous aquifer. Key geological complexities include multiple layers with varying groundwater flow rates, connections with overlying and underlying geological basins, and the presence of faults. In general, the aquifer sediments outcrop as sandstone and siltstone along its eastern margin where they are recharged by rainwater (Habermehl 1980, Habermehl 1982, Radke et al. 2000) and, to a lesser extent, along the western margin where they outcrop. Groundwater generally flows from the eastern margin towards the southern, western and northern margins of the Basin, and from the western margin towards the south-west (Figure 55). Some artesian groundwater in the GAB is more than one million years old and, generally, groundwater recharge rates and travel or residence times are reasonably well understood (Habermehl 2001).

© Copyright, Habermehl (1982), Fensham & Fairfax (2003).

Figure 55 Map of the Great Artesian Basin. The shaded grey area indicates the eastern recharge areas. Dominant flow directions are indicated by arrows. Spring supergroups are represented by dotted lines.

## Geological basins

The GAB comprises the Eromanga, Surat and Carpentaria basins, as well as parts of the underlying Bowen and Gallilee basins (Figure 54 in Chapter 7). The Euroka Arch separates the Eromanga and Carpentaria basins, and the Nebine Ridge in Queensland separates the Eromanga Basin and northern part of the Surat Basin (Habermehl 1980). The basins formed in a downwarp of continental crust during Jurassic times and west of a volcanic system during the Cretaceous. The volcanic system was located offshore of the present coastline, on the eastern Australian Plate boundary. The basins accumulated continental sediments during the Jurassic and Early Cretaceous, and marine and continental sediments later during the Cretaceous (Shaw 1990).

The constituent sedimentary basins of the GAB are continuous across the shallow basement ridges and platforms of older sedimentary, metamorphic and igneous rocks. The basin consists of aquifers made up of sheet-like quartzose sandstone deposits. These aquifers are of mostly continental origin from the Triassic, Jurassic and Cretaceous ages, and alternate with confining beds of siltstone and mudstone with low permeability. The aquifers of the GAB are bound by the Winton Formation at the top and the Rewan Group at the bottom. Not all sequences are present across the whole basin (Habermehl 1980, 2001; see also Figure 56).

The Eromanga and Surat basin sediments originate from many large-scale cycles of sediment deposition by rivers and streams (fluvial), and by lake and river environments (fluviolacustrine). Fluvial deposits created widespread sandstone units that make up important aquifers, including the Wyandra Sandstone Member of the Cadna-owie Formation, and the Hooray, Adori, Hutton and Precipice sandstones and their stratigraphic equivalents. Units of fluviolacustrine origin are generally aquitards made up of siltstone, mudstone and fine-grained sandstone. They include the lower parts of the Cadna-owie Formation, large parts of the Wallumbilla Formation as, well as the Westbourne, Birkhead and Evergreen formations (Habermehl 2001).

## Aquifers and aquitards of the Great Artesian Basin

The Clematis, Precipice, Hutton, Adori and Hooray sandstones; the Wyandra Sandstone Member of the Cadna-owie Formation; and the Winton–Mackunda Formations and their equivalents contain aquifers (Habermehl 1980, 2001; Figure 56). Most artesian waterbores in the GAB tap these aquifers (Habermehl 2001), with most flowing artesian waterbores tapping the Jurassic – Lower Cretaceous aquifers (Hooray Sandstone and Wyandra Sandstone Member). The Mackunda and Winton formations form the shallowest aquifers of the Cretaceous Rolling Downs Groups. Figure 56 shows the basic hydrostratigraphic sequence of the GAB, including the Eromanga Basin and Surat Basin sequence, and the stratigraphic equivalents in the Coonamble Embayment of the Surat Basin in New South Wales.

Major confining beds include the Rewan Group, Moolayember, Evergreen, Birkhead, Westbourne, Wallumbilla and Toolebuc formations; and Allaru Mudstone and their equivalents. Parts of the Mackunda and Winton formations also consist of aquitards (Habermehl 1980, 2001).

The aquifers are continuous across large extents of the GAB. Faults locally displace or disconnect aquifers, and obstruct part or all of the groundwater flow in the main Lower Cretaceous – Jurassic aquifers. Faults can act as permeable or impermeable barriers to groundwater flow or to hydrocarbons migrating in the sandstones (Radke et al. 2000).

Before exploitation of the aquifers began in around 1880, the potentiometric surface of the artesian aquifers in the Lower Cretaceous and Jurassic sequence (e.g. Hooray Sandstone and equivalents) was above the ground surface across almost the entire basin (Habermehl 1980, 1982). Discharge from waterbores caused the regional potentiometric surface of the exploited aquifers to drop dramatically in heavily developed areas. In some areas, near the margins and in the south-east to central part of the Basin, the potentiometric surface has fallen below the ground surface (Habermehl 1980, 2001).

Rainfall along the western and part of the eastern slopes of the Great Dividing Range provides recharge to the GAB aquifers. Most of the Jurassic and Cretaceous sandstone aquifers outcrop on the eastern margin, allowing rainwater to enter directly or to flow into the aquifer through overlying sandy surface sediments (Habermehl 1980, 2001). Recharge also occurs along the western marginal zone, where aquifer sandstones are exposed or subcrop. Drainage systems such as the Finke River contribute substantial seasonal recharge within this arid zone. Discharge occurs in the southern, south-western and north-western regions of the GAB (see Figure 55).

cr = creek, fm = formation, gp = group, mbr = member, mdst = mudstone, NSW = New South Wales, NT = Northern Territory, Qld = Queensland, SA = South Australia, sst = sandstone.   © Copyright, Habermehl (1980), Radke et al. (2000).

Figure 56 Hydrostratigraphic sequence of the Great Artesian Basin

# Hydrochemistry of the Great Artesian Basin

The range of recharge mechanisms and environmental processes within the Great Artesian Basin (GAB) means that the basin’s hydrochemistry is not homogeneous, both among the constituent sedimentary basins, and within regional recharge zones and within the aquifers throughout the Basin (Herczeg et al. 1991, Radke et al. 2000). Hydrochemical change occurs during water infiltration into an aquifer and within the aquifers. As water flows through the aquifer rocks, ions in the water react with the rocks. The chemical characteristics of spring water reflect the characteristics of the aquifer or aquifers from which the water originates.

Regions and individual aquifers can broadly classify groundwater within the GAB. Hydrochemical differences delineate the eastern flow system from the western flow system. The westward flowing water is of a Na-HCOR3R-Cl type. The eastward-flowing water is of the Na-Cl-SOR4R type (Habermehl 1980, 2002, Herczeg et al. 1991). Artesian groundwater in the eastern and central parts of the GAB flows westward (Na-HCOR3R-Cl type waters). Carbon dioxide dissolves into water at the eastern marginal recharge zone as rainwater flows through near-surface soil. Water interactions with silicate and carbonate minerals in rock results in ion exchange. This contributes to elevated calcium and magnesium ion concentrations, and elevated alkalinity levels (Herczeg et al. 1991).

As the groundwater flows towards the Basin, sodium cations associated with the aquifer sediments are exchanged with calcium and magnesium cations in the water. The removal of calcium and magnesium from the groundwater by exchange with sodium in clay minerals within the aquifer framework may also cause hydrogen ions to be released. This further promotes carbonate dissolution (Herczeg et al. 1991, Radke et al. 2000). Further, diffusion of sodium and chlorine ions, and some calcium, magnesium and sulfate ions from overlying aquitards increases total ion concentration and influences the groundwater type.

Through the ion-exchange processes described above, the chemistry of the groundwater evolves from Ca-Mg-HCOR3R-Cl groundwater towards Na-HCOR3R-Cl. Na-HCOR3R-Cl type water is characterised by higher alkalinity and sodium dominance. It cannot, however, be confirmed that Ca-Mg-HCOR3R-Cl groundwater is a precursor of the Na-HCOR3R-Cl groundwater (Radke et al. 2000). This is because groundwater mixing and evolution make it difficult to accurately model ion exchange.

In the southwest part of the Basin, the eastward-flowing groundwater is characterised by Na-Cl-SOR4R- and Na-Ca-Cl-SOR4R-type waters. These higher calcium and sulfite concentrations are acquired from recharge and, possibly, from diffusion from overlying mudstones (Habermehl 1980, 2001, Radke et al. 2000).

## Salinity

Typically, deeper Jurassic–Cretaceous or Jurassic aquifers have lower salinity (as measured by electrical conductivity) levels than younger Cretaceous aquifers (Habermeh 1980, 2001, Radke et al. 2000, EHA 2009). Salinity also varies both spatially and temporally within the same aquifer. Areas closer to the recharge source are generally fresher than deeper sections of the same aquifer. The general trend in the main Lower Cretaceous – Jurassic aquifer (Hooray Sandstone and Wyandra Sandstone Member of the Cadna-owie Formation aquifer system) is for salinity to increase down the potentiometric gradient from the recharge zones. Areas of deeper aquifer burial are characterised by relatively higher salinity groundwater. This trend may also be explained by mixing of fresher recharge waters with saline waters in deeper parts of the Basin, along with the dissolution carbonates and ion exchange with clay minerals into the groundwater along the flow path (Herczeg et. al. 1991). The differences in salinity between aquifers and the change in salinity levels along flow paths make salinity one of the indicators for determining the source aquifer for GAB springs.

## Total dissolved solids

Total dissolved solids (TDSs) in the Lower Cretaceous – Jurassic aquifer ranges from less than 200 milligrams per litre (mg/L) to more than 10 000 mg/L, with higher TDS values in the deeper parts of the GAB. Generally, TDS ranges between 500 mg/L and 1500 mg/L in most of the GAB (Habermehl 1980, 2001). It is lowest in the Pilliga region of the Coonamble Embayment, the recharge zones of the northern Eromanga Basin and the northern part of the Carpentaria Basin. TDS is highly variable in the eastern Eromanga Basin recharge zone. It is consistently higher in all other recharge areas, especially in the western Eromanga Basin, which exceeds 1500 mg/L, though near the Finke River region salinities decrease to 400 mg/L (Radke et al. 2000).

TDS values of up to 10 000 mg/L can be found in the central Eromanga Depocentre and its southward extension. The central Eromanga Depocentre is separated from the western Eromanga Basin flow system by an arcuate zone of relatively lower TDS. The western Eromanga Basin flow system extends from the eastern recharge zone to the southern discharge zone, and around the northern and western margins of the Eromanga Depocentre. It is characterised by an increase in TDS from 300 mg/L to 1300 mg/L. Groundwaters have a TDS range of 300–1000 mg/L from the Cheepie Shelf and across the Eulo–Nebine Ridge area. Eastwards into the western Surat Basin and south in the Coonamble Embayment, TDS ranges from 600 mg/L to1000 mg/L; TDS is higher (up to 2000 mg/L) in the eastern Surat Basin (Radke et al. 2000).

## Temperature and pH

Temperature can indicate the origin of discharged waters. The slow flow rate of spring water from the aquifer to the surface means that spring water temperature will usually be lower than groundwater temperatures from bores that tap the same aquifer. If spring water temperatures are significantly higher than groundwater temperature measured in nearby bores, then it is likely that the spring is sourced from a deeper aquifer than that of the bore (EHA 2009).

Surface temperatures for groundwater bores that tap the Lower Cretaceous – Jurassic sequence usually range from 30 °C to 100 °C. Springs usually have temperatures around 20–45 °C. Groundwaters near basin margins have temperatures of up to 40 °C. Groundwaters in deeper parts of the GAB have warmer temperatures, between 60 °C and 100 °C, but these temperatures can also be found at shallower depths, as the aquifer and groundwater temperatures are determined by the local geothermal gradients (Habermehl 2001).

Groundwater pH measurements can be used to compare groundwater chemistry, but the accuracy of pH values for comparison are not always reliable, as pH is easily affected by environmental and physical changes (Parsons Brinckerhoff Australia Pty Ltd 2010). Confined aquifers from the Lower Cretaceous – Jurassic usually contain groundwater with pH values ranging between 7.5 and 8.5 (Habermehl 2001, Radke et al. 2000).

## Ion composition

The concentrations and ratios of major and minor ions are a good source of information for identifying source aquifers. Major ions include sodium, calcium, magnesium, potassium, chloride, bicarbonate, carbonate and sulfate. The relative concentrations of these ions are used to indicate the type of water and the possible flow pattern.

Chloride is a reliable indicator of influences from recharge, and subsequent diffusion and mixing processes. This is due to its low reactivity in water and rock (Radke et al. 2000). The main Lower Cretaceous – Jurassic aquifer (Hooray Sandstone and Wyandra SandstoneMember of the Cadna-owie Formation) has chloride concentrations ranging from 25 mg/L to more than 10 000 mg/L. The normal trend of a steady increase in chloride concentration down potentiometric gradients is only apparent in the northern Eromanga Basin and Coonamble Embayment. Groundwater adjacent to the Eulo supergroup springs and within the Pilliga region of the eastern Coonamble Embayment is predominantly less than 200 mg/L, with regions of significantly lower chloride concentrations (Radke et al. 2000).

## Alkalinity

Alkalinity is also a reliable indicator of groundwater evolution in confined flow systems. Total alkalinity has distinctive Basin-wide trends. Most recharge zones have waters with low alkalinity (less than 200 mg/L). Alkalinity increases down the potentiometric gradient away from the recharge zones and into the confined aquifer.

## Environmental isotopes data

Isotopes data, both stable and radioactive, can also be used to determine the origin and age of groundwater, as seen in Table 14.

Table 14 Isotopes used to identify groundwater origins.

| Isotope type | Isotope | Description |
| --- | --- | --- |
| Stable | Oxygen-18 and deuterium | Affected by meteorological processes that provide a fingerprint of their origin. Used to infer recharge conditions, calculate mixing between waters, interpret palaeoclimatic conditions and assess evaporation. |
| Stable | Carbon-13 | Interpretation of geochemical and microbiological processes in groundwater. Used as tracers of discharge contributions of different flow paths to streams. |
| Radiogenic | Carbon-14 | Used for dating regional and intermediate flow systems. Half-life of 5730 years, which covers a timescale of 500 to 30 000 years. |
| Radiogenic | Chlorine-36 | Half-life of approximately 301 000 years. Used for dating deeper, more mature groundwater with an age range of 46 000 to 1 000 000 years old. |
| Stable | Helium-3 | Indicator for mixing of endogenic fluids and groundwater. Used to determine sources of recharge. |

© Copyright, Parsons Brinckerhoff Australia Pty Ltd (2010).

Carbon-14 has been used extensively on waterbores in the GAB to determine groundwater ages, and residence times or travel times along artesian groundwater flow lines. It is, however, only present above background levels within 200 kilometres of the GAB recharge zones due to its relatively short half-life (Bentley et al. 1986, Radke et al. 2000). It is therefore useful for determining whether springs in deeper parts of the Basin are receiving water from a local source, rather than from the deeper aquifers where carbon-14 concentration would be low or undetectable due to the age of the water (Parsons Brinckerhoff Australia Pty Ltd 2010). Anthropogenic substances such as chlorofluorocarbons and tritium can be used as tracers and dating tools for relatively young (less than 50-year-old) groundwater (Parsons Brinckerhoff Australia Pty Ltd 2010).

Chlorine-36 has been used in the GAB to determine groundwater ages, and residence times or travel times along artesian groundwater flow lines (Bentley et al. 1986, Torgersen et al. 1991, Radke et al. 2000, Mahara et al. 2009). Most of the chlorine-36 applications have been carried out on waterbores in the Eromanga and Carpentaria basins, and Coonamble Embayment. Chlorine-36 is well suited for the determination of the long groundwater residence times in the GAB because of its long half-life. Carbon-14 and chlorine-36 isotopes have confirmed the recharge, regional groundwater movement rates, directions and patterns determined from hydrogeological data. Carbon-14 and chlorine-36 samples from springs originate mainly from artesian springs in the south-western part of the Eromanaga Basin in South Australia.

Helium-3/helium-4 ratios can be used to determine the origin of helium in artesian groundwater (Torgersen et al. 1987, 1991, 1992, Mahara et al. 2009, Love et al. 2010). Discharge springs and bores from the GAB have been found to show persistent but variable inputs of endogenic (deeply derived) fluids that contain helium-3 from Earth’s mantle. Up to 40 per cent of the helium in some GAB groundwater may be from the mantle (Love et al. 2010). These findings indicate that water quality in the aquifer is a product of groundwater evolution along flow paths, as well as local input of fluids from deeper rock strata.

# Springsure supergroup springs, Queensland

The Springsure supergroup consists of 71 active and inactive spring complexes, located in central to eastern Queensland, south of the town of Springsure (Figure 57, Table 15 and Table 16), within the Surat Basin. The Springsure supergroup lies within the Surat North Management Area of the Water Resources (Great Artesian Basin) Plan 2006 (Qld), and is the most easterly of the spring supergroups. Springs within this supergroup are the most likely springs to be affected by coal seam gas development and production, because they are closest to coal seams in the Walloon Coal Measures of the Injune Creek Group and Bandanna Coal Measures currently being exploited. The Springsure supergroup comprises mostly recharge springs, although some discharge springs are present, sometimes in the same area as recharge spring vents.

Hydrogeological profiles of the Lucky Last Spring Rock Creek and Abyss, Scott’s Creek, Cockatoo Creek, Boggomoss, Dawson River 2, Dawson River 6 and Dawson River 8 spring complexes are detailed in Volume 2, pages 3–101 of this report. The locations of these spring complexes are highlighted in Figure 57.

Table 15 Inactive spring complexes of the Springsure supergroup, Queensland (Springs Database).

| Inactive | Complex no. |
| --- | --- |
| 325 | 325 |
| Castle | 253 |
| Timor | 587 |

Table 16 Active spring complexes of the Springsure supergroup, Queensland (Springs Database).

| ActivePa | Complex no. | ActivePa | Complex no. |
| --- | --- | --- | --- |
| 16 | 16 | Dawson River 8 | 8 |
| 16 mile | 238 | Den Springs | 245 |
| 254 | 254 | Eddy Owen | 255 |
| 267 | 267 | Eden Vale | 76 |
| 296 | 297 | Elgin | 307 |
| 302 | 302 | Exped Range | 304 |
| 306 | 306 | Goodlife | 248 |
| 308 | 308 | Googenya | 242 |
| 309 | 309 | Hell hole | 234 |
| 310 | 310 | Ladybird | 246 |
| 311 | 311 | Lenore Hills | 583 |
| 326 | 326 | Lonely Eddie | 339 |
| 327 | 327 | Lucky Last | 230 |
| 328 | 328 | Lc1 | 599 |
| 331 | 331 | Lc2 | 600 |
| 332 | 332 | Lc3 | 601 |
| 334 | 334 | Lc4 | 602 |
| 335 | 335 | Lc5 | 603 |
| 336 | 336 | Lc6 | 604 |
| 337 | 337 | Major Mitchell | 241 |
| 35 | 35 | Manan | 258 |
| 78 | 78 | Marburg 1 | 598 |
| Abercorn | 261 | Marlong | 236 |
| Abyss | 592 | Merivale | 589 |
| Barngo | 562 | Moffat | 235 |
| Barton | 283 | Moffat Basalt | 595 |
| Bevan | 268 | Moolayember | 233 |
| Big Tree | 247 | Newton | 85 |
| Binnalong | 509 | Oaky 1 | 597 |
| Boggomoss | 5 | Paddy’s | 237 |
| Bowenvale | 585 | Ponies | 229 |
| Boxvale | 586 | Prices | 580 |
| Buckland | 563 | Rainbow Spring | 1 |
| Bunbunc | 240 | Rock Art | 263 |
| Bunya | 596 | Scott’s Creek | 260 |
| Carnarvetc | 297 | SF212 | 68 |
| Canaveron George | 296 | Spa | 262 |
| 69Cera | 264 | Spring Grove | 298 |
| Channin | 564 | Springwood | 588 |
| Cleanskins | 510 | Spring Rock Creek | 561 |
| Cockatoo 3 | 593 | Spring Ridge | 506 |
| Cockatoo Creek | 9 | Surveyor’s | 262 |
| Conom | 84 | Top | 243 |
| Crystal ball | 232 | Unfairplay | 272 |
| Dam dyke | 256 | VI mile | 507 |
| Dawson River 2 | 2 | Wambo | 584 |
| Dawson River 3 | 3 | Yebna | 74 |
| Dawson River 4 | 4 | Yebna 2 | 591 |
| Dawson River 6 | 6 |  |  |

a Note that active spring complexes often contain inactive vents and some of the numbered complexes are generally included in other complexes.

Figure 57 The Springsure supergroup springs investigated in this report.

## Regional geology

The springs of the Springsure supergroup lie on the eastern margin of the Great Artesian Basin (GAB), where the major aquifers are present at or close to the surface. The distribution of Jurassic rocks, which include all the main artesian aquifers in the region, is controlled by the underlying Permian Bowen Basin and Taroom Trough, on which Triassic and Jurassic GAB sediments were deposited. Folding and faulting resulted in many north–south-directed structures, including anticlines, synclines and faults. Late Cainozoic uplift of the Great Dividing Range increased the Basin-ward tilt towards the south-west, after which the GAB remained stable and the northen margin was extensively eroded (Exon 1976, DNR 1996). As a result, the spring complexes of the Springsure supergroup have a range of geological settings, depending on which formation is present at the surface. The folding and faulting has caused a large number of major and minor faults in the region, with most springs in the area believed to be associated with faults.

## Regional aquifers and aquitards

### Injune Creek Group

The Injune Creek Group of Middle to Late Jurassic age comprises three distinct formations that cover most of the Springsure supergroup region: the Westbourne Formation, the Adori Sandstone/Springbok Sandstone and Birkhead Formation. It also includes the Eurombah Formation and the Walloon Coal Measures. The Injune Creek Group consists of calcareous lithic sandstone, siltstone, mudstone, coal and conglomerate (DNRM 2005, GA 2012).

#### Westbourne Formation

The Westbourne Formation is primarily a confining bed consisting of fine-grained sandstone, siltstone and mudstone. It ranges in thickness from 50 metres to 120 metres (DNRM 2005, GA 2012).

#### Springbok Sandstone

The Springbok Sandstone is the main aquifer of the Injune Creek Group in the Surat Basin, and comprises fine- to medium-grained sandstone, siltstone, mudstone and minor pebble layers (DNRM 2005, GA 2012).

#### Birkhead Formation

The Birkhead Formation acts mainly as a confining bed, and consists of fine-grained sandstone, siltstone and mudstone, with some coal. Where sandstone is present, it acts as a minor aquifer (DNRM 2005, GA 2012).

### Hutton Sandstone

The Early Jurassic Hutton Sandstone contains a major GAB aquifer and overlies the Evergreen Formation. It consists of poorly sorted coarse- to medium-grained sandstone at its base, and fine-grained, well-sorted quartzose sandstone at the top of the formation. Minor amounts of carbonaceous siltstone, mudstone and coal are also present. The formation produces high yields of water (up to 50 L/s) and reaches maximum thicknesses in the centre of the Eromanga Basin of about 250 metres (DNRM 2005, GA 2012).

### Evergreen Formation

The Early Jurassic Evergreen Formation represents a predominantly confining bed. It consists of sandstone, carbonaceous mudstone and siltstone. The formation includes the Boxvale Sandstone Member and Westgrove Ironstone Formation (DNRM 2005, GA 2012).

#### Boxvale Sandstone Member

The Boxvale Sandstone Member of the Evergreen Formation consists of fine- to medium-grained quartz sandstone and fossil wood. It varies in thickness from 45 metres to 90 metres, and produces water of good quality and high yields (DNRM 2005, GA 2012).

### Precipice Formation

The Jurassic Precipice Sandstone and its equivalents are the deepest Jurassic aquifer in the Surat Basin and part of the eastern Eromanga Basin. The Precipice Sandstone consists of thick, cross-bedded and pebbly quartzose sandstone, siltstone and mudstone laid down in the Early Jurassic. Thickness of the formation varies from 45 metres to 150 metres. Water quality is also variable and often influenced by geological structures such as faults. The Precipice Sandstone aquifer recharges along the eastern margin of the GAB and springs occur in this area (DNRM 2005, GA 2012).

### Clematis Sandstone

The Clematis Sandstone aquifer, which this report includes in the GAB, is the oldest aquifer in this region and forms the only major Triassic aquifer in the Surat Basin part of the GAB. It comprises medium- to coarse-grained quartzose sandstone interbedded with siltstone and mudstone, as well as granule- to pebble-sized conglomerate. Waterbores tap the Clematis Sandstone where it outcrops along the eastern edge of the GAB. Towards the centre of the Basin, it is deep and not usually tapped (DNRM 2005, GA 2012).

## Hydrogeological summary

### Conceptual spring types

The occurrence of the Springsure supergroup is attributable primarily to its location near the eastern margin of the GAB, where major confined aquifers are close to the surface, but also have a high-enough artesian pressure for groundwater to come to the surface through any confining beds, usually along faults.

Lucky Last, Abyss and Spring Rock Creek complexes, located alongside each other, are located near a major fault and sit on top of a smaller fault that forms a geological boundary between the Hutton Sandstone and the Evergreen Formation. The conceptual spring type for Lucky Last and Spring Rock Creek would therefore be a discharge spring emanating from an artesian aquifer through a fault (conceptual spring type E and F). The Abyss Complex is likely a recharge spring emanating from the Hutton Sandstone.

The geological complexity of the region makes it difficult to attribute conceptual spring types to all of the spring complexes. Other spring complexes, including Cockatoo Creek, Boggomoss, Dawson River 6 and Dawson River 2 are most likely associated with unmapped faults (EHA 2009). Another conceptual model that may apply to these springs is for discharge springs where the confining layer has been thinned by erosion and the underlying artesian aquifer is able to flow to the surface (see conceptual spring type C at Appendix E).

Scott’s Creek spring complex lies on the eastern limb of the Arcadia Anticline. The likely conceptual model for Scott’s Creek is therefore spring discharge resulting from an anticline elevating the aquifer relative to the surface (see conceptual spring type D in Appendix E).

### Potential source aquifers

Source aquifers for the Springsure spring complexes vary depending on their geological setting. Volume 2 of this report gives a detailed examination of each spring complex. The predominant source aquifer for the investigated springs is likely to be the Precipice Sandstone, which is known to have a potentiometric surface of about 250 metres (AHL) in the area, while the springs are located at about 244–245 m Australian Height Datum. Cockatoo Creek, Boggomoss, Dawson River 6 and Dawson River 2 are all likely to have the Precipice Sandstone as their source. Chemical analysis of spring groundwater and water from waterbores supports the conclusion that Cockatoo Creek is sourced from Precipice Sandstone. Waterbore chemistry varies spatially in the area of Boggomoss and Dawson River 6 spring complexes, making any attribution of a source aquifer less certain. In some instances, mixing with overlying aquifers in the Boxvale Member of the Evergreen Formation might also occur. Dawson River 8 spring complex is likely sourced from the Hutton Sandstone, with influence from the overlying Birkhead Formation.

Scott’s Creek and Lucky Last spring complexes are located on younger Jurassic sediments. Hutton Sandstone is therefore a potential source aquifer for these springs. A comparison of water chemistry for Scott’s Creek springs and waterbores within a 10-kilometre radius suggests Hutton Sandstone as the likely source aquifer. However, again, there may be a contribution from a minor aquifer in the Birkhead Formation, which overlies Hutton Sandstone.

Hydrochemical analysis suggests that artesian groundwater from spring vents in the Lucky Last, Spring Rock Creek and Abyss (located next to Lucky Last, see Figure 57) complexes may have different source aquifers, depending on their location. For example, springs 287 and 340 (Lucky Last) are located on an outcrop of Evergreen Formation, and are thought to be associated with a fault that allows upward migration of groundwater from the Precipice Sandstone or adjacent Hutton Sandstone. On the other hand, springs 286 and 716 (Abyss) are underlain by Hutton Sandstone and located near the contact between the Hutton Sandstone and Birkhead Formation. The springs are thought to be fed by groundwater from the Hutton Sandstone emerging at the edge of the Birkhead Formation, making them recharge springs. Springs 287, 286 and 716 do have similar water chemistry, suggesting that they are all fed by Hutton Sandstone (see Volume 2 of this report).

Although likely source aquifers for the Springsure supergroup can be identified with more confidence than for the Eulo supergroup, it is not certain whether the Hutton and Precipice sandstones are the only source aquifers. This is due to a lack of chemical and potentiometric data for the Clematis Sandstone Group, as well as the Boxvale Sandstone Member and Birkhead Formation.

# Eulo supergroup springs, Queensland

The Eulo supergroup consists of 53 active and inactive spring complexes, located in southern Queensland south-west of the town of Eulo (Figure 58, Table 17), within the Eromanga Basin.The Eulo supergroup lies within the Warrego Management Area of the Water Resources (Great Artesian Basin) Plan 2006 (Qld). This region has experienced the greatest loss of pressure in the Great Artesian Basin (GAB); on average, the water level has decreased by 39 metres and ranges from 0 metres to 120 metres across the supergroup (Habermehl 1980, Fairfax & Fensham 2003). Collectively, these springs are considered to currently discharge about one-third of their early 20th century output (Fairfax & Fensham 2003). Many bores were sunk adjacent to springs in the region, typically within a few metres. Springs in the region have been reported to experience notable natural variation in flow rates. Although some springs were recorded to have a constant flow rate, others have been noted to fluctuate daily (e.g. Twomanee Springs), and others show intermittent spring activity for a longer time. Intermittent spring activity may be attributed to changes to the permeability and/or the chemical precipitates in the vertical pathways (EHA 2009). Anecdotal evidence does, however, suggest a link between weather patterns, such as high- and low-pressure systems, and short-term variation in spring flow.

Hydrogeological profiles of the Carpet, Dead Sea, Eulo Town, Granite, Merimo, Tunga, Wooregym and Yowah Creek spring complexes have been completed, and are included in Volume 2, pages 102–189 of this report. The names and location of these spring complexes are in Figure 58.

## Regional geology

The Eulo Springs overlie the Eulo Ridge, an area of granitic bedrock exposures and regionally shallow depth to the granite basement. The Eulo Ridge is characterised by a marked thinning of the Eromanga Basin sedimentary sequence (Senior et al. 1971). Wireline-logged waterbores in the region (Habermehl 2001) indicate that the Eromanga Basin aquifers beneath the Hooray Sandstone are not present within the vicinity of the spring complexes (EHA 2009). Outcrops of the Winton Formation, and the Doncaster and Coreena Members of the Wallumbilla Formation are present in the region. Several spring complexes, most notably the Dead Sea complex, are coincident with mapped geological fault structures (see Toompine 1:250 000 SG55-13 geological map sheet series—Ingram and Senior 1970).

Table 17 Spring complexes of the Eulo supergroup, Queensland.

| ActivePa | Complex no. | Inactive | Complex no. |
| --- | --- | --- | --- |
| Bokeen | n/a | Banger | 146 |
| Car Window | 113 | Baroona | 159 |
| Carpet | 148 | Bush | 134 |
| Corina | 123 | Curracunya | 123 |
| Dead Sea | 157 | Currawinya | 108 |
| Dewella | 150 | Goonerah | 117 |
| Eulo Town | 144 | Kungie | 115 |
| Fish | 110 | Minyeburra | 141 |
| Fish (Tarko) (formerly Jubilee) | 129 | Ningaling | 271 |
| Garrawin | 57 | Riley | 105 |
| Goonamurra | 153 | ShellRam | 151 |
| Gooning | 130 | Shire Tank | 121 |
| Granite | 133 | Taleroo | 109 |
| Jacombe | n/a | Thorlindah | 104 |
| Jubilee | 129 | Thorlindah Mud | n/a |
| Kopanyu | 111 | Tynghynia | n/a |
| Magic Wand | 137 | Waihora | 143 |
| Merimo | 142 | Warroo | n/a |
| Muddy | 145 | Wiggera | 138 |
| Ooliman | 149 | Woolshed | 116 |
| Panic Sandhill | n/a | Youleen | 114 |
| Pikes | 147 |  |  |
| Pitherty | 118 |  |  |
| Tareen | 124 |  |  |
| Tilberry | 132 |  |  |
| Tunca | 135 |  |  |
| Tunga | 139 |  |  |
| Tunkata | 125 |  |  |
| Wacno | 131 |  |  |
| Wooregym | 136 |  |  |
| Yowah Creek | 156 |  |  |
| Yowah Mud | 155 |  |  |

n/a = not applicable.
a Note that active spring complexes often contain inactive vents.

Figure 58 The Eulo supergroup springs in the Eromanga Basin investigated in this report.

## Regional aquifers

The major GAB aquifers present in the Eulo supergroup region include the Hooray Sandstone and the Wyandra Sandstone of the Cadna-owie Formation. Local artesian aquifers also occur in the shallow Winton Formation, and within the Coreena and Doncaster members of the Wallumbilla Formation in the south-eastern part of the Eromanga Basin. However, the latter is generally referred to as a confining bed by Habermehl (2001) (see Figure 56 in Section 8.2).

### Winton/Mackunda Formation

The Winton Formation of the Rolling Downs Group is a Cretaceous formation located in central to western Queensland, South Australia and the Northern Territory across a large part of the GAB. It consists of sedimentary rocks including sandstone, siltstone and claystone. The average depth to the top of the water-bearing strata is 77 metres, with an estimated average saturated thickness of 6 metres in the Eulo area. The Winton Formation reaches a maximum thickness of 400 metres.

The Mackunda Formation of the Rolling Downs Group conformably underlies the Winton Formation, and consists of labile sandstone, mudstone and calcareous siltstone. The average depth to the top of the water-bearing strata in the Mackunda Formation is 179 metres with an average thickness of 6 metres across its extent. The Winton/Mackunda Groundwater Management Unit consists of two nonflowing or subartesian aquifer systems. The (semi-)confined aquifers occur within laterally discontinuous or lenticular sandstone deposits (Habermehl 1980, Australian Natural Resource Atlas 2009).

### Wallumbilla Formation

The Cretaceous Wallumbilla Formation of the Rolling Downs Group is a sedimentary unit comprised dominantly by mudstone and siltstone with grey limestone and minor sandstone. The average thickness of the formation is 250 metres, and ranges from 100 metres to 600 metres (Radke et al. 2003). Aquifers within the Wallumbilla Formation (Coreena and Doncaster members) are generally lenticular with an uncertain hydraulic connection between adjacent aquifers. Yields are low and salinity is typically high, ranging from 1500 microsiemens per centimetre (µS/cm) to more than 10 000 µS/cm. The named beds include the Coreena, Doncaster, Jones Valley, Ranmoor and Trimble members, although only the Coreena and Doncaster members occur in the Eulo area. It reaches a maximum thickness of 600 metres. The Coreena and Doncaster members also contain sandstone and form minor aquifers. The Wallumbilla Formation overlies the Wyandra Sandstone Member of the Cadna-owie Formation in the Eromanga Basin (DNRM 2005, GA 2012).

#### Coreena Member

The Coreena Member comprises siltstone and fine sandstone interbedded with mudstone. It has a maximum thickness of 165 metres (DNRM 2005, GA 2012).

#### Doncaster Member

The Doncaster Member comprises mudstone, siltstone, minor quartz sandstone in part glauconitic, silty limestone and gypsum (Eromanga and Surat basins). It has a maximum thickness of 275 metres (DNRM 2005, GA 2012).

#### Wyandra Sandstone (Cadna-owie Formation)

The Wyandra Sandstone Member of the Cadna-owie Formation forms a significant aquifer in parts of Queensland, overlying the Cadna-owie Formation in the Eromanga Basin. It is the shallowest artesian aquifer producing flowing artesian waterbores in the Basin, though some flowing bores are also known from the Correna–Doncaster aquifers. The formation is a medium- to coarse-grained quartzose sandstone, generally less than 20 metres in thickness (DNRM 2005, GA 2012).

#### Hooray Sandstone

The Hooray Sandstone and its hydrogeological equivalents are generally the shallowest major artesian aquifer intercepted by flowing artesian waterbores in the GAB in Queensland. The Hooray Sandstone comprises fine-to-coarse white quartzose sandstone with a kaolinitic matrix. This is the most highly developed artesian aquifer across a large extent of the Eromanga Basin, except in the centre of the basin where it is too deep for economic development by waterbores. It typically produces very good yields (DNRM 2005, GA 2012).

## Hydrogeological summary

### Conceptual spring types

The Eulo spring complex is primarily attributable to the shallowing of the GAB aquifers due to elevated granitic basement structures. The Granite and Eulo Town spring complexes are situated directly adjacent to granite outcroppings, and may be considered as discharge spring complexes emanating from contact between onlapping sediments and outcropping basement structures (see conceptual spring types I and J at Appendix E and Figure 59).

The Carpet, Merimo and Wooregym spring complexes occur at greater distances from mapped granite outliers, although are likely still associated with elevated granite basement in the region. The most likely conceptual spring model for these vents are discharge springs associated with weakness or thinning, and, possibly, the fracturing-associated basement structure (conceptual spring types G and H, Appendix E).

The Dead Sea spring complex is one of the most active spring complexes in the Eulo region, with 24 active vents recorded during the 2012 field surveys. This spring group lies directly above the Boondoona fault (see conceptual spring type E and F, Appendix E), although the elevated basement structure also should be considered as a relevant conceptual spring type for this complex (type I and J). The Yowah Creek spring complex may also be associated with the extension of the Boondoona Fault, and the Tunga spring complex may be associated with an unmapped fault or fracture system, because there is a fault running through the inlier of granite at the headwaters of Boorara and Twomanee creeks (EHA 2009).

Senior et al. (1971) refers to carbonate coatings and deposits of tufa in the Eulo area. These are likely to be similar to carbonate deposits found on artesian GAB springs in northern New South Wales, western Queensland and in the south-western part of the GAB in South Australia.

Figure 59 Granite spring complex—Massey Spring.

### Potential source aquifers

The likely source aquifers of the Eulo Spring groups are considered to be the Wyandra Sandstone Member of the Cadna-owie Formation and/or the Hooray Sandstone, although it is likely that some of these waters mix with shallower aquifers en route to surface (e.g. type J springs). Although considered less likely, there is insufficient evidence to discount conclusively the shallower aquifers of the Winton Formation, and the discontinuous aquifers present in the Doncaster and Coreena Members of the Wallumbilla Formation as being potential source aquifers for some of the Eulo springs. This is due to:

the artesian status of several bores in the region tapping these shallow aquifers

no differentiation between the water chemistry of springs in the region and groundwater bores, regardless of which aquifer the bores are recorded to be tapping (see, for example, Carpet in Figure 60).

All waterbores known to be tapping either the Wyandra Sandstone Member of the Cadna-owie Formation or the Hooray Sandstone with available standing water-level data assessed in this report have maintained artesian pressure over time. Although there are many examples of shallower waterbores with artesian pressure tapping either the Winton or Wallumbilla formation, there are also several examples of waterbores tapping these aquifers being recorded as nonflowing or subartesian upon drilling, or being recorded as ceasing to flow post-drilling. EHA (2009) postulate that there may be sufficient interconnection between the deeper aquifers (the Wyandra Sandstone and the Hooray Sandstone) and the shallower aquifers (Wallumbilla and Winton formations) to allow for the development of artesian conditions in the upper formations that are not regionally considered to produce flowing artesian waterbores.

Water-quality analyses of spring and bore water did not conclusively attribute or discount source aquifers for the Eulo spring complexes analysed here. This could be explained by the waterbores in the region lacking casing information, such as missing information as to whether the bore had screens or used only an open hole at the bottom of the casing, and which aquifers were contributing water. A number of bores also tap multiple aquifers and the major ion composition of water from bores in the area is similar, regardless of what aquifers they are tapping (Figure 60). This could result in groundwater mixing, thereby erasing any water-quality signatures. An alternative explanation is that these waterbores and springs could be sourced from the same aquifer. There is currently no data to confirm whether the water chemistry for these waterbores differs from the groundwater chemistry for overlying or underlying aquifers.

Figure 60 Carpet spring complex—Piper plot of the spring water and bore water chemistry.

# Bourke supergroup springs, New South Wales

The Bourke supergroup consists of 53 active and inactive discharge spring complexes, located in northern New South Wales, near the town of Bourke (Figure 61 and Table 18), within the Eromanga Basin part of the Great Artesian Basin (GAB). Tego and Towry are located in Queensland. Most of the springs in this area are inactive. In comparison to the Springsure and Eulo supergroups, there are very few Bourke spring complexes with high-value rankings (see Part 1) and many spring wetland areas have been excavated.

Hydrogeological profiles of the spring complexes with conservation rankings 1a—4a; Peery Lakes, Sweetwater, Thooro mud Tego, and Towry, have been complete39Td39T and are found in Volume 2 of this report. The locations of these spring complexes are shown in Figure 61.

Table 18 Spring complexes of the Bourke supergroup.

| Spring complexes with active springs | Inactive spring complexes |  |
| --- | --- | --- |
| Boongunyarrah | Bathing | Sandy |
| Colless | Bingewilpa | Sledgehammer |
| Lake Eliza | Coonbilly | Sweetwater |
| Mascot | Gerara | Thooro |
| Mooronowa North | Goonery | Thully |
| Mooronowa South | Gooroomero | Tooloomi |
| Nulty | Hawkes | Wapweela |
| Peery Lakes | Kallara | Yantabangee |
| Scrubber | Kullyna | Yantabulla |
| Tego | Lila | Yoorltoo |
| Throo Mud | Native Dog | Yotomi |
| Toulby | Peery | Youngerina |
| Towry | Pullamonga |  |

## Regional geology

The Bourke spring complex lies near the southern edge of the Eromanga Basin in New South Wales, and just north of the New South Wales – Queensland border. Interpreted stratigraphy for this area is not as detailed as the stratigraphy available for the areas around the Springsure and Eulo supergroups. Available stratigraphic information indicates that the geological units present in the area include the Rolling Downs Group and Hooray Sandstone. Deeper GAB aquifers such as Hutton or Precipice Sandstone are not present (Kuske et al. 2010). Many geological structures, including faults and lineaments, are evident in the region, and many of the springs appear to be associated with these geological features (Figure 61).

GAB = Great Artesian Basin.

Figure 61 Bourke supergroup, Eromanga & Surat basins, New South Wales (NSW) and Queensland (Qld).

## Regional aquifers

### Wallumbilla Formation

The Cretaceous Wallumbilla Formation is a sedimentary unit comprised mainly of mudstone and siltstone with grey limestone and minor sandstone. It is the lowest stratigraphic unit in the Rolling Downs Group. The average thickness of the formation is 250 metres, and ranges from 100 metres to 600 metres (Radke et al. 2000). The Coreena Member comprises siltstone and fine sandstone interbedded with mudstone. It has a maximum thickness of 165 metres (DNRM 2005, GA 2012). The Doncaster Member comprises mudstone, siltstone, minor quartz sandstone in part glauconitic, silty limestone and gypsum (Eromanga and Surat basins). It has a maximum thickness of 275 metres (DNRM 2005, GA 2012).

Aquifers within the Wallumbilla Formation are generally lenticular with an uncertain hydraulic connection between adjacent aquifers. Yields are low and salinity is typically high, ranging from 1500 microsiemens per centimetre (µS/cm) to more than 100 000 µS/cm. The aquifers include the Coreena and Doncaster members, which are present in the Bourke area. The Wallumbilla Formation overlies the Wyandra Sandstone Member of the Cadna-owie Formation in the Eromanga Basin (DNRM 2005, GA 2012).

### Wyandra Sandstone (Cadna-owie Formation)

The Wyandra Sandstone Member of the Cadna-owie Formation forms a significant aquifer, overlying the Cadna-owie Formation in the Eromanga Basin, and is the shallowest artesian aquifer in the GAB that generally produces flowing artesian waterbores, though some flowing bores also originate from the Wallumbilla Formation. The formation is a medium- to coarse-grained quartzose sandstone, generally less than 20 metres in thickness (DNRM 2005, GA 2012).

### Hooray Sandstone

The Lower Cretaceous – Jurassic Hooray Sandstone and its hydrogeological equivalents are also generally the shallowest major artesian aquifers in the GAB, producing flowing artesian waterbores in the Eromanga Basin. The Wyandra Sandstone Member of the Cadna-owie Formation and the Hooray Sandstone are often considered as a single aquifer unit. The Hooray Sandstone comprises fine-to-coarse white quartzose sandstone with a kaolinitic matrix. This is the most highly developed artesian aquifer across a large extent of the Eromanga Basin, except in the centre where it is too deep for economic waterbore development. The Hooray Sandstone typically produces very good yields (DNRM 2005, GA 2012).

## Hydrogeological summary

### Conceptual spring types

Peery Lakes spring complex and other spring complexes on the edge of the GAB are likely to be springs emanating from a downgradient at the edge of the Basin (see conceptual spring type K at Appendix E).

The extensive faulting and other geological structures, including lineaments in the area, suggest that most of the spring complexes in the Bourke supergroup are discharge springs emanating from an artesian aquifer through a fault (conceptual spring types E and F). For example, there are a number of faults and lineaments running north-west to south-east, and lines of springs running parallel to them, such as the line of springs running from Youngerina to Bourke (Rade 1954). A line of springs, including Sweetwater, also follow a fault trending northeast to southwest (Figure 61).

It is harder to attribute a conceptual spring type to Tego and Towry spring complexes, because they lie on the Cunnamulla Shelf in an area with very few geological features and no mapped basement highs (Cunnamulla 1:250 000 geological sheet SH55-2, 1st edition; Senior & Thomas 1971). Nevertheless, the most likely conceptual types to apply would be springs emanating from a fault (type E or F), or a spring resulting from anticline or monocline elevating the aquifer relative to the surface (type D).

### Potential source aquifers

The likely source aquifer for most spring complexes in the Bourke supergroup—including Peery Lakes, Sweetwater and Thooro Mud—is the Hooray Sandstone. It is possible that the springs are fed from shallower aquifers including the Wyandra Sandstone Member of the Cadna-owie Formation, or minor aquifers in the Coreena and Doncaster members of the Wallumbilla Formation. The Wyandra Sandstone Member of the Cadna-owie Formation is also a likely aquifer for Tego and Towry springs, as the Hooray Sandstone is at some depth in this area.

Although considered less likely, there is insufficient evidence to discount the discontinuous aquifers present in the Wallumbilla Formation as being potential source aquifers for the Bourke springs, due to:

the artesian status of waterbores in the region tapping these shallow aquifers

no interpreted stratigraphy to determine what aquifers the waterbores are tapping

little to no groundwater chemistry data to distinguish between aquifers and assign a source aquifer for a spring.

Many of the Bourke springs are now inactive, suggesting that whatever aquifer was their source no longer has sufficient artesian pressure for groundwater to reach the surface and produce artesian springs. The springs are located in the region extending across a large part of southern Queensland and northern New South Wales that has experienced extensive drawdown due to large-scale exploitation of artesian groundwater since the late 1890s (Habermehl 1980).

# Bogan River supergroup springs, New South Wales

The Bogan River supergroup consists of four active and inactive discharge spring complexes, located in northern New South Wales (Figure 62 and Table 19). The spring complexes extend in a line running south-south-west to north-north-east starting near the New South Wales town of Colossal and ending near the Queensland town of Hebel. The most southerly complex (Coolabah) lies on the edge of the Great Artesian Basin (GAB). Cumborah, Cuddie and Coorigul are located in the Coonamble Embayment of the Surat Basin. The geology and characteristics of Coorigul and Cumborah spring complexes indicate that they are not GAB springs, but sourced by local aquifers.

Hydrogeological profiles of all spring complexes have been completed and are in Volume 2, pages 235–264 of this report. The location of these spring complexes is shown in Figure 62 and Figure 63.

Table 19 Spring complexes of the Bogan River supergroup, New South Wales.

| Spring complexes with active springs | Inactive spring complexes |
| --- | --- |
| Coorigul | Coolabah |
|  | Cuddie |
|  | Cumborah |

## Regional geology

The Bogan River springs extend from the north-western part of the Coonamble Embayment into the Surat Basin (Figure 63). The geology for each of the spring complexes is based on the stratigraphy of the Surat Basin and Coonamble Embayment. Coorigul and Cumborah spring complexes are located along a ridge of sedimentary rocks that contain opal. The town of Lightning Ridge, famous for its opals, is located about halfway between Coorigul and Cumborah. Drillers’ logs from waterbores and mapped depths of the Hooray Sandstone and its hydrogeological equivalents indicate that Mooga Sandstone and Gubberamunda Sandstone are underlying the Cuddie, Cumborah and Coorigul springs. Coolabah Spring is likely located at the edge of the GAB—shown within a small triangle shown as Surat Basin on the geological framework side map as part of the Bourke 1: 250 000 SH 55-10 Geological Series map sheet (Brunker 1971) and on the edge of the GAB hydrogeological unit boundary map produced by Habermehl and Lau (1997) (Figure 62). However, it maps just outside the boundary of Surat Basin using Australian Geological Provinces (sedimentary basins) maps (Stewart et al. 2013) (Figure 63).

GAB = Great Artesian Basin, QLD = Queensland.

Figure 62 Bogan River supergroup—Eromanga and Surat basins, New South Wales (NSW).

GAB = Great Artesian Basin, QLD = Queensland.

Figure 63 Boundaries of GAB hydrogeological units in northern New South Wales (NSW)—Bogan River supergroups.

## Regional aquifers

### Bungil Formation

The Bungil Formation occurs in the Surat Basin and the Coonamble Embayment. It consists of three members: the Minmi Member, the Nullawurt Sandstone Member and the Kingulli Member, and comprises sandstone, siltstone and mudstone. The Kingull Member is mostly a confining bed. The Minmi Member is primarily a confining bed, but does contain minor aquifers. The Nallawurt Sandstone Member is the main aquifer and comprises a quartzose, and lithic sandstone and siltstone. The Bungil Formation is a lateral equivalent of the Cadna-owie Formation in the Eromanga Basin and, along with the Mooga Sandstone, is informally divided into the Drildool beds and Keelindi beds in the Coonamble Embayment. It provides water of generally good quality with yields of more than 1.5 litres per second (L/s) (DNRM 2005, GA 2012). The Bungil Formation is informally known as the Drildool beds in the southern Coonamble Embayment to delineate the limit of marine influenced sediments (Watkins & Meakin 1964).

### Mooga Sandstone

Mooga Sandstone is a hydrogeological equivalent of the Hooray Sandstone. The formation is thin but continuous throughout the Surat Basin and the Coonamble Embayment, and supplies artesian groundwater of good quality with yields of up to 35 L/s. The formation occurs between only the Nebine Ridge and the Kumbarilla Ridge. Mooga Sandstone comprises fluvial quartzose to sublabile sandstone. It is thinly bedded with siltstone and mudstone (DNRM 2005, GA 2012). The Mooga Sandstone has informally been named the Drildool and Keelindi beds in the Coonamble Embayment due to the difficulty in distinguishing the Mooga Sandstone from the Orallo Formation in the southern Coonamble Embayment (Watkins & Meakin 1964).

### Gubberamunda Sandstone

The Gubberamunda Sandstone is also a hydrogeological equivalent of the Hooray Sandstone and laterally equivalent to the Pilliga Sandstone in the Coonamble Embayment. It comprises fluviatile sandstone, siltsone, mudstone and minor conglomerate. The Gubberamunda Sandstone provides artesian flows of good-quality water (DNRM 2005, GA 2012).

## Hydrogeological summary

### Conceptual spring types

The Coorigul and Cumborah spring complexes are most likely non-GAB springs emanating from the base of Tertiary sandstone where it contacts a confining layer.

Cuddie spring complex lies in the Coonamble Embayment, in an area with a number of lineaments trending north-south and east-west (NSW DTIRE 2004). The conceptual spring types that would describe the mechanism for discharge would be a discharge spring emanating from an artesian aquifer through a fault (see conceptual spring types E and F at Appendix E).

According to available stratigraphy and the mapped extent of the GAB by Habermehl & Lau (1997), Coolabah spring complex lies just on the edge of the GAB and the Lachlan Fold Belt. As it appears to be a GAB spring, the most likely conceptual spring type would be a spring emanating from a downgradient at the edge of the Basin (see conceptual spring type K in Appendix E).

### Potential source aquifer

The likely source aquifers for the GAB springs of the Bogan River supergroup (Cuddie and Coolabah) located in the Coonamble Embayment include the Mooga Sandstone (informally named the Drildool beds and Keelindi beds), the Bungil Formation and the Pilliga Sandstone.

The Cuddie spring complex is located in the Surat Basin. Mooga Sandstone is the most likely source aquifer for the Cuddie spring. It appears as though most bores in the area tap the Mooga Sandstone. Some bores are very deep, and appear to extend beyond the depth of Mooga Sandstone to deeper GAB aquifers such as the Pilliga Sandstone.

As the Coolabah spring complex is located near the southern edge of the GAB, where the Mooga Sandstone (Drildool beds and/or the Keelindi beds) come to the surface, one or more of these aquifers is the most likely source aquifer for springs in this complex. The Pilliga Sandstone appears to thin out to the east between Marra Creek and Macquarie River, and does not extend as far west as Bogan River (geological cross-section on Walgett SH 55-11 Geological Series map sheet 1:250 000 by Meakin et al. 1996), leaving the Mooga Sandstone as the most likely option.

# Risk assessment

Coal seam gas extraction in the Surat and Bowen basins may pose a threat to Great Artesian Basin (GAB) springs through the depressurisation of the coal seams in the Walloon Coal Measures (Surat Basin) and the Bandanna Coal Measures (Bowen Basin), and the subsequent effects of depresurisation on overlying and underlying aquifers.

The process of depressurising a coal seam involves pumping out groundwater to lower the artesian pressure in the coal seams. Gas then desorbs from the coal and can be extracted. Groundwater from overlying and underlying aquifers may then flow into the depressurised coal seams (QWC 2012a). The extent to which the coal seams and adjacent aquifers are hydraulically connected determines the resulting decrease in artesian groundwater pressure of those aquifer(s). Where a formation containing a targeted coal seam is separated from an aquifer by low-permeability rocks, such as siltstone and mudstone, groundwater flow between the coal seams and aquifer will be greatly reduced and coal seam gas extraction could have a limited impact on the aquifer.

Coalmining has the potential to impact GAB springs through similar mechanisms. To prevent groundwater from infiltrating mine pits and underground mine shafts, water is pumped out of surrounding formations. This creates a cone of depression around a mine. Like a depressurised coal seam, water from overlying or underlying aquifers (and from formations surrounding the depressurised zone) may flow into the depressurised zone.

Any reduction of the artesian groundwater pressure in an aquifer may result in a decrease in the artesian groundwater levels of in a waterbore or spring that is fed by the aquifer (QWC 2012a). The closer a GAB spring is to coal seam gas production wells or coalmines, the greater the likelihood of the spring being affected, if coal seam gas extraction or mine dewatering has an effect on the source aquifer. Figure 64 shows the location of the Springsure, Eulo, Bourke and Bogan River supergroups in relation to current coal seam gas production wells, coal seam gas tenements and potential future sites of coal seam gas production. Figure 65 shows the location of the Springsure, Eulo, Bourke and Bogan River supergroups in relation to current coalmining leases and coal exploration permit titles. From these figures, it can be seen that the Springsure supergroup is in close proximity to coal seam gas and coalmining activity. The other supergroups (Eulo, Bourke and Bogan River) are relatively isolated and are not likely to be impacted from current activities. However, it should be noted that groundwater impacts can be slow to manifest and the springs should be monitored to ensure that negative effects do not develop, particularly through cumulative impacts of groundwater extraction for both groundwater bores, and coal and gas development.

ATP = authority to prospect, CSG = coal seam gas, EPP = exploration permit for petroleum, GAB = Great Artesian Basin.

Figure 64 Coal seam gas tenures and springs of the Springsure, Eulo, Bourke and Bogan River supergroups.

GAB = Great Artesian Basin, NSW = New South Wales, Qld = Queensland.

Figure 65 Coal tenures and springs of the Springsure, Eulo, Bourke and Bogan River supergroups.

The Walloon Coal Measures in the Surat Basin and the Bandanna Formation in the Bowen Basin are the two formations currently targeted for coal seam gas extraction (DERM 2010). They are not major aquifers of the GAB; however, some bores do tap the Walloon Coal Measures and, on occasion, water is extracted from the Bandanna Formation for agricultural purposes.

For most of the extent of the Walloon Coal Measures, low-permeability mudstones and siltstones are present at the top and bottom of the formation. The siltstone and mudstones at the bottom of the formation are quite thick in some areas and act as an aquitard separating the Walloon Coal Measures from the Hutton Sandstone below. The Queensland Water Commission (QWC 2012a) predicted the maximum impact on Hutton Sandstone to be less than five metres drawdown across most of its extent. There are, however, areas where the maximum drawdown may be up to 18 metres. The confining siltstone and mudstone at the top of the Walloon Coal Measures is much thinner and, in some places, has been eroded away. Connectivity between the Walloon Coal Measures and the overlying Springbok Sandstone aquifer is therefore highly variable and poorly understood at this stage (QWC 2012a). The QWC (2012a) predicted the maximum impact on the Springbok Sandstone across most of its extent to be less than a 20-metre drawdown; however, drawdown in an area south of Miles may reach more than 90 metres.

Depressurisation of the Bandanna Formation is unlikely to affect underlying formations, as they have very low permeability. It is also unlikely to affect overlying aquifers across most of its extent, because it is separated from overlying aquifers by the Rewan Group, which is made up of mudstones with very low permeability (QWC 2012a). The Rewan Group and overlying Clematis Sandstone were eroded away in a narrow zone that trends north-south and is situated east of Injune. The Precipice Sandstone in that location is in direct contact with the Bandanna Formation and the two formations are likely to have a high degree of interaction. The QWC predicted a maximum drawdown of 10 metres in this area, which is also close to the Fairview and Spring Gully coal seam gas production fields.

The potential for drawdown of tens of metres in adjacent aquifers in some areas resulted in the QWC investigating the potential effects of the predicted drawdowns on springs in the Springsure supergroup, which is in the vicinity of coal seam gas production fields. The QWC (2012a) has identified springs in the Fairview and Spring Gully coal seam gas production field areas as the springs most likely to be affected by coal seam gas extraction. The Surat underground water impact report (QWC 2012a) gave Lucky Last, Scott’s Creek and Yebna 2 spring complexes a risk assessment score of five out of five based on the likelihood of a reduction in the artesian groundwater flow from springs (drawdown of more than 0.2 metres) and the consequences of a reduced flow on ecological values.

Lucky Last and Scott’s Creek spring complexes are of particular concern, because they have a high-value ranking (as per Fensham et al. 2010) and a high likelihood of being affected by coal seam gas activities. Figure 66 shows the location of Lucky Last and Scott’s Creek spring complexes in relation to coal seam gas tenures and wells in the area. Spring complexes predicted to be impacted, but with a lower risk rank (e.g. because drawdown is predicted to be less than 0.2 metres) include Dawson River 8, Yebna, Ponies, Barton, 311, Spring Ridge, VI Mile, Spring Rock Creek, Wambo and Abyss. Other springs of concern include Dawson River 8, 311 and Spring Rock Creek, because they have fairly high-risk rankings and are discharge springs. The drawdown risk threshold of 0.2 metres—as set in s. 379(3) Water Act 2000 (Qld)—used to assess and rank springs likely to be impacted by coal seam gas is, however, more of a reflection on the predictive capability of the QWC’s groundwater model, as opposed to the actual level of drawdown that would have an impact on a spring’s ecological values.

Lucky Last spring complex is predicted to have a reduction in artesian groundwater pressure of 1.0–1.5 metres in the next 40–60 years, with an estimated timeframe of 5–40 years before the impact is predicted to exceed 0.2 metres (QWC 2012a). This is because the Precipice Sandstone (with some contribution from the Boxvale Sandstone Member of the Evergreen Formation) has been identified as the source aquifer for Lucky Last Springs, and there is a high degree of connectivity between artesian groundwater in the Precipice Sandstone and the Bandanna Coal Formation in this area. This is also the case for Yebna 2 spring complex. Once the drawdown exceeds 0.2 metres, the ‘make good obligations’ under the Water Act 2000 (Qld) will be triggered.

Scott’s Creek spring complex is predicted to have a reduction in artesian groundwater pressure of 0.2–0.5 m in the next 30–35 years. It is estimated that it will take 20–40 years before the predicted impact will exceed 0.2 metres (QWC 2012a). The Hutton Sandstone has been identified as the source aquifer for Scott’s Creek spring complex39T.39T In the area of Scott’s Creek, the lower Walloon Coal Measures aquitard is thin, and there would therefore be greater connectivity between the Hutton Sandstone and Walloon Coal Measures.

The QWC has developed a Spring Impact Management Strategy (SIMS) (QWC 2012a) as part of the underground water impact report. Along with identifying potentially affected artesian springs and assessing the connectivity of underlying aquifers, the SIMS also provides for a monitoring program and spring impact mitigation strategy. The spring mitigation strategy is currently focused on investigating potential actions to mitigate or prevent the predicted impacts at spring locations. Petroleum tenure holders are required to implement a spring monitoring program and a spring mitigation program for springs with a predicted impact of more than 0.2 metres under the Water Act 2000 (Qld). The spring complexes that fall under this requirement include Lucky Last, Scott’s Creek, Barton, Spring Rock Creek and 311/Yebna 2. The other springs investigated in this report had a predicted drawdown of less than 0.2 metres and are therefore unlikely to be significantly impacted by coal seam gas extraction. Further investigation into Dawson River 8 would be advisable, because it is of high conservation value. One bore in the area, with similar groundwater chemistry to the spring, is listed as tapping the Walloon Coal Measures (see Hydrogeological profile of Dawson River 8), although this may be an error. Table 20 shows the risk assessment score (QWC 2012b) for the Springsure spring complexes given a conservation ranking of 1a or 1b by Fensham et al. (2010) and focused on in this report. Springs listed as ‘not potentially impacted’ are in an area where no underlying aquifer has a predicted drawdown of more than 0.2 metres, and are at least 10 kilometres from an area with a predicted drawdown of more than 0.2 metres.

All springs likely to be affected by coal seam gas development are located in the Surat Basin, although some of them, such as Lucky Last spring complex, may be impacted by drawdown in underlying Bowen Basin Sediments. All springs located in the Bowen Basin, with the exception of Elgin 2 are recharge springs—conceptual types A or B (Figure 67). Recharge springs are supplied by groundwater from an unconfined GAB aquifer. Water drains out of the aquifer either by the water intersecting and running along a layer of low permeability or a fracture, or where the saturated zone of the aquifer intersects with the surface. Water residence time in the outcropping aquifer is fairly short. Spring flow is dynamic and correlated to recent rainfall (Habermehl 1982). For this reason recharge springs are less likely to be affected by dewatering though coal seam gas activity or mining than discharge springs, as spring flow is less dependent on artesian pressure.

APLNG = Australian Pacific LNG, ATP = authority to prospect, CSG = coal seam gas, EPP = exploration permit for petroleum.

Figure 66 Coal seam gas tenures and wells near Lucky Last and Scott’s Creek spring complexes.

Table 20 Risk assessment results for Springsure spring complexes with 1a and 1b conservation ranking.

| Spring complex name | QWC complex number | Risk assessment score (1–5)Pa |
| --- | --- | --- |
| Lucky Last | 230 | 5 |
| Scott’s Creek | 260 | 5 |
| Dawson River 8 | 8 | 3 |
| Cockatoo Creek | 9 | Not a potentially impacted spring |
| Boggomoss | 5 | Not a potentially impacted spring |
| Dawson River 6 | 6 | Not a potentially impacted spring |
| Dawson River 2 | 2 | Not a potentially impacted spring |

QWC = Queensland Water Commission.
a As assessed in QWC (2012a).

As only a few spring complexes within the Springsure supergroup are likely to be affected by coal seam gas extractions, it is unlikely that spring complexes further away such as Eulo, Bourke and Bogan River will be affected in the short term. However, exploration permits for petroleum (EPPs) and authorities to prospect (ATPs) have been granted for areas close to the Eulo and Bourke supergroups (Figure 64). Because the QWC, now the Office of Groundwater Impact Assessment, has a defined area of jurisdiction (currently the Surat Cumulative Management Area), spring complexes other than Springsure are outside of the area of modelled impact by coal seam gas development on groundwater. No model currently exists in enough detail to predict drawdown caused by coal seam gas development or coalmining for the Eulo, Bourke and Bogan River supergroups. Any effects that future coal seam gas extraction might have on springs in the Eulo and Bourke supergroups will depend on the hydraulic connectivity between the targeted coal seams and the Cadna-owie–Hooray aquifer and equivalent Surat Basin aquifers, the distance between the springs and coal seam gas extraction sites, and scale of the groundwater level drawdown (QWC 2012a). However, given the long distances between current coal seam gas and coalmining activity, impacts from drawdown are not expected. Potential effects of coal seam gas extraction and coalmining on the Bogan River supergroups are even less likely. Should coal seam gas or coalmining activities occur in closer proximity to these spring complexes, the risk assessment should be revisited.

Other impacts, such as drawdown caused by the use of flowing artesian waterbores; excavation; trampling and grazing by domestic stock, and native and feral species; and introduced pests, also pose a threat to the GAB springs. Historically, decreased aquifer pressure caused by the use of artesian waterbores has resulted in a dramatic reduction in artesian spring flows and a loss of many springs in the GAB (Fairfax & Fensham 2003). The Eulo, Bourke and Bogan River supergroups and the communities of native species dependent on their flows are more likely to be threatened by these land-use practises than by coal seam gas extraction.

Figure 67 Discharge and recharge springs of the Springsure supergroup with coal seam gas (CSG) and mine tenements in the Bowen and Surat basins.

Bore rehabilitation and regulation was suggested as the most useful strategy for the recovery of artesian springs in the Eulo and Bourke supergroups (Smerdon et al. 2012). Any successful strategy aimed at protecting GAB springs must therefore take into account the historical and local threats to particular spring supergroups and complexes.

# Concluding remarks

Identification of the source aquifer/s for the spring complexes in the Springsure supergroup was achieved with a fair degree of confidence. Ample information was available for these springs, because they have been the subject of in-depth investigation by Klohn Crippen Berger (QWC 2012c) for the Queensland Water Commission (QWC) for the Underground water impact report for the Surat Cumulative Management Area (QWC 2012a). The QWC report (2012a) provides a detailed discussion on how spring complexes have been identified as being vulnerable to coal seam gas impacts. The detailed discussion and analysis by the QWC were relied upon for informing the risk analysis.

Identification of some potential source aquifers was possible for the Eulo, Bourke and Bogan River supergroups. Our ability to identify a particular source aquifer for the springs in the Eulo supergroup was limited, because there is the potential for mixing of artesian groundwater from different aquifers before the spring water comes to the ground surface. Further limitations include the reduced amount of bore-screening data, and the similarity in hydrochemistry between all waterbores and springs. Identifying a source aquifer with any high degree of certainty for the Bourke and Bogan River supergroups was not possible due to the lack of bore water chemistry data, and sufficiently detailed and reliable stratigraphic information for the areas.

The methodology defined by Parsons Brinckerhoff (2010) allowed for a comprehensive investigation to determine the artesian source aquifer for a spring. Processes such as vertical mixing of artesian groundwater do, however, restrict the conclusions that can be drawn from some steps in the methodology, such as hydrochemistry analysis. Although the methodology involved using a variety of data and sources of information, the ability to successfully identify a spring’s source aquifer/s is heavily dependent on the availability of certain key pieces of information.

The ability to identify artesian source aquifers for springs is highly dependent on information about waterbores in the vicinity of the artesian spring. Both the type of information and quality is important. To identify a source aquifer with any confidence, information on the drillers’ logs, geological and geophysical logs, stratigraphy, bore data (including screening and casing, contributing aquifers, depth of groundwater cut in nearby waterbores), as well as the hydrochemistry of groundwater from the bores is essential. For example, in the Springsure supergroup, more information on waterbore screens was available, and the information about ‘screening’ (i.e. the interval where the slots appear in the casing or were an open hole exists below the bottom of the casing) was more specific. This made it easier to use this information— along with the interpreted stratigraphy of the bore’s drilling log, and other geological or hydrogeological information—to come to a conclusion as to what aquifer the bore was tapping.

In contrast, little screening information was available for the Eulo supergroup area. For example, the information on bore casing did not include information on screening and did not specify whether the bore was accessing water via an open hole at the bottom. The information on aquifers encountered during drilling of the bore often did not specify which aquifers were contributing to the bore. Given the Queensland 61TDepartment of Environment and Resource Management’s61T Groundwater Database had codes to specify whether a bore was accessing water via an open hole at the bottom and specify contributing aquifers, assumptions were not made regarding the contributing aquifers in the absence of this information. The bores at Eulo also passed through several aquifers, making it difficult to determine whether the bores were sourcing groundwater from one or more artesian aquifers. Without the information about what aquifer or aquifers a bore is tapping, any hydrochemistry data are less valuable, as it cannot be linked to a specific aquifer and used to identify a spring’s aquifer.

The lack of sufficiently detailed stratigraphic information made it very difficult to identify potential source aquifers for the artesian springs of the Bourke and Bogan River supergroups. No waterbores in New South Wales had interpreted stratigraphy from bore drilling logs. The New South Wales Department of Trade and Investment, Division of Resources and Energy 1:250 000 geological maps provided some interpreted stratigraphy for the area. These maps often did not indicate the presence of Great Artesian Basin aquifers, such as the Hooray Sandstone or its equivalents in the Surat Basin, including the Pilliga and Mooga Sandstones, which are known to be present in the area of the Bourke and Bogan River supergroup springs (H Haridharan 2013, pers. comm.). Stratigraphy provided from one of the stratigraphic bore in the Bourke area and stratigraphy from Queensland waterbores near the border with New South Wales was more relied on, along with published literature on the geology of the region, such as that by Habermehl (2001), Exon (1976), and Hawke and Cramsie (1984). To identify source aquifers for the Bourke and Bogan River supergroup springs in the future, more detailed information on stratigraphy and the waterbores in the area is needed.

# References

Abbott WE 1881, ‘Notes of a journey on the Darling’, Journal and Proceedings of the Royal Society of NSW, vol. 15, pp. 41–70.

Ah Chee D 2002, ‘Kwatye (water) in the Great Artesian Basin’, Environment South Australia, vol. 9, pp. 20–21.

Anderson C & Fletcher HO 1934, [title illegible] Nambour Chronicle and North Coast Advertiser, February 16, 1934, p.2.

Anon. 1858, ‘Sullivan on Cumberland: book review of J Sullivan’s 1857 Cumberland and Westmorland, ancient and modern, the people, dialects, supersitions and customs’, Blackwood’s Edinburgh Magazine vol. 83, pp. 344–56.

Anon. 1889, ‘A tramp across Australia, no. IX’, South Australian Register, 10 October, p. 3.

Anon. 1894, ‘To the far south-west. IV. The never-never country: Coongoola Station, a model homestead on the Warrego’, The Brisbane Courier, 4 June, p. 8.

Anon. 1904, Mud springs in western Queensland, The Queenslander, 19 November, p. 28.

Australian Natural Resource Atlas 2009, Water resources—management and development—Queensland, Australian Government Department of Sustainability, Environment, Water, Population and Communities84T.

Bailey J 2006, Mr Stuart's track: the forgotten life of Australia's greatest explorer, Pan Macmillan, Sydney.

Ball LC 1918, ‘A mound spring at Crystalbrook’, Queensland Government Mining Journal, vol. 19, pp. 508–9.

Bayly IAE 1999, ‘Review of how Indigenous people managed for water in desert regions of Australia’, Journal of the Royal Society of Western Australia, vol. 82, pp. 17–25.

Bedford CT 1886, ‘Reminiscences of a surveying trip from Boulia to the South Australian Border’, Royal Geographical Society of Queensland, vol. 2, pp. 99–113.

Bentley HW, Phillips FM, Davis SN, Habermehl MA, Airey PL, Calf GE, Elmore D, Gove HE & Torgersen T 1986, ‘Chlorine 36 dating of very old groundwater. The Great Artesian Basin, Australia’, Water Resources Research, vol. 22, no. 13, pp. 1991-2001.

Bourke and District Historical Society 1969, The papers presented by the members of the Bourke and District Historical Society on the history Bourke, Bourke and District Historical Society, Bourke.

Bowen R & Presey B 1993, Localities and habitats of plants with restricted distributions in the Western District of New South Wales, occasional paper no. 175, New South Wales National Parks and Wildlife Service, Sydney.

Bowman M 1998, ‘Belief, legend and perceptions of the sacred in contemporary Bath’, Folklore, vol. 109, pp. 25–31.

Boyd WE 1990, ‘Mound Springs’, in MJ Tyler, CR Twindale, M Davies & CB Wells (eds), Natural history on the north east deserts, Royal Society of South Australia, Adelaide, pp. 107–18.

Brunker RL 1971, Bourke 1:250 000 geological map, sheet SH/55-10, 1st edn, Geological Survey of New South Wales, Sydney.

Cathcart M 2009, The water dreamers: the remarkable history of our dry continent, Text Publishing, Melbourne.

Chapelle FH 1997, The hidden sea: ground water, springs, and wells, Geoscience Press, Tuscon, Arizona.

CoA 2014, Ecological and hydrogeological survey of the Great Artesian Basin springs—Springsure, Eulo, Bourke and Bogan River supergroups, Knowledge project, Volume 2: hydrogeological profiles, report prepared by UniQuest for the Department of the Environment, Commonwealth of Australia.

David TW 1891, ‘Artesian water in New South Wales’, Journal and Proceedings of the Royal Society of New South Wales, vol. 25, pp. 286–96.

Davies RJ-P, Craigie AI, Mackay DA, Whalen MA, Cheong JP-E & Leach GJ 2007, ‘Resolution of the taxonomy of Eriocaulon (Eriocaulaceae) taxa endemic to the Australian mound springs, using morphometrics and AFLP markers’, Australian Systematic Botany, vol. 20, pp. 428—47.

Davis JA, Harrington SA & Friend JA 1993, ‘Invertebrate communities of relict streams in the arid zone: the George Gill range, central Australia’, Australian Journal of Marine and Freshwater Research, vol. 44, pp. 483–505.

De Deckker P 1986, ‘What happened to the Australian aquatic biota 18 000 years ago?’, in P De Deckker & WD Williams (eds), Limnology in Australia, CSIRO, Australia, pp. 487–96.

Deveson B 2004, ‘The origin of precious opal’, Australian Gemmologist, vol. 22, pp. 50–8.

Dickson DR 1987, The fountain of living waters: the typology of the waters of life in Herbert, Vaughan, and Traherne, University of Missouri Press, Columbia.

DERM 2010, Managing impacts of CSG extraction on underground water in the Great Artesian Basin, Queensland Department of Environment and Resource Management, Brisbane, accessed 25 March 2013, <www.ehp.qld.gov.au/factsheets/pdf/water/w198.pdf>.

DERM 2012, DERM Groundwater Database, information received November 2012, Queensland Department of Environment and Resource Management, Brisbane.

DNR 1996, Report on the impact of Nathan Dam on boggomosses and regional hydrology, Resource Sciences Centre, Resource Condition and Trend Unit, Water and Assessment Planning, Queensland Department of Natural Resources, Brisbane.

DNRM 2005, Hydrogeological framework report for the Great Artesian Basin water resource plan area, version 1.0, Queensland Department of Natural Resources and Mines, Queensland Government.

Dowden K 2000, European paganism: the realities of cult from antiquity to the Middle Ages, Routledge, New York.

Duncan-Kemp AM 1934, Our sandhill country: man and nature in south-western Queensland, 2nd edn, Angus & Robertson, Sydney.

Durack M 1959, Kings in grass castles, Transworld Publishers, Sydney.

Edwards, E. 1911 Artesian bore reports. Item ID 291260, Queensland State Archive.

Edwards E 1912, Artesian bore reports. Item ID 291266, Queensland State Archive.

EHA 2009, Identification of source aquifers to significant springs dependent on groundwater flow from the Great Artesian Basin, report prepared by Environmental Hydrology Associates for the Queensland Department of Environment and Resource Management.

Exon NF 1976, Geology of the Surat Basin in Queensland, Bureau of Mineral Resources, Geology and Geophysics, Australia. Bulletin 166.

Fatchen, T 2001, Vegetated area as an index of mound spring flows’, Proceedings of the 4PthP Mound Spring Researchers Forum, National Parks and Wildlife, Adelaide, South Australia, 23 Feb 2001, pp. 5-8.

Fairfax RJ & Fensham RJ 2002, ‘In the footsteps of J Alfred Griffiths: a cataclysmic history of Great Artesian Basin springs in Queensland’, Australian Geographical Studies, vol. 40, pp. 210–30.

Fairfax RJ & Fensham RJ 2003, ‘Great Artesian Basin springs in southern Queensland 1911–2000’, Memoirs of the Queensland Museum, vol. 49, pp. 285–93.

Fairfax R, Fensham R, Wager R, Brook S, Webb A & Unmack P 2007, ‘Recovery of the red-finned blue-eye: an endangered fish from springs of the Great Artesian Basin’, Wildlife Research, vol. 34, pp. 156–66.

Fatchen T 2000, Mound springs management planning. Management issues, strategies and prescriptions for Mound Springs in far north South Australia, report prepared for the South Australian Department for Environment and Heritage with the support of the Australian Government’s Natural Heritage Trust.

Fensham RJ & Fairfax RJ 2003, ‘Spring wetlands of the Great Artesian Basin, Queensland, Australia’, Wetland Ecology and Management, vol. 11, pp. 343–62.

Fensham RJ, Fairfax RJ & Sharpe PR 2004, ‘Spring wetlands in seasonally arid Queensland: floristics, environmental relations, classification and conservation values’, Australian Journal of Botany, vol. 52, pp. 583–95.

Fensham RJ, Pennay C & Drime J 2012, Ecological and botanical survey of springs in the Surat cumulative management area, prepared for the Queensland Water Commission, Queensland Herbarium, Brisbane.

Fensham RJ, Ponder WF & Fairfax RJ 2010, Recovery plan for the community of native species dependent on natural discharge of groundwater from the Great Artesian Basin, report to the Australian Government Department of the Environment, Water, Heritage and the Arts, Queensland Department of Environment and Resource Management, Brisbane.

Fensham RJ, Powell O & Horn J 2011a, ‘Rail survey plans to remote sensing: vegetation change in the Mulga Lands of eastern Australia and its implications for land use’, The Rangeland Journal, vol. 33, pp. 229–38.

Fensham RJ, Silcock JL, Kerezsy A & Ponder W 2011b, ‘Four desert waters: setting arid zone wetland conservation priorities through understanding patterns of endemism’, Biological Conservation, vol. 144, pp. 2459–67.

Field JH 2006, ‘Trampling through the Pleistocene: does taphonomy matter at Cuddie Springs?’, Australian Archaeology, vol. 63, pp. 9–15.

Field JH, Dodson JR & Prosse IP 2002, ‘A late Pleistocene vegetation history from the Australian semi-arid zone’, Quaternary Science Reviews, vol. 21, pp. 1023–7.

Finlayson HH 1935, The Red Centre: man and beast in the heart of Australia, Angus & Robertson, Sydney.

GA 2012, Stratigraphic Units Database, online database, Geoscience Australia, Canberra, <http://dbforms.ga.gov.au/pls/www/geodx.strat_units.int>.

Great Artesian Basin Consultative Council (GABCC) 1998, ‘Great Artesian Basin Resource Study (November 1998)’, Great Artesian Basin Consultative Council, Canberra.

GABCC 2000, Great Artesian Basin: strategic management plan, Great Artesian Basin Consultative Council, Canberra.

Gibbs LM 2006, ‘Valuing water: variability and the Lake Eyre Basin, central Australia’, Australian Geographer, vol. 37, pp. 73–85.

Gilliat HA 1885, ‘Royal Commission of Water: notes on the mud springs and some shafts and borings of underground supply on the road between Ford’s Bridge (on the Warrego River) and Hungerford (on the Paroo River)’, Votes and Proceedings of the Legislative Council, vol. 6, pp. 827–28.

Glover CJM 1989, ‘Fishes’, in W Zeidler & WF Ponder (eds), Natural history of Dalhousie Springs, South Australian Museum, Adelaide.

Godwin L 2001, ‘The bureaucracy, the law and Black’s Palace: a history of management of one site in the central Queensland highlands’, Ngulaig, vol. 18, pp. 1–26.

Habermehl M A 1980 ‘The Great Artesian Basin, Australia’, BMR Journal of Australian Geology & Geophysics, vol 5, pp. 9 – 31.

Habermehl MA 1982, Springs in the Great Artesian Basin, Australia: their origin and nature, Bureau of Mineral Resources, Geology and Geophysics, Canberra.

Habermehl MA 2001, Wire-line logged waterbores in the Great Artesian Basin: digital data of logs and waterbore data acquired by AGSO, Bureau of Rural Science, Canberra.

Habermehl MA 2002, Hydrogeology, hydrochemistry and isotope hydrology of the Great Artesian Basin, Bureau of Rural Sciences, Canbera.

Harris C 2002, ‘Culture and geography: South Australia's mound springs as trade and communication routes’, Historic Environment, vol. 18, pp. 8–11.

Hawke JM & Cramsie JN (eds) 1984, ‘Contributions to the geology of the Great Australian Basin in New South Wales’, Geological Survey of New South Wales, Bulletin 31.

Herczeg A, Torgersen T, Chivas A & Habermehl M 1991, ‘Geochemistry of ground waters from the Great Artesian Basin, Australia’, Journal of Hydrogeology, vol. 126, pp. 225–45.

Hodgkinson WO 1877, North-west explorations, report to Parliament, Government Printer, Brisbane.

Holdaway SJ, Fanning PC & Witter D. 2000, ‘Prehistoric Aboriginal occupation of the rangelands: interpreting the surface archaeological record of far western New South Wales’, Rangeland Journal, vol. 22, pp. 44–57.

Horwitz P, Rogan R, Halse S, Davis J & Somme B 2009, ‘Wetland invertebrate richness and endemism on the Swan Coastal Plain, Western Australia’, Marine and Freshwater Research, vol. 60, pp. 1006–20.

Ingram JA & Senior DA 1970, Toompine, Queensland, 1:250 000 geological series map, sheet SG/55-13, 1st edn, Bureau of Mineral Resources, Australia.

Jack RL 1900, Artesian water in the state of Queensland, Australia, Harrison and Sons, London.

Jensen HI 1926, ‘Geological reconniassiance betwenn Roma, Springsure, Tambo, Taroom’, Quensland Geological Survey, vol. 277, pp. 97–114.

Jowett A & Christie P 2007, ‘Results of a search for Eriocaulon carsonii on Peery Lake at Paroo-Darling National Park’, unpublished report to the NSW Department of Environment, Climate Change and Water.

Junk WJ 2006, ‘The comparative biodiversity of seven globally important wetlands’, Aquatic Sciences, vol. 68, p. 239.

Kanso S & Patel BKC 2003, ‘Microvirga subterranean gen. nov., a moderate thermophile from a deep subsurface Australian thermal aquifer’, International Journal of Systematic and Evolutionary Microbiology, vol. 53, pp. 401–6.

Keith DA 2000, ‘Sampling designs, field techniques and analytical methods for systematic plant population surveys’, Ecological Management and Restoration, vol. 1, pp. 125–39.

Kelley JE 1882, ‘Well sinking, water storing, and irrigation in the interior’, Sydney Morning Herald, 1 May, p. 8.

Kuske T, van der Wielen S, Dunsmore R, Ransley T, Draper J & Feitz AJ 2010, Hydrochemical baseline mapping of the Great Artesian Basin groundwater, Geoscience Australia, Canberra.

Love A, Karlstrom K, Crossey L, Shand P, Rousseau-Gueutin P & Priestley S 2010, ‘Developing a new model for the Great Artesian Basin of Australia: hydrologic mixing, multi-scale flow systems, fault-partitioned sub-basins, and mantle influences on groundwater quality, superimposed on regional flow systems’, in National Groundwater Conference 2010 Proceedins. Groundwater 2010: the challenge of sustainable management, Canberra, ACT, Australia (206) 31October – 4 November 2010.

Madigan CT 1946, Crossing the dead heartI, Georgian House, Melbourne.

Mahara Y, Habermehl MA, Hasegawa T, Nakata K, Ransleu TR, Hatano T, Mizuochi Y, Kobayashi H, Ninomiya A, Senior BR, Yasuda H & Ohata T 2009, ‘Groundwater dating by estimation of groundwater flow velocity and dissolved P4PHe accumulation rate calibrated by P36PCl in the Great Artesian Basin, Australia’, Earth and Planetary Science Letters, vol. 287, p. 43–56.

McDonald E, Coldrick B & Villiers L 2005, Study of groundwater-related Aboriginal cultural values of the Gnangara Mound, Western Australia’, report for the Department of Environment, Estill & Associates, Perth.

McKellar H 1984, Matya-Mundu: a history of the Aboriginal people of south-west Queensland, Cunnamulla Native Welfare Association, Cunnamulla.

Meakin S, Henley H, Pecover S & Watkins K 1996, Walgett 1:250 000 geological map, sheet SH/55-11, 1st edn, Geological Survey of New South Wales, Sydney.

Mitchell TL 1847, Journal of an expedition into tropical Australia in search of a route from Sydney to the Gulf of Carpentaria, Longman, Brown, Green, and Longmans, London, available from the University of Adelaide Library Electronic Texts Collection, Adelaide.

Mulvaney D & Joyce E 1965, ‘Archaeological and geomorphological investigations on Mt Moffat Station, Queensland, Australia’, Proceedings of the Prehistoric Society, vol. 31, pp. 147–212.

Murphy NP, Guzik MT & Worthington Wilmer J 2010, ‘The influence of landscape on population structure of four invertebrates in groundwater springs’, Freshwater Biology, vol. 55, pp. 2499–2509.

Nameerf D 1881, ‘From the Balonne to the Bulloo in search of droving’, The Queenslander, 19 March, p. 365.

NSW DTIRE 2004, Major geological boundaries and structures 02/11/2004, Geological Survey of New South Wales, NSW Department of Trade, Investment, Resources and Energy, MinView.

NSW OoW 2010, PINNEENA version 3.2 Groundwater works, New South Wales Office of Water, Department of Environment, Climate Chante and Water, Sydney.

Noble JC, Habermehl MA, James CD, Landsberg J, Langston AC & Morton SR 1998, ‘Biodiversity implications of water management in the Great Artesian Basin’, The Rangeland Journal, vol. 20, 275–300.

Nolan C 2003, Sandhills and channel country, Diamantina Shire Council, Bedourie.

NSW NPWS 2003, Salt pipewort (Eriocaulon carsonii) recovery plan, Approved recovery plan, New South Wales National Parks and Wildlife Service, Hurstville.

Ogilvie, C. 1912 Artesian bore reports. Item ID 291266, Queensland State Archive.

Ogilvie C 1945, ‘The hydrology of the Queensland portion of the Great Australian Artesian Basin’, in Artesian water supplies: first interim report, pp. 23–61.

Parsons Brinckerhoff Australia Pty Ltd 2010, Manual for the identification of source aquifers to significant springs dependent on groundwater flow from the Great Artesian Basin, Queensland Department of Environment and Resource Management, Brisbane.

Paterson AB 1896, ‘Song of the artesian water’, The Bulletin, 12 December, p. 10.

Percy A 1906, The drought antidote for the north-west, or, Utilisation of the artesian resources of New South Wales, Government Printer, Sydney.

Pickard J 1992, Artesian springs in the Western Division of New South Wales, Graduate School of the Environment, Macquarie University, Sydney.

Ponder, WF 1986, ‘Mound springs of the Great Artesian Basin’ in P De Dekker & WD Williams (eds.), Limnology in Australia, CSIRO, Melbourne & Dr W. Junk Publishers, Dortrecht, pp. 403-420.

Ponder WF 1995, ‘Mound spring snails of the Great Artesian Basin’, in KE Alison (ed.), The conservation biology of molluscs, World Conservation Union, pp. 13–18.

Ponder WF 2004, ‘Endemic aquatic macroinvertebrates of artesian springs of the Great Artesian Basin—progress and future directions’, Records of the South Australian Museum Monograph Series, vol. 7, pp. 101–10.

Ponder WF & Clark GA 1990, ‘A radiation of hydrobiid snails in threatened atresian springs in western Queensland’, Records of the Australian Museum, vol. 42, pp. 301–63.

Ponder WF, Hershler R & Jenkins B 1989, ‘An endemic radiation of hydrobiid snails from artesian springs in northern South Australia: their taxonomy, physiology, distribution and anatomy’, Malacologia, vol. 31, pp. 1–140.

Powell O 2012, ‘Song of the artesian water: aridity, drought and disputation along Queensland’s pastoral frontier in Australia’, Rangeland Journal, vol. 34, pp. 305–17.

Quarantotto P 1989, Hydrogeology of the Surat Basin, Queensland, Geological survey of Queensland, QDM record 1989 26, Queensland Department of Mines, Brisbane.

QWC 2012a, Underground water impact report for the Surat cumulative management area, Queensland Water Commission, Brisbane.

QWC 2012b, Assessment of risks and potential consequences to springs in the Surat cumulative management area, Queensland Water Commission, Brisbane.

QWC 2012c, Hydrogeological attributes associated with springs in the Surat cumulative management area, Queensland Water Commission, Brisbane.

Rade J 1954, ‘Geology and sub-surface waters of the area north of the Darling River between longitudes 145Po Pand 149PoPE, New South Wales’, Journal & Proceedings of the Royal Society of New South Wales, vol. 88, pp. 24–32.

Rade J 1955, ‘Geology and sub-surface waters of the Coonamble Basin, New South Wales’, Journal & Proceedings of the Royal Society of New South Wales, vol. 88, pp. 77–88.

Radke BM, Ferguson J, Creswell RG, Ransely TR & Habermehl MA 2000, Hydrochemistry and implied hydrodynamics of the Cadna-owie-Hooray Aquifer Great Artesian Basin, Bureau of Rural Sciences, Canberra.

Robins RP 1993, ‘Archaeology and the Currawinya Lakes: towards a prehistory of arid lands of southwest Queensland’, PhD thesis, Griffith University.

Robins RP 1997, ‘Patterns in the landscape: a case study in nonsite archaeology from southwest Queensland’, Memoirs of the Queensland Museum, Cultural heritage series, vol 1, pp. 23-56.

Robins RP 1998, ‘Archaeological investigations at Youlain Springs, southwest Queensland’, Memoirs of the Queensland Museum, Cultural heritage series, vol. 1, pp. 57–74.

Robins RP 1999, ‘Clocks for rocks: an archaeological perspective on the Currawinya Lakes’, in RT Kingsford (ed.), A free-flowing river: the ecology of the Paroo River, National Parks and Wildlife Service, Sydney.

Robins RP & Swanson CE 2006, ‘Of fairy rings and telegraph poles: the importance of accounting for evidence of absence in archaeological surveys’, in S Ulm (ed.), An archaeological life: papers in honour of Jay Hall’, University of Queensland Aboriginal and Torres Strait Islander Studies Unit, Brisbane, pp. 73–88.

Schmiechen J 2004, Lake Eyre Basin heritage tourism: future directions, Lake Eyre Basin Coordinating Group, <www.lakeeyrebasin.org.au/archive/media/future_directions.pdf>.

Schwalbaum WJ 1997, Understanding groundwater, Nova Science Publishing, New York.

Senior D & Thomas BM 1971, Cunnamulla, Queensland, 1:250 000 geological series map, sheet SH/55-02, 1st edn, Bureau of Mineral Resources, Australia.

Shaw R 1990, The seismo-tectonics of southeastern Queensland, Bureau of Mineral Resources, Geology and Geophysics, Canberra.

Silcock J 2009, Identification of permanent refuge waterbodies in the Cooper Creek and Georgina-Diamantina catchments, South Australia Arid Lands Natural Resource Management Board, Adelaide.

Silcock JL, Fensham RJ & Martin TG 2011, ‘Assessing rarity and threat in an arid zone flora’, Australian Journal of Botany, vol. 59, pp. 336–50.

Smerdon BD, Ransley TR, Radke BM & Kellett JR 2012, Water resource assessment for the Great Artesian Basin, report to the Australian Government from the CSIRO Great Artesian Basin Water Resource Assessment, CSIRO Water for a Healthy Country Flagship, Australia.

Smith CF 1912, ‘Summer rain and springs of water in Greek and Hebrew poetry’, The Classical Journal, vol. 8, pp. 17–24.

Stewart AJ, Raymond OL, Totterdell JM, Zhang W & Gallagher R 2013, Australian geological provinces, 2013.01 edition, digital dataset, Geoscience Australia, Canberra, <www.ga.gov.au>.

Strang V 2004, The meaning of water, Berg, Oxford.

Torgersen T, Clark WB & Haberhehl MA 1987, ‘Helium isotopic evidence for recent subcrustal volcanism in eastern Australia’, Geophysical Research Letters, vol. 14, no. 12, pp. 1215–8.

Torgersen T, Habermehl MA & Clarke WB 1992, ‘Crustal helium fluxes and heat flow in the Great Artesian Basin, Australia’, Chemical Geology: Isotope Geoscience section, vol. 102, pp. 139–52.

Torgersen T, Haberhehl MA, Phillips FM, Elmore D, KubilP, Jones BG, Hemmick T & Gove HE 1991, ‘Chlorine-36 dating of very old groundwater 3. Further studies in the Great Artesian Basin, Australia’, Water Resources Research, vol. 27, no. 12, pp. 3201–13.

Trueman CNG, Field JH, Dortch J, Charle B & Wroe S 2005, ‘Prolonged existence of humans and megafauna in Pleistocene Australia’, PNAS, vol. 102, pp. 8381–85.

Veth PM 1993, Islands in the interior: the dynamics of prehistoric adaptations within the arid zone of Australia, International Monographs in Prehistory, Michigan.

Wager R 1995, Recovery plan for Queensland artesian springs fishes, Australian Nature Conservation Agenncy Endangered Species Program, Australian Nature Conservation Agency, Canberra.

Watkins JJ & Meakin NS 1996, Nyngan and Walgett 1:250 000 geological maps, sheets SH/55-15 and SH55-11: Explanatory Notes, vii +112pp. Geological Survey of New South Wales, Sydney.

Witter DC 2004, ‘Regional variation of the archaeology in western New South Wales’, Rangeland Journal, vol. 26, pp. 129–49.

Worthington Wilmer JW, Elkin C, Wilcox C, Murray L, Niejalke D & Possingham H 2008, ‘The influence of multiple dispersal mechanisms and landscape structure on population clustering and connectivity in fragmented artesian spring snail populations’, Molecular Ecology Notes, vol. 17, pp. 3733–51.

Yu S 2002, ‘Ngapa kunangkul (living water): an Indigenous view of groundwater’, in A Gaynor, M Trinca & A Haebich (eds), Country: visions of land and people in Western AustraliaI, Western Australian Museum, Perth.

Zeidler W 1997, ‘A new species of freshwater amphipod, Austrochiltonia dalhousiensis sp. nov., (Crustacea: Amphipoda: Hyalellidae) from Dalhousie Springs, South Australia’, Transactions of the Royal Society of South Australia, vol. 121, pp. 29–42.

# Appendix A:	Metadata for Queensland Springs Database

Dataset COMPILERS

C Pennay, J Drimer, O Powell, R Fensham, R Rossini

Dataset DATE

March 2013

Dataset CUSTODIAN

Queensland Herbarium, Department of Science, Information Technology, Innovation and the Arts

Dataset JURISDICTION

Queensland

Description ABSTRACT

The information in this dataset is an inventory of permanent springs within the Eulo, Springsure, Bourke and Bogan River regional supergroups (Habermehl 1982) of springs in the Great Artesian Basin in Queensland. The dataset is current at March 2013, and includes information for all springs on: location; status (active/inactive); grouping (complex), and where available information on: physical properties; general morphology; water chemistry; floristic composition; disturbance; faunal composition; survey effort; photographic documentation; historical descriptions. Where sufficient information was available, springs have been given a conservation ranking at both the individual spring and complex levels.

Description SEARCH WORD(S)

Spring, spring wetland, wetland, Queensland, Great Artesian Basin, GAB

Description GEOGRAPHIC EXTENT NAMES(S)

Access STORED DATASET FORMAT(S)

Point locations

PROJECTION: Universal Transverse Mercator (UTM)

DATUM: Geocentric Datum of Australia (GDA94) or Australian Geodetic Datum (AGD84)

ACCESS AVAILABLE FORMAT TYPE(S)

Microsoft Access database

Data Quality LINEAGE

Table A1	Site details

| Field name | Description | Allowed values |
| --- | --- | --- |
| Site number | Unique identifier for spring wetland | >0 |
| Preliminary site number | Preliminary unique identifier given to spring wetlands during field survey | >0 (mandatory) |
| Data source | Source of data for individual spring wetland | text, blank |
| Date | Most recent date spring wetland visited (see Historical data for a list and report from all previous visits) | text, blank |
| Recorders | Name of person(s) who recorded data | text, blank |
| Site name | Unique site name | text, blank |
| Property name | Name of property/pastoral station/national park on which spring wetland is located | text, blank |
| Flora species list | If TRUE there is a link to a list of plant taxa associated with this spring wetland, a list of species records for a spring wetland is necessary to undertake a conservation ranking. This can contain data from numerous trips. If the spring has not been surveyed yet, or is inactive must equal FALSE. | true/false |
| Adjacent environment RE | Field description of vegetation adjoining spring wetland including regional ecosystem (RE) code(s). For a full description of regional ecosystem codes refer to www.derm.qld.gov.au/wildlife-ecosystems/biodiversity/regional_ecosystems/index.php | text, blank |
| Spring RE | Regional ecosystem (RE) code for the spring wetland. For a full description of regional ecosystem codes refer to www.derm.qld.gov.au/wildlife-ecosystems/biodiversity/regional_ecosystems/index.php | single RE code |
| Notes | Additional information | text, blank |
| Complex name | Name of spring complex to which spring wetland belongs. A complex represents a group of springs or spring groups such that no adjacent pair of springs or spring-groups is more than about 6km distant and all springs within the spring complex are in a similar geomorphic setting. Geomorphic setting includes geological unit, landform, landscape position or soil type. Complexes can contain both active and inactive springs. | text, blank |
| Complex number | Unique numerical identifier for spring complex | any integer >0, blank |
| Supergroup name | Name of spring supergroup to which spring wetland belongs. A supergroup represents a major regional cluster of spring complexes with some consistent hydrogeological characteristics as defined by Habermehl (1982), Ponder (1986), GABCC (1998) and Fensham et al. (2003) | text, blank |
| Discharge | A spring emanating from the Great Artesian Basin (GAB) aquifers other than those within outcrop areas of the following sandstone formations on the eastern margins of the GAB: Adori, Blantyre, Boxvale, Clematis, Expedition, Gilbert River, Gubberamunda, Hampstead, Hooray, Hutton, Mooga, Namur, Nullawurt, Precipice, Springbok, Warrang and Wyandra sandstones, the Bulimba, Bungil, Digby, Eurombah, Glenidal, Pilliga and Murta formations, and the Albany Pass, Drildool, Helby, Kumbarilla and Ronlow beds. | true/false |
| Active | The spring wetland is actively expressing water to maintain the spring wetland in a damp condition | true/false |
| GAB | Spring wetland derived from aquifers associated with Great Artesian Basin (GAB) | true/false |
| Region | Non-GAB Region (EG Carnarvon Basalt) | text, blank |
| Latitude (decimal degrees) | Latitude, recorded as decimal degrees to 7 decimal places (0.01 m accuracy). Negative values indicate location in southern hemisphere | (mandatory) |
| Longitude (decimal degrees) | Longitude, recorded as decimal degrees to 7 decimal places (0.01 m accuracy). Positive values indicate location in eastern hemisphere | (mandatory) |
| Horizontal coordinate system | Coordinate system used for horizontal location. 1. Australian Geodetic Datum of 1984 (AGD84), 2. Map Grid of Australia 1994 (MGA1994) | AGD84, MGA1994 |
| Zone | Zone within Map Grid of Australia 1994 (MGA1994) coordinate system | 54, 55, 56 |
| Easting | Metres east within zone, recorded to 2 decimal places (0.1 m accuracy), MGA1994 coordinate system | (mandatory) |
| Northing | Metres north within zone, recorded to 2 decimal places (0.1 m accuracy), MGA1994 coordinate system | (mandatory) |
| Location precision | Indication of the precision (accuracy) of location data (horizontal coordinates, latitude and longitude). 1. OmniSTAR Differential GPS has a mean horizontal positional error of ± 0.1 m. 2. Handheld GPS units have a mean horizontal positional error of ± 10 m. 3. Other measures of location have been given a horizontal positional error range from 0.1–10 km. 4. Some spring wetland locations have had their location masked to protect populations of endangered species, these have been given a horizontal positional error of >10 km. | OmniSTAR Differential ~0.1 m accuracy, Garmin handheld ~10 m accuracy, Other 0.1–10 km accuracy, Masked >10 km accuracy |
| Vertical coordinate system | Australian Height Datum (AHD) | Australian Height Datum (AHD), blank |
| Elevation—AHD (metres) | Mean elevation referenced to Australian Height Datum (AHD). Calculated from ellipsoidal height using AUSGeoid09 V1.01 (11 April 2011) www.ga.gov.au/earth-monitoring/geodesy/geodetic-datums/geoid.html (accessed 27/09/2011). Recorded in metres to 2 decimal places (0.1 m accuracy). | >0, blank |
| Dominant surface composition | Field description of the dominant surface composition of the vent. 1. Peat, 2. Mud (exuded), 3. Rocky seep (fractured), 4. Sand/Silt, 5. Carbonate (travertine), 6. Water/soak, 7. Other | Peat, Mud (exuded), Rocky Seep (fractured), Sand/Silt, Carbonate (travertine), Water/soak, Other, blank |
| General morphology | Field description of the general morphology of the vent. Mound, Flat, Closed depression (concave), Open depression (watercourse) bed, Open depression (watercourse) bank, Terraced, Other | Mound, Flat, Closed Depression (concave), Oped Depression (watercourse) bed, Open Depression (watercourse) bank, Terraced, Other, blank |
| Mound length (metres) | Visual estimate of the length (metres) of mound | >0, blank |
| Mound width (metres) | Visual estimate of the width (metres) of mound | >0, blank |
| Relative mound height (metres) | Visual estimate of the relative height (metres) of mound to the adjoining non-mounded area | >0, blank |
| Soak Length (metres) | Visual estimate of the length (metres) of the saturated spring wetland | >0, blank |
| Soak width (metres) | Visual estimate of the width (metres) of the saturated spring wetland | >0, blank |
| Spring wetland area (square metres) | Area (square metres) covered by ≥50% spring wetland vegetation | >0, blank |
| Area derivation | Method used to calculate ‘Spring wetland area’. (a) Differential GPS–GIS polygon. Field traverse of the spring wetland boundary (defined as having >50% cover of perennial wetland plant species) using differential GPS to generate a GIS polygon feature for which an area could be calculated using Albers equal areas projection. (b) Visual estimate—multiplication of values recorded for spring wetland length and width | Differential GPS–GIS polygon, visual estimate, blank |
| Estimated spring flow (L/min) | Estimated discharge rate (litres per minute) of water from spring wetland | >0, blank |
| Flow derivation | Method used to calculate ‘Estimated spring flow’. (a) Fatchen model based on vegetated area (spring wetland area), for details refer to Fatchen (2001) Vegetated wetland area as an index of mound spring flows. Proceedings of the 4th Mound Spring researchers forum Friday 23 February 2001, Department of Environment and Heritage, Adelaide, pp. 5–8, accessed 10 March 2008, www.gabcc.org.au/tools/getFile.aspx?tbl=tblContentItem&id=47.(b) Visual estimate. Field estimate of visible flow rate. | Fatchen, visual estimate, blank |
| Water sample taken | Field water sample collected from site if free water was available. If TRUE, links to a separate water chemistry database containing estimates of all water chemistry measures. This database can contain measurements on numerous dates. | true/false |
| Morphological type (landscape situation) | Field description of the landscape situation of the spring wetland using morphological types as defined in CSIRO (2009) | crest, hillock, ridge, simple slope, upper slope, mid-slope, lower slope, flat, open depression, closed depression, blank |
| Landform (landscape) element | Field description of landform element (landform with 20-m radius) of the spring wetland using landform element types described and defined in CSIRO (2009, pp. 34–35) | Refer to CSIRO (2009) |
| Erosional landform pattern | Field description of erosional pattern (landform with 300-m radius) of the spring wetland using erosional landform pattern classes described in CSIRO (2009, pp. 47–48) | Refer to CSIRO (2009) |
| Landform pattern 2 | Field description of landform pattern (landform with 300-m radius) of the spring wetland using landform pattern types described and defined in CSIRO (2009. p. 49) | Refer to CSIRO (2009) |
| Surface geology | Field description of surface geology with reference to 1:250,000 geology mapping units | text, blank |
| Excavation damage (proportion) | Visual estimate of excavation damage recorded as one of five classes based on proportion of wetland area affected | 0=none; 1=adjacent to spring wetland; 2=spring wetland less than 50% effected; 3=spring wetland more than 50% effected, but not totally eradicated; 4=spring wetland totally eradicated, blank |
| Excavation damage (type) | Description of excavation damage to site. 1. Bored, 2. Milled, 3. Dammed, 4. Drained, 5. Boxed well, 6. Pumped, 7. Other | text, blank |
| Pig damage | Visual estimate of pig damage recorded as one of four classes based on proportion of wetland area affected | 0=absent; 1<10% affected; 2=10–50% affected; 3=>50% affected, blank |
| Stock damage | Visual estimate of stock damage recorded as one of four classes based on proportion of wetland area affected | 0=absent; 1<10% affected; 2=10–50% affected; 3=>50% affected, blank |
| Macroinvertebrate survey | Five-minute sweep of vegetated spring wetland area with 15-cm sieve (1 mm mesh). If TRUE, links to a separate macroinvertebrate database containing abundance data for all macroinvertebrates on all sampling occasions. | true/false |
| Molluscs survey | Where surveyed a five-minute sweep of vegetated spring wetland area with 15 cm sieve (1 mm mesh). If molluscs have been found and the macroinvertebrate survey conducted see previous field for abundance information. | Surveyed and found, surveyed and not found, not surveyed |
| Fish survey | Presence or absence of fish gauged by visual investigation | Surveyed and found, surveyed and not found, not surveyed |
| Inundation frequency | Known or inferred inundation history, based on information from landholders and floristic composition of spring wetland (perennial obligate wetland species) | Ephemeral, Intermittent, Seasonal, Permanent, Unknown, blank |
| Spring endemic | Presence in spring wetland of flora and/or fauna taxa endemic to GAB springs | true/false |
| Endemic details | List of flora and/or fauna taxa present in spring wetland endemic to GAB springs. For a full list of flora you can also link to the Flora database, connected to the field ‘Flora species list’ | List of endemic species, blank |
| Scald endemic | List of flora taxa present in any scald associated with the spring | List of endemic species, blank |
| EPBC community | Spring wetland falls within the definition of The community of native species dependent on natural discharge of groundwater from the Great Artesian Basin listed as a threatened ecological community under the Environment Protection and Biodiversity Conservation Act 1999 (Cwlth) | true/false |
| EPBC spp. | Presence in spring wetland of flora and/or fauna taxa listed as threatened under the Environment Protection and Biodiversity Conservation Act 1999 (Cwlth) | TRUE/FALSE |
| EPBC spp. detail | Presence in spring wetland of flora and/or fauna taxa listed as endangered or vulnerable or near threatened under the Environment Protection and Biodiversity Conservation Act 1999 (Cwlth) | List of EPBC species, blank |
| NCA spp. | Presence in spring wetland of flora and/or fauna taxa listed as endangered or vulnerable or near threatened under the Nature Conservation Act 1992 (Qld) | true/false |
| NCA spp. detail | List of flora and/or fauna taxa present in spring wetland and listed as endangered or vulnerable under the Nature Conservation Act 1992 (Qld) | List of NCA species, blank |
| Invasive species | Presence in spring wetland of invasive flora and/or fauna (i.e. Invasive species refers to flora and fauna species that have adetrimental effect on spring wetlands and associated biological values) | true/false |
| Invasive species detail | List of invasive flora and/or fauna taxa present in spring wetland | List of invasive species, blank |
| Disjunct/isolated population | Occurrence of populations of plant and/or animal taxa not known from habitat other than spring wetlands within 250 km, listed in Appendix R. Based on confirmed species records held in the Queensland Herbarium HERBRECS database | true/false |
| Conservation Ranking 2012 (vent level) | Spring wetland conservation ranking applied at individual spring wetland/vent level as per the following ruleset: Category 1a: Contains at least one GAB endemic species not known from any other location beyond this spring complex. Category 1b: Contains endemic species known from more than one spring complex; or has populations of threatened species listed under state or Commonwealth legislation that do not conform to Category 1a. Category 2: Provides habitat for populations of plant and/or animal species not known from habitat other than spring wetlands within 250km. Category 3: Spring wetland vegetation without isolated populations (Category 2) with at least one native plant species that is not a weedy cosmopolitan plant species (Appendix R). Category 4: a) Spring wetland vegetation comprised of exotic and/or only native weedy cosmopolitan wetland plant species (Appendix R) or a spring devoid of vegetation or b) the original spring wetland is destroyed by impoundment or excavation. The probability of important biological values being identified in the future is very low. No other wetland species. Category 5: All springs inactive. Conservation ranking can only be applied to spring wetlands which have associated flora and fauna records | 1a, 1b, 2, 3, 4a, 4b, 5 |
| Conservation Ranking rationale 2012 (vent level) | Detail of the attribute which results (trigger) in the spring receiving a particular conservation rank. Refer to field Conservation Ranking 2012 (Vent Level) | text, blank |
| Conservation Ranking 2012 (complex level) | Spring wetland conservation ranking aggregated to the spring complex level. All spring wetlands within a spring complex are given the highest rank (Conservation Ranking (spring wetland/vent)) achieved by any constituent spring wetlands | 1a, 1b, 2, 3, 4a, 4b, 5 |
| Conservation Ranking rationale 2012 (complex level) | Detail of the attribute which results (trigger) in the spring complex receiving a particular conservation rank. Refer to fields Conservation Ranking 2012 (complex level) and Conservation Ranking 2012 (vent level) | text, blank |
| Additional Info | Directory path to the folder containing all additional information regarding this spring (e.g. Hydrogeological report, historical information, survey plans) | text, blank |

Table A2 Photo details

| Field name | Description | Allowed values |
| --- | --- | --- |
| Site number | Unique identifier for spring wetland | >0 (mandatory) |
| Preliminary site number | Preliminary unique identifier given to spring wetlands during field survey | >0 |
| Description | Caption describing photo (keep short, less than 100 characters) | text, blank |
| Date taken | Date photo was taken (can be gathered from EXIF data for recent digital camera images), for older photos where exact date not known default to the 1st day of known month or 1st day of known year | text, blank |
| File name | Unique identifier for site photo; full file name including extension (e.g. QWC_1044.jpg) | text (mandatory) |
| Locality | Brief text describing locality (e.g. property name) | text, blank |
| Direction | Azimuth of photo direction measured clockwise in degrees from magnetic north (e.g. 270) | 0–360 |
| Regional ecosystem | Regional ecosystem (RE) code for the spring wetland. For a full description of regional ecosystem codes refer to www.derm.qld.gov.au/wildlife-ecosystems/biodiversity/regional_ecosystems/index.php> | single RE code |
| Latitude (decimal degrees) | Latitude at point photo taken, recorded as decimal degrees to 4 decimal places. Negative values indicate location in southern hemisphere | –29.5 to –9.5 |
| Longitude (decimal degrees) | Longitude at point photo taken, recorded as decimal degrees to 4 decimal places. Positive values indicate location in eastern hemisphere | 137.9 to 154.0 |
| Photographer | Name of person(s)/institution who took the photo (e.g. JS Smith) | text, blank |
| Bioregion | 3 letter code indicating bioregion as per Sattler and Williams (1999) | BBN, BBS, CYP, CHC, CQC, DEU, EIU, GUP,MGD, NWH, MUL, NET, SEQ, WET, blank |
| Supergroup | Name of spring supergroup to which spring wetland belongs. A supergroup represents a major regional cluster of spring complexes with some consistent hydrogeological characteristics as defined by Habermehl (1982), Ponder (1986), GABCC (1998) and Fensham et al. (2003) | text, blank |

Table A3 Flora records

| Field name | Description | Allowed values |
| --- | --- | --- |
| Record number | Unique record number for all flora records | autonumber (mandatory) |
| Site number | Unique identifier for spring wetland | >0 (mandatory) |
| Preliminary site number | Preliminary unique identifier given to spring wetlands during field survey | >0 |
| Taxon number | Unique identifier for floristic taxa—equivalent to BC_NR in Queensland Herbarium HERBRECS database | >0 (mandatory) |
| Taxon name | Accepted taxon name, without author details | text |
| Date | Date taxa recorded at spring wetland (i.e. date of floristic survey) | text (mandatory) |
| Supergroup | Name of spring supergroup to which spring wetland belongs. A supergroup represents a major regional cluster of spring-complexes with some consistent hydrogeological characteristics as defined by Habermehl (1982), Ponder (1986), GABCC (1998) and Fensham et al. (2003) | text, blank |

Table A4 Flora taxa

| Field name | Description | Allowed values |
| --- | --- | --- |
| Taxon ID | Unique identifier for floristic taxa equivalent to BC_NR in Queensland Herbarium HERBRECS database | >0 (mandatory) |
| Taxon name including author | Accepted full taxon name including author(s) | text (mandatory) |
| Taxon name | Accepted full taxon name excluding author(s) | text (mandatory) |
| Taxa with isolated population | Taxon occurs as isolated populations in spring wetlands (i.e. not known from habitat other than spring wetlands within a radius of 250 km, though may be common elsewhere), Based on confirmed species records held in the Queensland Herbarium HERBRECS database | true/false |
| Spring endemic | Taxon endemic to GAB springs | true/false |
| NCA | Taxon listed under Nature Conservation Act 1992 (Qld) | true/false |
| EPBC | Taxon listed as threatened under the Environment Protection and Biodiversity Conservation Act 1999 (Cwlth) | true/false |
| Naturalised | Taxon recognised by Queensland Herbarium as naturalised | true/false |
| Widespread colonisers | Taxon recognised as common and widespread in a broad range of wetland habitats (cosmopolitan wetland taxa) |  |
| Non-wetland incidental taxa | Taxon not normally associated with wetland habitats | true/false |
| Invasive taxa | Naturalised taxon recognised as potentially invasive in wetland habitats (e.g. ‘ponded pasture’ taxa) | true/false |

Table A5 Water chemistry

| Field name | Description | Allowed values |
| --- | --- | --- |
| Record number | Unique record number for all water chemistry records | autonumber (mandatory) |
| Site number | Unique identifier for spring wetland | >0 |
| Date | Date spring wetland water data collected | Any date, blank |
| Distance from vent (m) | The distance (metres) from main spring vent to the point at which water chemistry data was collected | >0 |
| pH | Hydrogen ion potential of field water sample | 0–14, blank |
| Temperature | Water temperature in degrees Celsius (recorded to 1 decimal place) | >0, blank |
| Conductivity | Measure of electrical conductivity. Measured in microsiemens per centimetre. | >0, blank |
| Chemistry data source | 1. Data collected by Herbarium staff; 2. Data collected by others | 1, 2, blank |
| Hydrogeological report exists | A hydrogeological report exists for the spring or an associated bore | true/false |
| Notes | Additional information | text, blank |

Contact information CONTACT ORGANISATION

Queensland Herbarium, Queensland Department of Science, Information Technology, Innovation and the Arts

Contact information CONTACT POSITION

Director, Botanical Services

Contact information MAIL ADDRESS

Brisbane Botanic Gardens, Mt Coot-tha, Mt Coot-tha Road

Contact information SUBURB/PLACE/LOCALITY

Toowong

Contact information STATE/LOCALITY 2

Qld

Contact information COUNTRY

Australia

Contact information POSTCODE

4066

Contact information TELEPHONE

07 3896 9326

Contact information FACSIMILE

07 3896 9624

Contact information ELECTRONIC MAIL ADDRESS

Queensland.Herbarium@derm.qld.gov.au

Metadata Date METADATA DATE

06 March 2013

References

Cox R & Barron A (eds) 1998, Great Artesian Basin resource study, Great Artesian Basin Consultative Council, Canberra.

CSIRO 2009, Australian soil and land survey field handbook, 3rd edn, CSIRO Publishing, Collingwood.

Fatchen T 2001, ‘Vegetated wetland area as an index of mound spring flows’, in Proceedings of the 4th Mound Spring Researchers Forum,National Parks and Wildlife, Adelaide, South Australia, 23 Feb. 2001, pp. 5–8,

Fensham RJ & Fairfax RJ 2003, ‘Spring wetlands of the Great Artesian Basin, Queensland, Australia’, Wetland Ecology and Management, vol. 11, pp. 343–62.

Fensham RJ, Pennay C & DrimerJ 2011, Ecological and botanical survey of springs in the Surat cumulative management area, Queensland Herbarium, Queensland Department of Science, Information Technology, Innovation and the Arts, Brisbane.

Habermehl MA 1982, Springs in the Great Artesian Basin, Australia: their origin and nature, Bureau of Mineral Resources, Geology and Geophysics, Canberra.

Ponder WF 1986, ‘Mound springs of the Great Artesian Basin’, in P De Deckker & WD Williams (eds), Limnology in Australia, CSIRO, Australia, pp. 403–20.

Queensland Herbarium 2011, Regional Ecosystem Description Database (REDD), version 6.0b, January 2011, Department of Environment and Resource Management, Brisbane, accessed 28 September 2011,

Sattler PS & Williams RD (eds) 1999, The conservation status of Queensland’s bioregional ecosystems, Environmental Protection Agency, Brisbane.

# Appendix B:	Example spring report, Wiggera Spring, Eulo supergroup

WIGGERA SPRING

Status: Inactive

Location: –28.26836°S, 144.56864°E (Wiggera), –28.26625°S, 144.56600°E (Wiggera Carpet Springs)

Property: Granite Springs

Preliminary site #: 381B, 381Dnv

Site #: 905 (Wiggera), 905.1 (Wiggera carpet springs)

Field survey date: 18 October 2012

Informant: Graeme Pfitzner, Ian Pike, Gordon Warner

Inspected plans: Perambulator Survey of Runs on Bundilla, Acklands, Bingara and Dewalla Creeks, Warrego District, 21 February (1876); survey plan W.51.40 (c.1870); 4 mile series 1 sheet 3C (1931); 4 mile series 2 sheet 7 (1959)

Marked on survey plan W51.40 (1870) as unnamed spring, Perambulator Survey of Runs on Bundilla, Acklands, Bingara and Dewalla creeks, Warrego District (1876); and four-mile maps. The Adelaide Chronicle reported on extensive mud springs between Eulo and Thargomindah and that ‘a hot lake is found near the southern boundary of the (Dynevor) run’ (24 May 1919, p. 5). This probably relates to the area formerly flooded by Wiggera Spring.

Survey plan W.51.40 (1870), with Wiggera Spring marked in bottom centre of image.

Perambulator Survey of Runs on Bundilla, Acklands, Bingara and Dewalla creeks, Warrego District, 21 February (1876), showing Wiggera Spring.

Four-mile series 1, sheet 3C (1931) showing Wiggera Spring.

Bore inspectors Ogilvie and Edwards visited Wiggera Spring. Ogilvie wrote:

‘Situated at the head of Wiggera Crek in the south-east corner of Dynevor Downs. The surrounding country on all sides consists of very steep desert sandstone ridges. The water issues from several holes in a basin about 10 foot wide and 2 or 3 feet deep and runs the flat bed of the watercourse for about ½ mile. A few reeds flourish on the edge of the pool, and good feed is growing in the watercourse, which, however, is rather boggy. The temperature is 95°F (also 96°F) and the water of good quality. 42,100 gpd. measured.

A few small carpet springs occur at the main spring and 4 or 5 about 15 chains west, but these are barely noticeable and do not overflow.’

© Copyright, Ogilvie (1912)

Edwards (13/12/1911) described Wiggera as:

‘Situated in the centre of a basin about ½ mile in width and almost surrounded by desert sandstone ranges. Wiggera Spring is one of the largest and most valuable in the district and was at one time the subject of litigation between the owners of Dynevor Downs and Boorara runs as their division boundary passed through the spring. I made a rough estimate of the flow which is about U40,000 g.p.d.U In winter this runs Wiggera Creek for 3 miles. The bottom of the spring is of loose free sand and has numerous ‘pipes’ or fissures up which water and gas rise. Reaching as far as possible down these ‘pipes’ I secured some water-worn stones which belong to the white sandstone and quartzites of the desert sandstone series.

Improvements in Boorara- 200 foot of 2 inch piping leading to about 75 foot of troughing for watering of stock. In Dynevor a short length of drain has been made to lead water into Wiggera Creek. A strong flow of gas has been given off from the spring.

Quality: in taste similar to Wooregym bore water the HR2RS being noticeable; other gases are probably present but impart no taste to the water. Temperature about 96°F. Wiggera Spring water is hotter than both Tunga and Wooregym bores and almost as hot as Bingara. This evidence together with the great similarity in taste and presence of HR2RS gas seems to indicate an artesian source of supply for the spring.’

© Copyright, Edwards (1912)

[Vide note WIGGERA SPRINGS, Rpt No. 13 p.15] in Edwards 18/2/12:

‘The supply is now very small but a boundary rider who was on the run before the adjacent bore was sunk states that the spring gave out when the bore first struck water. Originally the spring maintained the same length of drain as the bore now does. Quality: excellent.’

© Copyright, Edwards (1912)

The site of Wiggera Spring is a dry, stony clay depression at the southern end of a vast barren plain between low hills. It sits in a shallow dry creekline vegetated with Geijera parviflora, Acacia tetragonophylla and Atalaya hemiglauca. The only sign of its former considerable outflow is a rusty pipe emanating from this depression. Sections of the pipe are scattered across the adjacent stony flat. Just to the north is a depression that appears to have been scraped out with a drain leading to the north-west. Numerous old fencelines converge at the site. Long-term residents of the area remember Wiggera as an active spring until about the 1940s, when it was no longer regularly maintained by scooping out and it went dry.

The Carpet Springs described by Ogilvie are evident as dry, shallow depressions fringed by sedges near the main spring at –28.26835, 144.56828 and 300 m to the west-north-west on the edge of the vast pebbly plain, where there are about five such depressions across a small area. Ian Pike remembers Eriocaulon carsonii growing in the Wiggera Spring wetland in the 1940s, while the groundwater indicator species Sporobolus partimpatens, Calocephalus sp. (Eulo) and Trianthema sp. (Coorabulka RW Purdie 1404) are still common at the site. Aboriginal stone flakes are scattered across the area.

B

A

D

C

(A) Site of Wiggera spring, now a dry, shallow depression in the bed of Wiggera Creek (Photo 109), (B) rusty, old pipe emanating from the spring depression (Photo 110) and lying on the adjacent pebbly flat (Photo 113), (C) the only sign of the former 40 000 gpd flow that emanated from the spring and flowed Wiggera Creek for five kilometres, and (D) shallow depressions on stony, barren flat between hills, filled with sedges and Sporobolus partimpatens; the Carpet Springs west of the main Wiggera Spring, as described by Ogilvie in 1912 (Photo 116).

# Appendix C:	Survey status; Springsure, Eulo and Bourke supergroups

Table A6 Survey status for Springsure, Eulo and Bourke supergroups, March 2013, with notes on distance to tenure (petroleum production leases), elevation and access

| Site | Supergroup | Latitude | Longitude | Elevation (m) | EPBC community | EPBC-listed species | EPBC species | NCA- listed species | NCA species | Distance to tenure (km) | Access |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 1 | Springsure | –25.4341 | 150.0253 | 186.36 | Yes | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 44.58 | Surveyed |
| 2 | Springsure | –25.4576 | 150.0215 | 175.5 | Yes | No | n/a | No | n/a | 41.95 | Surveyed |
| 3 | Springsure | –25.4525 | 150.0292 | 177.88 | Yes | No | n/a | No | n/a | 42.51 | Surveyed |
| 4 | Springsure | –25.4458 | 150.0237 | 179.42 | Yes | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 43.27 | Surveyed |
| 5 | Springsure | –25.4455 | 150.0260 | 187.85 | Yes | Yes | Arthraxon hispidus | Yes | Thelypteris confluens, Arthraxon hispidus | 43.30 | Surveyed |
| 6 | Springsure | –25.4275 | 150.0227 | 197.56 | Yes | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 45.32 | Surveyed |
| 7 | Springsure | –25.4268 | 150.0282 | 203.03 | Yes | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 45.40 | Surveyed |
| 8 | Springsure | –25.4154 | 150.0237 | 208.68 | Yes | No | n/a | No | n/a | 46.68 | Surveyed |
| 9 | Springsure | –25.4148 | 150.0236 | 210.31 | Yes | No | n/a | No | n/a | 46.74 | Surveyed |
| 10 | Springsure | –25.4125 | 150.0209 | 212.54 | Yes | No | n/a | No | n/a | 47.01 | Surveyed |
| 11 | Springsure | –25.4512 | 150.0436 | 173.37 | Yes | No | n/a | No | n/a | 42.67 | Surveyed |
| 12 | Springsure | –25.4414 | 150.0563 | 173.38 | Yes | No | n/a | No | n/a | 43.77 | Surveyed |
| 13 | Springsure | –25.4409 | 150.0563 | 174.6 | Yes | No | n/a | No | n/a | 43.82 | Surveyed |
| 14 | Springsure | –25.4476 | 150.0427 | 170.98 | Yes | No | n/a | No | n/a | 43.06 | Surveyed |
| 15 | Springsure | –25.4435 | 150.0337 | 188.23 | Yes | No | n/a | No | n/a | 43.53 | Surveyed |
| 16 | Springsure | –25.4484 | 150.1541 | 158.929 | No | No | n/a | No | n/a | 43.30 | Surveyed |
| 17 | Springsure | –25.4604 | 150.1331 | 171.835 | No | No | n/a | No | n/a | 41.75 | Surveyed |
| 18 | Springsure | –25.4603 | 150.1301 | 163.134 | No | No | n/a | No | n/a | 41.74 | Surveyed |
| 19 | Springsure | –25.4564 | 150.1341 | 161.424 | No | No | n/a | No | n/a | 42.21 | Surveyed |
| 20 | Springsure | –25.4614 | 150.1321 | 172.229 | No | No | n/a | No | n/a | 41.63 | Surveyed |
| 21 | Springsure | –25.4554 | 150.1351 | 160.631 | No | No | n/a | No | n/a | 42.33 | Surveyed |
| 22 | Springsure | –25.4280 | 150.0275 | 199.67 | Yes | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 45.26 | Surveyed |
| 23 | Springsure | –25.4359 | 150.0256 | 188.14 | Yes | Yes | Arthraxon hispidus | Yes | Thelypteris confluens, Arthraxon hispidus | 44.38 | Surveyed |
| 24 | Springsure | –25.4384 | 150.0256 | 185.01 | Yes | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 44.10 | Surveyed |
| 25 | Springsure | –25.4368 | 150.0233 | 184.86 | Yes | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 44.27 | Surveyed |
| 26 | Springsure | –25.5524 | 149.8071 | 190 | Yes | No | n/a | No | n/a | 31.32 | Surveyed |
| 27 | Springsure | –25.4323 | 150.0254 | 191.59 | Yes | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 44.79 | Surveyed |
| 28 | Springsure | –25.5524 | 149.8071 | 190 | Yes | No | n/a | No | n/a | 31.32 | Surveyed |
| 29 | Springsure | –25.4516 | 150.0205 | 178.17 | Yes | No | n/a | No | n/a | 42.62 | Surveyed |
| 30 | Springsure | –25.4423 | 150.0055 | 181.33 | Yes | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 43.66 | Surveyed |
| 31 | Springsure | –25.4409 | 150.0049 | 183.16 | Yes | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 43.82 | Surveyed |
| 32 | Springsure | –25.4412 | 150.0260 | 182.94 | Yes | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 43.79 | Surveyed |
| 33 | Springsure | –25.4591 | 150.0286 | 177.98 | Yes | No | n/a | No | n/a | 41.77 | Surveyed |
| 34 | Springsure | –25.4584 | 150.1231 | 163.104 | No | No | n/a | No | n/a | 41.91 | Surveyed |
| 35 | Springsure | –25.4594 | 150.1251 | 162.707 | No | No | n/a | No | n/a | 41.81 | Surveyed |
| 36 | Springsure | –25.4564 | 150.1261 | 164.178 | No | No | n/a | No | n/a | 42.15 | Surveyed |
| 37 | Springsure | –25.4608 | 150.0797 | 174.76 | Yes | No | n/a | No | n/a | 41.59 | Surveyed |
| 37.1 | Springsure | –25.4608 | 150.0798 | 174.76 | Yes | No | n/a | No | n/a | 41.58 | Surveyed |
| 38 | Springsure | –25.5685 | 149.8021 | 182.67 | Yes | No | n/a | No | n/a | 29.51 | Surveyed |
| 39 | Springsure | –25.4764 | 150.1271 | 177.162 | No | No | n/a | No | n/a | 39.92 | Surveyed |
| 40 | Springsure | –25.4753 | 150.1287 | 179.88 | Yes | Yes | Eriocaulon carsonii | Yes | Eriocaulon carsonii, Livistona nitida | 40.06 | Surveyed |
| 41 | Springsure | –25.4767 | 150.1309 | 180.43 | Yes | Yes | Eriocaulon carsonii | Yes | Eriocaulon carsonii, Livistona nitida | 39.91 | Surveyed |
| 42 | Springsure | –25.5150 | 150.0586 | 181.47 | Yes | No | n/a | No | n/a | 35.51 | Surveyed |
| 43 | Springsure | –25.4633 | 150.0567 | 172.83 | Yes | No | n/a | No | n/a | 41.31 | Surveyed |
| 44 | Springsure | –25.4586 | 150.0476 | 221 | Yes | No | n/a | No | n/a | 41.83 | Surveyed |
| 45 | Springsure | –25.4714 | 150.1221 | 172.22 | No | No | n/a | No | n/a | 40.45 | Surveyed |
| 46 | Springsure | –25.4713 | 150.1201 | 173.037 | No | No | n/a | No | n/a | 40.45 | Surveyed |
| 47 | Springsure | –25.4709 | 150.1175 | 169.86 | No | No | n/a | No | n/a | 40.49 | Surveyed |
| 47.1 | Springsure | –25.4710 | 150.1181 | 168.23 | No | No | n/a | No | n/a | 40.47 | Surveyed |
| 47.2 | Springsure | –25.4711 | 150.1181 | 178 | No | No | n/a | No | n/a | 40.46 | Surveyed |
| 47.3 | Springsure | –25.4712 | 150.1182 | 177 | No | No | n/a | No | n/a | 40.45 | Surveyed |
| 47.4 | Springsure | –25.4714 | 150.1183 | 177 | No | No | n/a | No | n/a | 40.44 | Surveyed |
| 47.5 | Springsure | –25.4710 | 150.1184 | 167.32 | No | No | n/a | No | n/a | 40.48 | Surveyed |
| 47.6 | Springsure | –25.4705 | 150.1183 | 166.99 | No | No | n/a | No | n/a | 40.54 | Surveyed |
| 47.7 | Springsure | –25.4705 | 150.1177 | 167.44 | No | No | n/a | No | n/a | 40.54 | Surveyed |
| 47.8 | Springsure | –25.4699 | 150.1178 | 167.13 | No | No | n/a | No | n/a | 40.60 | Surveyed |
| 47.9 | Springsure | –25.4696 | 150.1178 | 166.95 | No | No | n/a | No | n/a | 40.63 | Surveyed |
| 48 | Springsure | –25.4721 | 150.1187 | 174.74 | No | No | n/a | Yes | Livistona nitida | 40.36 | Surveyed |
| 49 | Springsure | –25.4714 | 150.1191 | 173.891 | No | No | n/a | No | n/a | 40.44 | Surveyed |
| 50 | Springsure | –25.4696 | 150.1186 | 166.18 | No | No | n/a | No | n/a | 40.63 | Surveyed |
| 51 | Springsure | –25.4684 | 150.1201 | 169.779 | No | No | n/a | No | n/a | 40.78 | Surveyed |
| 52 | Springsure | –25.4773 | 150.1257 | 181.42 | Yes | Yes | Eriocaulon carsonii | Yes | Eriocaulon carsonii | 39.81 | Surveyed |
| 53 | Springsure | –25.4549 | 150.0210 | 174.82 | Yes | No | n/a | No | n/a | 42.25 | Surveyed |
| 54 | Springsure | –25.4454 | 150.0591 | 170.34 | Yes | No | n/a | No | n/a | 43.32 | Surveyed |
| 55 | Springsure | –25.4219 | 150.0227 | 202.18 | Yes | No | n/a | No | n/a | 45.95 | Surveyed |
| 56 | Springsure | –25.4247 | 150.0217 | 197.05 | Yes | No | n/a | No | n/a | 45.63 | Surveyed |
| 56.1 | Springsure | –25.4246 | 150.0210 | 196.57 | Yes | No | n/a | No | n/a | 45.65 | Surveyed |
| 57 | Springsure | –25.4202 | 150.0265 | 201.4 | Yes | No | n/a | No | n/a | 46.14 | Surveyed |
| 58 | Springsure | –25.4225 | 150.0271 | 201.61 | Yes | No | n/a | No | n/a | 45.88 | Surveyed |
| 59 | Springsure | –25.4421 | 150.0235 | 182.32 | Yes | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 43.69 | Surveyed |
| 60 | Springsure | –25.4302 | 150.0216 | 190.99 | Yes | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 45.02 | Surveyed |
| 61 | Springsure | –25.4544 | 150.0274 | 172.43 | Yes | No | n/a | No | n/a | 42.31 | Surveyed |
| 62 | Springsure | –25.4311 | 150.0252 | 193.45 | Yes | No | n/a | No | n/a | 44.91 | Surveyed |
| 63 | Springsure | –25.4502 | 150.0464 | 171.64 | Yes | No | n/a | No | n/a | 42.78 | Surveyed |
| 64 | Springsure | –25.7177 | 150.2409 | 223.08 | No | Yes | Eriocaulon carsonii | Yes | Myriophyllum artesium, Eriocaulon carsonii | 18.86 | Surveyed |
| 64.1 | Springsure | –25.7178 | 150.2407 | 217.29 | Yes | No | n/a | No | n/a | 18.83 | Surveyed |
| 65 | Springsure | –25.7165 | 150.2391 | 219.2 | Yes | Yes | Eriocaulon carsonii | Yes | Myriophyllum artesium, Eriocaulon carsonii | 18.82 | Surveyed |
| 65.1 | Springsure | –25.7163 | 150.2390 | 218.29 | Yes | No | n/a | No | n/a | 18.83 | Surveyed |
| 65.2 | Springsure | –25.7162 | 150.2391 | 222.13 | Yes | No | n/a | No | n/a | 18.84 | Surveyed |
| 66 | Springsure | –25.7194 | 150.2381 | 217.928 | Yes | No | n/a | No | n/a | 18.52 | Surveyed |
| 67 | Springsure | –25.4755 | 150.1272 | 176.92 | Yes | No | n/a | No | n/a | 40.02 | Surveyed |
| 68 | Springsure | –25.4312 | 150.0270 | 196.09 | Yes | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 44.91 | Surveyed |
| 68.1 | Springsure | –25.4312 | 150.0267 | 197.07 | Yes | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 44.90 | Surveyed |
| 69 | Springsure | –25.4644 | 150.1221 | 184.274 | No | No | n/a | No | n/a | 41.23 | Surveyed |
| 157 | Springsure | –25.2797 | 151.1614 | 175.16696 | No | No | n/a | No | n/a | 121.67 | Surveyed |
| 158 | Springsure | –25.1197 | 151.1358 | 197.85609 | No | No | n/a | No | n/a | 130.03 | Surveyed |
| 159 | Springsure | –25.1156 | 151.1347 | 199.84569 | No | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 130.24 | Surveyed |
| 160 | Springsure | –25.1208 | 151.1372 | 201.69513 | No | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 130.06 | Surveyed |
| 161 | Springsure | –24.8675 | 147.4419 | 592.86096 | No | No | n/a | No | n/a | 82.55 | Surveyed |
| 162 | Springsure | –24.8242 | 147.4897 | 586.63873 | No | No | n/a | No | n/a | 77.77 | Surveyed |
| 163 | Springsure | –24.8414 | 147.3467 | 593.59277 | No | No | n/a | No | n/a | 92.05 | Surveyed |
| 164 | Springsure | –24.8597 | 147.1892 | 456.66693 | No | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 107.79 | Surveyed |
| 165 | Springsure | –24.8458 | 147.1947 | 443.65741 | No | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 107.23 | Surveyed |
| 166 | Springsure | –24.8267 | 147.1942 | 436.55573 | No | No | n/a | No | n/a | 107.30 | Surveyed |
| 167 | Springsure | –24.8339 | 147.1989 | 438.87662 | No | No | n/a | No | n/a | 106.83 | Surveyed |
| 168 | Springsure | –24.8106 | 147.2022 | 435.22226 | No | No | n/a | No | n/a | 106.51 | Surveyed |
| 169 | Springsure | –24.8247 | 147.4900 | 588.13892 | No | No | n/a | No | n/a | 77.74 | Surveyed |
| 170 | Springsure | –24.7975 | 147.6911 | 831.58331 | No | No | n/a | No | n/a | 57.65 | Surveyed |
| 171 | Springsure | –24.7886 | 147.6772 | 764.51538 | No | No | n/a | No | n/a | 59.04 | Surveyed |
| 172 | Springsure | –24.7886 | 147.6711 | 761.77777 | No | No | n/a | No | n/a | 59.66 | Surveyed |
| 173 | Springsure | –24.7808 | 147.6583 | 766.88861 | No | No | n/a | No | n/a | 60.94 | Surveyed |
| 174 | Non–GAB | –24.8034 | 147.8591 | 942 | No | No | n/a | No | n/a | 40.85 | Surveyed |
| 175 | Non–GAB | –24.7894 | 147.8461 | 883 | No | No | n/a | No | n/a | 42.16 | Surveyed |
| 176 | Springsure | –24.8006 | 147.8236 | 834.99377 | No | No | n/a | No | n/a | 44.40 | Surveyed |
| 177 | Non–GAB | –24.7439 | 147.7473 | 838 | No | No | n/a | No | n/a | 52.06 | Surveyed |
| 178 | Springsure | –25.3625 | 147.1003 | 475.22223 | No | No | n/a | No | n/a | 123.91 | Surveyed |
| 179 | Springsure | –25.3667 | 147.1036 | 476.04626 | No | No | n/a | No | n/a | 123.76 | Surveyed |
| 180 | Springsure | –25.1172 | 147.4231 | 725.67926 | No | No | n/a | No | n/a | 85.62 | Surveyed |
| 181 | Springsure | –25.0942 | 147.4411 | 711.4538 | No | No | n/a | No | n/a | 83.43 | Surveyed |
| 182 | Springsure | –24.8797 | 147.7689 | 802.58954 | No | No | n/a | No | n/a | 49.88 | Surveyed |
| 183 | Springsure | –24.8731 | 147.7961 | 823.37347 | No | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 47.20 | Surveyed |
| 184 | Springsure | –26.2334 | 148.8686 | 406.38 | No | No | n/a | No | n/a | 11.11 | Surveyed |
| 185 | Springsure | –26.2321 | 148.8694 | 404.16 | No | No | n/a | No | n/a | 11.25 | Surveyed |
| 186 | Springsure | –26.2367 | 148.8690 | 398.72 | No | No | n/a | No | n/a | 10.73 | Surveyed |
| 187 | Springsure | –26.2190 | 148.6703 | 382.7 | No | No | n/a | No | n/a | 20.61 | Surveyed |
| 188 | Springsure | –26.2695 | 148.7054 | 368 | No | No | n/a | No | n/a | 14.56 | Surveyed |
| 189 | Springsure | –25.8915 | 149.2860 | 245.61 | Yes | Yes | Eriocaulon carsonii | Yes | Eriocaulon carsonii | 6.75 | Surveyed |
| 190 | Springsure | –25.8884 | 149.2874 | 244.35 | Yes | Yes | Eriocaulon carsonii | Yes | Eriocaulon carsonii | 6.90 | Surveyed |
| 191 | Springsure | –25.8918 | 149.2875 | 244.45 | Yes | Yes | Eriocaulon carsonii | Yes | Eriocaulon carsonii | 6.90 | Surveyed |
| 192 | Springsure | –25.8890 | 149.2790 | 244.85 | Yes | Yes | Eriocaulon carsonii | Yes | Eriocaulon carsonii | 6.07 | Surveyed |
| 192.1 | Springsure | –25.8881 | 149.2792 | 247 | Yes | No | n/a | No | n/a | 6.08 | Surveyed |
| 193 | Springsure | –26.2244 | 147.7164 | 453.4198 | No | No | n/a | No | n/a | 110.58 | Surveyed |
| 194 | Springsure | –26.2264 | 147.7119 | 444.99689 | No | No | n/a | No | n/a | 110.79 | Surveyed |
| 196 | Bourke | –28.8502 | 146.7913 | 140 | Yes | No | n/a | No | n/a | 238.30 | Surveyed |
| 196.1 | Bourke | –28.8501 | 146.7914 | 135.093 | Yes | No | n/a | No | n/a | 238.28 | Surveyed |
| 196.2 | Bourke | –28.8500 | 146.7917 | 140 | Yes | No | n/a | No | n/a | 238.25 | Surveyed |
| 196.3 | Bourke | –28.8496 | 146.7919 | 135.681 | Yes | No | n/a | No | n/a | 238.22 | Surveyed |
| 196.4 | Bourke | –28.8488 | 146.7921 | 140 | Yes | No | n/a | No | n/a | 238.16 | Surveyed |
| 196.5 | Bourke | –28.8493 | 146.7909 | 135.075 | Yes | No | n/a | No | n/a | 238.29 | Surveyed |
| 197 | Bourke | –28.9708 | 146.9237 | 134.606 | Yes | No | n/a | No | n/a | 233.33 | Surveyed |
| 197.1 | Bourke | –28.9716 | 146.9255 | 134.245 | Yes | No | n/a | No | n/a | 233.22 | Surveyed |
| 197.2 | Bourke | –28.9707 | 146.9251 | 134.154 | Yes | No | n/a | No | n/a | 233.21 | Surveyed |
| 197.3 | Bourke | –28.9732 | 146.9262 | 135.108 | Yes | No | n/a | No | n/a | 233.25 | Surveyed |
| 197.4 | Bourke | –28.9743 | 146.9268 | 136.019 | Yes | No | n/a | No | n/a | 233.26 | Surveyed |
| 197.5 | Bourke | –28.9746 | 146.9263 | 136.131 | Yes | No | n/a | No | n/a | 233.33 | Surveyed |
| 197.6 | Bourke | –28.9728 | 146.9292 | 136.163 | Yes | No | n/a | No | n/a | 232.98 | Surveyed |
| 198 | Bourke | –28.9491 | 146.8874 | 133.803 | Yes | No | n/a | No | n/a | 235.16 | Surveyed |
| 199 | Bourke | –28.9781 | 146.8917 | 138.076 | No | No | n/a | No | n/a | 236.40 | Surveyed |
| 200 | Bourke | –28.9690 | 146.9088 | 144.764 | No | No | n/a | No | n/a | 234.47 | Surveyed |
| 201 | Eulo | –28.3333 | 144.5354 | 185 | Yes | No | n/a | No | n/a | 412.51 | Surveyed |
| 202 | Eulo | –28.3275 | 144.5329 | 185 | Yes | Yes | Hydrocotyle dipleura | Yes | Hydrocotyle dipleura, Myriophyllum arteisum | 412.56 | Surveyed |
| 203 | Eulo | –28.3108 | 144.5501 | 194 | Yes | Yes | Eragrostis fenshami | Yes | Myriophyllum artesium, Eragrostis fenshami | 410.43 | Surveyed |
| 203.1 | Eulo | –28.3095 | 144.5504 | 196 | Yes | No | n/a | No | n/a | 410.37 | Surveyed |
| 204 | Eulo | –28.4682 | 144.5794 | 172 | Yes | No | n/a | No | n/a | 412.95 | Surveyed |
| 204.1 | Eulo | –28.4737 | 144.5878 | 167 | Yes | No | n/a | No | n/a | 412.38 | Surveyed |
| 204.2 | Eulo | –28.4790 | 144.5900 | 164 | Yes | No | n/a | No | n/a | 412.36 | Surveyed |
| 204.3 | Eulo | –28.4851 | 144.5891 | 163 | Yes | No | n/a | No | n/a | 412.67 | Surveyed |
| 205.1 | Eulo | –28.4026 | 144.5177 | 174 | Yes | No | n/a | No | n/a | 416.39 | Surveyed |
| 205.2 | Eulo | –28.4023 | 144.5176 | 174 | Yes | No | n/a | No | n/a | 416.38 | Surveyed |
| 205.3 | Eulo | –28.4022 | 144.5174 | 174 | Yes | No | n/a | No | n/a | 416.40 | Surveyed |
| 205.4 | Eulo | –28.4032 | 144.5182 | 174 | Yes | No | n/a | Yes | Myriophyllum artesium | 416.36 | Surveyed |
| 205.5 | Eulo | –28.4031 | 144.5179 | 174 | Yes | No | n/a | No | n/a | 416.38 | Surveyed |
| 205.6 | Eulo | –28.4039 | 144.5198 | 176 | Yes | No | n/a | No | n/a | 416.24 | Surveyed |
| 205.7 | Eulo | –28.4012 | 144.5198 | 174 | Yes | No | n/a | No | n/a | 416.14 | Surveyed |
| 205.8 | Eulo | –28.3974 | 144.5253 | 177 | Yes | No | n/a | No | n/a | 415.51 | Surveyed |
| 205.9 | Eulo | –28.3861 | 144.5338 | 178 | Yes | No | n/a | No | n/a | 414.35 | Surveyed |
| 206.1 | Eulo | –28.4378 | 144.5076 | 170 | Yes | No | n/a | No | n/a | 418.50 | Surveyed |
| 206.2 | Eulo | –28.4379 | 144.5063 | 170 | Yes | No | n/a | No | n/a | 418.62 | Surveyed |
| 206.3 | Eulo | –28.4352 | 144.5054 | 170 | Yes | No | n/a | No | n/a | 418.62 | Surveyed |
| 206.4 | Eulo | –28.4254 | 144.5065 | 171 | Yes | No | n/a | No | n/a | 418.18 | Surveyed |
| 206.5 | Eulo | –28.4215 | 144.5083 | 172 | Yes | No | n/a | No | n/a | 417.89 | Surveyed |
| 207 | Eulo | –28.3905 | 144.3054 | 135.778 | Yes | No | n/a | No | n/a | 435.69 | Surveyed |
| 207.1 | Eulo | –28.3933 | 144.2979 | 134.321 | Yes | No | n/a | No | n/a | 436.47 | Surveyed |
| 207.2 | Eulo | –28.3950 | 144.2960 | 141 | Yes | No | n/a | No | n/a | 436.70 | Surveyed |
| 208 | Eulo | –28.2897 | 144.3166 | 143.239 | Yes | No | n/a | No | n/a | 431.62 | Surveyed |
| 208.1 | Eulo | –28.2880 | 144.3179 | 149 | Yes | No | n/a | No | n/a | 431.46 | Surveyed |
| 208.2 | Eulo | –28.2932 | 144.3133 | 149 | Yes | No | n/a | No | n/a | 432.03 | Surveyed |
| 208.3 | Eulo | –28.2941 | 144.3129 | 149 | Yes | No | n/a | No | n/a | 432.10 | Surveyed |
| 209 | Eulo | –28.1767 | 145.0221 | 155.897 | Yes | No | n/a | No | n/a | 362.31 | Surveyed |
| 210 | Eulo | –28.7185 | 144.4899 | 136.475 | Yes | No | n/a | No | n/a | 430.70 | Surveyed |
| 211.1 | Eulo | –28.6729 | 144.3962 | 129 | Yes | No | n/a | No | n/a | 437.31 | Surveyed |
| 211.1 | Eulo | –28.6720 | 144.3975 | 129 | Yes | No | n/a | No | n/a | 437.16 | Surveyed |
| 211.1 | Eulo | –28.6721 | 144.3970 | 129 | Yes | No | n/a | No | n/a | 437.20 | Surveyed |
| 211.1 | Eulo | –28.6718 | 144.3969 | 130 | Yes | No | n/a | No | n/a | 437.20 | Surveyed |
| 211.1 | Eulo | –28.6719 | 144.3963 | 130 | Yes | No | n/a | No | n/a | 437.26 | Surveyed |
| 211.1 | Eulo | –28.6716 | 144.3960 | 130 | Yes | No | n/a | No | n/a | 437.28 | Surveyed |
| 211.2 | Eulo | –28.6721 | 144.3957 | 130 | Yes | No | n/a | No | n/a | 437.32 | Surveyed |
| 211.2 | Eulo | –28.6723 | 144.3960 | 130 | Yes | No | n/a | No | n/a | 437.30 | Surveyed |
| 211.2 | Eulo | –28.6725 | 144.3959 | 129 | Yes | No | n/a | No | n/a | 437.32 | Surveyed |
| 211.2 | Eulo | –28.6725 | 144.3954 | 130 | Yes | No | n/a | No | n/a | 437.36 | Surveyed |
| 211.2 | Eulo | –28.6724 | 144.3950 | 130 | Yes | No | n/a | No | n/a | 437.40 | Surveyed |
| 211.2 | Eulo | –28.6727 | 144.3963 | 129 | Yes | No | n/a | No | n/a | 437.29 | Surveyed |
| 211.2 | Eulo | –28.6727 | 144.3951 | 130 | Yes | No | n/a | No | n/a | 437.39 | Surveyed |
| 211.2 | Eulo | –28.6729 | 144.3954 | 129 | Yes | No | n/a | No | n/a | 437.38 | Surveyed |
| 211.2 | Eulo | –28.6732 | 144.3951 | 129 | Yes | No | n/a | No | n/a | 437.42 | Surveyed |
| 211.2 | Eulo | –28.6735 | 144.3951 | 129 | Yes | No | n/a | No | n/a | 437.43 | Surveyed |
| 211.2 | Eulo | –28.6737 | 144.3949 | 129 | Yes | No | n/a | No | n/a | 437.45 | Surveyed |
| 211.3 | Eulo | –28.6734 | 144.3945 | 129 | Yes | No | n/a | No | n/a | 437.48 | Surveyed |
| 211.3 | Eulo | –28.6739 | 144.3945 | 129 | Yes | No | n/a | No | n/a | 437.50 | Surveyed |
| 211.3 | Eulo | –28.6745 | 144.3940 | 125.696 | Yes | No | n/a | No | n/a | 437.57 | Surveyed |
| 211.3 | Eulo | –28.6748 | 144.3930 | 129 | Yes | No | n/a | No | n/a | 437.67 | Surveyed |
| 211.3 | Eulo | –28.6750 | 144.3931 | 129 | Yes | No | n/a | No | n/a | 437.67 | Surveyed |
| 211.3 | Eulo | –28.6727 | 144.3965 | 129 | Yes | No | n/a | No | n/a | 437.27 | Surveyed |
| 211.3 | Eulo | –28.6759 | 144.3918 | 129 | Yes | No | n/a | No | n/a | 437.82 | Surveyed |
| 211.3 | Eulo | –28.6759 | 144.3914 | 130 | Yes | No | n/a | No | n/a | 437.86 | Surveyed |
| 211.3 | Eulo | –28.6766 | 144.3909 | 129 | Yes | No | n/a | No | n/a | 437.93 | Surveyed |
| 211.3 | Eulo | –28.6768 | 144.3918 | 129 | Yes | No | n/a | No | n/a | 437.86 | Surveyed |
| 211.3 | Eulo | –28.6762 | 144.3930 | 129 | Yes | No | n/a | No | n/a | 437.73 | Surveyed |
| 211.4 | Eulo | –28.6759 | 144.3930 | 128 | Yes | No | n/a | No | n/a | 437.71 | Surveyed |
| 211.4 | Eulo | –28.6758 | 144.3937 | 129 | Yes | No | n/a | No | n/a | 437.64 | Surveyed |
| 211.4 | Eulo | –28.6757 | 144.3939 | 129 | Yes | No | n/a | No | n/a | 437.62 | Surveyed |
| 211.4 | Eulo | –28.6754 | 144.3942 | 129 | Yes | No | n/a | No | n/a | 437.59 | Surveyed |
| 211.4 | Eulo | –28.6752 | 144.3941 | 129 | Yes | No | n/a | No | n/a | 437.59 | Surveyed |
| 211.4 | Eulo | –28.6725 | 144.3987 | 129 | Yes | No | n/a | No | n/a | 437.07 | Surveyed |
| 211.4 | Eulo | –28.6747 | 144.3942 | 129 | Yes | No | n/a | No | n/a | 437.56 | Surveyed |
| 211.5 | Eulo | –28.6722 | 144.3991 | 129 | Yes | No | n/a | No | n/a | 437.01 | Surveyed |
| 211.6 | Eulo | –28.6716 | 144.3982 | 129 | Yes | No | n/a | No | n/a | 437.07 | Surveyed |
| 211.7 | Eulo | –28.6714 | 144.3981 | 130 | Yes | No | n/a | No | n/a | 437.08 | Surveyed |
| 211.8 | Eulo | –28.6720 | 144.3982 | 129 | Yes | No | n/a | No | n/a | 437.09 | Surveyed |
| 211.9 | Eulo | –28.6723 | 144.3979 | 129 | Yes | No | n/a | No | n/a | 437.12 | Surveyed |
| 212 | Eulo | –28.4769 | 144.4870 | 161 | Yes | No | n/a | No | n/a | 421.74 | Surveyed |
| 213 | Eulo | –28.4702 | 144.5003 | 166 | Yes | No | n/a | No | n/a | 420.29 | Surveyed |
| 214 | Eulo | –28.4725 | 144.5117 | 170 | Yes | No | n/a | No | n/a | 419.32 | Surveyed |
| 215 | Eulo | –28.4360 | 144.4051 | 147 | Yes | No | n/a | No | n/a | 427.91 | Surveyed |
| 216 | Eulo | –28.4617 | 144.3840 | 134.48 | Yes | No | n/a | No | n/a | 430.71 | Surveyed |
| 217 | Eulo | –28.7139 | 144.5063 | 134.085 | Yes | No | n/a | No | n/a | 429.04 | Surveyed |
| 218 | Eulo | –28.7114 | 144.5086 | 131.897 | Yes | No | n/a | No | n/a | 428.73 | Surveyed |
| 219 | Eulo | –28.7156 | 144.5186 | 133.491 | Yes | No | n/a | No | n/a | 428.00 | Surveyed |
| 220 | Eulo | –28.1563 | 144.7296 | 177.598 | Yes | No | n/a | No | n/a | 389.10 | Surveyed |
| 220.1 | Eulo | –28.1642 | 144.7234 | 183 | Yes | No | n/a | No | n/a | 389.90 | Surveyed |
| 220.2 | Eulo | –28.1639 | 144.7231 | 185 | Yes | No | n/a | No | n/a | 389.93 | Surveyed |
| 221.1 | Eulo | –28.1375 | 144.8635 | 160 | Yes | No | n/a | No | n/a | 376.00 | Surveyed |
| 221.2 | Eulo | –28.1381 | 144.8629 | 160 | Yes | No | n/a | No | n/a | 376.07 | Surveyed |
| 221.3 | Eulo | –28.1391 | 144.8624 | 160 | Yes | No | n/a | No | n/a | 376.14 | Surveyed |
| 221.4 | Eulo | –28.1387 | 144.8630 | 160 | Yes | No | n/a | No | n/a | 376.08 | Surveyed |
| 221.5 | Eulo | –28.1386 | 144.8637 | 159 | Yes | No | n/a | No | n/a | 376.01 | Surveyed |
| 221.6 | Eulo | –28.1387 | 144.8632 | 159.661 | Yes | No | n/a | No | n/a | 376.06 | Surveyed |
| 221.7 | Eulo | –28.1363 | 144.8649 | 160 | Yes | No | n/a | No | n/a | 375.83 | Surveyed |
| 222 | Eulo | –28.0857 | 145.0261 | 159.519 | Yes | No | n/a | No | n/a | 359.27 | Surveyed |
| 222.1 | Eulo | –28.0856 | 145.0251 | 162 | Yes | No | n/a | No | n/a | 359.35 | Surveyed |
| 223 | Eulo | –27.9521 | 144.7699 | 171.457 | Yes | No | n/a | No | n/a | 380.24 | Surveyed |
| 224 | Eulo | –27.9535 | 144.7696 | 171.591 | Yes | No | n/a | No | n/a | 380.30 | Surveyed |
| 225 | Eulo | –27.9487 | 144.7731 | 172.089 | Yes | No | n/a | No | n/a | 379.87 | Surveyed |
| 225.1 | Eulo | –27.9486 | 144.7734 | 174 | Yes | No | n/a | No | n/a | 379.84 | Surveyed |
| 226 | Eulo | –27.9424 | 144.7774 | 172.209 | Yes | No | n/a | No | n/a | 379.32 | Surveyed |
| 227 | Eulo | –27.9412 | 144.7784 | 172.152 | Yes | No | n/a | No | n/a | 379.20 | Surveyed |
| 227.1 | Eulo | –27.9408 | 144.7790 | 176 | Yes | No | n/a | No | n/a | 379.13 | Surveyed |
| 227.2 | Eulo | –27.9407 | 144.7788 | 176 | Yes | No | n/a | No | n/a | 379.15 | Surveyed |
| 228 | Eulo | –27.9100 | 144.8724 | 179.768 | Yes | No | n/a | No | n/a | 369.57 | Surveyed |
| 228.1 | Eulo | –27.9100 | 144.8724 | 180 | Yes | No | n/a | No | n/a | 369.57 | Surveyed |
| 229 | Eulo | –27.9384 | 144.8575 | 173.816 | Yes | No | n/a | No | n/a | 371.60 | Surveyed |
| 229.1 | Eulo | –27.9323 | 144.8588 | 178 | Yes | No | n/a | No | n/a | 371.34 | Surveyed |
| 229.2 | Eulo | –27.9384 | 144.8575 | 178 | Yes | No | n/a | No | n/a | 371.60 | Surveyed |
| 230.1 | Eulo | –27.9275 | 144.8852 | 180.288 | Yes | No | n/a | No | n/a | 368.71 | Surveyed |
| 230.2 | Eulo | –27.9282 | 144.8849 | 178.981 | Yes | No | n/a | No | n/a | 368.75 | Surveyed |
| 230.3 | Eulo | –27.9279 | 144.8849 | 178.47 | Yes | No | n/a | No | n/a | 368.75 | Surveyed |
| 230.4 | Eulo | –27.9286 | 144.8859 | 179.269 | Yes | No | n/a | No | n/a | 368.66 | Surveyed |
| 230.5 | Eulo | –27.9299 | 144.8871 | 177.88 | Yes | No | n/a | No | n/a | 368.58 | Surveyed |
| 230.6 | Eulo | –27.9305 | 144.8877 | 178.16 | Yes | No | n/a | No | n/a | 368.54 | Surveyed |
| 230.7 | Eulo | –27.9258 | 144.8874 | 180.447 | Yes | No | n/a | No | n/a | 368.47 | Surveyed |
| 231.1 | Eulo | –27.8678 | 144.9102 | 184.847 | Yes | No | n/a | No | n/a | 365.09 | Surveyed |
| 231.2 | Eulo | –27.8674 | 144.9099 | 185.057 | Yes | No | n/a | No | n/a | 365.11 | Surveyed |
| 231.3 | Eulo | –27.8636 | 144.9102 | 188 | Yes | No | n/a | No | n/a | 365.00 | Surveyed |
| 232.1 | Eulo | –27.9162 | 144.8816 | 181 | Yes | No | n/a | No | n/a | 368.81 | Surveyed |
| 232.2 | Eulo | –27.9151 | 144.8811 | 178.206 | Yes | No | n/a | No | n/a | 368.84 | Surveyed |
| 232.3 | Eulo | –27.9154 | 144.8812 | 178.144 | Yes | No | n/a | No | n/a | 368.84 | Surveyed |
| 232.4 | Eulo | –27.9157 | 144.8810 | 178.255 | Yes | No | n/a | No | n/a | 368.86 | Surveyed |
| 233.1 | Eulo | –27.9058 | 144.8888 | 178.87 | Yes | No | n/a | No | n/a | 367.91 | Surveyed |
| 233.2 | Eulo | –27.9023 | 144.8907 | 178.774 | Yes | No | n/a | No | n/a | 367.65 | Surveyed |
| 233.3 | Eulo | –27.9014 | 144.8902 | 179.082 | Yes | No | n/a | No | n/a | 367.68 | Surveyed |
| 233.4 | Eulo | –27.9013 | 144.8905 | 182 | Yes | No | n/a | No | n/a | 367.65 | Surveyed |
| 233.5 | Eulo | –27.9004 | 144.8897 | 179.011 | Yes | No | n/a | No | n/a | 367.71 | Surveyed |
| 234 | Eulo | –28.1585 | 145.0363 | 157.34 | Yes | Yes | Eragrostis fenshamii | Yes | Eragrostis fenshamii, Myriophyllum artesium | 360.42 | Surveyed |
| 235.1 | Eulo | –28.1390 | 144.8570 | 160 | Yes | No | n/a | No | n/a | 376.65 | Surveyed |
| 235.2 | Eulo | –28.1390 | 144.8570 | 157.877 | Yes | No | n/a | No | n/a | 376.65 | Surveyed |
| 235.3 | Eulo | –28.1397 | 144.8558 | 160 | Yes | No | n/a | No | n/a | 376.79 | Surveyed |
| 236 | Eulo | –28.1817 | 144.8056 | 161.051 | Yes | No | n/a | No | n/a | 382.70 | Surveyed |
| 237 | Eulo | –28.1923 | 144.8434 | 153.892 | Yes | No | n/a | No | n/a | 379.47 | Surveyed |
| 237.1 | Eulo | –28.1942 | 144.8423 | 155.968 | Yes | No | n/a | No | n/a | 379.63 | Surveyed |
| 237.2 | Eulo | –28.1949 | 144.8359 | 155.984 | Yes | No | n/a | No | n/a | 380.25 | Surveyed |
| 237.3 | Eulo | –28.1962 | 144.8333 | 155.822 | Yes | No | n/a | No | n/a | 380.53 | Surveyed |
| 237.4 | Eulo | –28.1958 | 144.8335 | 155.271 | Yes | No | n/a | No | n/a | 380.50 | Surveyed |
| 238 | Eulo | –28.1864 | 144.8481 | 157 | Yes | Yes | Eragrostis fenshamii | Yes | Eragrostis fenshamii, Myriophyllum artesium | 378.86 | Surveyed |
| 238.1 | Eulo | –28.1853 | 144.8484 | 155.006 | Yes | No | n/a | No | n/a | 378.79 | Surveyed |
| 239 | Eulo | –28.1820 | 144.9727 | 153.799 | Yes | No | n/a | No | n/a | 367.08 | Surveyed |
| 239.1 | Eulo | –28.1819 | 144.9736 | 157 | Yes | No | n/a | No | n/a | 366.99 | Surveyed |
| 239.2 | Eulo | –28.1799 | 144.9681 | 155.972 | Yes | No | n/a | No | n/a | 367.44 | Surveyed |
| 284 | Springsure | –25.8296 | 149.0414 | 356.98 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 285 | Springsure | –25.7627 | 148.7698 | 356.81 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 286 | Springsure | –25.7982 | 148.7691 | 348.2 | Yes | Yes | Eriocaulon carsonii | Yes | Eriocaulon carsonii | 0.00 | Surveyed |
| 286.1 | Springsure | –25.7982 | 148.7703 | 350 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 286.2 | Springsure | –25.7980 | 148.7702 | 350 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 286.3 | Springsure | –25.7976 | 148.7687 | 351 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 287 | Springsure | –25.7981 | 148.7756 | 349.48 | Yes | Yes | Eriocaulon carsonii | Yes | Eriocaulon carsonii | 0.00 | Surveyed |
| 299 | Springsure | –24.7475 | 146.5372 | 490.38879 | No | No | n/a | No | n/a | 173.08 | Surveyed |
| 300 | Springsure | –24.7736 | 146.9881 | 507.26233 | No | No | n/a | No | n/a | 127.96 | Surveyed |
| 301 | Springsure | –24.8569 | 147.3997 | 611.38898 | No | No | n/a | No | n/a | 86.75 | Surveyed |
| 302 | Springsure | –25.0710 | 147.6760 | 808 | No | No | n/a | No | n/a | 59.88 | Surveyed |
| 303 | Springsure | –24.8781 | 147.7903 | 812.33026 | No | No | n/a | No | n/a | 47.75 | Surveyed |
| 304 | Non–GAB | –24.8534 | 147.8631 | 861 | No | No | n/a | No | n/a | 40.44 | Surveyed |
| 305 | Springsure | –24.8817 | 147.8239 | 791.61115 | No | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 44.38 | Surveyed |
| 306 | Springsure | –24.8844 | 147.8208 | 786.18518 | No | No | n/a | No | n/a | 44.67 | Surveyed |
| 307 | Springsure | –24.8825 | 147.8186 | 789.55554 | No | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 44.90 | Surveyed |
| 308 | Springsure | –24.9281 | 147.8264 | 784.70679 | No | No | n/a | No | n/a | 44.08 | Surveyed |
| 309 | Springsure | –24.9434 | 147.9611 | 786.5 | No | No | n/a | No | n/a | 30.62 | Surveyed |
| 310 | Springsure | –24.9464 | 147.9581 | 784.411 | No | No | n/a | No | n/a | 30.92 | Surveyed |
| 311 | Non–GAB | –24.8684 | 147.9391 | 1016 | No | No | n/a | No | n/a | 32.92 | Surveyed |
| 312 | Non–GAB | –24.8244 | 147.9111 | 1032 | No | No | n/a | No | n/a | 35.65 | Surveyed |
| 313 | Non–GAB | –24.8154 | 147.8891 | 983 | No | No | n/a | No | n/a | 37.85 | Surveyed |
| 314 | Non–GAB | –24.8894 | 148.0261 | 904 | No | No | n/a | No | n/a | 24.16 | Surveyed |
| 315 | Springsure | –24.9424 | 147.9601 | 785.611 | No | No | n/a | No | n/a | 30.72 | Surveyed |
| 316 | Springsure | –25.1064 | 148.0781 | 720.111 | No | No | n/a | No | n/a | 23.52 | Surveyed |
| 316.1 | Springsure | –25.0604 | 148.0551 | 742.608 | No | No | n/a | No | n/a | 22.98 | Surveyed |
| 316.2 | Springsure | –25.0604 | 148.0341 | 754.756 | No | No | n/a | No | n/a | 24.92 | Surveyed |
| 317 | Springsure | –25.5054 | 147.9771 | 481.232 | No | No | n/a | No | n/a | 65.44 | Surveyed |
| 317.1 | Springsure | –25.5064 | 147.9781 | 490.736 | No | No | n/a | No | n/a | 65.49 | Surveyed |
| 317.2 | Springsure | –25.5084 | 147.9731 | 485.55 | No | No | n/a | No | n/a | 65.92 | Surveyed |
| 318 | Springsure | –25.5084 | 147.9771 | 482.361 | No | No | n/a | No | n/a | 65.74 | Surveyed |
| 319 | Springsure | –25.7255 | 150.2689 | 228.83 | Yes | No | n/a | Yes | Myriophyllum artesium | 20.36 | Surveyed |
| 320 | Springsure | –25.7318 | 150.2519 | 227.64 | Yes | No | n/a | Yes | Myriophyllum artesium | 18.65 | Surveyed |
| 320.1 | Springsure | –25.7319 | 150.2520 | 232 | Yes | No | n/a | No | n/a | 18.64 | Surveyed |
| 321 | Springsure | –25.7294 | 150.2500 | 225.62 | Yes | Yes | Eriocaulon carsonii | Yes | Myriophyllum artesium, Eriocaulon carsonii | 18.70 | Surveyed |
| 321.1 | Springsure | –25.7297 | 150.2499 | 224.85 | Yes | No | n/a | No | n/a | 18.67 | Surveyed |
| 321.2 | Springsure | –25.7297 | 150.2500 | 224.5 | Yes | No | n/a | No | n/a | 18.67 | Surveyed |
| 321.3 | Springsure | –25.7298 | 150.2501 | 224.99 | Yes | No | n/a | No | n/a | 18.68 | Surveyed |
| 321.4 | Springsure | –25.7296 | 150.2499 | 225.39 | Yes | No | n/a | No | n/a | 18.67 | Surveyed |
| 321.5 | Springsure | –25.7294 | 150.2498 | 225.4 | Yes | No | n/a | No | n/a | 18.68 | Surveyed |
| 321.6 | Springsure | –25.7293 | 150.2501 | 225.47 | Yes | No | n/a | No | n/a | 18.70 | Surveyed |
| 321.7 | Springsure | –25.7289 | 150.2501 | 225.17 | Yes | No | n/a | No | n/a | 18.74 | Surveyed |
| 321.8 | Springsure | –25.7287 | 150.2502 | 225.99 | Yes | No | n/a | No | n/a | 18.76 | Surveyed |
| 322 | Springsure | –24.8922 | 147.5239 | 636.61108 | No | No | n/a | No | n/a | 74.31 | Surveyed |
| 340 | Springsure | –25.7940 | 148.7732 | 350.06 | Yes | Yes | Eriocaulon carsonii | Yes | Eriocaulon carsonii | 0.00 | Surveyed |
| 380 | Eulo | –28.1328 | 144.5029 | 171.571 | Yes | No | n/a | No | n/a | 409.86 | Surveyed |
| 380.1 | Eulo | –28.1326 | 144.5029 | 174 | Yes | No | n/a | No | n/a | 409.85 | Surveyed |
| 380.2 | Eulo | –28.1317 | 144.5028 | 174 | Yes | No | n/a | No | n/a | 409.84 | Surveyed |
| 381 | Eulo | –28.2356 | 144.6275 | 201 | Yes | No | n/a | Yes | Myriophyllum artesium | 400.95 | Surveyed |
| 381.1 | Eulo | –28.2357 | 144.6274 | 201 | Yes | No | n/a | No | n/a | 400.96 | Surveyed |
| 381.2 | Eulo | –28.2357 | 144.6274 | 201 | Yes | No | n/a | No | n/a | 400.96 | Surveyed |
| 381.3 | Eulo | –28.2214 | 144.6338 | 199 | Yes | No | n/a | No | n/a | 399.94 | Surveyed |
| 382.1 | Eulo | –28.2537 | 144.7417 | 167.532 | Yes | No | n/a | Yes | Myriophyllum artesium | 390.82 | Surveyed |
| 382.2 | Eulo | –28.2536 | 144.7421 | 167.581 | Yes | No | n/a | Yes | Myriophyllum artesium | 390.78 | Surveyed |
| 382.3 | Eulo | –28.2530 | 144.7420 | 167.7 | Yes | No | n/a | Yes | Myriophyllum artesium | 390.77 | Surveyed |
| 382.4 | Eulo | –28.2527 | 144.7421 | 167.569 | Yes | No | n/a | Yes | Myriophyllum artesium | 390.75 | Surveyed |
| 382.5 | Eulo | –28.2524 | 144.7424 | 167.318 | Yes | No | n/a | No | n/a | 390.72 | Surveyed |
| 382.6 | Eulo | –28.2521 | 144.7424 | 167.477 | Yes | No | n/a | No | n/a | 390.70 | Surveyed |
| 383 | Eulo | –28.1309 | 144.7278 | 178 | Yes | No | n/a | No | n/a | 388.58 | Surveyed |
| 383.1 | Eulo | –28.1331 | 144.7273 | 178 | Yes | No | n/a | No | n/a | 388.69 | Surveyed |
| 383.2 | Eulo | –28.1359 | 144.7269 | 180 | Yes | No | n/a | No | n/a | 388.80 | Surveyed |
| 385 | Eulo | –28.2740 | 144.9861 | 151.075 | Yes | No | n/a | No | n/a | 368.77 | Surveyed |
| 385.1 | Eulo | –28.2716 | 144.9826 | 151.463 | Yes | No | n/a | No | n/a | 369.02 | Surveyed |
| 385.2 | Eulo | –28.2720 | 144.9840 | 156 | Yes | No | n/a | No | n/a | 368.90 | Surveyed |
| 386.1 | Eulo | –28.3199 | 144.2126 | 143.203 | Yes | No | n/a | No | n/a | 442.24 | Surveyed |
| 386.2 | Eulo | –28.3207 | 144.2112 | 148 | Yes | No | n/a | No | n/a | 442.39 | Surveyed |
| 386.3 | Eulo | –28.3131 | 144.2161 | 143.095 | Yes | No | n/a | No | n/a | 441.71 | Surveyed |
| 386.4 | Eulo | –28.3240 | 144.2049 | 143.721 | Yes | No | n/a | No | n/a | 443.07 | Surveyed |
| 387 | Eulo | –28.4990 | 144.1883 | 146.191 | Yes | No | n/a | No | n/a | 450.03 | Surveyed |
| 408 | Springsure | –25.1795 | 148.5704 | 335.28 | No | No | n/a | No | n/a | 26.31 | Surveyed |
| 497 | Springsure | –25.1410 | 151.1370 | 193 | No | No | n/a | No | n/a | 128.63 | Surveyed |
| 498 | Springsure | –24.9790 | 151.0410 | 295 | No | No | n/a | No | n/a | 133.69 | Surveyed |
| 499 | Springsure | –25.7002 | 149.1289 | 248.27 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 500 | Springsure | –25.7198 | 149.1048 | 257.84 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 500.1 | Springsure | –25.7282 | 149.1005 | 251.01 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 534 | Springsure | –25.7326 | 149.1028 | 264.24 | Yes | No | n/a | No | n/a | 0.00 | Surveyed |
| 535 | Springsure | –25.7202 | 149.0275 | 289 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 536 | Springsure | –25.7145 | 149.0654 | 275.28 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 536.1 | Springsure | –25.7136 | 149.0654 | 270.78 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 536.2 | Springsure | –25.7155 | 149.0648 | 278.61 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 537 | Springsure | –25.7283 | 149.0939 | 254.29 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 538 | Springsure | –25.3544 | 149.3631 | 262 | No | No | n/a | Yes | Lobelia membranacea listed under former name Pratia podenzanae | 27.72 | Surveyed |
| 539 | Springsure | –25.0994 | 149.2731 | 410 | No | No | n/a | Yes | Lobelia membranacea listed under former name Pratia podenzanae | 31.84 | Surveyed |
| 539.1 | Springsure | –25.1134 | 149.2331 | 480 | No | No | n/a | No | n/a | 27.63 | Surveyed |
| 540 | Springsure | –24.5341 | 149.1696 | 212.82 | Yes | No | n/a | No | n/a | 66.32 | Surveyed |
| 541 | Springsure | –24.3159 | 149.1678 | 220 | No | No | n/a | No | n/a | 77.51 | Surveyed |
| 542 | Springsure | –24.2984 | 149.1791 | 222 | No | No | n/a | No | n/a | 76.08 | Surveyed |
| 543 | Springsure | –24.3144 | 149.1551 | 233 | No | No | n/a | No | n/a | 76.92 | Surveyed |
| 544 | Springsure | –24.3084 | 149.1741 | 261 | No | No | n/a | No | n/a | 76.94 | Surveyed |
| 544.1 | Springsure | –24.3094 | 149.1701 | 219 | No | No | n/a | No | n/a | 76.91 | Surveyed |
| 544.2 | Springsure | –24.3094 | 149.1741 | 213 | No | No | n/a | No | n/a | 77.05 | Surveyed |
| 544.3 | Springsure | –24.3084 | 149.1751 | 210 | No | No | n/a | No | n/a | 76.98 | Surveyed |
| 545 | Springsure | –24.1724 | 149.1561 | 248 | No | No | n/a | No | n/a | 62.17 | Surveyed |
| 546 | Springsure | –24.2574 | 149.1347 | 248 | No | No | n/a | Yes | Lobelia membranacea listed under former name Pratia podenzanae | 70.22 | Surveyed |
| 547 | Springsure | –23.8174 | 149.2231 | 213 | No | No | n/a | No | n/a | 36.34 | Surveyed |
| 548 | Springsure | –23.7754 | 149.1751 | 213 | No | No | n/a | No | n/a | 29.92 | Surveyed |
| 549 | Springsure | –23.8614 | 149.1001 | 737 | No | No | n/a | No | n/a | 29.50 | Surveyed |
| 550 | Springsure | –23.7594 | 149.1041 | 750 | No | No | n/a | No | n/a | 22.70 | Surveyed |
| 551 | Springsure | –24.0344 | 149.0221 | 308 | No | No | n/a | No | n/a | 42.99 | Surveyed |
| 552 | Springsure | –24.0354 | 149.0351 | 547 | No | No | n/a | No | n/a | 43.47 | Surveyed |
| 553 | Springsure | –25.0794 | 148.2296 | 573 | No | No | n/a | Yes | Lobelia membranacea listed under former name Pratia podenzanae | 11.58 | Surveyed |
| 554 | Springsure | –25.0435 | 148.2009 | 575 | No | No | n/a | Yes | Lobelia membranacea listed under former name Pratia podenzanae | 9.61 | Surveyed |
| 554.3 | Springsure | –25.0354 | 148.2042 | 653 | No | No | n/a | No | n/a | 8.74 | Surveyed |
| 555 | Springsure | –25.0574 | 148.2101 | 536 | No | No | n/a | No | n/a | 10.25 | Surveyed |
| 556 | Springsure | –25.0573 | 148.2136 | 521 | No | No | n/a | No | n/a | 10.05 | Surveyed |
| 675 | Springsure | –25.1792 | 148.5640 | 323.97 | No | No | n/a | No | n/a | 26.94 | Surveyed |
| 676 | Springsure | –25.1795 | 148.5635 | 325.89 | No | No | n/a | No | n/a | 27.00 | Surveyed |
| 677 | Springsure | –25.0378 | 148.2000 | 619.03 | No | No | n/a | Yes | Livistona nitida | 9.23 | Surveyed |
| 678 | Springsure | –25.0372 | 148.2013 | 624.83 | No | No | n/a | Yes | Livistona nitida | 9.09 | Surveyed |
| 679 | Springsure | –26.2785 | 148.6959 | 353.6 | No | No | n/a | No | n/a | 14.96 | Surveyed |
| 680 | Springsure | –26.2731 | 148.6868 | 366.46 | No | No | n/a | No | n/a | 16.02 | Surveyed |
| 680.1 | Springsure | –26.2736 | 148.6873 | 364.69 | No | No | n/a | No | n/a | 15.95 | Surveyed |
| 681 | Springsure | –25.4361 | 150.0230 | 185.63 | Yes | Yes | Arthraxon hispidus | Yes | Arthraxon hispidus | 44.36 | Surveyed |
| 682 | Springsure | –25.8081 | 148.7342 | 355.6 | No | No | n/a | No | n/a | 1.68 | Surveyed |
| 683 | Springsure | –25.4531 | 150.0206 | 175.74 | Yes | No | n/a | No | n/a | 42.45 | Surveyed |
| 684 | Springsure | –25.7258 | 150.2694 | 229.73 | Yes | No | n/a | No | n/a | 20.37 | Surveyed |
| 685 | Springsure | –25.7231 | 150.2665 | 219.16 | No | No | n/a | No | n/a | 20.36 | Surveyed |
| 686 | Springsure | –25.7948 | 148.7734 | 346.86 | Yes | No | n/a | No | n/a | 0.00 | Surveyed |
| 687 | Springsure | –25.7948 | 148.7738 | 351.97 | Yes | Yes | Eriocaulon carsonii | Yes | Eriocaulon carsonii | 0.00 | Surveyed |
| 687.1 | Springsure | –25.7946 | 148.7738 | 348.58 | Yes | No | n/a | No | n/a | 0.00 | Surveyed |
| 687.2 | Springsure | –25.7946 | 148.7738 | 348.4 | Yes | No | n/a | No | n/a | 0.00 | Surveyed |
| 687.3 | Springsure | –25.7942 | 148.7736 | 348.96 | Yes | No | n/a | No | n/a | 0.00 | Surveyed |
| 687.4 | Springsure | –25.7941 | 148.7735 | 349.22 | Yes | Yes | Eriocaulon carsonii | Yes | Eriocaulon carsonii | 0.00 | Surveyed |
| 687.5 | Springsure | –25.7937 | 148.7733 | 350.65 | Yes | No | n/a | No | n/a | 0.00 | Surveyed |
| 687.6 | Springsure | –25.7936 | 148.7733 | 350.31 | Yes | Yes | Eriocaulon carsonii | Yes | Eriocaulon carsonii | 0.00 | Surveyed |
| 688 | Springsure | –25.7951 | 148.7737 | 348.91 | Yes | No | n/a | No | n/a | 0.00 | Surveyed |
| 689 | Springsure | –25.7940 | 148.7728 | 348.47 | Yes | Yes | Eriocaulon carsonii | Yes | Eriocaulon carsonii | 0.00 | Surveyed |
| 690 | Springsure | –25.4370 | 150.0880 | 181.837 | No | No | n/a | Yes | Livistona nitida | 44.25 | Surveyed |
| 691 | Springsure | –25.4873 | 150.0471 | 170.93 | Yes | No | n/a | No | n/a | 38.62 | Surveyed |
| 692 | Springsure | –25.7260 | 149.1037 | 219.48 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 693 | Springsure | –25.7207 | 149.0296 | 279.75 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 694 | Springsure | –25.7124 | 149.0726 | 271.13 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 695 | Springsure | –25.7254 | 149.0869 | 267.57 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 696 | Springsure | –25.7255 | 149.0869 | 266.06 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 697 | Springsure | –25.7256 | 149.0867 | 267.97 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 698 | Springsure | –25.7256 | 149.0867 | 268.21 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 699 | Springsure | –25.7258 | 149.0866 | 267.68 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 700 | Springsure | –25.2107 | 148.1189 | 611.01 | No | No | n/a | No | n/a | 29.64 | Surveyed |
| 701 | Springsure | –25.2043 | 148.2292 | 564.79 | No | No | n/a | No | n/a | 25.23 | Surveyed |
| 702 | Springsure | –26.2703 | 149.2433 | 338.42 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 703 | Springsure | –26.2853 | 149.2345 | 359.72 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 704 | Springsure | –25.6797 | 149.1273 | 255.39 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 705 | Springsure | –25.2303 | 148.6591 | 380.48 | No | No | n/a | No | n/a | 17.46 | Surveyed |
| 706 | Springsure | –25.4791 | 148.7315 | 398.7 | No | No | n/a | No | n/a | 2.91 | Surveyed |
| 707 | Springsure | –25.4787 | 148.7321 | 399.55 | No | No | n/a | No | n/a | 2.91 | Surveyed |
| 708 | Springsure | –25.4785 | 148.7326 | 401.24 | No | No | n/a | No | n/a | 2.89 | Surveyed |
| 709 | Springsure | –25.4785 | 148.7335 | 402.48 | No | No | n/a | No | n/a | 2.83 | Surveyed |
| 710 | Springsure | –24.6597 | 149.1141 | 295.22 | No | No | n/a | No | n/a | 51.20 | Surveyed |
| 711 | Springsure | –26.8742 | 150.4371 | 289.2 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 711.1 | Springsure | –26.8740 | 150.4372 | 289.65 | No | No | n/a | No | n/a | 0.00 | Surveyed |
| 712 | Springsure | –25.1312 | 148.1904 | 455.62 | No | Yes | Phaius australis | Yes | Phaius australis | 18.43 | Surveyed |
| 713 | Springsure | –25.0907 | 148.2290 | 455.62 | No | No | n/a | Yes | Livistona nitida | 12.80 | Surveyed |
| 714 | Springsure | –25.0888 | 148.2238 | 820.29 | No | No | n/a | No | n/a | 12.77 | Surveyed |
| 715 | Springsure | –25.0338 | 148.2046 | 677 | No | No | n/a | No | n/a | 8.59 | Surveyed |
| 716 | Springsure | –25.8077 | 148.7340 | 358.21 | No | No | n/a | No | n/a | 1.70 | Surveyed |
| 850 | Springsure | –25.0960 | 149.5413 | 316.24 | No | No | n/a | No | n/a | 52.62 | Surveyed |
| 851 | Springsure | –24.9467 | 149.6135 | 289.32 | No | No | n/a | No | n/a | 68.14 | Surveyed |
| 852 | Springsure | –23.9330 | 149.1860 | 344.26 | No | Yes | Phaius australis | Yes | Phaius australis | 41.26 | Surveyed |
| 900 | Eulo | –28.2856 | 144.7332 | 162 | Yes | No | n/a | No | n/a | 392.61 | Surveyed |
| 901 | Eulo | –28.2031 | 144.6347 | 202 | Yes | No | n/a | No | n/a | 399.33 | Surveyed |
| 901.1 | Eulo | –28.2058 | 144.6334 | 208 | Yes | No | n/a | No | n/a | 399.53 | Surveyed |
| 902 | Eulo | –28.2523 | 144.3553 | 146 | Yes | No | n/a | No | n/a | 426.94 | Surveyed |
| 902.1 | Eulo | –28.2549 | 144.3588 | 158 | Yes | No | n/a | No | n/a | 426.68 | Surveyed |
| 903 | Eulo | –28.3735 | 144.5564 | 187 | Yes | No | n/a | No | n/a | 411.84 | Surveyed |
| 903.1 | Eulo | –28.3817 | 144.5237 | 181 | Yes | No | n/a | No | n/a | 415.14 | Surveyed |
| 904 | Eulo | –28.3425 | 144.5586 | 199 | Yes | No | n/a | No | n/a | 410.64 | Surveyed |
| 905 | Eulo | –28.2684 | 144.5686 | 216 | Yes | No | n/a | No | n/a | 407.42 | Surveyed |
| 905.1 | Eulo | –28.2663 | 144.5660 | 212 | Yes | No | n/a | No | n/a | 407.60 | Surveyed |
| 906 | Eulo | –28.3186 | 144.5547 | 196 | Yes | No | n/a | No | n/a | 410.25 | Surveyed |
| 907 | Eulo | –28.4834 | 144.4964 | 163 | Yes | No | n/a | No | n/a | 421.11 | Surveyed |
| 908.1 | Eulo | –28.2464 | 144.3536 | 155 | Yes | No | n/a | No | n/a | 426.94 | Surveyed |
| 908.1 | Eulo | –28.2470 | 144.3545 | 154 | Yes | No | n/a | No | n/a | 426.87 | Surveyed |
| 908.1 | Eulo | –28.2465 | 144.3547 | 154 | Yes | No | n/a | No | n/a | 426.84 | Surveyed |
| 908.1 | Eulo | –28.2459 | 144.3548 | 155 | Yes | No | n/a | No | n/a | 426.82 | Surveyed |
| 908.1 | Eulo | –28.2458 | 144.3549 | 155 | Yes | No | n/a | No | n/a | 426.80 | Surveyed |
| 908.1 | Eulo | –28.2459 | 144.3533 | 156 | Yes | No | n/a | No | n/a | 426.96 | Surveyed |
| 908.2 | Eulo | –28.2462 | 144.3530 | 156 | Yes | No | n/a | No | n/a | 426.99 | Surveyed |
| 908.3 | Eulo | –28.2464 | 144.3529 | 156 | Yes | No | n/a | No | n/a | 427.01 | Surveyed |
| 908.4 | Eulo | –28.2476 | 144.3530 | 155 | Yes | No | n/a | No | n/a | 427.03 | Surveyed |
| 908.5 | Eulo | –28.2477 | 144.3532 | 155 | Yes | No | n/a | No | n/a | 427.02 | Surveyed |
| 908.6 | Eulo | –28.2485 | 144.3536 | 154 | Yes | No | n/a | No | n/a | 427.00 | Surveyed |
| 908.7 | Eulo | –28.2482 | 144.3540 | 154 | Yes | No | n/a | No | n/a | 426.95 | Surveyed |
| 908.8 | Eulo | –28.2480 | 144.3541 | 154 | Yes | No | n/a | No | n/a | 426.93 | Surveyed |
| 908.9 | Eulo | –28.2477 | 144.3539 | 154 | Yes | No | n/a | No | n/a | 426.95 | Surveyed |
| 909.1 | Eulo | –28.2386 | 144.3348 | 149 | Yes | No | n/a | No | n/a | 428.49 | Surveyed |
| 909.2 | Eulo | –28.2369 | 144.3342 | 149 | Yes | No | n/a | No | n/a | 428.51 | Surveyed |
| 909.3 | Eulo | –28.2353 | 144.3344 | 150 | Yes | No | n/a | No | n/a | 428.44 | Surveyed |
| 909.4 | Eulo | –28.2310 | 144.3342 | 149 | Yes | No | n/a | No | n/a | 428.35 | Surveyed |
| 909.5 | Eulo | –28.2290 | 144.3328 | 150 | Yes | No | n/a | No | n/a | 428.43 | Surveyed |
| 910.1 | Eulo | –28.1442 | 145.1260 | 162.348 | Yes | No | n/a | No | n/a | 351.61 | Surveyed |
| 910.2 | Eulo | –28.1443 | 145.1262 | 161.588 | Yes | No | n/a | No | n/a | 351.60 | Surveyed |
| 910.3 | Eulo | –28.1442 | 145.1263 | 165 | Yes | No | n/a | No | n/a | 351.58 | Surveyed |
| 910.4 | Eulo | –28.1424 | 145.1242 | 164 | Yes | No | n/a | No | n/a | 351.72 | Surveyed |
| 910.5 | Eulo | –28.1416 | 145.1223 | 161.497 | Yes | No | n/a | No | n/a | 351.88 | Surveyed |
| 911 | Eulo | –28.1876 | 145.0274 | 156.071 | Yes | No | n/a | No | n/a | 362.15 | Surveyed |
| 911.1 | Eulo | –28.1877 | 145.0282 | 160 | Yes | No | n/a | No | n/a | 362.07 | Surveyed |
| 911.2 | Eulo | –28.1881 | 145.0284 | 160 | Yes | No | n/a | No | n/a | 362.07 | Surveyed |
| 911.3 | Eulo | –28.1893 | 145.0283 | 160 | Yes | No | n/a | No | n/a | 362.12 | Surveyed |
| 912 | Eulo | –28.1815 | 145.0237 | 160 | Yes | No | n/a | No | n/a | 362.31 | Surveyed |
| 912.1 | Eulo | –28.1852 | 145.0240 | 161 | Yes | No | n/a | No | n/a | 362.39 | Surveyed |
| 912.2 | Eulo | –28.1804 | 145.0240 | 160 | Yes | No | n/a | No | n/a | 362.24 | Surveyed |
| 913 | Eulo | –28.4543 | 144.9946 | 150.292 | Yes | No | n/a | No | n/a | 374.54 | Surveyed |
| 914 | Eulo | –28.4970 | 145.0070 | 151 | Yes | No | n/a | No | n/a | 375.11 | Surveyed |
| 914.1 | Eulo | –28.4857 | 145.0088 | 148.725 | Yes | No | n/a | No | n/a | 374.49 | Surveyed |
| 914.2 | Eulo | –28.4856 | 145.0091 | 148.324 | Yes | No | n/a | No | n/a | 374.46 | Surveyed |
| 914.3 | Eulo | –28.4832 | 145.0164 | 148.678 | Yes | No | n/a | No | n/a | 373.70 | Surveyed |
| 914.4 | Eulo | –28.4828 | 145.0165 | 148.727 | Yes | No | n/a | No | n/a | 373.68 | Surveyed |
| 915 | Eulo | –28.4391 | 145.0205 | 152.842 | Yes | No | n/a | No | n/a | 371.60 | Surveyed |
| 916.1 | Eulo | –28.1967 | 145.1730 | 167 | Yes | No | n/a | No | n/a | 348.90 | Surveyed |
| 916.1 | Eulo | –28.1973 | 145.1732 | 167 | Yes | No | n/a | No | n/a | 348.90 | Surveyed |
| 916.1 | Eulo | –28.1971 | 145.1732 | 167 | Yes | No | n/a | No | n/a | 348.89 | Surveyed |
| 916.2 | Eulo | –28.1963 | 145.1730 | 167 | Yes | No | n/a | No | n/a | 348.89 | Surveyed |
| 916.3 | Eulo | –28.1962 | 145.1739 | 166 | Yes | No | n/a | No | n/a | 348.80 | Surveyed |
| 916.4 | Eulo | –28.1955 | 145.1743 | 167 | Yes | No | n/a | No | n/a | 348.74 | Surveyed |
| 916.5 | Eulo | –28.1967 | 145.1735 | 167 | Yes | No | n/a | No | n/a | 348.85 | Surveyed |
| 916.6 | Eulo | –28.1969 | 145.1738 | 167 | Yes | No | n/a | No | n/a | 348.83 | Surveyed |
| 916.7 | Eulo | –28.1972 | 145.1741 | 167 | Yes | No | n/a | No | n/a | 348.81 | Surveyed |
| 916.8 | Eulo | –28.1977 | 145.1744 | 167 | Yes | No | n/a | No | n/a | 348.80 | Surveyed |
| 916.9 | Eulo | –28.1974 | 145.1738 | 167 | Yes | No | n/a | No | n/a | 348.85 | Surveyed |
| 917.1 | Eulo | –28.1319 | 145.1204 | 163 | Yes | No | n/a | No | n/a | 351.76 | Surveyed |
| 917.1 | Eulo | –28.1300 | 145.1200 | 163 | Yes | No | n/a | No | n/a | 351.74 | Surveyed |
| 917.1 | Eulo | –28.1294 | 145.1205 | 163 | Yes | No | n/a | No | n/a | 351.68 | Surveyed |
| 917.1 | Eulo | –28.1294 | 145.1214 | 164 | Yes | No | n/a | No | n/a | 351.59 | Surveyed |
| 917.1 | Eulo | –28.1290 | 145.1214 | 164 | Yes | No | n/a | No | n/a | 351.58 | Surveyed |
| 917.2 | Eulo | –28.1315 | 145.1205 | 163 | Yes | No | n/a | No | n/a | 351.74 | Surveyed |
| 917.3 | Eulo | –28.1321 | 145.1178 | 164 | Yes | No | n/a | No | n/a | 352.01 | Surveyed |
| 917.4 | Eulo | –28.1285 | 145.1152 | 165 | Yes | No | n/a | No | n/a | 352.14 | Surveyed |
| 917.5 | Eulo | –28.1284 | 145.1160 | 165 | Yes | No | n/a | No | n/a | 352.07 | Surveyed |
| 917.6 | Eulo | –28.1281 | 145.1160 | 165 | Yes | No | n/a | No | n/a | 352.06 | Surveyed |
| 917.7 | Eulo | –28.1281 | 145.1167 | 165 | Yes | No | n/a | No | n/a | 351.99 | Surveyed |
| 917.8 | Eulo | –28.1273 | 145.1172 | 165 | Yes | No | n/a | No | n/a | 351.92 | Surveyed |
| 917.9 | Eulo | –28.1306 | 145.1197 | 163 | Yes | No | n/a | No | n/a | 351.79 | Surveyed |
| 918.1 | Eulo | –28.1174 | 145.1267 | 163 | Yes | No | n/a | No | n/a | 350.73 | Surveyed |
| 918.1 | Eulo | –28.1189 | 145.1286 | 163 | Yes | No | n/a | No | n/a | 350.60 | Surveyed |
| 918.1 | Eulo | –28.1182 | 145.1285 | 163 | Yes | No | n/a | No | n/a | 350.59 | Surveyed |
| 918.1 | Eulo | –28.1182 | 145.1278 | 163 | Yes | No | n/a | No | n/a | 350.66 | Surveyed |
| 918.1 | Eulo | –28.1174 | 145.1287 | 162 | Yes | No | n/a | No | n/a | 350.55 | Surveyed |
| 918.2 | Eulo | –28.1174 | 145.1262 | 163 | Yes | No | n/a | No | n/a | 350.79 | Surveyed |
| 918.3 | Eulo | –28.1184 | 145.1268 | 163 | Yes | No | n/a | No | n/a | 350.76 | Surveyed |
| 918.4 | Eulo | –28.1191 | 145.1270 | 163 | Yes | No | n/a | No | n/a | 350.76 | Surveyed |
| 918.5 | Eulo | –28.1191 | 145.1264 | 163 | Yes | No | n/a | No | n/a | 350.82 | Surveyed |
| 918.6 | Eulo | –28.1193 | 145.1268 | 163 | Yes | No | n/a | No | n/a | 350.78 | Surveyed |
| 918.7 | Eulo | –28.1193 | 145.1276 | 163 | Yes | No | n/a | No | n/a | 350.71 | Surveyed |
| 918.8 | Eulo | –28.1190 | 145.1277 | 163 | Yes | No | n/a | No | n/a | 350.69 | Surveyed |
| 918.9 | Eulo | –28.1191 | 145.1279 | 163 | Yes | No | n/a | No | n/a | 350.68 | Surveyed |
| 919 | Eulo | –28.5733 | 145.0051 | 153 | Yes | No | n/a | No | n/a | 378.45 | Surveyed |
| 919.1 | Eulo | –28.5734 | 145.0048 | 152 | Yes | No | n/a | No | n/a | 378.48 | Surveyed |
| 920 | Eulo | –28.6883 | 145.2356 | 146 | Yes | No | n/a | No | n/a | 363.35 | Surveyed |
| 921 | Eulo | –28.9662 | 144.5862 | 142 | Yes | No | n/a | No | n/a | 433.29 | Surveyed |
| 922.1 | Eulo | –28.9154 | 144.7658 | 138 | Yes | No | n/a | No | n/a | 415.23 | Surveyed |
| 922.2 | Eulo | –28.9150 | 144.7660 | 138 | Yes | No | n/a | No | n/a | 415.19 | Surveyed |
| 922.3 | Eulo | –28.9152 | 144.7657 | 138 | Yes | No | n/a | No | n/a | 415.23 | Surveyed |
| 922.4 | Eulo | –28.9155 | 144.7656 | 138 | Yes | No | n/a | No | n/a | 415.25 | Surveyed |
| 923 | Eulo | –28.9222 | 144.7596 | 136 | Yes | No | n/a | No | n/a | 416.10 | Surveyed |
| 924 | Eulo | –28.5497 | 144.6572 | 147 | Yes | No | n/a | No | n/a | 408.86 | Surveyed |
| 925 | Eulo | –28.7117 | 144.1545 | 137 | Yes | No | n/a | No | n/a | 460.71 | Surveyed |
| 925.1 | Eulo | –28.7140 | 144.1573 | 136 | Yes | No | n/a | No | n/a | 460.54 | Surveyed |
| 926 | Eulo | –28.7889 | 144.2635 | 124.98 | Yes | No | n/a | No | n/a | 453.93 | Surveyed |
| 926.1 | Eulo | –28.7887 | 144.2659 | 127 | Yes | No | n/a | No | n/a | 453.70 | Surveyed |
| 926.2 | Eulo | –28.7890 | 144.2656 | 127 | Yes | No | n/a | No | n/a | 453.74 | Surveyed |
| 926.3 | Eulo | –28.7890 | 144.2658 | 127 | Yes | No | n/a | No | n/a | 453.72 | Surveyed |
| 926.4 | Eulo | –28.7877 | 144.2661 | 127 | Yes | No | n/a | No | n/a | 453.64 | Surveyed |
| 926.5 | Eulo | –28.7868 | 144.2669 | 126.329 | Yes | No | n/a | No | n/a | 453.53 | Surveyed |
| 926.6 | Eulo | –28.7870 | 144.2667 | 127 | Yes | No | n/a | No | n/a | 453.55 | Surveyed |
| 927 | Eulo | –28.6639 | 144.6378 | 142 | Yes | No | n/a | No | n/a | 415.16 | Surveyed |
| 928.1 | Eulo | –28.7048 | 144.4002 | 132 | Yes | No | n/a | No | n/a | 438.22 | Surveyed |
| 928.1 | Eulo | –28.7083 | 144.3953 | 129 | Yes | No | n/a | No | n/a | 438.79 | Surveyed |
| 928.1 | Eulo | –28.7085 | 144.3951 | 129 | Yes | No | n/a | No | n/a | 438.82 | Surveyed |
| 928.1 | Eulo | –28.7080 | 144.3937 | 129 | Yes | No | n/a | No | n/a | 438.92 | Surveyed |
| 928.1 | Eulo | –28.7018 | 144.3978 | 130 | Yes | No | n/a | No | n/a | 438.30 | Surveyed |
| 928.2 | Eulo | –28.7049 | 144.3983 | 131 | Yes | No | n/a | No | n/a | 438.39 | Surveyed |
| 928.3 | Eulo | –28.7064 | 144.3971 | 130 | Yes | No | n/a | No | n/a | 438.55 | Surveyed |
| 928.4 | Eulo | –28.7072 | 144.3973 | 130 | Yes | No | n/a | No | n/a | 438.57 | Surveyed |
| 928.5 | Eulo | –28.7075 | 144.3973 | 130 | Yes | No | n/a | No | n/a | 438.59 | Surveyed |
| 928.6 | Eulo | –28.7076 | 144.3962 | 130 | Yes | No | n/a | No | n/a | 438.68 | Surveyed |
| 928.7 | Eulo | –28.7078 | 144.3960 | 130 | Yes | No | n/a | No | n/a | 438.71 | Surveyed |
| 928.8 | Eulo | –28.7082 | 144.3960 | 129 | Yes | No | n/a | No | n/a | 438.73 | Surveyed |
| 928.9 | Eulo | –28.7089 | 144.3964 | 130 | Yes | No | n/a | No | n/a | 438.72 | Surveyed |
| 929 | Eulo | –28.8400 | 144.4923 | 134 | Yes | No | n/a | No | n/a | 435.69 | Surveyed |
| 930 | Eulo | –29.1391 | 144.6640 | 123 | Yes | No | n/a | No | n/a | 435.39 | Surveyed |
| 931.1 | Eulo | –29.2304 | 144.6982 | 120.4 | Yes | No | n/a | No | n/a | 437.47 | Surveyed |
| 931.1 | Eulo | –29.2282 | 144.6993 | 123 | Yes | No | n/a | No | n/a | 437.26 | Surveyed |
| 931.1 | Eulo | –29.2285 | 144.6994 | 123 | Yes | No | n/a | No | n/a | 437.26 | Surveyed |
| 931.1 | Eulo | –29.2289 | 144.7000 | 123 | Yes | No | n/a | No | n/a | 437.23 | Surveyed |
| 931.1 | Eulo | –29.2289 | 144.6996 | 123 | Yes | No | n/a | No | n/a | 437.27 | Surveyed |
| 931.1 | Eulo | –29.2283 | 144.7004 | 123 | Yes | No | n/a | No | n/a | 437.16 | Surveyed |
| 931.2 | Eulo | –29.2282 | 144.7000 | 123 | Yes | No | n/a | No | n/a | 437.19 | Surveyed |
| 931.2 | Eulo | –29.2303 | 144.6983 | 123 | Yes | No | n/a | No | n/a | 437.45 | Surveyed |
| 931.3 | Eulo | –29.2306 | 144.6980 | 123 | Yes | No | n/a | No | n/a | 437.49 | Surveyed |
| 931.4 | Eulo | –29.2299 | 144.6977 | 123 | Yes | No | n/a | No | n/a | 437.48 | Surveyed |
| 931.5 | Eulo | –29.2270 | 144.6980 | 123 | Yes | No | n/a | No | n/a | 437.29 | Surveyed |
| 931.6 | Eulo | –29.2264 | 144.6966 | 123 | Yes | No | n/a | No | n/a | 437.38 | Surveyed |
| 931.7 | Eulo | –29.2263 | 144.6981 | 118.664 | Yes | No | n/a | No | n/a | 437.25 | Surveyed |
| 931.8 | Eulo | –29.2277 | 144.6990 | 123 | Yes | No | n/a | No | n/a | 437.25 | Surveyed |
| 931.9 | Eulo | –29.2279 | 144.6993 | 123 | Yes | No | n/a | No | n/a | 437.23 | Surveyed |
| 932.1 | Eulo | –29.2142 | 144.7051 | 119.136 | Yes | No | n/a | No | n/a | 435.99 | Surveyed |
| 932.2 | Eulo | –29.2144 | 144.7054 | 119.136 | Yes | No | n/a | No | n/a | 435.98 | Surveyed |
| 933 | Eulo | –29.0792 | 144.6412 | 121 | Yes | No | n/a | No | n/a | 434.19 | Surveyed |
| 934 | Eulo | –28.8714 | 144.4110 | 127.013 | Yes | No | n/a | No | n/a | 444.28 | Surveyed |
| 935.1 | Eulo | –28.8307 | 144.3499 | 126.03 | Yes | No | n/a | No | n/a | 447.93 | Surveyed |
| 935.2 | Eulo | –28.8307 | 144.3499 | 128 | Yes | No | n/a | No | n/a | 447.93 | Surveyed |
| 935.3 | Eulo | –28.8304 | 144.3500 | 128 | Yes | No | n/a | No | n/a | 447.91 | Surveyed |
| 936 | Eulo | –28.8670 | 144.1846 | 155.673 | Yes | No | n/a | No | n/a | 464.23 | Surveyed |
| 936.1 | Eulo | –28.8294 | 144.1956 | 138 | Yes | No | n/a | No | n/a | 461.68 | Surveyed |
| 937.1 | Eulo | –28.7806 | 144.4623 | 139 | Yes | No | n/a | No | n/a | 435.77 | Surveyed |
| 937.1 | Eulo | –28.7859 | 144.4517 | 133 | Yes | No | n/a | No | n/a | 436.94 | Surveyed |
| 937.1 | Eulo | –28.7873 | 144.4380 | 132 | Yes | No | n/a | No | n/a | 438.22 | Surveyed |
| 937.1 | Eulo | –28.7891 | 144.4361 | 131 | Yes | No | n/a | No | n/a | 438.47 | Surveyed |
| 937.1 | Eulo | –28.7857 | 144.4347 | 130 | Yes | No | n/a | No | n/a | 438.45 | Surveyed |
| 937.1 | Eulo | –28.7857 | 144.4356 | 131 | Yes | No | n/a | No | n/a | 438.37 | Surveyed |
| 937.2 | Eulo | –28.7838 | 144.4361 | 131 | Yes | No | n/a | No | n/a | 438.24 | Surveyed |
| 937.2 | Eulo | –28.7817 | 144.4403 | 132 | Yes | No | n/a | No | n/a | 437.79 | Surveyed |
| 937.2 | Eulo | –28.7799 | 144.4629 | 140 | Yes | No | n/a | No | n/a | 435.69 | Surveyed |
| 937.3 | Eulo | –28.7792 | 144.4623 | 140 | Yes | No | n/a | No | n/a | 435.72 | Surveyed |
| 937.4 | Eulo | –28.7794 | 144.4601 | 141 | Yes | No | n/a | No | n/a | 435.92 | Surveyed |
| 937.5 | Eulo | –28.7801 | 144.4594 | 139 | Yes | No | n/a | No | n/a | 436.01 | Surveyed |
| 937.6 | Eulo | –28.7817 | 144.4604 | 137 | Yes | No | n/a | No | n/a | 435.98 | Surveyed |
| 937.7 | Eulo | –28.7837 | 144.4595 | 136 | Yes | No | n/a | No | n/a | 436.16 | Surveyed |
| 937.8 | Eulo | –28.7840 | 144.4577 | 136 | Yes | No | n/a | No | n/a | 436.33 | Surveyed |
| 937.9 | Eulo | –28.7866 | 144.4561 | 135 | Yes | No | n/a | No | n/a | 436.58 | Surveyed |
| 938 | Eulo | –28.7473 | 144.4311 | 134 | Yes | No | n/a | No | n/a | 437.16 | Surveyed |
| 939 | Eulo | –28.7094 | 144.4361 | 142 | Yes | No | n/a | No | n/a | 435.16 | Surveyed |
| 939.1 | Eulo | –28.7096 | 144.4179 | 134 | Yes | No | n/a | No | n/a | 436.81 | Surveyed |
| 940.1 | Eulo | –28.7875 | 144.4022 | 125 | Yes | No | n/a | No | n/a | 441.43 | Surveyed |
| 940.1 | Eulo | –28.7892 | 144.4162 | 128 | Yes | No | n/a | No | n/a | 440.26 | Surveyed |
| 940.1 | Eulo | –28.7893 | 144.4167 | 127 | Yes | No | n/a | No | n/a | 440.21 | Surveyed |
| 940.1 | Eulo | –28.7901 | 144.4161 | 127 | Yes | No | n/a | No | n/a | 440.31 | Surveyed |
| 940.1 | Eulo | –28.7902 | 144.4185 | 126.895 | Yes | No | n/a | No | n/a | 440.09 | Surveyed |
| 940.1 | Eulo | –28.7902 | 144.4193 | 127 | Yes | No | n/a | No | n/a | 440.02 | Surveyed |
| 940.2 | Eulo | –28.7888 | 144.4173 | 128 | Yes | No | n/a | No | n/a | 440.14 | Surveyed |
| 940.2 | Eulo | –28.7845 | 144.4230 | 129 | Yes | No | n/a | No | n/a | 439.45 | Surveyed |
| 940.2 | Eulo | –28.7879 | 144.4040 | 126 | Yes | No | n/a | No | n/a | 441.29 | Surveyed |
| 940.3 | Eulo | –28.7869 | 144.4051 | 126.333 | Yes | No | n/a | No | n/a | 441.15 | Surveyed |
| 940.4 | Eulo | –28.7870 | 144.4087 | 125 | Yes | No | n/a | No | n/a | 440.83 | Surveyed |
| 940.5 | Eulo | –28.7868 | 144.4090 | 125 | Yes | No | n/a | No | n/a | 440.80 | Surveyed |
| 940.6 | Eulo | –28.7865 | 144.4089 | 125 | Yes | No | n/a | No | n/a | 440.79 | Surveyed |
| 940.7 | Eulo | –28.7869 | 144.4099 | 126 | Yes | No | n/a | No | n/a | 440.72 | Surveyed |
| 940.8 | Eulo | –28.7865 | 144.4097 | 126 | Yes | No | n/a | No | n/a | 440.72 | Surveyed |
| 940.9 | Eulo | –28.7893 | 144.4157 | 127 | Yes | No | n/a | No | n/a | 440.31 | Surveyed |
| 941 | Eulo | –28.7147 | 144.5000 | 134.487 | Yes | No | n/a | No | n/a | 429.63 | Surveyed |
| 941.1 | Eulo | –28.7152 | 144.5003 | 137 | Yes | No | n/a | No | n/a | 429.63 | Surveyed |
| 942 | Eulo | –28.7215 | 144.4874 | 140 | Yes | No | n/a | No | n/a | 431.05 | Surveyed |
| 943 | Eulo | –28.5571 | 144.4413 | 142.062 | Yes | No | n/a | No | n/a | 428.81 | Surveyed |
| 944.1 | Eulo | –28.3668 | 144.1398 | 150.887 | Yes | No | n/a | No | n/a | 450.41 | Surveyed |
| 944.2 | Eulo | –28.3667 | 144.1395 | 154 | Yes | No | n/a | No | n/a | 450.43 | Surveyed |
| 944.3 | Eulo | –28.3663 | 144.1398 | 149.805 | Yes | No | n/a | No | n/a | 450.39 | Surveyed |
| 944.4 | Eulo | –28.3639 | 144.1422 | 150.394 | Yes | No | n/a | No | n/a | 450.10 | Surveyed |
| 944.5 | Eulo | –28.3709 | 144.1488 | 158 | Yes | No | n/a | No | n/a | 449.69 | Surveyed |
| 944.6 | Eulo | –28.3552 | 144.1596 | 149.473 | Yes | No | n/a | No | n/a | 448.22 | Surveyed |
| 944.7 | Eulo | –28.3567 | 144.1748 | 152 | Yes | No | n/a | No | n/a | 446.84 | Surveyed |
| 945.1 | Eulo | –28.2824 | 144.3238 | 151 | Yes | No | n/a | No | n/a | 430.74 | Surveyed |
| 945.1 | Eulo | –28.2809 | 144.3260 | 150 | Yes | No | n/a | No | n/a | 430.49 | Surveyed |
| 945.1 | Eulo | –28.2812 | 144.3254 | 150 | Yes | No | n/a | No | n/a | 430.56 | Surveyed |
| 945.1 | Eulo | –28.2813 | 144.3252 | 150 | Yes | No | n/a | No | n/a | 430.58 | Surveyed |
| 945.2 | Eulo | –28.2818 | 144.3260 | 150 | Yes | No | n/a | No | n/a | 430.53 | Surveyed |
| 945.3 | Eulo | –28.2814 | 144.3278 | 150 | Yes | No | n/a | No | n/a | 430.34 | Surveyed |
| 945.4 | Eulo | –28.2811 | 144.3277 | 150 | Yes | No | n/a | No | n/a | 430.34 | Surveyed |
| 945.5 | Eulo | –28.2808 | 144.3276 | 150 | Yes | No | n/a | No | n/a | 430.34 | Surveyed |
| 945.6 | Eulo | –28.2805 | 144.3268 | 150 | Yes | No | n/a | No | n/a | 430.41 | Surveyed |
| 945.7 | Eulo | –28.2798 | 144.3268 | 150 | Yes | No | n/a | No | n/a | 430.40 | Surveyed |
| 945.8 | Eulo | –28.2805 | 144.3261 | 150 | Yes | No | n/a | No | n/a | 430.47 | Surveyed |
| 945.9 | Eulo | –28.2809 | 144.3264 | 150 | Yes | No | n/a | No | n/a | 430.46 | Surveyed |
| 946 | Eulo | –28.2989 | 144.3016 | 139.616 | Yes | No | n/a | No | n/a | 433.30 | Surveyed |
| 947 | Eulo | –28.3370 | 144.3002 | 137.545 | Yes | No | n/a | No | n/a | 434.54 | Surveyed |
| 948.1 | Eulo | –28.3595 | 144.3246 | 145 | Yes | No | n/a | No | n/a | 432.94 | Surveyed |
| 948.1 | Eulo | –28.3582 | 144.3243 | 146 | Yes | No | n/a | No | n/a | 432.93 | Surveyed |
| 948.1 | Eulo | –28.3579 | 144.3238 | 145 | Yes | No | n/a | No | n/a | 432.96 | Surveyed |
| 948.1 | Eulo | –28.3582 | 144.3232 | 145 | Yes | No | n/a | No | n/a | 433.03 | Surveyed |
| 948.1 | Eulo | –28.3583 | 144.3234 | 145 | Yes | No | n/a | No | n/a | 433.01 | Surveyed |
| 948.1 | Eulo | –28.3597 | 144.3212 | 144 | Yes | No | n/a | No | n/a | 433.26 | Surveyed |
| 948.2 | Eulo | –28.3589 | 144.3271 | 146 | Yes | No | n/a | No | n/a | 432.69 | Surveyed |
| 948.3 | Eulo | –28.3590 | 144.3295 | 146 | Yes | No | n/a | No | n/a | 432.47 | Surveyed |
| 948.4 | Eulo | –28.3588 | 144.3299 | 146 | Yes | No | n/a | No | n/a | 432.43 | Surveyed |
| 948.5 | Eulo | –28.3590 | 144.3305 | 147 | Yes | No | n/a | No | n/a | 432.38 | Surveyed |
| 948.6 | Eulo | –28.3591 | 144.3230 | 145 | Yes | No | n/a | No | n/a | 433.08 | Surveyed |
| 948.7 | Eulo | –28.3589 | 144.3233 | 145 | Yes | No | n/a | No | n/a | 433.05 | Surveyed |
| 948.8 | Eulo | –28.3585 | 144.3237 | 145 | Yes | No | n/a | No | n/a | 432.99 | Surveyed |
| 948.9 | Eulo | –28.3584 | 144.3241 | 141.127 | Yes | No | n/a | No | n/a | 432.95 | Surveyed |
| 949.1 | Eulo | –28.4670 | 144.3592 | 138 | Yes | No | n/a | No | n/a | 433.18 | Surveyed |
| 949.1 | Eulo | –28.4690 | 144.3624 | 136 | Yes | No | n/a | No | n/a | 432.95 | Surveyed |
| 949.1 | Eulo | –28.4671 | 144.3612 | 136 | Yes | No | n/a | No | n/a | 433.00 | Surveyed |
| 949.1 | Eulo | –28.4711 | 144.3640 | 137 | Yes | No | n/a | No | n/a | 432.88 | Surveyed |
| 949.2 | Eulo | –28.4672 | 144.3592 | 138 | Yes | No | n/a | No | n/a | 433.19 | Surveyed |
| 949.3 | Eulo | –28.4673 | 144.3596 | 138 | Yes | No | n/a | No | n/a | 433.16 | Surveyed |
| 949.4 | Eulo | –28.4675 | 144.3600 | 137 | Yes | No | n/a | No | n/a | 433.12 | Surveyed |
| 949.5 | Eulo | –28.4674 | 144.3609 | 137 | Yes | No | n/a | No | n/a | 433.04 | Surveyed |
| 949.6 | Eulo | –28.4677 | 144.3613 | 136 | Yes | No | n/a | No | n/a | 433.01 | Surveyed |
| 949.7 | Eulo | –28.4680 | 144.3614 | 137 | Yes | No | n/a | No | n/a | 433.01 | Surveyed |
| 949.8 | Eulo | –28.4680 | 144.3616 | 136 | Yes | No | n/a | No | n/a | 432.99 | Surveyed |
| 949.9 | Eulo | –28.4679 | 144.3619 | 136 | Yes | No | n/a | No | n/a | 432.96 | Surveyed |
| 950.1 | Eulo | –28.4772 | 144.3736 | 136 | Yes | No | n/a | No | n/a | 432.19 | Surveyed |
| 950.1 | Eulo | –28.4738 | 144.3731 | 136 | Yes | No | n/a | No | n/a | 432.13 | Surveyed |
| 950.1 | Eulo | –28.4734 | 144.3733 | 136 | Yes | No | n/a | No | n/a | 432.10 | Surveyed |
| 950.1 | Eulo | –28.4735 | 144.3735 | 136 | Yes | No | n/a | No | n/a | 432.08 | Surveyed |
| 950.1 | Eulo | –28.4738 | 144.3734 | 136 | Yes | No | n/a | No | n/a | 432.10 | Surveyed |
| 950.1 | Eulo | –28.4742 | 144.3743 | 136 | Yes | No | n/a | No | n/a | 432.03 | Surveyed |
| 950.2 | Eulo | –28.4734 | 144.3744 | 136 | Yes | No | n/a | No | n/a | 431.99 | Surveyed |
| 950.2 | Eulo | –28.4726 | 144.3727 | 135 | Yes | No | n/a | No | n/a | 432.12 | Surveyed |
| 950.2 | Eulo | –28.4723 | 144.3730 | 135 | Yes | No | n/a | No | n/a | 432.09 | Surveyed |
| 950.2 | Eulo | –28.4719 | 144.3730 | 136 | Yes | No | n/a | No | n/a | 432.07 | Surveyed |
| 950.2 | Eulo | –28.4714 | 144.3726 | 136 | Yes | No | n/a | No | n/a | 432.09 | Surveyed |
| 950.2 | Eulo | –28.4764 | 144.3734 | 136 | Yes | No | n/a | No | n/a | 432.18 | Surveyed |
| 950.2 | Eulo | –28.4712 | 144.3731 | 136 | Yes | No | n/a | No | n/a | 432.04 | Surveyed |
| 950.2 | Eulo | –28.4711 | 144.3736 | 136 | Yes | No | n/a | No | n/a | 431.99 | Surveyed |
| 950.2 | Eulo | –28.4707 | 144.3738 | 136 | Yes | No | n/a | No | n/a | 431.96 | Surveyed |
| 950.2 | Eulo | –28.4705 | 144.3745 | 136 | Yes | No | n/a | No | n/a | 431.88 | Surveyed |
| 950.2 | Eulo | –28.4684 | 144.3759 | 136 | Yes | No | n/a | No | n/a | 431.68 | Surveyed |
| 950.3 | Eulo | –28.4755 | 144.3736 | 136 | Yes | No | n/a | No | n/a | 432.14 | Surveyed |
| 950.4 | Eulo | –28.4758 | 144.3732 | 136 | Yes | No | n/a | No | n/a | 432.18 | Surveyed |
| 950.5 | Eulo | –28.4764 | 144.3730 | 135 | Yes | No | n/a | No | n/a | 432.22 | Surveyed |
| 950.6 | Eulo | –28.4766 | 144.3732 | 132.555 | Yes | No | n/a | No | n/a | 432.21 | Surveyed |
| 950.7 | Eulo | –28.4760 | 144.3734 | 136 | Yes | No | n/a | No | n/a | 432.17 | Surveyed |
| 950.8 | Eulo | –28.4745 | 144.3731 | 136 | Yes | No | n/a | No | n/a | 432.15 | Surveyed |
| 950.9 | Eulo | –28.4739 | 144.3727 | 135 | Yes | No | n/a | No | n/a | 432.17 | Surveyed |
| 951 | Eulo | –28.4910 | 144.3411 | 130.17 | Yes | No | n/a | No | n/a | 435.66 | Surveyed |
| 952 | Eulo | –28.4146 | 144.2744 | 132.782 | Yes | No | n/a | No | n/a | 439.32 | Surveyed |
| 953.1 | Eulo | –27.9994 | 144.7868 | 172 | Yes | No | n/a | No | n/a | 379.70 | Surveyed |
| 953.2 | Eulo | –27.9993 | 144.7867 | 172 | Yes | No | n/a | No | n/a | 379.71 | Surveyed |
| 953.3 | Eulo | –27.9992 | 144.7876 | 173 | Yes | No | n/a | No | n/a | 379.62 | Surveyed |
| 953.4 | Eulo | –27.9993 | 144.7876 | 173 | Yes | No | n/a | No | n/a | 379.61 | Surveyed |
| 954.1 | Eulo | –28.1295 | 144.7861 | 171 | Yes | No | n/a | No | n/a | 383.06 | Surveyed |
| 954.2 | Eulo | –28.1295 | 144.7855 | 171 | Yes | No | n/a | No | n/a | 383.11 | Surveyed |
| 954.3 | Eulo | –28.1296 | 144.7852 | 166.519 | Yes | No | n/a | No | n/a | 383.15 | Surveyed |
| 955.1 | Eulo | –28.1509 | 144.9569 | 160 | Yes | No | n/a | No | n/a | 367.62 | Surveyed |
| 955.2 | Eulo | –28.1511 | 144.9565 | 160 | Yes | No | n/a | No | n/a | 367.67 | Surveyed |
| 955.3 | Eulo | –28.1533 | 144.9576 | 160 | Yes | No | n/a | No | n/a | 367.63 | Surveyed |
| 955.4 | Eulo | –28.1532 | 144.9570 | 160 | Yes | No | n/a | No | n/a | 367.68 | Surveyed |
| 955.5 | Eulo | –28.1518 | 144.9611 | 157.222 | Yes | No | n/a | No | n/a | 367.25 | Surveyed |
| 956 | Eulo | –28.1457 | 145.0434 | 156.979 | Yes | No | n/a | No | n/a | 359.38 | Surveyed |
| 957 | Eulo | –27.8185 | 145.6715 | 198.513 | Yes | No | n/a | No | n/a | 291.32 | Surveyed |
| 958 | Eulo | –27.8194 | 145.6474 | 195.694 | Yes | No | n/a | No | n/a | 293.64 | Surveyed |
| 959 | Eulo | –28.4748 | 144.4943 | 163 | Yes | No | n/a | No | n/a | 421.00 | Surveyed |
| 960.1 | Bourke | –29.5294 | 145.8310 | – | Yes | No | n/a | No | n/a | 355.56 | Surveyed |
| 960.1 | Bourke | –29.5244 | 145.8339 | – | Yes | No | n/a | No | n/a | 355.04 | Surveyed |
| 960.1 | Bourke | –29.5249 | 145.8338 | – | Yes | No | n/a | No | n/a | 355.07 | Surveyed |
| 960.1 | Bourke | –29.5277 | 145.8301 | – | Yes | No | n/a | No | n/a | 355.53 | Surveyed |
| 960.1 | Bourke | –29.5279 | 145.8298 | – | Yes | No | n/a | No | n/a | 355.57 | Surveyed |
| 960.1 | Bourke | –29.5280 | 145.8294 | – | Yes | No | n/a | No | n/a | 355.61 | Surveyed |
| 960.2 | Bourke | –29.5277 | 145.8284 | – | Yes | No | n/a | No | n/a | 355.68 | Surveyed |
| 960.2 | Bourke | –29.5278 | 145.8282 | – | Yes | No | n/a | No | n/a | 355.69 | Surveyed |
| 960.2 | Bourke | –29.5287 | 145.8313 | – | Yes | No | n/a | No | n/a | 355.50 | Surveyed |
| 960.3 | Bourke | –29.5269 | 145.8325 | – | Yes | No | n/a | No | n/a | 355.29 | Surveyed |
| 960.4 | Bourke | –29.5264 | 145.8329 | – | Yes | No | n/a | No | n/a | 355.23 | Surveyed |
| 960.5 | Bourke | –29.5261 | 145.8331 | – | Yes | No | n/a | No | n/a | 355.20 | Surveyed |
| 960.6 | Bourke | –29.5256 | 145.8325 | – | Yes | No | n/a | No | n/a | 355.22 | Surveyed |
| 960.7 | Bourke | –29.5254 | 145.8329 | – | Yes | No | n/a | No | n/a | 355.17 | Surveyed |
| 960.8 | Bourke | –29.5254 | 145.8332 | – | Yes | No | n/a | No | n/a | 355.15 | Surveyed |
| 960.9 | Bourke | –29.5249 | 145.8336 | – | Yes | No | n/a | No | n/a | 355.09 | Surveyed |
| 961 | Bourke | –29.7160 | 146.2843 | – | Yes | No | n/a | No | n/a | 330.84 | Surveyed |
| 961.1 | Bourke | –29.7162 | 146.2849 | 110.829 | Yes | No | n/a | No | n/a | 330.81 | Surveyed |
| 962 | Bourke | –29.4570 | 145.1003 | 130 | Yes | No | n/a | No | n/a | 414.23 | Surveyed |
| 963 | Bourke | –29.4540 | 145.1013 | 122.883 | Yes | No | n/a | No | n/a | 414.00 | Surveyed |
| 964 | Bourke | –29.2410 | 146.4054 | 136.628 | No | No | n/a | No | n/a | 291.66 | Surveyed |
| 965 | Bourke | –29.2677 | 146.3830 | 131.292 | No | No | n/a | No | n/a | 295.02 | Surveyed |
| 966 | Bourke | –29.1922 | 146.5875 | 128.304 | No | No | n/a | No | n/a | 273.80 | Surveyed |
| 967 | Bourke | –29.1040 | 146.6200 | 149 | No | No | n/a | No | n/a | 266.12 | Surveyed |
| 967.1 | Bourke | –29.0957 | 146.6540 | 144 | No | No | n/a | No | n/a | 262.81 | Surveyed |
| 968 | Bourke | –29.4181 | 146.1150 | 154.611 | No | No | n/a | No | n/a | 325.74 | Surveyed |
| 969 | Bourke | –29.4188 | 146.2112 | 140.364 | No | No | n/a | No | n/a | 317.88 | Surveyed |
| 970 | Bourke | –29.5096 | 146.1470 | 156.976 | No | No | n/a | No | n/a | 328.48 | Surveyed |
| 971 | Bourke | –30.0238 | 145.1099 | 101.816 | Yes | No | n/a | No | n/a | 443.13 | Surveyed |
| 972 | Bourke | –29.8258 | 145.5143 | 120.874 | Yes | No | n/a | No | n/a | 398.62 | Surveyed |
| 973 | Bourke | –29.5246 | 145.1296 | 122.994 | Yes | No | n/a | No | n/a | 415.00 | Surveyed |
| 973.1 | Bourke | –29.5395 | 145.1149 | – | Yes | No | n/a | No | n/a | 417.01 | Surveyed |
| 974.1 | Bourke | –29.5351 | 145.2476 | 123.972 | Yes | No | n/a | No | n/a | 405.31 | Surveyed |
| 974.1 | Bourke | –29.5328 | 145.2568 | 123.972 | Yes | No | n/a | No | n/a | 404.40 | Surveyed |
| 974.1 | Bourke | –29.5326 | 145.2566 | – | Yes | No | n/a | No | n/a | 404.41 | Surveyed |
| 974.1 | Bourke | –29.5319 | 145.2553 | – | Yes | No | n/a | No | n/a | 404.49 | Surveyed |
| 974.1 | Bourke | –29.5320 | 145.2576 | – | Yes | No | n/a | No | n/a | 404.30 | Surveyed |
| 974.1 | Bourke | –29.5318 | 145.2580 | 123.102 | Yes | No | n/a | No | n/a | 404.25 | Surveyed |
| 974.2 | Bourke | –29.5312 | 145.2592 | – | Yes | No | n/a | No | n/a | 404.12 | Surveyed |
| 974.2 | Bourke | –29.5323 | 145.2590 | – | Yes | No | n/a | No | n/a | 404.19 | Surveyed |
| 974.2 | Bourke | –29.5323 | 145.2597 | – | Yes | No | n/a | No | n/a | 404.13 | Surveyed |
| 974.2 | Bourke | –29.5318 | 145.2606 | – | Yes | No | n/a | No | n/a | 404.03 | Surveyed |
| 974.2 | Bourke | –29.5303 | 145.2617 | – | Yes | No | n/a | No | n/a | 403.86 | Surveyed |
| 974.2 | Bourke | –29.5365 | 145.2491 | – | Yes | No | n/a | No | n/a | 403.73 | Surveyed |
| 974.2 | Bourke | –29.5289 | 145.2624 | – | Yes | No | n/a | No | n/a | 405.25 | Surveyed |
| 974.3 | Bourke | –29.5345 | 145.2493 | – | Yes | No | n/a | No | n/a | 405.13 | Surveyed |
| 974.4 | Bourke | –29.5325 | 145.2551 | – | Yes | No | n/a | No | n/a | 404.54 | Surveyed |
| 974.5 | Bourke | –29.5324 | 145.2555 | – | Yes | No | n/a | No | n/a | 404.50 | Surveyed |
| 974.6 | Bourke | –29.5324 | 145.2557 | – | Yes | No | n/a | No | n/a | 404.48 | Surveyed |
| 974.7 | Bourke | –29.5327 | 145.2559 | – | Yes | No | n/a | No | n/a | 404.47 | Surveyed |
| 974.8 | Bourke | –29.5334 | 145.2565 | – | Yes | No | n/a | No | n/a | 404.46 | Surveyed |
| 974.9 | Bourke | –29.5330 | 145.2570 | – | Yes | No | n/a | No | n/a | 404.40 | Surveyed |
| 975 | Bourke | –29.5147 | 145.2780 | 121.843 | Yes | No | n/a | No | n/a | 401.69 | Surveyed |
| 976.1 | Bourke | –29.3917 | 145.3041 | 138 | Yes | No | n/a | No | n/a | 393.43 | Surveyed |
| 976.1 | Bourke | –29.3976 | 145.3064 | 137 | Yes | No | n/a | No | n/a | 393.52 | Surveyed |
| 976.1 | Bourke | –29.3974 | 145.3067 | 137 | Yes | No | n/a | No | n/a | 393.48 | Surveyed |
| 976.1 | Bourke | –29.3976 | 145.3070 | 137 | Yes | No | n/a | No | n/a | 393.46 | Surveyed |
| 976.1 | Bourke | –29.3974 | 145.3077 | 138 | Yes | No | n/a | No | n/a | 393.40 | Surveyed |
| 976.1 | Bourke | –29.3970 | 145.3079 | 132.64 | Yes | No | n/a | No | n/a | 393.36 | Surveyed |
| 976.2 | Bourke | –29.3968 | 145.3073 | 137 | Yes | No | n/a | No | n/a | 393.40 | Surveyed |
| 976.2 | Bourke | –29.3966 | 145.3070 | 137 | Yes | No | n/a | No | n/a | 393.42 | Surveyed |
| 976.2 | Bourke | –29.3953 | 145.3067 | 137 | Yes | No | n/a | No | n/a | 393.38 | Surveyed |
| 976.2 | Bourke | –29.3933 | 145.3057 | 137 | Yes | No | n/a | No | n/a | 393.37 | Surveyed |
| 976.2 | Bourke | –29.3937 | 145.3063 | 137 | Yes | No | n/a | No | n/a | 393.34 | Surveyed |
| 976.2 | Bourke | –29.3918 | 145.3042 | 131.148 | Yes | No | n/a | No | n/a | 393.41 | Surveyed |
| 976.2 | Bourke | –29.3921 | 145.3046 | 138 | Yes | No | n/a | No | n/a | 393.43 | Surveyed |
| 976.3 | Bourke | –29.3918 | 145.3044 | 138 | Yes | No | n/a | No | n/a | 393.42 | Surveyed |
| 976.4 | Bourke | –29.3922 | 145.3040 | 137 | Yes | No | n/a | No | n/a | 393.46 | Surveyed |
| 976.5 | Bourke | –29.3924 | 145.3044 | 137 | Yes | No | n/a | No | n/a | 393.44 | Surveyed |
| 976.6 | Bourke | –29.3926 | 145.3048 | 137 | Yes | No | n/a | No | n/a | 393.42 | Surveyed |
| 976.7 | Bourke | –29.3937 | 145.3048 | 129.609 | Yes | No | n/a | No | n/a | 393.47 | Surveyed |
| 976.8 | Bourke | –29.3948 | 145.3038 | 136 | Yes | No | n/a | No | n/a | 393.60 | Surveyed |
| 976.9 | Bourke | –29.3968 | 145.3029 | 136 | Yes | No | n/a | No | n/a | 393.78 | Surveyed |
| 978 | Bourke | –29.3972 | 145.3215 | 130.263 | Yes | No | n/a | No | n/a | 392.19 | Surveyed |
| 978.1 | Bourke | –29.3977 | 145.3230 | 130.05 | Yes | No | n/a | No | n/a | 392.09 | Surveyed |
| 978.2 | Bourke | –29.3963 | 145.3207 | 133.784 | Yes | No | n/a | No | n/a | 392.22 | Surveyed |
| 979 | Bourke | –29.3801 | 145.3405 | 125.384 | Yes | No | n/a | No | n/a | 389.72 | Surveyed |
| 979.1 | Bourke | –29.3748 | 145.3371 | 133 | Yes | No | n/a | No | n/a | 389.76 | Surveyed |
| 980 | Bourke | –29.3695 | 145.3755 | 129.758 | Yes | No | n/a | No | n/a | 386.18 | Surveyed |
| 981 | Bourke | –29.2731 | 145.3510 | 129.244 | Yes | No | n/a | No | n/a | 383.76 | Surveyed |
| 982.1 | Bourke | –29.1368 | 145.2519 | 128.356 | Yes | No | n/a | No | n/a | 386.44 | Surveyed |
| 982.2 | Bourke | –29.1370 | 145.2517 | 133 | Yes | No | n/a | No | n/a | 386.48 | Surveyed |
| 982.3 | Bourke | –29.1372 | 145.2516 | 133 | Yes | No | n/a | No | n/a | 386.49 | Surveyed |
| 982.4 | Bourke | –29.1374 | 145.2516 | 127.467 | Yes | No | n/a | No | n/a | 386.52 | Surveyed |
| 982.5 | Bourke | –29.1386 | 145.2514 | 132 | Yes | No | n/a | No | n/a | 386.60 | Surveyed |
| 983.1 | Bourke | –29.1511 | 145.2326 | 130 | Yes | No | n/a | No | n/a | 388.88 | Surveyed |
| 983.1 | Bourke | –29.1508 | 145.2280 | 130 | Yes | No | n/a | No | n/a | 389.24 | Surveyed |
| 983.1 | Bourke | –29.1526 | 145.2338 | 130 | Yes | No | n/a | No | n/a | 388.87 | Surveyed |
| 983.2 | Bourke | –29.1512 | 145.2276 | 130 | Yes | No | n/a | No | n/a | 389.29 | Surveyed |
| 983.3 | Bourke | –29.1512 | 145.2272 | 130 | Yes | No | n/a | No | n/a | 389.33 | Surveyed |
| 983.4 | Bourke | –29.1516 | 145.2262 | 130 | Yes | No | n/a | No | n/a | 389.43 | Surveyed |
| 983.5 | Bourke | –29.1527 | 145.2255 | 130 | Yes | No | n/a | No | n/a | 389.55 | Surveyed |
| 983.6 | Bourke | –29.1530 | 145.2265 | 130 | Yes | No | n/a | No | n/a | 389.49 | Surveyed |
| 983.7 | Bourke | –29.1525 | 145.2273 | 130 | Yes | No | n/a | No | n/a | 389.40 | Surveyed |
| 983.8 | Bourke | –29.1518 | 145.2284 | 130 | Yes | No | n/a | No | n/a | 389.26 | Surveyed |
| 983.9 | Bourke | –29.1523 | 145.2280 | 130 | Yes | No | n/a | No | n/a | 389.32 | Surveyed |
| 989.1 | Bourke | –29.4235 | 145.0590 | 124.083 | Yes | No | n/a | No | n/a | 416.25 | Surveyed |
| 989.2 | Bourke | –29.4236 | 145.0593 | 131 | Yes | No | n/a | No | n/a | 416.22 | Surveyed |
| 989.3 | Bourke | –29.4237 | 145.0591 | 131 | Yes | No | n/a | No | n/a | 416.24 | Surveyed |
| 989.4 | Bourke | –29.4237 | 145.0592 | 131 | Yes | No | n/a | No | n/a | 416.24 | Surveyed |
| 989.5 | Bourke | –29.4238 | 145.0593 | 131 | Yes | No | n/a | No | n/a | 416.23 | Surveyed |
| 989.6 | Bourke | –29.4238 | 145.0594 | 131 | Yes | No | n/a | No | n/a | 416.22 | Surveyed |
| 989.7 | Bourke | –29.4235 | 145.0594 | 131 | Yes | No | n/a | No | n/a | 416.21 | Surveyed |
| 989.8 | Bourke | –29.4235 | 145.0596 | 131 | Yes | No | n/a | No | n/a | 416.19 | Surveyed |
| 990 | Bourke | –29.3398 | 145.0033 | 120.494 | Yes | No | n/a | No | n/a | 417.35 | Surveyed |
| 991 | Bogan River | –29.5289 | 145.2624 | 174.989 | No | No | n/a | No | n/a | 403.73 | Surveyed |
| 992 | Bogan River | –29.7411 | 147.7644 | 153.608 | No | No | n/a | No | n/a | 237.01 | Surveyed |
| 992.1 | Bogan River | –29.7411 | 147.7647 | – | No | No | n/a | No | n/a | 236.99 | Surveyed |
| 992.2 | Bogan River | –29.7413 | 147.7644 | – | No | No | n/a | No | n/a | 237.02 | Surveyed |
| 992.3 | Bogan River | –29.7412 | 147.7646 | – | No | No | n/a | No | n/a | 237.00 | Surveyed |
| 993 | Bogan River | –30.3486 | 147.3434 | 123.225 | Yes | No | n/a | No | n/a | 316.06 | Surveyed |
| 994 | Bogan River | –30.8338 | 146.9499 | 141.107 | Yes | No | n/a | No | n/a | 381.91 | Surveyed |
| 995 | Bourke | –30.6615 | 143.8270 | – | Yes | No | n/a | No | n/a | 584.53 | Surveyed |
| 996 | Bourke | –29.0193 | 146.9304 | 141.762 | Yes | No | n/a | No | n/a | 235.54 | Surveyed |
| 997 | Bourke | –29.1377 | 146.5993 | 136 | No | No | n/a | No | n/a | 269.72 | Surveyed |
| 998 | Bourke | –29.2014 | 146.5510 | 133 | No | No | n/a | No | n/a | 277.34 | Surveyed |
| 1000 | Bourke | –30.7201 | 143.5694 | – | Yes | No | n/a | No | n/a | 608.93 | Surveyed |
| 1000 | Bourke | –30.7208 | 143.5700 | – | Yes | No | n/a | No | n/a | 608.92 | Surveyed |
| 1000 | Bourke | –30.7286 | 143.5733 | – | Yes | Yes | Eriocaulon carsonii | Yes | Eriocaulon carsonii | 609.08 | Surveyed |
| 1000 | Bourke | –30.7287 | 143.5738 | – | Yes | No | n/a | No | n/a | 609.06 | Surveyed |
| 1001 | Bourke | –30.7294 | 143.5739 | – | Yes | No | n/a | No | n/a | 609.08 | Surveyed |
| 1001 | Bourke | –30.7289 | 143.5743 | – | Yes | No | n/a | No | n/a | 609.02 | Surveyed |
| 1001 | Bourke | –30.7288 | 143.5744 | – | Yes | No | n/a | No | n/a | 609.01 | Surveyed |
| 1001 | Bourke | –30.7300 | 143.6130 | – | Yes | No | n/a | No | n/a | 605.91 | Surveyed |
| 1001 | Bourke | –30.5772 | 143.1014 | – | Yes | No | n/a | No | n/a | 640.51 | Surveyed |
| 1002 | Bourke | –30.5250 | 143.1304 | – | Yes | No | n/a | No | n/a | 635.46 | Surveyed |
| 1003 | Bourke | –30.3984 | 143.8354 | – | Yes | No | n/a | No | n/a | 569.37 | Surveyed |
| 1004 | Bourke | –30.7317 | 144.2435 | – | Yes | No | n/a | No | n/a | 555.26 | Surveyed |
| 1005 | Bourke | –30.6315 | 144.4223 | – | Yes | No | n/a | No | n/a | 534.92 | Surveyed |
| 1006 | Bourke | –29.5635 | 146.0671 | – | Yes | No | n/a | No | n/a | 338.19 | Surveyed |
| 1006 | Bourke | –29.5634 | 146.0687 | – | Yes | No | n/a | No | n/a | 338.05 | Surveyed |
| 1007 | Bourke | –29.4823 | 145.7902 | 137 | Yes | No | n/a | No | n/a | 356.38 | Surveyed |
| 1007 | Bourke | –29.4799 | 145.7898 | 136 | Yes | No | n/a | No | n/a | 356.28 | Surveyed |
| 1008 | Bourke | –29.4697 | 145.7577 | 129 | Yes | No | n/a | No | n/a | 358.43 | Surveyed |
| 1009 | Bourke | –29.4308 | 145.7323 | 129 | Yes | No | n/a | No | n/a | 358.53 | Surveyed |
| 1010 | Bourke | –28.9347 | 146.9332 | 140.033 | No | No | n/a | No | n/a | 230.53 | Surveyed |
| 1011 | Springsure | –24.5303 | 149.1203 | 221.47 | No | No | n/a | No | n/a | 65.02 | Surveyed |
| 1012 | Springsure | –24.5834 | 149.1321 | 264.59 | No | No | n/a | No | n/a | 59.80 | Surveyed |
| 301A | Springsure | –24.5920 | 146.8610 | 489.5 | No | No | n/a | No | n/a | 141.88 | Surveyed |
| nv10 | Springsure | –24.7480 | 147.7460 | 831 | No | No | n/a | No | n/a | 52.19 | High-elevation recharge |
| nv11 | Springsure | –24.8300 | 147.7420 | 768.40002 | No | No | n/a | No | n/a | 52.55 | No vehicle access; Remote |
| nv12 | Springsure | –24.7430 | 147.7330 | 859 | No | No | n/a | No | n/a | 53.49 | High-elevation recharge |
| nv126 | Eulo | –28.1454 | 144.4906 | 179 | Yes | No | n/a | No | n/a | 411.34 | No vehicle access; Remote |
| nv13 | Springsure | –24.7111 | 147.6921 | 753 | No | No | n/a | No | n/a | 57.74 | No vehicle access; Remote |
| nv14 | Springsure | –24.7660 | 147.6950 | 965 | No | No | n/a | No | n/a | 57.28 | High-elevation recharge |
| nv15 | Springsure | –24.7550 | 147.6920 | 898 | No | No | n/a | No | n/a | 57.58 | High-elevation recharge |
| nv16 | Springsure | –25.2033 | 147.5883 | 629.61121 | No | No | n/a | No | n/a | 72.16 | No vehicle access; Remote |
| nv17 | Springsure | –25.4000 | 147.5500 | 558.75 | No | No | n/a | No | n/a | 85.50 | No vehicle access; Remote |
| nv18 | Springsure | –25.2667 | 147.4667 | 640.5061 | No | No | n/a | No | n/a | 86.01 | No vehicle access; Remote |
| nv19 | Springsure | –25.0333 | 147.4500 | 602.85413 | No | No | n/a | No | n/a | 81.81 | No vehicle access; Remote |
| nv2 | Springsure | –24.1814 | 149.1511 | 268 | No | No | n/a | No | n/a | 62.89 | No vehicle access; Remote |
| nv20 | Springsure | –25.4000 | 147.3000 | 502.5 | No | No | n/a | No | n/a | 107.21 | No vehicle access; Remote |
| nv21 | Springsure | –24.8600 | 147.2100 | 557 | No | No | n/a | No | n/a | 105.71 | No vehicle access; Remote |
| nv22 | Springsure | –24.8500 | 147.2000 | 461.5 | No | No | n/a | No | n/a | 106.70 | No vehicle access; Remote |
| nv3 | Springsure | –24.2834 | 149.1341 | 296 | No | No | n/a | No | n/a | 72.96 | No vehicle access; Remote |
| nv329 | Springsure | –25.6484 | 149.2011 | 314 | No | No | n/a | No | n/a | 3.31 | Not permanent |
| nv331 | Springsure | –25.3984 | 149.4011 | 245.035 | No | No | n/a | No | n/a | 30.98 | Not permanent |
| nv332 | Springsure | –25.4154 | 149.3681 | 245.035 | No | No | n/a | No | n/a | 27.28 | Not permanent |
| nv337 | Springsure | –23.9134 | 149.1125 | 697 | No | No | n/a | No | n/a | 34.78 | No vehicle access; Remote |
| nv339 | Springsure | –23.7314 | 149.0841 | 372 | No | No | n/a | No | n/a | 19.54 | No vehicle access; Remote |
| nv340 | Springsure | –23.9154 | 149.0201 | 413 | No | No | n/a | No | n/a | 30.33 | <5 km hike |
| nv341 | Springsure | –23.8404 | 149.2291 | 432 | No | No | n/a | No | n/a | 38.13 | No vehicle access; Remote |
| nv343 | Springsure | –23.9214 | 149.0281 | 423 | No | No | n/a | No | n/a | 31.26 | <5 km hike |
| nv348 | Springsure | –24.1614 | 149.1391 | 284 | No | No | n/a | No | n/a | 60.35 | No vehicle access; Remote |
| nv349 | Springsure | –24.2074 | 149.1361 | 257 | No | No | n/a | No | n/a | 65.02 | No vehicle access; Remote |
| nv350 | Springsure | –24.2284 | 149.1331 | 249 | No | No | n/a | No | n/a | 67.11 | No vehicle access; Remote |
| nv351 | Springsure | –24.2554 | 149.1291 | 257 | No | No | n/a | No | n/a | 69.83 | No vehicle access; Remote |
| nv352 | Springsure | –24.2834 | 149.1411 | 265 | No | No | n/a | No | n/a | 73.18 | No vehicle access; Remote |
| nv353 | Springsure | –24.2104 | 149.1461 | 238 | No | No | n/a | No | n/a | 65.69 | No vehicle access; Remote |
| nv354 | Springsure | –24.2824 | 149.1491 | 254 | No | No | n/a | No | n/a | 73.34 | No vehicle access; Remote |
| nv355 | Springsure | –24.3224 | 149.1321 | 242 | No | No | n/a | No | n/a | 77.05 | <5 km hike |
| nv356 | Springsure | –24.3764 | 149.1431 | 248 | No | No | n/a | No | n/a | 82.17 | <5 km hike |
| nv357 | Springsure | –24.3854 | 149.1431 | 248 | No | No | n/a | No | n/a | 81.20 | <5 km hike |
| nv358 | Springsure | –24.4054 | 149.1261 | 253 | No | No | n/a | No | n/a | 78.58 | <5 km hike |
| nv359 | Springsure | –24.4669 | 149.1148 | 287 | No | No | n/a | No | n/a | 71.65 | Not permanent |
| nv363 | Springsure | –24.8154 | 147.9841 | 754 | No | No | n/a | No | n/a | 28.35 | No vehicle access; Remote |
| nv365 | Springsure | –23.9094 | 149.0861 | 674 | No | No | n/a | No | n/a | 32.88 | No vehicle access; Remote |
| nv366 | Springsure | –23.9164 | 149.1051 | 587 | No | No | n/a | No | n/a | 34.62 | No vehicle access; Remote |
| nv367 | Springsure | –23.8864 | 149.2811 | 374 | No | No | n/a | No | n/a | 45.31 | No vehicle access; Remote |
| nv368 | Springsure | –23.8374 | 149.1121 | 388 | No | No | n/a | No | n/a | 28.43 | No vehicle access; Remote |
| nv369 | Springsure | –23.8274 | 149.1121 | 439 | No | No | n/a | No | n/a | 27.69 | No vehicle access; Remote |
| nv370 | Springsure | –25.0134 | 149.2241 | 428 | No | No | n/a | No | n/a | 31.50 | No vehicle access; Remote |
| nv371 | Springsure | –24.0904 | 149.2171 | 229 | No | No | n/a | No | n/a | 57.06 | No vehicle access; Remote |
| nv372 | Springsure | –24.4660 | 149.1110 | 291 | No | No | n/a | No | n/a | 71.64 | Not permanent |
| nv373 | Springsure | –24.4740 | 149.1320 | 262 | No | No | n/a | No | n/a | 71.38 | Not permanent |
| nv374 | Springsure | –24.9564 | 149.4031 | 303 | No | No | n/a | No | n/a | 50.42 | No vehicle access; Remote |
| nv379 | Springsure | –24.5952 | 149.0859 | 313 | No | No | n/a | No | n/a | 57.04 | Not permanent |
| nv383 | Springsure | –25.0009 | 148.9881 | 337 | No | No | n/a | No | n/a | 11.46 | Unreliable source; Remote |
| nv384 | Springsure | –25.0610 | 148.8801 | 426 | No | No | n/a | No | n/a | 2.33 | Unreliable source; Remote |
| nv385 | Springsure | –25.4994 | 148.9831 | 435 | No | No | n/a | No | n/a | 0.00 | No vehicle access; Remote |
| nv386 | Springsure | –23.8574 | 149.3181 | 282 | No | No | n/a | No | n/a | 46.91 | High-elevation recharge |
| nv387 | Springsure | –24.9484 | 148.1351 | 1134 | No | No | n/a | No | n/a | 13.24 | High-elevation recharge |
| nv388 | Springsure | –24.9654 | 148.1581 | 1057 | No | No | n/a | No | n/a | 10.95 | High-elevation recharge |
| nv389 | Springsure | –24.9804 | 148.1751 | 1041 | No | No | n/a | No | n/a | 9.25 | High-elevation recharge |
| nv390 | Springsure | –25.0014 | 148.2111 | 1065 | No | No | n/a | No | n/a | 6.07 | High-elevation recharge |
| nv391 | Springsure | –24.9594 | 148.1001 | 1046 | No | No | n/a | No | n/a | 16.74 | High-elevation recharge |
| nv392 | Springsure | –25.0504 | 148.2161 | 465 | No | No | n/a | No | n/a | 9.26 | No vehicle access; Remote |
| nv394 | Springsure | –25.0314 | 148.2111 | 728 | No | No | n/a | No | n/a | 7.93 | No vehicle access; Remote |
| nv395 | Springsure | –25.0744 | 148.2021 | 823 | No | No | n/a | No | n/a | 12.28 | High-elevation recharge |
| nv396 | Springsure | –25.0284 | 148.1991 | 649 | No | No | n/a | No | n/a | 8.62 | No vehicle access; Remote |
| nv399 | Springsure | –25.0544 | 148.1561 | 815 | No | No | n/a | No | n/a | 13.80 | No vehicle access; Remote |
| nv4 | Springsure | –23.7984 | 149.0711 | 827 | No | No | n/a | No | n/a | 22.44 | <5 km hike |
| nv400 | Springsure | –25.0594 | 148.1621 | 685 | No | No | n/a | No | n/a | 13.67 | No vehicle access; Remote |
| nv401 | Springsure | –25.0454 | 148.1461 | 729 | No | No | n/a | No | n/a | 14.08 | No vehicle access; Remote |
| nv402 | Springsure | –25.3929 | 150.1486 | 184 | No | No | n/a | No | n/a | 49.43 | No vehicle access; Remote |
| nv403 | Springsure | –25.3192 | 150.0847 | 226 | No | No | n/a | No | n/a | 57.46 | No vehicle access; Remote |
| nv404 | Springsure | –25.3933 | 150.1714 | 263 | No | No | n/a | No | n/a | 49.65 | No vehicle access; Remote |
| nv405 | Springsure | –25.4235 | 150.1926 | 185 | No | No | n/a | No | n/a | 46.66 | No vehicle access; Remote |
| nv408 | Springsure | –24.9170 | 147.5500 | 710 | No | No | n/a | No | n/a | 71.68 | No vehicle access; Remote |
| nv425 | Springsure | –24.9900 | 147.3100 | 575 | No | No | n/a | No | n/a | 95.60 | No vehicle access; Remote |
| nv426 | Springsure | –24.6726 | 147.7284 | 580 | No | No | n/a | No | n/a | 54.63 | No vehicle access; Remote |
| nv427 | Springsure | –24.7930 | 147.6346 | 692 | No | No | n/a | No | n/a | 63.30 | No vehicle access; Remote |
| nv428 | Springsure | –24.8113 | 147.5964 | 638 | No | No | n/a | No | n/a | 67.11 | No vehicle access; Remote |
| nv429 | Springsure | –24.8484 | 147.6968 | 770 | No | No | n/a | No | n/a | 57.06 | No vehicle access; Remote |
| nv435 | Springsure | –25.0754 | 148.2311 | 552 | No | No | n/a | No | n/a | 11.11 | High-elevation recharge |
| nv437 | Springsure | –25.3070 | 148.4478 | 580 | No | No | n/a | No | n/a | 37.02 | No vehicle access; Remote |
| nv438 | Springsure | –25.4784 | 148.2158 | 477 | No | No | n/a | No | n/a | 53.24 | No vehicle access; Remote |
| nv5 | Springsure | –24.9184 | 148.9411 | 451.5 | No | No | n/a | No | n/a | 18.45 | Unreliable source; Remote |
| nv6 | Springsure | –25.2314 | 148.3511 | 823.838 | No | No | n/a | No | n/a | 28.42 | Unreliable source; Remote |
| nv600 | Springsure | –25.1312 | 148.1312 | 673 | No | No | n/a | No | n/a | 21.59 | Surveyed |
| nv601 | Springsure | –25.1762 | 148.1182 | 620 | No | No | n/a | No | n/a | 26.41 | Surveyed |
| nv602 | Springsure | –25.1555 | 148.0844 | 632 | No | No | n/a | No | n/a | 26.71 | Surveyed |
| nv603 | Springsure | –25.1477 | 148.1190 | 654 | No | No | n/a | No | n/a | 23.79 | Surveyed |
| nv604 | Springsure | –25.0795 | 148.1230 | 809 | No | No | n/a | No | n/a | 18.12 | High-elevation recharge |
| nv605 | Springsure | –25.1286 | 148.1972 | 660 | No | No | n/a | No | n/a | 17.89 | Surveyed |
| nv606 | Springsure | –25.0813 | 148.1263 | 774 | No | No | n/a | No | n/a | 17.99 | High-elevation recharge |
| nv607 | Springsure | –25.1104 | 148.1371 | 713 | No | No | n/a | No | n/a | 19.44 | Surveyed |
| nv608 | Springsure | –25.0962 | 148.1626 | 879 | No | No | n/a | No | n/a | 16.57 | High-elevation recharge |
| nv609 | Springsure | –24.5778 | 148.2525 | 286 | No | No | n/a | No | n/a | 19.18 | No vehicle access; Remote |
| nv610 | Springsure | –24.5633 | 148.2464 | 351 | No | No | n/a | No | n/a | 20.85 | No vehicle access; Remote |
| nv611 | Springsure | –24.5680 | 148.2306 | 386 | No | No | n/a | No | n/a | 20.56 | No vehicle access; Remote |
| nv612 | Springsure | –24.5252 | 148.2210 | 386 | No | No | n/a | No | n/a | 25.44 | No vehicle access; Remote |
| nv613 | Springsure | –24.5276 | 148.2368 | 398 | No | No | n/a | No | n/a | 24.94 | High-elevation recharge |
| nv614 | Springsure | –24.5324 | 148.2068 | 406 | No | No | n/a | No | n/a | 24.96 | No vehicle access; Remote |
| nv615 | Springsure | –24.5516 | 148.1758 | 540 | No | No | n/a | No | n/a | 23.89 | High-elevation recharge |
| nv616 | Springsure | –24.5324 | 148.1989 | 434 | No | No | n/a | No | n/a | 25.17 | No vehicle access; Remote |
| nv617 | Springsure | –24.5083 | 148.2141 | 418 | No | No | n/a | No | n/a | 27.43 | High-elevation recharge |
| nv618 | Springsure | –24.4987 | 148.2102 | 415 | No | No | n/a | No | n/a | 28.56 | High-elevation recharge |
| nv619 | Springsure | –24.4967 | 148.2194 | 390 | No | No | n/a | No | n/a | 28.62 | High-elevation recharge |
| nv620 | Springsure | –24.4811 | 148.2120 | 407 | No | No | n/a | No | n/a | 30.46 | High-elevation recharge |
| nv621 | Springsure | –24.6317 | 149.0629 | 381 | No | No | n/a | No | n/a | 52.47 | No vehicle access; Remote |
| nv622 | Springsure | –24.6225 | 149.1018 | 310 | No | No | n/a | No | n/a | 54.65 | No vehicle access; Remote |
| nv623 | Bourke | –30.7020 | 143.7163 | – | Yes | No | n/a | No | n/a | 595.88 | No vehicle access; Remote |
| nv624 | Bourke | –30.0300 | 142.6610 | – | Yes | No | n/a | No | n/a | 651.66 | No vehicle access; Remote |
| nv625 | Eulo | –28.6840 | 144.3090 | 125 | No | No | n/a | No | n/a | 445.63 | Unable to locate |
| nv626 | Eulo | –28.1870 | 144.4420 | 207 | No | No | n/a | No | n/a | 417.01 | Unable to locate |
| nv627 | Eulo | –28.5230 | 144.5150 | 206 | No | No | n/a | No | n/a | 420.82 | Unable to locate |
| nv628 | Eulo | –28.5230 | 144.5150 | 206 | No | No | n/a | No | n/a | 420.82 | Unable to locate |
| nv7 | Springsure | –25.2314 | 148.3341 | 758.773 | No | No | n/a | No | n/a | 28.17 | No vehicle access; Remote |
| nv8 | Springsure | –25.0314 | 148.1311 | 757 | No | No | n/a | No | n/a | 14.73 | No vehicle access; Remote |
| nv9 | Springsure | –24.7484 | 147.8811 | 777.25 | No | No | n/a | No | n/a | 38.67 | No vehicle access; Remote |
| SP1 | Springsure | –27.6024 | 151.9395 | 667.12 | No | No | n/a | No | n/a | 50.88 | Non–GAB |
| SP10 | Springsure | –27.8998 | 151.9456 | 452.29 | No | No | n/a | No | n/a | 57.74 | Non–GAB |
| SP11 | Springsure | –28.0253 | 151.9794 | 476.16 | No | No | n/a | No | n/a | 67.96 | Non–GAB |
| SP15 | Springsure | –27.5701 | 151.8476 | 510.57 | No | No | n/a | No | n/a | 41.92 | Non–GAB |
| SP2 | Springsure | –26.8771 | 151.5959 | 579.7 | No | No | n/a | No | n/a | 42.24 | Non–GAB |
| SP3 | Springsure | –27.3946 | 151.5736 | 376 | No | No | n/a | No | n/a | 19.16 | Non–GAB |
| SP37 | Springsure | –27.9521 | 151.9928 | 491.97 | No | No | n/a | No | n/a | 64.57 | Non–GAB |
| SP38 | Springsure | –27.9596 | 151.9998 | 493.52 | No | No | n/a | No | n/a | 65.58 | Non–GAB |
| SP4 | Springsure | –27.3278 | 151.4570 | 360 | No | No | n/a | No | n/a | 11.98 | Non–GAB |
| SP42 | Springsure | –27.6036 | 151.9110 | 597.99 | No | No | n/a | No | n/a | 48.10 | Non–GAB |
| SP43 | Springsure | –27.5815 | 151.8899 | 548.08 | No | No | n/a | No | n/a | 46.05 | Non–GAB |
| SP44 | Springsure | –27.4265 | 151.9071 | 502.82 | No | No | n/a | No | n/a | 48.45 | Non–GAB |
| SP45 | Springsure | –27.4393 | 151.9561 | 621.86 | No | No | n/a | No | n/a | 52.97 | Non–GAB |
| SP5 | Springsure | –27.2186 | 151.7609 | 457 | No | No | n/a | No | n/a | 41.86 | Non–GAB |
| SP6 | Springsure | –27.6213 | 151.6208 | 384.09 | No | No | n/a | No | n/a | 19.80 | Non–GAB |
| SP7 | Springsure | –27.6854 | 151.6235 | 462.83 | No | No | n/a | No | n/a | 20.18 | Non–GAB |
| SP8 | Springsure | –27.6651 | 151.5749 | 460.04 | No | No | n/a | No | n/a | 15.32 | Non–GAB |
| SP9 | Springsure | –27.7107 | 151.7254 | 469.34 | No | No | n/a | No | n/a | 30.41 | Non–GAB |
| x328 | Springsure | –25.3654 | 149.3591 | 245.035 | No | No | n/a | No | n/a | 27.32 | Not permanent |
| x346 | Springsure | –25.4754 | 150.1271 | 181 | No | No | n/a | No | n/a | 40.03 | No spring found |
| x375 | Springsure | –24.4995 | 149.1287 | 278 | No | No | n/a | No | n/a | 68.56 | No spring found |
| x377 | Springsure | –24.5637 | 149.1189 | 281 | No | No | n/a | No | n/a | 61.42 | No spring found |
| x380 | Springsure | –24.6464 | 149.0871 | 317 | No | No | n/a | No | n/a | 51.63 | No spring found |
| x381 | Springsure | –24.6764 | 149.0811 | 352 | No | No | n/a | No | n/a | 48.26 | No spring found |
| x431 | Springsure | –25.7206 | 149.0285 | 287 | No | No | n/a | No | n/a | 0.00 | No spring found |
| x436 | Springsure | –25.6718 | 148.4547 | 455 | No | No | n/a | No | n/a | 29.40 | No spring found |

# Appendix D:	Nomination form and guidelines for changing the conservation status of a species under the Nature Conservation Act 1992 (Qld)

Species nomination form and guidelines for adding or changing the category of a native species listing under the Queensland Nature Conservation Act 1992 (NCA Act)

For: Calocephalus sp. (Eulo ME Ballingall MEB2590)

General notes

The purpose of this document is to nominate a species for assessment under the NCA by the Department of Environment and Resource Management Species Technical Committee (STC) for its consideration and subsequent advice to the Minister for Climate Change and Sustainability.

Please use one nomination form for each species. The form may be submitted electronically, however, the original, signed, hard copy must also be lodged. Lodgement instructions are provided at the end of the form. The STC will not consider nominations submitted in any other format.

Each section of the form needs to be completed with as much detail as possible, and Uindicate when there is no information availableU. Identify your Ureferences / information sourcesU, document reasons and supportive data. Indicate the quality of facts/information, for example was it based on research or anecdotal data; on observed data or estimated or inferred from data; or suspected to be the case. Identify Uconfidential materialU and explain the sensitivity.

The STC will not consider incomplete nominations or nominations with insufficient information. Your nomination will be returned to you if inadequate information is provided.

Your nomination must be supported with referenced summaries of relevant information from the scientific literature. Full bibliographic details are to be provided. The opinion of appropriate scientific experts may also be cited, provided they authorise you to do so. The names of the expert(s), their qualifications and full contact details must also be provided if they are cited.

The STC assesses nominations against the IUCN Red List Categories and Criteria (version 3.1) for the categories of extinct in the wild, endangered, vulnerable, near threatened and least concern. The IUCN updates its red list guidelines regularly and the STC uses the most recent version (version 8.0). This form will be updated in accord with revisions of IUCN 40Tcriteria, if necessary. A full description of the IUCN categories and criteria can be found in: IUCN 2001. IUCN Red List Categories: Version 3.1. Prepared by the IUCN Species Survival Commission. IUCN, Gland, Switzerland and Cambridge, UK.

40Twww.iucnredlist.org/documents/redlist_cats_crit_en.pdf http://www.iucnredlist.org/documents/RedListGuidelines.pdf .

USpecies—Uapplies to the entity nominated under the Nature Conservation Act

UPopulationU—refers to populations within a species or total population numbers for a species.

Section 1. Summary

1.1 Scientific and common name of species (or subspecies)

Calocephalus sp. (Eulo ME Ballingal MEB2590)

1.2 If the species is not conventionally accepted, please provide: a taxonomic description of the species in a form suitable for publication in conventional scientific literature. State where this description has been submitted for publication; or evidence that a scientific institution has a specimen of the species and a written statement signed by a person who is a taxonomist with relevant expertise (has worked, or is a published author, on the class of species nominated) that the species is new. Details of the qualifications and experience of the taxon expert need to be provided. For a specimen lodged at a museum or herbarium, state where the specimen is held, the collector name, collection date and collection/voucher number.

Specimens are currently on loan to DNA (Alice Springs) to Phillip Short who is revising the group.

1.3 If a population is being nominated, justify why the population should be considered separately from the species as a whole. This will generally require evidence why the nominated population is considered genetically distinct and/or geographically separate and/or severely threatened in comparison with all other populations of the species.

1.4 Please provide a description of the species or population that is sufficient to distinguish it from other species or populations.

Differs from all other Calocephalus and similar species in the area by the stems and leaves that are completely glabrous and glandular (microscopic); compound heads broader than long and much wider than C. sonderi, with a whitish appearance from very dense white woolly bracts, with florets pale yellow and barely topping the woolly bracts.

1.5 Current conservation status under Nature Conservation Act 1992 and the EPBC Act

NCA: NT EPBC: none

1.6 Proposed conservation status under the Nature Conservation Act 1992 and the EPBC Act

Vulnerable

1.7 IUCN Criteria under which the species is eligible for listing. The species should be judged against the criteria described in Attachment B: Categories and criteria used for assessing the status of species. The categories for extinct in the wild, endangered, vulnerable and near threatened use the most recent version of IUCN criteria.

Vulnerable (D2)

Section 2. Species ecology/biology

2.1 Is this species conventionally accepted? If not, explain why. Is there any controversy on the taxonomy?

Accepted but awaiting formal description (Phillip Short ms.)

2.2 Give a brief description of the species’: appearance, including size and/or weight, and sex and age variation if appropriate; social structure and dispersion (e.g. solitary/clumped/flocks)

Annual tufted herb, with multiple stems to 20 cm high. Stems and leaves glabrous, with minute scattered gland dots (microscopic). The leaves are alternate or sub-opposite, linear–oblong, 4–10 mm long and up to 1 mm wide, with a rounded apex. The compound heads are depressed globosa to 13 mm wide, and white woolly, the general involucral bracts are green. The partial heads have 5 to 7 pale yellow florets and about the same number of woolly bracts with flat white tips and often a green central strip at the base. Achenes to 1 mm long, hairy, and with a pappus of with 6–8 separate plumose bristles (Queensland Herbarium 2012).

2.3 Describe the species’ habitat (e.g. aspect, topography, substrate, climate, forest type, associated species, sympatric species).

Associated only with Great Artesian Basin (GAB) springs in the Eulo Supergroup (Habermehl 1982). The species occurs on scalded claypans and plains, and low lunettes which typically surround GAB springs, and on the sides of inactive and active mound springs.

2.4 What is the species’ generation length? Note: Generation length is the average age of parents of the current cohort (i.e. newborn individuals in the population).

The species is short-lived but has a large, woody taproot and can behave as a perennial in good seasons.

2.5 Is there any other information regarding the species ecology or biology relevant to a conservation status assessment?

Only associated with GAB springs. Flowers in spring and summer. Widespread death of mature plants occurs most years, followed by mass germination under the dead plants.

Section 3. Conservation status

| 3.1 Describe the species’ distribution in Australia and attach a map of known localities. Please include details of which Natural Resource Management and IBRA Bioregions the species occurs in. |  |  |  |  |
| --- | --- | --- | --- | --- |
| Confined to south-western Queensland in the Mulga Lands IBRA region. It occurs in a band west of Eulo, extending to Bundoona/Yowah Creek in the north, Granite Springs in the west and Currawinya National Park in the south. Map 1 shows all records of the species, as well as absences, based on a comprehensive 2012 survey of springs in the Eulo supergroup. |  |  |  |  |
| 3.2 What is the species’ total extent of occurrence (in kmP2P) (see Attachment A) |  |  |  |  |
| The species occurs in an area (rectangle) of 7000 kmP2 P(100 km × 70 km). |  |  |  |  |
| 3.3 What is the species’ total area of occupancy (in kmP2P) (see Attachment A) |  |  |  |  |
| The estimated area of occupancy is 3.2 kmP2P. The area occupied by each of the 18 known populations ranged from 800 mP2P to 1 kmP2P. The total area of occupancy is a tiny fraction of the extent of occurrence, as the species is confined to springs and their surrounds, often in very small patches. |  |  |  |  |
| 3.4 What is the species’ total population size in terms of number of mature individuals? |  |  |  |  |
| In spring–summer 2012, the total estimated population size was about 15 000 individuals. At large populations, density of plants in a subset of the population and the total area of occupancy were used to estimate population size. The largest population encountered contained >5000 plants, while some populations contained only 50 individuals. However, population size is known to vary substantially with seasonal conditions (see below). |  |  |  |  |
| 3.5 How many locations do you consider the species occurs in and why? Where are these located? Note: The term 'location' defines a geographically or ecologically distinct area. |  |  |  |  |
| The species occurs at 18 distinct locations. While it is likely that a handful of populations were missed in large spring areas (such as on Currawinya NP and Granite Springs), these data are based on a comprehensive survey of all spring groups in the Eulo Supergroup in Qld and NSW. (Prior to the 2012 surveys, the species was known from five localities, but there were no data on population size or area of occupancy; all these known locations were visited in 2012, so the old records are not included in the table below.) |  |  |  |  |
| 3.6 For UfloraU, and where applicable, for UfaunaU, detail the location, land tenure, survey date, estimated number of individuals and area of occupancy. This is optional for taxa nominated as near threatened or least concern. Summary distribution information such as a map and list of localities should be provided for taxa nominated as near threatened or least concern. |  |  |  |  |
| Location | Land tenure | Date of most recent survey | Number of individuals at location | Area of occupancy at location (kmP2PPP) |
| Fenced spring complex, Currawinya National Park, 13 km north of ranger headquarters | National Park | 5/12/2012 | 5000 | 0.25 |
| Dead Horse Spring, Currawinya National Park, 13km north of ranger headquarters | National Park | 5/12/2012 | 300 | 0.04 |
| West of Yowah Creek, Merimo | Private | 16/10/2012 | 200 | 0.0008 |
| Wooregym Bore and springs, Bingara | Private | 16/10/2012 | 500 | 0.50 |
| Fish Springs (fenced), Granite Springs | Private | 18/10/2012 | 2000 | 0.25 |
| Near Bull Spring, Granite Springs | Private | 18/10/2012 | 200 | 0.25 |
| Wiggera Spring, Granite Springs/Bingara boundary | Private | 18/10/2012 | 200 | 1.00 |
| Double Well and spring, Boorara, just south of Granite Springs boundary | Private | 19/10/2012 | 200 | 0.04 |
| Tunkata Well and spring, Boorara, just south of Granite Springs boundary | Private | 19/10/2012 | 500 | 0.04 |
| Boundary Springs, Granite Springs – Dynevor Downs boundary | Private | 19/10/2012 | 100 | 0.005 |
| Dewalla Springs, Bingara | Private | 20/10/2012 | 50 | 0.20 |
| Eulo Town Common, west of Pitherty road, 2.5 km south-west of town | Town common | 20/10/2012 | 200 | 0.04 |
| East of Shire Tank, Currawinya National Park | National Park | 5/12/2012 | 2500 | 0.25 |
| Wombula–Granite springs boundary, north-east of Tunca Spring | Private | 8/12/2012 | 50 | 0.003 |
| Yowah Creek Springs, Bundoona | Private | 10/12/2012 | 2000 | 0.01 |
| Little Bundoona Springs, Dead Sea, Penaroo | Private | 11/12/2012 | 200 | 0.25 |
| Pretty Plains Springs, Tilbooroo | Private | 12/12/2012 | 200 | 0.05 |
| 1.7 km north of Eulo, on western side of Paroo River | Town Common | 13/12/2012 | 300 | 0.0002 |
| 3.7 Is the species’ distribution severely fragmented? If so, what is the cause of this fragmentation? Note: Severely fragmented refers to the situation in which increased extinction risk to the taxon results from most individuals being found in small and relatively isolated populations (in certain circumstances this may be inferred from habitat information). These small populations may go extinct, with a reduced probability of recolonisation |  |  |  |  |
| Distribution is naturally fragmented, confined to springs and their immediate surrounds. |  |  |  |  |
| 3.8 Does the species undergo extreme natural fluctuations in population numbers, extent of occurrence or area of occupancy? To what extent and why? Note: Extreme fluctuations can be said to occur in a number of taxa when population size or distribution area varies widely, rapidly and frequently, typically with a variation greater than one order of magnitude (i.e. a tenfold increase or decrease). |  |  |  |  |
| The extent of occurrence and area of occupancy remain relatively constant. However, population numbers undergo large natural fluctuations in response to prevailing seasonal conditions. For example, a population on Currawinya National Park that has been visited repeatedly during the past five years fluctuates from occurring as scattered plants to forming the dominant groundcover across a couple of hectares. It seems to respond to winter–spring rainfall in particular. Nearly all mature plants in any population can die off simultaneously, and such events are usually followed by mass germination of seedlings, often under the standing remains of dead plants. |  |  |  |  |
| 3.9 What data are there to indicate past trends in the species’ population size, distribution, extent or quality of habitat? (if available, include data that indicates the percentage decline over the past 10 years or 3 generations whichever is longer)? |  |  |  |  |
| The collecting record does not indicate a decline in this species across its range. Transect data from the past two years inside and outside fenced areas documents substantial natural population fluctuations (see section 3.8 above), and an increase in the abundance and fecundity of the species when grazing pressure is removed. Thus it is likely that the species was more abundant prior to the introduction of feral and domestic herbivores. |  |  |  |  |
| 3.10 What data are there to indicate future changes in the species’ population size, distribution, extent or quality of habitat? (if available, include data that indicates the percentage decline over 10 years or 3 generations whichever is longer (up to a maximum of 100 years in the future) where the time period is a continuous period that may include a component of the past? |  |  |  |  |
| Spring habitats are extremely vulnerable to habitat degradation, through aquifer drawdown, concentrated grazing and trampling pressure, excavation and sowing of introduced pasture species (Fensham & Fairfax 2003). With the exception of grazing pressure, these impacts don’t affect the habitat of Calocephalus sp., as it occurs in scalded country adjacent to springs which remains suitable even after damage or destruction of springs. However, concentrated pressure from goats, sheep, cattle and, to an extent, native macropods, is likely to result in continued degradation of habitat and a decline in the abundance and fecundity of the species. |  |  |  |  |
| 3.11 Has the species been reasonably well surveyed? Is the species’ current known distribution and/or population size likely to be its actual distribution and/or population size? |  |  |  |  |
| Eighty-six spring group, representing all but three groups in the Eulo supergroup, were surveyed between October and December 2012. Surveys were also undertaken on springs in the Bourke and Bogan River supergroups (NSW) and all supergroups in Queensland. Thus, current estimates are likely to closely reflect the actual distribution, area of occupancy and population size. However, it is probable that a small number of populations that occur in large spring areas, such as on Currawinya National Park and Granite Springs, have been missed. |  |  |  |  |
| 3.12 For species considered eligible for listing as extinct or extinct in the wild, please provide details of the most recent known collection, or authenticated sighting of the species in the wild and whether additional populations are likely to exist. |  |  |  |  |

4. Threats and threat abatement

4.1 Identify past, current and future threats indicating whether they are actual or potential. For each threat describe: a. how and where it impacts on this species b. what its effect has been so far (indicate whether it is known or suspected; does it only affect certain populations) c. what is its expected effect in the future (is the threat only suspected; does it only affect certain populations)

Habitat destruction through aquifer drawdown, excavation of habitat, elevated grazing pressure, feral pig damage and introduced pasture species are observed threats to spring-dependent species (Silcock et al. 2011). However, these impacts do not affect the persistence of Calocephalus sp., which grows around even long-extinct (potentially pre-European) springs. The major threat to this species is high-grazing pressure. It is highly palatable and is nearly always heavily grazed by feral animals (goats) and domestic livestock (cattle and sheep). Macropods may also contribute to this grazing pressure. In 13 of the 18 known populations, plants were heavily grazed, usually by goats and/or cattle. Plants were typically cropped to ground level, although most still had a few flowers. Ungrazed plants (usually in fenced areas) have a very different growth form to grazed plants, spreading to 40 cm tall and up to 80 cm across, and being laden with flowers. Transects inside and outside fenced areas show a clear effect of grazing on fecundity and abundance of this species in areas where grazing pressure is high. However, plants survive and can produce seedlings even under very heavy grazing pressure. Thus, the long-term effects of grazing on the persistence of this species are unknown. Ongoing monitoring of these transects will shed further light on these grazing impacts.

4.2 Where possible, provide information on threats for each occurrence/location. This is optional for taxa nominated as near threatened or least concern. Summary information should be provided for taxa nominated as near threatened or least concern.

Location/population; Grazing pressure (as recorded October–December 2012). Fenced spring complex, Currawinya National Park, 13 km north of ranger headquarters; All plants outside fence very heavily grazed by goats, cropped to ground level; plants inside fenced area recovered rapidly and form dominant groundcover after winter–spring rain. Dead Horse Spring, Currawinya National Park, 13 km north of ranger headquarters; All plants outside fence very heavily grazed by goats, cropped to ground level; fence only completed in September 2012, so no signs of recovery yet. West of Yowah Creek, Merimo; Some plants grazed by goats and macropods. Wooregym Bore and springs, Bingara; Most plants heavily grazed and trampled by cattle, but some still flowering. Fish Springs (fenced), Granite Springs; Plants outside fence heavily grazed by goats and cattle, but some seedlings still present; seedlings also present inside fenced area. Near Bull Spring, Granite Springs; Some plants grazed by goats. Wiggera Spring, Granite Springs/Bingara boundary; Plants all very dry and shrivelled at time of survey, so difficult to assess grazing pressure. Double Well and spring, Boorara, just south of Granite Springs boundary; Some plants grazed by goats. Tunkata Well and spring, Boorara, just south of Granite Springs boundary; Some plants grazed by goats. Boundary Springs, Granite Springs – Dynevor Downs boundary; Plants look healthy/leafy, although most are heavily grazed by goats and cattle. Dewalla Springs, Bingara; Not heavily grazed. Eulo Town Common, west of Pitherty road, 2.5 km south-west of town; Very healthy, large plants, mostly ungrazed. East of Shire Tank, Currawinya National Park; Woody plants, all very heavily grazed by goats. Wombula – Granite Springs boundary, north-east of Tunca Spring; All plants grazed to <5 cm high. Yowah Creek Springs, Bundoona; All plants grazed to ground level, but some still flowering. Little Bundoona Springs, Dead Sea, Penaroo; All plants grazed to ground level by goats and sheep, but some still flowering. Pretty Plains Springs, Tilbooroo; Heavily grazed by goats and cattle; some older plants have recently died and seedlings to <3 cm (as yet ungrazed) are growing up underneath standing dead plants. 1.7 km north of Eulo, on western side of Paroo River; Not heavily grazed despite high cattle grazing pressure at site; mostly young plants in population, including tiny seedlings to <3 cm.

4.3 Identify and explain any additional biological characteristics particular to the species that are threatening to its survival.

None known.

4.4 Give an overview of how threats are being abated/could be abated and other recovery actions under way/proposed. Identify who is undertaking these activities and how successful the activities have been to date.

Two populations on Currawinya National Park and one population on Granite Springs have been recently fenced to exclude all large herbivores (the fenced populations are identified in Section 3.6 above). Fencing has been undertaken under a Caring for Country project led by Stephen Peck from Charleville National Parks. Permanent monitoring transects have been established inside and outside the fences to examine the effects of removal of grazing pressure on the species. Continued monitoring will shed further light on population dynamics and grazing effects. There is a Nature Refuge agreement over the Dead Sea springs, which contains a population of the species, however, the area remains very heavily grazed by goats, macropods and sheep. Concerted and ongoing goat control across the area, through aerial mustering and targeted on-ground shooting, will alleviate much of the grazing pressure on this species. Wooregym Springs on Bingara would benefit from fencing, which would protect the Calocephalus and other spring endemics from grazing and trampling by cattle.

4.5 Identify key management documentation for the species (e.g. recovery plans, conservation plans, threat abatement plans).

The species is not mentioned in the Currawinya National Park Management Plan (2001).

4.6 Are there any management or research recommendations from the documents mentioned in 4.5 or otherwise, that will assist in the conservation of the species?

No.

5. Compilers, referees and references

| 5.1 Compiler(s) details |  |
| --- | --- |
| Name(s) | Ailsa Holland |
| Organisation(s) | Queensland Herbarium |
| Contact details |  |
| Postal address | Brisbane Botanic Gardens, Mt Coot-tha, Mt Coot-tha road, Toowong, 4066 |
| Email | Ailsa.Holland@derm.qld.gov.au |
| Phone | (07) 38969317 Fax (07) 38969324 |
| Date | 9 October 2012 |
| 5.2 Has this document been refereed? If so, indicate by whom |  |
| 5.3 Reference List |  |
| Fensham RJ & Fairfax RJ 2003, Spring wetlands of the Great Artesian Basin, Queensland, Australia’, Wetlands Ecology and Management, vol. 11, pp. 343–62. Habermehl MA 1982, Springs in the Great Artesian Basin, Australia: their origin and nature,. Bureau of Mineral Resources, Geology and Geophysics, Canberra. Queensland Herbarium 2012, HERBRECS, specimen label information, accessed 11 October 2012. Silcock JL 2011, ‘Assessing rarity and threat in an arid-zone flora’, Australian Journal of Botany, vol. 59, pp. 336–50. |  |

# Appendix E:	Conceptual diagrams of Great Artesian Basin spring types

For all diagrams:

EPBC = 113TEnvironment Protection and Biodiversity Conservation Act 1999 (Cwlth)

GAB = Great Artesian Basin.

Type A. Springs emanating from outcropping sediments of the GAB. Water is not flowing to the spring under aquifer pressure. Water flows to the spring either along low-permeability layer or through a fracture. These springs occur in areas of high relief in the landscape and springs (GAB springs, but not EPBC listed).

Type B. Spring emanating from outcropping sediments of the GAB where the saturated zone of the aquifer (watertable) intersects with the ground surface. These springs are in gullies or valleys. (GAB recharge springs, not EPBC listed) .

Type C. A discharge spring where the confining layer (aquitard) has been thinned by erosion and the underlying artesian aquifer is able to flow to the surface (GAB springs, EPBC listed).

Type D. Discharge spring resulting from an anticline or a monocline elevating the aquifer relative to the surface (GAB springs, EPBC listed).

Type E. Discharge spring emanating from uppermost artesian aquifer through fault (GAB springs, EPBC listed).

Type F. Discharge spring emanating from deep artesian aquifer where the fault is sufficiently transmissive to allow discharge from sequences beneath the uppermost aquifer layer (GAB springs, EPBC listed).

Type G. Discharge spring occurring on the top of the basement dome, ridge or platform, where the aquitards has been weakened or fractured (GAB springs, EPBC listed).

Type H. Discharge spring associated with weakness or thinning, and possibly the fracturing of the aquitard associated with basement structure (GAB springs, EPBC listed).

Type I. Spring emanating from contact between onlapping sediments and outcropping basement where the groundwater source is the upper-most aquifer (GAB springs, EPBC listed).

Type J. Spring emanating from contact between onlapping sediments and outcropping basement with mixing of the groundwater from multiple aquifer sequences (GAB springs, EPBC listed).

Type K. Spring emanating from downgradient (western or southern) edge of the GAB (GAB springs, EPBC listed).

Type L. Springs emanating from within volcanic capping above outcropping sediments, often from porous layers in the basalt (non-GAB springs, not EPBC listed).

Type M. Springs emanating from contact between volcanic capping and outcropping sediments (non-GAB springs, not EPBC listed).

Type N. Springs emanating from within Tertiary sandstone overlying the GAB. Water flows to the spring either along a low-permeability layer or through a fracture (non-GAB springs, not EPBC listed).

Type O. Springs emanating from the base of Tertiary sandstone where it contacts the confining layer above aquifer sediments (non-GAB springs, not EPBC listed).

Costello, John (1838–1923), in Australian dictionary of biography, National Centre of Biography, Australian National University, Canberra.