Department of Environment Hazardous Waste Pathways PRN1314-0523 June 2014

This report: has been prepared by GHD for Department of Environment  and may only be used and relied on by Department of Environment  for the purpose agreed between GHD and the Department of Environment  as set out in section 1.2 of this report.

GHD otherwise disclaims responsibility to any person other than Department of Environment  arising in connection with this report. GHD also excludes implied warranties and conditions, to the extent legally permissible.

The services undertaken by GHD in connection with preparing this report were limited to those specifically detailed in the report and are subject to the scope limitations set out in the report.

The opinions, conclusions and any recommendations in this report are based on conditions encountered and information reviewed at the date of preparation of the report.  GHD has no responsibility or obligation to update this report to account for events or changes occurring subsequent to the date that the report was prepared.

The opinions, conclusions and any recommendations in this report are based on assumptions made by GHD described in this report in section 1.3. GHD disclaims liability arising from any of the assumptions being incorrect.

GHD has prepared this report on the basis of information provided by Department of Environment  and others who provided information to GHD (including Government authorities)], which GHD has not independently verified or checked beyond the agreed scope of work. GHD does not accept liability in connection with such unverified information, including errors and omissions in the report which were caused by errors or omissions in that information.

GHD excludes and disclaims all liability for all claims, expenses, losses, damages and costs, including indirect, incidental or consequential loss, legal costs, special or exemplary damages and loss of profits, savings or economic benefit, Department of Environment  may incur as a direct or indirect result of the Hazardous Waste Pathways database for any reason being inaccurate, incomplete or incapable of being processed on Department of Environment ’s equipment or systems or failing to achieve any particular purpose. To the extent permitted by law, GHD excludes any warranty, condition, undertaking or term, whether express or implied, statutory or otherwise, as to the condition, quality, performance, merchantability or fitness for purpose of the Hazardous Waste Pathways database

GHD does not guarantee that the Hazardous Waste Pathways database is free of computer viruses or other conditions that may damage or interfere with data, hardware or software with which it might be used. Department of Environment  absolves GHD from any consequence of Department of Environment  or other person’s use of or reliance on, the Hazardous Waste Pathways Database.

Table of contents

1.	Introduction	1

1.1	Background	1

1.2	Scope and limitations	2

1.3	Assumptions	2

2.	Methodology	3

2.1	Hazardous Waste Background Review	3

2.2	Hazardous Waste Classification	3

2.3	Hazardous Waste Handling Transport and Tracking	3

2.4	Hazardous Waste Treatment	3

3.	Hazardous Waste Production	4

4.	Hazardous Waste Pathways	5

4.1	Waste Description and Classification	5

4.2	Description of primary hazardous waste categories	7

4.3	Waste Tracking	10

4.4	Hazardous Waste Treatment Processes	12

4.5	Hazardous Waste Treatment in Australia	16

5.	Waste Treatment Guidelines	17

5.1	Pre-acceptance Procedures	17

5.2	Acceptance Procedures	17

5.3	Storage and Handling Procedures	17

6.	Hazardous Waste Export	19

7.	References	20

8.	Appendix A - Hazardous Waste Classification and Treatment Options in Australia	22

Table index

Table 1 NEPM 75 Waste Categories and Codes	5

Table 2 EU Waste Directive Disposal Treatment	12

Table 3 EU Waste Directive Recovery Treatment	12

Figure index

Figure 1 Waste Identification and Classification	5

Figure 2. Waste Tracking Procedure	10

# Introduction

## Background

GHD has been engaged by the Hazardous Waste Section of the Department of Environment to produce a description of key pathways taken by hazardous wastes in Australia, from arising to fate. There are variable approaches to hazardous waste management in Australia as different jurisdictions (State and Territory Governments) control the management, treatment and disposal of hazardous waste materials. The Commonwealth Government under the Hazardous Waste (Regulation of Exports and Imports) Act 1989 (Hazardous Waste  Act) aims to regulate the export and import of hazardous waste so that humans and the environment are protected from possible harmful effects of the waste material.

The objectives of the Act in relation to hazardous waste include:

- Processing of export, import and transit permit applications under the Act

- Ensuring compliance and enforcement of the Act

- Preparing, implementing and amending legislation relating to the international movements of hazardous waste to, from or through Australia

- Formulating and implementing policies relating to the international movements of hazardous waste to, from or through Australia

- Participating in International forums such as the Basel Convention and Organisation for Economic Cooperation and Development (OECD) which deal with the international movements of hazardous waste

- Consulting, preparing and providing information to stakeholders on the Act and the permit process.

The Act considers environmental responsibilities both within and outside of Australia.

Waste can be classified as hazardous based on material or source; such as waste containing materials that are themselves considered to be hazardous (e.g arsenic) or waste generated from sources that are known to produce hazardous wastes (eg metal galvinising).

Properties or characteristics of  liquids, gas and solid wastes that can be classified as hazardous include substances that can be::

- Explosive

- Flammable

- Spontaneously combust

- Corrosive

- Toxic

- Radioactive

- Infectious

- Ecotoxic.

Waste is classified by the Environment Protection Authority (EPA) Victoria into three categories according to the hazard they pose. Category A waste is banned from landfills; it must be reused, recycled or treated before it can be disposed of. Category B waste, which poses a high hazard, and Category C waste, which represents the lowest hazard and can be disposed of at approved landfills. This waste classification is also used by New South Wales, other Australian states and territories use alternative classifications and categories for hazardous waste.

The National Environment Protection (Movement of Controlled Waste between States and Territories) Measure 2004 (NEPM) provides a national regulatory framework for management of hazardous waste materials in Australia. It includes handling of controlled waste material, tracking of waste movements, and licensing for those generating, storing, transporting, treating and disposing of waste.

## Scope and limitations

The scope of work for the report is to describe major pathways for hazardous waste in Australia. The purpose of the data will be to publish this material in a user friendly interface on the Department’s website to assist the general public with understanding waste types, pathways and options.

The report provides a summary of:

- Description of waste type, name and alternatives and derivatives

- Acceptable re-use, recovery, treatment or disposal paths options

- Details of the types of facilities that can manage, treat or contain the waste

- Details on export options or paths.

## Assumptions

The following assumptions have been applied to the development of the Hazardous Waste Pathways report.

- The NEPM 75 list has been used to classify hazardous waste types.

- The NEPM list of wastes was consolidated into 15 waste categories that align with common chemical and treatment characteristics.

- The pathways have been prepared on the basis that the Department will use this data to develop into a functional public web interface so the general public can access this information.

- GHD has intentionally avoided reference to specific sites or companies in terms of pathways and treatment to avoid issues with competitive disadvantage for specific companies and facilities, but also to maintain longer term currency of the information as the status of sites and companies will change on an ongoing basis.

- There are many waste practices in Australia that are not considered best practice – GHD has attempted to outline best practice approaches but minimise reference to practices that may be applied that are not adequate for commercial or regulatory reasons.

# Methodology

The following methodology was used for the development of hazardous waste pathways:

## Hazardous Waste Background Review

GHD carried out a preliminary review of background information on waste types and contaminants components. This consisted of desktop analysis of international best practice and guidelines to define and classify hazardous waste across Australia.

## Hazardous Waste Classification

Based on the brief provided by the Department of Environment, waste classification for the report has been based on the National Environmental Protection (Movement of Controlled Waste between States and Territories) Measure (NEPM) list which lists 75 waste types.

GHD has grouped the NEPM list into broader categories to simplify the identification and classification of hazardous waste by the general public, waste producers, transporters and treatment and disposal facilities. This grouping into 15 common categories represents the approach applied in other groupings such as the Victoria EPA and in the Tasmania Controlled Waste Strategy. It also aims to facilitate waste management and selection of treatment processes.

## Hazardous Waste Handling Transport and Tracking

Where relevant, GHD has made reference to specific waste handling, transport and tracking requirements, however as these vary across state and territories, the detail provided will refer back to state or territory hazardous/controlled waste regulation and requirements.

## Hazardous Waste Treatment

GHD has carried out a review of the best available technologies for treatment of hazardous waste internationally and has based this benchmark on the European Union Hazardous Waste Framework Directive (Directive 2006/13/EC) Annex IIA and Annex IIB, which provides a comprehensive approach to best practice waste treatment processes. Many other guides or references to treatment including those in Australia and other states refer to the standards applied in this directive.

# Hazardous Waste Production

Hazardous Waste in Australia is produced by a diverse range of waste generators from ordinary households (e.g. paint, asbestos, chemicals, e-waste, medical waste) to small commercial operations (e.g. resins, solvents, lead acid batteries, tyres), institutions and organisations such as hospitals and quarantine facilities (e.g. sharps, pharmaceuticals, body fluids, quarantine ship waste) as well as industrial waste generators (e.g. contaminated soil, zinc compounds, oils). The diversity of origin and types of waste make hazardous waste management and regulation in Australia very challenging. Combined with the fact that most state and territories have an alternative approach to classification and regulation also creates further challenges.

The table below provides a list of primary hazardous waste (by density) generated in Australia. These nine waste types make up approximately 92% of Australian hazardous waste generation.

Primary Hazardous Waste Generation in Australia - 2012

| - Y code or supplementary code | - Waste name | - Tonnes generated | - % of total hazardous waste generation |
| --- | --- | --- | --- |
| - Y18 | Residues arising from industrial waste disposal operations | 1,725,091 | - 26% |
| - 6 | Soils contaminated with residues of substances in Basel Y-codes 19-45 | 1,541,227 | - 23% |
| - 4 | Putrescible/ organic waste | 777,147 | - 12% |
| - Y36 | Asbestos (dust and fibres) | 477,257 | - 7% |
| - 8 | Tyres | 415,411 | - 6% |
| - Y9 | Waste oils/water, hydrocarbons/water mixtures, emulsion | 411,107 | - 6% |
| - Y8 | Waste mineral oils unfit for their originally intended use | 345,605 | - 5% |
| - Y35 | Basic solutions or bases in solid form | 335,371 | - 5% |
| - Y23 | Zinc compounds | 144,462 | - 2% |

- Source: Blue Environment 2012

# Hazardous Waste Pathways

This section provides an overview of hazardous waste identification, classification and pathways as summarised in Figure 1.

Figure 1 Waste Identification and Classification

## Waste Description and Classification

Currently, there is a number of different waste classification systems used in each Australian jurisdiction. In 2011 the Department of Sustainability, Environment, Water, Population and Communities commissioned Hyder to undertake a comparison between the National Waste Classification System (develop early 1990s) and the waste classification systems used in each of the Australian jurisdictions (Hyder report).

For the purpose of this report, the waste list from the National Environmental Protection (Movement of controlled waste between States and Territories) Measure (NEPM) has been adopted as the basis for the development of the hazardous waste pathways, as shown in Table 1 below.

Table 1 NEPM 75 Waste Categories and Codes

| Waste Code | Waste stream or wastes having as constituents: |
| --- | --- |
| A100 | Waste resulting from surface treatment of metals and plastics |
| A110 | Waste from heat treatment and tempering operations containing cyanides |
| A130 | Cyanides (inorganic) |
| B100 | Acidic solutions or acids in solid form |
| C100 | Basic solutions or bases in solid form |
| D100 | Metal carbonyls |
| D110 | Inorganic fluorine compounds excluding calcium fluoride |
| D120 | Mercury; mercury compounds |
| D130 | Arsenic; arsenic compounds |
| D140 | Chromium compounds (hexavalent and trivalent) |
| D150 | Cadmium; cadmium compounds |
| D160 | Beryllium; beryllium compounds |
| D170 | Antimony; antimony compounds |
| D180 | Thallium; thallium compounds |
| D190 | Copper compounds |
| D200 | Cobalt compounds |
| D210 | Nickel compounds |
| D220 | Lead; lead compounds |
| D230 | Zinc compounds |
| D240 | Selenium; selenium compounds |
| D250 | Tellurium, tellurium compounds |
| D270 | Vanadium compounds |
| D290 | Barium compounds (excluding barium sulphate) |
| D300 | Non-toxic salts |
| D310 | Boron compounds |
| D330 | Inorganic sulfides |
| D340 | Perchlorates |
| D350 | Chlorates |
| D360 | Phosphorus compounds excluding mineral phosphates |
| E100 | Waste containing peroxides other than hydrogen peroxide |
| F100 | Waste from the production, formulation and use of inks, dyes, pigments, paints, lacquers and varnish |
| F110 | Waste from the production, formulation and use of resins, latex, plasticisers, glues and adhesives |
| G100 | Ethers |
| G110 | Organic solvents excluding halogenated solvents |
| G150 | Halogenated organic solvents |
| G160 | Waste from the production, formulation and use of organic solvents |
| H100 | Waste from the production, formulation and use of biocides and phytopharmaceuticals |
| H110 | Organic phosphorus compounds |
| H170 | Waste from the manufacture, formulation and use of wood-preserving chemicals |
| J100 | Waste mineral oils unfit for their original intended use |
| J120 | Waste oil/water, hydrocarbons/water mixtures or emulsions |
| J160 | Waste tarry residues arising from refining, distillation, and any pyrolytic treatment |
| K100 | Animal effluent and residues (abattoir effluent, poultry and fish processing waste) |
| K110 | Grease trap waste |
| K130 | Sewage sludge and residues including nightsoil and septic tank sludge |
| K140 | Tannery wastes (including leather dust, ash, sludges and flours) |
| K190 | Wool scouring waste |
| M100 | Waste, substances and articles containing or contaminated with polychlorinated biphenyls (PCBs), polychlorinated naphthalenes (PCNs), polychlorinated terphenyls (PCTs) and/or polybrominated biphenyls (PBBs) |
| M150 | Phenols, phenol compounds including chlorophenols |
| M160 | Organohalogen compounds - other than substances referred to in this list |
| M170 | Polychlorinated dibenzo-furan (any congener) |
| M180 | Polychlorinated dibenzo-p-dioxin (any congener) |
| M210 | Cyanides (organic) |
| M220 | Isocyanate compounds |
| M230 | Triethylamine catalysts for setting foundry sands |
| M250 | Surface active agents (surfactants), containing principally organic constituents and which may contain metals and inorganic materials |
| M260 | Highly odorous organic chemicals (including mercaptans and acrylates) |
| N100 | Containers and drums that are contaminated with residues of waste referred to in this Table |
| N100 | Containers and drums that are contaminated with residues of substances referred to in this list |
| N120 | Soils contaminated with a substance or waste referred to in this Table |
| N120 | Soils contaminated with a substance or waste referred to in this Table |
| N140 | Fire debris and fire washwaters |
| N150 | Fly ash |
| N160 | Encapsulated, chemically-fixed, solidified or polymerised wastes |
| N190 | Filter cake |
| N205 | Residues from industrial waste treatment/disposal operations. |
| N220 | Asbestos |
| N230 | Ceramic-based fibres with physico-chemical characteristics similar to those of asbestos |
| R100 | Clinical and related wastes |
| R120 | Waste pharmaceuticals, drugs and medicines |
| R140 | Waste from the production and preparation of pharmaceutical products |
| T100 | Waste chemical substances arising from research and development or teaching activities including those which are not identified and/or are new and whose effects on human health and/or the environment are not known |
| T120 | Waste from the production, formulation and use of photographic chemicals and processing materials |
| T140 | Tyres |
| T200 | Waste of an explosive nature not subject to other legislation |

## Description of primary hazardous waste categories

There are 75 Waste classifications in Australia listed on the NEPM 75 list. GHD has broken these into 15 groups based on common chemical and treatment characteristics which are outlined below. Each waste group is summarised in terms of where the waste is generated, how it is commonly dealt with and other specific issues relating to management of the specific waste type.

### Cyanides

Cyanide is a widely-used industrial chemical with significant chemical properties. It is used in the mining sector as a leaching agent for removing gold and silver from ore, and is also used in industrial processes such as steel hardening/metal finishing, plastics production, and manufacture of goods such as adhesives, computer electronics, fire retardants, cosmetics, dyes, nylon, paints and pharmaceuticals.

Cyanide wastes rarely require disposal. In gold and silver mining, the contaminated wastewater (tailings) is retained in dams whilst the cyanide decomposes naturally by exposure to sunlight and air. It can also be destroyed by means of neutralisation and oxidation.

### Acids and Alkaline Bases

Acids and bases have a pH less than or equal to 2 or greater than or equal to 12.5 and are highly corrosive. Acids are commonly used in mining and heavy industry but also a broad range of other sectors such as medical and photographic use. The most common are sulphuric acid, nitric acid and hydrochloric acid (H2SO4, HNO3 and HCl). Alkaline waste waters occur in various kinds of construction, cleaning and industrial activities.

Acids and bases will often require neutralisation or treatment prior to re-use or disposal. Neutralisation and disposal to landfill, transport to a wastewater treatment plant, or on-site management, immobilisation/solidification, incineration. Acids and alkalis can be used for other waste treatments, for example neutralisation, precipitation and oil/water separation (emulsion breaking). Silver can be recovered from photographic chemicals.

### Inorganic Chemicals

Inorganic chemicals comprise batteries and a broad range of compounds commonly used in industry. They consist of compounds containing mineral or non-biological origins. Many inorganic chemicals have commercial value and can be recycled or recovered.

Current practices include on-site management through recycling and other forms of re-use at major industrial sites (for example hydro-metallurgical metal salts recovery and pyro-metallurgical metals recovery).

### Reactive Chemicals

Reactive chemical waste includes materials that are unstable or undergo rapid or violent chemical reaction when exposed to air, water or other material, generate toxic gases or vapours when mixed with water or when exposed to pH conditions between 2 and 12.5 (as in the case with cyanide or sulphide containing materials), forms potentially explosive mixtures with water, are capable of detonation or explosive reaction when heated or subjected to shock. Examples include acetyl chloride, chromic acid, cyanides, hypochlorides, organic peroxides, perchlorates, permanganates, sulphides, some plating materials and bleaches. Waste generators include Quarry and mine operations (explosives) and Industrial waste water treatment plants - organic peroxides used as chemical additives.

Under carefully controlled conditions, specialised disposal companies employ some waste oxidant streams (hypochlorite and peroxide) in the treatment of heavy metal wastes. They can also be safely disposed to sewer under controlled conditions.

### Paints, Lacquers, Varnish, Resins, Inks Adhesives

Paints, lacquers, varnish, resins, inks and adhesives comprise of a broad range of domestic and industrial wastes that contain a wide range of toxic and hazardous compounds. They require special handling management or disposal. There is a broad range of re-use, recycling and recovery options for these wastes in addition to disposal. Waste generators include domestic premises generating small quantities and in small containers. This would also include commercial operators such as painting contractors, carpet cleaners, dry-cleaners as well as manufacturing industries making paints and adhesives, and those using these materials

Collected contaminated containers are often left to dry out, and then cleaned (dry) and baled for scrap. Larger quantities of liquid residues are recovered into waste oil fuels. There are enterprising operators in the paints and coatings industry who produce low cost primer and undercoat by blending compatible paint residues.

### Organic Solvents, Solvents Residues

Organic solvents and solvent residues are generally toxic and highly flammable. They consist of a range of products used broadly in domestic, commercial and industrial applications such as paint thinners and dry cleaning fluids. They require special treatment or disposal to minimise contamination and/or health risks.

Organic solvents and solvent residues are either processed on-site by generators, stockpiled awaiting further management (such as re-use by blending into waste oils for use as fuel), or transported for treatment and recovery processes.

### Pesticides

Pesticides are commonly used by households, government and industry and include herbicides, fungicides, insecticides, fumigants, bactericides, rodenticides, baits, lures and repellents to control and/or destroy pests.

Care should be undertaken in the management and disposal of pesticides by rinsing containers (some containers can be re-used, refilled, returned or recycled). The ‘Drum-muster’ program operates successfully, requiring triple rinsing of pesticide/herbicide containers prior to depositing at a council collection point for crushing and recycling.

### Oils, Hydrocarbons, Emulsions

Oils, hydrocarbons and emulsions are common wastes from the automotive, commercial and industrial uses.

Most of this waste stream can be recovered and/or recycled to avoid the requirement for disposal. Most waste oils are converted to energy as fuel to kilns.

### Putrescible/Organic Wastes

Putrescible organic wastes consist of a broad range of organic waste material being predominantly animal waste, food waste, grease trap waste and sewage. This waste can cause serious health and environmental issues if not treated appropriately. Primary waste generators include food processors and sewage treatment plants. Commercial and domestic grease trap waste mostly water contaminated with oils and fats, and organic solids is also produced.

Most of this is waste is sent to sewage treatment plants or to landfill and some waste is used for land farming, irrigation or spreading.

### Organic Chemicals

Organic chemicals consist of a broad range of organic compounds that contain carbon. This waste is generated by electrical equipment and the mining sector containing PCBs, electricity generators and distributors. It consists of harmful chemicals derived from industrial processes.

Specific treatment processes are required to treat much of this waste stream due to the toxicity and danger to health from exposure. A number of specialist treatment facilities exist to safely treat these wastes.

### Solids/Sludges Requiring Special Handling

Solids and sludges requiring special handling consist of a broad range of wastes including contaminated soils, asbestos, contaminated containers, fly ash and filter cake. Contamination and toxicity levels vary significantly and there are a broad range of treatment options.

Based on the characteristics of waste materials they can be sent to landfill

### Clinical and Pharmaceutical Wastes

Materials generated by the health care industry and other clinical settings, which have the potential to cause infection, injury or public offence e.g. anatomical waste and body fluids, sharps, cytotoxics, waste pharmaceuticals and chemical waste.

Clinical waste is sent to landfill, liquid wastes such as body fluids and cytotoxic wastes and body parts, are handled appropriately by discharge to sewer, transfer to destruction facilities, and cremation. Opportunities for re-use and resource recovery of clinical and related waste are particularly limited, due to the potential for infection.

### Miscellaneous

Miscellaneous wastes comprise a range of chemical wastes derived from various activities including industrial or treatment processes. These wastes require special handling and treatment.

### Tyres

Waste tyres are generated predominantly from domestic vehicle use, road transport, mining and heavy industry. Approximately 48 million end of life tyres require management and disposal in Australia each year.

Waste tyres in Australia should be sent to appropriately licensed recycling or disposal facilities. Only a small percentage (10-20%) of tyres is recycled each year, most are sent to landfill, stockpiled or illegally dumped.

## Waste Tracking

Most hazardous waste must be monitored from its production to its final disposal. Producers, waste collectors, transporters, treatment and disposal facilities must comply with data management and tracking of waste in accordance with Schedule B of the NPEM, which supports national consistency in tracking controlled waste. The NEPM is designed to ensure that controlled wastes movements between states and territories are properly identified, transported and handled, in an environmentally sound manner and that they reach a licensed or approved facility for treatment, recycling, storage and/or disposal. Figure 2 below summaries the waste tracking procedure for hazardous waste.

Figure 2. Waste Tracking Procedure

Each participating State or Territory should ensure that all controlled wastes transported in accordance with this Measure are accompanied by the information set out in part 1 and part 2 of Schedule B, and that part 3 of Schedule B is completed by a facility upon acceptance of the waste

Schedule B consists of three parts:

Part 1	Information to be provided by producers:

Description of the waste(s) [Use proper shipping name/technical name if applicable for Dangerous Goods]

The physical nature of the waste

Waste code(s)

Contaminant(s)

UN Number(s)

UN Code(s)

Dangerous Goods Class(es) (UN Class(es)) [and Subsidiary Risk if applicable for Dangerous Goods]

Packaging Group number

Amount of waste(s)

Waste origin code (ANZ Standard Industry Code)

Type of package (eg bulk) [and number of packages of each type if applicable for Dangerous Goods]

Facility name

Facility address

Facility licence number

State/Territory of destination

Name of waste producer

Address of waste source

Producer’s telephone number

Emergency contact number in the event of accident or spillage

Consignment authorisation number

Producer identification number

Date of dispatch.

Part 2	Information to be provided by transporters:

Name of transporter(s)

Address of transporter(s)

Vehicle registration number(s)

Name(s) of transit State(s)/Territory or Territories

Transport licence number(s)

Date of transport

Type of transport eg train, truck.

Part 3	Information to be provided by facilities:

Type of treatment at facility

Date of receipt at facility

Any discrepancies noted in information provided in parts 1 and 2 Schedule B should be reported as required by the relevant agency in the jurisdiction in which the facility is located.

## Hazardous Waste Treatment Processes

The European Union has established a legislative framework for the handling of waste. The Waste Framework Directive (WFD) or Directive defines waste, recovery and disposal treatments, and provides guidelines for the management of waste.

Treatment processes and codes for the report have been based on the EU Directive Annex IIA and Annex IIB.

EU Waste Directive Treatment Codes - Annex IIA

This Annex is intended to list disposal operations such as they occur in practice. In accordance with Article 4, waste must be disposed of without endangering human health and without the use of processes or methods likely to harm the environment.

Table 2 EU Waste Directive Disposal Treatment

| Treatment Code | Waste Treatment |
| --- | --- |
| D1 | Deposit into or on to land (eg landfill, etc.) |
| D2 | Land treatment (eg biodegradation of liquid or sludgy discards in soils, etc.) |
| D3 | Deep injection (eg injection of pumpable discards into wells, salt domes or naturally occurring repositories, etc.) |
| D4 | Surface impoundment (e.g. placement of liquid or sludgy discards into pits, ponds or lagoons, etc.) |
| D5 | Specially engineered landfill (eg placement into lined discrete cells which are capped and isolated from one another and the environment, etc.) |
| D6 | Release into a water body except seas/oceans |
| D7 | Release into seas/oceans including sea‑bed insertion |
| D8 | Biological treatment not specified elsewhere in this Annex which results in final compounds or mixtures which are discarded by means of any of the operations numbered D1 to D7 and D9 to D12 |
| D9 | Physico-chemical treatment not specified elsewhere in this Annex which results in final compounds or mixtures which are discarded by means of any of the operations numbered D 1 to D 8 and D 10 to D 12 (eg evaporation, drying, calcination, etc.) |
| D10 | Incineration on land |
| D11 | Incineration at sea |
| D12 | Permanent storage (e.g. emplacement of containers in a mine, etc.) |
| D13 | Blending or mixing prior to submission to any of the operations numbered D1 to D12 |
| D14 | Repackaging prior to submission to any of the operations numbered D1 to D13 |
| D15 | Storage pending any of the operations numbered D 1 to D 14 (excluding temporary storage, pending collection, on the site where it is produced) |

EU Waste Directive Treatment Codes - Annex IIB

This Annex is intended to list recovery operations as they occur in practice. In accordance with Article 4, waste must be recovered without endangering human health and without the use of processes or methods likely to harm the environment.

Table 3 EU Waste Directive Recovery Treatment

| Treatment Code | Waste Treatment |
| --- | --- |
| R1 | Use principally as a fuel or other means to generate energy |
| R2 | Solvent reclamation/regeneration |
| R3 | Recycling/reclamation of organic substances which are not used as solvents (including composting and other biological transformation processes) |
| R4 | Recycling/reclamation of metals and metal compounds |
| R5 | Recycling/reclamation of other inorganic materials |
| R6 | Regeneration of acids or bases |
| R7 | Recovery of components used for pollution abatement |
| R8 | Recovery of components from catalysts |
| R9 | Oil re-refining or other reuses of oil |
| R10 | Land treatment resulting in benefit to agriculture or ecological improvement |
| R11 | Use of wastes obtained from any of the operations numbered R 1 to R 10 |
| R12 | Exchange of wastes for submission to any of the operations numbered R 1 to R 11 |
| R13 | Storage of wastes pending any of the operations numbered R 1 to R 12 (excluding temporary storage, pending collection, on the site where it is produced) |

### Disposal Processes

D1: Deposit into or onto land

Waste is permanently stored on authorised landfills, compacted and finally covered with top soil or impermeable material cover.

D2: Land treatment

Land treatment consists of biodegradation of waste by spreading it on land, often followed by the incorporation of the waste into the soil. Land treatment is generally applied to non-hazardous biodegradation of liquid or sludgy wastes, e.g. disposal of dredging sludge.

D3: Deep Injection

Injection of pumpable discards into natural or artificial cavities (wells, salt domes) and porous formations of rock.

D4: Surface impoundment

The deposit of liquid or sludgy waste in natural or engineered ponds, pits or lagoons, e.g. management of tailings in mining operations; impoundment of dredging sludge, etc.

D5: Specially engineered landfill

Placement into lined discrete cells which are capped and isolated from one another and the environment. Hazardous waste requires landfilling to avoid in-ground leachate.

D6: Release into a water body except seas/oceans

Deposit of non-hazardous dredging sludge and other non-hazardous sludge in surface water including the bed and the subsoil.

D7: Release into seas/oceans including sea bed insertion

For example, discharge of fish processing waste and inert materials of natural origin.

D8: Biological treatment

Biological treatment includes activated sludge, aerated lagoons, composting, aerobic and anaerobic digestion. They are applied for the treatment of municipal waste, contaminated soil, sludge or mineral wastes, if followed by disposal.

D9: Physico-Chemical Treatments

Physico-chemical treatments are used in practice in a very broad sense to treat liquid, sludgy and solid wastes, such as neutral aqueous inorganics, acids, alkalis, oils, contaminated containers, cyanides, organic sludges, reactive chemicals.

Physico-chemical processes include: dewatering, filtration, precipitation, sedimentation, acid neutralization, alkali treatment, chromic acid treatment, distillation, immobilisation and stabilisation.

D10: Incineration on Land

Combustion of organic substances contained waste materials to be converted into ash, flue gas and heat. Incinerators are used to burn hazardous waste primarily for waste destruction/treatment purposes and for potential energy or material recovery. Incineration destroys toxic organic components in hazardous waste and reduces the volume of the waste. Incineration is not suitable for metal-bearing hazardous wastes, as metals will not combust. Types of waste incinerators include: rotary kilns, fluidized bed units among others.

D11: Incineration at Sea

This operation is prohibited by international conventions.

D12: Permanent Storage

Permanent storage includes emplacement of containers in a mine, landfills for the underground storage of waste.

D13: Blending or Mixing

Basic sorting activities prior to submission to any of the operations numbered D1 to D12; such as. crushing and shredding of waste in order to reduce the volume for transport or land filling. They are also used to achieve certain waste physical and chemical characteristics such as homogenisation, conditioning and solidification.

D14: Repackaging

Transfer and compaction of waste prior to submission to any of the operations numbered D 1 to D 13. Due to the disaggregated nature of some types of waste, it is sometimes necessary to compact them to make them easier to use in the following process. Pressure machinery is used to pack the waste into a certain physical form.

D15: Storage

Temporary storage pending any of the operations numbered D 1 to D 14. Does not apply to storage of waste prior collection at the site where it is produced.

### Recovery Processes

R1: Waste Recovery as a Fuel

Liquid waste fuel can be produced from hazardous wastes such as waste oils, waste solvents not suitable for regeneration, and distillation bottoms (residue from distillation), and tyres. Waste oils normally need pre-treatment to remove sediments and water. Examples of combustion plants that may use waste as fuel are the combustion plants for the marine engines, cement kilns, blast furnaces in iron and steel production, brick kilns in the production of ceramics, lime kilns and asphalt production.

R2: Solvent Reclamation/Regeneration

Solvent regeneration facilities separate contaminants from waste solvents to regenerate the solvent to its original quality or to a lower grade product. The processes used for solvent recovery are distillation, filtration, evaporation, centrifugation and stripping.

R3: Recycling/reclamation

Recycling or reclamation of organic wastes which are not used as solvents, e.g. recycling of waste paper and board, reprocessing and recycling of plastic waste, composting of biowaste and green waste, fermentation of biodegradable waste for biogas production.

R4: Recycling/reclamation of metals and metal compounds

Recycling of scrap and production waste in steelworks; shredding and reprocessing, thermal treatment of cables or oil-contaminated metals; battery recycling; and electrolytic recovery of silver from photo chemicals.

R5: Recycling/reclamation of other inorganic materials

Reprocessing of construction and demolition waste; reprocessing and recycling of glass waste; use as secondary raw material in cement kilns; asphalt mixing plants; and use for underground stowage in mines.

R6: Regeneration of acids or bases

Re-concentration of spent acids; and the thermal decomposition of spent sulphuric acid for use as feedstock in sulphuric acid production.

R7: Recovery of components used for pollution abatement

Regeneration of activated carbon from water purification and flue gas treatment, mainly by thermal treatment; and the regeneration of resins by solvent washing.

R8: Recovery of components from catalysts

Regeneration of catalysts to be reused as catalysts; and the recovery of catalyst components, mainly of metal components, e.g. recycling of precious metals from catalytic converters in vehicle exhausts.

R9: Oil re-refining or other reuses of oil

Re-refining into base oils which can be used to manufacture lubricating products; use to generate fuel which can be used as a substitute for coal, diesel and light fuel.

R10: Landspreading

Land treatment resulting in benefit to agriculture or ecological improvement. Use of sewage sludge; the spreading on land of compost from the treatment of separately collected bio waste; the use of and mineral wastes as fertilisers in compliance with national legislation; landscape restoration, e.g. as final landfill cover; restoration of old disused quarries.

R11: Use of residuals

Use of wastes obtained from any of the operations numbered R 1 to R 10.

Energy recovery of sorting residues; shredder light fraction; distillation sludge from oil-refining; and the use of slag from co-incineration for underground stowage.

R12: Exchange of wastes for submission to any of the operations numbered R 1 to R 11

Basic sorting activities; mixing of waste from different generators before it is sent to a recovery facility; transfer and compaction of waste; shredding of wood waste prior to energy recovery.

R13: Storage of wastes pending any of the operations numbered R 1 to R 12 (excluding temporary storage, pending collection, on the site where it is produced)

Interim storage of waste prior to recovery.

## Hazardous Waste Treatment in Australia

Hazardous waste classification and treatment options are shown in Appendix A. The most commonly used disposal treatments in Australia are physico-chemical treatments (D9), repackaging or mixing (D14) and storage (D15). These are pre-treatment operations, which must be followed by one of the recovery or disposal operations, e.g landfill (D1).

Recovery processes such as waste recovery as a fuel (R1), solvent reclamation/regeneration (R2), recycling/reclamation of organic waste (R3) are the main recovery treatment options practiced in Australia.

# Waste Treatment Guidelines

As there are no current guidelines for waste treatment in Australia other than those applied by specific state and territories, we have based the Guidelines for waste treatment operations on the European Commission Reference Document on Best Available Techniques (BAT) for the Waste Treatment Industries (BREF).

This section provides BAT guidelines for the pre-treatment and post-treatment processes and facilities. The following steps should be followed by waste treatment plants: acceptance, storage, treatment, storage of residues and emissions.

## Pre-acceptance Procedures

Pre-acceptance testing prior to waste transport to the facility is required to determine the type of waste and check that the facility is licenced to appropriately treat or dispose of the waste. Waste identification or samples for testing need to be provided before the transport of waste to the facility.

## Acceptance Procedures

After completing all pre-acceptance testing the waste is then transported to the facility. Hazardous wastes must be tracked and controlled waste carriers must hold the appropriate licences. The acceptance procedures are defined by the treatment facilities and may include physical inspection when they arrive at the site, identification, sampling and analysis and definition of treatment programme.

## Storage and Handling Procedures

Storing liquid hazardous waste requires extra care. It should be stored under cover and in a bunded and secure area that contains any leaks or spills and prevents wastes from coming in contact with the ground or escaping to the environment via stormwater drains or gutters. Liquids may be stored in tanks and/or containers (e.g. glass containers, drums, big containers), storage cells, storage buildings and outside storage (e.g. waste waters).

The basic Australian Standards’ requirements for bunding of hazardous liquid wastes are:

Bunds should be impervious and preferentially constructed of concrete, bricks or stones

Bunds should be designed to contain 110% of the volume of the largest vessel within the bund

Vessels should be at least 1 m from the bund walls

Bund walls should preferably be between 0.5 to 1.5 m in height.

Hazardous waste must be assessed and classified before transferring to a treatment or disposal facility. When sending hazardous waste for treatment or disposal, the following must take place:

Follow waste tracking rules

The transporter is appropriately licensed

The waste is being sent to and accepted by an approved or licensed.

Storage and handling requirements of hazardous waste are outlined in the Australian Dangerous Goods Code and in the following Australian Standards:

AS/NZS 3833-2007: The storage and handling of mixed classes of dangerous goods.

AS 1940-2004: The storage and handling of flammable and combustible liquids

AS 3780-2008: The storage and handling of corrosive substances

AS 4326-2008: The storage and handling of oxidising agents.

# Hazardous Waste Export

Hazardous wastes can be exported from Australia but require a Hazardous Waste Export permit from the Australian Commonwealth Government.

Hazardous waste can be exported under the Basel Convention and the OECD control system.

Generally, a hazardous waste export license will only be granted if the waste that is being exported is for recovery or recycling. In exceptional circumstances, hazardous waste may be exported for disposal, if no licensed disposal options exist in Australia and specific criteria are met.

Primary waste exported from Australia for recycling or recovery include:

Brass or Lead Dross

Spent Catalyst

Nickel-Cadmium Batteries

Aluminium Ashes and Residues

Used Lead Acid Batteries (ULABs)

Electronic Scrap

Zinc Ashes and Residues

# References

- Environment Protection Authority Victoria website. www.epa.vic.gov.au

- The National Environment Protection (Movement of Controlled Waste between States and Territories) Measure 2004 (NEPM)

- European Union Hazardous Waste Framework Directive (Directive 2006/13/EC) Annex IIA and Annex IIB

- Blue Environment 2012

- European Commission Reference Document on Best Available Techniques (BAT) for the Waste Treatment Industries (BREF)

Appendix A - Hazardous Waste Classification and Treatment Options in Australia

GHD 133 Castlereagh St Sydney NSW 2000 - T: +61 2 9239 7100 F: +61 2 9239 7199 E: sydmail@ghd.com.au © GHD 2014 This document is and shall remain the property of GHD. The document may only be used for the purpose for which it was commissioned and in accordance with the Terms of Engagement for the commission. Unauthorised use of this document in any form whatsoever is prohibited. G:\21\23557\WP\199835.docx Document Status Rev No.; Author; Reviewer; Approved for Issue. Name; Signature; Name; Signature; Date. 0; M. Valencia – S.Thompson; D. Kovac; S. Thompson; 30/06/14.

www.ghd.com

ANZSIC classification has four levels.