# Unit conversion factors

3 July 2017

PREPARED FOR

Department of the Environment and Energy

PREPARED IN ASSOCIATION WITH

This document contains default unit conversion factors for converting Australian hazardous waste data from cubic metres or numbers of items into tonnes. Factors are provided for waste categories as set out in the National Environment Protection (Movement of Controlled Waste between States and Territories) Measure.

The factors were first used in Hazardous Waste in Australia 2017 and have been incorporated into the Australian Hazardous Waste Data and Reporting Standard 2017 (the standard). The standard is designed to diminish the differences between regulatory systems, reduce costs and provide more certainty for regulators and businesses, and help compile national data in a consistent manner.

The method used to develop the factors was:

Compiled tracking system data for 2010-11 were partitioned into waste form: liquid, sludge, solid and occasionally mixtures for each waste.

For each waste (and the most predominant waste form), a density figure was estimated based on:

EPA Victoria’s ‘Waste Materials – Density Data’

NSW’s ‘Online waste tracking system - conversion factors’ (not published)

Merck Index chemical/ physical properties (where there is some confidence in the chemistry and concentration of the waste)

MSDS information

Ascend’s The Health and Environmental Impacts of Hazardous Wastes report (prepared for the Department) and

logic relating to knowledge of the wastes and their sources.

For those wastes that may be reported in units, such as tyres and drums, the supplied figure was a unit-to-mass conversion, as well as a volume-to-mass conversion.

Where no reliable information was found for density estimation a default density of 1 t/m3 was assumed.

To comply with the standard, states and territories and the Australian Government should use the densities and unit conversion factors provided below to convert hazardous waste data to a consistent tonnage basis.

Unit conversion factors

|  |  |  | Waste description (NEPM Schedule A, List 1) | Waste density (t/m3) | Conversion factor (t/unit) |
| --- | --- | --- | --- | --- | --- |
| A | Plating and heat treatment | A100 | Waste resulting from surface treatment of metals & plastics | 1.5 |  |
|  |  | A110 | Waste from heat treatment & tempering operations containing cyanides | 2.0 |  |
|  |  | A130 | Cyanides (inorganic) | 1.2 |  |
| B | Acids | B100 | Acidic solutions or acids in solid form | 1.2 |  |
| C | Alkalis | C100 | Basic solutions or bases in solid form | 1.3 |  |
| D | Inorganic chemicals | D100 | Metal carbonyls | 1.0 |  |
|  |  | D110 | Inorganic fluorine compounds excluding calcium fluoride | 1.4 |  |
|  |  | D120 | Mercury; mercury compounds | 0.3 |  |
|  |  | D130 | Arsenic; arsenic compounds | 1.7 |  |
|  |  | D140 | Chromium compounds (hexavalent & trivalent) | 1.9 |  |
|  |  | D150 | Cadmium; cadmium compounds | 1.0 |  |
|  |  | D160 | Beryllium; beryllium compounds | 4.1 |  |
|  |  | D170 | Antimony; antimony compounds | 1.0 |  |
|  |  | D180 | Thallium; thallium compounds | 1.0 |  |
|  |  | D190 | Copper compounds | 1.8 |  |
|  |  | D200 | Cobalt compounds | 1.0 |  |
|  |  | D210 | Nickel compounds | 1.0 |  |
|  |  | D220 | Lead; lead compounds | 7.5 |  |
|  |  | D230 | Zinc compounds | 1.8 |  |
|  |  | D240 | Selenium; selenium compounds | 1.0 |  |
|  |  | D250 | Tellurium; tellurium compounds | 1.0 |  |
|  |  | D270 | Vanadium compounds | 1.0 |  |
|  |  | D290 | Barium compounds (excluding barium sulphate) | 1.0 |  |
|  |  | D300 | Non-toxic salts | 1.2 |  |
|  |  | D310 | Boron compounds | 1.0 |  |
|  |  | D330 | Inorganic sulfides | 0.8 |  |
|  |  | D340 | Perchlorates | 1.0 |  |
|  |  | D350 | Chlorates | 1.0 |  |
|  |  | D360 | Phosphorus compounds excluding mineral phosphates | 1.0 |  |
| E | Reactive chemicals | E100 | Waste containing peroxides other than hydrogen peroxide | 1.0 |  |
| F | Paints, resins, inks, organic sludges | F100 | Waste from production, formulation & use of inks, dyes, pigments, paints, lacquers & varnish | 1.3 |  |
|  |  | F110 | Waste from the production, formulation & use of resins, latex, plasticisers, glues & adhesives | 1.3 |  |
| G | Organic solvents | G100 | Ethers | 0.7 |  |
|  |  | G110 | Organic solvents excluding halogenated solvents | 0.9 |  |
|  |  | G150 | Halogenated organic solvents | 1.5 |  |
|  |  | G160 | Waste from the production, formulation & use of organic solvents | 1.0 |  |
| H | Pesticides | H100 | Waste from the production, formulation & use of biocides & phytopharmaceuticals | 1.0 |  |
|  |  | H110 | Organic phosphorous compounds | 1.0 |  |
|  |  | H170 | Waste from manufacture, formulation & use of wood-preserving chemicals | 1.2 |  |
| J | Oils | J100 | Waste mineral oils unfit for their original intended use | 0.9 |  |
|  |  | J120 | Waste oil/water, hydrocarbons/water mixtures or emulsions | 1.0 |  |
|  |  | J160 | Waste tarry residues arising from refining, distillation, & any pyrolytic treatment | 1.2 |  |
| K | Putrescible/ organic waste | K100 | Animal effluent & residues (abattoir effluent, poultry & fish processing wastes) | 0.9 |  |
|  |  | K110 | Grease trap waste | 0.9 |  |
|  |  | K140 | Tannery wastes (incl. leather dust, ash, sludges & flours) | 1.0 |  |
|  |  | K190 | Wool scouring wastes | 1.0 |  |
| M | Organic chemicals | M100 | Waste substances & articles containing or contaminated with polychlorinated biphenyls, polychlorinated naphthalenes, polychlorinated terphenyls and/or polybrominated biphenyls | 1.0 |  |
|  |  | M150 | Phenols, phenol compounds including chlorophenols | 1.2 |  |
|  |  | M160 | Organohalogen compounds—other than substances referred to in this Table | 1.0 |  |
|  |  | M170 | Polychlorinated dibenzo-furan (any congener) | 1.0 |  |
|  |  | M180 | Polychlorinated dibenzo-p-dioxin (any congener) | 1.0 |  |
|  |  | M210 | Cyanides (organic) | 1.0 |  |
|  |  | M220 | Isocyanate compounds | 1.0 |  |
|  |  | M230 | Triethylamine catalysts for setting foundry sands | 1.0 |  |
|  |  | M250 | Surface active agents (surfactants), containing principally organic constituents & which may contain metals & inorganic materials | 1.0 |  |
|  |  | M260 | Highly odorous organic chemicals (including mercaptans & acrylates) | 1.0 |  |
| N | Soil/ sludge | N100 | Containers & drums that are contaminated with residues of substances referred to in this list | 0.1 | 0.018 |
|  |  | N120 | Soils contaminated with a controlled waste | 0.9 |  |
|  |  | N140 | Fire debris & fire wash waters | 1.0 |  |
|  |  | N150 | Fly ash, excluding fly ash generated from Australian coal fired power stations | 1.7 |  |
|  |  | N160 | Encapsulated, chemically-fixed, solidified or polymerised wastes referred to in this list | 0.8 |  |
|  |  | N190 | Filter cake contaminated with residues of substances referred to in this list | 1.0 |  |
|  |  | N205 | Residues from industrial waste treatment/disposal operations | 0.7 |  |
|  |  | N220 | Asbestos | 0.8 |  |
|  |  | N230 | Ceramic-based fibres with physico-chemical characteristics similar to those of asbestos | 0.8 |  |
| R | Clinical and pharmaceutical | R100 | Clinical & related wastes | 0.2 |  |
|  |  | R120 | Waste pharmaceuticals, drugs & medicines | 0.3 |  |
|  |  | R140 | Waste from the production & preparation of pharmaceutical products | 1.0 |  |
| T | Miscellaneous | T100 | Waste chemical substances arising from research & development or teaching activities, including those which are not identified and/or are new & whose effects on human health and/or the environment are not known | 1.0 |  |
|  |  | T120 | Waste from the production, formulation & use of photographic chemicals & processing materials | 1.0 |  |
|  |  | T140 | Tyres | 0.3 | 0.008 |
|  |  | T200 | Waste of an explosive nature not subject to other legislation | 1.0 |  |
| Other |  | Other |  | 1.0 |  |