internal  report

561

Results of a gamma dose rate survey at the rehabilitated Sleisbeck mine, Northern Territory, Australia

Bollhöfer A & Fawcett M

April 2009

(Release status - unrestricted)

# Results of a gamma dose rate survey at the rehabilitated Sleisbeck mine, Northern Territory, Australia

A Bollhöfer & M Fawcett1

1 Fawcett Mine Rehabilitation Services Pty Ltd, Darwin NT

Supervising Scientist Division

GPO Box 461, Darwin NT 0801

April 2009

Registry File SG2004/0169

(Release status – unrestricted)

How to cite this report:

Bollhöfer A & Fawcett M 2009. Results of a gamma dose rate survey at the rehabilitated Sleisbeck mine, Northern Territory, Australia. Internal Report 561, April, Supervising Scientist, Darwin. Unpublished paper.

Location of final PDF file in SSD Explorer

\Publications Work\Publications and other productions\Internal Reports (IRs)\Nos 500 to 599\IR561_Sleisbeck gamma survey post rehab (Bollhöfer & Fawcett)\IR561_Results gamma dose rate survey Sleisbeck mine (Bollhöfer & Fawcett).pdf

Authors of this report:

Andreas Bollhöfer – Environmental Research Institute of the Supervising Scientist, GPO Box 461, Darwin NT 0801, Australia

Mike Fawcett – Fawcett Mine Rehabilitation Services Pty Ltd, PO Box 781, Howard Springs, NT 0835 Australia

The Supervising Scientist is part of the Australian Government Department of the Environment, Water, Heritage and the Arts.

© Commonwealth of  Australia 2009

Supervising Scientist

Department of the Environment, Water, Heritage and the Arts

GPO Box 461, Darwin NT 0801 Australia

This work is copyright. Apart from any use as permitted under the Copyright Act 1968, no part may be reproduced by any process without prior written permission from the Supervising Scientist. Requests and inquiries concerning reproduction and rights should be addressed to Publications Inquiries, Supervising Scientist, GPO Box 461, Darwin NT 0801.

e-mail: publications_ssd@environment.gov.au

Internet: www.environment.gov.au/ssd   (www.environment.gov.au/ssd/publications)

The views and opinions expressed in this report do not necessarily reflect those of the Commonwealth of Australia. While reasonable efforts have been made to ensure that the contents of this report are factually correct, some essential data rely on the references cited and the Supervising Scientist and the Commonwealth of Australia do not accept responsibility for the accuracy, currency or completeness of the contents of this report, and shall not be liable for any loss or damage that may be occasioned directly or indirectly through the use of, or reliance on, the report. Readers should exercise their own skill and judgment with respect to their use of the material contained in this report.

Printed and bound in Darwin NT by Supervising Scientist Division

# Contents

Executive summary	iv

1  Introduction	1

2  Methods	6

3  Results	7

4  Conclusion	9

References	10

Appendix A1  Eastings, northings, counts and comments recorded on the fieldsheets	11

Appendix A2  Results of the gamma dose rate measurements	19

## Figures

Figure 1  Location of the Sleisbeck mine in Kakadu National Park	1

Figure 2  The pumped out Sleisbeck pit	3

Figure 3  Sleisbeck pit during backfilling	3

Figure 4  Placing of the low radioactivity cover layer over the pit	4

Figure 5  Rippig the footprint of the former truck dumps	4

Figure 6  View of the Sleisbeck open pit before and after rehabilitation	5

Figure 7  Aerial view of the rehabilitated Sleisbeck pit and the ripped truck dump area	5

Figure 8  Response of the Mini Instruments Environmental survey monitor Model 6–80/MC–71 digital scalar to 137Cs, 0-20 μGyh-1 range – April 2008	7

Figure 9  Response of the Mini Instruments Environmental survey monitor Model 6–80/MC–71 digital scalar to 137Cs, 0-2 μGyh-1 range – April 2008	7

Figure 10  Gamma dose rates [µGy∙hr-1] measured at the rehabilitated Sleisbeck mine overlaid on Quickbird data acquired in August 2003 (before mine rehabilitation)	8

Figure 11  Histogram and probability plot of terrestrial gamma dose rates at Sleisbeck post rehabilitation	9

## Executive summary

This report summarises the results of a gamma dose rate survey conducted at the historic Sleisbeck mine in the Gunlom Land Trust in July 2008, after rehabilitation of the site was completed. 360 dose rate reading were taken over an approximately 7.6 ha area. The median terrestrial gamma dose rate measured across the area surveyed is 0.13 µGy∙hr-1. The arithmetic average is 0.23 µGy∙hr-1. The geometric mean, or ‘typical value’, is 0.14 µGy∙hr-1, similar to environmental background gamma dose rates measured in the area.

The remediation works have reduced the average terrestrial gamma dose rates across the Sleisbeck area by about threefold compared to the average pre rehabilitation dose rates in the area. Ninety-nine percent of the area surveyed exhibit terrestrial gamma dose rates below 1.25 µGy∙hr-1. This has resulted in a reduction of the annual doses received by the public accessing the area. Assuming that the area was occupied for one month during the dry season, effective doses from exposure to terrestrial gamma radiation are well below the annual dose constraint for the public for existing exposure situations of 10 mSv and lower than the 0.3 mSv dose constraint recommended in current ICRP (2007) guidelines for prolonged exposure from planned exposure situations.

# Results of a gamma dose rate survey at the rehabilitated Sleisbeck mine, Northern Territory, Australia

A Bollhöfer & M Fawcett

## 1  Introduction

The old Sleisbeck mine was one of the many small abandoned uranium mine sites located in the South of the World Heritage listed Kakadu National Park, Northern Territory, Australia. It is located approximately 32 km south east of Guratba, or Coronation Hill, in the Katherine River catchment (Figure 1). It was one of the first uranium deposits discovered in the area by George Sleis, a Czechoslovakian geologist, in 1954 (Annabell 1977). The deposit was worked in 1957, but only ~600 tonnes of uranium ore were mined at Sleisbeck and 3 tonnes of U3O8 were produced (at Rum Jungle).

Figure 1  Location of the Sleisbeck mine in Kakadu National Park

The mine workings consisted of an open pit, approximately 100 m x 30 m in dimensions and three major waste rock dumps (‘truck dumps’) approximately 100 m to the south of the pit, which contained overburden and waste rock with low grade uranium mineralisation (Pfitzner et al 2003). When the deposit proved un-economic the pit, waste rock and mineralised material dumped to the south of the open pit, and the Sleisbeck camp, approximately 3 km west of the pit, were abandoned without any substantial effort to clean up or remediate the area.

In 1996 land granted to the Gunlom Aboriginal Land Trust, including the area of the old Sleisbeck mine, was leased to the Director of National Parks to be managed as part of Kakadu National Park. The lease agreement requires the Director to implement a plan of environmental rehabilitation of Guratba (Coronation Hill) and other mine sites and associated workings in the area so as to limit and where possible reverse the impact on the environment of any mining activities previously carried out (Supervising Scientist, 2008).

In 2002, an airborne gamma survey was flown of the area around Sleisbeck, as part of a program to characterise the extent and magnitude of radiological contamination in the Gunlom Land Trust area (Pfitzner et al 2003). This survey identified the truck dumps as the main source of the gamma signal measured aboard the plane. Ground truthing was conducted in June 2003 and July 2006 (Bollhöfer et al 2007), and found the areal extent of contamination to be ~1.2 ha. Maximum external gamma dose rates determined from the ground-truthed airborne gamma survey data were ~2.3 μGy∙hr-1 across a 7 m x 7 m area (Bollhöfer et al 2008). However, there were localised spots within the area where terrestrial gamma dose rates of above 5 μGy∙hr-1 were measured (Bollhöfer et al 2007). Trenches were also dug into the waste rock dumps, and the average height of mineralised material that was dumped on the topsoil was determined to be approximately 0.7 m. It was estimated that a total of ~8600 m3 of material needed to be removed to clean up the area and that this would result in a reduction of external gamma dose rates to less than 0.3 μGy∙hr-1 (Bollhöfer et al 2008).

In May 2006, the Australian Government allocated $7.3 million over four years to the Director of National Parks to rehabilitate Guratba (Coronation Hill) and other mine sites and associated workings in the area. The rehabilitation program is being managed by Parks Australia, on behalf of the Director, and SSD is providing specialist assistance with the radiological assessment (Supervising Scientist, 2008). As part of the remediation and general clean up of the abandoned mine sites in the South Alligator River Valley, the rehabilitation of the former Sleisbeck mine site started in the dry season of 2007 and was finalised in December 2007.

The main works undertaken at Sleisbeck mine were in three phases (Waggitt & Fawcett 2008):

- Upgrading of the access track between Guratba and Sleisbeck.

- Pumping the water out (Figure 2) and backfilling of the pit with material from the truck dumps followed by placement of a cover layer of clean material sourced from nearby costean spoil piles and disused former tracks.

- Backfilling of selected costeans in the vicinity of the Sleisbeck pit that presented a potential safety hazard to humans and/or fauna.

The waste rock and low grade material from the truck dumps to the South of the pit was removed and dumped into the pit, and shaped to cover a mineralised area in the pit wall that exhibited very high external gamma dose rates of up to 38 μGy∙hr-1 (Tims & Ryan 1998) (Figure 3). Before remediation this hot spot area in the pit wall was only accessible by boat. The clean (ie background radioactivity) material for placement over the truck dump material was sourced from spoil piles located immediately to the east of the Sleisbeck pit. This material was spread across the backfilled pit surface in a single layer to a nominal depth of 700 mm (Figure 4). The second source of cover material was a disused track to the north-east of the pit. This provided material for the upper 300 mm layer (Waggitt & Fawcett 2008).

Figure 2  The pumped out Sleisbeck pit (photo: M Fawcett)

Figure 3  Sleisbeck pit during backfilling (photo: M Fawcett)

Once all the waste rock had been recovered from the old truck dumps, the footprint of each area was ripped to a depth of ~ 300 mm in order to break up compaction and provide a moisture retaining seedbed for subsequent revegetation (Figure 5). Revegetation of the site was undertaken in December 2007 using local provenance seed to the greatest extent possible (Waggitt & Fawcett 2008). Figure 6 shows a comparison view of the Sleisbeck pit before and after rehabilitation. Figure 7 shows an aerial view of the Sleisbeck pit area after rehabilitation.

Figure 4  Placing of the low radioactivity cover layer over the pit (photo: M Fawcett)

Figure 5  Ripping the footprint of the former truck dumps (photo: M Fawcett)

Figure 6  View of the Sleisbeck open pit before (left, photo by A Bollhöfer) and after (right, photo by M Fawcett) rehabilitation

Figure 7  Aerial view of the rehabilitated Sleisbeck pit and the ripped truck dump area 
(photo by M Fawcett)

A ground gamma survey was conducted of the area in July 2008, to determine average external gamma dose rates on site after rehabilitation. Three conventional Geiger Müller tubes (GM680-Environmental dose rate meters) were used for the survey. This report summarises the result of the gamma dose rate survey, and provides a statistical analysis of the data. Its primary purpose is as a data summary to demonstrate that the rehabilitation objective, to limit and where possible reverse the impact on the environment of any historic mining activities carried out in the Gunlom Land Trust area, has been met.

The rehabilitation of the historic mine sites in the South Alligator River valley is regarded as an intervention (NHMRC 1995). This is because the sources of exposure and exposure pathways were already present due to earlier practices that preceded regulatory control, it is an existing exposure situation. For these situations it is recommended that doses be optimized below a dose constraint. In ICRP 82 (1999), paragraph 4.1, it is recommended that:

An existing annual dose approaching about 10 mSv may be used as a generic reference level below which intervention is not likely to be justifiable for some prolonged exposure situation.

Although annual doses for people in the South Alligator River valley from historic mining activities are well below this dose constraint (Bollhöfer et al 2002), the radiological standard for the South Alligator River Valley rehabilitation has been set at a threshold external gamma dose rate of 1.25 μGy∙hr-1 (±20 %), following discussion between Parks Australia, OSS, Fawcett Mine Rehabilitation Services and O’Kane Consultants in 2008.

## 2  Methods

The gamma survey of the rehabilitated Sleisbeck mine area was conducted on 9–10 July 2008. Three dose rate instruments of the same type and model were used for the measurement of the external gamma dose rates. The specifications of the instruments are given below (Esparon 2007).

Description:	Environmental dose rate meter

Manufacturer: 	Mini-Instruments

Model:	6–80

Serial Number:	01065

Serial Number:	01049

Serial Number:	01064

Sensitivity:	18 counts/second per 1 µGy∙hr-1within the energy range of 55 keV to 2 MeV with a deviation of less than 25%

The instrument with serial number 01065 (GM 1) was calibrated on 21 April 2008 by Australian Radiation Services Pty Ltd, by testing the response of the monitor to a collimated beam of gamma rays from a 137Cs source (Figures 8 & 9). The results obtained in the calibration were typical for this type of instrument and complied with the manufacturer’s specifications. GM tube 3 (serial number 01049) and GM tube 2 (serial number 01064) were cross calibrated against GM tube 1, and measured count rates were corrected.

Gamma dose rates were measured in the field at a height of 1 m above ground. Count rates were registered for a period of 100 seconds and external gamma dose rates were determined using the calibration regression fits below.

Figure 8  Response of the Mini Instruments Environmental survey monitor Model 6–80/MC–71 digital scalar to 137Cs, 0–20 μGyh-1 range – April 2008

Figure 9  Response of the Mini Instruments Environmental survey monitor Model 6–80/MC–71 digital scalar to 137Cs, 0-2 μGyh-1 range – April 2008

## 3  Results

Tables A1 and A2 in the Appendix summarise eastings, northings, counts and comments recorded on the fieldsheets, and the results of the dose rate measurements at the Sleisbeck mine, respectively. Data were collected in the WGS84 coordinate system within Zone 53 and it is important to note that the precision of the GPS is typically around ±10 m. Approximately 7.6 ha were surveyed.

Figure 10 shows the locations and magnitude of the individual measurements overlaid on the Quickbird satellite image acquired in 2003, before rehabilitation of the mine site (notice that the pit is still present and filled with water).

Figure 10  Gamma dose rates [µGy∙hr-1] measured at the rehabilitated Sleisbeck mine overlaid on Quickbird data acquired in August 2003 (before mine rehabilitation)

Marten (1992) has previously determined the dose rate from cosmic radiation in the Alligator Rivers Region to 0.07 µGy∙hr-1. This value needs to be subtracted from the above measurements to determine terrestrial gamma dose rates.

Figure 11 shows a histogram and probability plot of the terrestrial gamma dose rates measured at Sleisbeck after rehabilitation.

The median terrestrial gamma dose rate measured across the area surveyed (7.6 ha) is 0.13 µGy∙hr-1. Geometric and arithmetic averages are 0.14 and 0.23 µGy∙hr-1, respectively.

The probability plot shows that 99% of the area surveyed exhibited terrestrial gamma dose rates below 1.25 µGy∙hr-1 assuming a lognormal distribution of the terrestrial gamma dose rates. There is a small area immediately to the east of the old access track to the rehabilitated pit of approximately 4 m2 in size, where terrestrial gamma dose rates of above 3 µGy∙hr-1 were measured. This area was part of the old track to the Sleisbeck pit, and mineralised material may have been used as road fill. This would also explain the occasional high readings obtained on the access track. However, this area is very small and takes up less than 0.05 per mill of the total area surveyed.

Figure 11  Histogram and probability plot of terrestrial gamma dose rates at Sleisbeck post rehabilitation. The probability plot shows that ~99% of the area surveyed exhibits terrestrial gamma dose rates below the threshold value of 1.25 µGy∙hr-1.

## 4  Conclusion

The rehabilitation of the old truck dumps and pit area at the Sleisbeck mine has reduced the average terrestrial gamma dose rates in the area by about threefold, from an average of between 0.57–0.76 µGy∙hr-1 pre-rehabilitation (Bollhöfer et al 2008) to 0.23 µGy∙hr-1 post rehabilitation. Typical dose rates measured are 0.14 µGy∙hr-1, indistinguishable from environmental background dose rates reported in the area, which range from 0.06–0.15 µGy∙hr-1 (Tims et al 2000, Waggitt 2004, Bollhöfer et al 2008).

Ninety-nine percent of the area surveyed exhibits dose rates below the threshold value of 1.25 µGy∙hr-1 (±20%). However, there are a couple of small areas where dose rates measured on ground are above this threshold value. These areas are close to or on existing tracks leading to the backfilled pit, indicating that some mineralised material may have been used as road fill when the tracks were built in the 1950s.

Assuming the unlikely scenario that the site is occupied for one month per year in the dry season, the average terrestrial gamma dose rate on site will lead to an effective dose from exposure to terrestrial gamma radiation of 0.11 and 0.13 mSv for an adult and child, respectively, using conversion factors of 0.69 Sv∙Gy-1 (adults) and 0.79 Sv∙Gy-1 (child) given in Table VIII of Annex A of UNSCEAR (2000). Approximately half of this dose originates from exposure to background radiation. These doses are not only well below the dose constraint of 10 mSv per year for existing situations, recommended by the International Commission on Radiation Protection (ICRP 1999), but also below the ICRP’s most recent recommendations (ICRP 2007) of a dose constraint of about 0.3 mSv for prolonged exposure from planned exposure situations.

## References

Annabel R 1971. The uranium hunters. Rigby Publishers Ltd, Adelaide.

Bollhöfer A, Ryan B, Pfitzner K, Martin P & Iles M 2002. A radiation dose estimate for visitors of the South Alligator River valley, Australia, from remnants of uranium mining and milling activities. In Uranium mining and hydrogeology III, eds BJ Merkel, B Planer-Friedrich & C Wolkersdorfer, Technical University, Bergakademie Freiberg, 931940.

Bollhöfer A, Pfitzner K, Ryan B & Fawcett M 2007. Ground truthing of an airborne gamma survey and assessment of the radiological conditions of the Sleisbeck mine area. Internal Report 526, June, Supervising Scientist, Darwin. Unpublished paper.

Bollhöfer A, Pfitzner K, Ryan B, Martin P, Fawcett M & Jones DR 2008. Airborne gamma survey of the historic Sleisbeck mine area in the Northern Territory, Australia, and its use for site rehabilitation planning. Journal of Environmental Radioactivity 99, 1770–1774.

ICRP 1999. Protection of the public in situations of prolonged radiation exposure. Publication 82, International Commission on Radiological Protection, Pergamon Press, Oxford.

ICRP 2007. The 2007 Recommendations of the International Commission on Radiological Protection. Publication 103, International Commission on Radiological Protection, Elsevier Ltd.

Marten R 1992. External gamma dose rate survey of the Ranger Uranium Mine land application plot. In Proceedings of the Workshop on Land Application of Effluent Water from Uranium Mines in the Alligator Rivers Region. Supervising Scientist for the Alligator Rivers Region, AGPS, Canberra, 252–266.

NHMRC 1995. Recommendations for limiting exposure to ionizing radiation and National standard for limiting occupational exposure to ionizing radiation. Radiation Health series 39, National Health and Medical Research Council, Australian Government Publishing Service, Canberra.

Pfitzner K, Ryan B & Martin P 2003. Airborne gamma survey of the Sleisbeck mine area. Internal Report 400, January, Supervising Scientist, Darwin. Unpublished paper.

Supervising Scientist 2008. Annual Report 2007–2008. Supervising Scientist, Darwin.

Tims S & Ryan B 1998. Radiation surveys of El Sherana camp site and Sleisbeck pit and waste rock piles. Internal report 284, Supervising Scientist, Canberra. Unpublished paper.

Tims S, Ryan B & Waggitt PW 2000. γ-radiation survey of exposed tailings in the area around Rockhole mine. Internal Report 332, Supervising Scientist, Darwin. Unpublished paper.

UNSCEAR 2000. United Nations Scientific Committee on the Effects of Atomic Radiation 2000 Report Vol. I. Sources and effects of ionizing radiation. Report to the General Assembly, with scientific annexes, United Nations Publication, New York.

Waggitt PW 2004. Uranium mine rehabilitation: the story of the South Alligator Valley intervention. Journal of Environmental Radioactivity 76, 51–66.

Waggitt P & Fawcett M 2008. Small scale uranium mine remediation in northern Australia. In Uranium mining and hydrogeology V, eds BJ Merkel & A Hasche-Berger, Springer-Verlag Berlin & Heidelberg New York, 243–250.

## Appendix A1  Eastings, northings, counts and comments recorded on the fieldsheets

| GM tube | WPT | Easting | Northing | counts per 100 sec | comments |
| --- | --- | --- | --- | --- | --- |
| GM1 | WPT001 | 264869 | 8475637 | 240 | Environmental |
| GM1 | WPT002 | 264880 | 8475635 | 285 | “ |
| GM1 | WPT003 | 264892 | 8475637 | 290 | Env edge |
| GM1 | WPT004 | 264905 | 8475632 | 1360 | Edge of dump |
| GM1 | WPT005 | 264914 | 8475626 | 531 | Dump I + II |
| GM1 | WPT006 | 264925 | 8475621 | 822 | “ |
| GM1 | WPT007 | 264937 | 8475619 | 402 | “ |
| GM1 | WPT008 | 264946 | 8475614 | 378 | “ |
| GM1 | WPT009 | 264963 | 8475612 | 514 | “ |
| GM1 | WPT010 | 264968 | 8475607 | 489 | Dump 1 |
| GM1 | WPT011 | 264981 | 8475605 | 612 | Dump I + II |
| GM1 | WPT012 | 264992 | 8475603 | 678 | Dump I / East edge |
| GM1 | WPT013 | 264999 | 8475601 | 1605 | Edge next to track |
| GM1 | WPT014 | 265008 | 8475603 | 1978 | On track |
| GM1 | WPT015 | 265033 | 8475598 | 6137 | Pad next to track |
| GM1 | WPT016 | 265037 | 8475591 | 1269 |  |
| GM1 | WPT017 | 265048 | 8475581 | 778 | WRD III, not rehab |
| GM1 | WPT018 | 265060 | 8475578 | 622 | WRD III |
| GM1 | WPT019 | 265071 | 8475571 | 705 | WRD III edge |
| GM1 | WPT020 | 265082 | 8475554 | 723 | WRD III rehab |
| GM1 | WPT021 | 265084 | 8475545 | 1160 | End of dump |
| GM1 | WPT022 | 265084 | 8475536 | 793 | “ |
| GM1 | WPT023 | 265095 | 8475532 | 461 |  |
| GM1 | WPT024 | 265104 | 8475519 | 396 |  |
| GM1 | WPT025 | 265115 | 8475510 | 373 | Environmental |
| GM1 | WPT026 | 265122 | 8475499 | 367 | “ |
| GM1 | WPT027 | 265119 | 8475514 | 342 | “ |
| GM1 | WPT028 | 265113 | 8475521 | 354 | “ |
| GM1 | WPT029 | 265106 | 8475530 | 377 |  |
| GM1 | WPT030 | 265097 | 8475538 | 417 |  |
| GM1 | WPT031 | 265088 | 8475549 | 509 |  |
| GM1 | WPT032 | 265077 | 8475560 | 583 |  |
| GM1 | WPT033 | 265073 | 8475567 | 698 |  |
| GM1 | WPT034 | 265073 | 8475578 | 708 | Edge of dump 3, not rehab |
| GM1 | WPT035 | 265066 | 8475580 | 725 | “ |
| GM1 | WPT036 | 265049 | 8475587 | 693 |  |
| GM1 | WPT037 | 265039 | 8475592 | 883 | Dump / rehab |
| GM1 | WPT038 | 265029 | 8475600 | 1435 |  |
| GM1 | WPT039 | 265022 | 8475603 | 2766 | Grey material / ‘pad’ |
| GM1 | WPT040 | 265011 | 8475607 | 1543 |  |
| GM1 | WPT041 | 265001 | 8475614 | 3190 | On track |
| GM1 | WPT042 | 264986 | 8475618 | 751 | Within dump II |
| GM1 | WPT043 | 264977 | 8475625 | 940 |  |
| GM1 | WPT044 | 264963 | 8475625 | 943 | Heap around tree |
| GM1 | WPT045 | 264961 | 8475632 | 624 | Dump I |
| GM1 | WPT046 | 264953 | 8475634 | 492 | “ |
| GM1 | WPT047 | 264941 | 8475634 | 411 | “ |
| GM1 | WPT048 | 264925 | 8475637 | 775 |  |
| GM1 | WPT049 | 264912 | 8475643 | 574 |  |
| GM1 | WPT050 | 264908 | 8475647 | 1562 | On rubble pile around tree |
| GM1 | WPT051 | 264896 | 8475652 | 1076 | Edge of dump I |
| GM1 | WPT052 | 264885 | 8475656 | 311 | Env |
| GM1 | WPT053 | 264877 | 8475663 | 289 | “ |
| GM1 | WPT054 | 264902 | 8475739 | 255 | “ |
| GM1 | WPT055 | 264911 | 8475731 | 262 | “ |
| GM1 | WPT056 | 264924 | 8475728 | 278 | Disturbed |
| GM1 | WPT057 | 264936 | 8475724 | 285 |  |
| GM1 | WPT058 | 264956 | 8475724 | 392 |  |
| GM1 | WPT059 | 264969 | 8475719 | 496 |  |
| GM1 | WPT060 | 264980 | 8475714 | 418 | Edge of pit |
| GM1 | WPT061 | 264989 | 8475712 | 276 | Above pit |
| GM1 | WPT062 | 264998 | 8475708 | 340 | “ |
| GM1 | WPT063 | 265005 | 8475703 | 294 | “ |
| GM1 | WPT064 | 265018 | 8475699 | 308 | “ |
| GM1 | WPT065 | 265023 | 8475694 | 341 |  |
| GM1 | WPT066 | 265034 | 8475685 | 321 | Hill / rocks |
| GM1 | WPT067 | 265043 | 8475681 | 295 | “ |
| GM1 | WPT068 | 265056 | 8475674 | 248 | “ |
| GM1 | WPT069 | 265072 | 8475668 | 245 | “ |
| GM1 | WPT070 | 265081 | 8475661 | 261 | “ |
| GM1 | WPT071 | 265098 | 8475652 | 271 | “ |
| GM1 | WPT072 | 265107 | 8475641 | 233 | “ |
| GM1 | WPT073 | 265118 | 8475624 | 228 |  |
| GM1 | WPT074 | 265134 | 8475619 | 257 |  |
| GM1 | WPT075 | 265145 | 8475610 | 199 |  |
| GM1 | WPT076 | 265161 | 8475601 | 206 |  |
| GM1 | WPT077 | 265168 | 8475590 | 228 |  |
| GM1 | WPT078 | 265179 | 8475584 | 191 |  |
| GM1 | WPT079 | 265183 | 8475603 | 217 | Rocky slope / hill |
| GM1 | WPT080 | 265159 | 8475612 | 187 | “ |
| GM1 | WPT081 | 265141 | 8475625 | 227 | “ |
| GM1 | WPT082 | 265125 | 8475637 | 236 | “ |
| GM1 | WPT083 | 265108 | 8475647 | 235 | “ |
| GM1 | WPT084 | 265088 | 8475657 | 281 | “ |
| GM1 | WPT085 | 265074 | 8475668 | 247 | “ |
| GM1 | WPT086 | 265061 | 8475677 | 222 | “ |
| GM1 | WPT087 | 265045 | 8475687 | 252 | “ |
| GM1 | WPT088 | 265043 | 8475687 | 255 | “ |
| GM1 | WPT089 | 265036 | 8475696 | 316 | “ |
| GM1 | WPT090 | 265027 | 8475697 | 370 | Edge of hill / pit |
| GM1 | WPT091 | 265016 | 8475707 | 328 | Above pit |
| GM1 | WPT092 | 265009 | 8475714 | 271 | “ |
| GM1 | WPT093 | 265000 | 8475717 | 292 | “ |
| GM1 | WPT094 | 264987 | 8475721 | 301 | “ |
| GM1 | WPT095 | 264980 | 8475727 | 591 | “ |
| GM1 | WPT096 | 264969 | 8475732 | 565 | Top of west pit wall |
| GM1 | WPT097 | 264960 | 8475736 | 442 | Disturbed area |
| GM1 | WPT098 | 264947 | 8475741 | 383 |  |
| GM1 | WPT099 | 264940 | 8475750 | 358 | Disturbed area |
| GM1 | WPT100 | 264925 | 8475754 | 309 | Rel undisturbed |
| GM1 | WPT101 | 264916 | 8475757 | 329 | Env |
| GM1 | WPT102 | 264970 | 8475747 | 631 | In pit |
| GM1 | WPT103 | 264983 | 8475739 | 636 | “ |
| GM1 | WPT104 | 264992 | 8475730 | 662 | “ |
| GM1 | WPT105 | 265003 | 8475734 | 602 | On top of gamma hot spot in wall |
| GM1 | WPT106 | 264989 | 8475741 | 732 | Next to pit wall |
| GM1 | WPT107 | 264922 | 8475473 | 302 | Undisturbed |
| GM1 | WPT108 | 264910 | 8475470 | 392 | “ |
| GM1 | WPT109 | 264903 | 8475469 | 1117 |  |
| GM1 | WPT110 | 264895 | 8475468 | 1389 |  |
| GM1 | WPT111 | 264886 | 8475467 | 692 |  |
| GM1 | WPT112 | 264872 | 8475471 | 838 |  |
| GM1 | WPT113 | 264865 | 8475471 | 742 |  |
| GM1 | WPT114 | 264867 | 8475458 | 708 |  |
| GM1 | WPT115 | 264859 | 8475460 | 245 |  |
| GM1 | WPT116 | 264881 | 8475467 | 1126 |  |
| GM1 | WPT117 | 264892 | 8475469 | 1307 |  |
| GM1 | WPT118 | 264903 | 8475466 | 960 |  |
| GM1 | WPT119 | 264912 | 8475458 | 1522 |  |
| GM1 | WPT120 | 264923 | 8475457 | 262 |  |
| GM1 | WPT121 | 264916 | 8475438 | 295 |  |
| GM1 | WPT122 | 264896 | 8475440 | 329 |  |
| GM1 | WPT123 | 264885 | 8475442 | 263 |  |
| GM1 | WPT124 | 264968 | 8475660 | 6478 |  |
| GM2 | WPT002 | 264865 | 8475622 | 118 |  |
| GM2 | WPT003 | 264878 | 8475619 | 217 |  |
| GM2 | WPT004 | 264890 | 8475621 | 273 |  |
| GM2 | WPT005 | 264903 | 8475610 | 1103 |  |
| GM2 | WPT006 | 264914 | 8475610 | 652 |  |
| GM2 | WPT007 | 264921 | 8475610 | 538 |  |
| GM2 | WPT008 | 264939 | 8475604 | 383 |  |
| GM2 | WPT009 | 264948 | 8475599 | 402 |  |
| GM2 | WPT010 | 264957 | 8475601 | 703 | Tree mound |
| GM2 | WPT011 | 264970 | 8475595 | 782 |  |
| GM2 | WPT012 | 264979 | 8475592 | 625 |  |
| GM2 | WPT013 | 264985 | 8475590 | 1279 | Tree mound |
| GM2 | WPT014 | 264992 | 8475588 | 614 |  |
| GM2 | WPT015 | 265003 | 8475592 | 677 |  |
| GM2 | WPT016 | 265013 | 8475588 | 2344 | track |
| GM2 | WPT017 | 265024 | 8475581 | 1328 |  |
| GM2 | WPT018 | 265033 | 8475574 | 1460 |  |
| GM2 | WPT019 | 265044 | 8475570 | 617 |  |
| GM2 | WPT020 | 265059 | 8475567 | 709 |  |
| GM2 | WPT021 | 265064 | 8475552 | 1005 |  |
| GM2 | WPT022 | 265066 | 8475545 | 1226 |  |
| GM2 | WPT023 | 265068 | 8475528 | 875 |  |
| GM2 | WPT024 | 265079 | 8475523 | 531 |  |
| GM2 | WPT025 | 265093 | 8475523 | 429 | environment |
| GM2 | WPT026 | 265099 | 8475506 | 383 |  |
| GM2 | WPT027 | 265110 | 8475499 | 330 |  |
| GM2 | WPT028 | 265119 | 8475492 | 350 |  |
| GM2 | WPT029 | 265126 | 8475529 | 363 |  |
| GM2 | WPT030 | 265117 | 8475536 | 302 | new line |
| GM2 | WPT031 | 265109 | 8475545 | 350 |  |
| GM2 | WPT032 | 265100 | 8475558 | 463 |  |
| GM2 | WPT033 | 265091 | 8475567 | 568 |  |
| GM2 | WPT034 | 265082 | 8475578 | 860 |  |
| GM2 | WPT035 | 265075 | 8475587 | 689 |  |
| GM2 | WPT036 | 265066 | 8475594 | 932 |  |
| GM2 | WPT037 | 265060 | 8475602 | 877 |  |
| GM2 | WPT038 | 265055 | 8475609 | 751 |  |
| GM2 | WPT039 | 265044 | 8475615 | 477 |  |
| GM2 | WPT040 | 265035 | 8475620 | 981 |  |
| GM2 | WPT041 | 265018 | 8475629 | 1565 |  |
| GM2 | WPT042 | 265006 | 8475638 | 559 |  |
| GM2 | WPT043 | 264997 | 8475636 | 590 | track |
| GM2 | WPT044 | 264988 | 8475644 | 620 |  |
| GM2 | WPT045 | 264980 | 8475638 | 1425 | Tree mound |
| GM2 | WPT046 | 264968 | 8475643 | 794 | Big hill |
| GM2 | WPT047 | 264955 | 8475653 | 814 |  |
| GM2 | WPT048 | 264948 | 8475658 | 681 |  |
| GM2 | WPT049 | 264935 | 8475658 | 730 |  |
| GM2 | WPT050 | 264924 | 8475658 | 510 |  |
| GM2 | WPT051 | 264910 | 8475654 | 1013 |  |
| GM2 | WPT052 | 264905 | 8475659 | 1638 |  |
| GM2 | WPT053 | 264895 | 8475669 | 295 | environment |
| GM2 | WPT054 | 264883 | 8475674 | 274 |  |
| GM2 | WPT055 | 264899 | 8475720 | 258 | new line |
| GM2 | WPT056 | 264906 | 8475717 | 222 |  |
| GM2 | WPT057 | 264920 | 8475717 | 285 |  |
| GM2 | WPT058 | 264933 | 8475715 | 302 |  |
| GM2 | WPT059 | 264949 | 8475717 | 321 |  |
| GM2 | WPT060 | 264956 | 8475710 | 386 |  |
| GM2 | WPT061 | 264967 | 8475702 | 437 |  |
| GM2 | WPT062 | 264985 | 8475699 | 472 | edge of pit |
| GM2 | WPT063 | 264996 | 8475697 | 478 |  |
| GM2 | WPT064 | 265007 | 8475694 | 417 |  |
| GM2 | WPT065 | 265018 | 8475688 | 740 |  |
| GM2 | WPT066 | 265027 | 8475683 | 393 |  |
| GM2 | WPT067 | 265034 | 8475675 | 389 |  |
| GM2 | WPT068 | 265043 | 8475670 | 342 | edge of pit |
| GM2 | WPT069 | 265051 | 8475663 | 268 |  |
| GM2 | WPT070 | 265069 | 8475650 | 243 |  |
| GM2 | WPT071 | 265069 | 8475650 | 247 |  |
| GM2 | WPT072 | 265078 | 8475646 | 252 |  |
| GM2 | WPT073 | 265090 | 8475637 | 229 |  |
| GM2 | WPT074 | 265100 | 8475630 | 242 |  |
| GM2 | WPT075 | 265103 | 8475623 | 221 |  |
| GM2 | WPT076 | 265116 | 8475613 | 199 |  |
| GM2 | WPT077 | 265127 | 8475612 | 233 |  |
| GM2 | WPT078 | 265136 | 8475604 | 195 |  |
| GM2 | WPT079 | 265147 | 8475599 | 185 |  |
| GM2 | WPT080 | 265158 | 8475592 | 215 |  |
| GM2 | WPT081 | 265168 | 8475584 | 231 | track |
| GM2 | WPT082 | 265183 | 8475577 | 226 |  |
| GM2 | WPT083 | 265229 | 8475666 | 205 |  |
| GM2 | WPT084 | 265211 | 8475686 | 197 |  |
| GM2 | WPT085 | 265189 | 8475693 | 181 |  |
| GM2 | WPT086 | 265166 | 8475701 | 184 |  |
| GM2 | WPT087 | 265147 | 8475717 | 198 |  |
| GM2 | WPT088 | 265126 | 8475724 | 190 |  |
| GM2 | WPT089 | 265111 | 8475739 | 192 |  |
| GM2 | WPT090 | 265093 | 8475746 | 174 |  |
| GM2 | WPT091 | 265084 | 8475753 | 166 |  |
| GM2 | WPT092 | 265080 | 8475761 | 215 |  |
| GM2 | WPT093 | 265069 | 8475768 | 192 |  |
| GM2 | WPT094 | 265057 | 8475771 | 167 |  |
| GM2 | WPT095 | 265042 | 8475779 | 206 |  |
| GM2 | WPT096 | 265040 | 8475784 | 174 |  |
| GM2 | WPT097 | 265030 | 8475790 | 205 |  |
| GM2 | WPT098 | 265028 | 8475808 | 219 |  |
| GM2 | WPT099 | 265020 | 8475814 | 206 |  |
| GM3 | WPT001 | 264858 | 8475609 | 245 | New line |
| GM3 | WPT002 | 264871 | 8475609 | 249 |  |
| GM3 | WPT003 | 264882 | 8475604 | 268 |  |
| GM3 | WPT004 | 264898 | 8475595 | 434 |  |
| GM3 | WPT005 | 264907 | 8475591 | 417 |  |
| GM3 | WPT006 | 264920 | 8475588 | 493 |  |
| GM3 | WPT007 | 264930 | 8475582 | 471 |  |
| GM3 | WPT008 | 264936 | 8475591 | 862 | Large mound of dirt around standing trees |
| GM3 | WPT009 | 264939 | 8475584 | 580 |  |
| GM3 | WPT010 | 264950 | 8475577 | 561 |  |
| GM3 | WPT011 | 264965 | 8475575 | 478 |  |
| GM3 | WPT012 | 264976 | 8475572 | 432 |  |
| GM3 | WPT013 | 264977 | 8475581 | 987 | Large mound of dirt around standing trees |
| GM3 | WPT014 | 264983 | 8475579 | 461 |  |
| GM3 | WPT015 | 264997 | 8475568 | 539 |  |
| GM3 | WPT016 | 265008 | 8475561 | 518 |  |
| GM3 | WPT017 | 265021 | 8475557 | 1001 | Built up dirt area |
| GM3 | WPT018 | 265030 | 8475550 | 874 |  |
| GM3 | WPT019 | 265041 | 8475546 | 736 |  |
| GM3 | WPT020 | 265048 | 8475539 | 1125 | Cleared land |
| GM3 | WPT021 | 265048 | 8475535 | 2112 | Small mound of dirt around standing tree |
| GM3 | WPT022 | 265046 | 8475526 | 750 | Cleared land |
| GM3 | WPT023 | 265050 | 8475517 | 1151 | Small mound of dirt |
| GM3 | WPT024 | 265061 | 8475510 | 630 | Cleared land |
| GM3 | WPT025 | 265063 | 8475504 | 396 | Undisturbed bushland |
| GM3 | WPT026 | 265070 | 8475495 | 345 | “ |
| GM3 | WPT027 | 265079 | 8475489 | 314 | “ |
| GM3 | WPT028 | 265090 | 8475480 | 295 | Dugout channel |
| GM3 | WPT029 | 265103 | 8475477 | 328 | Undisturbed bushland |
| GM3 | WPT030 | 265133 | 8475543 | 288 | New line |
| GM3 | WPT031 | 265131 | 8475555 | 294 | Undisturbed bushland |
| GM3 | WPT032 | 265125 | 8475562 | 289 | “ |
| GM3 | WPT033 | 265113 | 8475580 | 348 | “ |
| GM3 | WPT034 | 265098 | 8475580 | 593 | “ |
| GM3 | WPT035 | 265094 | 8475593 | 597 | Cleared land |
| GM3 | WPT036 | 265089 | 8475598 | 725 | “ |
| GM3 | WPT037 | 265082 | 8475600 | 685 | “ |
| GM3 | WPT038 | 265073 | 8475611 | 583 | Edge of cleared area |
| GM3 | WPT039 | 265062 | 8475620 | 479 |  |
| GM3 | WPT040 | 265051 | 8475626 | 486 |  |
| GM3 | WPT041 | 265044 | 8475635 | 435 |  |
| GM3 | WPT042 | 265033 | 8475642 | 373 |  |
| GM3 | WPT043 | 265027 | 8475646 | 462 |  |
| GM3 | WPT044 | 265018 | 8475651 | 499 |  |
| GM3 | WPT045 | 265006 | 8475660 | 519 |  |
| GM3 | WPT046 | 264998 | 8475666 | 604 |  |
| GM3 | WPT047 | 264984 | 8475666 | 466 |  |
| GM3 | WPT048 | 264971 | 8475658 | 1008 | Large mound of dirt around standing trees |
| GM3 | WPT049 | 264971 | 8475666 | 597 |  |
| GM3 | WPT050 | 264966 | 8475675 | 525 |  |
| GM3 | WPT051 | 264953 | 8475680 | 409 |  |
| GM3 | WPT052 | 264946 | 8475680 | 296 |  |
| GM3 | WPT053 | 264937 | 8475684 | 312 |  |
| GM3 | WPT054 | 264930 | 8475684 | 283 |  |
| GM3 | WPT055 | 264915 | 8475685 | 303 |  |
| GM3 | WPT056 | 264902 | 8475689 | 276 |  |
| GM3 | WPT057 | 264895 | 8475691 | 234 |  |
| GM3 | WPT058 | 264886 | 8475700 | 237 | New line |
| GM3 | WPT059 | 264893 | 8475709 | 258 |  |
| GM3 | WPT060 | 264904 | 8475706 | 278 |  |
| GM3 | WPT061 | 264913 | 8475706 | 275 |  |
| GM3 | WPT062 | 264928 | 8475700 | 309 |  |
| GM3 | WPT063 | 264938 | 8475695 | 320 |  |
| GM3 | WPT064 | 264951 | 8475691 | 360 |  |
| GM3 | WPT065 | 264960 | 8475688 | 451 |  |
| GM3 | WPT066 | 264973 | 8475682 | 536 | Cleared land |
| GM3 | WPT067 | 264980 | 8475677 | 996 | Large mound of dirt around standing trees |
| GM3 | WPT068 | 264995 | 8475675 | 1360 | Cleared land / edge of old pit |
| GM3 | WPT069 | 265000 | 8475671 | 770 | “ |
| GM3 | WPT070 | 265009 | 8475664 | 600 | “ |
| GM3 | WPT071 | 265020 | 8475660 | 375 | Bushy area |
| GM3 | WPT072 | 265031 | 8475651 | 345 | Cleared land |
| GM3 | WPT073 | 265040 | 8475648 | 390 | “ |
| GM3 | WPT074 | 265049 | 8475642 | 320 | Bushy, rocky area on hill slope |
| GM3 | WPT075 | 265058 | 8475635 | 279 | “ |
| GM3 | WPT076 | 265069 | 8475630 | 261 | “ |
| GM3 | WPT077 | 265076 | 8475622 | 293 | “ |
| GM3 | WPT078 | 265083 | 8475615 | 277 | “ |
| GM3 | WPT079 | 265093 | 8475608 | 328 | Edge of cleared area |
| GM3 | WPT080 | 265103 | 8475606 | 341 | “ |
| GM3 | WPT081 | 265111 | 8475593 | 365 | “ |
| GM3 | WPT082 | 265118 | 8475584 | 305 | Undisturbed bushland |
| GM3 | WPT083 | 265127 | 8475580 | 264 | “ |
| GM3 | WPT084 | 265134 | 8475573 | 252 | “ |
| GM3 | WPT085 | 265145 | 8475566 | 278 | “ |
| GM3 | WPT086 | 265158 | 8475558 | 251 | “ |
| GM3 | WPT087 | 265170 | 8475559 | 268 | “ |
| GM3 | WPT088 | 265188 | 8475631 | 206 | New line / line follows highest point of hill |
| GM3 | WPT089 | 265172 | 8475642 | 221 | Highest point of hill |
| GM3 | WPT090 | 265152 | 8475652 | 215 | “ |
| GM3 | WPT091 | 265139 | 8475662 | 183 | “ |
| GM3 | WPT092 | 265124 | 8475671 | 191 | “ |
| GM3 | WPT093 | 265104 | 8475680 | 222 | “ |
| GM3 | WPT094 | 265092 | 8475685 | 228 | “ |
| GM3 | WPT095 | 265074 | 8475694 | 201 | “ |
| GM3 | WPT096 | 265063 | 8475698 | 215 | “ |
| GM3 | WPT097 | 265056 | 8475703 | 238 | “ |
| GM3 | WPT098 | 265047 | 8475709 | 270 | “ |
| GM3 | WPT099 | 265037 | 8475714 | 279 | “ |
| GM3 | WPT100 | 265030 | 8475725 | 324 | “ |
| GM3 | WPT101 | 265023 | 8475727 | 429 | “ |
| GM3 | WPT102 | 265017 | 8475736 | 291 | “ |
| GM3 | WPT103 | 265012 | 8475745 | 270 | “ |
| GM3 | WPT104 | 265001 | 8475753 | 587 | “ |
| GM3 | WPT105 | 264997 | 8475760 | 317 | “ |
| GM3 | WPT106 | 264987 | 8475767 | 281 | “ |
| GM3 | WPT107 | 264983 | 8475771 | 275 | “ |
| GM3 | WPT108 | 264976 | 8475776 | 227 | “ |
| GM3 | WPT109 | 264965 | 8475780 | 306 | “ |
| GM3 | WPT110 | 264954 | 8475785 | 218 | “ |
| GM3 | WPT111 | 264945 | 8475787 | 236 | “ |
| GM3 | WPT112 | 264934 | 8475761 | 295 | bottom of crest leading towards pit / east |
| GM3 | WPT113 | 264942 | 8475754 | 289 | “ |
| GM3 | WPT114 | 264951 | 8475750 | 426 | “ |
| GM3 | WPT115 | 264958 | 8475743 | 380 | “ |
| GM3 | WPT116 | 264967 | 8475739 | 489 | “ |
| GM3 | WPT117 | 264978 | 8475747 | 588 | “ |
| GM3 | WPT118 | 264987 | 8475732 | 280 | New line |
| GM3 | WPT119 | 264912 | 8475484 | 287 |  |
| GM3 | WPT120 | 264899 | 8475480 | 542 |  |
| GM3 | WPT121 | 264890 | 8475482 | 706 |  |
| GM3 | WPT122 | 264881 | 8475484 | 1587 | Large mound of dirt around standing trees |
| GM3 | WPT123 | 264870 | 8475482 | 1354 |  |
| GM3 | WPT124 | 264863 | 8475478 | 354 |  |
| GM3 | WPT125 | 264870 | 8475491 | 283 | New line |
| GM3 | WPT126 | 264884 | 8475486 | 307 |  |
| GM3 | WPT127 | 264897 | 8475484 | 257 |  |
| GM3 | WPT128 | 264908 | 8475481 | 234 |  |
| GM3 | WPT129 | 264917 | 8475481 | 288 |  |
| GM3 | WPT130 | 264910 | 8475455 | 300 | New line |
| GM3 | WPT131 | 264901 | 8475457 | 490 |  |
| GM3 | WPT132 | 264888 | 8475456 | 388 |  |
| GM3 | WPT133 | 264876 | 8475456 | 468 |  |
| GM3 | WPT134 | 264869 | 8475456 | 622 |  |
| GM3 | WPT135 | 264854 | 8475462 | 237 |  |
| GM3 | WPT136 | 264852 | 8475447 | 208 | New line |
| GM3 | WPT137 | 264863 | 8475438 | 222 |  |
| GM3 | WPT138 | 264871 | 8475434 | 220 |  |

## Appendix A2  Results of the gamma dose rate measurements

|  | Waypoint | Eastings | Northings | γ-dose rate µGy∙hr-1 | ± µGy∙hr-1 |
| --- | --- | --- | --- | --- | --- |
| GM1 | WPT001 | 264868.7 | 8475637 | 0.12 | 0.01 |
| GM1 | WPT002 | 264879.5 | 8475635 | 0.15 | 0.01 |
| GM1 | WPT003 | 264892.1 | 8475637 | 0.15 | 0.01 |
| GM1 | WPT004 | 264904.8 | 8475632 | 0.75 | 0.02 |
| GM1 | WPT005 | 264913.9 | 8475626 | 0.29 | 0.01 |
| GM1 | WPT006 | 264924.7 | 8475621 | 0.45 | 0.02 |
| GM1 | WPT007 | 264937.4 | 8475619 | 0.21 | 0.01 |
| GM1 | WPT008 | 264946.4 | 8475614 | 0.20 | 0.01 |
| GM1 | WPT009 | 264962.7 | 8475612 | 0.28 | 0.01 |
| GM1 | WPT010 | 264968.1 | 8475607 | 0.26 | 0.01 |
| GM1 | WPT011 | 264980.8 | 8475605 | 0.33 | 0.01 |
| GM1 | WPT012 | 264991.6 | 8475603 | 0.37 | 0.01 |
| GM1 | WPT013 | 264998.8 | 8475601 | 0.89 | 0.02 |
| GM1 | WPT014 | 265007.8 | 8475603 | 1.10 | 0.02 |
| GM1 | WPT015 | 265033.1 | 8475598 | 3.65 | 0.05 |
| GM1 | WPT016 | 265036.8 | 8475591 | 0.70 | 0.02 |
| GM1 | WPT017 | 265047.7 | 8475581 | 0.42 | 0.02 |
| GM1 | WPT018 | 265060.3 | 8475578 | 0.34 | 0.01 |
| GM1 | WPT019 | 265071.2 | 8475571 | 0.38 | 0.01 |
| GM1 | WPT020 | 265082.2 | 8475554 | 0.39 | 0.02 |
| GM1 | WPT021 | 265084.1 | 8475545 | 0.64 | 0.02 |
| GM1 | WPT022 | 265084.2 | 8475536 | 0.43 | 0.02 |
| GM1 | WPT023 | 265095 | 8475532 | 0.25 | 0.01 |
| GM1 | WPT024 | 265104.1 | 8475519 | 0.21 | 0.01 |
| GM1 | WPT025 | 265115 | 8475510 | 0.20 | 0.01 |
| GM1 | WPT026 | 265122.3 | 8475499 | 0.19 | 0.01 |
| GM1 | WPT027 | 265118.6 | 8475514 | 0.18 | 0.01 |
| GM1 | WPT028 | 265113.1 | 8475521 | 0.19 | 0.01 |
| GM1 | WPT029 | 265105.8 | 8475530 | 0.20 | 0.01 |
| GM1 | WPT030 | 265096.8 | 8475538 | 0.22 | 0.01 |
| GM1 | WPT031 | 265087.6 | 8475549 | 0.27 | 0.01 |
| GM1 | WPT032 | 265076.7 | 8475560 | 0.31 | 0.01 |
| GM1 | WPT033 | 265073.1 | 8475567 | 0.38 | 0.01 |
| GM1 | WPT034 | 265073 | 8475578 | 0.38 | 0.01 |
| GM1 | WPT035 | 265065.7 | 8475580 | 0.39 | 0.02 |
| GM1 | WPT036 | 265049.4 | 8475587 | 0.38 | 0.01 |
| GM1 | WPT037 | 265038.6 | 8475592 | 0.48 | 0.02 |
| GM1 | WPT038 | 265029.5 | 8475600 | 0.79 | 0.02 |
| GM1 | WPT039 | 265022.2 | 8475603 | 1.54 | 0.03 |
| GM1 | WPT040 | 265011.4 | 8475607 | 0.85 | 0.02 |
| GM1 | WPT041 | 265000.5 | 8475614 | 1.77 | 0.03 |
| GM1 | WPT042 | 264986.1 | 8475618 | 0.41 | 0.02 |
| GM1 | WPT043 | 264977 | 8475625 | 0.51 | 0.02 |
| GM1 | WPT044 | 264962.6 | 8475625 | 0.52 | 0.02 |
| GM1 | WPT045 | 264960.7 | 8475632 | 0.34 | 0.01 |
| GM1 | WPT046 | 264953.5 | 8475634 | 0.26 | 0.01 |
| GM1 | WPT047 | 264940.8 | 8475634 | 0.22 | 0.01 |
| GM1 | WPT048 | 264924.6 | 8475637 | 0.42 | 0.02 |
| GM1 | WPT049 | 264911.9 | 8475643 | 0.31 | 0.01 |
| GM1 | WPT050 | 264908.3 | 8475647 | 0.86 | 0.02 |
| GM1 | WPT051 | 264895.6 | 8475652 | 0.59 | 0.02 |
| GM1 | WPT052 | 264884.8 | 8475656 | 0.16 | 0.01 |
| GM1 | WPT053 | 264877.5 | 8475663 | 0.15 | 0.01 |
| GM1 | WPT054 | 264902 | 8475739 | 0.13 | 0.01 |
| GM1 | WPT055 | 264911.1 | 8475731 | 0.13 | 0.01 |
| GM1 | WPT056 | 264923.8 | 8475728 | 0.14 | 0.01 |
| GM1 | WPT057 | 264936.4 | 8475724 | 0.15 | 0.01 |
| GM1 | WPT058 | 264956.2 | 8475724 | 0.21 | 0.01 |
| GM1 | WPT059 | 264968.9 | 8475719 | 0.27 | 0.01 |
| GM1 | WPT060 | 264979.8 | 8475714 | 0.22 | 0.01 |
| GM1 | WPT061 | 264988.8 | 8475712 | 0.14 | 0.01 |
| GM1 | WPT062 | 264997.9 | 8475708 | 0.18 | 0.01 |
| GM1 | WPT063 | 265005.1 | 8475703 | 0.15 | 0.01 |
| GM1 | WPT064 | 265017.8 | 8475699 | 0.16 | 0.01 |
| GM1 | WPT065 | 265023.2 | 8475694 | 0.18 | 0.01 |
| GM1 | WPT066 | 265034.1 | 8475685 | 0.17 | 0.01 |
| GM1 | WPT067 | 265043.2 | 8475681 | 0.15 | 0.01 |
| GM1 | WPT068 | 265055.9 | 8475674 | 0.13 | 0.01 |
| GM1 | WPT069 | 265072.1 | 8475668 | 0.13 | 0.01 |
| GM1 | WPT070 | 265081.2 | 8475661 | 0.13 | 0.01 |
| GM1 | WPT071 | 265097.5 | 8475652 | 0.14 | 0.01 |
| GM1 | WPT072 | 265106.6 | 8475641 | 0.12 | 0.01 |
| GM1 | WPT073 | 265117.6 | 8475624 | 0.12 | 0.01 |
| GM1 | WPT074 | 265133.9 | 8475619 | 0.13 | 0.01 |
| GM1 | WPT075 | 265144.8 | 8475610 | 0.10 | 0.01 |
| GM1 | WPT076 | 265161.1 | 8475601 | 0.10 | 0.01 |
| GM1 | WPT077 | 265168.4 | 8475590 | 0.12 | 0.01 |
| GM1 | WPT078 | 265179.3 | 8475584 | 0.09 | 0.01 |
| GM1 | WPT079 | 265182.7 | 8475603 | 0.11 | 0.01 |
| GM1 | WPT080 | 265159.2 | 8475612 | 0.09 | 0.01 |
| GM1 | WPT081 | 265141 | 8475625 | 0.12 | 0.01 |
| GM1 | WPT082 | 265124.7 | 8475637 | 0.12 | 0.01 |
| GM1 | WPT083 | 265108.4 | 8475647 | 0.12 | 0.01 |
| GM1 | WPT084 | 265088.5 | 8475657 | 0.15 | 0.01 |
| GM1 | WPT085 | 265073.9 | 8475668 | 0.13 | 0.01 |
| GM1 | WPT086 | 265061.2 | 8475677 | 0.11 | 0.01 |
| GM1 | WPT087 | 265044.9 | 8475687 | 0.13 | 0.01 |
| GM1 | WPT088 | 265043.1 | 8475687 | 0.13 | 0.01 |
| GM1 | WPT089 | 265035.8 | 8475696 | 0.16 | 0.01 |
| GM1 | WPT090 | 265026.8 | 8475697 | 0.20 | 0.01 |
| GM1 | WPT091 | 265015.9 | 8475707 | 0.17 | 0.01 |
| GM1 | WPT092 | 265008.6 | 8475714 | 0.14 | 0.01 |
| GM1 | WPT093 | 264999.6 | 8475717 | 0.15 | 0.01 |
| GM1 | WPT094 | 264986.9 | 8475721 | 0.16 | 0.01 |
| GM1 | WPT095 | 264979.7 | 8475727 | 0.32 | 0.01 |
| GM1 | WPT096 | 264968.8 | 8475732 | 0.30 | 0.01 |
| GM1 | WPT097 | 264959.7 | 8475736 | 0.24 | 0.01 |
| GM1 | WPT098 | 264947.1 | 8475741 | 0.20 | 0.01 |
| GM1 | WPT099 | 264939.8 | 8475750 | 0.19 | 0.01 |
| GM1 | WPT100 | 264925.3 | 8475754 | 0.16 | 0.01 |
| GM1 | WPT101 | 264916.3 | 8475757 | 0.17 | 0.01 |
| GM1 | WPT102 | 264970.5 | 8475747 | 0.34 | 0.01 |
| GM1 | WPT103 | 264983.2 | 8475739 | 0.34 | 0.01 |
| GM1 | WPT104 | 264992.2 | 8475730 | 0.36 | 0.01 |
| GM1 | WPT105 | 265003 | 8475734 | 0.33 | 0.01 |
| GM1 | WPT106 | 264988.5 | 8475741 | 0.40 | 0.02 |
| GM1 | WPT107 | 264922.5 | 8475473 | 0.16 | 0.01 |
| GM1 | WPT108 | 264909.9 | 8475470 | 0.21 | 0.01 |
| GM1 | WPT109 | 264902.7 | 8475469 | 0.61 | 0.02 |
| GM1 | WPT110 | 264895.5 | 8475468 | 0.77 | 0.02 |
| GM1 | WPT111 | 264886.5 | 8475467 | 0.38 | 0.01 |
| GM1 | WPT112 | 264872 | 8475471 | 0.46 | 0.02 |
| GM1 | WPT113 | 264864.8 | 8475471 | 0.40 | 0.02 |
| GM1 | WPT114 | 264866.7 | 8475458 | 0.38 | 0.01 |
| GM1 | WPT115 | 264859.5 | 8475460 | 0.13 | 0.01 |
| GM1 | WPT116 | 264881 | 8475467 | 0.62 | 0.02 |
| GM1 | WPT117 | 264891.8 | 8475469 | 0.72 | 0.02 |
| GM1 | WPT118 | 264902.7 | 8475466 | 0.53 | 0.02 |
| GM1 | WPT119 | 264911.8 | 8475458 | 0.84 | 0.02 |
| GM1 | WPT120 | 264922.6 | 8475457 | 0.13 | 0.01 |
| GM1 | WPT121 | 264915.6 | 8475438 | 0.15 | 0.01 |
| GM1 | WPT122 | 264895.7 | 8475440 | 0.17 | 0.01 |
| GM1 | WPT123 | 264884.9 | 8475442 | 0.14 | 0.01 |
| GM1 | WPT124 | 264967.6 | 8475660 | 1.20 | 0.02 |
| GM2 | WPT002 | 264865.2 | 8475622 | 0.06 | 0.01 |
| GM2 | WPT003 | 264877.9 | 8475619 | 0.12 | 0.01 |
| GM2 | WPT004 | 264890.5 | 8475621 | 0.15 | 0.01 |
| GM2 | WPT005 | 264903.2 | 8475610 | 0.66 | 0.03 |
| GM2 | WPT006 | 264914 | 8475610 | 0.38 | 0.02 |
| GM2 | WPT007 | 264921.2 | 8475610 | 0.32 | 0.02 |
| GM2 | WPT008 | 264939.3 | 8475604 | 0.22 | 0.01 |
| GM2 | WPT009 | 264948.4 | 8475599 | 0.23 | 0.02 |
| GM2 | WPT010 | 264957.4 | 8475601 | 0.42 | 0.02 |
| GM2 | WPT011 | 264970 | 8475595 | 0.46 | 0.03 |
| GM2 | WPT012 | 264979.1 | 8475592 | 0.37 | 0.02 |
| GM2 | WPT013 | 264984.5 | 8475590 | 0.77 | 0.04 |
| GM2 | WPT014 | 264991.7 | 8475588 | 0.36 | 0.02 |
| GM2 | WPT015 | 265002.5 | 8475592 | 0.40 | 0.02 |
| GM2 | WPT016 | 265013.4 | 8475588 | 1.42 | 0.07 |
| GM2 | WPT017 | 265024.2 | 8475581 | 0.80 | 0.04 |
| GM2 | WPT018 | 265033.3 | 8475574 | 0.88 | 0.04 |
| GM2 | WPT019 | 265044.2 | 8475570 | 0.36 | 0.02 |
| GM2 | WPT020 | 265058.6 | 8475567 | 0.42 | 0.02 |
| GM2 | WPT021 | 265064.2 | 8475552 | 0.60 | 0.03 |
| GM2 | WPT022 | 265066 | 8475545 | 0.74 | 0.04 |
| GM2 | WPT023 | 265068 | 8475528 | 0.52 | 0.03 |
| GM2 | WPT024 | 265078.9 | 8475523 | 0.31 | 0.02 |
| GM2 | WPT025 | 265093.3 | 8475523 | 0.25 | 0.02 |
| GM2 | WPT026 | 265098.8 | 8475506 | 0.22 | 0.01 |
| GM2 | WPT027 | 265109.7 | 8475499 | 0.19 | 0.01 |
| GM2 | WPT028 | 265118.8 | 8475492 | 0.20 | 0.01 |
| GM2 | WPT029 | 265125.7 | 8475529 | 0.21 | 0.01 |
| GM2 | WPT030 | 265116.6 | 8475536 | 0.17 | 0.01 |
| GM2 | WPT031 | 265109.3 | 8475545 | 0.20 | 0.01 |
| GM2 | WPT032 | 265100.2 | 8475558 | 0.27 | 0.02 |
| GM2 | WPT033 | 265091.1 | 8475567 | 0.33 | 0.02 |
| GM2 | WPT034 | 265082 | 8475578 | 0.51 | 0.03 |
| GM2 | WPT035 | 265074.7 | 8475587 | 0.41 | 0.02 |
| GM2 | WPT036 | 265065.6 | 8475594 | 0.56 | 0.03 |
| GM2 | WPT037 | 265060.1 | 8475602 | 0.52 | 0.03 |
| GM2 | WPT038 | 265054.6 | 8475609 | 0.45 | 0.02 |
| GM2 | WPT039 | 265043.8 | 8475615 | 0.28 | 0.02 |
| GM2 | WPT040 | 265034.7 | 8475620 | 0.59 | 0.03 |
| GM2 | WPT041 | 265018.4 | 8475629 | 0.94 | 0.05 |
| GM2 | WPT042 | 265005.7 | 8475638 | 0.33 | 0.02 |
| GM2 | WPT043 | 264996.7 | 8475636 | 0.35 | 0.02 |
| GM2 | WPT044 | 264987.6 | 8475644 | 0.37 | 0.02 |
| GM2 | WPT045 | 264980.5 | 8475638 | 0.86 | 0.04 |
| GM2 | WPT046 | 264967.8 | 8475643 | 0.47 | 0.03 |
| GM2 | WPT047 | 264955.1 | 8475653 | 0.48 | 0.03 |
| GM2 | WPT048 | 264947.8 | 8475658 | 0.40 | 0.02 |
| GM2 | WPT049 | 264935.2 | 8475658 | 0.43 | 0.02 |
| GM2 | WPT050 | 264924.4 | 8475658 | 0.30 | 0.02 |
| GM2 | WPT051 | 264910 | 8475654 | 0.61 | 0.03 |
| GM2 | WPT052 | 264904.5 | 8475659 | 0.99 | 0.05 |
| GM2 | WPT053 | 264895.5 | 8475669 | 0.17 | 0.01 |
| GM2 | WPT054 | 264882.8 | 8475674 | 0.15 | 0.01 |
| GM2 | WPT055 | 264898.6 | 8475720 | 0.14 | 0.01 |
| GM2 | WPT056 | 264905.8 | 8475717 | 0.12 | 0.01 |
| GM2 | WPT057 | 264920.3 | 8475717 | 0.16 | 0.01 |
| GM2 | WPT058 | 264932.9 | 8475715 | 0.17 | 0.01 |
| GM2 | WPT059 | 264949.1 | 8475717 | 0.18 | 0.01 |
| GM2 | WPT060 | 264956.4 | 8475710 | 0.22 | 0.02 |
| GM2 | WPT061 | 264967.3 | 8475702 | 0.25 | 0.02 |
| GM2 | WPT062 | 264985.3 | 8475699 | 0.27 | 0.02 |
| GM2 | WPT063 | 264996.2 | 8475697 | 0.28 | 0.02 |
| GM2 | WPT064 | 265007 | 8475694 | 0.24 | 0.02 |
| GM2 | WPT065 | 265017.9 | 8475688 | 0.44 | 0.02 |
| GM2 | WPT066 | 265026.9 | 8475683 | 0.23 | 0.02 |
| GM2 | WPT067 | 265034.2 | 8475675 | 0.22 | 0.02 |
| GM2 | WPT068 | 265043.3 | 8475670 | 0.20 | 0.01 |
| GM2 | WPT069 | 265050.6 | 8475663 | 0.15 | 0.01 |
| GM2 | WPT070 | 265068.7 | 8475650 | 0.14 | 0.01 |
| GM2 | WPT071 | 265068.7 | 8475650 | 0.14 | 0.01 |
| GM2 | WPT072 | 265077.7 | 8475646 | 0.14 | 0.01 |
| GM2 | WPT073 | 265090.4 | 8475637 | 0.13 | 0.01 |
| GM2 | WPT074 | 265099.5 | 8475630 | 0.13 | 0.01 |
| GM2 | WPT075 | 265103.2 | 8475623 | 0.12 | 0.01 |
| GM2 | WPT076 | 265115.9 | 8475613 | 0.11 | 0.01 |
| GM2 | WPT077 | 265126.7 | 8475612 | 0.13 | 0.01 |
| GM2 | WPT078 | 265135.8 | 8475604 | 0.11 | 0.01 |
| GM2 | WPT079 | 265146.7 | 8475599 | 0.10 | 0.01 |
| GM2 | WPT080 | 265157.6 | 8475592 | 0.12 | 0.01 |
| GM2 | WPT081 | 265168.4 | 8475584 | 0.13 | 0.01 |
| GM2 | WPT082 | 265182.9 | 8475577 | 0.12 | 0.01 |
| GM2 | WPT083 | 265229 | 8475666 | 0.11 | 0.01 |
| GM2 | WPT084 | 265210.8 | 8475686 | 0.11 | 0.01 |
| GM2 | WPT085 | 265189.1 | 8475693 | 0.10 | 0.01 |
| GM2 | WPT086 | 265165.6 | 8475701 | 0.10 | 0.01 |
| GM2 | WPT087 | 265147.4 | 8475717 | 0.11 | 0.01 |
| GM2 | WPT088 | 265125.7 | 8475724 | 0.10 | 0.01 |
| GM2 | WPT089 | 265111.2 | 8475739 | 0.10 | 0.01 |
| GM2 | WPT090 | 265093.1 | 8475746 | 0.09 | 0.01 |
| GM2 | WPT091 | 265084 | 8475753 | 0.09 | 0.01 |
| GM2 | WPT092 | 265080.3 | 8475761 | 0.12 | 0.01 |
| GM2 | WPT093 | 265069.4 | 8475768 | 0.10 | 0.01 |
| GM2 | WPT094 | 265056.8 | 8475771 | 0.09 | 0.01 |
| GM2 | WPT095 | 265042.3 | 8475779 | 0.11 | 0.01 |
| GM2 | WPT096 | 265040.4 | 8475784 | 0.09 | 0.01 |
| GM2 | WPT097 | 265029.6 | 8475790 | 0.11 | 0.01 |
| GM2 | WPT098 | 265027.6 | 8475808 | 0.12 | 0.01 |
| GM2 | WPT099 | 265020.3 | 8475814 | 0.11 | 0.01 |
| GM3 | WPT001 | 264858.1 | 8475609 | 0.13 | 0.01 |
| GM3 | WPT002 | 264870.7 | 8475609 | 0.13 | 0.01 |
| GM3 | WPT003 | 264881.6 | 8475604 | 0.14 | 0.01 |
| GM3 | WPT004 | 264897.9 | 8475595 | 0.23 | 0.01 |
| GM3 | WPT005 | 264907 | 8475591 | 0.22 | 0.01 |
| GM3 | WPT006 | 264919.6 | 8475588 | 0.26 | 0.01 |
| GM3 | WPT007 | 264930.5 | 8475582 | 0.25 | 0.01 |
| GM3 | WPT008 | 264935.8 | 8475591 | 0.47 | 0.02 |
| GM3 | WPT009 | 264939.5 | 8475584 | 0.31 | 0.02 |
| GM3 | WPT010 | 264950.4 | 8475577 | 0.30 | 0.02 |
| GM3 | WPT011 | 264964.8 | 8475575 | 0.26 | 0.01 |
| GM3 | WPT012 | 264975.7 | 8475572 | 0.23 | 0.01 |
| GM3 | WPT013 | 264977.4 | 8475581 | 0.54 | 0.02 |
| GM3 | WPT014 | 264982.8 | 8475579 | 0.25 | 0.01 |
| GM3 | WPT015 | 264997.3 | 8475568 | 0.29 | 0.02 |
| GM3 | WPT016 | 265008.2 | 8475561 | 0.28 | 0.01 |
| GM3 | WPT017 | 265020.9 | 8475557 | 0.55 | 0.02 |
| GM3 | WPT018 | 265029.9 | 8475550 | 0.48 | 0.02 |
| GM3 | WPT019 | 265040.8 | 8475546 | 0.40 | 0.02 |
| GM3 | WPT020 | 265048.1 | 8475539 | 0.62 | 0.02 |
| GM3 | WPT021 | 265048.1 | 8475535 | 1.17 | 0.04 |
| GM3 | WPT022 | 265046.4 | 8475526 | 0.41 | 0.02 |
| GM3 | WPT023 | 265050.1 | 8475517 | 0.63 | 0.02 |
| GM3 | WPT024 | 265060.9 | 8475510 | 0.34 | 0.02 |
| GM3 | WPT025 | 265062.8 | 8475504 | 0.21 | 0.01 |
| GM3 | WPT026 | 265070.1 | 8475495 | 0.18 | 0.01 |
| GM3 | WPT027 | 265079.2 | 8475489 | 0.16 | 0.01 |
| GM3 | WPT028 | 265090.1 | 8475480 | 0.15 | 0.01 |
| GM3 | WPT029 | 265102.7 | 8475477 | 0.17 | 0.01 |
| GM3 | WPT030 | 265132.8 | 8475543 | 0.15 | 0.01 |
| GM3 | WPT031 | 265130.9 | 8475555 | 0.15 | 0.01 |
| GM3 | WPT032 | 265125.4 | 8475562 | 0.15 | 0.01 |
| GM3 | WPT033 | 265112.6 | 8475580 | 0.18 | 0.01 |
| GM3 | WPT034 | 265098.2 | 8475580 | 0.32 | 0.02 |
| GM3 | WPT035 | 265094.5 | 8475593 | 0.32 | 0.02 |
| GM3 | WPT036 | 265089 | 8475598 | 0.39 | 0.02 |
| GM3 | WPT037 | 265081.8 | 8475600 | 0.37 | 0.02 |
| GM3 | WPT038 | 265072.7 | 8475611 | 0.31 | 0.02 |
| GM3 | WPT039 | 265061.8 | 8475620 | 0.26 | 0.01 |
| GM3 | WPT040 | 265050.9 | 8475626 | 0.26 | 0.01 |
| GM3 | WPT041 | 265043.6 | 8475635 | 0.23 | 0.01 |
| GM3 | WPT042 | 265032.7 | 8475642 | 0.20 | 0.01 |
| GM3 | WPT043 | 265027.3 | 8475646 | 0.25 | 0.01 |
| GM3 | WPT044 | 265018.2 | 8475651 | 0.27 | 0.01 |
| GM3 | WPT045 | 265005.5 | 8475660 | 0.28 | 0.01 |
| GM3 | WPT046 | 264998.2 | 8475666 | 0.33 | 0.02 |
| GM3 | WPT047 | 264983.8 | 8475666 | 0.25 | 0.01 |
| GM3 | WPT048 | 264971.3 | 8475658 | 0.55 | 0.02 |
| GM3 | WPT049 | 264971.2 | 8475666 | 0.32 | 0.02 |
| GM3 | WPT050 | 264965.7 | 8475675 | 0.28 | 0.01 |
| GM3 | WPT051 | 264953 | 8475680 | 0.22 | 0.01 |
| GM3 | WPT052 | 264945.8 | 8475680 | 0.15 | 0.01 |
| GM3 | WPT053 | 264936.8 | 8475684 | 0.16 | 0.01 |
| GM3 | WPT054 | 264929.6 | 8475684 | 0.15 | 0.01 |
| GM3 | WPT055 | 264915.1 | 8475685 | 0.16 | 0.01 |
| GM3 | WPT056 | 264902.5 | 8475689 | 0.14 | 0.01 |
| GM3 | WPT057 | 264895.3 | 8475691 | 0.12 | 0.01 |
| GM3 | WPT058 | 264886.2 | 8475700 | 0.12 | 0.01 |
| GM3 | WPT059 | 264893.3 | 8475709 | 0.13 | 0.01 |
| GM3 | WPT060 | 264904.1 | 8475706 | 0.14 | 0.01 |
| GM3 | WPT061 | 264913.1 | 8475706 | 0.14 | 0.01 |
| GM3 | WPT062 | 264927.6 | 8475700 | 0.16 | 0.01 |
| GM3 | WPT063 | 264938.5 | 8475695 | 0.17 | 0.01 |
| GM3 | WPT064 | 264951.1 | 8475691 | 0.19 | 0.01 |
| GM3 | WPT065 | 264960.2 | 8475688 | 0.24 | 0.01 |
| GM3 | WPT066 | 264972.9 | 8475682 | 0.29 | 0.02 |
| GM3 | WPT067 | 264980.1 | 8475677 | 0.55 | 0.02 |
| GM3 | WPT068 | 264994.6 | 8475675 | 0.75 | 0.03 |
| GM3 | WPT069 | 265000 | 8475671 | 0.42 | 0.02 |
| GM3 | WPT070 | 265009.1 | 8475664 | 0.32 | 0.02 |
| GM3 | WPT071 | 265019.9 | 8475660 | 0.20 | 0.01 |
| GM3 | WPT072 | 265030.8 | 8475651 | 0.18 | 0.01 |
| GM3 | WPT073 | 265039.9 | 8475648 | 0.21 | 0.01 |
| GM3 | WPT074 | 265048.9 | 8475642 | 0.17 | 0.01 |
| GM3 | WPT075 | 265058 | 8475635 | 0.14 | 0.01 |
| GM3 | WPT076 | 265068.9 | 8475630 | 0.13 | 0.01 |
| GM3 | WPT077 | 265076.2 | 8475622 | 0.15 | 0.01 |
| GM3 | WPT078 | 265083.4 | 8475615 | 0.14 | 0.01 |
| GM3 | WPT079 | 265092.5 | 8475608 | 0.17 | 0.01 |
| GM3 | WPT080 | 265103.3 | 8475606 | 0.186 | 0.01 |
| GM3 | WPT081 | 265110.7 | 8475593 | 0.19 | 0.01 |
| GM3 | WPT082 | 265118 | 8475584 | 0.16 | 0.01 |
| GM3 | WPT083 | 265127 | 8475580 | 0.14 | 0.01 |
| GM3 | WPT084 | 265134.3 | 8475573 | 0.13 | 0.01 |
| GM3 | WPT085 | 265145.2 | 8475566 | 0.14 | 0.01 |
| GM3 | WPT086 | 265157.9 | 8475558 | 0.13 | 0.01 |
| GM3 | WPT087 | 265170.5 | 8475559 | 0.14 | 0.01 |
| GM3 | WPT088 | 265187.9 | 8475631 | 0.10 | 0.01 |
| GM3 | WPT089 | 265171.5 | 8475642 | 0.11 | 0.01 |
| GM3 | WPT090 | 265151.6 | 8475652 | 0.11 | 0.01 |
| GM3 | WPT091 | 265138.9 | 8475662 | 0.09 | 0.01 |
| GM3 | WPT092 | 265124.4 | 8475671 | 0.10 | 0.01 |
| GM3 | WPT093 | 265104.5 | 8475680 | 0.11 | 0.01 |
| GM3 | WPT094 | 265091.8 | 8475685 | 0.13 | 0.01 |
| GM3 | WPT095 | 265073.7 | 8475694 | 0.10 | 0.01 |
| GM3 | WPT096 | 265062.9 | 8475698 | 0.11 | 0.01 |
| GM3 | WPT097 | 265055.6 | 8475703 | 0.12 | 0.01 |
| GM3 | WPT098 | 265046.5 | 8475709 | 0.14 | 0.01 |
| GM3 | WPT099 | 265037.5 | 8475714 | 0.14 | 0.01 |
| GM3 | WPT100 | 265030.2 | 8475725 | 0.17 | 0.01 |
| GM3 | WPT101 | 265022.9 | 8475727 | 0.23 | 0.01 |
| GM3 | WPT102 | 265017.4 | 8475736 | 0.15 | 0.01 |
| GM3 | WPT103 | 265011.9 | 8475745 | 0.14 | 0.01 |
| GM3 | WPT104 | 265001.1 | 8475753 | 0.32 | 0.02 |
| GM3 | WPT105 | 264997.4 | 8475760 | 0.17 | 0.01 |
| GM3 | WPT106 | 264986.5 | 8475767 | 0.15 | 0.01 |
| GM3 | WPT107 | 264982.9 | 8475771 | 0.14 | 0.01 |
| GM3 | WPT108 | 264975.6 | 8475776 | 0.12 | 0.01 |
| GM3 | WPT109 | 264964.8 | 8475780 | 0.16 | 0.01 |
| GM3 | WPT110 | 264953.9 | 8475785 | 0.11 | 0.01 |
| GM3 | WPT111 | 264944.9 | 8475787 | 0.12 | 0.01 |
| GM3 | WPT112 | 264934.3 | 8475761 | 0.15 | 0.01 |
| GM3 | WPT113 | 264941.6 | 8475754 | 0.15 | 0.01 |
| GM3 | WPT114 | 264950.6 | 8475750 | 0.23 | 0.01 |
| GM3 | WPT115 | 264957.9 | 8475743 | 0.20 | 0.01 |
| GM3 | WPT116 | 264966.9 | 8475739 | 0.26 | 0.01 |
| GM3 | WPT117 | 264977.7 | 8475747 | 0.32 | 0.02 |
| GM3 | WPT118 | 264986.8 | 8475732 | 0.14 | 0.01 |
| GM3 | WPT119 | 264911.5 | 8475484 | 0.15 | 0.01 |
| GM3 | WPT120 | 264899 | 8475480 | 0.29 | 0.02 |
| GM3 | WPT121 | 264889.9 | 8475482 | 0.38 | 0.02 |
| GM3 | WPT122 | 264880.9 | 8475484 | 0.88 | 0.03 |
| GM3 | WPT123 | 264870.1 | 8475482 | 0.75 | 0.03 |
| GM3 | WPT124 | 264862.9 | 8475478 | 0.19 | 0.01 |
| GM3 | WPT125 | 264870 | 8475491 | 0.15 | 0.01 |
| GM3 | WPT126 | 264884.5 | 8475486 | 0.16 | 0.01 |
| GM3 | WPT127 | 264897.1 | 8475484 | 0.13 | 0.01 |
| GM3 | WPT128 | 264908 | 8475481 | 0.12 | 0.01 |
| GM3 | WPT129 | 264917 | 8475481 | 0.15 | 0.01 |
| GM3 | WPT130 | 264910 | 8475455 | 0.16 | 0.01 |
| GM3 | WPT131 | 264901 | 8475457 | 0.26 | 0.01 |
| GM3 | WPT132 | 264888.4 | 8475456 | 0.21 | 0.01 |
| GM3 | WPT133 | 264875.7 | 8475456 | 0.25 | 0.01 |
| GM3 | WPT134 | 264868.5 | 8475456 | 0.34 | 0.02 |
| GM3 | WPT135 | 264854.1 | 8475462 | 0.12 | 0.01 |
| GM3 | WPT136 | 264852.4 | 8475447 | 0.10 | 0.01 |
| GM3 | WPT137 | 264863.3 | 8475438 | 0.11 | 0.01 |
| GM3 | WPT138 | 264870.5 | 8475434 | 0.11 | 0.01 |