616

internal report

A Bollhöfer, A Beraldo, K Pfitzner, A Esparon & G Carr

February 2013

Release status – unrestricted

Project number – RES-2005-001

Pre-mining radiological conditions in the Ranger Project Area

This page has been left blank intentionally.

# Pre-mining radiological conditions in the Ranger Project Area

A Bollhöfer, A Beraldo, K Pfitzner, A Esparon & G Carr

Supervising Scientist Division

GPO Box 461, Darwin NT 0801

February 2013

Project number RES-2005-001

Registry File SG2005/0127

(Release status – unrestricted)

How to cite this report:

A Bollhöfer, A Beraldo, K Pfitzner, A Esparon & G Carr 2013. Pre-mining radiological conditions in the Ranger Project Area. Internal Report 616, February, Supervising Scientist, Darwin.

Project number – RES-2005-001

Location of final PDF file in SSDX Sharepoint:

http://publications.nt.environment.gov.au/PublicationWork/Publications and Productions/Internal Reports (IRs)/Nos 600 to 699/IR616_Pre-mining radiological conditions

Location of all key data files for this report in SSDX:

http://ssd.nt.environment.gov.au/SSDX/Chemical and Radiological Site Assessment/SPIRE Radiological/SPIRE Anomaly 2

Authors of this report:

Andreas Bollhöfer – Environmental Research Institute of the Supervising Scientist, GPO Box 461, Darwin NT 0801, Australia

Annamarie Beraldo – Environmental Research Institute of the Supervising Scientist, GPO Box 461, Darwin NT 0801, Australia. Address at time of publication: Sinclair Knight Merz, GPO Box 2145 Parap NT 0804, Australia

Kirrilly Pfitzner – Environmental Research Institute of the Supervising Scientist, GPO Box 461, Darwin NT 0801, Australia

Andrew Esparon – Environmental Research Institute of the Supervising Scientist, GPO Box 461, Darwin NT 0801, Australia

G Carr – Environmental Research Institute of the Supervising Scientist, GPO Box 461, Darwin NT 0801, Australia. Address at time of publication: Northern Land Council, GPO Box 1222, Darwin, NT 0801, Australia

The Supervising Scientist is part of the Australian Government Department of Sustainability, Environment, Water, Population and Communities.

© Commonwealth of Australia 2013

Supervising Scientist

Department of Sustainability, Environment, Water, Population and Communities

GPO Box 461, Darwin NT 0801 Australia

This work is copyright. Apart from any use as permitted under the Copyright Act 1968, no part may be reproduced by any process without prior written permission from the Supervising Scientist. Requests and enquiries concerning reproduction and rights should be addressed to Publications Enquiries, Supervising Scientist, GPO Box 461, Darwin NT 0801.

e-mail: publications_ssd@environment.gov.au

Internet: www.environment.gov.au/ssd   (www.environment.gov.au/ssd/publications)

The views and opinions expressed in this report do not necessarily reflect those of the Commonwealth of Australia. While reasonable efforts have been made to ensure that the contents of this report are factually correct, some essential data rely on references cited and/or the data and/or information of other parties, and the Supervising Scientist and the Commonwealth of Australia do not accept responsibility for the accuracy, currency or completeness of the contents of this report, and shall not be liable for any loss or damage that may be occasioned directly or indirectly through the use of, or reliance on, the report. Readers should exercise their own skill and judgment with respect to their use of the material contained in this report.

Printed and bound in Darwin NT by Supervising Scientist Division

# Contents

Executive summary	ix

1  Introduction	1

1.1  Background	1

1.1.1  Ranger Environmental Requirements	1

1.1.2  Use of airborne gamma surveys in exploration and site characterisation	2

1.1.3  Existing pre-mining ground data at Ranger	3

1.2  Objective	4

2  Selection of an undisturbed radiologically anomalous site	5

2.1  The MODAT database	5

2.2  Existing airborne gamma surveys	6

2.2.1  The Alligator River Geophysical Survey	7

2.2.2  1996 Rio Tinto Airborne Gamma survey	8

3  Methods	10

3.1  1976 airborne gamma survey specifications	10

3.2  1997 airborne gamma survey specifications	10

3.3  Ground truthing	11

3.3.1  Gamma dose rate measurements	11

3.3.2  Radon flux density measurements	13

3.3.3  Radon in air measurements	15

3.3.4  Soil activity concentration measurements	16

4  Results	18

4.1  1976 airborne gamma survey data	18

4.2  1997 airborne gamma survey	19

4.3  Gamma dose rates measured in the field	20

4.4  Radon	21

4.4.1  Radon flux densities	21

4.4.2  Radon activity concentration in air	21

4.5  Soil activity concentrations	22

4.5.1  Laboratory HPGe measurements	22

4.5.2  In situ NaI measurements	25

5  Conversion factors	26

5.1  Terrestrial gamma dose rate to 226Ra soil activity concentration	26

5.2  226Ra soil activity concentration to 222Rn flux density	27

5.3  Terrestrial gamma dose rate to 222Rn flux density	28

5.4  Converting the 1976 AGS U data to terrestrial dose rate in the field	29

5.4.1  Modelling the 1997 AGS line data to the field gamma dose rate data	30

5.4.2  Modelling the 1976 raster to the 1997 raster airborne gamma data	35

5.4.3  Converting 1976 eU data to terrestrial gamma dose rates and radon flux densities	36

6  Discussion	37

6.1  Pre-mining external γ dose rates, radon flux densities and 226Ra activity concentrations in the greater Ranger region	37

6.2  Orebody 1, Orebody 3 and Anomaly 2	40

6.2.1  Gamma dose rates and 226Ra activity concentrations	40

6.2.2 Radon flux density	40

6.2.3  Airborne radon activity concentration	41

6.2  Djalkmara and Corridor Creek Land Application Areas	42

6.3  Environmental background	42

7  Conclusions	44

References	45

Appendix A  Results of external gamma dose rate and in-situ soil activity concentration measurements	49

Appendix B  Radon flux density	88

Appendix C  Airborne radon activity concentrations	90

Appendix D  Soil activity concentrations	92

## Figures

Figure 1  Contours of total counts measured during the 1969 AGS over the Ranger area (Ryan 1972)	3

Figure 2  Known uranium (and uranium/gold) anomalies in the ARR (from Pfitzner & Martin 2003)	5

Figure 3  Spatial coverage of the existing AGS data available over the ARR, as available from NTGS (from Pfitzner & Martin 2003)	6

Figure 4  (a) Ikonos satellite data (2001, displayed as false colour composite) subset shown in Figure 3 and (b) corresponding 1976 equivalent uranium (eU) AGS raster data, with MODAT uranium anomalies overlaid.	8

Figure 5  Eupene et al (1975) map of aerial radiometric contours of total count (TC) overlaid on airborne data from the 1976 AGS.	9

Figure 6  Extent and flight lines of the 1997 Rio Tinto airborne gamma survey, overlaid on an aerial photograph from March 2007.	9

Figure 7  Location of the 2007–08 ground gamma survey points	12

Figure 8  Location of the 2007–08 ground gamma survey points overlaid on the total counts signal above Anomaly 2 from the 1997 AGS	13

Figure 9  Location of the radon exhalation sampling points overlaid on the total counts signal above Anomaly 2 from the 1997 AGS	15

Figure 10  Set up of the GS-512 field gamma spectrometer	16

Figure 11  Counts (> 25 counts per sec) in the uranium channel of the 1976 AGS over the whole scene	18

Figure 12  Counts (>250 counts per second) in the uranium channel of the 1976 AGS  over the Ranger mine region	19

Figure 13  Counts (>250 counts per second) in the uranium channel of the 1976 AGS over Anomaly 2	19

Figure 14  Counts (>4500 counts per second) in the TC channel of the 1997 AGS over Anomaly 2	20

Figure 15  Field gamma dose rates measured over Anomaly 2 between 2007–09	20

Figure 16  Radon flux densities measured over the Anomaly 2 area between 2007–09	21

Figure 17  Radon activity concentrations measured in air at 30 cm, 50 cm and 150 cm above ground over the Anomaly 2 area for the dry season 2009	22

Figure 18  (a) Measured 234Th activity concentration plotted against derived 238U for sample activity concentrations between (a) 40 – 40  000 Bq∙kg-1, and (b) 40 – 10 000 Bq∙kg-1	23

Figure 19  (a) 226Ra activity concentration and (b) 226Ra/238U activity concentration ratio plotted against 238U activity concentrations in samples from Anomaly 2	23

Figure 20  Calculated terrestrial gamma dose rates [µGy∙hr-1] from laboratory HPGe radionuclide activity concentration measurements plotted versus the measured terrestrial gamma dose rates at sites 1A-25A.	25

Figure 21  Calculated terrestrial gamma dose rates (using equation 2) in µGy∙hr-1 plotted versus measured terrestrial gamma dose rates performed in August 2007 using the NaI detector.	25

Figure 22  In situ and laboratory measured soil 226Ra activity concentration plotted against the measured terrestrial gamma dose rates, all data shown.	26

Figure 23  In situ (NaI) (o) and laboratory (HPGe) (•) measured soil 226Ra activity concentration plotted against measured terrestrial gamma dose rates (< 2 µGy∙hr-1)	27

Figure 24  222Rn flux densities plotted against the measured (HPGe) 226Ra soil activity concentrations. Only the loamy sand and fine gravel sample types are included in the line of best fit.	28

Figure 25  222Rn flux densities plotted against the measured (in situ) terrestrial gamma dose rates.	29

Figure 26  Shapefile created in ArcGIS for the 2007-2009 ground survey (grey), flight lines and flight line numbers (black arrows) and individual ground survey points (orange)	31

Figure 27  Variable buffers overlaid on shapefile created in ArcGIS for the 2007–2009 ground survey.	32

Figure 28  Coefficient of determination R2 obtained between average ground gamma dose rates for varying buffer radii measured in the field and TC measured in 1997 on board the plane	32

Figure 29  Correlation between averaged terrestrial gamma dose rates and the TC (s-1) channel of the 1997 AGS. Buffer radius of 90 m, n=54.	33

Figure 30  Correlation between averaged terrestrial gamma dose rates and eU (Bq∙kg-1) channel of the 1997 AGS. Buffer radius r=90 m, n=54.	33

Figure 31  Buffers (n=29) with even distribution of ground survey points chosen to establish the correlation between the ground survey and the 1997 AGS data. Buffer radius r=90 m	34

Figure 32 Correlation between averaged terrestrial gamma dose rates and the TC (s-1) channel of the 1997 AGS for buffers with evenly distributed ground survey points only. r = 90 m, n = 29	34

Figure 33  Correlation between averaged terrestrial gamma dose rates and eU (Bq∙kg-1) channel of the 1997 AGS for buffers with evenly distributed ground survey points only. r=90 m, n=29.	35

Figure 34  Full extent of the 1997 raster imagery. The total counts of the 1997 AGS are shown, with light colours indicating high values.	35

Figure 35  Averaged eU data (Bq∙kg-1) from the 1997 AGS plotted versus the 1976 eU data (s-1) of the respective grid cell.	36

Figure 36  Shapefiles overlaid on aerial photo of the greater Ranger region (left) and on the counts in the eU channel of the 1976 AGS of the same extent (right).	38

Figure 37  Shapefiles overlaid on aerial photo of Ranger only (left) and on the counts in the eU channel of the 1976 AGS of the same extent (right)	39

Figure 38  Radon activity concentrations measured in air at 30 cm, 50 cm and 150 cm above ground over the Anomaly 2 area for the dry season 2009.	41

## Tables

Table 1  Uranium concentrations, terrestrial gamma dose rates 226Ra activity concentrations and radon flux densities determined for orebodies 1 and 3 and background sites from Kvasnicka and Auty (1994)	4

Table 2  Survey parameters of the 1976 Rio Tinto Alligator River geophysical survey	10

Table 3  Survey parameters of the 1997 Rio Tinto Ranger uranium mine AGS flown by World Geoscience Corporation Ltd	11

Table 4  Details of the instruments used for the gamma dose rate surveys at Anomaly 2	11

Table 5  Conversion factors used to calculate 238U, 232Th and 40K activity concentrations and expected terrestrial gamma dose rates from given soil uranium, thorium and potassium concentrations	24

Table 6  Comparison of typical footprints for ground-based and airborne gamma surveys	30

Table 7  Pre-mining mean (95% confidence intervals) external gamma dose rates E [µGy∙hr-1] (including cosmic component), 222Rn flux densities and soil 226Ra activity concentrations in the Ranger area	37

Appendix A1  γ dose rates and in-situ γ-spectrometry results from Anomaly 2 survey (4 pixels at 70 m x 70 m each) conducted 21–24 August 2007.	49

Appendix A2  γ dose rate results from Anomaly 2A survey conducted 21-24 July 2008. Measured values were read off the analogue dial in the field, calculated values used the calibration equation.	53

Appendix A3  γ dose rate results from Anomaly 2B survey (northeast of Anomaly 2A) conducted 1–5 September 2008	67

Appendix A4  γ dose rate results from survey between Anomalies 2A and 2B, and on top of Anomaly 2C (southwest of Anomaly 2A), conducted 8–10 October 2008	77

Appendix B  Radon flux density results for individual charcoal canisters deployed during Anomaly 2 survey conducted 14–17 July 2009	88

Appendix C  Results of airborne radon activity concentrations [Bq∙m-3] for sites 1A-25A	90

Appendix D  Results of soil activity concentrations [Bq∙kg-1] and gamma dose rate measurements [µGy∙hr-1] for sites 1A-25A at Anomaly 2.	92

# Executive summary

Ground gamma surveys, radon exhalation and soil 226Ra activity concentration measurements were conducted between 2007 and 2009 over an undisturbed analogue site, Anomaly 2, to the south of Ranger mine. This anomaly exhibits radiation levels that are higher than typical background levels in the region, similar to the outcropping orebodies 1 and 3, that have been mined out between 1981 and 2012. The purpose of the measurements was to groundtruth a historic airborne gamma survey (AGS) that was flown over the area in 1976, before mining started, to be able to determine the pre-mining radiation source term and extrapolate to the whole extent of the AGS including the unmined RPA.

Algorithms were developed to upscale the ground gamma data in ArcGIS to make the spatial resolution comparable to the resolution of the 1976 AGS (300 m line spacing). This was done via an intermediate step, correlating the ground data with data from a higher resolution AGS flown in 1997 by Rio Tinto (100 m line spacing) that also covered the area of Anomaly 2.

The minimum footprint area that can be assessed is set by the optimum buffer radius determined when up-scaling the external gamma dose rates measured on ground to the AGS data. For the current case this is approximately 4 ha. Thus, the correlation models developed allow estimates to be made of the pre-mining baseline gamma dose rates, 226Ra soil activity concentrations and 222Rn fluxes for any selected area larger than 4 ha covered by the 1976 AGS over the greater Ranger area.

Comparison with published data on external gamma dose rates, 226Ra soil activity concentrations and 222Rn exhalation flux densities in the Ranger region before mining started shows that our model estimates are in good agreement with radiation and radionuclide activity levels estimated previously via direct measurement on top of orebody 3 and orebody 1, and from previous environmental radiation surveys.

The GIS model will also allow an estimate of pre-mining uptake of uranium series radionuclides into biota over the footprint of the Ranger mine, assuming secular equilibrium of the radionuclides in soils and using uptake factors determined for bushtucker in the region (eg Martin et al 1998, Ryan et al 2005, Doering et al in prep). This will facilitate the calculation of pre-mining ingestion doses to humans from the consumption of traditional foods harvested on site, in addition to an estimate of the internal and external radiation doses to wildlife.

Our approach for determining pre-mining radiological conditions can be used at any mine and industrial site where historical AGS data are available and an undisturbed analogue exists to groundtruth the AGS data. Areal extent and thus resources for the ground survey will ultimately depend on the resolution of the historic AGS.

# 1  Introduction

## 1.1  Background

### 1.1.1  Ranger Environmental Requirements

The Environmental Requirements (ERs) for the Ranger uranium mine set out the Commonwealth’s environmental protection conditions with which the company (Rio Tinto) must comply. These are conditions of the Authority to mine prescribed substances on the Ranger Project Area, issued under section 41 of the Atomic Energy Act 1953. The Primary Environmental Objectives of the ERs are to ensure that World Heritage attributes and biodiversity are maintained and the health of ecosystems and members of the public are protected during operation of the mine, and to rehabilitate the Ranger Project Area to:

an environment similar to the adjacent areas of Kakadu National Park such that, in the opinion of the Minister with the advice of the Supervising Scientist, the rehabilitated area could be incorporated into the Kakadu National Park (Commonwealth of Australia 1999).

With regards to the radiological conditions after rehabilitation of the Ranger Project Area the major objectives are:

stable radiological conditions on areas impacted by mining so that, the health risk to members of the public, including traditional owners, is as low as reasonably achievable; members of the public do not receive a radiation dose which exceeds applicable limits recommended by the most recently published and relevant Australian standards, codes of practice, and guidelines; and there is a minimum of restrictions on the use of the area.

The ‘Code of Practice and Safety Guide for Radiation Protection’ and ‘Radioactive Waste Management in Mining and Mineral Processing’ (ARPANSA 2005) are the most recently published and relevant documents for benchmarking the radiation protection objectives of the ERs. The Code

applies to the control of occupational and public radiation exposures, and the management of radioactive waste generated, at all stages of mining and mineral processing from exploration to final site rehabilitation

and provides a regulatory framework

to manage the protection of workers, members of the public and the environment from harmful effects of radiation exposures arising from mining or mineral processing and from the waste resulting from these activities both now and in the future.

The Code specifies a public dose limit of 1 mSv in a year, which is derived from ARPANSA’s Recommendations for Limiting Exposure to Ionizing Radiation (2002). In addition, both the Code and Safety Guide make clear that the principle of optimisation of protection is to be applied to radiation practices, including site rehabilitation, to help ensure that the magnitude of individual doses, the number of people exposed, and the likelihood of incurring exposures are kept as low as reasonably achievable, economic and social factors being taken into account.

Natural background sources are generally excluded from regulatory control, and the doses received from natural background sources are not amenable to dose limits. The ERs state that

Radiation doses received from natural background sources or as the result of undergoing medical procedures are not subject to the system and are not to be included in the calculation of radiation doses.

Consequently, radiological conditions in the area before mining commenced need to be established in order to determine the above natural background magnitude of exposure to radiation after rehabilitation of Ranger mine. Such a baseline assessment needs to summarise all potential exposure pathways, which include the terrestrial gamma, the inhalation and the ingestion pathways.

In a high natural background radiation area such as the area around Ranger mine, where uranium orebodies 1 and 3 and several other anomalies are known to exist (Eupene et al 1975, Hegge et al 1980), determining the difference between post-rehabilitation and pre-mining annual radiation doses presents a challenge. Although there are some early studies that assessed the uranium mineralisation that occurred at and around the Ranger ore bodies, these studies were usually focussed on determining the radiation levels and the extent of the outcropping anomaly or orebody (eg Eupene et al 1975), rather than determining an area wide average gamma dose rate or soil uranium activity concentration. In addition, although many pre-mining environmental studies have been conducted (see for example Conway et al 1974) they provide relatively little quantitative radiological data that are spatially referenced appropriately to accurately assess the baseline radiological conditions in the greater Ranger region. Only Kvasnicka (1993) and Kvasnicka and Auty (1994) have reported outcomes from a pre-mining dose assessment for the Ranger uranium mine.

### 1.1.2  Use of airborne gamma surveys in exploration and site characterisation

Airborne gamma surveys (AGS) coupled with ground truthing surveys have previously been used for area wide assessments of current radiological conditions at rehabilitated and historic mine sites. Martin et al (2006) for example determined the radiological conditions at the rehabilitated Nabarlek mine through extensive groundtruthing of an AGS flown in July 1997 at a nominal line spacing of 100 m and a flying height of approximately 50 m. In 2000, an AGS was flown across the South Alligator River valley at a line spacing and flying height of 50 m (Pfitzner & Martin, 2000). This survey was flown to identify location and magnitude of radiological contamination in the South Alligator River valley from uranium mining and milling activities in the 1950s and 60s and to aid in rehabilitation planning for the area (Pfitzner et al 2001, Bollhöfer et al 2002). At the historic Sleisbeck mine an AGS was flown in August 2002, and geophysical data were collected along 25 m spaced flight lines at an average flying height of only 40 m (Pfitzner et al 2003, Bollhöfer et al 2007a, 2008). A similar tight line spacing of the AGS was used for an assessment of the radiological conditions at the old Rum Jungle mine site in October 2006 (Bollhöfer et al 2007a). This is an unusually tight line spacing but it is required for mine site assessment, where features can be very small in size, often less than 0.1 ha.

In contrast, surveys for mineral exploration are usually flown at a much coarser resolution with a line spacing in the order of 500 m and flying height > 100 m. AGSs were first extensively used for uranium exploration in 1949 in the Northwest Territories of Canada (Darnley 1972). In the Pine Creek Geosyncline, airborne gamma spectrometry has also been the principle tool for uranium exploration (Tucker et al 1980). The Ranger orebodies for example were discovered during an AGS flown in 1969 by Noranda (Australia) (Ryan 1972). Large anomalies were also detected over Nabarlek (Tipper & Lawrence 1972) and Koongarra in 1970 (Foy & Pedersen 1975). The Rum Jungle South uranium orebody was discovered in late 1959 by ground follow-up of weak radiometric anomalies detected by AGSs flown in 1952, 1956 and 1957 (AAEC 1963, Berkmann 1968).

Despite the coarser resolution of AGSs flown for mineral exploration compared to site remediation assessment, the method is still sufficient to detect and qualitatively map landscape scale changes in radiological conditions. An AGS flown in the late 1970s by the Bureau of Mineral Resources for example (1500 m line spacing, 150 m flying height) detected large radiometric anomalies at the former Rum Jungle mines, due to contamination of large areas with uranium bearing material (Tucker et al 1980). Despite this it was not until the late 1990s that AGS were routinely used to assess the radiological status of historical mine sites or other areas contaminated by naturally occurring radioactive material (NORM) residues (eg Winkelmann et al 2001, Coetzee et al 2006, Martin et al 2006, Bollhöfer et al 2008).

### 1.1.3  Existing pre-mining ground data at Ranger

Ryan (1972) and Eupene et al (1975) published qualitative results of an AGS flown by Noranda in 1969 in the greater Ranger region. Contour lines are shown of the total counts measured during that AGS (Figure 1).

Figure 1  Contours of total counts measured during the 1969 AGS over the Ranger area (Ryan 1972).

Apart from Anomalies 1 and 3 (now pits 1 and 3), the data confirm the location of additional radiological anomalies that exist in the greater Ranger region. Anomaly 4 to the north of orebody 3 is now located within the Djalkmarra land application area and Anomaly 5 is located within the Corridor Creek land application area (see also Bollhöfer et al 2010). The pre-mining data in particular highlight an existing anomaly to the south of the Ranger lease, which was called Anomaly 2.

Subsequent work on the ground at Ranger focussed on mapping the area and delineating the extent and magnitude of the radiological anomalies, and indicated the possible presence of substantial mineralisation (Eupene et al 1975). The first proposal for mining of uranium in the Alligator Rivers Region came jointly from the then Australian Atomic Energy Commission and Ranger Uranium Mines Pty Ltd for the development of the Ranger deposits (Johnston & Needham in Supervising Scientist 1999). An Inquiry into this proposal, the Ranger Uranium Environmental Inquiry (RUEI), or Fox Inquiry, was set up by the Commonwealth Government in 1975. Although the Inquiry investigated site specific environmental aspects of the proposed Ranger project, little data on the radiation background in the region is given in the two reports. For the lowland surrounding the Ranger prospect, a typical background external gamma dose rate of 0.1 µGy∙hr-1 is given.

Data from an assessment of the pre-mining radiation background for the Ranger uranium mine are published in Kvasnicka (1993) and Kvasnicka and Auty (1994). A summary of uranium concentrations, terrestrial gamma dose rates 226Ra activity concentrations and radon flux densities given in these publications is shown in Table 1.

Table 1  Uranium concentrations, terrestrial gamma dose rates 226Ra activity concentrations and radon flux densities determined for orebodies 1 and 3 and background sites from Kvasnicka and Auty (1994).

|  | U3O8 [%] | γ-dose rate [µGy/hr] | 226Ra [Bq/g] | Rn [Bq/m2/s] |
| --- | --- | --- | --- | --- |
| Orebody 3 | 0.037 ± 0.030a | 0.58 | 1.35 ± 1.09c | 2.5 ± 2.0e |
| Orebody 1 | 0.061 ± 0.063a | 0.96b | 2.23 ± 2.30d | 4.1 ± 3.4e |
| Background |  | 0.06 – 0.08 | 0.07 – 0.12 | 0.13 – 0.22e |

a: determined from results of U3O8 in the top 1 m of core samples from exploratory drilling on the orebodies

b: determined from the ratio of U3O8 of orebodies 1 and 3, multiplied by the dose rate measured on top of orebody 3.

c: determined from the measured γ-dose rate on orebody 3.

d: determined from the ratio of U3O8 of orebodies 1 and 3 and measured γ-dose rate on orebody 3

e: determined using a radon flux density per radium activity concentration ratio of 1.85 (Bq/m2/s)/(Bq/g) measured on orebody 3

## 1.2  Objective

The objective of this study was to use and groundtruth historic AGS data to retrospectively determine the pre-mining radiological conditions in the greater Ranger region. This required identifying a suitable undisturbed radiological anomaly from the historic AGS data, groundtruthing the AGS data for the anomaly through field measurements and then extrapolating the groundtruthed data for the anomaly to the greater Ranger region.

# 2  Selection of an undisturbed radiologically anomalous site

## 2.1  The MODAT database

The Mineral Occurrence Database (MODAT) was developed within the Northern Territory Geological Survey to give spatially accurate point data of known mineral occurrences within the Northern Territory (NTGS 2007). More than 2800 metallic and non-metallic mineral occurrences are currently listed in this database.

The database also provides an overview of radiation anomalies in the Northern Territory and Pfitzner and Martin (2003) have investigated this database with a particular focus on uranium (and thorium) commodities in the Alligator Rivers Region. They have identified 198 known commodities in the Alligator Rivers Region, 164 of which are uranium anomalies of various sizes from 1 tonne to 10000 tonnes of uranium contained in the deposit. These uranium deposits were also classified according to their status.

Operating Mine: significant mineral deposit that is currently being mined.

Abandoned Mine: significant mineral deposit with recorded past production.

Prospect: mineral deposit with an exploration history but no production recorded.

Mineral Occurrence: minor mineral deposit, no production or significant exploration history.

Figure 2 shows the uranium anomalies in the Alligator Rivers Region with some of the anomalies annotated. For more detailed information, refer to Pfitzner and Martin (2003).

Figure 2  Known uranium (and uranium/gold) anomalies in the ARR (from Pfitzner & Martin 2003).

Ranger 1 No 1 and 3 are examples of an operating uranium mine, the historic Sleisbeck mine and Nabarlek are examples of abandoned (and now rehabilitated) mine sites. The Koongarra, Ranger 1 No 2, 4 and 9 and the North Ranger 1 uranium deposits are examples of known uranium prospects in the Alligator Rivers Region that have a an exploration history but have no production recorded. As the purpose of this study was to determine pre-mining radiological conditions, it was these types of deposits, deposits that were not disturbed significantly over the past three decades, that were of interest to our project.

## 2.2  Existing airborne gamma surveys

Existing airborne gamma-ray survey (AGS) data covering the ARR were reviewed in order to evaluate potential undisturbed anomalies that could be groundtruthed as a pre-mining analogue to the Ranger ore bodies.

Some AGS data is available free of charge from the Northern Territory Geological Survey, and Figure 3 shows the extent of the available data in addition to the extent of high resolution AGS data acquired over the rehabilitated Nabarlek mine (Martin et al 2006), the historic Sleisbeck area (Pfitzner et al 2003, Bollhöfer et al 2008), and the upper South Alligator River valley (Pfitzner & Martin 2001, Pfitzner et al 2001a,b). Not the entire Alligator Rivers Region has been surveyed, and only the Koolpin and Alligator River Geological surveys were flown before mining started, in 1974 and 1976, respectively. The Ranger 1 deposits were covered by the Alligator River Geological Survey only.

To avoid correcting for inter-survey differences in resolution, flying height, detector volume and other parameters, ideally the pre-mining AGS data from the Ranger deposits and data from a potential analogue should have been acquired during the same survey, using the same survey specifications and instrumentation. Consequently, it was decided to study the Alligator River Geological Survey in more detail (labelled Alligator in Figure 3) and investigate whether this survey delineated radiation anomalies that may be suitable as a pre-mining analogue for the Ranger 1 and 3 orebodies.

Figure 3  Spatial coverage of the existing AGS data available over the ARR, as available from NTGS (from Pfitzner & Martin 2003).

### 2.2.1  The Alligator River Geophysical Survey

The Alligator River Geophysical Survey was flown in 1976. The 1976 AGS data were acquired from Rio Tinto by the NT Government, and are available on the public domain. Data were re-processed in 2000 by the Northern Territory Geological Survey (NTGS) and then resampled by NTGS at a pixel size of 70 m x70 m in 2003. These data were acquired by eriss from the Northern Territory Geological Survey (NTGS) in 2003, and were re-processed in 2005 and then re-supplied by the Northern Territory Department of Mines and Energy in 2006. Figure 3 shows the coverage of the Alligator River Geophysical Survey.

During the conceptual stage of this project, the Koongarra uranium deposit was considered a possible location as a pre-mining analogue for Ranger mine. The deposit was discovered in 1969 during several AGS flown by Noranda Ltd in the Alligator Rivers Region, which also identified other high intensity anomalies in the Ranger ore body 1 region (Ryan 1972, Eupene et al 1975).

Subsequent groundtruthing of the identified anomaly at Koongarra showed a surface radiation anomaly that increased 4-fold in intensity in 3.4 m depth below the surface (Giblin 2004). The uranium mineralisation at Koongarra occurs in two ore bodies separated by about 100 m, with the primary ore mineral being pitchblende (UO2). Koongarra orebody 1 is covered by a 30 m deep layer of weathered schists (Dickson & Snelling, 1980) with secondary mineralisation present from just below the surface to the base of the weathered zone approximately 25–30 m down. Orebody 2 is about 50 m below the surface with only minor development of secondary mineralisation (Snelling 1980).

A comparison of signal intensity in the 1976 AGS with the location of known uranium occurrences in the MODAT database (Figure 4a) suggested that the signal in the AGS from the Koongarra orebodies was weak (Figure 4b), due to the fact that much of the mineralisation of the Koongarra deposit is covered by superficial sands, sometimes several metres thick. Consequently, the gamma signal from the uranium ore body is attenuated and weak at the surface and the signal measured on board the plane is small. It is important to note that generally, the gamma signal measured in air originates from radionuclides located in the top 0.5 m of the soil, while deeper lying radionuclides only contribute a few percent or less (depending on photon energy) to the signal (ICRU 1994, Saito & Jacobs 1994).

In contrast, Ranger 1 Anomaly 2 to the south of the Ranger lease is outcropping in places and characterised on ground by three strong separate radiometric anomalies (Hegge et al 1980). It also exhibits a strong airborne gamma signal in the 1976 data, as can be seen in Figure 4b. Anomaly 2 has been excluded from the Ranger Project Area, following the recommendations of the Ranger Uranium Environmental Inquiry in 1975. Although some drilling occurred on the anomaly (Ryan 1972), it is largely undeveloped.

In Figure 5 (left) radiation contours published in Eupene et al (1975), which show the total counts acquired during the 1969 Noranda airborne survey of Ranger, are overlaid on the airborne gamma data from the 1976 Alligator Rivers Geophysical Survey. In Figure 5 (right) the pre-mining 1976 airborne signal in the Ranger vicinity by extent and intensity (top 70% of values found in Ranger subset are shown coloured) is overlaid on the land surface image acquired by the IKONOS satellite in 2001 (from Esparon et al 2009). Based on the assessment of the historical AGS data it was decided to obtain groundtruthed data in the greater region of Anomaly 2 and to achieve this objective, an extensive fieldwork program to the south of the Ranger lease was commissioned in 2007.

Figure 4  (a) Ikonos satellite data (2001, displayed as false colour composite) subset shown in Figure 3 and (b) corresponding 1976 equivalent uranium (eU) AGS raster data, with MODAT uranium anomalies overlaid. Light colours in (b) indicate a high intensity signal in the eU channel. Note the dark colour in the Koongarra region.

### 2.2.2  1996 Rio Tinto Airborne Gamma survey

In 2008 AGS data was made available to SSD by ERA. This AGS was commissioned by ERA and flown by World Geoscience Corporation Ltd in 1997, 20 years after the Alligator Rivers Geophysical Survey, at a lower flying height and a higher spatial resolution than the 1976 survey. The survey covered the Anomaly 2 area and it was considered appropriate to investigate whether data from that survey could be used for groundtruthing of the 1976 AGS as well. Initially, the data was used to delineate the exact location of the Anomalies.

Figure 6 shows the extent of the 1997 AGS. Although the survey included all of the active mining area, including the land application areas and Jabiru East, only the total counts (TC) data measured on board the plane to the south of the lease are shown, with the red colour indicating count rates above 13 500 counts per second. The locations of Anomalies 2A and 2B are clearly visible in this image.

N

N

Figure 5  Eupene et al (1975) map of aerial radiometric contours of total count (TC) overlaid on airborne data (left) from the 1976 AGS. In addition, 1976 TC data with the top 70% of the values coloured are overlaid on an IKONOS (2001, displayed as true colour) optical satellite image on the right.

Figure 6  Extent and flight lines of the 1997 Rio Tinto airborne gamma survey, overlaid on an aerial photograph from March 2007. Only the total counts measured during the AGS south of the Ranger lease are shown. Anomalies 2A and 2B are clearly visible. A third Anomaly, 2C, lies to the southwest of Anomaly 2A.

# 3  Methods

## 3.1  1976 airborne gamma survey specifications

Figure 3 shows the coverage of the Alligator River Geophysical Survey overlaid on Ikonos satellite data from the Alligator Rivers Region. The survey was flown in 1976 by Geometrics, and, both magnetic and radiometric data were acquired. The data was then obtained by the Northern Territory Geological Survey from Rio Tinto in September 2000. Data were processed by the NTGS and supplied to eriss in 2003. Data were then re-processed in 2005 for the Northern Territory Geological Survey by Intrepid Geophysics, separated into magnetic and radiometric files and the flight direction added, and re-supplied to eriss in 2006.

Although the NTGS data has had a height correction applied (the digital numbers have changed from integer to floating point) no height information is available. There is no documentation on how the data was re-processed apart from the fact that the radon has been stripped (in the ‘reprocessed’ column), whereas the ‘original’ column has only been stripped of the noise. Radon was not stripped point by point but rather an average has been stripped as a result of radon in air on the overall image. The ‘reprocessed’ column also probably had some levelling applied (for the high frequency noise) (R Clifton, NTGS, pers comm). The algorithms applied to correct the data in the reprocessed columns are, however, not known.

Table 2 shows a summary of the known survey parameters. There is no information available on the specifications of the NaI detector on board the plane. Readings for radiometrics were taken every second and raw counts per second in the equivalent uranium (eU), equivalent thorium (eTh), potassium (K) and total count (TC) were measured. Line data were then processed and raster data were produced with a pixel size of 70 m x 70 m.

Table 2  Survey parameters of the 1976 Rio Tinto Alligator River geophysical survey.

| Survey parameter |  |
| --- | --- |
| Survey date | 1976 |
| Flight line direction | 030 – 210 |
| Line spacing | 300 m |
| Flying height | unknown |
| Total km flown | 21700 (approx) |
| Counts reported | TC, eU, eTh, K |
| Spectrometer | Unknown |
| Volume | Unknown |

## 3.2  1997 airborne gamma survey specifications

ERA has made available data from an AGS flown in 1997 by World Geoscience Corporation Ltd over the Ranger area, including Anomaly 2. Table 3 shows a summary of the survey parameters of the 1997 Rio Tinto AGS.

## 3.3  Ground truthing

### 3.3.1  Gamma dose rate measurements

Gamma dose rates were measured using three environmental dose rate meters. The details of the three instruments (GM1, GM2 & GM3) are provided in Table 3. GM1 has been calibrated by Australian Radiation Services Pty. Ltd in June 2006, April 2008 and December 2010. Results of the calibration showed that the environmental dose rate meter operated within its specifications.

Table 3  Survey parameters of the 1997 Rio Tinto Ranger uranium mine AGS flown by World Geoscience Corporation Ltd.

| Survey parameter |  |
| --- | --- |
| Survey date | July – August 1997 |
| Flight line direction | 090 – 270 |
| Line spacing | 200 m |
| Flying height | 50 m |
| Total km flown | Unknown |
| Counts reported | eU, eTh, K, TC |
| Spectrometer | 256 channel PGAM-1000 |
| Volume | 33.56 litres |

The Mini-Instruments Environmental Meter Type 6-80 have both an analogue display giving the measured dose rate in µGy∙hr-1 and a digital display of counts in a pre-selected time, usually 100 s. Gamma dose rates can be calculated from the measured count rate. GM2 and GM3 were cross calibrated against GM1 at three areas that exhibited gamma dose rates between 0.1 and 1 µGy∙hr-1 and the values for the corrections factors applied to the measured count rates are shown in Table 4.

Table 4  Details of the instruments used for the gamma dose rate surveys at Anomaly 2.

|  | GM1 | GM2 | GM3 |
| --- | --- | --- | --- |
| Description | Environmental dose rate meter |  |  |
| Manufacturer | Mini-instruments |  |  |
| Model | 6-80 | 6-80 | 6-80 |
| Serial number | 01065 | 01064 | 01049 |
| Correction factor (Aug 07 – Sep 08) |  | 0.92 ± 0.04 | 0.92 ± 0.02 |
| Correction factor (Oct 2008) |  | 1.01 ± 0.02 | 0.97 ± 0.02 |

#### 3.3.1.1  August 2007

Initially, four 70 m x 70 m pixels of the 1976 AGS were surveyed using GM1. It was planned to take measurements within a pixel characterised by low counts in the AGS (background), on Anomaly 2A and 2B and between the two anomalies. Following recommendations made in ICRU Report 75 (2006), gamma dose rates were measured at 40 random points within each of those 70 m x 70 m pixels, and the average dose rates for each 70 m x 70 m area was determined.

160 gamma dose rates and in-situ soil activity concentrations were measured. GPS readings were taken at each measurement site for all gamma dose rate and soil activity concentration surveys. Figure 7 shows the locations of gamma dose rate measurements. Figure 8 shows a close up of Anomaly 2 and the location of the gamma dose rate measurements taken in 2007 and 2008, overlaid on the total counts measured during the 1997 AGS of the area.

#### 3.3.1.2  July 2008

A gamma dose rate survey was conducted over the main Anomaly (Anomaly 2A) in July 2008. The purpose of this survey was to delineate the exact position and the extent of Anomaly 2A. The line spacing of the survey was approximately 15 m, and the distance between measurement points approximately 10 m. An area of approximately 9 ha was surveyed and 663 gamma dose rates measured.

Figure 7  Location of the 2007–08 ground gamma survey points.

Figure 8  Location of the 2007–08 ground gamma survey points overlaid on the total counts signal above Anomaly 2 from the 1997 AGS.

#### 3.3.1.3  September 2008

A gamma dose rate survey was performed in September 2008. This survey aimed to delineate the exact position and the extent of Anomaly 2B which is located to the north of Anomaly 2A. The line spacing was approximately 10 m with measurements taken at 10 m intervals.  An area of approximately 6.6 ha was surveyed and 491 dose rate measurements taken.

#### 3.3.1.4  October 2008

A fourth gamma dose rate survey was performed in October 2008, to fill in the area between Anomalies 2A and 2B, and to extend the area surveyed to the south west of Anomaly 2, to delineate the intensity and position of a third Anomaly, which will be called Anomaly 2C. The survey extended to a small tributary to Gulungul Creek, which can be seen on Figure 8 just south of Anomaly 2A. In this survey 533 measurements were taken.

### 3.3.2  Radon flux density measurements

Radon gas is produced in rock or soil grains from the decay of 226Ra to 222Rn through alpha decay. Radon then exhales from the ground into the atmosphere. Radon exhalation is a two-step process. First, a small quantity of the total radon produced in the ground emanates from the rock or soil grain surface into the soil gas. In the second step, a fraction of the radon gas diffuses to the ground surface and exhales from the soil into the air above (see for example Porstendörfer 1994, Lawrence 2005, Akber et al 2011).

Emanation from the solid grain into the soil gas depends on various factors, such as: the 222Rn production rate (which depends on the 226Ra activity concentration), the distribution of 226Ra within the grain (production near the surface leads to higher probability of escape into soil gas), and the grain size (the smaller the grain the higher is the surface area to volume ratio and emanation coefficients increase).

Exhalation of radon from the soil gas to the surface also depends on a number of variables including soil porosity and permeability, soil thickness, soil moisture, precipitation, wind velocity, barometric pressure and temperature (Porstendörfer 1994). Porosity facilitates radon exhalation and disturbing the soil allows any trapped radon to escape. Deeper layers of the soil make a decreasing contribution to surface exhalation but typically radon can reach the surface from depths of several meters. An estimate of diffusion length for 100 µm dry grain size material is 2  2.5 metres and a typical value for soil is 1.5 m (Porstendörfer 1994). Radon exhalation also has a complex dependence on soil moisture – values increase as the moisture content increases from dryness, and then decrease to nearly zero as soil moisture approaches saturation (Lawrence 2005, Lawrence et al 2009).

#### 3.3.2.1  Deployment of radon cups

Radon flux density can be measured by trapping the radon that exhales from the ground surface in canisters filled with activated charcoal. The charcoal canisters used in this study were brass cylindrical design (‘radon cups’) with an internal diameter of 0.061 m. If the ‘open face’ of a brass charcoal canister is sealed against a surface, then all the radon emanating from the surface will diffuse into the canister and be adsorbed onto the charcoal. Deployment of a number of radon cups provides simultaneous measurements of numerous locations and was the method of choice for the Anomaly 2 survey. The radon flux density over the period of exposure can then be estimated using:

(1)

where J (Bq·m-2·s-1) is the average radon flux density, R is the net count rate (s-1)  after background subtraction  obtained during the counting period tc (s),  (s-1) is the decay constant for radon, td (s) is the delay period from the end of exposure to the beginning of the counting interval,  (s-1·Bq-1) is the counting efficiency of the system, a (m2) is the area of the canister, te (s) is the period of exposure of the charcoal in the canister (Spehr et al 1983).

The derivation of equation 1 is based on a number of assumptions (Bollhöfer et al 2005) including that the radon exhalation from the ground is constant over the exposure period. Existing data indicate that diurnal variations in 222Rn flux densities in the Alligator Rivers Region are small (Todd et al 1998, Lawrence et al 2009), and so the assumption of constant radon flux densities is reasonable. In addition, our data are not corrected for the effects of water vapour uptake by the charcoal. Provided that adsorption of gases (primarily water vapour) does not lead to saturation of the charcoal, as is the case during the dry season, all of the radon exhaled is adsorbed on the charcoal, and no correction for humidity is required.

The charcoal canisters contain 25 grams of charcoal retained behind a wire mesh. Prior to deployment the cups are heated overnight at ~110 C to drive out any residual radon and water adsorbed on the charcoal.

Radon flux density was measured over the extent of Anomaly 2 in July 2009 at 25 locations. The locations of the sampling points are shown in Figure 9. Three radon cups were deployed at each location on 14 July 2009 and collected after a period of three days. The locations are shown in Figure 9. To prevent leakage of radon around the edge of the canisters they were embedded in the earth to a depth of about 1 cm or, if the surface was too hard or irregular, the rim was sealed with mud or a ‘putty’. There was no rainfall during the deployment period.

Figure 9  Location of the radon exhalation sampling points overlaid on the total counts signal above Anomaly 2 from the 1997 AGS.

#### 3.3.2.2  Setup of counting system

A NaI gamma spectrometer, housed in a lead castle to reduce background, at the eriss laboratories in Darwin was used to determine the activity of radon decay products adsorbed on the charcoal. Each radon cup was counted for 600 s. Four regions of interest (ROI) representing the photopeaks for 214Pb at 242, 295 and 353 keV and for 214Bi at 609 keV were used to determine the activity of radon progeny adsorbed onto the charcoal. The net count rate R was obtained by subtracting the background count rate from the gross count rate in those regions of interest.

The system was calibrated against a charcoal canister of known  226Ra activity. This calibration canister was prepared using twenty-five grams of charcoal and a solution containing 327 Bq activity of 226Ra that was carefully added to the canister (Bollhöfer et al 2005). The canister was sealed after the solution had dried and, after delay of 25 days to allow 222Rn to reach secular equilibrium with 226Ra, was counted several times to obtain a statistically robust measurement of counting efficiency. The average counting efficiency was 10.6% ± 0.2%.

The procedure for counting radon cups includes a delay of at least 3 hours between the collection of the canisters and the start of the count period to allow the progeny, 214Pb (half-life 27 mins) and 214Bi (half-life 20 mins), to in-grow towards a secular equilibrium with their progenitor, 222Rn (half-life 3.82 days).

### 3.3.3  Radon in air measurements

The activity concentration of radon in air [Bq∙m-3] was measured using track etch detectors supplied by Radiation Detection Systems (RDS), Adelaide. Two track etch detectors were deployed at about 1.5 m height above ground for 3 months (14 July to 22 October 2009) at each radon exhalation measurement location (1A to 25A). A small shade made of polystyrene was attached to the track etch detectors to avoid the track etch detector housing from exposure to direct sunlight. At 6 locations (sites 3A, 13A, 14A, 15A, 17A, 21A) two additional track etch detectors were deployed each at 30 cm and 50 cm height, respectively, to determine radon concentrations for a person lying down or sitting on the ground. Durrani and Ilic (1997) describe in detail the methodology of radon measurements using track etch detectors.

### 3.3.4  Soil activity concentration measurements

#### 3.3.4.1  In-situ soil activity concentration measurements

Soil activity concentrations were measured with a 512 channel portable NaI gamma detector (Geofyzika, now SatisGeo, model GS-512) during the August 2007 gamma survey at each gamma dose rate measurement point. Measurements were taken for 600 seconds, 1 metre above ground using a tripod as shown in Figure 10.

The GS-512 displays and stores the concentrations of K, U, Th in %, or mg∙kg-1 respectively. It is assumed that 226Ra is in secular equilibrium with 238U, and the uranium measurements are generally reported as equivalent uranium (eU). However, it is the 226Ra activity concentration that is determined, using the 214Bi gamma rays detected at 1.73-1.76 MeV as a proxy for 226Ra, rather than measuring uranium directly. Data for 40K (1.46 MeV) and the thorium-series radionuclide 208Tl (2.31 MeV) as a proxy for 232Th were also obtained and are reported in Appendix A1.

Spectra are stored in the 512 channels of the GS-512 spectrometer. K, U and Th concentrations are determined by so called stripping (IAEA, 1989), through a matrix multiplication of the count rates with pre determined calibration constants. The instrument was calibrated in October 2003, and its stability was tested regularly by measuring the activity concentration on a test spot at the Darwin laboratories.

Figure 10  Set up of the GS-512 field gamma spectrometer.

A 137Cs source located in the instrument is used to establish the position of the regions of interest for eU, eTh and K by reference to the 661.6 keV gamma peak of 137Cs. At locations with high uranium concentrations the instrument failed to stabilise the spectrum due to interference from lower energy peaks of 214Bi with the location of the 137Cs peak, and Compton scattering originating from the decay of uranium series radionuclides, and no results were obtained.

The internal memory of the GS-512 allows to store 4300 readings of 4 window ROI data or 200 complete 512 channel spectra. After each day of measurements, the spectrum data were downloaded to a computer hard drive.

#### 3.3.4.2  Laboratory soil activity concentration measurements

Soil samples were taken from the 25 locations where radon flux densities were determined in July 2009. At each flux density site (sites 1A to 25A), the top 2 cm of soil was sampled at 5 random locations within 1 m of the deployed radon cups and the 5 samples were combined in a plastic bag. Samples were then transported to Darwin for radionuclide analyses.

238U, 226Ra, 228Ra, 228Th, 210Pb and 40K activities in the samples were determined using the High Purity Germanium (HPGe) gamma detectors from the Environmental Radioactivity section at eriss. An in-house program is used for analysis of sample activity concentrations (Esparon & Pfitzner 2010, Pfitzner 2010). Details of the gamma spectrometry methods are described in Murray et al (1987) and Marten (1992). Procedures for sample collection, preparation and measurements of radionuclide activity concentrations via gamma spectroscopy at the Environmental Radioactivity laboratory are described in Marten (1992).

The stability and background of the detectors is checked weekly with a multi isotope standard containing radionuclides of the uranium and thorium decay chains and a blank matrix (empty container), respectively. Detection limits for soil samples using gamma spectrometry are dependent on sample size, detector efficiency and background count rates of the given nuclide, but were typically ~3 Bq∙kg-1 for 226Ra for a one day count in the present study.

# 4  Results

## 4.1  1976 airborne gamma survey data

Figure 11 shows the results of the counts in the uranium channel over the whole of the 1976 AGS, overlaid on an aerial photo mosaic of Kakadu National Park from 2004. The locations of Ranger orebodies 1 and 3, and Anomaly 2 are clearly visible in the 1976 AGS data. A subset of the uranium channel data, only including count rates above 250 counts per second, is overlaid on the Ranger region photo in Figure 12 and on the Anomaly 2 photo in Figure 13.

Figure 11  Counts (> 25 counts per sec) in the uranium channel of the 1976 AGS over the whole scene.

Figure 12  Counts (>250 counts per second) in the uranium channel of the 1976 AGS 
over the Ranger mine region.

Figure 13  Counts (>250 counts per second) in the uranium channel of the 1976 AGS over Anomaly 2.

## 4.2  1997 airborne gamma survey

Figure 14 shows the results of the total counts measured south of the Ranger lease over Anomaly 2 during the 1997 AGS flown by Rio Tinto. It appears that an additional Anomaly is present (labelled Anomaly 2C) to the south of Anomalies 2A and 2B, in agreement with Hegge et al (1980) who stated that Anomaly 2 to the south of the Ranger lease is outcropping in places and characterised on ground by three strong separate radiometric anomalies.

Figure 14  Counts (>4500 counts per second) in the TC channel of the 1997 AGS over Anomaly 2.

## 4.3  Gamma dose rates measured in the field

Figure 15 shows the results of the field gamma dose rates (in µGy∙hr-1) measured in the region of Anomaly 2 between 2007 and 2009. The three separate radiation anomalies identified by AGS are clearly visible on the ground. Results are tabulated in Appendices A1–A4. Results of individual measurements are tabulated in Appendices A1–A4.

Figure 15  Field gamma dose rates measured over Anomaly 2 between 2007–09.

## 4.4  Radon

### 4.4.1  Radon flux densities

Figure 16 shows the geometric means of the three individual radon flux density measurements at sites 1A to 25A, colour-coded according to the magnitude of the calculated radon flux density (in mBq∙m-2∙s-1). Results for the individual sampling points and GPS locations of the sites are given in Appendix B.

Figure 16  Radon flux densities measured over the Anomaly 2 area between 2007–09.

Higher radon flux densities were measured on top of the Anomalies, with the highest radon flux density of ~10 Bq∙m-2∙s-1 measured at site 13A immediately on top of the Anomaly. This site is dominated by coarse gravel of a few centimetres thickness overlying rock strata.  Similar soil morphology was encountered at sites 14A and 15A, where a relatively thin layer of gravel and sand, respectively, was overlying rocky terrain. Site 12A was located in a small depression, where fine gravel had accumulated, most likely erosion products from the nearby higher activity soil of Anomaly 2A.

### 4.4.2  Radon activity concentration in air

Typical uncertainties for individual radon activity concentration results provided by RDS are 40% (Kvasnicka, pers comm). Averages and standard deviations for the two radon activity concentration results per site were calculated. All calculated averages had a relative standard deviation of less than 100%, however, from a total of 37 averages calculated at Anomaly 2, 6 averages exhibited a relative standard deviation above 57% (which is the combined uncertainty of the individual measurements). Typical relative standard deviation of two track etch detectors deployed at the same site was 25%. Results of individual and site averaged radon activity concentrations are tabulated in Appendix C.

The maximum radon activity concentration in air 1.5 m above ground was 430 Bq∙m-3 measured at site 14A in the immediate vicinity of Anomaly 2A. The next highest values at 332 and 331 Bq∙m-3 were measured at sites 7A and 16A. Despite the high radon activity flux density at site 13A on top of Anomaly 2A (9532 mBq∙m-2∙s-1) it showed an activity concentration in air 1.5 m above ground of 220 Bq∙m-3 only. Sites 17A (east of Anomaly 2A), 3A (close to Anomaly 2B) and 21A (southwest of Anomaly 2C) show radon activity concentrations of 160, 120 and 70 Bq∙m-3. The remainder of the sites showed radon activity concentrations between 45 and ~200 Bq/m3.

At 50 cm height above ground radon activity concentrations in air were highest 
(860–1550 Bq∙m-3) at sites 14A, 15A and 13A in the immediate vicinity of Anomaly 2A, whereas sites 3A, 17A and 21A showed radon activity concentrations in air about 10 times lower (20–100 Bq∙m-3). At 30 cm height above ground radon activity concentrations in air increase to up to 2000 Bq∙m-3 on top of Anomaly 2A, whereas sites 3A, 17A and 21a exhibit radon activity concentrations in air of 100–200 Bq∙m-3. Figure 17 summarises the results of the radon in air measurements.

Figure 17  Radon activity concentrations measured in air at 30 cm, 50 cm and 150 cm above ground over the Anomaly 2 area for the dry season 2009.

## 4.5  Soil activity concentrations

### 4.5.1  Laboratory HPGe measurements

Surface soil radionuclide activity concentrations were measured in soil samples collected from radon exhalation measurement sites 1A-25A (Figure 17) using the eriss HPGe detectors in the Darwin laboratories. Results are given in Appendix D.

238U activity concentrations were calculated from the contribution of 235U to the 186 keV line in the gamma emission spectrum, and then multiplying the calculated 235U activity concentration with the natural 238U/235U activity ratio of 21.7. 238U activity concentrations can also be estimated from the 234Th activities, assuming equilibrium between 238U and 234Th. Figure 18a shows the derived 238U activity concentrations (ie calculated from the contribution of 235U to the 186 keV line) plotted versus the measured 234Th activity concentrations for activity concentrations between 40 and 40 000 Bq∙kg-1. The slope of the linear best fit to the measured values indicates that, on average, the 234Th activity concentration was measured at about 90% of the 238U activity concentration. This slope is dominated by values above 10 000 Bq∙kg-1, for values below 10 000 Bq∙kg-1 the slope is approximately 1 (Figure 18b). 238U activity concentrations given in Appendix D are the error weighted mean of the 238U and 234Th activity concentrations measured in a sample.

Figure 18  (a) Measured 234Th activity concentration plotted against derived 238U for sample activity concentrations between (a) 40 – 40  000 Bq∙kg-1, and (b) 40 – 10 000 Bq∙kg-1.

Figure 19  (a) 226Ra activity concentration and (b) 226Ra/238U activity concentration ratio plotted against 238U activity concentrations in samples from Anomaly 2.

Figures 19a and b show the activity concentration of 226Ra and the 226Ra/238U activity concentration ratio plotted versus the 238U activity concentration in the soil samples. 226Ra activity concentrations measured in the lower activity samples is close to secular equilibrium with 238U. However, 226Ra/238U activity concentration ratios vary between 0.8 and 2.0, and are higher especially in samples with activity concentrations of 238U between 100 and 1000 Bq∙kg-1. On average 226Ra is approximately 40% higher than the activity concentrations of 238U in the samples.

It is important to note that:

1.	equivalent uranium concentrations (eU) measured with the in-situ NaI gamma spectrometer and given in Appendix A are not representative of the uranium concentration. Much rather the NaI gamma spectrometer determines the activity of 214Bi in soil from its gamma emission line at 1.76 MeV. As 214Bi can be assumed to be in radioactive equilibrium with 226Ra in soil, the eU values measured using the hand held gamma spectrometer should be multiplied with 12.35 Bq∙kg-1 per ppmU to calculate and report activity concentrations of 226Ra measured in situ.

2.	226Ra activity concentration measured in surface scrapes in the laboratory may not be representative of the 226Ra activity concentration in deeper sections of the soil profile.

To illustrate the importance of the second point, the expected terrestrial gamma dose rate (He) has been calculated from the soil activity concentrations at sites 1A – 25A measured using the laboratory HPGe detector:

He = a1∙238U + a2∙232Th + a3∙40K	(2)

with the conversion factors ai given in Table 5, assuming equilibrium between all members in the uranium and thorium decay chains, respectively (Saito & Jacob 1994), ie the measured 226Ra was used as a proxy for 238U activity concentrations.

Table 5  Conversion factors used to calculate 238U, 232Th and 40K activity concentrations and expected terrestrial gamma dose rates from given soil uranium, thorium and potassium concentrations.

|  | Uranium-238 | Thorium-232 | Potassium-40 |
| --- | --- | --- | --- |
| ai | 0.462 (nGy.hr-1)/(Bq.kg-1) | 0.604 (nGy.hr-1)/(Bq.kg-1) | 0.0417 (nGy.hr-1)/(Bq.kg-1) |
| ci | 12.35 BqU-238/mgU | 4.07 BqTh232/mgTh | 31.6 BqK40/gK |

Figure 20a shows the terrestrial gamma dose rates as calculated from the activity concentrations measured in the surface soil scrapes, plotted versus the measured terrestrial gamma dose rate at sites 1A to 25A. A value of 0.066 µGy∙hr-1 has been subtracted from the measured values, as the contribution from cosmic radiation to the measured gamma dose rate in the field (Marten 1992).

The slope of this fit indicates that calculated terrestrial gamma dose rates using equation 2 are on average two times higher than measured terrestrial gamma dose rates. However, the correlation is dominated by the high terrestrial gamma dose rates calculated for sites 14 A and 15A, which are both located on top of Anomaly 2. These two samples had the highest 226Ra soil activity concentrations measured in this study (see Appendix D). Figure 20b illustrates that data regression for calculated values are only about 5% higher than measured terrestrial gamma dose rates for sites excluding 14A and 15A, and also excluding site 24A, which was located on top of a rock pile and exhibited relatively smaller measured dose rates due to the geometry of the surface (the measurement was done on the pile crest) where the measurement was conducted.

Most of the gamma dose rate measured in air at a height of 1 m above the ground originates from radionuclides located in the top 0.5 m of the soil (ICRU 1994, Saito & Jacobs 1992). Hence, gamma dose rates calculated from radionuclide activity concentrations measured in surface soils (0 – ~5 cm) may over (or under) estimate the actual terrestrial gamma dose rate if soil activity concentrations are higher (or lower) at the soil surface. At sites 14A and 15A on top of Anomaly 2A it appears that activity concentrations of the surface soil scrapes collected are higher than the average activity concentrations throughout the top ~0.5 m of soil.

Figure 20  Calculated terrestrial gamma dose rates [µGy∙hr-1] from laboratory HPGe radionuclide activity concentration measurements plotted versus the measured terrestrial gamma dose rates at sites 1A-25A. a) entire range of dose rates measured, b) less than 10 µGy∙hr-1.

### 4.5.2  In situ NaI measurements

Soil activity concentrations were also measured with a 512 channel portable NaI gamma detector (Geofyzika, now SatisGeo, model GS-512) during the August 2007 gamma survey at each gamma dose rate measurement point. Results are shown in Appendix A.

It is important to note that, similar to a dose rate meter, the signal measured by a portable NaI gamma detector at a height of 1 m above the ground originates from radionuclides located in the top ~0.5 m of the soil rather than the surface soil only. In addition, the footprints of both measurement methods are comparable. Thus, calculated (from NaI measurements) and measured terrestrial gamma dose rates should be more readily comparable than those using laboratory measurements of radionuclide activity concentration in surface soil scrapes.

The portable NaI gamma detector gives equivalent uranium (eU) and equivalent thorium (eTh) concentrations in ppm, potassium concentrations are given in %. These concentrations have been used to calculate soil 226Ra, 232Th and 40K activity concentrations, using the conversion factors ci shown in Table 4 (Figure 21).

Figure 21  Calculated terrestrial gamma dose rates (using equation 2) in µGy∙hr-1 plotted versus measured terrestrial gamma dose rates performed in August 2007 using the NaI detector; a) entire range of dose rates measured, b) less than 2 µGy∙hr-1.

Figure 21a shows a plot of calculated terrestrial gamma dose rates versus gamma dose rates measured in the field in August 2007. It shows a good agreement between calculated and measured values, with slightly higher calculated gamma dose rates. Figure 21b shows the calculated terrestrial gamma dose rates versus gamma dose rates measured in the field for gamma dose rates from 0-2 µGy∙hr-1 (as the calibration for GM is strictly applicable for dose rates from 0-2 µGy∙hr-1 only). The comparison shows good agreement between calculated and measured terrestrial dose rates.

# 5  Conversion factors

## 5.1  Terrestrial gamma dose rate to 226Ra soil activity concentration

In Figure 22 the measured soil 226Ra activity concentration is plotted against measured terrestrial gamma dose rates. This plot includes all laboratory measurements using the eriss HPGe detectors and also the in-situ NaI measurements. A fit to all 226Ra data is shown, in addition to a fit to the NaI measurements only.

The yellow sample points in Figure 22 indicate scrape samples that were taken immediately on top of Anomaly 2 (sites 13A-15A, see Figure 17). Sites 14A and 15A are samples from rocky terrain and the substrate consisted of coarse gravel and some rocks. 226Ra activity concentrations were higher than 238U in these surface scrapes. Site 16A also consisted of gravel but 226Ra and 238U in this sample were close to equilibrium. As mentioned previously, the measurement of the gamma dose rate at site 24A was taken on top of a rock heap next to an old trench, which most likely lead to lower measured terrestrial gamma dose rates due to the change in surface geometry. Consequently, it is justified to omit these samples from the fit of terrestrial gamma dose rates to 226Ra soil activity concentrations.

Figure 22  In situ and laboratory measured soil 226Ra activity concentration plotted against the measured terrestrial gamma dose rates, all data shown. Filled symbols (yellow) indicate the results from laboratory measurements. In addition a fit to the NaI data only (open symbols) is shown.

Figure 23 shows the in-situ measured 226Ra activity concentration plotted against the measured terrestrial dose rate, for dose rates between 0-2 µGy∙hr-1, and including the HPGe measurements in that range. A conversion factor of 2330 Bq∙kg-1 per µGy∙hr-1 has been determined with a 95% confidence interval of (2280-2380) Bq∙kg-1 per µGy∙hr-1. The value of the x-intercept (y = 0) is (0.07±0.01) µGy∙hr-1, which is only slightly higher, than the average terrestrial gamma dose rate calculated from the measured soil 232Th and 40K activity concentrations alone of 0.034±0.030 µGy∙hr-1 (95% confidence) using equation (2) above.

Figure 23  In situ (NaI) (o) and laboratory (HPGe) (•) measured soil 226Ra activity concentration plotted against measured terrestrial gamma dose rates (< 2 µGy∙hr-1).

Using equation (3) we can convert terrestrial gamma dose rates measured 1 m above the ground to 226Ra soil activity concentrations.

226RaBqkg=(2330±50) Bq/kgμGy/hr∙Eterr-(165±20) [Bqkg]	(3)

## 5.2  226Ra soil activity concentration to 222Rn flux density

Similar to the terrestrial gamma radiation, where most of the signal originates from the top approximately 50 cm of the soil (ICRU 1994), soil layers down to several metres can contribute to radon exhalation at the surface, with a typical diffusion length for 222Rn of 1.5 m (Porstendörfer 1994). In addition, it has been shown that radon exhalation for different geomorphic sites, or soil types, can differ for similar 226Ra activity concentration in the soil (Lawrence et al 2009). For example in the greater Ranger area barren compacted surfaces with no vegetation show a 226Ra activity concentration to 222Rn flux density conversion factor RE-R of 0.27 [(mBq.m-2.s-1)/(Bq.kg-1)] whereas the conversion factor for non compacted material can be up to 10 times higher.

Figure 24 shows the geometric mean of the 222Rn flux densities at each site plotted against the measured 226Ra soil activity concentrations.

Figure 24  222Rn flux densities plotted against the measured (HPGe) 226Ra soil activity concentrations. Only the loamy sand and fine gravel sample types are included in the line of best fit.

Equation (4) below can be used to calculate the expected 222Rn flux density for loamy sands and gravel, if soil 226Ra activity concentrations are known. This has been derived excluding data from sites 13A – 15A on top of Anomaly 2A, and site 24A, which was on top of a rockpile in the vicinity of Anomaly 2C, not representative of the general soil morphology in the area.

222RnmBqm2.s=(0.9±0.4) mBqm2.sBqkg∙226Ra  [Bqkg]	(4)

For coarser material and rocks that can be found on top of the Anomalies it appears that, whereas 226Ra activity concentration increases in these areas, the 222Rn flux densities do not. This discrepancy may be due to the reduced surface area of the coarser gravel and rocks relative to the sand and gravel sites. Consequently, this equation does not apply immediately on top of Anomaly 2, and also most likely not immediately on top of the anomalies associated with orebodies 1 and 3 before mining started. The average 222Rn flux density immediately on top of Anomaly 2A is 6000±2300 mBq∙m-2∙s-1, while the 226Ra activity concentration varies by more than one order of magnitude.

## 5.3  Terrestrial gamma dose rate to 222Rn flux density

Approximately 56% of the terrestrial gamma signal measured 1 m above the ground originate from a circle of about 2 m (IAEA 1989) and the measurement of the terrestrial gamma dose rate is more representative of the radioactivity in the top 50 cm of soil, rather than the surface soils only. The typical diffusion length for 222Rn is about 1.5 m (Porstendörfer 1994) and in our study radon exhalation at the sites has been determined as the geometric mean of three individual radon cup results within a radius of approximately 1 m.

Hence, a conversion factor has also been calculated to determine 222Rn flux density from the measured terrestrial gamma dose rates. Terrestrial gamma dose rates have been calculated by subtracting 0.066 µGy∙hr-1 as the contribution from cosmic radiation from the gamma dose rate measured on ground (Marten 1992).

The plot of the geometric means of the radon flux densities at each site against the measured terrestrial gamma dose rates is shown in Figure 25. The line shows the best fit to the data points, but excludes sites 13A, 14A and 15A directly on top of Anomaly 2A for the reasons mentioned above.

Figure 25  222Rn flux densities plotted against the measured (in situ) terrestrial gamma dose rates. The line of best fit has been drawn including loamy sand and fine gravel substrate types, but excluding measurements from directly on top of the Anomaly (circled) and sample 24.

The coefficient of determination R2 between the averaged 222Rn flux densities and the measured terrestrial dose rates in the field is significantly better than the correlation between 222Rn flux densities and measured 226Ra soil activity concentration in the surface scrapes (Figure 24). The variability in terrestrial gamma dose rates accounts for approximately 80% of the variability observed in radon activity flux densities, using a linear model. This is not surprising, due to the depth related effects discussed above.

The equation to convert terrestrial gamma dose rates into 222Rn flux densities can thus be written as:

222RnBqm2∙s=(3.4±0.7) Bqm2∙suGyhr∙Eterr[uGyhr]	(5)

## 5.4  Converting the 1976 AGS U data to terrestrial dose rate in the field

Differences in survey parameters, height of the detector above ground, detector type and calibration do not allow direct comparison of spatial data from the two (ie 1976 and 1997) AGSs and the ground-based gamma dose rate survey, respectively. Upscaling of the ground-based survey is required to make the footprints of the ground-based survey and the AGSs comparable. Bollhöfer et al (2008) show a table which compares the typical footprints of AGSs with the footprint of ground-based surveys usually performed at 1 m above ground. This table is re-produced here (Table 6). Spatial referencing and data processing (which is largely unknown for the 1976 AGS) can furthermore complicate the process of upscaling the data.

Table 6  Comparison of typical footprints for ground-based and airborne gamma surveys.

|  | Altitude | Footprint, radius r | % signal from within r | Reference |
| --- | --- | --- | --- | --- |
| Airborne gamma survey | 120 m | 180 m | 50 | Bierwirth et al 1996 |
| Airborne gamma survey | 50 m | 50 m | 40 | Duval et al 1971 |
| Airborne gamma survey | 30 m | 36 m | 56 | Billings & Hovgard 1999 |
| Ground gamma survey | 1 m | 2 m | 56 | IAEA 1989 |

In this study, data from two AGSs were available over the greater Anomaly 2 area. One survey was flown by Rio Tinto in 1997, at a flying height above Anomaly 2 of approximately 50 m and a flight line spacing of 100 m. The other survey was flown in 1976 before mining started, at an unknown flying height and a line spacing of 300 m. In addition, the 1976 data have been processed and pixilated (70 m x 70 m) but processing algorithms are largely unknown.

Due to the better resolution of the 1997 AGS data it was decided that the upscaling of the gamma dose rates measured in the field to the eU (s-1) data measured during the 1976 AGS should be performed via an intermediate step, using the eU (Bq∙kg-1) line data above Anomaly 2 from the 1997 survey. The underlying assumption for the upscaling of our data is that between 1976 and the time of the groundtruthing, no changes that would significantly alter the radiological conditions at Anomaly 2 have occurred. This assumption appears reasonable, as Anomaly 2 has been excluded from the Ranger Project Area after recommendations made in the second Fox Report (RUEI, 1977), due to its close proximity to Mount Brockman and its Aboriginal sacred sites, Djidbidjidbi and Dadbe, sites of significant value to the Mirrar Aboriginal people.

### 5.4.1  Modelling the 1997 AGS line data to the field gamma dose rate data

The AGS data originally received from Rio Tinto as projected coordinates of the Australian geodetic datum 1984, were reprojected into the WGS84 map datum, UTM Zone 53S. A shapefile was then created, defined by the boundary of the 2007 to 2009 field data obtained for the Anomaly 2 area (Figures 15 and 26).

Airborne gamma survey points which lie within the boundary of the shapefile and have integer time records were then extracted. The basic assumption here is that all non-integer second data is interpolated based on the integer second records. The integer records were then given a unique id (ID_97) and the uranium (eU in Bq∙kg-1) and total count (TC in s-1) data was used for modeling purposes.

Figure 26  Shapefile created in ArcGIS for the 2007-2009 ground survey (grey), flight lines and flight line numbers (black arrows) and individual ground survey points (orange).

Lines were then created which represent the flight path for each of the records and each line adopted the TC and eU values from the integer time stamp of the AGS data file. In addition, the average altitude of the flight lines was calculated using the average altitude of the previous and current records.

Table 6 above illustrates the influence of survey height on the effective footprint for various AGS specifications. Consequently, the ground data need to be upscaled. A circular footprint is justified for a stationary plane, but the average distance the plane travels between individual measurements is 60 m. Consequently, a series of buffers of the shape shown in Figure 27 was created in ArcGIS and the buffer radius was varied.

The calculated mean gamma dose rates [µGy∙hr-1] measured on ground within the chosen buffers for each line were then calculated for various buffer radii chosen. These averages were then plotted against airborne total counts (TC) of the respective flight lines to obtain the buffer radius giving the best correlation between averaged dose rates and TC measured on board of the plane.

Figure 28 shows the coefficient of determination R2 obtained in the GIS analysis between averaged gamma dose rates on ground and total counts in the 1997 AGS for buffer radii ranging from 60 m – 130 m. The radii were varied in 5 m steps. Analysis of the data indicated that the best correlation between averaged gamma dose rates measured on ground and total counts measured on board the plane was gained for a buffer radius of 90 m.

Figure 27  Variable buffers overlaid on shapefile created in ArcGIS for the 2007–2009 ground survey.

Figure 28  Coefficient of determination R2 obtained between average ground gamma dose rates for varying buffer radii measured in the field and TC measured in 1997 on board the plane.

Figures 29 and 30 show the correlations between the averaged (buffer radius = 90 m; n = 54) terrestrial gamma dose rates and the TC (s-1) and eU (Bq∙kg-1) channels, respectively. Terrestrial gamma dose rates have been calculated by subtracting 0.066 µGy∙hr-1 as the contribution from cosmic radiation from the gamma dose rate measured on ground (Marten 1992).

Figure 29  Correlation between averaged terrestrial gamma dose rates and the TC (s-1) channel of the 1997 AGS. Buffer radius of 90 m, n=54.

Figure 30  Correlation between averaged terrestrial gamma dose rates and eU (Bq∙kg-1) channel of the 1997 AGS. Buffer radius r=90 m, n=54.

To ensure that results of the correlations were not affected by variations in ground sample spacing, only those buffers which have a consistent distribution of ground points within have been used.  Figure 31 shows those buffers that have been selected out and were then used for further analysis. A total of 29 buffers were extracted.

Figures 32 and 33 show the correlations between the averaged (buffer radius = 90 m; n = 29) terrestrial gamma dose rates and the TC (s-1) and eU (Bq∙kg-1) channels, respectively. The equation to convert the eU data from the 1997 AGS into terrestrial dose rates measured in the field can thus be written as:

EterrμGyhr=0.00126±0.00028μGy∙kghr∙Bq∙eU1997[Bq kg-1]-(0.023±0.086)μGyhr	(6)

Figure 31  Buffers (n=29) with even distribution of ground survey points chosen to establish the correlation between the ground survey and the 1997 AGS data. Buffer radius r=90 m.

Figure 32  Correlation between averaged terrestrial gamma dose rates and the TC (s-1) channel of the 1997 AGS for buffers with evenly distributed ground survey points only. r = 90 m, n = 29.

Figure 33  Correlation between averaged terrestrial gamma dose rates and eU (Bq∙kg-1) channel of the 1997 AGS for buffers with evenly distributed ground survey points only. r=90 m, n=29.

### 5.4.2  Modelling the 1976 raster to the 1997 raster airborne gamma data

Both the 1976 (70 m x 70 m) and 1997 (25 m x 25 m) raster datasets were reprojected to WGS84 UTM53S and a subset of the raster data was created (Figure 34). Both the 1997 and 1976 data were then clipped to this subset. The subset includes the full extent of the 1997 raster imagery including Anomaly 2 at the southern edge of the dataset, but excludes the mine footprint. The assumption is, that apart from areas on site, radiation levels have not changed between 1976 and 1997.

Figure 34  Full extent of the 1997 raster imagery. The total counts of the 1997 AGS are shown, with light colours indicating high values. The area within the blue outline was excluded and includes the disturbed footprint of the Ranger mine site.

A vector grid of the 1976 data was then created using the Hawths Tool in ArcGIS. Each grid cell was populated with both the TC (s-1) and eU (s-1) values of the 1976 raster, with the centre of each grid cell taking on the respective values. The 1997 raster data (TC in s-1 and eU in Bq∙kg-1) was then overlaid and the averages were calculated of those 1997 raster data that are completely contained within the respective 1976 vector grid cell using the Spatial Analyst Tool in ArcGIS. Using this method a total of 6916 records were obtained. Results of the correlation between averaged 1997 eU data and the values of the respective 1976 vector grid cell, and the R2 are shown in Figure 35.

Figure 35  Averaged eU data (Bq∙kg-1) from the 1997 AGS plotted versus the 1976 eU data (s-1) of the respective grid cell.

The equation to convert the eU data (s-1) from the 1976 AGS into averaged eU (Bq∙kg-1) for the 1997 AGS can thus be written as:

eU1997Bq kg-1=0.58±0.01[Bq kg-1 s]∙eU1976s-1+(14±1)Bq kg-1			(7)

### 5.4.3  Converting 1976 eU data to terrestrial gamma dose rates and radon flux densities

Combining equations (3), (5), (6) and (7) allows us to determine conversion equations and associated uncertainties (at the 95% confidence level) to determine terrestrial gamma dose rates, radon flux densities and 226Ra soil actyivity concentrations from the count rates in the eU channel of the 1976 AGS.

Equation (8), (9) and (10) are the conversion equations and associated 95% confidence intervals to convert the 1976 eU data (s-1) into terrestrial gamma dose rates measured 1m above ground (equation 8), radon flux densities (equation 9) and soil 226Ra activity concentrations (equation 10), respectively.

1976 eU data to terrestrial gamma dose rates:

EterrμGyhr=0.00073±0.00016μGy∙shr∙eU1976[s-1]-(0.005±0.086)μGyhr 			(8)

1976 eU data to radon flux densities:

222RnmBqm2∙s=2.48±0.76 [mBqm2] ∙eU1976 s-1-(20±290)[mBqm2∙s] 				(9)

1976 eU data to 226Ra activity concentrations:

226RaBqkg=1.70±0.38[Bq∙skg] ∙eU1976 s-1-160±200[Bqkg] 				(10)

# 6  Discussion

## 6.1  Pre-mining external γ dose rates, radon flux densities and 226Ra activity concentrations in the greater Ranger region

Equations 8, 9 and 10 above have been used to determine pre-mining terrestrial gamma dose rates, radon flux densities and soil 226Ra activity concentrations in the greater Ranger region from the U counts (s-1) of the 1976 AGS. For ease of comparison with measurements of gamma dose rates conducted in the field, the contribution from cosmic rays (0.066 µGy∙hr-1; Marten 1992b) has been added to the modelled terrestrial gamma dose rates.

Various shapefiles have been created in ArcGIS. For each of those shapefiles the sum of the counts per second in the eU channel of the 1976 AGS was determined using the Zonal Statistics tool in ArcGIS. This number was used in conversion equations 8-10, and the result divided by the number of pixels within the shapefile, to determine mean pre-mining gamma dose rates, 222Rn flux densities and soil 226Ra activity concentrations for those various areas. Results are shown in Table 7.

Table 7  Pre-mining mean (95% confidence intervals) external gamma dose rates E [µGy∙hr-1] (including cosmic component), 222Rn flux densities and soil 226Ra activity concentrations in the Ranger area.

| Infrastructure | Sum [s-1] | n | Area [ha] | Eave [µGy∙hr-1] | 226Raave [Bq∙g-1] | Rnave [Bq∙m-2∙s-1] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 - Pit 1 | 89599 | 81 | 40 | 0.87 ± 0.18 | 1.8 ± 0.4 | 2.7 ± 0.9 |
| 2 - Pit 3 | 81930 | 157 | 77 | 0.44 ± 0.09 | 0.88 ± 0.20 | 1.3 ± 0.4 |
| 3 - Anomaly 2 | 28458 | 77 | 38 | 0.33 ± 0.06 | 0.63 ± 0.14 | 0.92 ± 0.28 |
| 4 - Djalkmara LAA | 3549 | 19 | 9 | 0.20 ± 0.03 | 0.31 ± 0.07 | 0.46 ± 0.14 |
| 5 - Corridor Ck LAAs | 28316 | 284 | 139 | 0.14 ± 0.02 | 0.17 ± 0.04 | 0.25 ± 0.08 |
| 6 - Tailings Dam | 14247 | 225 | 110 | 0.11 ± 0.01 | 0.11 ± 0.03 | 0.16 ± 0.05 |
| 7 - Magela LAAs | 7823 | 114 | 56 | 0.12 ± 0.01 | 0.11 ± 0.03 | 0.17 ± 0.05 |
| 8 - RP1 | 1932 | 35 | 17 | 0.11 ± 0.01 | 0.09 ± 0.02 | 0.14 ± 0.04 |
| 9 - RP1 LAAs | 4159 | 77 | 38 | 0.11 ± 0.01 | 0.09 ± 0.02 | 0.13 ± 0.04 |
| 10 - Djalkmara ex LAA | 916 | 19 | 9 | 0.10 ± 0.01 | 0.07 ± 0.02 | 0.12 ± 0.04 |
| 11 - Jabiru East LAA | 5308 | 103 | 50 | 0.10 ± 0.01 | 0.09 ± 0.02 | 0.13 ± 0.04 |
| 12 - Jabiru | 34177 | 624 | 306 | 0.11 ± 0.01 | 0.09 ± 0.02 | 0.14 ± 0.04 |
| Ranger Project Area | 996080 | 16082 | 7881 | 0.11 ± 0.01 | 0.11 ± 0.02 | 0.15 ± 0.05 |

Figure 36 shows the shapefiles that were created, including the area of the township of Jabiru, overlaid on an aerial photo from 2004 of the region. In addition, the pixilated eU counts per second of the 1976 AGS over the same region are shown. Figure 37 shows the same two plots, but over the Ranger mine area only.

38

Figure 36  Shapefiles overlaid on aerial photo of the greater Ranger region (left) and on the counts in the eU channel of the 1976 AGS of the same extent (right). 1: Pit 1; 2: Pit 3; 3: Anomaly 2; 4: Djalkmara land application area (LAA); 5: Corridor Creek LAA; 6: Tailings Dam; 7: Magela LAAs; 8: Retention  Pond 1; 9: Retention Pond 1 LAAs; 10: Djalkmara extension LAA; 11: Jabiru East LAA; 12: Jabiru.

39

Figure 37  Shapefiles overlaid on aerial photo of Ranger only (left) and on the counts in the eU channel of the 1976 AGS of the same extent (right).

## 6.2  Orebody 1, Orebody 3 and Anomaly 2

### 6.2.1  Gamma dose rates and 226Ra activity concentrations

Count rates in the eU channel of the 1976 AGS are highest over Orebody 1, Orebody 3 and Anomaly 2 (Figure 39) and thus these features exhibit the highest calculated external gamma dose rates, radon flux densities and soil 226Ra activity concentrations. The average pre-mining external gamma dose rates calculated from the radiological GIS data for orebodies 1 and 3 and for the Anomaly 2 area are 0.87, 0.44 and 0.33 µGy∙hr-1, respectively (95% confidence intervals are given in Table 6). Average 226Ra activity concentrations are 1900, 900 and 600 Bq∙kg-1. The average has been determined in ArcGIS for the surface area (shapefile) of the two pits, rather than the surface area of the outcropping orebodies.

Kvasnicka (1993) and Kvasnicka & Auty (1994) have determined gamma dose rates for the two orebodies (see table 1). The terrestrial gamma dose rate on orebody 1 was 0.96 µGy∙hr-1 and on orebody 3 it was 0.58 µGy∙hr-1. Whereas the dose rate on top of orebody 3 was measured directly, dose rates on top of orebody 1 were estimated from radionuclide activity concentrations in drill core taken above the orebody. 226Ra activity concentrations on top of the orebodies were estimated to 2230 and 1350 Bq∙kg-1, respectively.

Differences in average values between our study and Kvasnicka and Auty (1994) most likely arise from the different areal extent of the anomalies investigated. The surface area of orebody 1 investigated by Kvasnicka and Auty (1994) was 44 ha, which is similar to the area used for averaging in our study, and average external gamma dose rates and 226Ra soil activity concentrations agree well. In contrast, measurements of the gamma dose rates on top of orebody 3 were only taken over an area of 66 ha by Kvasnicka and Auty (1994) compared with an average calculated above 77 ha in our study. Most likely relatively more lower values at the outer edges of the orebody have been included in our average of the external gamma dose rate, leading to the difference observed between the two studies.

The average external gamma dose rates determined above Anomaly 2 using the GIS data is 0.33 µGy∙hr-1, whereas the average of all field measurements is 0.40 µGy∙hr-1. This translates to an average soil 226Ra activity concentration of 600 Bq∙kg-1 at Anomaly 2.

### 6.2.2 Radon flux density

The average pre-mining radon flux density calculated from the radiological GIS data for orebodies 1 and 3 and for the Anomaly 2 area are 2.7, 1.3 and 0.9 Bq∙m-2∙s-1, respectively

Kvasnicka & Auty (1994) also measured the radon flux density on top of orebody 3 at three locations and the average amounted to (4.4±1.0 Bq∙m-2∙s-1). They furthermore use the measurements of radon flux densities and soil 226Ra activity concentrations at those three locations to calculate a conversion factor of 1.85±0.23 [(mBq∙m-2∙s-1)/(Bq∙kg-1)] to determine the 222Rn flux density for known soil 226Ra activity concentrations across the two orebodies from the estimated average 226Ra activity concentrations. Using this conversion factor, they estimate radon flux densities of 4.1 Bq∙m-2∙s-1 for orebody 1 and 2.5 Bq∙m-2∙s-1 for orebody 3, respectively. These values are higher but within the 95% confidence intervals determined in our study.

Some of the difference in radon exhalation flux densities for orebody 3 can be explained by the smaller areal extent of the orebody 3 study area compared to our study (see above). In addition, the conversion factor of 1.85±0.23 [(mBq.m-2.s-1)/(Bq∙kg-1)] used by Kvasnicka and Auty (1994) to calculate radon flux densities is much larger than the conversion factor estimated in our study from direct radon flux density measurements and 226Ra measurements in soil samples via HPGe gamma spectrometry (equation 4). The conversion factor determined in our study is 0.9±0.4 (mBq.m-2.s-1)/(Bq∙kg-1) similar to values published in Lawrence et al (2009) for vegetated woodland (0.61±0.03). Kvasnicka and Auty’s (1994) conversion factor may potentially be too high, due to radon potentially diffusing from deeper layers, with higher 226Ra activity concentrations than the surface few centimetres of the soil. This may lead to higher estimated radon flux densities.

The terrestrial gamma dose rate to radon flux density conversion described by equation (5) provides a more reliable approach to convert 226Ra activity concentrations to average radon flux densities. Using equation (3) a conversion factor of (1.4±0.3) (mBq∙m-2∙s-1)/(Bq∙kg-1) was calculated, which is in agreement with direct measurements. Using this conversion factor, radon flux densities for orebodies 1 and 3 from Kvasnicka & Auty (1994) amount to 3.2 mBq∙m-2∙s-1 and 1.9 mBq∙m-2∙s-1, similar to the averages determined in our study.

The average radon flux density determined from measurement in the field above Anomaly 2 were 2.2 Bq∙m-2∙s-1 compared to 0.9 Bq∙m-2∙s-1 determined using the radiological GIS data. This difference is due to the over-representation of radon exhalation measurements on top of the radiation anomalies (Figure 9) compared to other areas. Repeated measurements on high radon flux areas will skew the average value of the field measurements towards a higher value. The typical radon flux density, ie the geometric mean of all individual field measurements taken above the Anomaly, is 0.7 Bq∙m-2∙s-1, similar to the average determined using the GIS model.

### 6.2.3  Airborne radon activity concentration

Airborne radon activity concentrations were measured at 1.5 m height on Anomaly 2 in the dry season 2009. Figure 41 shows the measured radon activity concentrations at various heights (30 cm, 50 cm and 150 cm) above ground (from Bollhöfer et al 2011) and its dependence on 226Ra soil activity concentrations at the sites.

Figure 38  Radon activity concentrations measured in air at 30 cm, 50 cm and 150 cm above ground over the Anomaly 2 area for the dry season 2009. Also given are soil 226Ra activity concentrations.

Whereas the soil activity concentrations vary by almost 4 orders of magnitude (14 – 111 000 Bq∙kg-1) radon activities in air at 1.5 m height vary much less (45 – 427 Bq∙m-3). This indicates rapid aerial dispersion of radon and effective mixing at this height. The typical dry season radon concentration (ie the geometric mean of the measured values) 1.5 m above the area investigated is ~150 Bq∙m3 (the arithmetic average is 175 Bq∙m3). This is about 2.5 times higher than the dry season radon concentration measured at Jabiru East of 60 Bq∙m3 (Akber & Pfitzner 1994) and about 6 times higher than annual average radon concentration measured at Jabiru of 24 Bq∙m3 (Akber et al 1992).

There is a positive correlation between the radon concentration at 1.5 m height and the 226Ra activity concentrations in the underlying soil (p < 0.005; R2 = 0.4), but radon concentration only increases slightly above the 150 Bq∙m3, rising by ~1 Bq∙m3 for every 370 Bq∙kg -1 increase in soil 226Ra activity concentration. Wet season radon concentrations in air will generally be lower than the values cited above, which has previously been shown at other areas in the Alligator Rivers Region (Martin et al 2004).

Figure 40 illustrates that at areas away from ‘hot spots’, radon concentration is relatively uniform vertically, but concentrations are higher when sitting or lying in close vicinity to the outcropping uranium anomalies that exhibit elevated 226Ra activity concentrations. This potential exposure route and its dependence on height needs to be factored into the broader land use requirements of local indigenous people when assessing potential radiation doses to humans that accessed the sites prior to mining and after rehabilitation.

No attempt has been made in our study to model the radon activity concentrations in air to the counts in the uranium channel of the 1997 or 1976 AGSs. Atmospheric dispersion does not allow concentrations in air to be correlated directly with radon exhalation or 226Ra activity concentrations in the underlying soil. Rather, the measured radon activity concentration values will be used to validate atmospheric transport models used to calculate radon activity concentrations in air close to sources of known radon activity fluxes.

## 6.2  Djalkmara and Corridor Creek Land Application Areas

The Djalkmara and Corridor Creek land application areas are located on top of natural radiation anomalies, as can be seen in Figure 39. The Djalkmara LAA lies on top of radiation Anomaly 4, whereas parts of the Corridor Creek land application areas lie on top of radiation Anomaly 5 (Ryan 1972; Eupene et al 1975). These anomalies are smaller in intensity and size than anomalies 1, 2 and 3, but both external gamma dose rates and radon flux densities are somewhat higher than the values for the surrounding areas.The average pre-mining values for the external gamma dose rate, radon activity flux and 226Ra activity concentration estimated from our study for the Djalkmara LAA are 0.2 µGy∙hr-1, 0.45 Bq∙m-2∙s-1 and 310 Bq∙kg-1 respectively. For the Corridor Creek land application areas the averages are 0.13 µGy∙hr-1, 0.23 Bq∙m-2∙s-1 and 160 Bq∙kg-1.

The slightly higher than background external dose rates and radon flux densities measured in the Djalkmara and Corridor Creek land application areas are in agreement with measurements performed recently within these areas, as part of a project to determine the radiological status of the Ranger land application areas. The average external gamma dose rate determined for the Djalkmara LAA was 0.29±0.09 µGy∙hr-1 excluding the contribution from applied radionuclides, and 0.21±0.06 µGy∙hr-1 for the Corridor Creek LAA (Akber et al 2011a). The average radon flux density at the Djalkmara LAA was 0.66 Bq∙m-2∙s-1 and most of this radon exhalation was attributed to the existing radiation anomaly rather than applied radionuclides (Akber et al 2011b). The Corridor Creek average was 0.28 Bq∙m-2∙s-1, in good agreement with the values determined using the GIS model.

## 6.3  Environmental background

There are many data published on environmental background dose rates in the Alligator Rivers Region and close to Ranger uranium mine. Akber et al (2011) report background external gamma dose rates for Jabiru East and the South Alligator Ranger Station of 0.09 and 0.07 µGy∙hr-1, respectively. Storm and Martin (1995) also measured background external gamma dose rates at Jabiru East and report values between 0.12 and 0.13 µGy∙hr-1. Lawrence (2005) measured external gamma dose rates (unpublished data) at Jabiru Water Tower, Jabiru East and the Mudginberri radon station and values there amounted to 0.14, 0.12 and 0.21 µGy∙hr-1, respectively.

In the Draft Jabiluka Environmental Impact Statement (ERA 1996) gamma dose rates are given for a 300m x 300m area covering parts of the Jabiluka evaporation pond and infrastructure, before works commenced. Assuming that the values reported are given in µGy∙hr-1 (rather than mGy∙hr-1, see Table 9.5 in Main Report) the average at Jabiluka is 0.05 µGy∙hr-1, with a range of 0.035-0.083 µGy∙hr-1. These values are generally in agreement with the magnitude of background dose rates determined using our GIS data, which provides around 0.10 µGy∙hr-1 for Jabiru and Jabiru East, 0.08 µGy∙hr-1 for the South Alligator Ranger Station and 0.08 µGy∙hr-1 for Jabiluka.

Radon flux densities for environmental background sites calculated using the conversion factors above (equation 9) and assuming about 35 counts per second (typical for environmental background) up to 55 counts per second (determined for the Jabiru area) amount to 0.06 – 0.12 Bq∙m-2∙s-1. This is in agreement with values reported previously for the area (Lawrence 2005; Todd et al 1994).

# 7  Conclusions

Pre-mining radiological conditions have been determined from historic AGS data by groundtruthing an undisturbed radiological anomaly and extrapolating to the whole extent of the AGS including the unmined Ranger Project Area. The minimum footprint area that can be assessed is set by the optimum buffer radius determined when up-scaling the external gamma dose rates measured on the ground to the AGS data. For the current case this is approximately 4 ha. Thus, the correlation models developed allow estimates to be made of the pre-mining baseline gamma dose rates, 226Ra soil activity concentrations and 222Rn fluxes for any selected area larger than 4 ha covered by the 1976 AGS over the greater Ranger area.

Comparison with published data on external gamma dose rates, 226Ra activity concentrations and 222Rn flux densities measured in the greater Ranger region shows that our model estimates are in good agreement with radiation levels estimated previously via direct measurement on top of orebody 3, and from previous environmental radiation surveys. It has thus been shown that the model can reliably estimate the pre-mining radiological conditions at Ranger mine. Whereas the gamma dose rates, 226Ra soil activity concentrations and 222Rn fluxes for the Ranger Project Area as a whole (7881 ha) are on average not different from typical backgrounds measured in the region, there are areas, notably orebodies 1 (40 ha) and 3 (77 ha) and Anomaly 2 (38 ha), that exhibited gamma dose rates 3 to 9 times higher, and 226Ra soil activity concentrations and 222Rn flux densities, respectively, 7 to 20 times higher than typical environmental background. This needs to be taken into account when estimating pre-mining radiation doses that may have been received by indigenous people accessing the area around the orebodies before mining started.

The GIS model will also allow an estimate of pre-mining uptake of uranium series radionuclides into biota over the footprint of the Ranger mine, assuming secular equilibrium of the radionuclides in soils and using concentration ratios determined for bushtucker in the region (eg Martin et al 1998; Ryan et al 2003) or from the BRUCE database (Doering et al in prep). This will facilitate the calculation of pre mining ingestion doses from the consumption of traditional foods harvested on site, in addition to an estimate of the internal and external radiation doses to humans and the environment.

# References

Akber RA & Pfitzner JL 1994. Atmospheric concentrations of radon and radon daughters in Jabiru East. Technical memorandum 45, Supervising Scientist for the Alligator Rivers Region, AGPS, Canberra.

Akber R, Pfitzner J & Johnston A 1992. Radon transport from Ranger Uranium Mine: a review of the public radiation dose estimates. Radiation Protection in Australia 10, 41–46.

Akber R, Lu P & Bollhöfer A 2011. 222Rn in the land application areas. Energy Resources of Australia Ltd, Darwin.

Australian Atomic Energy Commission (AAEC) 1963. Rum Jungle Project. A report on the uranium mining and treatment industry established by the Commonwealth Government at Rum Jungle, and its contribution to the development of the Northern Territory of Australia.

Australian Radiation Protection and Nuclear Safety Agency (ARPANSA), 2005. Code of Practice and Safety Guide for Radiation Protection and Radioactive Waste Management in Mining and Mineral Processing. Radiation Protection Series Publication No 9.

Australian Radiation Protection and Nuclear Safety Agency (ARPANSA) 2002. Recommendations for limiting exposure to ionizing radiation (1995) (Guidance note [NOHSC:3022(1995)]) and National standard for limiting occupational exposure to ionizing radiation [NOHSC:1013(1995)], Radiation Protection Series Publication No 1, Republished March 2002.

Berkman DA 1968. The geology of the Rum Jungle uranium deposits. In Uranium in Australia: symposium at Rum Jungle NT, June 16–21, 1968. Australasian Institute of Mining and Metallurgy, Darwin Branch, 12–31.

Bollhöfer A, Ryan B, Pfitzner K, Martin P & Iles M 2002. A radiation dose estimate for visitors of the South Alligator River valley, Australia, from remnants of uranium mining and milling activities. In Uranium mining and hydrogeology III, eds BJ Merkel, B Planer-Friedrich & C Wolkersdorfer. Technical University, Bergakademie Freiberg, 931–940.

Bollhöfer A, Storm J, Martin P & Tims S 2005. Geographic variability in radon exhalation at a rehabilitated uranium mine in the Northern Territory, Australia. Environmental Monitoring and Assessment 114, 313–330.

Bollhöfer A, Pfitzner K, Ryan B & Fawcett M 2007a. Ground truthing of an airborne gamma survey and assessment of the radiological conditions of the Sleisbeck mine area. Internal Report 526, June, Supervising Scientist, Darwin. Unpublished paper.

Bollhöfer A, Pfitzner K, Ryan B, Esparon A, Brazier J & Jones D 2007b. Radiological assessment of Rum Jungle mine, Northern Territory. Report to the Department of Regional Development, Primary Industry, Fisheries and Resources, Northern Territory Government, Darwin.

Bollhöfer A, Pfitzner K, Ryan B, Martin P, Fawcett M & Jones DR 2008. Airborne gamma survey of the historic Sleisbeck mine area in the Northern Territory, Australia, and its use for site rehabilitation planning. Journal of Environmental Radioactivity 99, 1770–1774.

Bollhöfer A, Esparon A & Pfitzner K 2011. Pre-mining radiological conditions at Ranger mine. In eriss research summary 2009–2010. eds Jones DR & Webb A, Supervising Scientist Report 202, Supervising Scientist, Darwin NT, 101–106.

Coetzee H, Wade P & Winde F 2006, Understanding environmental geophysical anomalies - An interdisciplinary case study from the West Rand. South African Journal of Geology 109, 495–502.

Commonwealth of Australia, 1999. Environmental Requirements of the Commonwealth of Australia for the Operation of Ranger Uranium Mine.

Conway NF, Davy DR, Giles MS, Newton PJF & Pollard DA 1974. The Alligator Rivers Area Fact Finding Study. Four Reports by the Australian Atomic Energy Commission (AAEC).

Darnley AG 1972. Airborne gamma-ray survey techniques. In Proceedings of a NATO sponsored Advanced Study Institute on Methods of Prospecting for Uranium Minerals, eds Bowie SHU, Davis M & Ostle D, London Inst. Min. Metall, 174–211.

Dickson BL & Snelling AA 1980. Movemenets of uranium and daughter isotopes in the Koongarra uranium deposit. In Uranium in the Pine Creek Geosyncline. Proceedings of the International Uranium Symposium on the Pine Creek Geosyncline. International Atomic Energy Agency, Vienna, 499–508.

Durrani SA & Ilic R (eds) 1997. Radon measurements by etched track detectors: Applications in radiation protection, earth sciences, and the environment. World Scientific, Singapore.

Esparon A, Pfitzner K, Bollhöfer A, Ryan B 2009. Pre-mining radiological conditions at Ranger mine. In eriss research summary 2007–2008. eds Jones DR & Webb A, Supervising Scientist Report 200, Supervising Scientist, Darwin NT, 111–116.

Esparon A & Pfitzner J 2010. Visual gamma: eriss gamma analysis technical manual. Internal Report 539, December, Supervising Scientist, Darwin.

Eupene GS, Fee PH & Colville RG 1975. Ranger 1 uranium deposits. In: Economic geology of Australia and Papua New Guinea. 1. Metals. ed Knight CL, Australasian Institute of Mining & Metallurgy; Monograph 5, 308–317.

Foy MF & Pedersen CP 1975. Koongarra uranium deposit. In Economic geology of Australia and Papua New Guinea. 1. Metals. ed Knight CL, Australasian Institute of Mining & Metallurgy; Monograph 5, 317–321.

Giblin A 2004. Alligator Rivers Uranium Deposits (Koongarra, Nabarlek and Ranger One) Northern Territory. CRC LEME, CSIRO Exploration and Mining.

Hegge MR, Mosher DV, Eupene GS & Anthony PJ 1980. Geologic setting of the East Alligator uranium deposits and prospects. In Uranium in the Pine Creek Geosyncline. Proceedings of the International Uranium Symposium on the Pine Creek Geosyncline. International Atomic Energy Agency, Vienna, 259–272.

International Atomic Energy Agency (IAEA) 1989. Construction and use of calibration facilities for radiometric field equipment. Technical Reports Series No 309, International Atomic Energy Agency, Vienna.

International Commission on Radiation Units and Measurements (ICRU) 1994. Gamma-ray spectrometry in the environment. ICRU Report 53.

International Commission on Radiological Protection (ICRP) 2000. Radiation protection recommendations as applied to the disposal of long-lived solid radioactive waste. ICRP Publication 81, Pergamon Press, Oxford.

Knoll GF 1979. Radiation detection and measurement. 2nd edn, John Wiley & Sons, New York.

Kvasnicka J & Auty RF 1994. Assessment of background radiation exposures at Ranger Uranium Mine. Radiation Protection in Australia October 12(4), 126–134.

Lawrence CE 2005. Measurement of 222Rn exhalation rates and 210Pb deposition rates in a tropical environment. PhD thesis. Queensland University of Technology, Brisbane.

Lawrence CE, Akber RA, Bollhöfer A & Martin P 2009. Radon-222 exhalation from open ground on and around a uranium mine in the wet-dry tropics. Journal of Environmental Radioactivity 100, 1–8.

Marten R 1992. External gamma dose rate survey of the Ranger Uranium Mine land application plot. In: Proceedings of the Workshop on Land Application of Effluent Water from Uranium Mines in the Alligator Rivers Region. Supervising Scientist for the Alligator Rivers Region, AGPS, Canberra.

Marten R 1992. Procedures for routine analysis of naturally occurring radionuclides in environmental samples by gamma-ray spectrometry with HPGe detectors. Internal report 76, Supervising Scientist for the Alligator Rivers Region, Canberra. Unpublished paper.

Martin P, Tims S, Ryan B & Bollhöfer A 2004. A radon and meteorological measurement network for the Alligator Rivers Region, Australia. Journal of Environmental Radioactivity 76, 35–49.

Martin P, Tims S, McGill A, Ryan B & Pfitzner K 2006. Use of airborne γ-ray spectrometry for environmental assessment of the rehabilitated Nabarlek uranium mine, northern Australia. Environmental Monitoring and Assessment 115, 531–553.

Murray AS, Marten R, Johnston A & Martin P 1987. Analysis for naturally occurring radionuclides at environmental concentrations by gamma spectrometry. Journal of Radioanalytical and Nuclear Chemistry, Articles 115, 263–288.

Northern Territory Geological Survey (NTGS) 2007. MODAT, mineral occurrence database [electronic resource (CD)], Darwin. ISSN 1445-5358.

Pfitzner J 2010. eriss HPGe detector calibration. Internal Report 576, October, Supervising Scientist, Darwin. Unpublished paper.

Pfitzner K & Martin P 2000. Airborne gamma survey of the South Alligator River valley: First report. Internal Report 353, Supervising Scientist, Darwin. Unpublished paper.

Pfitzner K & Martin P 2003. An assessment of radiation anomalies in the Alligator Rivers Region – a review. Internal Report 446, July, Supervising Scientist, Darwin. Unpublished paper.

Pfitzner K, Martin P & Ryan B 2001a. Airborne gamma survey of the upper South Alligator River valley: Second Report. Internal Report 377, Supervising Scientist, Darwin. Unpublished paper.

Pfitzner K, Ryan B, Bollhöfer & Martin P 2001b. Airborne gamma survey of the upper South Alligator River valley: Third Report. Internal Report 383, Supervising Scientist, Darwin. Unpublished paper.

Pfitzner K, Ryan B & Martin P 2003. Airborne gamma survey of the Sleisbeck mine area. Internal Report 400, January, Supervising Scientist, Darwin. Unpublished paper.

Porstendörfer J 1994. Properties and behaviour of radon and thoron and their decay products in the air. Journal of Aerosol Science 25(2), 219–263.

Ryan GR 1972. Ranger 1: a case study. In Proceedings of a NATO sponsored Advanced Study Institute on Methods of Prospecting for Uranium Minerals, eds Bowie SHU, Davis M & Ostle D, London Institure of Mining and Metallurgy, 296–400.

Saito K & Jacob P 1995. Gamma ray fields in the air due to sources in the ground. Radiat. Prot. Dosim. 58(1), 29–45.

Snelling AA 1980. Uraninite and its alteration products, Koongarra uranium deposit. In Uranium in the Pine Creek Geosyncline. Proceedings of the International Uranium Symposium on the Pine Creek Geosyncline. International Atomic Energy Agency, Vienna, 487–498.

Spehr W & Johnston A 1983. The measurement of radon exhalation rates using activated charcoal. Radiation Protect. Australia 1(3), 113–116.

Tipper DB & Lawrence G 1972. The Nabarlek area, Arnhemland, Australia: a case history. In Proceedings of a NATO sponsored Advanced Study Institute on Methods of Prospecting for Uranium Minerals, eds Bowie SHU, Davis M & Ostle D, London Inst. Min. Metall, 301–305.

Todd R, Akber RA & Martin P 1998. 222Rn and 220Rn activity flux from the ground in the vicinity of Ranger Uranium Mine. Internal report 279, Supervising Scientist, Canberra. Unpublished paper.

Tucker DH, Stuart DC, Hone IG & Sampath N 1980. The characteristics and interpretation of regional gravity, magnetic and radiometric surveys in the Pine Creek Geosyncline. In Uranium in the Pine Creek Geosyncline. Proceedings of the International Uranium Symposium on the Pine Creek Geosyncline. International Atomic Energy Agency, Vienna, 101–140.

Winkelmann I, Thomas M & Vogl K 2001. Aerial measurements on uranium ore mining, milling and processing areas in Germany. Journal of Environmental Radioactivity 53, 301–311.

# Appendix A  Results of external gamma dose rate and in-situ soil activity concentration measurements

Appendix A1  γ dose rates and in-situ γ-spectrometry results from Anomaly 2 survey (4 pixels at 70 m x 70 m each) conducted 21–24 August 2007. GM tube 1 was used for dose rate measurements, a GS-512 hand held NaI gamma spectrometer (Geofyzika, now SatisGeo) was used for in-situ γ-spectrometry.

| line | pos | UTM easting | UTM northing | counts/ 100s | uGy/hr meas | uGy/hr calc | +- | K % | U ppm | Th ppm |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 37-16 | 1 | 272510.41 | 8593989 | 249 | 0.15 | 0.160 | 0.010 | 0.2 | 5.4 | 8.5 |
| 37-0 | 2 | 272519.43 | 8593993 | 245 | 0.175 | 0.158 | 0.010 | 0.1 | 7.8 | 9.2 |
| 37-39 | 3 | 272503.26 | 8593978 | 203 | 0.1 | 0.132 | 0.009 | 0.1 | 3.4 | 6.6 |
| 37-24 | 4 | 272515.9 | 8593982 | 250 | 0.14 | 0.161 | 0.010 | 0.2 | 6.1 | 8.2 |
| 37-14 | 5 | 272512.35 | 8593972 | 238 | 0.15 | 0.153 | 0.010 | 0.1 | 4.2 | 5.1 |
| 37-27 | 6 | 272519.57 | 8593976 | 223 | 0.125 | 0.144 | 0.009 | 0.1 | 4.9 | 6.1 |
| 37-02 | 7 | 272528.78 | 8593956 | 234 | 0.125 | 0.151 | 0.009 | 0.1 | 4.7 | 5.9 |
| 37-09 | 8 | 272528.72 | 8593963 | 245 | 0.14 | 0.158 | 0.010 | 0.3 | 5.3 | 7 |
| 37-06 | 9 | 272514.25 | 8593961 | 229 | 0.125 | 0.148 | 0.009 | 0.2 | 4.3 | 7 |
| 37-03 | 10 | 272528.78 | 8593956 | 249 | 0.14 | 0.160 | 0.010 | 0.1 | 6.2 | 6.2 |
| 37-33 | 11 | 272536.08 | 8593949 | 250 | 0.15 | 0.161 | 0.010 | 0.2 | 5.6 | 7.9 |
| 37-32 | 12 | 272528.86 | 8593947 | 225 | 0.14 | 0.145 | 0.009 | 0.2 | 5.4 | 7.1 |
| 37-35 | 13 | 272510.75 | 8593947 | 233 | 0.15 | 0.150 | 0.009 | 0.2 | 4 | 6.7 |
| 37-05 | 14 | 272516.24 | 8593939 | 230 | 0.125 | 0.149 | 0.009 | 0.2 | 4 | 5.8 |
| 37-11 | 15 | 272523.51 | 8593936 | 233 | 0.125 | 0.150 | 0.009 | 0.2 | 4.6 | 6.8 |
| 37-17 | 16 | 272530.77 | 8593934 | 223 | 0.125 | 0.144 | 0.009 | 0.2 | 3.8 | 6 |
| 37-20 | 17 | 272534.36 | 8593938 | 240 | 0.14 | 0.155 | 0.010 | 0.3 | 3.6 | 6.6 |
| 37-10 | 18 | 272539.87 | 8593928 | 235 | 0.15 | 0.152 | 0.010 | 0.1 | 4.4 | 6.8 |
| 37-15 | 19 | 272534.48 | 8593923 | 223 | 0.125 | 0.144 | 0.009 | 0 | 5.2 | 6.5 |
| 37-13 | 20 | 272541.72 | 8593923 | 235 | 0.14 | 0.152 | 0.010 | 0.1 | 4.8 | 6.4 |
| 37-34 | 21 | 272547.14 | 8593925 | 225 | 0.125 | 0.145 | 0.009 | 0.1 | 5 | 7.1 |
| 37-07 | 22 | 272543.46 | 8593932 | 249 | 0.13 | 0.160 | 0.010 | 0.2 | 4.3 | 5.7 |
| 37-08 | 23 | 272559.81 | 8593925 | 259 | 0.15 | 0.166 | 0.010 | 0.2 | 5.2 | 7.6 |
| 37-18 | 24 | 272567.03 | 8593929 | 242 | 0.1 | 0.156 | 0.010 | 0.1 | 7.5 | 7.8 |
| 37-29 | 25 | 272563.37 | 8593932 | 244 | 0.15 | 0.157 | 0.010 | 0.2 | 6.3 | 7.3 |
| 37-26 | 26 | 272568.69 | 8593947 | 279 | 0.14 | 0.179 | 0.010 | 0.2 | 8.5 | 8.3 |
| 37-04 | 27 | 272564.99 | 8593956 | 289 | 0.15 | 0.185 | 0.011 | 0.2 | 11.7 | 7.7 |
| 37-38 | 28 | 272543.31 | 8593951 | 237 | 0.14 | 0.153 | 0.010 | 0.1 | 6.1 | 5.7 |
| 37-30 | 29 | 272539.62 | 8593960 | 239 | 0.15 | 0.154 | 0.010 | 0.1 | 6.4 | 5.6 |
| 37-12 | 30 | 272555.91 | 8593960 | 289 | 0.16 | 0.185 | 0.011 | 0.3 | 8.8 | 7.5 |
| 37-31 | 31 | 272563.12 | 8593964 | 304 | 0.175 | 0.194 | 0.011 | 0.2 | 9.5 | 8.7 |
| 37-36 | 32 | 272543.21 | 8593964 | 232 | 0.14 | 0.150 | 0.009 | 0.2 | 6.2 | 7.9 |
| 37-28 | 33 | 272552.23 | 8593967 | 268 | 0.125 | 0.172 | 0.010 | 0.2 | 8 | 7.6 |
| 37-22 | 34 | 272537.72 | 8593971 | 255 | 0.14 | 0.164 | 0.010 | 0.1 | 6.3 | 7.5 |
| 37-21 | 35 | 272539.48 | 8593976 | 239 | 0.13 | 0.154 | 0.010 | 0.2 | 7 | 6.6 |
| 37-25 | 36 | 272535.85 | 8593978 | 254 | 0.15 | 0.163 | 0.010 | 0.2 | 6 | 6.3 |
| 37-19 | 37 | 272552.13 | 8593980 | 271 | 0.14 | 0.174 | 0.010 | 0.3 | 6.9 | 6.8 |
| 37-37 | 38 | 272568.42 | 8593980 | 315 | 0.15 | 0.201 | 0.011 | 0.2 | 11.5 | 10.4 |
| 37-01 | 39 | 272559.32 | 8593986 | 272 | 0.15 | 0.175 | 0.010 | 0.2 | 9.8 | 9.1 |
| 37-23 | 40 | 272550.23 | 8593991 | 252 | 0.15 | 0.162 | 0.010 | 0.2 | 9.4 | 10.1 |
| 266-99 | 1 | 273064 | 8593658 | 325 | 0.15 | 0.207 | 0.011 | 0.3 | 12.7 | 12.6 |
| 266-101 | 2 | 273063.5 | 8593658 | 322 | 0.15 | 0.206 | 0.011 | 0.3 | 12.7 | 12.4 |
| 266-92 | 3 | 273076.13 | 8593663 | 335 | 0.18 | 0.214 | 0.011 | 0.4 | 11.8 | 15.1 |
| 266-114 | 4 | 273083.39 | 8593662 | 346 | 0.16 | 0.220 | 0.012 | 0.2 | 14 | 12.6 |
| 266-81 | 5 | 273092.52 | 8593653 | 353 | 0.2 | 0.225 | 0.012 | 0.2 | 14.2 | 13.3 |
| 266-94 | 6 | 273085.35 | 8593643 | 334 | 0.175 | 0.213 | 0.011 | 0.2 | 12.5 | 11.3 |
| 266-112 | 7 | 273074.49 | 8593643 | 352 | 0.2 | 0.224 | 0.012 | 0.1 | 13.4 | 13.3 |
| 266-117 | 8 | 273101.6 | 8593649 | 323 | 0.2 | 0.206 | 0.011 | 0.1 | 13.6 | 12.1 |
| 266-107 | 9 | 273116.13 | 8593644 | 323 | 0.19 | 0.206 | 0.011 | 0.2 | 12.3 | 10.4 |
| 266-95 | 10 | 273123.34 | 8593647 | 354 | 0.19 | 0.225 | 0.012 | 0.2 | 12.3 | 10.7 |
| 266-100 | 11 | 273123.27 | 8593656 | 364 | 0.16 | 0.232 | 0.012 | 0.2 | 15.1 | 12.4 |
| 266-108 | 12 | 273108.77 | 8593658 | 368 | 0.2 | 0.234 | 0.012 | 0.3 | 14.5 | 12.2 |
| 266-97 | 13 | 273114.17 | 8593662 | 356 | 0.21 | 0.227 | 0.012 | 0.2 | 16.3 | 14 |
| 266-83 | 14 | 273106.87 | 8593669 | 346 | 0.2 | 0.220 | 0.012 | 0.3 | 14.3 | 12.8 |
| 266-102 | 15 | 273101.47 | 8593666 | 319 | 0.2 | 0.204 | 0.011 | 0.1 | 15.6 | 13.6 |
| 266-85 | 16 | 273094.25 | 8593662 | 362 | 0.19 | 0.230 | 0.012 | 0.2 | 13.7 | 13.3 |
| 266-105 | 17 | 273117.7 | 8593673 | 432 | 0.21 | 0.274 | 0.013 | 0.3 | 18.6 | 13.7 |
| 266-110 | 18 | 273124.9 | 8593679 | 392 | 0.2 | 0.249 | 0.012 | 0.2 | 20 | 15.4 |
| 266-104 | 19 | 273119.45 | 8593680 | 450 | 0.2 | 0.285 | 0.013 | 0.1 | 22.9 | 14.4 |
| 266-103 | 20 | 273115.79 | 8593686 | 374 | 0.25 | 0.238 | 0.012 | 0.3 | 22.9 | 13 |
| 266-89 | 21 | 273108.53 | 8593688 | 447 | 0.25 | 0.283 | 0.013 | 0.3 | 22.6 | 13.6 |
| 266-119 | 22 | 273103.17 | 8593678 | 406 | 0.21 | 0.258 | 0.012 | 0.2 | 21.3 | 13.1 |
| 266-116 | 23 | 273099.54 | 8593680 | 414 | 0.2 | 0.263 | 0.013 | 0.2 | 20.1 | 13 |
| 266-111 | 24 | 273104.88 | 8593691 | 411 | 0.21 | 0.261 | 0.013 | 0.1 | 21.9 | 12.9 |
| 266-115 | 25 | 273117.55 | 8593691 | 566 | 0.23 | 0.357 | 0.015 | 0.2 | 34.5 | 15.3 |
| 266-80 | 26 | 273126.53 | 8593701 | 573 | 0.24 | 0.361 | 0.015 | 0.2 | 38 | 15.7 |
| 266-96 | 27 | 273113.83 | 8593704 | 549 | 0.24 | 0.346 | 0.015 | 0.2 | 34.9 | 15.1 |
| 266-93 | 28 | 273108.41 | 8593702 | 483 | 0.24 | 0.305 | 0.014 | 0.4 | 32.1 | 15.1 |
| 266-86 | 29 | 273092.19 | 8593693 | 448 | 0.24 | 0.284 | 0.013 | 0.3 | 24.7 | 14.5 |
| 266-90 | 30 | 273090.41 | 8593689 | 406 | 0.22 | 0.258 | 0.012 | 0.2 | 21.9 | 12.9 |
| 266-84 | 31 | 273088.53 | 8593699 | 468 | 0.22 | 0.296 | 0.013 | 0.3 | 26.8 | 14.3 |
| 266-98 | 32 | 273081.27 | 8593700 | 477 | 0.25 | 0.302 | 0.014 | 0 | 28.1 | 13.7 |
| 266-87 | 33 | 273081.34 | 8593691 | 438 | 0.25 | 0.277 | 0.013 | 0.3 | 23.5 | 16.9 |
| 266-109 | 34 | 273081.37 | 8593688 | 390 | 0.19 | 0.248 | 0.012 | 0.2 | 19.7 | 15.4 |
| 266-118 | 35 | 273075.87 | 8593697 | 428 | 0.21 | 0.271 | 0.013 | 0 | 25.3 | 16.8 |
| 266-91 | 36 | 273075.78 | 8593708 | 533 | 0.23 | 0.336 | 0.014 | 0.3 | 33.3 | 16.6 |
| 266-82 | 37 | 273068.7 | 8593687 | 393 | 0.21 | 0.250 | 0.012 | 0.2 | 18.4 | 12.7 |
| 266-88 | 38 | 273061.5 | 8593682 | 379 | 0.18 | 0.241 | 0.012 | 0.3 | 15.2 | 13.8 |
| 266-113 | 39 | 273070.58 | 8593678 | 349 | 0.2 | 0.222 | 0.012 | 0.4 | 16.4 | 13.7 |
| 266-106 | 40 | 273076.02 | 8593678 | 360 | 0.2 | 0.229 | 0.012 | 0.2 | 15.2 | 12.7 |
| 267 | 1 | 273158 | 8593807 | 705 | 0.4 | 0.443 | 0.016 | 0.4 | 55.7 | 12 |
| 267 | 2 | 273161 | 8593793 | 545 | 0.35 | 0.344 | 0.014 | 0.3 | 40.3 | 9.6 |
| 267 | 3 | 273141 | 8593821 | 1089 | 0.6 | 0.681 | 0.020 | 0.7 | 103.6 | 9.3 |
| 267 | 4 | 273188 | 8593804 | 503 | 0.3 | 0.318 | 0.014 | 0.4 | 35.9 | 5.1 |
| 267 | 5 | 273162 | 8593840 | 654 | 0.4 | 0.411 | 0.016 | 0.4 | 50.1 | 10.7 |
| 267 | 6 | 273165 | 8593808 | 626 | 0.35 | 0.394 | 0.016 | 0.3 | 51.2 | 9.8 |
| 267 | 7 | 273153 | 8593814 | 683 | 0.4 | 0.429 | 0.016 | 0.7 | 51 | 8.7 |
| 267 | 8 | 273154 | 8593800 | 637 | 0.45 | 0.401 | 0.016 | 0.7 | 51.3 | 10.4 |
| 267 | 9 | 273157 | 8593800 | 596 | 0.35 | 0.375 | 0.015 | 0.3 | 43.2 | 8.2 |
| 267 | 10 | 273166 | 8593820 | 546 | 0.3 | 0.344 | 0.014 | 0.5 | 36.1 | 7.3 |
| 267 | 11 | 273174 | 8593806 | 552 | 0.25 | 0.348 | 0.015 | 0.3 | 37.4 | 6.1 |
| 267 | 12 | 273191 | 8593806 | 464 | 0.25 | 0.294 | 0.013 | 0.3 | 33.9 | 5.1 |
| 267 | 13 | 273197 | 8593803 | 591 | 0.4 | 0.372 | 0.015 | 0.5 | 43.7 | 5.6 |
| 267 | 14 | 273181 | 8593807 | 508 | 0.25 | 0.321 | 0.014 | 0.3 | 35.7 | 6.3 |
| 267 | 15 | 273185 | 8593816 | 517 | 0.3 | 0.326 | 0.014 | 0.2 | 37.1 | 6.7 |
| 267 | 16 | 273181 | 8593822 | 635 | 0.3 | 0.400 | 0.016 | 0.3 | 45.8 | 5.5 |
| 267 | 17 | 273177 | 8593825 | 628 | 0.3 | 0.395 | 0.016 | 0.4 | 44.2 | 5 |
| 267 | 18 | 273171 | 8593824 | 591 | 0.35 | 0.372 | 0.015 | 0.2 | 44.7 | 5.8 |
| 267 | 19 | 273156 | 8593821 | 755 | 0.35 | 0.474 | 0.017 | 0.5 | 60.7 | 10.8 |
| 267 | 20 | 273139 | 8593812 | 1120 | 0.6 | 0.700 | 0.021 | 0.6 | 108.4 | 7.3 |
| 267 | 21 | 273142 | 8593807 | 876 | 0.6 | 0.549 | 0.018 | 1 | 75.2 | 11.5 |
| 267 | 22 | 273135 | 8593803 | 1061 | 0.7 | 0.664 | 0.020 | 0.7 | 90.4 | 13.3 |
| 267 | 23 | 273131 | 8593794 | 958 | 0.6 | 0.600 | 0.019 | 0.5 | 91.7 | 11.4 |
| 267 | 24 | 273139 | 8593787 | 891 | 0.5 | 0.558 | 0.019 | 0.6 | 78.8 | 9.1 |
| 267 | 25 | 273150 | 8593790 | 607 | 0.35 | 0.382 | 0.015 | 0.4 | 44.9 | 8.3 |
| 267 | 26 | 273158 | 8593786 | 570 | 0.25 | 0.359 | 0.015 | 0.6 | 36.4 | 7.2 |
| 267 | 27 | 273158 | 8593783 | 495 | 0.25 | 0.313 | 0.014 | 0.4 | 33.8 | 7 |
| 267 | 29 | 273171 | 8593784 | 522 | 0.35 | 0.330 | 0.014 | 0.6 | 36.8 | 8.8 |
| 267 | 28 | 273156 | 8593821 | 549 | 0.35 | 0.346 | 0.015 | 0.3 | 41.5 | 10 |
| 267 | 30 | 273180 | 8593792 | 559 | 0.3 | 0.352 | 0.015 | 0.3 | 36 | 7.5 |
| 267 | 31 | 273184 | 8593791 | 514 | 0.3 | 0.325 | 0.014 | 0.4 | 36.9 | 7.9 |
| 267 | 32 | 273190 | 8593784 | 601 | 0.35 | 0.379 | 0.015 | 0.3 | 44.8 | 10.4 |
| 267 | 33 | 273186 | 8593782 | 614 | 0.35 | 0.387 | 0.015 | 0.4 | 43.7 | 10.7 |
| 267 | 34 | 273194 | 8593786 | 513 | 0.3 | 0.324 | 0.014 | 0.3 | 32.7 | 9.3 |
| 267 | 35 | 273195 | 8593785 | 511 | 0.3 | 0.323 | 0.014 | 0 | 33.5 | 8 |
| 267 | 36 | 273194 | 8593808 | 519 | 0.275 | 0.328 | 0.014 | 0.4 | 36.4 | 6.2 |
| 267 | 37 | 273181 | 8593849 | 1008 | 0.5 | 0.631 | 0.020 | 0.7 | 96.1 | 7.9 |
| 267 | 38 | 273155 | 8593844 | 1305 | 0.85 | 0.815 | 0.022 | 0.8 | 133.1 | 7.2 |
| 267 | 39 | 273144 | 8593848 | 986 | 0.6 | 0.617 | 0.019 | 0.6 | 92.4 | 4.9 |
| 267 | 40 | 273151 | 8593852 | 1273 | 0.7 | 0.795 | 0.022 | 0.8 | 124 | 5.9 |
| 720 | 1 | 272874 | 8593490 | 1083 | 0.6 | 0.677 | 0.020 | 1.2 | 97 | 10.2 |
| 720 | 2 | 272866 | 8593493 | 872 | 0.55 | 0.547 | 0.018 | 1.1 | 65.4 | 8.8 |
| 720 | 3 | 272864 | 8593499 | 1115 | 0.55 | 0.697 | 0.021 | 1.2 | 94.4 | 10.3 |
| 720 | 4 | 272856 | 8593492 | 851 | 0.45 | 0.534 | 0.018 | 0.5 | 67.9 | 7.2 |
| 720 | 5 | 272859 | 8593482 | 963 | 0.5 | 0.603 | 0.019 | 0.8 | 80.9 | 9.5 |
| 720 | 6 | 272855 | 8593476 | 963 | 0.55 | 0.603 | 0.019 | 1 | 87.3 | 8.4 |
| 720 | 7 | 272868 | 8593477 | 2282 | 1.4 | 1.421 | 0.030 | 2.2 | 227.5 | 13.3 |
| 720 | 8 | 272876 | 8593473 | 2171 | 1.4 | 1.352 | 0.029 | 2.8 | 244.5 | 14.7 |
| 720 | 9 | 272862 | 8593467 | 1685 | 1.25 | 1.051 | 0.025 | 1.9 | 183.6 | 11.6 |
| 720 | 10 | 272860 | 8593461 | 1330 | 0.85 | 0.831 | 0.023 | 2.2 | 136.8 | 9.9 |
| 720 | 11 | 272865 | 8593460 | 5578 | 3.5 | 3.464 | 0.046 |  |  |  |
| 720 | 12 | 272863 | 8593454 | 5474 | 3.5 | 3.400 | 0.046 |  |  |  |
| 720 | 13 | 272866 | 8593444 | 17184 | 11 | 10.660 | 0.081 |  |  |  |
| 720 | 14 | 272866 | 8593437 | 19151 | 13 | 11.880 | 0.086 |  |  |  |
| 720 | 15 | 272871 | 8593444 | 13092 | 8 | 8.123 | 0.071 |  |  |  |
| 720 | 16 | 272878 | 8593448 | 3742 | 2.5 | 2.326 | 0.038 | 5.4 | 431.8 | 37.4 |
| 720 | 17 | 272872 | 8593454 | 3343 | 2 | 2.079 | 0.036 | 4.6 | 388.8 | 31.7 |
| 720 | 18 | 272874 | 8593463 | 2757 | 1.15 | 1.715 | 0.033 | 3.2 | 340.4 | 22.4 |
| 720 | 19 | 272884 | 8593454 | 3429 | 2 | 2.132 | 0.036 | 3.9 | 392 | 27.2 |
| 720 | hot | 272886 | 8593455 | 22015 | 15 | 13.655 | 0.092 |  |  |  |
| 720 | 20 | 272891 | 8593457 | 12906 | 8 | 8.008 | 0.070 |  |  |  |
| 720 | 21 | 272889 | 8593445 | 1811 | 1 | 1.129 | 0.026 | 1.2 | 175.7 | 7.9 |
| 720 | 22 | 272908 | 8593444 | 1306 | 0.7 | 0.816 | 0.022 | 1.1 | 121.9 | 5.9 |
| 720 | 23 | 272917 | 8593441 | 1000 | 0.6 | 0.626 | 0.020 | 0.7 | 80.1 | 3.9 |
| 720 | 24 | 272918 | 8593452 | 1715 | 1.1 | 1.069 | 0.026 | 1.4 | 170.5 | 7.5 |
| 720 | 25 | 272900 | 8593457 | 8129 | 5 | 5.046 | 0.056 |  |  |  |
| 720 | 27 | 272915 | 8593466 | 2694 | 1.5 | 1.676 | 0.032 | 3.2 | 306.3 | 19.8 |
| 720 | 28 | 272904 | 8593470 | 5224 | 3.5 | 3.245 | 0.045 |  |  |  |
| 720 | 26 | 272906 | 8593462 | 6893 | 4.5 | 4.280 | 0.051 |  |  |  |
| 720 | 29 | 272898 | 8593471 | 2089 | 1.3 | 1.301 | 0.028 | 1.8 | 222.5 | 11.9 |
| 720 | 30 | 272889 | 8593467 | 3193 | 2 | 1.986 | 0.035 | 3.1 | 321.1 | 21.6 |
| 720 | 31 | 272906 | 8593479 | 1820 | 1 | 1.134 | 0.026 | 2.1 | 187.9 | 11.3 |
| 720 | 32 | 272911 | 8593486 | 2284 | 1.4 | 1.422 | 0.030 | 1.8 | 254.3 | 14.3 |
| 720 | 33 | 272918 | 8593489 | 2241 | 1.25 | 1.395 | 0.029 | 2.3 | 263.8 | 18.6 |
| 720 | 34 | 272913 | 8593491 | 1288 | 0.8 | 0.804 | 0.022 | 0.4 | 123.7 | 10.5 |
| 720 | 35 | 272906 | 8593493 | 979 | 0.6 | 0.613 | 0.019 | 0.7 | 95.1 | 10 |
| 720 | 36 | 272904 | 8593498 | 1080 | 0.6 | 0.676 | 0.020 | 0.7 | 92.3 | 8.5 |
| 720 | 37 | 272899 | 8593490 | 1005 | 0.5 | 0.629 | 0.020 | 1.1 | 88.1 | 8.5 |
| 720 | 38 | 272898 | 8593501 | 993 | 0.55 | 0.622 | 0.020 | 0.8 | 85.1 | 8.9 |
| 720 | 39 | 272886 | 8593487 | 1109 | 0.65 | 0.693 | 0.021 | 1.2 | 92.1 | 7.8 |
| 720 | 40 | 272873 | 8593501 | 792 | 0.5 | 0.497 | 0.017 | 1 | 62.7 | 9.8 |

Appendix A2  γ dose rate results from Anomaly 2A survey conducted 21-24 July 2008. Measured values were read off the analogue dial in the field, calculated values used the calibration equation.

| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |  |
| --- | --- | --- | --- | --- | --- | --- |
| 272628.29 | 8593516.04 | 348 | 0.2 | 0.18 | 0.01 |
| 272642.76 | 8593518 | 346 | 0.2 | 0.18 | 0.01 |
| 272653.62 | 8593518.08 | 319 | 0.175 | 0.17 | 0.01 |
| 272660.86 | 8593518.14 | 379 | 0.2 | 0.20 | 0.01 |
| 272669.9 | 8593520.06 | 391 | 0.2 | 0.21 | 0.01 |
| 272680.76 | 8593520.15 | 356 | 0.2 | 0.19 | 0.01 |
| 272686.22 | 8593516.5 | 376 | 0.2 | 0.20 | 0.01 |
| 272693.46 | 8593516.56 | 317 | 0.175 | 0.17 | 0.01 |
| 272707.96 | 8593514.83 | 334 | 0.2 | 0.18 | 0.01 |
| 272720.64 | 8593514.93 | 381 | 0.25 | 0.20 | 0.01 |
| 272733.31 | 8593515.04 | 399 | 0.25 | 0.21 | 0.01 |
| 272747.79 | 8593515.15 | 428 | 0.25 | 0.23 | 0.01 |
| 272756.86 | 8593513.38 | 467 | 0.25 | 0.25 | 0.01 |
| 272769.5 | 8593517.17 | 509 | 0.25 | 0.27 | 0.01 |
| 272782.18 | 8593517.27 | 518 | 0.25 | 0.28 | 0.01 |
| 272794.81 | 8593522.91 | 665 | 0.35 | 0.36 | 0.01 |
| 272803.84 | 8593524.82 | 613 | 0.3 | 0.33 | 0.01 |
| 272809.31 | 8593521.18 | 637 | 0.35 | 0.34 | 0.01 |
| 272823.8 | 8593519.45 | 733 | 0.4 | 0.40 | 0.02 |
| 272836.51 | 8593515.87 | 744 | 0.4 | 0.40 | 0.02 |
| 272851.01 | 8593514.14 | 673 | 0.4 | 0.36 | 0.01 |
| 272858.25 | 8593514.2 | 623 | 0.35 | 0.34 | 0.01 |
| 272865.49 | 8593514.25 | 855 | 0.5 | 0.47 | 0.02 |
| 272874.54 | 8593514.33 | 826 | 0.45 | 0.45 | 0.02 |
| 272887.22 | 8593514.43 | 787 | 0.45 | 0.43 | 0.02 |
| 272905.29 | 8593518.26 | 624 | 0.4 | 0.34 | 0.01 |
| 272917.95 | 8593520.21 | 648 | 0.4 | 0.35 | 0.01 |
| 272925.22 | 8593516.58 | 609 | 0.4 | 0.33 | 0.01 |
| 272937.87 | 8593520.37 | 650 | 0.4 | 0.35 | 0.01 |
| 272950.54 | 8593520.47 | 558 | 0.35 | 0.30 | 0.01 |
| 272961.42 | 8593518.71 | 614 | 0.35 | 0.33 | 0.01 |
| 272975.92 | 8593516.99 | 794 | 0.45 | 0.43 | 0.02 |
| 272990.39 | 8593518.95 | 593 | 0.35 | 0.32 | 0.01 |
| 273004.87 | 8593519.06 | 457 | 0.25 | 0.24 | 0.01 |
| 273012.13 | 8593517.28 | 578 | 0.35 | 0.31 | 0.01 |
| 273021.19 | 8593515.51 | 519 | 0.3 | 0.28 | 0.01 |
| 273032.09 | 8593511.9 | 482 | 0.3 | 0.26 | 0.01 |
| 273046.57 | 8593512.02 | 386 | 0.25 | 0.20 | 0.01 |
| 273061.05 | 8593512.14 | 463 | 0.3 | 0.25 | 0.01 |
| 273070.11 | 8593512.21 | 454 | 0.3 | 0.24 | 0.01 |
| 273081 | 8593508.61 | 472 | 0.3 | 0.25 | 0.01 |
| 273091.86 | 8593508.7 | 473 | 0.3 | 0.25 | 0.01 |
| 273077.59 | 8593482.76 | 338 | 0.175 | 0.18 | 0.01 |
| 273064.9 | 8593484.5 | 470 | 0.3 | 0.25 | 0.01 |
| 273053.99 | 8593489.95 | 432 | 0.25 | 0.23 | 0.01 |
| 273041.3 | 8593491.69 | 481 | 0.3 | 0.26 | 0.01 |
| 273032.28 | 8593487.93 | 547 | 0.35 | 0.29 | 0.01 |
| 273021.39 | 8593491.53 | 664 | 0.4 | 0.36 | 0.01 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |  |
| 273010.51 | 8593493.29 | 486 | 0.3 | 0.26 | 0.01 |
| 272996.01 | 8593495.02 | 454 | 0.25 | 0.24 | 0.01 |
| 272986.94 | 8593496.79 | 491 | 0.3 | 0.26 | 0.01 |
| 272976.06 | 8593498.54 | 581 | 0.35 | 0.31 | 0.01 |
| 272961.58 | 8593498.43 | 716 | 0.4 | 0.39 | 0.01 |
| 272947.1 | 8593498.31 | 1222 | 0.7 | 0.67 | 0.02 |
| 272936.23 | 8593498.22 | 1164 | 0.7 | 0.64 | 0.02 |
| 272925.4 | 8593494.45 | 1325 | 0.8 | 0.73 | 0.02 |
| 272910.95 | 8593490.64 | 2261 | 1 | 1.25 | 0.03 |
| 272896.43 | 8593494.22 | 1027 | 0.6 | 0.56 | 0.02 |
| 272878.31 | 8593495.91 | 1242 | 0.7 | 0.68 | 0.02 |
| 272865.64 | 8593495.81 | 948 | 0.5 | 0.52 | 0.02 |
| 272856.57 | 8593497.58 | 932 | 0.5 | 0.51 | 0.02 |
| 272843.88 | 8593499.33 | 829 | 0.5 | 0.45 | 0.02 |
| 272834.85 | 8593497.41 | 712 | 0.4 | 0.39 | 0.01 |
| 272820.35 | 8593499.14 | 530 | 0.3 | 0.28 | 0.01 |
| 272813.12 | 8593497.23 | 516 | 0.3 | 0.28 | 0.01 |
| 272796.83 | 8593497.1 | 642 | 0.35 | 0.35 | 0.01 |
| 272778.68 | 8593502.49 | 562 | 0.35 | 0.30 | 0.01 |
| 272764.21 | 8593500.53 | 501 | 0.3 | 0.27 | 0.01 |
| 272755.14 | 8593502.3 | 480 | 0.3 | 0.26 | 0.01 |
| 272742.48 | 8593500.36 | 379 | 0.25 | 0.20 | 0.01 |
| 272731.62 | 8593500.27 | 414 | 0.25 | 0.22 | 0.01 |
| 272717.13 | 8593500.15 | 325 | 0.2 | 0.17 | 0.01 |
| 272708.05 | 8593503.77 | 271 | 0.15 | 0.14 | 0.01 |
| 272699.01 | 8593501.85 | 279 | 0.15 | 0.14 | 0.01 |
| 272684.56 | 8593498.05 | 299 | 0.15 | 0.16 | 0.01 |
| 272668.24 | 8593501.6 | 377 | 0.25 | 0.20 | 0.01 |
| 272655.59 | 8593497.81 | 338 | 0.25 | 0.18 | 0.01 |
| 272644.73 | 8593497.72 | 411 | 0.25 | 0.22 | 0.01 |
| 272633.91 | 8593492.1 | 447 | 0.25 | 0.24 | 0.01 |
| 272626.67 | 8593492.05 | 406 | 0.2 | 0.22 | 0.01 |
| 272630.63 | 8593449.66 | 1109 | 0.7 | 0.61 | 0.02 |
| 272639.67 | 8593451.58 | 1137 | 0.7 | 0.62 | 0.02 |
| 272654.17 | 8593449.85 | 1022 | 0.6 | 0.56 | 0.02 |
| 272674.07 | 8593451.85 | 863 | 0.5 | 0.47 | 0.02 |
| 272690.38 | 8593450.14 | 899 | 0.5 | 0.49 | 0.02 |
| 272697.6 | 8593452.04 | 857 | 0.5 | 0.47 | 0.02 |
| 272708.47 | 8593452.13 | 1164 | 0.6 | 0.64 | 0.02 |
| 272715.71 | 8593452.19 | 1572 | 0.9 | 0.87 | 0.02 |
| 272726.59 | 8593450.43 | 1085 | 0.6 | 0.60 | 0.02 |
| 272739.25 | 8593452.38 | 962 | 0.5 | 0.53 | 0.02 |
| 272753.74 | 8593450.65 | 487 | 0.3 | 0.26 | 0.01 |
| 272764.59 | 8593452.58 | 403 | 0.2 | 0.21 | 0.01 |
| 272773.66 | 8593450.81 | 593 | 0.35 | 0.32 | 0.01 |
| 272788.13 | 8593452.77 | 1031 | 0.6 | 0.57 | 0.02 |
| 272802.63 | 8593451.04 | 576 | 0.3 | 0.31 | 0.01 |
| 272813.49 | 8593451.13 | 789 | 0.4 | 0.43 | 0.02 |
| 272824.35 | 8593451.22 | 844 | 0.5 | 0.46 | 0.02 |
| 272840.65 | 8593451.35 | 960 | 0.5 | 0.53 | 0.02 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |  |
| 272847.89 | 8593451.41 | 1215 | 0.7 | 0.67 | 0.02 |
| 272856.94 | 8593451.48 | 2135 | 1.25 | 1.18 | 0.03 |
| 272867.82 | 8593449.72 | 10834 | 6 | 6.06 | 0.06 |
| 272880.48 | 8593451.67 | 3291 | 2 | 1.83 | 0.03 |
| 272889.53 | 8593451.74 | 29049 | 15 | 16.26 | 0.10 |
| 272896.77 | 8593451.8 | 2391 | 1.5 | 1.33 | 0.03 |
| 272904.03 | 8593450.01 | 1688 | 1 | 0.93 | 0.02 |
| 272913.07 | 8593451.93 | 2125 | 1.25 | 1.18 | 0.03 |
| 272923.93 | 8593452.02 | 1030 | 0.5 | 0.56 | 0.02 |
| 272940.23 | 8593452.15 | 638 | 0.4 | 0.35 | 0.01 |
| 272958.33 | 8593452.29 | 661 | 0.4 | 0.36 | 0.01 |
| 272972.83 | 8593450.57 | 513 | 0.3 | 0.28 | 0.01 |
| 272980.07 | 8593450.63 | 563 | 0.35 | 0.30 | 0.01 |
| 272999.97 | 8593452.63 | 709 | 0.4 | 0.39 | 0.01 |
| 273009.04 | 8593450.86 | 854 | 0.5 | 0.47 | 0.02 |
| 273025.33 | 8593450.99 | 470 | 0.3 | 0.25 | 0.01 |
| 273039.82 | 8593451.11 | 339 | 0.2 | 0.18 | 0.01 |
| 273052.48 | 8593453.05 | 300 | 0.2 | 0.16 | 0.01 |
| 273063.35 | 8593451.29 | 277 | 0.15 | 0.14 | 0.01 |
| 273079.65 | 8593451.43 | 223 | 0.15 | 0.11 | 0.01 |
| 273088.7 | 8593451.5 | 241 | 0.15 | 0.12 | 0.01 |
| 273079.91 | 8593418.23 | 229 | 1 | 0.12 | 0.01 |
| 273072.66 | 8593420.02 | 209 | 0.1 | 0.11 | 0.01 |
| 273059.98 | 8593419.91 | 213 | 0.1 | 0.11 | 0.01 |
| 273047.31 | 8593419.81 | 212 | 0.1 | 0.11 | 0.01 |
| 273034.64 | 8593419.71 | 233 | 0.15 | 0.12 | 0.01 |
| 273023.79 | 8593417.78 | 245 | 0.15 | 0.13 | 0.01 |
| 273011.1 | 8593419.52 | 271 | 0.15 | 0.14 | 0.01 |
| 272998.44 | 8593417.58 | 284 | 0.15 | 0.15 | 0.01 |
| 272982.13 | 8593419.29 | 377 | 0.2 | 0.20 | 0.01 |
| 272973.08 | 8593419.22 | 388 | 0.2 | 0.21 | 0.01 |
| 272953.18 | 8593417.21 | 496 | 0.3 | 0.27 | 0.01 |
| 272936.87 | 8593418.92 | 575 | 0.35 | 0.31 | 0.01 |
| 272929.63 | 8593418.87 | 728 | 0.4 | 0.40 | 0.02 |
| 272916.94 | 8593420.61 | 908 | 0.5 | 0.50 | 0.02 |
| 272906.09 | 8593418.68 | 1224 | 0.6 | 0.67 | 0.02 |
| 272895.21 | 8593420.43 | 895 | 0.5 | 0.49 | 0.02 |
| 272880.75 | 8593418.47 | 1443 | 0.7 | 0.80 | 0.02 |
| 272864.45 | 8593418.34 | 12714 | 7.5 | 7.11 | 0.06 |
| 272857.21 | 8593418.28 | 21443 | 12.5 | 12.00 | 0.08 |
| 272842.73 | 8593418.17 | 870 | 0.5 | 0.48 | 0.02 |
| 272826.43 | 8593418.04 | 1224 | 0.7 | 0.67 | 0.02 |
| 272810.12 | 8593419.75 | 1211 | 0.7 | 0.67 | 0.02 |
| 272792.02 | 8593419.6 | 926 | 0.5 | 0.51 | 0.02 |
| 272782.96 | 8593419.53 | 848 | 0.5 | 0.46 | 0.02 |
| 272770.29 | 8593419.43 | 876 | 0.5 | 0.48 | 0.02 |
| 272754 | 8593419.3 | 774 | 0.4 | 0.42 | 0.02 |
| 272743.13 | 8593419.21 | 868 | 0.5 | 0.47 | 0.02 |
| 272734.08 | 8593419.14 | 822 | 0.4 | 0.45 | 0.02 |
| 272725.03 | 8593419.07 | 881 | 0.5 | 0.48 | 0.02 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |  |
| 272708.73 | 8593418.93 | 1000 | 0.6 | 0.55 | 0.02 |
| 272699.68 | 8593418.86 | 839 | 0.5 | 0.46 | 0.02 |
| 272692.44 | 8593418.8 | 834 | 0.4 | 0.46 | 0.02 |
| 272679.77 | 8593418.7 | 806 | 0.4 | 0.44 | 0.02 |
| 272667.09 | 8593418.6 | 807 | 0.5 | 0.44 | 0.02 |
| 272654.42 | 8593418.5 | 725 | 0.4 | 0.39 | 0.02 |
| 272643.56 | 8593418.41 | 690 | 0.4 | 0.37 | 0.01 |
| 272632.68 | 8593420.17 | 646 | 0.4 | 0.35 | 0.01 |
| 272621.83 | 8593418.24 | 631 | 0.4 | 0.34 | 0.01 |
| 272629.32 | 8593386.94 | 683 | 0.4 | 0.37 | 0.01 |
| 272642 | 8593387.04 | 749 | 0.5 | 0.41 | 0.02 |
| 272647.44 | 8593385.24 | 900 | 0.5 | 0.49 | 0.02 |
| 272660.12 | 8593385.35 | 847 | 0.5 | 0.46 | 0.02 |
| 272669.17 | 8593385.42 | 966 | 0.5 | 0.53 | 0.02 |
| 272683.65 | 8593385.54 | 1627 | 0.9 | 0.90 | 0.02 |
| 272696.33 | 8593385.64 | 1130 | 0.6 | 0.62 | 0.02 |
| 272707.19 | 8593385.72 | 965 | 0.5 | 0.53 | 0.02 |
| 272721.67 | 8593385.84 | 756 | 0.45 | 0.41 | 0.02 |
| 272732.54 | 8593385.93 | 695 | 0.4 | 0.38 | 0.01 |
| 272745.21 | 8593386.03 | 784 | 0.5 | 0.43 | 0.02 |
| 272761.51 | 8593386.16 | 956 | 0.5 | 0.52 | 0.02 |
| 272775.99 | 8593386.28 | 1048 | 0.6 | 0.57 | 0.02 |
| 272788.66 | 8593386.38 | 1209 | 0.6 | 0.67 | 0.02 |
| 272801.34 | 8593386.48 | 1448 | 0.8 | 0.80 | 0.02 |
| 272810.39 | 8593386.55 | 1689 | 1 | 0.93 | 0.02 |
| 272819.44 | 8593386.63 | 1682 | 1 | 0.93 | 0.02 |
| 272830.3 | 8593386.72 | 1684 | 1 | 0.93 | 0.02 |
| 272844.79 | 8593386.83 | 2102 | 1.25 | 1.17 | 0.03 |
| 272861.1 | 8593385.12 | 10891 | 6 | 6.09 | 0.06 |
| 272875.58 | 8593385.23 | 2621 | 1.25 | 1.46 | 0.03 |
| 272884.63 | 8593385.31 | 1489 | 0.9 | 0.82 | 0.02 |
| 272895.5 | 8593385.39 | 871 | 0.5 | 0.48 | 0.02 |
| 272908.17 | 8593385.5 | 627 | 0.35 | 0.34 | 0.01 |
| 272924.46 | 8593385.63 | 430 | 0.25 | 0.23 | 0.01 |
| 272938.95 | 8593385.74 | 426 | 0.25 | 0.23 | 0.01 |
| 272953.43 | 8593385.86 | 304 | 0.2 | 0.16 | 0.01 |
| 272964.29 | 8593385.95 | 285 | 0.175 | 0.15 | 0.01 |
| 272975.16 | 8593386.04 | 234 | 0.125 | 0.12 | 0.01 |
| 272991.45 | 8593386.17 | 236 | 0.125 | 0.12 | 0.01 |
| 273000.5 | 8593386.24 | 212 | 0.125 | 0.11 | 0.01 |
| 273013.18 | 8593386.34 | 214 | 0.125 | 0.11 | 0.01 |
| 273025.85 | 8593386.44 | 220 | 0.125 | 0.11 | 0.01 |
| 273038.52 | 8593386.54 | 195 | 0.1 | 0.10 | 0.01 |
| 273053.02 | 8593384.82 | 208 | 0.125 | 0.10 | 0.01 |
| 273063.87 | 8593386.75 | 230 | 0.15 | 0.12 | 0.01 |
| 273080.17 | 8593386.88 | 216 | 0.125 | 0.11 | 0.01 |
| 273094.65 | 8593387 | 216 | 0.15 | 0.11 | 0.01 |
| 273080.34 | 8593364.75 | 199 | 0.125 | 0.10 | 0.01 |
| 273065.86 | 8593364.63 | 190 | 0.125 | 0.09 | 0.01 |
| 273056.81 | 8593364.56 | 198 | 0.125 | 0.10 | 0.01 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |  |
| 273045.94 | 8593364.47 | 198 | 0.125 | 0.10 | 0.01 |
| 273038.7 | 8593364.41 | 218 | 0.125 | 0.11 | 0.01 |
| 273027.84 | 8593364.33 | 212 | 0.125 | 0.11 | 0.01 |
| 273004.3 | 8593364.14 | 223 | 0.125 | 0.11 | 0.01 |
| 272986.2 | 8593363.99 | 212 | 0.125 | 0.11 | 0.01 |
| 272966.28 | 8593363.83 | 220 | 0.125 | 0.11 | 0.01 |
| 272946.37 | 8593363.67 | 274 | 0.15 | 0.14 | 0.01 |
| 272937.32 | 8593363.6 | 291 | 0.15 | 0.15 | 0.01 |
| 272926.45 | 8593363.51 | 343 | 0.2 | 0.18 | 0.01 |
| 272911.97 | 8593363.4 | 389 | 0.25 | 0.21 | 0.01 |
| 272893.86 | 8593363.25 | 557 | 0.35 | 0.30 | 0.01 |
| 272879.38 | 8593363.13 | 849 | 0.5 | 0.46 | 0.02 |
| 272868.5 | 8593364.89 | 850 | 0.5 | 0.46 | 0.02 |
| 272854.03 | 8593362.93 | 658 | 0.4 | 0.36 | 0.01 |
| 272839.55 | 8593362.81 | 579 | 0.35 | 0.31 | 0.01 |
| 272828.67 | 8593364.57 | 533 | 0.3 | 0.29 | 0.01 |
| 272817.81 | 8593364.48 | 505 | 0.3 | 0.27 | 0.01 |
| 272806.95 | 8593364.4 | 447 | 0.3 | 0.24 | 0.01 |
| 272790.65 | 8593364.26 | 413 | 0.25 | 0.22 | 0.01 |
| 272779.79 | 8593364.18 | 458 | 0.25 | 0.24 | 0.01 |
| 272772.55 | 8593364.12 | 518 | 0.3 | 0.28 | 0.01 |
| 272761.68 | 8593364.03 | 416 | 0.25 | 0.22 | 0.01 |
| 272743.58 | 8593363.89 | 428 | 0.25 | 0.23 | 0.01 |
| 272727.28 | 8593363.75 | 423 | 0.25 | 0.22 | 0.01 |
| 272718.23 | 8593363.68 | 675 | 0.4 | 0.37 | 0.01 |
| 272709.18 | 8593363.61 | 550 | 0.3 | 0.30 | 0.01 |
| 272696.49 | 8593365.35 | 476 | 0.3 | 0.25 | 0.01 |
| 272685.64 | 8593363.42 | 473 | 0.25 | 0.25 | 0.01 |
| 272672.97 | 8593363.32 | 427 | 0.25 | 0.23 | 0.01 |
| 272662.09 | 8593365.07 | 543 | 0.3 | 0.29 | 0.01 |
| 272645.81 | 8593363.1 | 445 | 0.25 | 0.24 | 0.01 |
| 272627.69 | 8593364.8 | 468 | 0.25 | 0.25 | 0.01 |
| 272629.9 | 8593540.02 | 286 | 0.13 | 0.16 | 0.01 |
| 272646.2 | 8593540.16 | 290 | 0.16 | 0.16 | 0.01 |
| 272655.25 | 8593540.23 | 329 | 0.18 | 0.19 | 0.01 |
| 272666.11 | 8593540.32 | 355 | 0.20 | 0.20 | 0.01 |
| 272673.37 | 8593538.53 | 343 | 0.18 | 0.20 | 0.01 |
| 272684.22 | 8593540.46 | 366 | 0.22 | 0.21 | 0.01 |
| 272700.53 | 8593538.75 | 340 | 0.20 | 0.19 | 0.01 |
| 272709.57 | 8593540.67 | 345 | 0.17 | 0.20 | 0.01 |
| 272725.87 | 8593538.95 | 344 | 0.20 | 0.20 | 0.01 |
| 272734.94 | 8593537.18 | 377 | 0.18 | 0.22 | 0.01 |
| 272745.81 | 8593537.27 | 433 | 0.26 | 0.25 | 0.01 |
| 272760.27 | 8593539.23 | 469 | 0.28 | 0.27 | 0.01 |
| 272769.31 | 8593541.15 | 567 | 0.29 | 0.33 | 0.01 |
| 272781.97 | 8593543.09 | 540 | 0.28 | 0.32 | 0.01 |
| 272798.28 | 8593541.38 | 546 | 0.29 | 0.32 | 0.01 |
| 272812.78 | 8593539.65 | 450 | 0.25 | 0.26 | 0.01 |
| 272830.88 | 8593539.8 | 435 | 0.28 | 0.25 | 0.01 |
| 272845.38 | 8593538.07 | 466 | 0.28 | 0.27 | 0.01 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |  |
| 272863.47 | 8593540.06 | 664 | 0.33 | 0.39 | 0.01 |
| 272877.96 | 8593540.18 | 502 | 0.29 | 0.29 | 0.01 |
| 272888.82 | 8593540.26 | 541 | 0.30 | 0.32 | 0.01 |
| 272897.89 | 8593538.49 | 520 | 0.32 | 0.30 | 0.01 |
| 272906.97 | 8593534.88 | 477 | 0.28 | 0.28 | 0.01 |
| 272925.03 | 8593540.55 | 557 | 0.30 | 0.33 | 0.01 |
| 272941.31 | 8593542.53 | 460 | 0.25 | 0.27 | 0.01 |
| 272955.81 | 8593540.8 | 468 | 0.25 | 0.27 | 0.01 |
| 272963.05 | 8593540.86 | 512 | 0.28 | 0.30 | 0.01 |
| 272975.72 | 8593540.96 | 491 | 0.28 | 0.29 | 0.01 |
| 272988.38 | 8593542.91 | 504 | 0.30 | 0.29 | 0.01 |
| 273002.88 | 8593541.18 | 503 | 0.27 | 0.29 | 0.01 |
| 273017.37 | 8593541.3 | 422 | 0.22 | 0.24 | 0.01 |
| 273031.86 | 8593539.57 | 409 | 0.23 | 0.24 | 0.01 |
| 273044.55 | 8593537.83 | 351 | 0.15 | 0.20 | 0.01 |
| 273055.4 | 8593539.76 | 370 | 0.20 | 0.21 | 0.01 |
| 273064.47 | 8593537.99 | 483 | 0.27 | 0.28 | 0.01 |
| 273073.51 | 8593539.9 | 396 | 0.22 | 0.23 | 0.01 |
| 273089.8 | 8593540.03 | 435 | 0.25 | 0.25 | 0.01 |
| 273079.2 | 8593506.75 | 371 | 0.18 | 0.21 | 0.01 |
| 273062.91 | 8593506.62 | 416 | 0.24 | 0.24 | 0.01 |
| 273044.8 | 8593506.47 | 443 | 0.28 | 0.26 | 0.01 |
| 273037.56 | 8593506.42 | 448 | 0.23 | 0.26 | 0.01 |
| 273026.7 | 8593506.33 | 488 | 0.25 | 0.28 | 0.01 |
| 273012.2 | 8593508.06 | 443 | 0.24 | 0.26 | 0.01 |
| 273003.15 | 8593507.98 | 424 | 0.28 | 0.25 | 0.01 |
| 272994.1 | 8593507.91 | 421 | 0.25 | 0.24 | 0.01 |
| 272985.06 | 8593505.99 | 431 | 0.23 | 0.25 | 0.01 |
| 272975.99 | 8593507.77 | 419 | 0.22 | 0.24 | 0.01 |
| 272961.51 | 8593507.65 | 552 | 0.32 | 0.32 | 0.01 |
| 272945.21 | 8593507.52 | 627 | 0.35 | 0.37 | 0.01 |
| 272934.35 | 8593507.43 | 848 | 0.45 | 0.50 | 0.02 |
| 272921.68 | 8593507.33 | 715 | 0.37 | 0.42 | 0.01 |
| 272907.19 | 8593507.21 | 889 | 0.52 | 0.53 | 0.02 |
| 272890.9 | 8593507.08 | 842 | 0.47 | 0.50 | 0.02 |
| 272885.47 | 8593507.04 | 888 | 0.51 | 0.53 | 0.02 |
| 272876.41 | 8593506.97 | 957 | 0.52 | 0.57 | 0.02 |
| 272865.55 | 8593506.88 | 818 | 0.49 | 0.49 | 0.02 |
| 272856.5 | 8593506.81 | 1350 | 0.85 | 0.81 | 0.02 |
| 272842.01 | 8593506.69 | 787 | 0.40 | 0.47 | 0.02 |
| 272831.15 | 8593506.6 | 665 | 0.35 | 0.39 | 0.01 |
| 272820.29 | 8593506.51 | 440 | 0.28 | 0.26 | 0.01 |
| 272811.24 | 8593506.44 | 443 | 0.29 | 0.26 | 0.01 |
| 272802.18 | 8593506.37 | 439 | 0.23 | 0.25 | 0.01 |
| 272787.68 | 8593508.1 | 632 | 0.32 | 0.37 | 0.01 |
| 272767.78 | 8593506.09 | 444 | 0.24 | 0.26 | 0.01 |
| 272758.72 | 8593507.86 | 454 | 0.28 | 0.26 | 0.01 |
| 272747.87 | 8593505.93 | 458 | 0.22 | 0.27 | 0.01 |
| 272731.57 | 8593505.8 | 295 | 0.14 | 0.17 | 0.01 |
| 272713.45 | 8593507.5 | 235 | 0.14 | 0.13 | 0.01 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |  |
| 272691.73 | 8593507.32 | 288 | 0.16 | 0.16 | 0.01 |
| 272680.86 | 8593507.24 | 286 | 0.16 | 0.16 | 0.01 |
| 272668.19 | 8593507.14 | 321 | 0.18 | 0.18 | 0.01 |
| 272648.28 | 8593506.97 | 346 | 0.18 | 0.20 | 0.01 |
| 272631.98 | 8593506.84 | 317 | 0.16 | 0.18 | 0.01 |
| 272634.06 | 8593473.66 | 719 | 0.45 | 0.43 | 0.02 |
| 272653.97 | 8593473.82 | 716 | 0.38 | 0.42 | 0.01 |
| 272668.46 | 8593473.94 | 580 | 0.30 | 0.34 | 0.01 |
| 272675.7 | 8593474 | 505 | 0.28 | 0.30 | 0.01 |
| 272684.75 | 8593474.07 | 433 | 0.22 | 0.25 | 0.01 |
| 272699.24 | 8593474.19 | 406 | 0.20 | 0.23 | 0.01 |
| 272708.29 | 8593474.26 | 443 | 0.27 | 0.26 | 0.01 |
| 272720.96 | 8593474.36 | 361 | 0.18 | 0.21 | 0.01 |
| 272733.64 | 8593474.46 | 297 | 0.17 | 0.17 | 0.01 |
| 272744.5 | 8593474.55 | 275 | 0.15 | 0.15 | 0.01 |
| 272762.6 | 8593474.7 | 240 | 0.14 | 0.13 | 0.01 |
| 272775.28 | 8593474.8 | 312 | 0.13 | 0.18 | 0.01 |
| 272789.76 | 8593474.92 | 296 | 0.15 | 0.17 | 0.01 |
| 272802.45 | 8593473.17 | 539 | 0.22 | 0.32 | 0.01 |
| 272813.31 | 8593473.26 | 680 | 0.41 | 0.40 | 0.01 |
| 272829.61 | 8593473.39 | 476 | 0.24 | 0.28 | 0.01 |
| 272840.47 | 8593473.48 | 719 | 0.43 | 0.43 | 0.02 |
| 272851.33 | 8593473.57 | 839 | 0.42 | 0.50 | 0.02 |
| 272864.01 | 8593473.67 | 1113 | 0.65 | 0.67 | 0.02 |
| 272880.3 | 8593473.8 | 1289 | 0.67 | 0.77 | 0.02 |
| 272894.78 | 8593473.92 | 1603 | 0.95 | 0.97 | 0.02 |
| 272907.46 | 8593474.02 | 2225 | 1.20 | 1.34 | 0.03 |
| 272916.51 | 8593474.09 | 2648 | 1.60 | 1.60 | 0.03 |
| 272927.37 | 8593474.18 | 2800 | 1.50 | 1.70 | 0.03 |
| 272947.29 | 8593474.34 | 1270 | 0.68 | 0.76 | 0.02 |
| 272958.15 | 8593474.42 | 687 | 0.38 | 0.41 | 0.01 |
| 272969.02 | 8593474.51 | 593 | 0.38 | 0.35 | 0.01 |
| 272985.31 | 8593474.64 | 600 | 0.34 | 0.35 | 0.01 |
| 272996.19 | 8593472.89 | 498 | 0.30 | 0.29 | 0.01 |
| 273005.24 | 8593472.96 | 611 | 0.30 | 0.36 | 0.01 |
| 273014.29 | 8593473.03 | 898 | 0.54 | 0.53 | 0.02 |
| 273026.95 | 8593474.98 | 786 | 0.44 | 0.47 | 0.02 |
| 273037.83 | 8593473.22 | 487 | 0.27 | 0.28 | 0.01 |
| 273052.3 | 8593475.18 | 391 | 0.19 | 0.23 | 0.01 |
| 273063.18 | 8593473.42 | 397 | 0.18 | 0.23 | 0.01 |
| 273075.85 | 8593473.53 | 334 | 0.16 | 0.19 | 0.01 |
| 273084.9 | 8593473.6 | 332 | 0.18 | 0.19 | 0.01 |
| 273077.91 | 8593442.19 | 194 | 0.09 | 0.11 | 0.01 |
| 273065.25 | 8593440.24 | 191 | 0.09 | 0.10 | 0.01 |
| 273047.13 | 8593441.94 | 269 | 0.13 | 0.15 | 0.01 |
| 273032.65 | 8593441.83 | 289 | 0.15 | 0.16 | 0.01 |
| 273021.79 | 8593441.74 | 378 | 0.19 | 0.22 | 0.01 |
| 273009.11 | 8593441.64 | 449 | 0.30 | 0.26 | 0.01 |
| 272992.82 | 8593441.51 | 593 | 0.29 | 0.35 | 0.01 |
| 272978.33 | 8593441.39 | 843 | 0.42 | 0.50 | 0.02 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |  |
| 272965.66 | 8593441.29 | 543 | 0.30 | 0.32 | 0.01 |
| 272952.99 | 8593441.19 | 601 | 0.28 | 0.35 | 0.01 |
| 272943.93 | 8593441.11 | 723 | 0.36 | 0.43 | 0.02 |
| 272931.26 | 8593441.01 | 667 | 0.38 | 0.39 | 0.01 |
| 272922.21 | 8593440.94 | 784 | 0.38 | 0.47 | 0.02 |
| 272907.73 | 8593440.82 | 1391 | 0.62 | 0.84 | 0.02 |
| 272896.86 | 8593440.73 | 1578 | 0.85 | 0.95 | 0.02 |
| 272887.81 | 8593440.66 | 1452 | 0.72 | 0.87 | 0.02 |
| 272878.76 | 8593440.59 | 1700 | 0.95 | 1.02 | 0.02 |
| 272864.27 | 8593440.47 | 17808 | 10.20 | 10.85 | 0.07 |
| 272849.79 | 8593440.36 | 1863 | 1.00 | 1.12 | 0.02 |
| 272831.68 | 8593440.21 | 848 | 0.42 | 0.50 | 0.02 |
| 272819.01 | 8593440.11 | 664 | 0.32 | 0.39 | 0.01 |
| 272804.51 | 8593441.84 | 1261 | 0.65 | 0.76 | 0.02 |
| 272797.27 | 8593441.78 | 1553 | 0.85 | 0.93 | 0.02 |
| 272786.41 | 8593441.69 | 1304 | 0.62 | 0.78 | 0.02 |
| 272771.92 | 8593441.57 | 918 | 0.53 | 0.55 | 0.02 |
| 272757.44 | 8593441.46 | 910 | 0.50 | 0.54 | 0.02 |
| 272744.77 | 8593441.36 | 839 | 0.44 | 0.50 | 0.02 |
| 272726.66 | 8593441.21 | 962 | 0.45 | 0.57 | 0.02 |
| 272717.61 | 8593441.14 | 1284 | 0.72 | 0.77 | 0.02 |
| 272710.37 | 8593441.08 | 1007 | 0.60 | 0.60 | 0.02 |
| 272701.31 | 8593441.01 | 998 | 0.45 | 0.60 | 0.02 |
| 272688.64 | 8593440.9 | 967 | 0.50 | 0.58 | 0.02 |
| 272677.78 | 8593440.82 | 938 | 0.55 | 0.56 | 0.02 |
| 272663.29 | 8593440.7 | 884 | 0.46 | 0.53 | 0.02 |
| 272656.05 | 8593440.64 | 891 | 0.42 | 0.53 | 0.02 |
| 272645.19 | 8593440.56 | 764 | 0.46 | 0.45 | 0.02 |
| 272628.89 | 8593440.42 | 911 | 0.50 | 0.54 | 0.02 |
| 272630.96 | 8593409.09 | 726 | 0.45 | 0.43 | 0.02 |
| 272641.82 | 8593409.17 | 724 | 0.43 | 0.43 | 0.02 |
| 272656.3 | 8593409.29 | 479 | 0.30 | 0.28 | 0.01 |
| 272667.18 | 8593407.53 | 689 | 0.40 | 0.41 | 0.01 |
| 272679.86 | 8593407.64 | 799 | 0.45 | 0.47 | 0.02 |
| 272694.34 | 8593407.75 | 962 | 0.54 | 0.57 | 0.02 |
| 272707.01 | 8593407.86 | 1157 | 0.62 | 0.69 | 0.02 |
| 272714.25 | 8593407.91 | 1097 | 0.63 | 0.66 | 0.02 |
| 272723.31 | 8593407.99 | 963 | 0.57 | 0.57 | 0.02 |
| 272739.6 | 8593408.12 | 834 | 0.45 | 0.50 | 0.02 |
| 272748.65 | 8593408.19 | 821 | 0.49 | 0.49 | 0.02 |
| 272763.14 | 8593408.31 | 807 | 0.50 | 0.48 | 0.02 |
| 272775.81 | 8593408.41 | 878 | 0.47 | 0.52 | 0.02 |
| 272790.3 | 8593408.52 | 838 | 0.48 | 0.50 | 0.02 |
| 272799.35 | 8593408.6 | 939 | 0.50 | 0.56 | 0.02 |
| 272806.59 | 8593408.66 | 1051 | 0.58 | 0.63 | 0.02 |
| 272817.47 | 8593406.9 | 1325 | 0.83 | 0.80 | 0.02 |
| 272833.75 | 8593408.87 | 1735 | 0.94 | 1.05 | 0.02 |
| 272846.42 | 8593408.98 | 1293 | 0.63 | 0.78 | 0.02 |
| 272862.73 | 8593407.26 | 2415 | 1.40 | 1.46 | 0.03 |
| 272875.4 | 8593407.36 | 1525 | 0.88 | 0.92 | 0.02 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |  |
| 272882.65 | 8593407.42 | 1161 | 0.73 | 0.70 | 0.02 |
| 272898.94 | 8593407.55 | 1040 | 0.58 | 0.62 | 0.02 |
| 272917.04 | 8593407.7 | 1021 | 0.50 | 0.61 | 0.02 |
| 272931.53 | 8593407.82 | 731 | 0.38 | 0.43 | 0.02 |
| 272947.82 | 8593407.95 | 489 | 0.30 | 0.29 | 0.01 |
| 272960.5 | 8593408.05 | 334 | 0.18 | 0.19 | 0.01 |
| 272974.98 | 8593408.17 | 311 | 0.16 | 0.18 | 0.01 |
| 272993.08 | 8593408.31 | 258 | 0.14 | 0.14 | 0.01 |
| 273000.33 | 8593408.37 | 259 | 0.14 | 0.14 | 0.01 |
| 273011.19 | 8593408.46 | 253 | 0.13 | 0.14 | 0.01 |
| 273023.86 | 8593408.56 | 244 | 0.13 | 0.14 | 0.01 |
| 273032.92 | 8593408.63 | 210 | 0.12 | 0.12 | 0.01 |
| 273049.21 | 8593408.76 | 189 | 0.15 | 0.10 | 0.01 |
| 273063.69 | 8593408.88 | 214 | 0.16 | 0.12 | 0.01 |
| 273070.94 | 8593408.94 | 206 | 0.12 | 0.11 | 0.01 |
| 273083.61 | 8593409.04 | 211 | 0.13 | 0.12 | 0.01 |
| 273082.08 | 8593373.98 | 217 | 0.10 | 0.12 | 0.01 |
| 273071.22 | 8593373.9 | 214 | 0.13 | 0.12 | 0.01 |
| 273054.91 | 8593375.61 | 234 | 0.14 | 0.13 | 0.01 |
| 273044.05 | 8593375.52 | 205 | 0.10 | 0.11 | 0.01 |
| 273027.75 | 8593375.39 | 184 | 0.11 | 0.10 | 0.01 |
| 273018.7 | 8593375.32 | 200 | 0.11 | 0.11 | 0.01 |
| 273009.65 | 8593375.25 | 222 | 0.12 | 0.12 | 0.01 |
| 272995.16 | 8593375.13 | 210 | 0.10 | 0.12 | 0.01 |
| 272980.68 | 8593375.01 | 204 | 0.12 | 0.11 | 0.01 |
| 272960.76 | 8593374.85 | 244 | 0.13 | 0.14 | 0.01 |
| 272948.09 | 8593374.75 | 266 | 0.15 | 0.15 | 0.01 |
| 272933.61 | 8593374.64 | 300 | 0.18 | 0.17 | 0.01 |
| 272924.55 | 8593374.56 | 302 | 0.17 | 0.17 | 0.01 |
| 272910.07 | 8593374.45 | 376 | 0.21 | 0.22 | 0.01 |
| 272899.21 | 8593374.36 | 655 | 0.36 | 0.39 | 0.01 |
| 272890.15 | 8593374.29 | 941 | 0.49 | 0.56 | 0.02 |
| 272879.29 | 8593374.2 | 1520 | 0.75 | 0.91 | 0.02 |
| 272866.6 | 8593375.94 | 2809 | 1.80 | 1.70 | 0.03 |
| 272850.32 | 8593373.97 | 3510 | 2.00 | 2.13 | 0.03 |
| 272835.82 | 8593375.69 | 1487 | 0.82 | 0.89 | 0.02 |
| 272824.96 | 8593375.61 | 1143 | 0.62 | 0.68 | 0.02 |
| 272808.67 | 8593375.48 | 834 | 0.43 | 0.50 | 0.02 |
| 272797.8 | 8593375.39 | 700 | 0.38 | 0.41 | 0.01 |
| 272783.32 | 8593375.27 | 712 | 0.37 | 0.42 | 0.01 |
| 272772.46 | 8593375.18 | 937 | 0.50 | 0.56 | 0.02 |
| 272759.78 | 8593375.08 | 644 | 0.37 | 0.38 | 0.01 |
| 272754.35 | 8593375.04 | 585 | 0.30 | 0.34 | 0.01 |
| 272738.06 | 8593374.91 | 617 | 0.31 | 0.36 | 0.01 |
| 272725.38 | 8593374.81 | 652 | 0.35 | 0.38 | 0.01 |
| 272709.09 | 8593374.67 | 707 | 0.37 | 0.42 | 0.01 |
| 272692.8 | 8593374.54 | 754 | 0.46 | 0.45 | 0.02 |
| 272680.12 | 8593374.44 | 864 | 0.50 | 0.51 | 0.02 |
| 272663.83 | 8593374.31 | 748 | 0.40 | 0.44 | 0.02 |
| 272656.57 | 8593376.1 | 755 | 0.38 | 0.45 | 0.02 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |  |
| 272647.53 | 8593374.18 | 627 | 0.35 | 0.37 | 0.01 |
| 272631.22 | 8593375.89 | 517 | 0.30 | 0.30 | 0.01 |
| 272631.8 | 8593528.97 | 329 | 0.12 | 0.19 | 0.01 |
| 272642.67 | 8593529.06 | 281 | 0.07 | 0.16 | 0.01 |
| 272649.89 | 8593530.96 | 280 | 0.1 | 0.16 | 0.01 |
| 272658.93 | 8593532.88 | 304 | 0.1 | 0.17 | 0.01 |
| 272667.98 | 8593532.95 | 292 | 0.1 | 0.16 | 0.01 |
| 272682.5 | 8593529.38 | 337 | 0.1 | 0.19 | 0.01 |
| 272695.16 | 8593531.33 | 353 | 0.11 | 0.20 | 0.01 |
| 272706 | 8593533.26 | 310 | 0.12 | 0.18 | 0.01 |
| 272716.87 | 8593533.35 | 306 | 0.09 | 0.17 | 0.01 |
| 272729.56 | 8593531.6 | 245 | 0.09 | 0.14 | 0.01 |
| 272738.64 | 8593527.99 | 290 | 0.1 | 0.16 | 0.01 |
| 272747.69 | 8593528.06 | 365 | 0.11 | 0.21 | 0.01 |
| 272758.52 | 8593531.84 | 420 | 0.15 | 0.24 | 0.01 |
| 272767.56 | 8593533.75 | 390 | 0.12 | 0.22 | 0.01 |
| 272782.06 | 8593532.03 | 592 | 0.22 | 0.35 | 0.01 |
| 272794.75 | 8593530.28 | 622 | 0.23 | 0.37 | 0.01 |
| 272809.25 | 8593528.56 | 574 | 0.22 | 0.34 | 0.01 |
| 272818.28 | 8593530.47 | 568 | 0.21 | 0.33 | 0.01 |
| 272832.75 | 8593532.43 | 538 | 0.2 | 0.31 | 0.01 |
| 272847.25 | 8593530.71 | 489 | 0.19 | 0.28 | 0.01 |
| 272854.51 | 8593528.92 | 465 | 0.16 | 0.27 | 0.01 |
| 272865.39 | 8593527.16 | 903 | 0.42 | 0.54 | 0.02 |
| 272876.24 | 8593529.1 | 714 | 0.3 | 0.42 | 0.01 |
| 272885.3 | 8593527.32 | 704 | 0.3 | 0.42 | 0.01 |
| 272892.51 | 8593531.07 | 565 | 0.23 | 0.33 | 0.01 |
| 272899.76 | 8593531.13 | 562 | 0.2 | 0.33 | 0.01 |
| 272914.23 | 8593533.09 | 561 | 0.28 | 0.33 | 0.01 |
| 272926.91 | 8593531.35 | 540 | 0.23 | 0.32 | 0.01 |
| 272939.56 | 8593535.14 | 610 | 0.26 | 0.36 | 0.01 |
| 272948.6 | 8593537.05 | 577 | 0.3 | 0.34 | 0.01 |
| 272959.49 | 8593533.45 | 553 | 0.22 | 0.32 | 0.01 |
| 272972.18 | 8593531.71 | 601 | 0.25 | 0.35 | 0.01 |
| 272981.23 | 8593531.78 | 713 | 0.3 | 0.42 | 0.01 |
| 272992.09 | 8593531.87 | 575 | 0.25 | 0.34 | 0.01 |
| 273004.75 | 8593533.82 | 503 | 0.23 | 0.29 | 0.01 |
| 273013.79 | 8593535.73 | 431 | 0.17 | 0.25 | 0.01 |
| 273026.49 | 8593532.15 | 451 | 0.16 | 0.26 | 0.01 |
| 273037.37 | 8593530.39 | 358 | 0.13 | 0.21 | 0.01 |
| 273053.66 | 8593530.52 | 326 | 0.17 | 0.19 | 0.01 |
| 273062.72 | 8593530.59 | 350 | 0.1 | 0.20 | 0.01 |
| 273071.75 | 8593532.51 | 389 | 0.1 | 0.22 | 0.01 |
| 273082.66 | 8593527.07 | 533 | 0.15 | 0.31 | 0.01 |
| 273077.48 | 8593495.67 | 379 | 0.12 | 0.22 | 0.01 |
| 273068.41 | 8593497.44 | 443 | 0.15 | 0.26 | 0.01 |
| 273055.77 | 8593493.65 | 372 | 0.15 | 0.21 | 0.01 |
| 273041.26 | 8593497.22 | 543 | 0.22 | 0.32 | 0.01 |
| 273030.36 | 8593500.83 | 462 | 0.21 | 0.27 | 0.01 |
| 273021.31 | 8593500.75 | 408 | 0.17 | 0.24 | 0.01 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |  |
| 273012.24 | 8593502.52 | 418 | 0.16 | 0.24 | 0.01 |
| 272999.57 | 8593502.42 | 464 | 0.17 | 0.27 | 0.01 |
| 272983.28 | 8593502.29 | 428 | 0.18 | 0.25 | 0.01 |
| 272974.24 | 8593500.37 | 454 | 0.2 | 0.26 | 0.01 |
| 272965.19 | 8593500.3 | 545 | 0.22 | 0.32 | 0.01 |
| 272957.94 | 8593500.24 | 676 | 0.25 | 0.40 | 0.01 |
| 272950.72 | 8593498.34 | 868 | 0.35 | 0.52 | 0.02 |
| 272936.22 | 8593500.07 | 906 | 0.4 | 0.54 | 0.02 |
| 272927.15 | 8593501.84 | 933 | 0.42 | 0.56 | 0.02 |
| 272914.48 | 8593501.74 | 1047 | 0.49 | 0.62 | 0.02 |
| 272901.77 | 8593505.32 | 985 | 0.28 | 0.59 | 0.02 |
| 272889.09 | 8593507.07 | 869 | 0.45 | 0.52 | 0.02 |
| 272878.22 | 8593506.98 | 938 | 0.47 | 0.56 | 0.02 |
| 272865.61 | 8593499.5 | 933 | 0.38 | 0.56 | 0.02 |
| 272856.57 | 8593497.58 | 1000 | 0.37 | 0.60 | 0.02 |
| 272843.91 | 8593495.64 | 897 | 0.4 | 0.53 | 0.02 |
| 272834.85 | 8593497.41 | 586 | 0.28 | 0.34 | 0.01 |
| 272818.55 | 8593497.28 | 434 | 0.19 | 0.25 | 0.01 |
| 272807.72 | 8593493.5 | 633 | 0.23 | 0.37 | 0.01 |
| 272798.65 | 8593495.27 | 594 | 0.24 | 0.35 | 0.01 |
| 272785.98 | 8593495.17 | 560 | 0.26 | 0.33 | 0.01 |
| 272778.72 | 8593496.96 | 547 | 0.23 | 0.32 | 0.01 |
| 272767.86 | 8593496.87 | 436 | 0.14 | 0.25 | 0.01 |
| 272755.18 | 8593496.77 | 569 | 0.17 | 0.33 | 0.01 |
| 272753.34 | 8593500.44 | 394 | 0.18 | 0.23 | 0.01 |
| 272742.51 | 8593496.67 | 340 | 0.17 | 0.19 | 0.01 |
| 272733.46 | 8593496.59 | 437 | 0.15 | 0.25 | 0.01 |
| 272724.42 | 8593494.68 | 326 | 0.13 | 0.19 | 0.01 |
| 272711.72 | 8593498.26 | 283 | 0.13 | 0.16 | 0.01 |
| 272699.04 | 8593498.16 | 321 | 0.08 | 0.18 | 0.01 |
| 272680.95 | 8593496.17 | 295 | 0.07 | 0.17 | 0.01 |
| 272670.06 | 8593499.77 | 292 | 0.07 | 0.16 | 0.01 |
| 272659.2 | 8593499.69 | 351 | 0.1 | 0.20 | 0.01 |
| 272646.54 | 8593497.74 | 328 | 0.11 | 0.19 | 0.01 |
| 272635.69 | 8593495.81 | 306 | 0.1 | 0.17 | 0.01 |
| 272623.02 | 8593495.71 | 309 | 0.12 | 0.18 | 0.01 |
| 272630.53 | 8593462.57 | 894 | 0.12 | 0.53 | 0.02 |
| 272643.19 | 8593464.51 | 978 | 0.33 | 0.58 | 0.02 |
| 272654.05 | 8593464.6 | 1090 | 0.57 | 0.65 | 0.02 |
| 272668.5 | 8593468.41 | 691 | 0.41 | 0.41 | 0.01 |
| 272679.37 | 8593468.49 | 643 | 0.31 | 0.38 | 0.01 |
| 272693.89 | 8593463.08 | 772 | 0.38 | 0.46 | 0.02 |
| 272704.74 | 8593465.01 | 651 | 0.33 | 0.38 | 0.01 |
| 272715.61 | 8593465.1 | 609 | 0.29 | 0.36 | 0.01 |
| 272724.66 | 8593465.17 | 555 | 0.31 | 0.33 | 0.01 |
| 272735.54 | 8593463.41 | 589 | 0.12 | 0.35 | 0.01 |
| 272746.38 | 8593465.35 | 327 | 0.13 | 0.19 | 0.01 |
| 272764.49 | 8593465.49 | 247 | 0.09 | 0.14 | 0.01 |
| 272771.75 | 8593463.7 | 274 | 0.13 | 0.15 | 0.01 |
| 272786.21 | 8593465.67 | 805 | 0.29 | 0.48 | 0.02 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |  |
| 272795.27 | 8593465.74 | 744 | 0.42 | 0.44 | 0.02 |
| 272806.13 | 8593465.83 | 707 | 0.27 | 0.42 | 0.01 |
| 272818.79 | 8593467.77 | 709 | 0.3 | 0.42 | 0.01 |
| 272829.67 | 8593466.01 | 619 | 0.31 | 0.36 | 0.01 |
| 272840.54 | 8593464.26 | 710 | 0.26 | 0.42 | 0.01 |
| 272849.57 | 8593468.02 | 862 | 0.35 | 0.51 | 0.02 |
| 272860.44 | 8593466.26 | 1175 | 0.52 | 0.70 | 0.02 |
| 272873.15 | 8593462.68 | 2688 | 1.3 | 1.63 | 0.03 |
| 272884.01 | 8593462.76 | 6285 | 2.9 | 3.82 | 0.04 |
| 272898.48 | 8593464.72 | 11192 | 8.1 | 6.81 | 0.06 |
| 272905.72 | 8593464.78 | 5804 | 4 | 3.52 | 0.04 |
| 272920.19 | 8593466.74 | 2916 | 1.4 | 1.76 | 0.03 |
| 272931.08 | 8593463.14 | 1323 | 0.62 | 0.79 | 0.02 |
| 272943.76 | 8593463.24 | 1048 | 0.5 | 0.63 | 0.02 |
| 272951 | 8593463.3 | 621 | 0.28 | 0.37 | 0.01 |
| 272956.4 | 8593467.03 | 764 | 0.32 | 0.45 | 0.02 |
| 272972.71 | 8593465.32 | 858 | 0.38 | 0.51 | 0.02 |
| 272985.38 | 8593465.42 | 491 | 0.22 | 0.29 | 0.01 |
| 273001.69 | 8593463.71 | 588 | 0.22 | 0.35 | 0.01 |
| 273008.92 | 8593465.61 | 1083 | 0.5 | 0.65 | 0.02 |
| 273019.8 | 8593463.85 | 953 | 0.4 | 0.57 | 0.02 |
| 273030.68 | 8593462.1 | 442 | 0.28 | 0.26 | 0.01 |
| 273043.33 | 8593464.04 | 389 | 0.16 | 0.22 | 0.01 |
| 273061.39 | 8593469.72 | 356 | 0.17 | 0.20 | 0.01 |
| 273072.32 | 8593462.43 | 247 | 0.12 | 0.14 | 0.01 |
| 273081.35 | 8593464.35 | 278 | 0.07 | 0.16 | 0.01 |
| 273079.83 | 8593429.29 | 175 | 0.06 | 0.09 | 0.01 |
| 273063.53 | 8593429.16 | 221 | 0.08 | 0.12 | 0.01 |
| 273050.86 | 8593429.06 | 237 | 0.07 | 0.13 | 0.01 |
| 273039.97 | 8593432.66 | 245 | 0.09 | 0.14 | 0.01 |
| 273027.31 | 8593430.72 | 244 | 0.07 | 0.14 | 0.01 |
| 273016.44 | 8593430.63 | 279 | 0.1 | 0.16 | 0.01 |
| 273005.58 | 8593430.54 | 310 | 0.11 | 0.18 | 0.01 |
| 272992.91 | 8593430.44 | 520 | 0.22 | 0.30 | 0.01 |
| 272978.42 | 8593430.32 | 491 | 0.14 | 0.29 | 0.01 |
| 272963.94 | 8593430.21 | 473 | 0.2 | 0.28 | 0.01 |
| 272953.08 | 8593430.12 | 522 | 0.22 | 0.31 | 0.01 |
| 272936.78 | 8593429.99 | 572 | 0.22 | 0.34 | 0.01 |
| 272924.09 | 8593431.73 | 686 | 0.26 | 0.40 | 0.01 |
| 272915.06 | 8593429.82 | 696 | 0.28 | 0.41 | 0.01 |
| 272904.19 | 8593429.73 | 897 | 0.4 | 0.53 | 0.02 |
| 272893.33 | 8593429.64 | 1400 | 0.85 | 0.84 | 0.02 |
| 272882.47 | 8593429.55 | 1039 | 0.5 | 0.62 | 0.02 |
| 272867.97 | 8593431.28 | 5151 | 2.8 | 3.13 | 0.04 |
| 272853.48 | 8593431.16 | 9071 | 8 | 5.51 | 0.05 |
| 272844.43 | 8593431.09 | 962 | 0.4 | 0.57 | 0.02 |
| 272833.58 | 8593429.16 | 673 | 0.26 | 0.40 | 0.01 |
| 272817.29 | 8593429.03 | 496 | 0.19 | 0.29 | 0.01 |
| 272797.36 | 8593430.71 | 732 | 0.28 | 0.43 | 0.02 |
| 272782.89 | 8593428.75 | 942 | 0.33 | 0.56 | 0.02 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |  |
| 272777.44 | 8593430.55 | 957 | 0.32 | 0.57 | 0.02 |
| 272764.77 | 8593430.45 | 885 | 0.38 | 0.53 | 0.02 |
| 272750.29 | 8593430.33 | 782 | 0.32 | 0.46 | 0.02 |
| 272739.42 | 8593430.25 | 802 | 0.35 | 0.48 | 0.02 |
| 272724.94 | 8593430.13 | 1004 | 0.42 | 0.60 | 0.02 |
| 272714.09 | 8593428.2 | 942 | 0.35 | 0.56 | 0.02 |
| 272705.02 | 8593429.97 | 817 | 0.33 | 0.48 | 0.02 |
| 272692.35 | 8593429.87 | 842 | 0.35 | 0.50 | 0.02 |
| 272683.3 | 8593429.8 | 842 | 0.36 | 0.50 | 0.02 |
| 272672.42 | 8593431.55 | 801 | 0.32 | 0.48 | 0.02 |
| 272661.56 | 8593431.47 | 812 | 0.36 | 0.48 | 0.02 |
| 272654.32 | 8593431.41 | 733 | 0.28 | 0.43 | 0.02 |
| 272643.45 | 8593431.32 | 618 | 0.21 | 0.36 | 0.01 |
| 272630.79 | 8593429.37 | 611 | 0.28 | 0.36 | 0.01 |
| 272621.74 | 8593429.3 | 622 | 0.23 | 0.37 | 0.01 |
| 272629.25 | 8593396.16 | 748 | 0.31 | 0.44 | 0.02 |
| 272647.36 | 8593396.31 | 731 | 0.3 | 0.43 | 0.02 |
| 272660.01 | 8593398.26 | 436 | 0.17 | 0.25 | 0.01 |
| 272665.46 | 8593396.45 | 734 | 0.42 | 0.43 | 0.02 |
| 272679.93 | 8593398.42 | 841 | 0.4 | 0.50 | 0.02 |
| 272689 | 8593396.64 | 1014 | 0.5 | 0.60 | 0.02 |
| 272703.48 | 8593396.76 | 1014 | 0.52 | 0.60 | 0.02 |
| 272712.53 | 8593396.83 | 954 | 0.51 | 0.57 | 0.02 |
| 272723.4 | 8593396.92 | 798 | 0.38 | 0.47 | 0.02 |
| 272734.26 | 8593397.01 | 731 | 0.39 | 0.43 | 0.02 |
| 272743.31 | 8593397.08 | 768 | 0.33 | 0.45 | 0.02 |
| 272754.17 | 8593397.17 | 842 | 0.33 | 0.50 | 0.02 |
| 272766.85 | 8593397.27 | 884 | 0.39 | 0.53 | 0.02 |
| 272783.14 | 8593397.4 | 1012 | 0.42 | 0.60 | 0.02 |
| 272790.38 | 8593397.46 | 920 | 0.5 | 0.55 | 0.02 |
| 272808.49 | 8593397.61 | 1118 | 0.52 | 0.67 | 0.02 |
| 272819.35 | 8593397.69 | 1335 | 0.58 | 0.80 | 0.02 |
| 272830.22 | 8593397.78 | 1427 | 0.75 | 0.86 | 0.02 |
| 272839.27 | 8593397.85 | 1409 | 0.8 | 0.85 | 0.02 |
| 272848.32 | 8593397.93 | 1603 | 0.75 | 0.96 | 0.02 |
| 272861.01 | 8593396.18 | 6078 | 2.9 | 3.69 | 0.04 |
| 272870.06 | 8593396.26 | 2358 | 1.05 | 1.42 | 0.03 |
| 272879.11 | 8593396.33 | 1598 | 0.8 | 0.96 | 0.02 |
| 272891.79 | 8593396.43 | 1182 | 0.6 | 0.71 | 0.02 |
| 272902.65 | 8593396.52 | 846 | 0.41 | 0.50 | 0.02 |
| 272918.94 | 8593396.65 | 771 | 0.29 | 0.46 | 0.02 |
| 272927.98 | 8593398.57 | 680 | 0.32 | 0.40 | 0.01 |
| 272946.09 | 8593398.71 | 479 | 0.21 | 0.28 | 0.01 |
| 272960.59 | 8593396.98 | 366 | 0.14 | 0.21 | 0.01 |
| 272971.45 | 8593397.07 | 286 | 0.12 | 0.16 | 0.01 |
| 272982.31 | 8593397.16 | 265 | 0.08 | 0.15 | 0.01 |
| 272991.36 | 8593397.23 | 264 | 0.09 | 0.15 | 0.01 |
| 273002.21 | 8593399.16 |  |  |  | 0.00 |
| 273002.23 | 8593397.32 | 241 | 0.07 | 0.13 | 0.01 |
| 273009.47 | 8593397.38 | 222 | 0.06 | 0.12 | 0.01 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |  |
| 273027.57 | 8593397.52 | 197 | 0.05 | 0.11 | 0.01 |
| 273043.87 | 8593397.65 | 204 | 0.05 | 0.11 | 0.01 |
| 273056.54 | 8593397.75 | 202 | 0.06 | 0.11 | 0.01 |
| 273067.4 | 8593397.84 | 209 | 0.04 | 0.11 | 0.01 |
| 273076.47 | 8593396.07 | 215 | 0.07 | 0.12 | 0.01 |
| 273090.94 | 8593398.03 | 168 | 0.04 | 0.09 | 0.01 |
| 272891.31 | 8593455.44 | 6032 | 4 | 3.66 | 0.04 |
| 272900.29 | 8593464.74 | 7790 | 5 | 4.73 | 0.05 |
| 272903.82 | 8593475.83 | 2120 | 1 | 1.28 | 0.03 |
| 272914.66 | 8593479.61 | 5603 | 3.5 | 3.40 | 0.04 |
| 272927.31 | 8593481.55 | 2300 | 1.1 | 1.39 | 0.03 |
| 272893.2 | 8593446.24 | 1611 | 0.9 | 0.97 | 0.02 |
| 272873.47 | 8593422.1 | 2578 | 1.3 | 1.56 | 0.03 |
| 272861.02 | 8593394.34 | 8348 | 5 | 5.07 | 0.05 |
| 272859.42 | 8593368.51 | 1198 | 0.7 | 0.72 | 0.02 |
| 272870 | 8593336 | 290 | 0.15 | 0.16 | 0.01 |

Appendix A3  γ dose rate results from Anomaly 2B survey (northeast of Anomaly 2A) conducted 1–5 September 2008.

| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |
| --- | --- | --- | --- | --- | --- |
| 273050.63 | 8593908.58 | 407 | 0.26 | 0.22 | 0.01 |
| 273063.3 | 8593908.68 | 471 | 0.25 | 0.25 | 0.01 |
| 273077.79 | 8593908.8 | 475 | 0.28 | 0.25 | 0.01 |
| 273090.46 | 8593908.9 | 539 | 0.29 | 0.29 | 0.01 |
| 273099.51 | 8593908.97 | 596 | 0.34 | 0.32 | 0.01 |
| 273108.56 | 8593909.04 | 728 | 0.39 | 0.40 | 0.01 |
| 273119.44 | 8593907.29 | 732 | 0.39 | 0.40 | 0.01 |
| 273130.29 | 8593909.22 | 734 | 0.48 | 0.40 | 0.02 |
| 273139.34 | 8593909.29 | 708 | 0.41 | 0.38 | 0.02 |
| 273153.83 | 8593909.41 | 742 | 0.41 | 0.40 | 0.02 |
| 273161.07 | 8593909.46 | 751 | 0.44 | 0.41 | 0.02 |
| 273170.08 | 8593915.07 | 853 | 0.45 | 0.47 | 0.02 |
| 273177.36 | 8593909.6 | 769 | 0.42 | 0.42 | 0.02 |
| 273179.17 | 8593909.61 | 813 | 0.48 | 0.44 | 0.02 |
| 273197.28 | 8593909.76 | 853 | 0.52 | 0.47 | 0.02 |
| 273213.57 | 8593909.89 | 637 | 0.39 | 0.34 | 0.02 |
| 273220.82 | 8593909.94 | 781 | 0.44 | 0.43 | 0.02 |
| 273229.88 | 8593908.17 | 721 | 0.45 | 0.39 | 0.02 |
| 273246.18 | 8593908.3 | 691 | 0.38 | 0.37 | 0.01 |
| 273262.46 | 8593910.28 | 740 | 0.42 | 0.40 | 0.02 |
| 273260.93 | 8593875.22 | 839 | 0.5 | 0.46 | 0.02 |
| 273250.05 | 8593876.98 | 910 | 0.51 | 0.50 | 0.02 |
| 273233.76 | 8593876.85 | 866 | 0.52 | 0.47 | 0.02 |
| 273217.48 | 8593874.88 | 955 | 0.55 | 0.52 | 0.02 |
| 273204.79 | 8593876.62 | 923 | 0.48 | 0.50 | 0.02 |
| 273192.11 | 8593876.52 | 1075 | 0.65 | 0.59 | 0.02 |
| 273179.44 | 8593876.41 | 1219 | 0.65 | 0.67 | 0.02 |
| 273166.77 | 8593876.31 | 1057 | 0.65 | 0.58 | 0.02 |
| 273150.47 | 8593876.18 | 968 | 0.6 | 0.53 | 0.02 |
| 273141.42 | 8593876.11 | 996 | 0.6 | 0.55 | 0.02 |
| 273126.94 | 8593875.99 | 851 | 0.5 | 0.46 | 0.02 |
| 273112.45 | 8593875.88 | 767 | 0.43 | 0.42 | 0.02 |
| 273101.59 | 8593875.79 | 712 | 0.4 | 0.39 | 0.01 |
| 273083.48 | 8593875.64 | 686 | 0.4 | 0.37 | 0.02 |
| 273067.19 | 8593875.51 | 564 | 0.35 | 0.30 | 0.01 |
| 273056.33 | 8593875.43 | 440 | 0.2 | 0.23 | 0.01 |
| 273047.26 | 8593877.2 | 363 | 0.2 | 0.19 | 0.01 |
| 273054.78 | 8593842.22 | 1091 | 0.65 | 0.60 | 0.02 |
| 273065.64 | 8593842.3 | 800 | 0.55 | 0.44 | 0.02 |
| 273078.33 | 8593840.56 | 1071 | 0.6 | 0.59 | 0.02 |
| 273083.74 | 8593844.29 | 904 | 0.5 | 0.49 | 0.02 |
| 273090.99 | 8593842.51 | 969 | 0.55 | 0.53 | 0.02 |
| 273103.65 | 8593844.45 | 1110 | 0.65 | 0.61 | 0.02 |
| 273112.7 | 8593844.53 | 1163 | 0.7 | 0.64 | 0.02 |
| 273123.58 | 8593842.77 | 1190 | 0.75 | 0.65 | 0.02 |
| 273134.43 | 8593844.7 | 953 | 0.6 | 0.52 | 0.02 |
| 273150.74 | 8593842.99 | 1237 | 0.7 | 0.68 | 0.02 |
| 273159.79 | 8593843.06 | 957 | 0.6 | 0.52 | 0.02 |
| 273176.09 | 8593843.19 | 827 | 0.45 | 0.45 | 0.02 |
| 273194.19 | 8593843.34 | 696 | 0.4 | 0.38 | 0.02 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |
| 273203.24 | 8593843.41 | 626 | 0.3 | 0.34 | 0.01 |
| 273214.11 | 8593843.5 | 723 | 0.4 | 0.39 | 0.01 |
| 273226.78 | 8593843.6 | 906 | 0.45 | 0.50 | 0.01 |
| 273235.83 | 8593843.67 | 940 | 0.5 | 0.51 | 0.02 |
| 273253.94 | 8593843.82 | 698 | 0.45 | 0.38 | 0.02 |
| 273264.8 | 8593843.9 | 642 | 0.35 | 0.35 | 0.01 |
| 273243.36 | 8593808.69 | 516 | 0.3 | 0.28 | 0.01 |
| 273230.67 | 8593810.43 | 548 | 0.3 | 0.29 | 0.01 |
| 273219.82 | 8593808.5 | 611 | 0.35 | 0.33 | 0.01 |
| 273208.94 | 8593810.26 | 763 | 0.45 | 0.42 | 0.02 |
| 273196.27 | 8593810.15 | 596 | 0.3 | 0.32 | 0.01 |
| 273189.03 | 8593810.1 | 519 | 0.33 | 0.28 | 0.01 |
| 273179.97 | 8593810.02 | 511 | 0.32 | 0.27 | 0.01 |
| 273165.49 | 8593809.91 | 547 | 0.33 | 0.29 | 0.01 |
| 273154.63 | 8593809.82 | 689 | 0.4 | 0.37 | 0.02 |
| 273143.76 | 8593809.73 | 888 | 0.5 | 0.49 | 0.02 |
| 273131.09 | 8593809.63 | 1159 | 0.7 | 0.64 | 0.02 |
| 273112.98 | 8593809.49 | 1708 | 1 | 0.94 | 0.02 |
| 273098.49 | 8593811.21 | 1247 | 0.8 | 0.69 | 0.02 |
| 273082.19 | 8593811.08 | 1357 | 0.8 | 0.75 | 0.02 |
| 273073.15 | 8593809.17 | 1306 | 0.8 | 0.72 | 0.02 |
| 273058.67 | 8593809.05 | 1119 | 0.65 | 0.61 | 0.02 |
| 273049.6 | 8593810.82 | 1090 | 0.5 | 0.60 | 0.02 |
| 273033.57 | 8593777.5 | 785 | 0.45 | 0.43 | 0.02 |
| 273044.44 | 8593777.58 | 770 | 0.45 | 0.42 | 0.02 |
| 273051.68 | 8593777.64 | 783 | 0.45 | 0.43 | 0.02 |
| 273057.11 | 8593777.68 | 792 | 0.45 | 0.43 | 0.02 |
| 273067.97 | 8593777.77 | 1092 | 0.5 | 0.60 | 0.02 |
| 273078.85 | 8593776.02 | 1136 | 0.65 | 0.62 | 0.02 |
| 273100.56 | 8593778.03 | 1035 | 0.55 | 0.57 | 0.02 |
| 273109.62 | 8593778.11 | 1025 | 0.6 | 0.56 | 0.02 |
| 273122.3 | 8593776.36 | 1100 | 0.65 | 0.60 | 0.02 |
| 273131.36 | 8593776.44 | 1031 | 0.55 | 0.57 | 0.02 |
| 273140.39 | 8593778.35 | 892 | 0.45 | 0.49 | 0.02 |
| 273153.07 | 8593778.46 | 573 | 0.35 | 0.31 | 0.01 |
| 273162.14 | 8593776.68 | 563 | 0.3 | 0.30 | 0.01 |
| 273169.38 | 8593776.74 | 547 | 0.28 | 0.29 | 0.01 |
| 273183.86 | 8593776.86 | 651 | 0.33 | 0.35 | 0.01 |
| 273192.91 | 8593776.93 | 643 | 0.3 | 0.35 | 0.01 |
| 273203.78 | 8593777.02 | 584 | 0.3 | 0.32 | 0.01 |
| 273216.45 | 8593777.12 | 524 | 0.3 | 0.28 | 0.01 |
| 273227.31 | 8593777.21 | 505 | 0.25 | 0.27 | 0.01 |
| 273238.18 | 8593777.29 | 579 | 0.3 | 0.31 | 0.01 |
| 273256.55 | 8593744.24 | 456 | 0.3 | 0.24 | 0.01 |
| 273249.31 | 8593744.19 | 508 | 0.3 | 0.27 | 0.01 |
| 273240.25 | 8593744.11 | 538 | 0.3 | 0.29 | 0.01 |
| 273227.58 | 8593744.01 | 468 | 0.3 | 0.25 | 0.01 |
| 273216.72 | 8593743.92 | 494 | 0.3 | 0.26 | 0.01 |
| 273209.47 | 8593743.87 | 554 | 0.35 | 0.30 | 0.01 |
| 273198.61 | 8593743.78 | 688 | 0.4 | 0.37 | 0.02 |
| 273191.37 | 8593743.72 | 653 | 0.4 | 0.35 | 0.02 |
| 273180.51 | 8593743.63 | 678 | 0.4 | 0.37 | 0.02 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |
| 273167.83 | 8593743.53 | 771 | 0.4 | 0.42 | 0.01 |
| 273153.35 | 8593743.42 | 850 | 0.5 | 0.46 | 0.02 |
| 273140.68 | 8593743.31 | 850 | 0.5 | 0.46 | 0.02 |
| 273129.81 | 8593743.23 | 795 | 0.45 | 0.43 | 0.02 |
| 273118.95 | 8593743.14 | 699 | 0.45 | 0.38 | 0.02 |
| 273106.26 | 8593744.88 | 736 | 0.48 | 0.40 | 0.02 |
| 273097.22 | 8593742.97 | 704 | 0.4 | 0.38 | 0.02 |
| 273088.16 | 8593744.74 | 677 | 0.4 | 0.37 | 0.02 |
| 273079.1 | 8593744.66 | 745 | 0.4 | 0.41 | 0.01 |
| 273062.81 | 8593744.53 | 694 | 0.4 | 0.38 | 0.02 |
| 273046.5 | 8593746.25 | 653 | 0.4 | 0.35 | 0.02 |
| 273032.03 | 8593744.29 | 543 | 0.35 | 0.29 | 0.02 |
| 273022.98 | 8593744.21 | 465 | 0.25 | 0.25 | 0.01 |
| 273012.11 | 8593744.13 | 487 | 0.22 | 0.26 | 0.01 |
| 272993.99 | 8593745.82 | 369 | 0.2 | 0.19 | 0.01 |
| 272979.53 | 8593743.86 | 396 | 0.2 | 0.21 | 0.01 |
| 272968.66 | 8593743.78 | 371 | 0.2 | 0.20 | 0.01 |
| 272982.97 | 8593766.02 | 369 | 0.15 | 0.19 | 0.01 |
| 272995.64 | 8593766.13 | 444 | 0.17 | 0.24 | 0.01 |
| 273004.69 | 8593766.2 | 517 | 0.25 | 0.28 | 0.01 |
| 273021 | 8593764.48 | 577 | 0.35 | 0.31 | 0.01 |
| 273026.44 | 8593764.53 | 578 | 0.3 | 0.31 | 0.01 |
| 273037.3 | 8593764.62 | 697 | 0.4 | 0.38 | 0.02 |
| 273037.2 | 8593777.52 | 808 | 0.4 | 0.44 | 0.01 |
| 273020.9 | 8593777.39 | 763 | 0.4 | 0.42 | 0.01 |
| 273008.23 | 8593777.29 | 628 | 0.35 | 0.34 | 0.01 |
| 272995.57 | 8593775.35 | 482 | 0.25 | 0.26 | 0.01 |
| 272984.69 | 8593777.1 | 401 | 0.2 | 0.21 | 0.01 |
| 272973.83 | 8593777.02 | 345 | 0.2 | 0.18 | 0.01 |
| 272964.76 | 8593778.79 | 292 | 0.15 | 0.15 | 0.01 |
| 272971.93 | 8593788.07 | 309 | 0.17 | 0.16 | 0.01 |
| 272975.53 | 8593789.94 | 353 | 0.2 | 0.19 | 0.01 |
| 272984.6 | 8593788.17 | 487 | 0.22 | 0.26 | 0.01 |
| 272999.09 | 8593788.28 | 676 | 0.4 | 0.37 | 0.02 |
| 273008.05 | 8593799.42 | 905 | 0.5 | 0.49 | 0.02 |
| 272995.36 | 8593801.16 | 490 | 0.25 | 0.26 | 0.01 |
| 272980.89 | 8593799.2 | 478 | 0.25 | 0.26 | 0.01 |
| 272977.26 | 8593801.02 | 368 | 0.2 | 0.19 | 0.01 |
| 272968.2 | 8593800.95 | 263 | 0.14 | 0.14 | 0.01 |
| 272968.13 | 8593810.17 | 325 | 0.17 | 0.17 | 0.01 |
| 272982.61 | 8593810.28 | 368 | 0.2 | 0.19 | 0.01 |
| 272993.48 | 8593810.37 | 418 | 0.25 | 0.22 | 0.01 |
| 273006.16 | 8593808.63 | 541 | 0.3 | 0.29 | 0.01 |
| 273017.03 | 8593808.72 | 833 | 0.5 | 0.45 | 0.02 |
| 273026.09 | 8593806.94 | 1063 | 0.6 | 0.58 | 0.02 |
| 273051.16 | 8593842.19 | 860 | 0.45 | 0.47 | 0.02 |
| 273034.85 | 8593843.9 | 866 | 0.45 | 0.47 | 0.02 |
| 273022.19 | 8593841.95 | 582 | 0.35 | 0.31 | 0.01 |
| 273013.14 | 8593841.88 | 506 | 0.3 | 0.27 | 0.01 |
| 273002.28 | 8593841.79 | 312 | 0.2 | 0.16 | 0.01 |
| 272991.4 | 8593843.55 | 309 | 0.17 | 0.16 | 0.01 |
| 272989.32 | 8593876.73 | 282 | 0.15 | 0.15 | 0.01 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |
| 273000.18 | 8593876.82 | 262 | 0.15 | 0.13 | 0.01 |
| 273020.1 | 8593876.98 | 331 | 0.2 | 0.17 | 0.01 |
| 273036.4 | 8593877.11 | 351 | 0.22 | 0.18 | 0.01 |
| 273037.95 | 8593908.48 | 378 | 0.22 | 0.20 | 0.01 |
| 273023.47 | 8593908.36 | 339 | 0.18 | 0.18 | 0.01 |
| 273012.61 | 8593908.27 | 289 | 0.18 | 0.15 | 0.01 |
| 273001.74 | 8593908.19 | 272 | 0.15 | 0.14 | 0.01 |
| 273010.69 | 8593921.17 | 266 | 0.14 | 0.14 | 0.01 |
| 273051.00 | 8593919.00 | 390 | 0.16 | 0.22 | 0.01 |
| 273066.00 | 8593919.00 | 458 | 0.22 | 0.27 | 0.01 |
| 273066.00 | 8593920.00 | 440 | 0.20 | 0.26 | 0.01 |
| 273089.00 | 8593920.00 | 541 | 0.25 | 0.32 | 0.01 |
| 273100.00 | 8593919.00 | 547 | 0.25 | 0.32 | 0.01 |
| 273115.00 | 8593920.00 | 560 | 0.22 | 0.33 | 0.01 |
| 273134.00 | 8593920.00 | 670 | 0.28 | 0.40 | 0.01 |
| 273142.00 | 8593920.00 | 736 | 0.35 | 0.44 | 0.01 |
| 273158.00 | 8593921.00 | 633 | 0.29 | 0.37 | 0.01 |
| 273173.00 | 8593920.00 | 637 | 0.29 | 0.38 | 0.01 |
| 273189.00 | 8593919.00 | 626 | 0.30 | 0.37 | 0.01 |
| 273202.00 | 8593921.00 | 612 | 0.30 | 0.36 | 0.01 |
| 273207.00 | 8593920.00 | 600 | 0.21 | 0.35 | 0.01 |
| 273218.00 | 8593920.00 | 561 | 0.20 | 0.33 | 0.01 |
| 273230.00 | 8593920.00 | 646 | 0.28 | 0.38 | 0.01 |
| 273242.00 | 8593920.00 | 643 | 0.28 | 0.38 | 0.01 |
| 273253.00 | 8593920.00 | 640 | 0.24 | 0.38 | 0.01 |
| 273263.00 | 8593920.00 | 645 | 0.30 | 0.38 | 0.01 |
| 273258.00 | 8593887.00 | 746 | 0.26 | 0.44 | 0.01 |
| 273247.00 | 8593888.00 | 768 | 0.29 | 0.46 | 0.01 |
| 273235.00 | 8593887.00 | 765 | 0.31 | 0.45 | 0.01 |
| 273224.00 | 8593887.00 | 902 | 0.33 | 0.54 | 0.01 |
| 273211.00 | 8593888.00 | 914 | 0.38 | 0.54 | 0.01 |
| 273201.00 | 8593888.00 | 858 | 0.40 | 0.51 | 0.01 |
| 273192.00 | 8593887.00 | 933 | 0.38 | 0.56 | 0.01 |
| 273181.00 | 8593887.00 | 1065 | 0.43 | 0.64 | 0.01 |
| 273166.00 | 8593887.00 | 1054 | 0.42 | 0.63 | 0.01 |
| 273154.00 | 8593887.00 | 932 | 0.39 | 0.56 | 0.01 |
| 273143.00 | 8593887.00 | 871 | 0.30 | 0.52 | 0.01 |
| 273129.00 | 8593887.00 | 758 | 0.35 | 0.45 | 0.01 |
| 273116.00 | 8593887.00 | 734 | 0.31 | 0.43 | 0.01 |
| 273100.00 | 8593887.00 | 640 | 0.28 | 0.38 | 0.01 |
| 273081.00 | 8593887.00 | 650 | 0.22 | 0.38 | 0.01 |
| 273068.00 | 8593887.00 | 500 | 0.20 | 0.29 | 0.01 |
| 273058.00 | 8593887.00 | 438 | 0.15 | 0.25 | 0.01 |
| 273043.00 | 8593888.00 | 335 | 0.12 | 0.19 | 0.01 |
| 273061.65 | 8593888.38 | 338 | 0.21 | 0.19 | 0.01 |
| 273045.37 | 8593886.40 | 332 | 0.17 | 0.19 | 0.01 |
| 273029.08 | 8593886.27 | 298 | 0.10 | 0.17 | 0.01 |
| 273020.01 | 8593888.04 | 285 | 0.12 | 0.16 | 0.01 |
| 273009.43 | 8593852.92 | 355 | 0.11 | 0.20 | 0.01 |
| 273025.71 | 8593854.89 | 306 | 0.16 | 0.17 | 0.01 |
| 273036.57 | 8593854.98 | 423 | 0.19 | 0.25 | 0.01 |
| 273049.26 | 8593853.24 | 433 | 0.23 | 0.25 | 0.01 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |
| 273058.31 | 8593853.31 | 419 | 0.20 | 0.24 | 0.01 |
| 273070.99 | 8593853.41 | 584 | 0.19 | 0.34 | 0.01 |
| 273080.03 | 8593855.33 | 768 | 0.25 | 0.46 | 0.01 |
| 273087.28 | 8593853.54 | 795 | 0.39 | 0.47 | 0.01 |
| 273098.15 | 8593853.63 | 778 | 0.28 | 0.46 | 0.01 |
| 273109.01 | 8593853.72 | 843 | 0.32 | 0.50 | 0.01 |
| 273116.25 | 8593853.78 | 1003 | 0.42 | 0.60 | 0.01 |
| 273128.92 | 8593853.88 | 1428 | 0.61 | 0.86 | 0.02 |
| 273137.96 | 8593855.79 | 1088 | 0.52 | 0.65 | 0.02 |
| 273145.22 | 8593854.01 | 1289 | 0.49 | 0.77 | 0.01 |
| 273152.46 | 8593854.07 | 1213 | 0.55 | 0.73 | 0.02 |
| 273161.51 | 8593854.14 | 1334 | 0.49 | 0.80 | 0.01 |
| 273174.19 | 8593854.24 | 1031 | 0.44 | 0.62 | 0.01 |
| 273185.05 | 8593854.33 | 847 | 0.35 | 0.50 | 0.01 |
| 273195.91 | 8593854.42 | 940 | 0.38 | 0.56 | 0.01 |
| 273204.97 | 8593854.49 | 818 | 0.29 | 0.49 | 0.01 |
| 273215.83 | 8593854.58 | 777 | 0.22 | 0.46 | 0.01 |
| 273230.31 | 8593854.69 | 817 | 0.28 | 0.49 | 0.01 |
| 273244.81 | 8593852.96 | 1134 | 0.47 | 0.68 | 0.01 |
| 273257.47 | 8593854.91 | 834 | 0.31 | 0.50 | 0.01 |
| 273266.52 | 8593854.98 | 734 | 0.28 | 0.43 | 0.01 |
| 273261.37 | 8593819.90 | 543 | 0.18 | 0.32 | 0.01 |
| 273252.31 | 8593821.67 | 583 | 0.21 | 0.34 | 0.01 |
| 273241.44 | 8593821.58 | 586 | 0.23 | 0.34 | 0.01 |
| 273230.58 | 8593821.50 | 684 | 0.18 | 0.40 | 0.01 |
| 273219.72 | 8593821.41 | 720 | 0.22 | 0.43 | 0.01 |
| 273205.23 | 8593821.29 | 664 | 0.24 | 0.39 | 0.01 |
| 273190.75 | 8593821.18 | 593 | 0.16 | 0.35 | 0.01 |
| 273181.70 | 8593821.10 | 586 | 0.27 | 0.34 | 0.01 |
| 273167.21 | 8593820.99 | 616 | 0.19 | 0.36 | 0.01 |
| 273156.35 | 8593820.90 | 657 | 0.22 | 0.39 | 0.01 |
| 273149.11 | 8593820.84 | 840 | 0.30 | 0.50 | 0.01 |
| 273134.62 | 8593820.73 | 1114 | 0.38 | 0.67 | 0.01 |
| 273118.31 | 8593822.44 | 1479 | 0.65 | 0.89 | 0.02 |
| 273100.22 | 8593820.45 | 1098 | 0.60 | 0.66 | 0.02 |
| 273087.55 | 8593820.35 | 1119 | 0.60 | 0.67 | 0.02 |
| 273080.29 | 8593822.13 | 1248 | 0.50 | 0.75 | 0.01 |
| 273064.01 | 8593820.16 | 1496 | 0.65 | 0.90 | 0.02 |
| 273054.94 | 8593821.93 | 1089 | 0.50 | 0.65 | 0.02 |
| 273042.29 | 8593819.98 | 1321 | 0.57 | 0.79 | 0.02 |
| 273029.60 | 8593821.73 | 1037 | 0.50 | 0.62 | 0.02 |
| 273016.92 | 8593821.63 | 820 | 0.38 | 0.49 | 0.01 |
| 273004.25 | 8593821.52 | 639 | 0.37 | 0.38 | 0.01 |
| 272991.58 | 8593821.42 | 420 | 0.20 | 0.24 | 0.01 |
| 272978.90 | 8593821.32 | 365 | 0.21 | 0.21 | 0.01 |
| 273028.05 | 8593788.52 | 944 | 0.55 | 0.56 | 0.02 |
| 273044.35 | 8593788.65 | 759 | 0.44 | 0.45 | 0.02 |
| 273055.23 | 8593786.89 | 874 | 0.49 | 0.52 | 0.02 |
| 273064.26 | 8593788.81 | 830 | 0.52 | 0.49 | 0.02 |
| 273075.13 | 8593788.90 | 964 | 0.55 | 0.58 | 0.02 |
| 273082.37 | 8593788.95 | 1014 | 0.58 | 0.61 | 0.02 |
| 273096.85 | 8593789.07 | 1597 | 0.77 | 0.96 | 0.02 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |
| 273107.73 | 8593787.31 | 1164 | 0.60 | 0.70 | 0.02 |
| 273120.40 | 8593787.41 | 918 | 0.47 | 0.55 | 0.02 |
| 273131.27 | 8593787.50 | 1021 | 0.60 | 0.61 | 0.02 |
| 273142.13 | 8593787.59 | 874 | 0.51 | 0.52 | 0.02 |
| 273154.80 | 8593787.69 | 610 | 0.29 | 0.36 | 0.01 |
| 273163.86 | 8593787.76 | 595 | 0.38 | 0.35 | 0.02 |
| 273171.10 | 8593787.82 | 523 | 0.28 | 0.31 | 0.01 |
| 273183.77 | 8593787.92 | 524 | 0.29 | 0.31 | 0.01 |
| 273198.26 | 8593788.04 | 484 | 0.29 | 0.28 | 0.01 |
| 273210.93 | 8593788.14 | 566 | 0.28 | 0.33 | 0.01 |
| 273219.98 | 8593788.21 | 430 | 0.23 | 0.25 | 0.01 |
| 273230.85 | 8593788.30 | 460 | 0.26 | 0.27 | 0.01 |
| 273245.33 | 8593788.42 | 480 | 0.25 | 0.28 | 0.01 |
| 273254.38 | 8593788.49 | 475 | 0.26 | 0.28 | 0.01 |
| 273258.27 | 8593755.32 | 505 | 0.25 | 0.30 | 0.01 |
| 273252.84 | 8593755.28 | 530 | 0.28 | 0.31 | 0.01 |
| 273241.98 | 8593755.19 | 533 | 0.28 | 0.31 | 0.01 |
| 273231.11 | 8593755.11 | 449 | 0.26 | 0.26 | 0.01 |
| 273222.06 | 8593755.03 | 522 | 0.25 | 0.31 | 0.01 |
| 273207.58 | 8593754.92 | 600 | 0.25 | 0.35 | 0.01 |
| 273193.09 | 8593754.80 | 667 | 0.30 | 0.39 | 0.01 |
| 273184.04 | 8593754.73 | 652 | 0.30 | 0.38 | 0.01 |
| 273173.16 | 8593756.48 | 701 | 0.35 | 0.41 | 0.01 |
| 273164.12 | 8593754.57 | 741 | 0.32 | 0.44 | 0.01 |
| 273156.88 | 8593754.51 | 785 | 0.38 | 0.47 | 0.01 |
| 273142.40 | 8593754.39 | 803 | 0.39 | 0.48 | 0.01 |
| 273133.33 | 8593756.17 | 748 | 0.38 | 0.44 | 0.01 |
| 273120.66 | 8593756.06 | 769 | 0.36 | 0.46 | 0.01 |
| 273106.17 | 8593755.95 | 730 | 0.32 | 0.43 | 0.01 |
| 273100.76 | 8593754.06 | 750 | 0.41 | 0.44 | 0.01 |
| 273091.70 | 8593753.99 | 765 | 0.40 | 0.45 | 0.01 |
| 273079.00 | 8593757.57 | 686 | 0.33 | 0.41 | 0.01 |
| 273071.79 | 8593753.83 | 653 | 0.30 | 0.39 | 0.01 |
| 273057.30 | 8593753.71 | 765 | 0.33 | 0.45 | 0.01 |
| 273046.43 | 8593755.47 | 681 | 0.30 | 0.40 | 0.01 |
| 273030.13 | 8593755.34 | 574 | 0.25 | 0.34 | 0.01 |
| 273017.47 | 8593753.39 | 548 | 0.28 | 0.32 | 0.01 |
| 273004.77 | 8593756.98 | 474 | 0.25 | 0.28 | 0.01 |
| 272988.49 | 8593755.00 | 398 | 0.22 | 0.23 | 0.01 |
| 272977.64 | 8593753.07 | 405 | 0.16 | 0.23 | 0.01 |
| 272967.03 | 8593721.63 | 351 | 0.16 | 0.20 | 0.01 |
| 272979.69 | 8593723.58 | 387 | 0.19 | 0.22 | 0.01 |
| 272988.76 | 8593721.81 | 407 | 0.21 | 0.24 | 0.01 |
| 272999.62 | 8593721.89 | 380 | 0.19 | 0.22 | 0.01 |
| 273015.91 | 8593722.02 | 402 | 0.20 | 0.23 | 0.01 |
| 273023.16 | 8593722.08 | 383 | 0.19 | 0.22 | 0.01 |
| 273037.64 | 8593722.20 | 451 | 0.25 | 0.26 | 0.01 |
| 273046.69 | 8593722.27 | 509 | 0.28 | 0.30 | 0.01 |
| 273055.74 | 8593722.34 | 491 | 0.25 | 0.29 | 0.01 |
| 273066.61 | 8593722.43 | 520 | 0.26 | 0.30 | 0.01 |
| 273073.85 | 8593722.49 | 556 | 0.32 | 0.33 | 0.01 |
| 273082.90 | 8593722.56 | 496 | 0.31 | 0.29 | 0.01 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |
| 273099.20 | 8593722.69 | 636 | 0.32 | 0.38 | 0.01 |
| 273111.88 | 8593720.95 | 608 | 0.33 | 0.36 | 0.01 |
| 273122.75 | 8593721.04 | 618 | 0.35 | 0.36 | 0.01 |
| 273129.98 | 8593722.94 | 646 | 0.31 | 0.38 | 0.01 |
| 273139.03 | 8593723.01 | 674 | 0.29 | 0.40 | 0.01 |
| 273166.20 | 8593721.39 | 661 | 0.37 | 0.39 | 0.01 |
| 273166.20 | 8593721.39 | 683 | 0.37 | 0.40 | 0.01 |
| 273177.06 | 8593721.47 | 655 | 0.31 | 0.39 | 0.01 |
| 273184.29 | 8593723.38 | 633 | 0.31 | 0.37 | 0.01 |
| 273196.98 | 8593721.63 | 635 | 0.28 | 0.37 | 0.01 |
| 273213.27 | 8593721.77 | 560 | 0.31 | 0.33 | 0.01 |
| 273220.52 | 8593721.82 | 532 | 0.25 | 0.31 | 0.01 |
| 273238.62 | 8593721.97 | 469 | 0.21 | 0.27 | 0.01 |
| 273247.67 | 8593722.04 | 408 | 0.19 | 0.24 | 0.01 |
| 273263.97 | 8593722.17 | 401 | 0.22 | 0.23 | 0.01 |
| 273052.53 | 8593897.53 | 373 | 0.2 | 0.21 | 0.01 |
| 273068.82 | 8593897.66 | 412 | 0.2 | 0.24 | 0.01 |
| 273079.68 | 8593897.75 | 522 | 0.3 | 0.31 | 0.01 |
| 273088.74 | 8593897.82 | 554 | 0.3 | 0.32 | 0.01 |
| 273101.41 | 8593897.92 | 599 | 0.35 | 0.35 | 0.01 |
| 273106.84 | 8593897.96 | 672 | 0.35 | 0.40 | 0.01 |
| 273119.52 | 8593898.07 | 653 | 0.35 | 0.38 | 0.01 |
| 273130.38 | 8593898.15 | 692 | 0.4 | 0.41 | 0.02 |
| 273139.43 | 8593898.23 | 690 | 0.4 | 0.41 | 0.02 |
| 273150.29 | 8593898.31 | 753 | 0.4 | 0.45 | 0.01 |
| 273162.97 | 8593898.41 | 807 | 0.5 | 0.48 | 0.02 |
| 273172.02 | 8593898.49 | 852 | 0.5 | 0.51 | 0.02 |
| 273190.13 | 8593898.63 | 854 | 0.5 | 0.51 | 0.02 |
| 273201 | 8593896.87 | 875 | 0.5 | 0.52 | 0.02 |
| 273208.25 | 8593896.93 | 796 | 0.45 | 0.47 | 0.02 |
| 273222.73 | 8593897.05 | 819 | 0.45 | 0.49 | 0.02 |
| 273233.59 | 8593897.14 | 703 | 0.35 | 0.42 | 0.01 |
| 273249.87 | 8593899.11 | 769 | 0.4 | 0.46 | 0.01 |
| 273258.94 | 8593897.34 | 677 | 0.4 | 0.40 | 0.02 |
| 273267.99 | 8593897.41 | 754 | 0.4 | 0.45 | 0.01 |
| 273255.57 | 8593865.96 | 843 | 0.5 | 0.50 | 0.02 |
| 273246.52 | 8593865.89 | 905 | 0.45 | 0.54 | 0.01 |
| 273235.66 | 8593865.8 | 966 | 0.5 | 0.58 | 0.02 |
| 273224.79 | 8593865.71 | 839 | 0.45 | 0.50 | 0.02 |
| 273217.55 | 8593865.65 | 884 | 0.45 | 0.53 | 0.02 |
| 273206.69 | 8593865.57 | 902 | 0.5 | 0.54 | 0.02 |
| 273197.63 | 8593865.49 | 898 | 0.5 | 0.53 | 0.02 |
| 273183.15 | 8593865.38 | 1208 | 0.6 | 0.72 | 0.02 |
| 273174.1 | 8593865.31 | 1006 | 0.55 | 0.60 | 0.02 |
| 273159.61 | 8593865.19 | 1255 | 0.8 | 0.75 | 0.02 |
| 273145.13 | 8593865.07 | 952 | 0.5 | 0.57 | 0.02 |
| 273128.84 | 8593864.94 | 892 | 0.5 | 0.53 | 0.02 |
| 273112.54 | 8593864.81 | 719 | 0.4 | 0.43 | 0.01 |
| 273099.87 | 8593864.71 | 577 | 0.35 | 0.34 | 0.01 |
| 273087.19 | 8593864.61 | 645 | 0.4 | 0.38 | 0.02 |
| 273074.5 | 8593866.35 | 537 | 0.3 | 0.31 | 0.01 |
| 273065.47 | 8593864.43 | 478 | 0.25 | 0.28 | 0.01 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |
| 273050.98 | 8593864.32 | 410 | 0.2 | 0.24 | 0.01 |
| 273040.11 | 8593866.07 | 355 | 0.2 | 0.20 | 0.01 |
| 273054.87 | 8593831.15 | 1324 | 0.6 | 0.79 | 0.02 |
| 273063.92 | 8593831.22 | 1148 | 0.6 | 0.69 | 0.02 |
| 273071.17 | 8593831.28 | 1232 | 0.6 | 0.74 | 0.02 |
| 273092.89 | 8593831.46 | 1114 | 0.6 | 0.67 | 0.02 |
| 273100.13 | 8593831.51 | 1202 | 0.6 | 0.72 | 0.02 |
| 273109.19 | 8593831.59 | 1057 | 0.55 | 0.63 | 0.02 |
| 273116.43 | 8593831.65 | 1096 | 0.6 | 0.65 | 0.02 |
| 273125.48 | 8593831.72 | 1149 | 0.6 | 0.69 | 0.02 |
| 273134.53 | 8593831.79 | 1260 | 0.6 | 0.75 | 0.02 |
| 273147.21 | 8593831.89 | 1123 | 0.7 | 0.67 | 0.02 |
| 273163.5 | 8593832.02 | 779 | 0.4 | 0.46 | 0.01 |
| 273170.74 | 8593832.08 | 695 | 0.4 | 0.41 | 0.02 |
| 273188.85 | 8593832.23 | 519 | 0.25 | 0.30 | 0.01 |
| 273196.09 | 8593832.29 | 452 | 0.25 | 0.26 | 0.01 |
| 273210.57 | 8593832.4 | 831 | 0.4 | 0.49 | 0.01 |
| 273225.06 | 8593832.52 | 745 | 0.4 | 0.44 | 0.01 |
| 273237.73 | 8593832.62 | 654 | 0.35 | 0.39 | 0.01 |
| 273252.22 | 8593832.74 | 612 | 0.35 | 0.36 | 0.01 |
| 273266.7 | 8593832.85 | 627 | 0.35 | 0.37 | 0.01 |
| 273257.93 | 8593797.74 | 451 | 0.3 | 0.26 | 0.01 |
| 273241.62 | 8593799.45 | 492 | 0.3 | 0.29 | 0.01 |
| 273230.76 | 8593799.37 | 492 | 0.25 | 0.29 | 0.01 |
| 273219.89 | 8593799.28 | 597 | 0.3 | 0.35 | 0.01 |
| 273209.03 | 8593799.19 | 558 | 0.35 | 0.33 | 0.01 |
| 273196.36 | 8593799.09 | 604 | 0.35 | 0.36 | 0.01 |
| 273183.68 | 8593798.99 | 528 | 0.3 | 0.31 | 0.01 |
| 273174.63 | 8593798.92 | 495 | 0.25 | 0.29 | 0.01 |
| 273165.58 | 8593798.84 | 531 | 0.3 | 0.31 | 0.01 |
| 273158.34 | 8593798.78 | 611 | 0.25 | 0.36 | 0.01 |
| 273147.47 | 8593798.7 | 625 | 0.35 | 0.37 | 0.01 |
| 273136.61 | 8593798.61 | 854 | 0.5 | 0.51 | 0.02 |
| 273118.51 | 8593798.47 | 1040 | 0.5 | 0.62 | 0.02 |
| 273111.26 | 8593798.41 | 1140 | 0.6 | 0.68 | 0.02 |
| 273104.02 | 8593798.35 | 1940 | 1 | 1.17 | 0.02 |
| 273084.09 | 8593800.03 | 1215 | 0.7 | 0.73 | 0.02 |
| 273066 | 8593798.04 | 935 | 0.5 | 0.56 | 0.02 |
| 273053.31 | 8593799.79 | 832 | 0.45 | 0.49 | 0.02 |
| 273042.46 | 8593797.85 | 803 | 0.4 | 0.48 | 0.01 |
| 273027.96 | 8593799.58 | 907 | 0.5 | 0.54 | 0.02 |
| 273053.58 | 8593766.59 | 660 | 0.4 | 0.39 | 0.02 |
| 273060.82 | 8593766.65 | 712 | 0.4 | 0.42 | 0.01 |
| 273071.68 | 8593766.74 | 869 | 0.5 | 0.52 | 0.02 |
| 273078.93 | 8593766.79 | 1156 | 0.6 | 0.69 | 0.02 |
| 273087.98 | 8593766.87 | 972 | 0.55 | 0.58 | 0.02 |
| 273097.03 | 8593766.94 | 923 | 0.5 | 0.55 | 0.02 |
| 273109.7 | 8593767.04 | 849 | 0.5 | 0.50 | 0.02 |
| 273126 | 8593767.17 | 868 | 0.5 | 0.52 | 0.02 |
| 273135.05 | 8593767.24 | 995 | 0.6 | 0.59 | 0.02 |
| 273147.74 | 8593765.5 | 827 | 0.45 | 0.49 | 0.02 |
| 273154.97 | 8593767.4 | 577 | 0.35 | 0.34 | 0.01 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |
| 273171.28 | 8593765.69 | 660 | 0.4 | 0.39 | 0.02 |
| 273176.71 | 8593765.73 | 678 | 0.4 | 0.40 | 0.02 |
| 273183.95 | 8593765.79 | 604 | 0.3 | 0.36 | 0.01 |
| 273193 | 8593765.87 | 526 | 0.3 | 0.31 | 0.01 |
| 273203.87 | 8593765.95 | 539 | 0.3 | 0.32 | 0.01 |
| 273218.35 | 8593766.07 | 562 | 0.3 | 0.33 | 0.01 |
| 273231.02 | 8593766.17 | 539 | 0.3 | 0.32 | 0.01 |
| 273243.7 | 8593766.27 | 531 | 0.3 | 0.31 | 0.01 |
| 273258.18 | 8593766.39 | 464 | 0.25 | 0.27 | 0.01 |
| 273258.45 | 8593733.19 | 390 | 0.25 | 0.22 | 0.01 |
| 273247.58 | 8593733.11 | 423 | 0.25 | 0.24 | 0.01 |
| 273236.72 | 8593733.02 | 436 | 0.25 | 0.25 | 0.01 |
| 273222.24 | 8593732.9 | 527 | 0.3 | 0.31 | 0.01 |
| 273213.18 | 8593732.83 | 557 | 0.3 | 0.33 | 0.01 |
| 273198.7 | 8593732.71 | 656 | 0.35 | 0.39 | 0.01 |
| 273191.46 | 8593732.66 | 681 | 0.35 | 0.40 | 0.01 |
| 273182.41 | 8593732.58 | 724 | 0.4 | 0.43 | 0.01 |
| 273169.73 | 8593732.48 | 660 | 0.35 | 0.39 | 0.01 |
| 273157.06 | 8593732.38 | 691 | 0.4 | 0.41 | 0.02 |
| 273142.57 | 8593732.26 | 719 | 0.4 | 0.43 | 0.01 |
| 273133.52 | 8593732.19 | 683 | 0.35 | 0.40 | 0.01 |
| 273117.23 | 8593732.06 | 625 | 0.35 | 0.37 | 0.01 |
| 273100.92 | 8593733.77 | 659 | 0.35 | 0.39 | 0.01 |
| 273090.06 | 8593733.69 | 625 | 0.35 | 0.37 | 0.01 |
| 273077.38 | 8593733.58 | 647 | 0.35 | 0.38 | 0.01 |
| 273066.52 | 8593733.5 | 503 | 0.3 | 0.29 | 0.01 |
| 273055.66 | 8593733.41 | 542 | 0.3 | 0.32 | 0.01 |
| 273044.79 | 8593733.32 | 540 | 0.3 | 0.32 | 0.01 |
| 273028.5 | 8593733.19 | 502 | 0.25 | 0.29 | 0.01 |
| 273017.63 | 8593733.1 | 399 | 0.2 | 0.23 | 0.01 |
| 273001.33 | 8593734.82 | 373 | 0.2 | 0.21 | 0.01 |
| 272983.22 | 8593734.67 | 350 | 0.16 | 0.20 | 0.01 |
| 272974.18 | 8593732.76 | 334 | 0.175 | 0.19 | 0.01 |
| 272963.32 | 8593732.67 | 358 | 0.2 | 0.21 | 0.01 |
| 272963.59 | 8593699.47 | 296 | 0.15 | 0.17 | 0.01 |
| 272976.26 | 8593699.57 | 323 | 0.2 | 0.18 | 0.01 |
| 272985.31 | 8593699.65 | 347 | 0.2 | 0.20 | 0.01 |
| 272997.99 | 8593699.75 | 325 | 0.2 | 0.18 | 0.01 |
| 273007.04 | 8593699.82 | 353 | 0.2 | 0.20 | 0.01 |
| 273016.09 | 8593699.89 | 345 | 0.2 | 0.20 | 0.01 |
| 273028.76 | 8593700 | 387 | 0.2 | 0.22 | 0.01 |
| 273045.06 | 8593700.13 | 425 | 0.25 | 0.25 | 0.01 |
| 273059.54 | 8593700.24 | 493 | 0.25 | 0.29 | 0.01 |
| 273072.22 | 8593700.35 | 494 | 0.25 | 0.29 | 0.01 |
| 273084.89 | 8593700.45 | 490 | 0.25 | 0.29 | 0.01 |
| 273097.56 | 8593700.55 | 463 | 0.25 | 0.27 | 0.01 |
| 273106.62 | 8593700.62 | 523 | 0.3 | 0.31 | 0.01 |
| 273113.86 | 8593700.68 | 499 | 0.25 | 0.29 | 0.01 |
| 273128.36 | 8593698.95 | 519 | 0.3 | 0.30 | 0.01 |
| 273139.21 | 8593700.88 | 596 | 0.35 | 0.35 | 0.01 |
| 273151.88 | 8593700.99 | 589 | 0.3 | 0.35 | 0.01 |
| 273166.38 | 8593699.26 | 563 | 0.35 | 0.33 | 0.01 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |
| 273180.86 | 8593699.37 | 566 | 0.3 | 0.33 | 0.01 |
| 273195.35 | 8593699.49 | 474 | 0.25 | 0.28 | 0.01 |
| 273209.83 | 8593699.61 | 480 | 0.25 | 0.28 | 0.01 |
| 273224.31 | 8593699.72 | 451 | 0.25 | 0.26 | 0.01 |
| 273238.8 | 8593699.84 | 435 | 0.25 | 0.25 | 0.01 |
| 273249.66 | 8593699.93 | 308 | 0.15 | 0.17 | 0.01 |
| 273262.33 | 8593700.03 | 296 | 0.15 | 0.17 | 0.01 |
| 273271.39 | 8593700.1 | 241 | 0.15 | 0.13 | 0.01 |
| 273047.63 | 8593831.09 | 1289 | 0.7 | 0.77 | 0.02 |
| 273031.32 | 8593832.81 | 967 | 0.5 | 0.58 | 0.02 |
| 273018.65 | 8593832.7 | 1011 | 0.5 | 0.60 | 0.02 |
| 273009.61 | 8593830.79 | 525 | 0.3 | 0.31 | 0.01 |
| 272996.92 | 8593832.53 | 369 | 0.2 | 0.21 | 0.01 |
| 272982.44 | 8593832.41 | 338 | 0.175 | 0.19 | 0.01 |
| 272985.79 | 8593865.64 | 239 | 0.15 | 0.13 | 0.01 |
| 272996.65 | 8593865.73 | 264 | 0.15 | 0.15 | 0.01 |
| 273009.34 | 8593863.98 | 279 | 0.15 | 0.16 | 0.01 |
| 273022.01 | 8593864.08 | 336 | 0.175 | 0.19 | 0.01 |
| 273040.11 | 8593866.07 | 374 | 0.2 | 0.21 | 0.01 |
| 273041.66 | 8593897.44 | 387 | 0.2 | 0.22 | 0.01 |
| 273028.98 | 8593899.18 | 347 | 0.175 | 0.20 | 0.01 |
| 273019.92 | 8593899.11 | 308 | 0.15 | 0.17 | 0.01 |
| 273019.76 | 8593919.4 | 284 | 0.15 | 0.16 | 0.01 |
| 273030.62 | 8593919.48 | 340 | 0.175 | 0.19 | 0.01 |
| 273041.49 | 8593919.57 | 373 | 0.2 | 0.21 | 0.01 |

Appendix A4  γ dose rate results from survey between Anomalies 2A and 2B, and on top of Anomaly 2C (southwest of Anomaly 2A), conducted 8–10 October 2008.

| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |
| --- | --- | --- | --- | --- | --- |
| 272574.28 | 8593701.88 | 267 | 0.13 | 0.14 | 0.01 |
| 272596.02 | 8593700.21 | 286 | 0.16 | 0.15 | 0.01 |
| 272626.82 | 8593698.61 | 295 | 0.15 | 0.16 | 0.01 |
| 272643.1 | 8593700.58 | 298 | 0.18 | 0.16 | 0.01 |
| 272666.63 | 8593700.77 | 263 | 0.15 | 0.14 | 0.01 |
| 272688.36 | 8593700.95 | 302 | 0.16 | 0.16 | 0.01 |
| 272706.46 | 8593701.09 | 282 | 0.14 | 0.15 | 0.01 |
| 272728.21 | 8593699.43 | 272 | 0.14 | 0.15 | 0.01 |
| 272746.31 | 8593699.57 | 291 | 0.16 | 0.16 | 0.01 |
| 272764.4 | 8593701.56 | 303 | 0.17 | 0.16 | 0.01 |
| 272787.95 | 8593699.91 | 277 | 0.15 | 0.15 | 0.01 |
| 272809.68 | 8593700.08 | 279 | 0.14 | 0.15 | 0.01 |
| 272835.01 | 8593702.13 | 295 | 0.18 | 0.16 | 0.01 |
| 272853.13 | 8593700.43 | 319 | 0.18 | 0.17 | 0.01 |
| 272871.24 | 8593700.58 | 315 | 0.19 | 0.17 | 0.01 |
| 272887.55 | 8593698.86 | 354 | 0.20 | 0.19 | 0.01 |
| 272912.88 | 8593700.91 | 303 | 0.19 | 0.16 | 0.01 |
| 272938.22 | 8593701.11 | 316 | 0.17 | 0.17 | 0.01 |
| 272963.59 | 8593699.47 | 315 | 0.16 | 0.17 | 0.01 |
| 272985.31 | 8593699.65 | 304 | 0.17 | 0.16 | 0.01 |
| 273016.09 | 8593699.89 | 320 | 0.18 | 0.17 | 0.01 |
| 273041.42 | 8593701.94 | 366 | 0.20 | 0.20 | 0.01 |
| 273064.97 | 8593700.29 | 461 | 0.23 | 0.25 | 0.01 |
| 273084.91 | 8593698.6 | 490 | 0.22 | 0.27 | 0.01 |
| 273115.67 | 8593700.69 | 521 | 0.29 | 0.29 | 0.01 |
| 273137.39 | 8593700.87 | 544 | 0.27 | 0.30 | 0.01 |
| 273151.89 | 8593699.14 | 602 | 0.32 | 0.34 | 0.01 |
| 273175.43 | 8593699.33 | 536 | 0.28 | 0.30 | 0.01 |
| 273224.28 | 8593703.41 | 467 | 0.26 | 0.26 | 0.01 |
| 273246.04 | 8593699.9 | 346 | 0.20 | 0.19 | 0.01 |
| 273274.99 | 8593701.97 | 283 | 0.15 | 0.15 | 0.01 |
| 273289.49 | 8593700.25 | 258 | 0.16 | 0.14 | 0.01 |
| 273268.25 | 8593639.21 | 449 | 0.24 | 0.25 | 0.01 |
| 273251.96 | 8593639.08 | 359 | 0.20 | 0.20 | 0.01 |
| 273222.98 | 8593640.69 | 274 | 0.20 | 0.15 | 0.01 |
| 273192.2 | 8593640.45 | 281 | 0.15 | 0.15 | 0.01 |
| 273170.47 | 8593640.27 | 333 | 0.18 | 0.18 | 0.01 |
| 273155.99 | 8593640.16 | 295 | 0.18 | 0.16 | 0.01 |
| 273123.38 | 8593641.74 | 344 | 0.20 | 0.19 | 0.01 |
| 273105.29 | 8593639.75 | 312 | 0.16 | 0.17 | 0.01 |
| 273087.17 | 8593641.45 | 331 | 0.20 | 0.18 | 0.01 |
| 273072.71 | 8593639.49 | 344 | 0.21 | 0.19 | 0.01 |
| 273052.79 | 8593639.33 | 348 | 0.20 | 0.19 | 0.01 |
| 273023.81 | 8593640.94 | 354 | 0.18 | 0.19 | 0.01 |
| 272994.84 | 8593640.71 | 359 | 0.18 | 0.20 | 0.01 |
| 272978.54 | 8593640.58 | 366 | 0.21 | 0.20 | 0.01 |
| 272940.54 | 8593638.43 | 448 | 0.22 | 0.25 | 0.01 |
| 272924.23 | 8593640.14 | 384 | 0.25 | 0.21 | 0.01 |
| 272897.07 | 8593639.92 | 378 | 0.19 | 0.21 | 0.01 |
| 272871.72 | 8593639.72 | 336 | 0.22 | 0.18 | 0.01 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |
| 272844.57 | 8593639.5 | 322 | 0.18 | 0.17 | 0.01 |
| 272815.6 | 8593639.27 | 350 | 0.20 | 0.19 | 0.01 |
| 272784.82 | 8593639.02 | 311 | 0.16 | 0.17 | 0.01 |
| 272768.51 | 8593640.73 | 363 | 0.16 | 0.20 | 0.01 |
| 272746.79 | 8593640.56 | 327 | 0.16 | 0.18 | 0.01 |
| 272732.3 | 8593640.44 | 334 | 0.17 | 0.18 | 0.01 |
| 272701.52 | 8593640.19 | 263 | 0.14 | 0.14 | 0.01 |
| 272683.42 | 8593640.05 | 300 | 0.16 | 0.16 | 0.01 |
| 272654.45 | 8593639.81 | 276 | 0.13 | 0.15 | 0.01 |
| 272630.91 | 8593639.62 | 279 | 0.14 | 0.15 | 0.01 |
| 272607.38 | 8593639.44 | 295 | 0.15 | 0.16 | 0.01 |
| 272582.03 | 8593639.23 | 316 | 0.17 | 0.17 | 0.01 |
| 272558.48 | 8593640.89 | 338 | 0.17 | 0.18 | 0.01 |
| 272573.45 | 8593580.14 | 262 | 0.15 | 0.14 | 0.01 |
| 272593.35 | 8593582.15 | 285 | 0.16 | 0.15 | 0.01 |
| 272627.77 | 8593580.58 | 250 | 0.14 | 0.13 | 0.01 |
| 272653.13 | 8593578.94 | 281 | 0.16 | 0.15 | 0.01 |
| 272682.08 | 8593581.02 | 278 | 0.15 | 0.15 | 0.01 |
| 272707.43 | 8593581.22 | 350 | 0.16 | 0.19 | 0.01 |
| 272732.78 | 8593581.43 | 323 | 0.17 | 0.17 | 0.01 |
| 272758.14 | 8593579.79 | 352 | 0.22 | 0.19 | 0.01 |
| 272779.86 | 8593579.96 | 423 | 0.19 | 0.23 | 0.01 |
| 272797.97 | 8593580.11 | 407 | 0.22 | 0.22 | 0.01 |
| 272816.07 | 8593580.25 | 447 | 0.24 | 0.25 | 0.01 |
| 272832.37 | 8593580.38 | 439 | 0.23 | 0.24 | 0.01 |
| 272850.47 | 8593580.53 | 504 | 0.23 | 0.28 | 0.01 |
| 272868.59 | 8593578.83 | 472 | 0.24 | 0.26 | 0.01 |
| 272886.68 | 8593580.82 | 311 | 0.21 | 0.17 | 0.01 |
| 272901.17 | 8593580.94 | 867 | 0.49 | 0.49 | 0.02 |
| 272933.77 | 8593579.35 | 369 | 0.22 | 0.20 | 0.01 |
| 272962.74 | 8593579.59 | 363 | 0.19 | 0.20 | 0.01 |
| 272984.46 | 8593579.76 | 431 | 0.22 | 0.24 | 0.01 |
| 273009.81 | 8593579.96 | 430 | 0.26 | 0.24 | 0.01 |
| 273027.92 | 8593580.11 | 459 | 0.26 | 0.25 | 0.01 |
| 273053.26 | 8593580.31 | 405 | 0.21 | 0.22 | 0.01 |
| 273074.99 | 8593580.49 | 357 | 0.19 | 0.19 | 0.01 |
| 273094.92 | 8593578.8 | 392 | 0.22 | 0.21 | 0.01 |
| 273120.25 | 8593580.85 | 335 | 0.19 | 0.18 | 0.01 |
| 273143.79 | 8593581.04 | 320 | 0.15 | 0.17 | 0.01 |
| 273167.33 | 8593581.23 | 298 | 0.18 | 0.16 | 0.01 |
| 273187.24 | 8593581.39 | 354 | 0.19 | 0.19 | 0.01 |
| 273212.59 | 8593581.59 | 371 | 0.22 | 0.20 | 0.01 |
| 273252.43 | 8593580.07 | 196 | 0.13 | 0.10 | 0.01 |
| 273281.4 | 8593580.3 | 180 | 0.07 | 0.09 | 0.01 |
| 272570.16 | 8593539.54 | 245 | 0.14 | 0.13 | 0.01 |
| 272590.07 | 8593539.7 | 248 | 0.14 | 0.13 | 0.01 |
| 272606.35 | 8593541.68 | 246 | 0.14 | 0.13 | 0.01 |
| 272631.73 | 8593538.19 | 285 | 0.14 | 0.15 | 0.01 |
| 272642.59 | 8593538.28 | 264 | 0.15 | 0.14 | 0.01 |
| 272630.07 | 8593519.74 | 323 | 0.16 | 0.17 | 0.01 |
| 272599.3 | 8593517.65 | 273 | 0.14 | 0.15 | 0.01 |
| 272577.55 | 8593521.16 | 232 | 0.15 | 0.12 | 0.01 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |
| 272557.63 | 8593521 | 248 | 0.15 | 0.13 | 0.01 |
| 272574.09 | 8593500.84 | 276 | 0.14 | 0.15 | 0.01 |
| 272599.44 | 8593501.05 | 273 | 0.14 | 0.15 | 0.01 |
| 272621.16 | 8593501.22 | 301 | 0.16 | 0.16 | 0.01 |
| 272635.65 | 8593501.34 | 304 | 0.21 | 0.16 | 0.01 |
| 272628.66 | 8593469.93 | 887 | 0.58 | 0.50 | 0.02 |
| 272614.17 | 8593469.81 | 760 | 0.42 | 0.43 | 0.02 |
| 272599.69 | 8593469.7 | 440 | 0.30 | 0.24 | 0.01 |
| 272577.96 | 8593469.52 | 367 | 0.19 | 0.20 | 0.01 |
| 272559.86 | 8593469.38 | 310 | 0.18 | 0.17 | 0.01 |
| 272572.71 | 8593447.35 | 439 | 0.23 | 0.24 | 0.01 |
| 272594.41 | 8593451.21 | 504 | 0.22 | 0.28 | 0.01 |
| 272616.15 | 8593449.54 | 752 | 0.45 | 0.42 | 0.02 |
| 272636.06 | 8593449.7 | 1066 | 0.62 | 0.61 | 0.02 |
| 272630.79 | 8593429.37 | 592 | 0.22 | 0.33 | 0.01 |
| 272610.86 | 8593431.06 | 572 | 0.31 | 0.32 | 0.01 |
| 272587.34 | 8593429.02 | 569 | 0.30 | 0.32 | 0.01 |
| 272563.79 | 8593430.68 | 537 | 0.30 | 0.30 | 0.01 |
| 272573.01 | 8593410.47 | 698 | 0.46 | 0.39 | 0.01 |
| 272591.11 | 8593410.61 | 821 | 0.43 | 0.46 | 0.02 |
| 272611.03 | 8593410.77 | 713 | 0.45 | 0.40 | 0.02 |
| 272629.34 | 8593385.1 | 689 | 0.35 | 0.39 | 0.01 |
| 272603.99 | 8593384.89 | 572 | 0.38 | 0.32 | 0.01 |
| 272585.89 | 8593384.75 | 498 | 0.30 | 0.28 | 0.01 |
| 272562.34 | 8593386.4 | 527 | 0.31 | 0.29 | 0.01 |
| 272575.23 | 8593358.84 | 488 | 0.25 | 0.27 | 0.01 |
| 272595.13 | 8593360.85 | 360 | 0.28 | 0.20 | 0.01 |
| 272615.06 | 8593359.16 | 472 | 0.24 | 0.26 | 0.01 |
| 272636.79 | 8593359.34 | 484 | 0.31 | 0.27 | 0.01 |
| 272638.76 | 8593339.07 | 395 | 0.22 | 0.21 | 0.01 |
| 272620.64 | 8593340.77 | 411 | 0.20 | 0.22 | 0.01 |
| 272602.54 | 8593340.62 | 416 | 0.20 | 0.22 | 0.01 |
| 272586.24 | 8593340.49 | 454 | 0.22 | 0.24 | 0.01 |
| 272559.09 | 8593340.27 | 359 | 0.19 | 0.19 | 0.01 |
| 272539.17 | 8593340.11 | 396 | 0.20 | 0.21 | 0.01 |
| 272512.01 | 8593339.89 | 407 | 0.23 | 0.22 | 0.01 |
| 272499.36 | 8593337.94 | 367 | 0.20 | 0.19 | 0.01 |
| 272481.24 | 8593339.64 | 359 | 0.20 | 0.19 | 0.01 |
| 272448.65 | 8593339.38 | 397 | 0.20 | 0.21 | 0.01 |
| 272428.72 | 8593341.06 | 392 | 0.20 | 0.21 | 0.01 |
| 272432.78 | 8593285.77 | 342 | 0.19 | 0.18 | 0.01 |
| 272452.71 | 8593284.08 | 432 | 0.22 | 0.23 | 0.01 |
| 272474.42 | 8593286.1 | 437 | 0.24 | 0.23 | 0.01 |
| 272501.6 | 8593284.48 | 411 | 0.23 | 0.22 | 0.01 |
| 272528.75 | 8593284.7 | 583 | 0.25 | 0.31 | 0.01 |
| 272552.29 | 8593284.89 | 631 | 0.25 | 0.34 | 0.01 |
| 272575.81 | 8593286.92 | 627 | 0.40 | 0.34 | 0.01 |
| 272602.98 | 8593285.29 | 443 | 0.23 | 0.24 | 0.01 |
| 272621.09 | 8593285.44 | 369 | 0.24 | 0.19 | 0.01 |
| 272642.81 | 8593285.61 | 360 | 0.18 | 0.19 | 0.01 |
| 272621.7 | 8593209.83 | 338 | 0.20 | 0.18 | 0.01 |
| 272668.77 | 8593210.21 | 426 | 0.22 | 0.23 | 0.01 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |
| 272650.67 | 8593210.06 | 402 | 0.22 | 0.21 | 0.01 |
| 272637.98 | 8593211.8 | 375 | 0.20 | 0.20 | 0.01 |
| 272605.4 | 8593209.7 | 352 | 0.20 | 0.19 | 0.01 |
| 272585.49 | 8593209.54 | 362 | 0.18 | 0.19 | 0.01 |
| 272563.75 | 8593211.21 | 334 | 0.19 | 0.18 | 0.01 |
| 272536.6 | 8593209.14 | 385 | 0.20 | 0.20 | 0.01 |
| 272516.67 | 8593210.83 | 391 | 0.20 | 0.21 | 0.01 |
| 272498.57 | 8593210.68 | 334 | 0.20 | 0.18 | 0.01 |
| 272478 | 8593209 | 315 | 0.18 | 0.16 | 0.01 |
| 272458.75 | 8593208.52 | 267 | 0.15 | 0.14 | 0.01 |
| 272437 | 8593212.03 | 312 | 0.17 | 0.16 | 0.01 |
| 272422.56 | 8593206.38 | 315 | 0.20 | 0.16 | 0.01 |
| 272550.97 | 8593449.02 | 380 | 0.20 | 0.20 | 0.01 |
| 272529.23 | 8593450.69 | 289 | 0.20 | 0.15 | 0.01 |
| 272556.71 | 8593410.33 | 545 | 0.25 | 0.29 | 0.01 |
| 272531.37 | 8593410.13 | 383 | 0.20 | 0.20 | 0.01 |
| 272509.64 | 8593409.96 | 351 | 0.21 | 0.18 | 0.01 |
| 272543.86 | 8593432.36 | 443 | 0.23 | 0.24 | 0.01 |
| 272522.16 | 8593428.5 | 453 | 0.22 | 0.24 | 0.01 |
| 272500.42 | 8593430.17 | 333 | 0.18 | 0.17 | 0.01 |
| 272533.38 | 8593384.33 | 352 | 0.13 | 0.19 | 0.01 |
| 272513.45 | 8593386.01 | 334 | 0.20 | 0.18 | 0.01 |
| 272493.55 | 8593384.01 | 325 | 0.16 | 0.17 | 0.01 |
| 272473.64 | 8593383.84 | 315 | 0.20 | 0.16 | 0.01 |
| 272466.17 | 8593411.45 | 351 | 0.20 | 0.18 | 0.01 |
| 272471.44 | 8593431.78 | 309 | 0.18 | 0.16 | 0.01 |
| 272478.52 | 8593452.12 | 247 | 0.12 | 0.13 | 0.01 |
| 272442.32 | 8593449.99 | 353 | 0.15 | 0.19 | 0.01 |
| 272440.68 | 8593429.69 | 260 | 0.15 | 0.13 | 0.01 |
| 272440.84 | 8593409.4 | 360 | 0.20 | 0.19 | 0.01 |
| 272411.87 | 8593409.17 | 279 | 0.15 | 0.14 | 0.01 |
| 272433.81 | 8593383.52 | 359 | 0.20 | 0.19 | 0.01 |
| 272406.65 | 8593383.31 | 327 | 0.18 | 0.17 | 0.01 |
| 272384.91 | 8593384.97 | 313 | 0.18 | 0.16 | 0.01 |
| 272390.55 | 8593359.2 | 290 | 0.15 | 0.15 | 0.01 |
| 272417.71 | 8593359.42 | 348 | 0.18 | 0.18 | 0.01 |
| 272448.48 | 8593359.67 | 317 | 0.18 | 0.17 | 0.01 |
| 272479.26 | 8593359.91 | 336 | 0.19 | 0.18 | 0.01 |
| 272513.66 | 8593360.19 | 466 | 0.26 | 0.25 | 0.01 |
| 272551.68 | 8593360.5 | 463 | 0.25 | 0.25 | 0.01 |
| 272675.17 | 8593090.38 | 260 | 0.15 | 0.13 | 0.01 |
| 272662.49 | 8593090.28 | 292 | 0.15 | 0.15 | 0.01 |
| 272642.58 | 8593090.12 | 999 | 0.60 | 0.55 | 0.02 |
| 272626.28 | 8593089.99 | 366 | 0.26 | 0.19 | 0.01 |
| 272586.47 | 8593087.82 | 397 | 0.20 | 0.21 | 0.01 |
| 272597.32 | 8593089.75 | 395 | 0.25 | 0.21 | 0.01 |
| 272561.11 | 8593089.46 | 898 | 0.50 | 0.49 | 0.02 |
| 272541.18 | 8593091.14 | 673 | 0.42 | 0.36 | 0.01 |
| 272517.64 | 8593090.95 | 693 | 0.40 | 0.38 | 0.01 |
| 272495.91 | 8593090.78 | 921 | 0.50 | 0.50 | 0.02 |
| 272479.64 | 8593088.8 | 635 | 0.45 | 0.34 | 0.01 |
| 272461.52 | 8593090.5 | 652 | 0.35 | 0.35 | 0.01 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |
| 272430.72 | 8593092.1 | 320 | 0.20 | 0.17 | 0.01 |
| 272414.43 | 8593091.97 | 272 | 0.16 | 0.14 | 0.01 |
| 272390.91 | 8593089.93 | 192 | 0.13 | 0.10 | 0.01 |
| 272378.25 | 8593087.99 | 188 | 0.12 | 0.09 | 0.01 |
| 272405.88 | 8593029.19 | 195 | 0.13 | 0.10 | 0.01 |
| 272440.27 | 8593031.31 | 190 | 0.10 | 0.09 | 0.01 |
| 272456.57 | 8593029.6 | 192 | 0.10 | 0.10 | 0.01 |
| 272476.5 | 8593027.92 | 183 | 0.11 | 0.09 | 0.01 |
| 272505.46 | 8593029.99 | 231 | 0.14 | 0.12 | 0.01 |
| 272525.39 | 8593028.31 | 229 | 0.10 | 0.12 | 0.01 |
| 272557.98 | 8593028.57 | 251 | 0.13 | 0.13 | 0.01 |
| 272588.75 | 8593028.82 | 201 | 0.10 | 0.10 | 0.01 |
| 272612.27 | 8593030.85 | 194 | 0.10 | 0.10 | 0.01 |
| 272630.39 | 8593029.16 | 201 | 0.10 | 0.10 | 0.01 |
| 272648.5 | 8593029.3 | 180 | 0.10 | 0.09 | 0.01 |
| 272661.17 | 8593029.4 | 192 | 0.11 | 0.10 | 0.01 |
| 272572.64 | 8593681.57 | 288 | 0.20 | 0.15 | 0.01 |
| 272592.63 | 8593672.51 | 306 | 0.20 | 0.16 | 0.01 |
| 272625 | 8593680 | 326 | 0.20 | 0.17 | 0.01 |
| 272661 | 8593680 | 247 | 0.15 | 0.13 | 0.01 |
| 272684.9 | 8593680.63 | 318 | 0.20 | 0.16 | 0.01 |
| 272701.21 | 8593678.92 | 301 | 0.20 | 0.16 | 0.01 |
| 272731.96 | 8593682.86 | 316 | 0.20 | 0.16 | 0.01 |
| 272759.15 | 8593679.39 | 281 | 0.20 | 0.14 | 0.01 |
| 272775.44 | 8593679.52 | 312 | 0.20 | 0.16 | 0.01 |
| 272808.03 | 8593679.78 | 314 | 0.15 | 0.16 | 0.01 |
| 272844.23 | 8593681.92 | 298 | 0.20 | 0.15 | 0.01 |
| 272871.4 | 8593680.29 | 309 | 0.20 | 0.16 | 0.01 |
| 272909.42 | 8593680.59 | 332 | 0.20 | 0.17 | 0.01 |
| 272936.59 | 8593678.97 | 369 | 0.20 | 0.19 | 0.01 |
| 272967.36 | 8593681.06 | 352 | 0.20 | 0.18 | 0.01 |
| 273005.39 | 8593679.52 | 353 | 0.20 | 0.18 | 0.01 |
| 273037.97 | 8593681.63 | 376 | 0.20 | 0.20 | 0.01 |
| 273066.95 | 8593680.02 | 373 | 0.20 | 0.20 | 0.01 |
| 273092.3 | 8593680.22 | 419 | 0.25 | 0.22 | 0.01 |
| 273123.06 | 8593682.31 | 437 | 0.25 | 0.23 | 0.01 |
| 273150.23 | 8593680.68 | 505 | 0.25 | 0.27 | 0.01 |
| 273186.44 | 8593680.98 | 378 | 0.20 | 0.20 | 0.01 |
| 273204.55 | 8593681.12 | 431 | 0.20 | 0.23 | 0.01 |
| 273226.27 | 8593681.3 | 308 | 0.20 | 0.16 | 0.01 |
| 273246.2 | 8593679.61 | 339 | 0.20 | 0.18 | 0.01 |
| 273262.5 | 8593679.74 | 322 | 0.15 | 0.17 | 0.01 |
| 273287.84 | 8593679.94 | 325 | 0.15 | 0.17 | 0.01 |
| 273266.61 | 8593618.91 | 472 | 0.30 | 0.25 | 0.01 |
| 273232.19 | 8593620.48 | 368 | 0.20 | 0.19 | 0.01 |
| 273194.17 | 8593620.18 | 324 | 0.20 | 0.17 | 0.01 |
| 273159.77 | 8593619.9 | 342 | 0.20 | 0.18 | 0.01 |
| 273128.99 | 8593619.65 | 391 | 0.20 | 0.21 | 0.01 |
| 273105.44 | 8593621.31 | 368 | 0.20 | 0.19 | 0.01 |
| 273071.06 | 8593619.19 | 377 | 0.20 | 0.20 | 0.01 |
| 273052.94 | 8593620.89 | 363 | 0.20 | 0.19 | 0.01 |
| 273033.04 | 8593618.88 | 381 | 0.20 | 0.20 | 0.01 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |
| 273016.73 | 8593620.6 | 436 | 0.22 | 0.23 | 0.01 |
| 273004.07 | 8593618.65 | 410 | 0.20 | 0.22 | 0.01 |
| 272989.57 | 8593620.38 | 421 | 0.23 | 0.22 | 0.01 |
| 272960.6 | 8593620.14 | 415 | 0.22 | 0.22 | 0.01 |
| 272926.2 | 8593619.87 | 389 | 0.22 | 0.20 | 0.01 |
| 272893.61 | 8593619.61 | 423 | 0.20 | 0.22 | 0.01 |
| 272864.65 | 8593619.37 | 371 | 0.15 | 0.19 | 0.01 |
| 272833.87 | 8593619.13 | 389 | 0.20 | 0.20 | 0.01 |
| 272803.07 | 8593620.72 | 351 | 0.20 | 0.18 | 0.01 |
| 272770.5 | 8593618.62 | 358 | 0.20 | 0.19 | 0.01 |
| 272746.95 | 8593620.27 | 366 | 0.20 | 0.19 | 0.01 |
| 272730.65 | 8593620.14 | 310 | 0.20 | 0.16 | 0.01 |
| 272703.5 | 8593619.92 | 314 | 0.20 | 0.16 | 0.01 |
| 272687.2 | 8593619.79 | 287 | 0.20 | 0.15 | 0.01 |
| 272665.48 | 8593619.62 | 329 | 0.20 | 0.17 | 0.01 |
| 272641.94 | 8593619.43 | 278 | 0.20 | 0.14 | 0.01 |
| 272611.16 | 8593619.18 | 320 | 0.20 | 0.17 | 0.01 |
| 272576.76 | 8593618.9 | 318 | 0.20 | 0.16 | 0.01 |
| 272558.64 | 8593620.6 | 325 | 0.20 | 0.17 | 0.01 |
| 272431.11 | 8593269.16 | 413 | 0.22 | 0.22 | 0.01 |
| 272452.83 | 8593269.33 | 498 | 0.27 | 0.27 | 0.01 |
| 272476.37 | 8593269.52 | 568 | 0.32 | 0.31 | 0.01 |
| 272501.72 | 8593269.72 | 590 | 0.32 | 0.32 | 0.01 |
| 272523.44 | 8593269.9 | 494 | 0.30 | 0.26 | 0.01 |
| 272548.79 | 8593270.1 | 578 | 0.38 | 0.31 | 0.01 |
| 272572.32 | 8593270.29 | 499 | 0.30 | 0.27 | 0.01 |
| 272595.88 | 8593268.64 | 406 | 0.24 | 0.22 | 0.01 |
| 272615.78 | 8593270.64 | 378 | 0.22 | 0.20 | 0.01 |
| 272642.95 | 8593269.02 | 418 | 0.22 | 0.22 | 0.01 |
| 272628.78 | 8593230.17 | 364 | 0.20 | 0.19 | 0.01 |
| 272668.61 | 8593230.49 | 393 | 0.24 | 0.21 | 0.01 |
| 272648.69 | 8593230.33 | 410 | 0.22 | 0.22 | 0.01 |
| 272637.83 | 8593230.25 | 381 | 0.20 | 0.20 | 0.01 |
| 272605.24 | 8593229.98 | 357 | 0.22 | 0.19 | 0.01 |
| 272583.51 | 8593229.81 | 377 | 0.22 | 0.20 | 0.01 |
| 272565.39 | 8593231.51 | 293 | 0.18 | 0.15 | 0.01 |
| 272545.49 | 8593229.5 | 408 | 0.22 | 0.22 | 0.01 |
| 272520.15 | 8593229.3 | 424 | 0.28 | 0.23 | 0.01 |
| 272498.41 | 8593230.97 | 455 | 0.26 | 0.24 | 0.01 |
| 272478.49 | 8593230.81 | 518 | 0.30 | 0.28 | 0.01 |
| 272456.78 | 8593228.79 | 420 | 0.25 | 0.22 | 0.01 |
| 272435.04 | 8593230.46 | 368 | 0.23 | 0.19 | 0.01 |
| 272418.75 | 8593230.33 | 386 | 0.22 | 0.20 | 0.01 |
| 272524.42 | 8593148.18 | 1224 | 0.80 | 0.67 | 0.02 |
| 272509.94 | 8593148.07 | 1065 | 0.60 | 0.58 | 0.02 |
| 272486.39 | 8593149.72 | 785 | 0.48 | 0.43 | 0.02 |
| 272461.03 | 8593151.36 | 728 | 0.40 | 0.40 | 0.01 |
| 272439.31 | 8593149.34 | 602 | 0.32 | 0.33 | 0.01 |
| 272415.78 | 8593149.15 | 594 | 0.35 | 0.32 | 0.01 |
| 272538.89 | 8593150.14 | 1060 | 0.60 | 0.58 | 0.02 |
| 272560.62 | 8593150.32 | 1037 | 0.59 | 0.57 | 0.02 |
| 272580.53 | 8593150.48 | 1254 | 0.68 | 0.69 | 0.02 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |
| 272600.45 | 8593150.64 | 1996 | 1.20 | 1.11 | 0.02 |
| 272620.38 | 8593148.96 | 2454 | 1.40 | 1.36 | 0.03 |
| 272642.1 | 8593149.13 | 3748 | 2.50 | 2.09 | 0.03 |
| 272660.19 | 8593151.12 | 3445 | 2.00 | 1.92 | 0.03 |
| 272683.73 | 8593151.31 | 807 | 0.45 | 0.44 | 0.02 |
| 272700.04 | 8593149.6 | 574 | 0.28 | 0.31 | 0.01 |
| 272721.76 | 8593149.77 | 373 | 0.18 | 0.20 | 0.01 |
| 272741.68 | 8593149.93 | 298 | 0.18 | 0.15 | 0.01 |
| 272763.4 | 8593150.11 | 308 | 0.16 | 0.16 | 0.01 |
| 272761.74 | 8593131.65 | 221 | 0.10 | 0.12 | 0.01 |
| 272741.84 | 8593129.65 | 241 | 0.12 | 0.13 | 0.01 |
| 272721.93 | 8593129.49 | 237 | 0.12 | 0.12 | 0.01 |
| 272698.38 | 8593131.14 | 348 | 0.17 | 0.19 | 0.01 |
| 272678.48 | 8593129.14 | 678 | 0.35 | 0.38 | 0.01 |
| 272658.55 | 8593130.82 | 1450 | 0.80 | 0.83 | 0.02 |
| 272645.89 | 8593128.87 | 3640 | 2.60 | 2.10 | 0.03 |
| 272631.39 | 8593130.6 | 2534 | 1.40 | 1.46 | 0.03 |
| 272611.49 | 8593128.6 | 2130 | 1.20 | 1.22 | 0.03 |
| 272591.56 | 8593130.28 | 1456 | 0.80 | 0.83 | 0.02 |
| 272569.83 | 8593130.11 | 1177 | 0.70 | 0.67 | 0.02 |
| 272549.92 | 8593129.95 | 1157 | 0.65 | 0.66 | 0.02 |
| 272530 | 8593129.78 | 999 | 0.56 | 0.57 | 0.02 |
| 272504.65 | 8593129.58 | 882 | 0.45 | 0.50 | 0.02 |
| 272484.74 | 8593129.42 | 737 | 0.45 | 0.41 | 0.02 |
| 272464.82 | 8593129.26 | 923 | 0.50 | 0.52 | 0.02 |
| 272446.72 | 8593129.11 | 761 | 0.45 | 0.43 | 0.02 |
| 272423.17 | 8593130.77 | 633 | 0.38 | 0.35 | 0.01 |
| 272403.25 | 8593130.61 | 472 | 0.26 | 0.26 | 0.01 |
| 272383.34 | 8593130.45 | 226 | 0.14 | 0.12 | 0.01 |
| 272361.61 | 8593130.27 | 198 | 0.10 | 0.10 | 0.01 |
| 272382.03 | 8593067.73 | 204 | 0.10 | 0.11 | 0.01 |
| 272401.92 | 8593071.58 | 209 | 0.10 | 0.11 | 0.01 |
| 272420.04 | 8593069.88 | 192 | 0.12 | 0.10 | 0.01 |
| 272443.57 | 8593070.07 | 254 | 0.13 | 0.13 | 0.01 |
| 272468.92 | 8593070.28 | 278 | 0.15 | 0.15 | 0.01 |
| 272487.03 | 8593070.42 | 413 | 0.20 | 0.23 | 0.01 |
| 272512.37 | 8593070.63 | 624 | 0.35 | 0.35 | 0.01 |
| 272534.1 | 8593070.8 | 758 | 0.45 | 0.43 | 0.02 |
| 272552.2 | 8593070.95 | 813 | 0.50 | 0.46 | 0.02 |
| 272573.94 | 8593069.28 | 376 | 0.18 | 0.21 | 0.01 |
| 272595.65 | 8593071.3 | 336 | 0.18 | 0.18 | 0.01 |
| 272617.39 | 8593069.63 | 351 | 0.17 | 0.19 | 0.01 |
| 272639.11 | 8593071.65 | 3008 | 1.80 | 1.73 | 0.03 |
| 272659.04 | 8593069.96 | 232 | 0.12 | 0.12 | 0.01 |
| 272680.76 | 8593070.14 | 167 | 0.11 | 0.08 | 0.01 |
| 272567.38 | 8593659.4 | 328 | 0.10 | 0.17 | 0.01 |
| 272587.28 | 8593661.41 | 287 | 0.08 | 0.15 | 0.01 |
| 272614.45 | 8593659.78 | 308 | 0.11 | 0.16 | 0.01 |
| 272632.56 | 8593659.93 | 276 | 0.11 | 0.14 | 0.01 |
| 272657.92 | 8593658.29 | 280 | 0.11 | 0.14 | 0.01 |
| 272683.25 | 8593660.33 | 263 | 0.07 | 0.13 | 0.01 |
| 272710.41 | 8593660.55 | 284 | 0.10 | 0.15 | 0.01 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |
| 272735.76 | 8593660.76 | 282 | 0.11 | 0.14 | 0.01 |
| 272766.54 | 8593661 | 285 | 0.11 | 0.15 | 0.01 |
| 272791.9 | 8593659.36 | 297 | 0.13 | 0.15 | 0.01 |
| 272824.49 | 8593659.62 | 324 | 0.12 | 0.17 | 0.01 |
| 272853.46 | 8593659.86 | 296 | 0.11 | 0.15 | 0.01 |
| 272886.05 | 8593660.12 | 308 | 0.13 | 0.16 | 0.01 |
| 272907.77 | 8593660.29 | 361 | 0.15 | 0.19 | 0.01 |
| 272936.74 | 8593660.53 | 390 | 0.15 | 0.21 | 0.01 |
| 272953.05 | 8593658.81 | 390 | 0.16 | 0.21 | 0.01 |
| 272978.38 | 8593660.86 | 400 | 0.16 | 0.21 | 0.01 |
| 273005.55 | 8593659.24 | 400 | 0.19 | 0.21 | 0.01 |
| 273029.09 | 8593659.42 | 373 | 0.17 | 0.20 | 0.01 |
| 273047.2 | 8593659.57 | 316 | 0.12 | 0.16 | 0.01 |
| 273070.73 | 8593659.76 | 381 | 0.14 | 0.20 | 0.01 |
| 273096.09 | 8593658.12 | 329 | 0.13 | 0.17 | 0.01 |
| 273128.67 | 8593660.22 | 353 | 0.03 | 0.18 | 0.01 |
| 273152.2 | 8593660.41 | 313 | 0.10 | 0.16 | 0.01 |
| 273175.74 | 8593660.6 | 264 | 0.10 | 0.13 | 0.01 |
| 273199.28 | 8593660.79 | 307 | 0.11 | 0.16 | 0.01 |
| 273222.83 | 8593659.14 | 299 | 0.11 | 0.15 | 0.01 |
| 273246.37 | 8593659.32 | 283 | 0.13 | 0.15 | 0.01 |
| 273269.9 | 8593659.51 | 304 | 0.15 | 0.16 | 0.01 |
| 273275.81 | 8593600.54 | 223 | 0.11 | 0.11 | 0.01 |
| 273246.84 | 8593600.31 | 450 | 0.17 | 0.24 | 0.01 |
| 273230.54 | 8593600.18 | 438 | 0.15 | 0.23 | 0.01 |
| 273208.82 | 8593600.01 | 349 | 0.14 | 0.18 | 0.01 |
| 273183.47 | 8593599.8 | 323 | 0.14 | 0.17 | 0.01 |
| 273147.25 | 8593601.36 | 390 | 0.14 | 0.21 | 0.01 |
| 273118.29 | 8593599.28 | 342 | 0.15 | 0.18 | 0.01 |
| 273098.38 | 8593599.12 | 436 | 0.15 | 0.23 | 0.01 |
| 273073.02 | 8593600.76 | 398 | 0.14 | 0.21 | 0.01 |
| 273049.48 | 8593600.57 | 374 | 0.13 | 0.20 | 0.01 |
| 273020.51 | 8593600.34 | 345 | 0.13 | 0.18 | 0.01 |
| 272993.35 | 8593600.12 | 401 | 0.14 | 0.21 | 0.01 |
| 272969.82 | 8593599.93 | 384 | 0.16 | 0.20 | 0.01 |
| 272948.09 | 8593599.76 | 347 | 0.12 | 0.18 | 0.01 |
| 272929.99 | 8593599.61 | 369 | 0.16 | 0.19 | 0.01 |
| 272904.64 | 8593599.41 | 453 | 0.15 | 0.24 | 0.01 |
| 272879.29 | 8593599.2 | 438 | 0.16 | 0.23 | 0.01 |
| 272857.57 | 8593599.03 | 437 | 0.14 | 0.23 | 0.01 |
| 272826.77 | 8593600.63 | 372 | 0.15 | 0.20 | 0.01 |
| 272801.43 | 8593600.42 | 342 | 0.14 | 0.18 | 0.01 |
| 272783.32 | 8593600.28 | 340 | 0.15 | 0.18 | 0.01 |
| 272759.78 | 8593600.09 | 334 | 0.10 | 0.17 | 0.01 |
| 272732.63 | 8593599.87 | 315 | 0.12 | 0.16 | 0.01 |
| 272714.51 | 8593601.57 | 294 | 0.10 | 0.15 | 0.01 |
| 272685.55 | 8593599.49 | 280 | 0.08 | 0.14 | 0.01 |
| 272658.4 | 8593599.27 | 260 | 0.10 | 0.13 | 0.01 |
| 272629.43 | 8593599.04 | 255 | 0.09 | 0.13 | 0.01 |
| 272611.31 | 8593600.74 | 274 | 0.09 | 0.14 | 0.01 |
| 272598.64 | 8593600.63 | 265 | 0.08 | 0.14 | 0.01 |
| 272578.73 | 8593598.63 | 276 | 0.09 | 0.14 | 0.01 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |
| 272564.25 | 8593598.51 | 306 | 0.08 | 0.16 | 0.01 |
| 272577.24 | 8593559.89 | 270 | 0.10 | 0.14 | 0.01 |
| 272595.34 | 8593560.03 | 259 | 0.12 | 0.13 | 0.01 |
| 272617.07 | 8593560.21 | 274 | 0.14 | 0.14 | 0.01 |
| 272640.6 | 8593560.4 | 287 | 0.13 | 0.15 | 0.01 |
| 272667.76 | 8593560.62 | 298 | 0.15 | 0.15 | 0.01 |
| 272698.54 | 8593560.86 | 387 | 0.17 | 0.20 | 0.01 |
| 272731.13 | 8593561.13 | 365 | 0.16 | 0.19 | 0.01 |
| 272752.87 | 8593559.46 | 413 | 0.20 | 0.22 | 0.01 |
| 272776.39 | 8593561.49 | 354 | 0.18 | 0.19 | 0.01 |
| 272801.75 | 8593559.85 | 405 | 0.21 | 0.21 | 0.01 |
| 272823.48 | 8593560.02 | 399 | 0.20 | 0.21 | 0.01 |
| 272854.26 | 8593560.27 | 413 | 0.17 | 0.22 | 0.01 |
| 272868.74 | 8593560.39 | 495 | 0.20 | 0.26 | 0.01 |
| 272895.91 | 8593558.76 | 568 | 0.23 | 0.30 | 0.01 |
| 272923.06 | 8593560.82 | 481 | 0.22 | 0.26 | 0.01 |
| 272946.59 | 8593561.01 | 433 | 0.20 | 0.23 | 0.01 |
| 272970.14 | 8593559.36 | 437 | 0.22 | 0.23 | 0.01 |
| 272995.49 | 8593559.56 | 593 | 0.28 | 0.32 | 0.01 |
| 273017.22 | 8593559.74 | 647 | 0.32 | 0.35 | 0.01 |
| 273033.51 | 8593559.87 | 645 | 0.32 | 0.35 | 0.01 |
| 273057.05 | 8593560.06 | 471 | 0.21 | 0.25 | 0.01 |
| 273086.02 | 8593560.29 | 442 | 0.22 | 0.23 | 0.01 |
| 273104.14 | 8593558.59 | 449 | 0.28 | 0.24 | 0.01 |
| 273129.47 | 8593560.64 | 342 | 0.19 | 0.18 | 0.01 |
| 273149.38 | 8593560.8 | 350 | 0.16 | 0.18 | 0.01 |
| 273171.12 | 8593559.13 | 305 | 0.16 | 0.16 | 0.01 |
| 273198.27 | 8593561.19 | 258 | 0.08 | 0.13 | 0.01 |
| 273225.42 | 8593561.41 | 196 | 0.09 | 0.10 | 0.01 |
| 273238.11 | 8593559.67 | 180 | 0.06 | 0.09 | 0.01 |
| 273258.03 | 8593559.83 | 198 | 0.06 | 0.10 | 0.01 |
| 273279.74 | 8593561.84 | 184 | 0.07 | 0.09 | 0.01 |
| 272640.81 | 8593309.57 | 404 | 0.20 | 0.21 | 0.01 |
| 272619 | 8593311 | 389 | 0.20 | 0.20 | 0.01 |
| 272601 | 8593310 | 479 | 0.23 | 0.25 | 0.01 |
| 272579.25 | 8593309.08 | 502 | 0.20 | 0.27 | 0.01 |
| 272557.53 | 8593308.9 | 508 | 0.30 | 0.27 | 0.01 |
| 272535.8 | 8593308.73 | 517 | 0.28 | 0.28 | 0.01 |
| 272518 | 8593312 | 526 | 0.30 | 0.28 | 0.01 |
| 272499.61 | 8593306.59 | 438 | 0.25 | 0.23 | 0.01 |
| 272474.22 | 8593311.92 | 394 | 0.18 | 0.21 | 0.01 |
| 272454.3 | 8593311.76 | 411 | 0.17 | 0.22 | 0.01 |
| 272427.13 | 8593313.39 | 441 | 0.20 | 0.23 | 0.01 |
| 272427.62 | 8593252.53 | 423 | 0.18 | 0.22 | 0.01 |
| 272449.36 | 8593250.86 | 457 | 0.25 | 0.24 | 0.01 |
| 272472.91 | 8593249.2 | 504 | 0.25 | 0.27 | 0.01 |
| 272500.05 | 8593251.27 | 574 | 0.30 | 0.31 | 0.01 |
| 272521.78 | 8593251.44 | 754 | 0.45 | 0.41 | 0.01 |
| 272545.3 | 8593253.48 | 547 | 0.30 | 0.29 | 0.01 |
| 272568.85 | 8593251.82 | 504 | 0.28 | 0.27 | 0.01 |
| 272592.37 | 8593253.86 | 432 | 0.21 | 0.23 | 0.01 |
| 272612.32 | 8593250.33 | 356 | 0.15 | 0.19 | 0.01 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |
| 272635.84 | 8593252.36 | 354 | 0.15 | 0.19 | 0.01 |
| 272614.63 | 8593187.64 | 511 | 0.30 | 0.27 | 0.01 |
| 272668.93 | 8593189.92 | 557 | 0.35 | 0.30 | 0.01 |
| 272645.41 | 8593187.89 | 736 | 0.50 | 0.40 | 0.01 |
| 272625.47 | 8593191.42 | 584 | 0.25 | 0.31 | 0.01 |
| 272598.32 | 8593189.35 | 416 | 0.20 | 0.22 | 0.01 |
| 272580.22 | 8593189.21 | 418 | 0.18 | 0.22 | 0.01 |
| 272560.3 | 8593189.05 | 351 | 0.15 | 0.18 | 0.01 |
| 272540.37 | 8593190.73 | 416 | 0.15 | 0.22 | 0.01 |
| 272515.04 | 8593188.68 | 471 | 0.20 | 0.25 | 0.01 |
| 272496.91 | 8593192.22 | 368 | 0.15 | 0.19 | 0.01 |
| 272480.66 | 8593186.56 | 345 | 0.15 | 0.18 | 0.01 |
| 272458.9 | 8593190.07 | 262 | 0.15 | 0.13 | 0.01 |
| 272433.56 | 8593189.87 | 346 | 0.15 | 0.18 | 0.01 |
| 272533.31 | 8593168.54 | 617 | 0.40 | 0.33 | 0.01 |
| 272511.6 | 8593166.52 | 706 | 0.40 | 0.38 | 0.01 |
| 272488.06 | 8593166.33 | 584 | 0.30 | 0.31 | 0.01 |
| 272473.56 | 8593168.06 | 631 | 0.30 | 0.34 | 0.01 |
| 272450.01 | 8593169.72 | 533 | 0.25 | 0.28 | 0.01 |
| 272421.05 | 8593169.48 | 499 | 0.20 | 0.27 | 0.01 |
| 272392.14 | 8593161.87 | 524 | 0.20 | 0.28 | 0.01 |
| 272551.45 | 8593165 | 617 | 0.30 | 0.33 | 0.01 |
| 272569.49 | 8593172.52 | 545 | 0.22 | 0.29 | 0.01 |
| 272591.25 | 8593169.01 | 596 | 0.22 | 0.32 | 0.01 |
| 272609.34 | 8593171 | 714 | 0.35 | 0.39 | 0.01 |
| 272632.89 | 8593169.34 | 926 | 0.50 | 0.50 | 0.02 |
| 272647.44 | 8593160.24 | 3365 | 2.00 | 1.86 | 0.03 |
| 272672.7 | 8593171.51 | 964 | 0.40 | 0.52 | 0.02 |
| 272692.63 | 8593169.82 | 893 | 0.40 | 0.48 | 0.02 |
| 272710.74 | 8593169.97 | 505 | 0.20 | 0.27 | 0.01 |
| 272730.65 | 8593170.13 | 409 | 0.15 | 0.22 | 0.01 |
| 272746.95 | 8593170.26 | 400 | 0.18 | 0.21 | 0.01 |
| 272768.67 | 8593170.44 | 311 | 0.15 | 0.16 | 0.01 |
| 272760.12 | 8593107.66 | 211 | 0.10 | 0.11 | 0.01 |
| 272740.19 | 8593109.35 | 218 | 0.10 | 0.11 | 0.01 |
| 272720.28 | 8593109.18 | 218 | 0.10 | 0.11 | 0.01 |
| 272700.36 | 8593109.02 | 231 | 0.10 | 0.12 | 0.01 |
| 272680.45 | 8593108.86 | 307 | 0.15 | 0.16 | 0.01 |
| 272660.52 | 8593110.55 | 611 | 0.20 | 0.33 | 0.01 |
| 272643 | 8593107 | 807 | 0.60 | 0.44 | 0.02 |
| 272622.48 | 8593112.09 | 903 | 0.35 | 0.49 | 0.02 |
| 272600.77 | 8593110.07 | 1276 | 0.70 | 0.70 | 0.02 |
| 272579.05 | 8593109.89 | 1006 | 0.50 | 0.55 | 0.02 |
| 272559.12 | 8593111.58 | 980 | 0.50 | 0.53 | 0.02 |
| 272539.22 | 8593109.57 | 902 | 0.40 | 0.49 | 0.02 |
| 272519.27 | 8593113.1 | 900 | 0.45 | 0.49 | 0.02 |
| 272503.02 | 8593107.44 | 648 | 0.35 | 0.35 | 0.01 |
| 272479.46 | 8593110.93 | 659 | 0.23 | 0.35 | 0.01 |
| 272455.95 | 8593107.06 | 646 | 0.20 | 0.35 | 0.01 |
| 272430.59 | 8593108.7 | 684 | 0.25 | 0.37 | 0.01 |
| 272407.05 | 8593108.51 | 293 | 0.13 | 0.15 | 0.01 |
| 272381.72 | 8593106.46 | 213 | 0.10 | 0.11 | 0.01 |
| UTM Eastings | UTM Northings | counts/ 100s | uGy/hr meas | uGy/hr calc | +- |
| 272385.85 | 8593043.79 | 192 | 0.10 | 0.09 | 0.01 |
| 272402.07 | 8593053.14 | 207 | 0.10 | 0.10 | 0.01 |
| 272420.19 | 8593051.44 | 215 | 0.10 | 0.11 | 0.01 |
| 272441.93 | 8593049.77 | 193 | 0.10 | 0.10 | 0.01 |
| 272461.86 | 8593048.09 | 215 | 0.10 | 0.11 | 0.01 |
| 272485.35 | 8593053.81 | 201 | 0.10 | 0.10 | 0.01 |
| 272508.88 | 8593054 | 255 | 0.10 | 0.13 | 0.01 |
| 272530.63 | 8593052.33 | 386 | 0.15 | 0.20 | 0.01 |
| 272550.53 | 8593054.33 | 451 | 0.18 | 0.24 | 0.01 |
| 272577.73 | 8593049.02 | 283 | 0.10 | 0.15 | 0.01 |
| 272601.22 | 8593054.74 | 233 | 0.15 | 0.12 | 0.01 |
| 272621.18 | 8593049.37 | 198 | 0.10 | 0.10 | 0.01 |
| 272641.09 | 8593049.53 | 206 | 0.10 | 0.10 | 0.01 |
| 272660.95 | 8593057.07 | 194 | 0.10 | 0.10 | 0.01 |
| 272682.75 | 8593048.02 | 176 | 0.10 | 0.09 | 0.01 |

# Appendix B  Radon flux density

Appendix B  Radon flux density results for individual charcoal canisters deployed during Anomaly 2 survey conducted 14–17 July 2009.

| Can # | site | Easting | Northing | Deployed Date/time | Retrieved Date/time | Rn [mBq.m-2.s-1] | ave±stdev geomean | Soil type |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 100 | 1A | 272968 | 8593810 | 14/07/2009 10:45 | 17/07/2009 10:37 | 252 ± 4 | 249±6 249 | sandy loam |
| 70 | 1A | 272968 | 8593810 | 14/07/2009 10:45 | 17/07/2009 10:37 | 253 ± 4 |  |  |
| 97 | 1A | 272968 | 8593810 | 14/07/2009 10:45 | 17/07/2009 10:37 | 241 ± 4 |  |  |
| 87 | 2A | 273084 | 8593837 | 14/07/2009 11:07 | 17/07/2009 10:41 | 818 ± 7 | 694±411 583 | fine gravel |
| 78 | 2A | 273084 | 8593837 | 14/07/2009 11:07 | 17/07/2009 10:41 | 235 ± 4 |  |  |
| 92 | 2A | 273084 | 8593837 | 14/07/2009 11:07 | 17/07/2009 10:41 | 1029 ± 7 |  |  |
| 88 | 3A | 273106 | 8593792 | 14/07/2009 11:25 | 17/07/2009 10:45 | 926 ± 7 | 1046±153 1039 | fine gravel |
| 68 | 3A | 273106 | 8593792 | 14/07/2009 11:25 | 17/07/2009 10:45 | 1219 ± 8 |  |  |
| 98 | 3A | 273106 | 8593792 | 14/07/2009 11:25 | 17/07/2009 10:45 | 992 ± 7 |  |  |
| 2E | 4A | 273141 | 8593890 | 14/07/2009 11:37 | 17/07/2009 10:48 | 771 ± 7 | 760±10 760 | fine gravel |
| 10E | 4A | 273141 | 8593890 | 14/07/2009 11:37 | 17/07/2009 10:48 | 759 ± 7 |  |  |
| 80 | 4A | 273141 | 8593890 | 14/07/2009 11:37 | 17/07/2009 10:48 | 750 ± 6 |  |  |
| 19E | 5A | 273273 | 8593846 | 14/07/2009 11:45 | 17/07/2009 10:50 | 751 ± 7 | 866±104 861 | sandy loam |
| 22E | 5A | 273273 | 8593846 | 14/07/2009 11:45 | 17/07/2009 10:50 | 894 ± 7 |  |  |
| 37E | 5A | 273273 | 8593846 | 14/07/2009 11:45 | 17/07/2009 10:50 | 952 ± 7 |  |  |
| 38E | 6A | 273263 | 8593696 | 14/07/2009 11:54 | 17/07/2009 10:54 | 271 ± 4 | 290±48 288 | sandy loam |
| 83 | 6A | 273263 | 8593696 | 14/07/2009 11:54 | 17/07/2009 10:54 | 254 ± 4 |  |  |
| 24E | 6A | 273263 | 8593696 | 14/07/2009 11:54 | 17/07/2009 10:54 | 345 ± 5 |  |  |
| 6E | 7A | 273200 | 8593597 | 14/07/2009 12:00 | 17/07/2009 10:55 | 864 ± 7 | 918±49 917 | sandy loam |
| 69 | 7A | 273200 | 8593597 | 14/07/2009 12:00 | 17/07/2009 10:55 | 959 ± 7 |  |  |
| 73 | 7A | 273200 | 8593597 | 14/07/2009 12:00 | 17/07/2009 10:55 | 932 ± 7 |  |  |
| 7E | 8A | 272988 | 8593601 | 14/07/2009 12:07 | 17/07/2009 10:58 | 209 ± 4 | 222±13 222 | fine gravel |
| 61 | 8A | 272988 | 8593601 | 14/07/2009 12:07 | 17/07/2009 10:58 | 235 ± 4 |  |  |
| O | 8A | 272988 | 8593601 | 14/07/2009 12:07 | 17/07/2009 10:58 | 222 ± 4 |  |  |
| 39E | 9A | 272793 | 8593599 | 14/07/2009 12:15 | 17/07/2009 11:01 | 286 ± 5 | 305±27 305 | fine gravel |
| 12E | 9A | 272793 | 8593599 | 14/07/2009 12:15 | 17/07/2009 11:01 | 324 ± 5 |  |  |
| 15 | 9A | 272793 | 8593599 | 14/07/2009 12:15 | 17/07/2009 11:01 | N/A |  |  |
| 35E | 10A | 272606 | 8593588 | 14/07/2009 12:20 | 17/07/2009 11:04 | 155 ± 3 | 140±14 139 | sandy loam |
| 9E | 10A | 272606 | 8593588 | 14/07/2009 12:20 | 17/07/2009 11:04 | 129 ± 3 |  |  |
| 74 | 10A | 272606 | 8593588 | 14/07/2009 12:20 | 17/07/2009 11:04 | 136 ± 4 |  |  |
| 4E | 11A | 272511 | 8593428 | 14/07/2009 12:28 | 17/07/2009 11:15 | 631 ± 6 | 612±27 612 | sandy loam |
| 29E | 11A | 272511 | 8593428 | 14/07/2009 12:28 | 17/07/2009 11:15 | 593 ± 6 |  |  |
| 65 | 11A | 272511 | 8593428 | 14/07/2009 12:28 | 17/07/2009 11:15 | N/A |  |  |
| 63 | 12A | 272712 | 8593449 | 14/07/2009 12:44 | 17/07/2009 11:18 | 6108 ± 17 | 6631±456 6621 | fine gravel |
| 8E | 12A | 272712 | 8593449 | 14/07/2009 12:44 | 17/07/2009 11:18 | 6946 ± 18 |  |  |
| 31 | 12A | 272712 | 8593449 | 14/07/2009 12:44 | 17/07/2009 11:18 | 6840 ± 18 |  |  |
| 84 | 13A | 272894 | 8593454 | 14/07/2009 12:55 | 17/07/2009 11:22 | 7541 ± 19 | 9983±3892 9532 | coarse gravel |
| 1E | 13A | 272894 | 8593454 | 14/07/2009 12:55 | 17/07/2009 11:22 | 7937 ± 19 |  |  |
| 23E | 13A | 272894 | 8593454 | 14/07/2009 12:55 | 17/07/2009 11:22 | 14472 ± 27 |  |  |
| 62 | 14A | 272894 | 8593453 | 14/07/2009 13:05 | 17/07/2009 11:33 | 1022 ± 7 | 5216±5420 3366 | coarse gravel |
| 76 | 14A | 272894 | 8593453 | 14/07/2009 13:05 | 17/07/2009 11:33 | 11336 ± 23 |  |  |
| 16E | 14A | 272894 | 8593453 | 14/07/2009 13:05 | 17/07/2009 11:33 | 3289 ± 13 |  |  |
| 86 | 15A | 272856 | 8593432 | 14/07/2009 13:12 | 17/07/2009 11:30 | 2586 ± 11 | 4982±2238 4594 | gravel/ sand on rock |
| 77 | 15A | 272856 | 8593432 | 14/07/2009 13:12 | 17/07/2009 11:30 | 7019 ± 19 |  |  |
| 95 | 15A | 272856 | 8593432 | 14/07/2009 13:12 | 17/07/2009 11:30 | 5341 ± 16 |  |  |
| 93 | 16A | 272867 | 8593385 | 14/07/2009 13:20 | 17/07/2009 11:26 | 4840 ± 15 | 5726±815 5686 | sandy |
| 67 | 16A | 272867 | 8593385 | 14/07/2009 13:20 | 17/07/2009 11:26 | 6443 ± 18 |  |  |
| 79 | 16A | 272867 | 8593385 | 14/07/2009 13:20 | 17/07/2009 11:26 | 5894 ± 17 |  |  |
| Can # | site | Easting | Northing | Deployed Date/time | Retrieved Date/time | Rn [mBq.m-2.s-1] | ave±stdev geomean | Soil type |
| 94 | 17A | 273092 | 8593445 | 14/07/2009 13:31 | 17/07/2009 11:42 | 93 ± 3 | 104±15 103 | sandy loam |
| 96 | 17A | 273092 | 8593445 | 14/07/2009 13:31 | 17/07/2009 11:42 | 98 ± 3 |  |  |
| 66 | 17A | 273092 | 8593445 | 14/07/2009 13:31 | 17/07/2009 11:42 | 121 ± 4 |  |  |
| 17E | 18A | 272798 | 8593299 | 14/07/2009 14:09 | 17/07/2009 11:49 | 143 ± 4 | 122±29 121 | fine gravel |
| 27E | 18A | 272798 | 8593299 | 14/07/2009 14:09 | 17/07/2009 11:49 | 102 ± 3 |  |  |
| 28E | 19A | 272993 | 8593295 | 14/07/2009 13:56 | 17/07/2009 11:46 | 217 ± 3 | 229±24 228 | sandy loam |
| 26E | 19A | 272993 | 8593295 | 14/07/2009 13:56 | 17/07/2009 11:46 | 213 ± 3 |  |  |
| 25E | 19A | 272993 | 8593295 | 14/07/2009 13:56 | 17/07/2009 11:46 | 256 ± 4 |  |  |
| 81E | 20A | 272495 | 8593306 | 14/07/2009 14:22 | 17/07/2009 11:53 | 822 ± 7 | 926±147 921 | sandy loam |
| 62E | 20A | 272495 | 8593306 | 14/07/2009 14:22 | 17/07/2009 11:53 | 1030 ± 8 |  |  |
| 40 | 21A | 272416 | 8593047 | 14/07/2009 14:30 | 17/07/2009 12:00 | 8 ± 3 | 6±2 6 | sandy loam |
| 64 | 21A | 272416.6 | 8593047.7 | 14/07/2009 14:30 | 17/07/2009 12:00 | 6 ± 3 |  |  |
| 30E | 21A | 272416.6 | 8593047.7 | 14/07/2009 14:30 | 17/07/2009 12:00 | 4 ± 3 |  |  |
| 20E | 22A | 272524.23 | 8593172.2 | 14/07/2009 14:38 | 17/07/2009 12:05 | 583 ± 6 | 556±26 555 | sandy loam |
| 3E | 22A | 272524.23 | 8593172.2 | 14/07/2009 14:38 | 17/07/2009 12:05 | 552 ± 6 |  |  |
| 72 | 22A | 272524.23 | 8593172.2 | 14/07/2009 14:38 | 17/07/2009 12:05 | 532 ± 6 |  |  |
| 21E | 23A | 272598.93 | 8593113.7 | 14/07/2009 14:46 | 17/07/2009 12:08 | 2496 ± 11 | 3513±1331 3359 | sandy loam |
| 85 | 23A | 272598.93 | 8593113.7 | 14/07/2009 14:46 | 17/07/2009 12:08 | 5020 ± 16 |  |  |
| 34E | 23A | 272598.93 | 8593113.7 | 14/07/2009 14:46 | 17/07/2009 12:08 | 3024 ± 13 |  |  |
| 75 | 24A | 272643.81 | 8593165 | 14/07/2009 14:58 | 17/07/2009 12:14 | 15684 ± 27 | 8710±6718 6634 | Rocky heap |
| 91 | 24A | 272643.81 | 8593165 | 14/07/2009 14:58 | 17/07/2009 12:14 | 2280 ± 11 |  |  |
| 18E | 24A | 272643.81 | 8593165 | 14/07/2009 14:58 | 17/07/2009 12:14 | 8166 ± 20 |  |  |
| 90 | 25A | 272731.02 | 8593124 | 14/07/2009 15:05 | 17/07/2009 12:21 | 278 ± 5 | 231±66 227 | sandy loam |
| 11E | 25A | 272731.02 | 8593124 | 14/07/2009 15:05 | 17/07/2009 12:21 | 185 ± 4 |  |  |

# Appendix C  Airborne radon activity concentrations

Appendix C  Results of airborne radon activity concentrations [Bq∙m-3] for sites 1A-25A.

| Site | Rn [Bq.m3] | Rn average [Bq.m3] |  | Site | Rn [Bq.m3] | Rn average [Bq.m3] |
| --- | --- | --- | --- | --- | --- | --- |
| 4A | 298±119 | 218±114 |  | 17A | 72±29 | 164±129 |
|  | 137±55 |  |  |  | 255±102 |  |
| 8A | 131±52 | 119±17 |  | 17A (30cm) | 236±94 | 144±131 |
|  | 107±43 |  |  |  | 51±20 |  |
| 9A | 39±16 | 45±8 |  | 17A (50cm) | 82±33 | 70±18 |
|  | 51±20 |  |  |  | 57±23 |  |
| 2A | 92±37 | 96±6 |  | 20A | 131±52 | 165±47 |
|  | 100±40 |  |  |  | 198±79 |  |
| 3A | 181±72 | 122±83 |  | 22A | 244±98 | 206±54 |
|  | 63±25 |  |  |  | 168±67 |  |
| 3A (30cm) | 175±70 | 206±43 |  | 23A | 88±35 | 120±45 |
|  | 236±94 |  |  |  | 151±60 |  |
| 3A (50cm) | 125±50 | 107±26 |  | 19A | 131±52 | 125±8 |
|  | 88±35 |  |  |  | 119±48 |  |
| 12A | 244±98 | 176±97 |  | 24A | 221±88 | 229±11 |
|  | 107±43 |  |  |  | 236±94 |  |
| 18A | 76±30 | 95±26 |  | 13A | 252±101 | 223±42 |
|  | 113±45 |  |  |  | 193±77 |  |
| 16A | 463±185 | 331±187 |  | 13A (30cm) | 3464±1386 | 1995±2078 |
|  | 198±79 |  |  |  | 525±210 |  |
| 21A | 63±25 | 68±6 |  | 13A (50cm) | 545±218 | 859±444 |
|  | 72±29 |  |  |  | 1173±469 |  |
| 21A (30cm) | n.r. | 100±40 |  | 14A | 422±169 | 427±7 |
|  | 100±40 |  |  |  | 432±173 |  |
| 21A (50cm) | n.r. | 20±8 |  | 14A (30cm) | 1657±663 | 1796±197 |
|  | 20±8 |  |  |  | 1935±774 |  |
| 1A | 82±33 | 113±44 |  | 14A (50cm) | 1225±490 | 1549±458 |
|  | 144±58 |  |  |  | 1873±749 |  |
| 25A | 46±18 | 46±18 |  | 15A | 259±104 | 283±33 |
|  | n.r. |  |  |  | 306±122 |  |
| 5A | 224±90 | 159±92 |  | 15A (30cm) | 267±107 | 401±190 |
|  | 94±38 |  |  |  | 535±214 |  |
| 6A | 298±119 | 237±87 |  | 15A (50cm) | 607±243 | 1233±885 |
|  | 175±70 |  |  |  | 1858±743 |  |
| Site | Rn [Bq.m3] | Rn average [Bq.m3] |  | Site | Rn [Bq.m3] | Rn average [Bq.m3] |
| 7A | 252±101 | 332±112 |  | unexposed | 15±6 | 15±6 |
|  | 411±164 |  |  |  |  |  |
| 10A | 57±23 | 110±74 |  |  |  |  |
|  | 162±65 |  |  |  |  |  |
| 11A | 181±72 | 166±22 |  |  |  |  |
|  | 150±60 |  |  |  |  |  |

n.r.: not reliable

# Appendix D  Soil activity concentrations

Appendix D  Results of soil activity concentrations [Bq∙kg-1] and gamma dose rate measurements [µGy∙hr-1] for sites 1A-25A at Anomaly 2. Soil activity concentrations were measured with the eriss HPGe gamma detectors. Eastings and Northings are given in Appendix B.

| Site | U-238 | Ra-226 | Pb-210 | Ra-228 | Th-228 | K-40 | Dose rate |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Site 1a | 131±4 | 224±2 | 193±5 | 27±1 | 34±1 | 36±3 | 0.21 ± 0.01 |
| Site 2a | 656±9 | 1330±10 | 803±14 | 24±1 | 24±1 | 10±4 | 0.47 ± 0.02 |
| Site 3a | 970±10 | 1230±10 | 930±20 | 52±2 | 58±1 | 0±5 | 0.72 ± 0.02 |
| Site 4a | 533±8 | 766±6 | 560±10 | 28±1 | 33±1 | 38±5 | 0.47 ± 0.02 |
| Site 5a | 241±6 | 429±3 | 414±8 | 24±1 | 28±1 | 26±4 | 0.30 ± 0.01 |
| Site 6a | 93±3 | 91±1 | 141±4 | 24±1 | 25±1 | 15±2 | 0.14 ± 0.01 |
| Site 7a | 124±3 | 130±1 | 197±4 | 21±1 | 25±1 | 16±2 | 0.16 ± 0.01 |
| Site 8a | 162±4 | 212±2 | 168±5 | 51±1 | 56±1 | 17±2 | 0.22 ± 0.02 |
| Site 9a | 103±3 | 168±1 | 159±5 | 32±1 | 36±1 | 35±3 | 0.16 ± 0.01 |
| Site 10a | 133±4 | 171±2 | 227±6 | 56±1 | 62±2 | 69±4 | 0.14 ± 0.01 |
| Site 11a | 226±5 | 291±2 | 379±7 | 25±1 | 26±1 | 35±3 | 0.21 ± 0.01 |
| Site 12a | 987±11 | 1330±10 | 1300±20 | 29±2 | 31±1 | 75±6 | 0.8 ± 0.01 |
| Site 13a | 7775±31 | 10580±70 | 6510±90 | 38±4 | 45±2 | 182±16 | 5.25 ± 0.06 |
| Site 14a | 78073±107 | 110700±800 | 64800±830 | N/A | N/A | N/A | 21.0 ± 0.7 |
| Site 15a | 32219±61 | 41080±290 | 29010±370 | 78±7 | 113±5 | 290±25 | 11.4 ± 0.22 |
| Site 16a | 8692±29 | 7800±60 | 6910±90 | 42±3 | 43±2 | 80±11 | 2.07 ± 0.03 |
| Site 17a | 30±2 | 38±1 | 77±3 | 18±1 | 20±1 | 21±2 | 0.11 ± 0.01 |
| Site 18a | 75±3 | 79±1 | 107±4 | 26±1 | 30±1 | 37±3 | 0.14 ± 0.01 |
| Site 19a | 46±2 | 39±1 | 109±3 | 18±1 | 20±1 | 27±2 | 0.13 ± 0.01 |
| Site 20a | 264±5 | 213±2 | 267±6 | 21±1 | 25±1 | 24±3 | 0.23 ± 0.01 |
| Site 21a | 16±1 | 14±1 | 45±3 | 15±1 | 16±1 | 44±3 | 0.10± 0.04 |
| Site 22a | 275±6 | 502±4 | 420±8 | 20±1 | 23±1 | 22±4 | 0.38 ± 0.02 |
| Site 23a | 373±7 | 654±5 | 729±12 | 16±1 | 16±1 | 20±4 | 0.70 ± 0.01 |
| Site 24a | 6289±29 | 9950±70 | 7660±100 | 77±4 | 98±3 | 616±22 | 2.00 ± 0.02 |
| Site 25a | 29±1 | 30±1 | 80±3 | 15±1 | 16±1 | 45±3 | 0.12 ± 0.01 |