645

internal report

C Pease, T Mooney, 
M Trenfield, C Costello and 
A Harford

September 2016

Release status – unrestricted

Updated procedure for the 72 hour algal growth inhibition toxicity test using Chlorella sp.

The Department acknowledges the traditional owners of country throughout Australia and their continuing connection to land, sea and community. We pay our respects to them and their cultures and to their elders both past and present.

# Updated procedure for the 72 hour algal growth inhibition toxicity test using Chlorella sp.

Ceiwen Pease, Thomas Mooney, Melanie Trenfield, Claire Costello and Andrew Harford

Supervising Scientist Branch

GPO Box 461, Darwin NT 0801

September 2016

Release status – unrestricted

How to cite this report:

Pease C, Mooney T, Trenfield M, Costello C & Harford A 2016. Updated procedure for the 72 hour algal growth inhibition toxicity test using Chlorella sp. Internal Report 645, September, Supervising Scientist, Darwin.

Authors of this report:

Ceiwen Pease –Supervising Scientist, GPO Box 461, Darwin NT 0801, Australia

Thomas Mooney –Supervising Scientist, GPO Box 461, Darwin NT 0801, Australia

Melanie Trenfield –Supervising Scientist, GPO Box 461, Darwin NT 0801, Australia

Claire Costello –Supervising Scientist, GPO Box 461, Darwin NT 0801, Australia

Andrew Harford –Supervising Scientist, GPO Box 461, Darwin NT 0801, Australia

The Supervising Scientist is a branch of the Australian Government Department of the Environment and Energy.

Supervising Scientist

Department of the Environment and Energy

GPO Box 461, Darwin NT 0801 Australia

environment.gov.au/science/supervising-scientist/publications

© Commonwealth of Australia 2016

IR645 is licensed by the Commonwealth of Australia for use under a Creative Commons By Attribution 3.0 Australia licence with the exception of the Coat of Arms of the Commonwealth of Australia, the logo of the agency responsible for publishing the report, content supplied by third parties, and any images depicting people. For licence conditions see: http://creativecommons.org/licenses/by/3.0/au/

Disclaimer

The views and opinions expressed in this publication are those of the authors and do not necessarily reflect those of the Australian Government or the Minister for the Environment and Energy.

While reasonable efforts have been made to ensure that the contents of this publication are factually correct, the Commonwealth does not accept responsibility for the accuracy or completeness of the contents, and shall not be liable for any loss or damage that may be occasioned directly or indirectly through the use of, or reliance on, the contents of this publication.

# Contents

Executive summary	v

1  Introduction	1

2  Objective	2

3  Principle of the test	3

4  Test organism	4

5  Dilution water	5

6  Chemical solutions	6

7  Test solutions	7

8  Test conditions and quality analysis	8

9  Apparatus and test equipment	9

9.1  Container preparation	9

9.2  Equipment	9

10  Area for test preparation	11

11  Recording data	12

12  Test procedure	13

13  Randomisation	15

14  Reference toxicants	16

15  Acceptability of test data	17

16  Analyses of test data	18

References	19

Appendix A Nutrient reduction trials	20

1  Aims	20

2  Method	20

3  Results	20

4  Discussion	21

Appendix B Use of HEPES buffer in algal toxicity tests	22

1  Aims	22

2  Methods	22

3  Results and discussion	23

Appendix C  Reference toxicity testing with uranium in Synthetic Soft Water	24

1  Aims	24

2  Method	24

3  Results	24

4  Discussion	26

Appendix D Synthetic Soft water preparation	27

1  Safety	27

2  Preparation of solution	27

3  Making stocks	27

Appendix E Analysis of algal densities using the Accuri C6 flow cytometer	29

1  Start-up Clean	29

2  Validation beads	29

3  Final clean	32

5  Shutdown procedure	34

Appendix F Calculation of algal growth constant	36

Appendix G Example end of test worksheet	40

Appendix H Lists of toxicity tests performed to determine nutrient concentrations and subsequent reference toxicity tests	41

Appendix I  Water Quality measurements	42

Appendix J  Chemical analyses	49

Appendix K  Analytical reports	56

# Executive summary

Chlorella sp. is one of five species routinely used in the Supervising Scientist ecotoxicology laboratory to derive site-specific water quality guideline values that are implemented in the creeks surrounding Ranger uranium mine in northern tropical Australia. Data for this species, which is native to the region, have contributed to the derivation of toxicity estimates for a number of contaminants of potential concern. The procurement of a new flow cytometer, which is able to quantify lower algal cell densities compared to previous methods, has allowed for the refinement of the test method to be more ecologically relevant.

Previous studies have demonstrated an increase in sensitivity of algae to metals associated with lower algal cell inoculation density (Moreno-Garrido et al 2000, Franklin et al 2002). Reducing the algal cell density within the test system also allows for a reduction in the concentration of nutrients added to the test medium. Dissolved metals and nutrients are known to interact, and this has the potential to interfere with the assessment of metal toxicity. Consequently, studies were undertaken to assess algal cell inoculation density and nutrient concentrations in order to optimise the sensitivity of the 72 h Chlorella sp. growth inhibition toxicity test. The outcome of these studies was an updated test method that uses an algal inoculation density of 
3 x 103 cells ml-1 (one order of magnitude lower than the original test method) and nitrate and phosphate concentrations of 3.63 mg L-1 nitrate and 34.5 µg L-1 phosphate (one quarter of the concentrations in the original test method). These refinements have resulted in a test system that is more representative of natural systems, which is an important requirement for developing toxicity tests. This report describes the revised test method and the studies used to define the test parameters.

# 1  Introduction

The tropical microalga, Chlorella sp. has been cultured continuously in the Supervising Scientist ecotoxicology laboratory for 25 years. This is one of five species routinely used to derive site-specific water quality guideline values, which are implemented in the creeks surrounding Ranger Uranium Mine in the Alligator Rivers Region in tropical northern Australia. The protocol, detailed in  (Riethmuller et al 2003) uses an algal density of 3 x 104 cells ml-1, and is measured by performing electronic cell enumeration using electro-zoning methods (i.e. Coulter Multisizer II Particle Analyser, Beckman-Coulter). This protocol has been effective for assessing the toxicity of contaminants to Chlorella sp.. The reproducibility of the protocol (as evident from results of the reference toxicity program) means a high level of reliability can be associated with the results of these studies. While the protocol is currently effective, it is important to continue to refine and improve protocols to make them as reproducible and sensitive as possible.

Studies have shown increased sensitivity to metals in toxicity tests when lower initial densities of algae are used  (Moreno-Garrido et al 2000, Franklin et al 2002). This reduced sensitivity is likely to be due to increased metal accumulation rates at lower densities  (Franklin et al 2002). Lower algae densities also enable nutrients, which are required within the test medium, to be reduced to concentrations that better reflect those which occur in the environment. Nutrients, such as phosphate, readily bind with some contaminants and can reduce their bioavailability and toxicity  (Mkandawire et al 2006, Mkandawire et al 2007). Therefore, reducing the levels of nutrients within the test medium often increases the sensitivity of the test. Therefore, it is ideal to minimise both initial algal cell densities and concentrations of nutrients to levels as close as possible to environmental concentrations.

The use of flow cytometry instead of electro-zonation or manual count methods for counting algal cells has made it possible to accurately measure much lower densities of microalgal cells per millilitre of test medium. The acquisition of a flow cytometer by the Supervising Scientist ecotoxicology laboratory has, thus, enabled the ability to optimise the 72 h Chlorella sp. growth inhibition toxicity test for both initial algal cell density and, consequently, nutrient concentrations. This report describes the revised method for the Chlorella sp. toxicity test, and also provides details of the studies undertaken to enable the method to be updated.

# 2  Objective

The objective of a test series (i.e. 3-4 definitive tests) is to determine the concentrations of a specified chemical or whole effluent that shows the median effect concentration (e.g. EC50). This is the concentration of a chemical in solution that is estimated to cause a 50% effect concentration of a sublethal response of test organisms. This is measured as the 50% effect population growth of the tropical alga Chlorella sp. over 72 h  (Riethmuller et al 2003).

# 3  Principle of the test

Exponentially growing cells of Chlorella sp. are exposed to a concentration range of a toxicant or water of interest over a 72 h period. The cell counts are performed at 48 h, 72 h, and occasionally 24 h, to calculate cell division rates for the algal population over time. The concentration-response relationship is modelled using non-linear regression and toxicity estimates are derived from this model. This test is based on international standards, specifically:

OECD  (2011) test number 201: Freshwater alga and cyanobacteria, growth inhibition test; and

USEPA  (2002) Methods for measuring the acute toxicity of effluents and receiving waters to freshwater and marine organisms, 5th edition.

# 4  Test organism

The unicellular green alga, Chlorella sp., has been continuously cultured in the ecotoxicology laboratory for the past 25 years (see Riethmuller et al (2003) for details). Chlorella sp. was isolated from surface waters of Magela Creek in Kakadu National Park (Georgetown Billabong) by Armando Padovan in 1991.

# 5  Dilution water

Depending on the aim of the test, either synthetic soft water (SSW) or uncontaminated natural water (often Magela Creek water (MCW) for Ranger related toxicity tests) is used as the test diluent.

Magela Creek water is collected from one of two locations along the creek. When there is flow in the creek during the wet season, water is collected by boat near Georgetown Billabong (Map Grid of Australia (MGA) Zone 53, 275320.954 East, 8597972.198 North). Throughout the rest of the year water is collected from Bowerbird Billabong closer to the source of the creek (MGA Zone 53, 287190 East, 8587265 North) where there is a persistent water body during the drier months.

Synthetic soft water was created to simulate the inorganic composition of MCW during the wet season. As a result, it lacks any form of organic carbon. Magela Creek water is very soft and slightly acidic with low alkalinity and low electrical conductivity. The SSW is made using the method in Appendix D. SSW is prepared as close to the start of an experiment as possible and is stored in the refrigerator in sealed polyethylene containers for up to two weeks.

# 6  Chemical solutions

All reagents used are analytical grade and stock solutions are made up in ultra-pure water (18 M, Milli-Q, Millipore). The date of stock preparation, source and the person who made the solution are marked on each bottle. A label displaying the chemical name, formula and any required hazard symbols and first aid information must also be included on the bottle. These labels must adhere to the Supervising Scientist Branch (SSB) chemical labelling protocol (See SSB document WHS-029).

# 7  Test solutions

All test waters, regardless of diluent, need to be supplemented with nutrients in order to promote algal growth that is sufficient to measure a toxicological response. A buffer (1 mM HEPES; 
N-2hydroxypiperazine-N’-2ethanesulfonic acid) is required to minimise pH drift throughout the 72 h test period (Appendix B). The following steps must be followed when preparing test solutions:

5 L of nutrient-supplemented test water is made (using either MCW or SSW) to final nitrate and phosphate concentrations of 3.63 mg L-1 nitrate (NO3-) and 34.5 µg L-1 phosphate (PO43), respectively, and a final HEPES concentration of 1 mM.

500 ml of the above test water is allocated for each treatment.

For each treatment, 50 ml aliquots are dispensed into three Erlenmeyer flasks. The remaining test solution for each treatment is used for the validation of toxicant and nutrient concentrations and water quality measurements.

The nutrient concentrations in this protocol differ from the original algal testing protocol (3.63 mg L-1 NO3- and 34.5 µg L-1 PO43- compared to 14.51 mg L-1 NO3- and 138 µg L-1 PO43-previously). The tests and method used to determine the new nutrient concentrations are shown in Appendix A.

# 8  Test conditions and quality analysis

At the start of each toxicity test, sub-samples (40 ml) of the control, a procedural blank and an ultra-pure water blank are collected in plastic sample bottles and acidified with 1% v/v nitric acid (HNO3, Chem-supply). Samples are analysed by an analytical chemistry laboratory for Al, Ca, Cd, Co, Cr, Cu, Fe, Mg, Mn, Na, Ni, Pb, Se, SO4, U and Zn using ICP-MS or ICP-OES. Samples are also taken from all other treatments and analysed for toxicants and modifying elements relevant to the test in question. Verification of nutrient concentrations (NO3- and PO43-) are performed by taking a 50 ml sub-sample from the control treatment and an ultra-pure water blank. Tests are conducted at 29 ± 1º C using a constant temperature growth cabinet with a 12 h light: 12 h dark photoperiod. Light intensity ranges between 100-150 µmol m-2 s-1 and is checked quarterly using a light meter.

Physico-chemical water quality parameters – i.e. electrical conductivity (EC), pH (WTW Multiline P4 Meter) and dissolved oxygen (DO, WTW inoLab Multiline Level 1) - are measured at the start and conclusion of the test using a pooled sample of each treatment. Temperature is monitored with a probe placed in a 50 ml vial containing 30 ml of ultrapure water at 5 min intervals and recorded using a remote logging system (Testo Saveris Professional, Lenzkirch, Germany). During the testing period, test containers are removed from the temperature controlled chamber for the minimum amount of time to maintain as constant temperature as possible throughout the test.

Results of the chemical analyses are considered acceptable if the control, procedural blank and ultra-pure water blank are free of contamination and the measured spiked toxicant concentrations are within 20% of nominal concentrations. Water quality data were considered acceptable if: the recorded temperature of the incubator remained at 29 ± 1°C; the recorded pH of the control group was within  1 unit of Day 1 values; the EC for the control solution was within 10% of the values obtained on Day 1; and the DO concentration was greater than 70% saturation throughout the test.

# 9  Apparatus and test equipment

## 9.1  Container preparation

All equipment that comes into contact with test organisms, control water or test solutions should be made of a material which is suitable to the toxicant in question. For metals, a plastic (e.g. polyethylene) should be used, where possible, whereas for organics, glass is a more suitable material. All plastic and glassware is washed by soaking in 5 % HNO3 for 24 h before undergoing a detergent wash (Neodisher Laboclean, phosphate free, Miele, Gütersloh, Germany) and two rinses in a laboratory dishwasher (Miele, Gütersloh, Germany) with deionised reverse osmosis water (Elix, Millipore, Massachusetts, USA). If metals are being tested, glassware used for the incubation of algae in toxicity tests must be silanised with 2% dimethyldichlorosilane in 
1,1,1-trichloroethane (Coatasil, Thermofisher Scientific, Massachusetts, USA) to decrease metal adsorption to the glass.

## 9.2  Equipment

Flow cytometer (e.g. Accuri C6, BD)

15 ml glass tissue homogeniser with Teflon pestle

Disposable 5 ml polystyrene round-bottomed tubes

Light-tight constant temperature growth chamber (set at 29 ºC)

Merck ultra-pure water purification system or similar

Refrigerator (set at 4 ºC)

pH, electrical conductivity and dissolved oxygen meters

A-grade volumetric flasks (500 ml and 5 L)

Chemicals and reagents

Analytical balance and weigh boats

Centrifuge – 4 x 500 ml capacity with swing out buckets

Plastic centrifuge tubes (50 ml)

Vortex mixer

Borosilicate glass 200 ml Erlenmeyer flasks with aluminium caps

Automatic adjustable pipettes (100 µL, 1 ml and 5 ml)

Disposable microlitre pipette tips

Light meter

Magnetic stirrer and stirrer bars

Polyethylene storage containers (1 L)

120 ml polystyrene water parameter vials

Cell scraper

Testo Saveris™ temperature monitoring system

Random number sheet

40 ml and 15 ml sterile plastic sample tubes

50 ml plastic bottles for nutrient samples

# 10  Area for test preparation

The preparation of test solutions should be carried out in an area with ample room and free of contamination from harmful vapours, dust or disturbance. Throughout the test workers should take care not to introduce any contaminants during daily observations and water exchanges by washing hands and arms and wearing disposable gloves. When working with uranium additional precautions must be taken such as using designated benches within the laboratory and disposing of uranium waste down specific sinks within the laboratory (See eriss ecotoxicology lab uranium ASA for the full requirements).

# 11  Recording data

The number of algal cells in each test flask is counted at 48 h, 72 h and, occasionally, 24 h after test commencement (see Section 12 and Appendix E for details). Two additional flasks are inoculated at the beginning of the test (i.e. Day 0) and these are counted to measure the starting algal cell density. These data are then used to calculate algal growth rate (Appendix G). The pH, dissolved oxygen and electrical conductivity are measured on a subsample of test waters at the start and end of the test. Continuous temperature data is collected throughout the test by the Testo Saveris™ temperature probe inside the incubator.

# 12  Test procedure

##### Day 0

Chlorella sp. cells are harvested from a 4-5 d old stock culture as these are in the exponential phase of growth. The algal cells should be centrifuged in a 50 ml plastic centrifuge tube at 1310 g for 7 minutes. The supernatant is then decanted and the cell-pellet resuspended in approximately 30 ml of ultra-pure water by vortexing. The washing process is repeated three times to ensure that the nutrient-enriched culture medium is removed.

After three rinses the pellet is resuspended in approximately 15 ml of ultra-pure water. Two dilutions of this cell suspension are prepared; a 1:10 dilution (e.g. 2 ml cells into 20 ml ultra-pure water) and a 1:100 dilution (0.2 ml of 1:10 dilution into 20 ml ultra-pure water). The 1:100 dilution is used to determine the volume of the 1:10 dilution inoculum required to have the correct final cell density.

Inoculum volume is determined from a 25 µL sub-sample using the flow cytometer (for complete instructions on measuring algal density and setting up the Accuri C6 flow cytometer, see Electronic cell enumeration section below and Appendix E).

The cell count from the flow cytometer is multiplied by the dilution factors (40 × for the 25 µL subsample taken by the flow cytometer and 100 × for the initial 1:100 dilution) to give the density of the washed cell suspension in cells ml-1. This value is then divided by the desired final cell density in each 50 ml aliquot of test solution to give the volume of the 1:10 dilution to add to each test flask (see example below). Two additional flasks are also inoculated and sampled on day 0 to confirm that the cell inoculation density is 3 × 103 cells ml-1.

Example:

Algal counts (P1 gate):		= 738, 728, 714, 657

x 							= 709.25

x100 						= 70925

x40							= 2.84 x 106

3 x 103 × 50 mL  2.84 x 106

= 0.0528

= 53 µL  added to each test flask of 50 ml test solution

Test solutions are prepared as described in section 6 and are equilibrated for 2 h.

Tests are performed with three replicates per treatment at an algal cell density of 3 x 103 cells ml-1.  Given the variability between replicates within a treatment is usually low (x=2.31 % CV in controls of tests to date), this design may be changed to two replicates per treatment if required. This also allows for more toxicant concentrations to be tested.

One flask with test solution but without algae is used as a background count correction for the flow cytometer for each treatment.

NOTE: If test solutions are similar in composition, one blank flask can use used for all treatments.

The test flasks are then placed randomly on the shelves in a light-tight constant temperature growth chamber at 29  1°C on a 12:12 h light, dark cycle at 100-150 photons PAR m-2 s-1.

##### Days 1-3

Each flask is gently agitated by hand twice daily throughout the test to avoid gas limitation; this is done by swirling the solution approximately six times in the clockwise and anti-clockwise direction.

The cell density in each flask is determined at both 48 and 72 h by using the flow cytometer (as per Step 3 and Appendix E).

#### Electronic cell enumeration

Each flask should be gently agitated using a cell scraper to resuspend the algal cells. A 2 ml sub-sample is then taken and placed in a glass tissue grinder and homogenised using a Teflon pestle to break down any clumps of algal cells. 500 µL of this sample is then placed in a clean 2 ml test tube and placed on the flow cytometer sampling stage. Three 25 µL sub-samples are analysed by the flow cytometer and an average taken of the number of events that occur within the gated region that represents algae auto-fluorescing in the red colour. This process is then repeated for each replicate per treatment with the blank flask containing no algae measured first to determine background levels of fluorescence.

The flow cytometer is back-flushed between each treatment. The homogeniser and pestle are also rinsed with ultra-pure water and two new test tubes are used for each treatment.

# 13  Randomisation

Each day, a new set of random numbers must be used to assign the position of each flask within the incubator. Each flask has a number written on the top of the tin foil lid and is placed into the incubator according to the order of the numbers on the random number sheet. The flasks need to be positioned on the edges of each shelf as close to the light source as possible to ensure that light exposure is maximised. Randomisation of flask position in the incubators is an important part of the experimental design. Random numbers are obtained from a random number table or generator for each day of the test. A set of random numbers is unique for each test and is not to be reused.

# 14  Reference toxicants

The use of reference toxicants enables the response of the test organism to be assessed over time to ensure the response is reproducible (see Appendix C for the current reference toxicity program). This process also checks the proficiency of operators and laboratory standards. Uranium (U, added as uranyl sulphate) is used in a concentration range from 5-80 µg L-1. Synthetic soft water is used as the diluent. The EC50 value, calculated from the concentration-response curve should fall within 3 standard deviations (SDs) of the mean on the quality control chart for the test species. If the value falls outside 2 SDs of the mean, it is a warning that there may be something wrong with the test or the sensitivity of the organism has changed. It is important to note that a control chart cannot be produced or considered reliable with less than 5 values.

# 15  Acceptability of test data

The test data are considered acceptable if:

The recorded temperature of the incubator remains within the prescribed limits (29 ± 1ºC),

The growth rate of the control algae is within the range 1.9 ± 0.15 (mean ± standard deviation) doublings per day,

There is <20% co-efficient of variability (CV) in the control growth rate,

The recorded pH is within the prescribed limits and,

The results of reference toxicity testing are within the set limits.

# 16  Analyses of test data

The growth rate of the algae in each flask is calculated using linear regression analyses (Appendix F). A regression is plotted for log10 cell density vs. time (h) to determine the slope of the line for each flask, which is equivalent to the cell division rate per h (µ) for each treatment. Doublings per day are calculated by multiplying the cell division rate by 24 (the hours in a day) then by the constant of 3.32. The algal population growth constant is based on calculations done by CSIRO and are shown in Appendix F. The growth rates of each treatment are presented as a function of the control response and these are plotted against measured toxicant concentrations. The endpoint of the algal growth test is measured as the 72 h EC50. Linear interpolation is used to calculate EC values when performing reference toxicity tests using uranium (U) to allow for comparison with historical reference toxicity data but in all other toxicant tests non-linear regressions (e.g. generally three parameter log-logistic models) are used as they provide a more accurate fit to the data. Toxicant concentration is log transformed in all analyses.

# References

Franklin NM, Stauber JL, Apte SC & Lim RP 2002. Effect of initial cell density on the bioavailability and toxicity of copper in microalgal bioassays. Environmental Toxicology and Chemistry 21 (4), 742-751.

Markich SJ 2002. Uranium speciation and bioavailability in aquatic systems: An overview. The Scientific World Journal 2, 707-729.

Mkandawire M, Taubert B & Dudel EG 2006. Limitations of growth-parameters in Lemna gibba bioassays for arsenic and uranium under variable phosphate availability. Ecotoxicology and Environmental Safety 65 (1), 118-128.

Mkandawire M, Vogel K, Taubert B & Dudel EG 2007. Phosphate regulates uranium (VI) toxicity to Lemna gibba L. G3. Environmental Toxicology 22 (1), 9-16.

Moreno-Garrido I, Lubián LM & Soares AMVM 2000. Influence of Cellular Density on Determination of EC50 in Microalgal Growth Inhibition Tests. Ecotoxicology and Environmental Safety 47 (2), 10//, 112-116.

OECD. 2011. Test No. 201: Freshwater Alga and Cyanobacteria, Growth Inhibition Test. OECD Publishing, Paris, http://dx.doi.org/10.1787/9789264069923-en.

Riethmuller N, Camilleri C, Franklin N, Hogan AC, King A, Koch A, Markich SJ, Turley C & van Dam RA 2003. Ecotoxicological testing protocols for Australian tropical freshwater ecosystems. Supervising Scientist Report 173, Supervising Scientist, Darwin, Northern Territory, Australia.

Trenfield MA, Ng JC, Noller BN, Markich SJ & Dam RAv 2011. Dissolved Organic Carbon Reduces Uranium Bioavailability and Toxicity. 2. Uranium[VI] Speciation and Toxicity to Three Tropical Freshwater Organisms. Environmental Science & Technology 45 (7), 2011/04/01, 3082-3089.

USEPA 2002. Methods for Measuring the Acute Toxicity of Effulents and Receiving Waters to Freshwater and Marine organisms, 5th edition. EPA-821-R-02-012, U.S. Environmental Protection Agency, Office of Water, Washington D.C., USA.

# Appendix A
Nutrient reduction trials

## 1  Aims

Tests were performed to establish the minimum concentration of nitrate (NO3-) and phosphate (PO43-) that can be added and produce acceptable algal growth over 72 h without nutrients being a limiting factor within the test system.

## 2  Method

Five tests were conducted with reduced cell density (3 x 103 cells ml-1) and reduced nutrients. Nutrient levels ranging from 1/64th of the original nutrient concentration (2.16 µg L-1 PO43- and 227 µg L-1 NO3-1) to the full ration (14.51 mg L-1 NO3- and 138 µg L-1 PO43-) were tested to determine the minimum concentration of nutrients required to maintain acceptable control growth (Table A1). The complete list of tests is shown in Appendix G. All tests were performed in SSW amended with different volumes of PO43- and NO3- added using the method described above.

Table A1 Summary of reduced nutrient trials showing growth rate range and nutrient concentrations tested

| Test Code | Growth rate range (doublings d-1) | Concentrations tested (µg L-1)a |  |
| --- | --- | --- | --- |
| 1366G | 0.62 – 1.70 | N | 453, 907, 1209, 1814, 3628, 7255, 14510 |
|  |  | P | 4.31, 8.63, 11.5, 17.3, 34.5, 69, 138 |
| 1373G | 1.35 – 1.89 | N | 907, 1209, 1814, 2418, 3628 |
|  |  | P | 8.63, 11.5, 17.3, 23, 34.5 |
| 1376G | 0.71 – 1.43 | N | 227, 453, 605, 907, 1209, 1814 |
|  |  | P | 2.16, 4.31, 5.75, 8.63, 11.5, 17.3 |
| 1378G | 0.91 – 1.70 | N | 907, 1209, 1451, 1814, 2418 |
|  |  | P | 8.63, 11.5, 13.8, 17.3, 23 |
| 1380G | 1.08 – 1.66 | N | 1209, 1451, 1814, 2418, 2902 |
|  |  | P | 11.5, 13.8, 17.3, 23, 27.6 |

a The original concentration of PO4-3 and NO3- added to test medium was 14.51 mg L-1 NO3- and 138 µg L-1 PO4-3

## 3  Results

Algal population growth rates consistently above the original test acceptability criterion of 
1.4 ± 0.3 doublings d-1 (Figure A1.) were observed in all concentrations of nutrients above 1/8 of the original nutrient provision (i.e. 1.815 mg L-1 NO3- and 17.3 µg L-1 PO43-). Based on this information, a new nutrient concentration of ¼ of the original nutrient dose was selected (i.e. 3.63 mg L-1 NO3- and 34.5 µg L-1 PO43-). This value was selected due to the consistently high growth rates and small variance observed. A lower concentration was not selected to ensure that nutrients would not become a limiting resource in the testing environment. All measured nutrient concentrations and quality control checks associated with these data are shown in Appendices I and J.

Figure A1 Pooled population growth rates for the five reduced nutrient trials performed (± S.E.). Red line represents the original population growth acceptability criteria of 1.4 doublings d-1.

## 4  Discussion

We established that a reduction of nutrients to a quarter of the original ration provided sufficient nutrients to Chlorella sp. at a reduced cell density of 3 x 103 cells ml-1. A nutrient reduction to 3.63 mg L-1 NO3- and 34.5 µg L-1 PO43- produced an average control growth rate over the nine successful reference toxicity tests of 1.9 doublings d-1. This growth rate was higher than the original test protocol acceptability criterion for population growth rate (1.4 doublings d-1) and seeing as this is a new methodology a new acceptability criterion was created. A higher growth rate would be expected with a reduced algal cell density due to there being a lower number of cells per ml of medium and, as such, more capacity for growth before nutrients become a limiting factor. It may have been possible to reduce the nutrient levels further, as there was little reduction in population growth rate observed at nutrient concentrations as low as 1/8 of the initial ration. However, it was decided to select slightly higher concentrations to ensure that nutrients did not become limiting. Statistical reports associated with this data can be found in Appendix K.

# Appendix B
Use of HEPES buffer in algal toxicity tests

## 1  Aims

Tests were performed to establish whether HEPES (N-2hydroxypiperazine-N’-2ethanesulfonic acid) was required in the new test system. Due to the reduction in nutrients and Chlorella sp. population starting density within the new system it was possible that pH would remain within the acceptability criterion of 1 pH unit in the absence of a buffer as fewer waste products would be produced by the algae present.

## 2  Methods

Three tests were conducted with an additional control treatment without HEPES included. Tests were performed in SSW with a Chlorella sp. starting cell density of 3 x 103 cells ml-1. Nutrients were added in the reduced concentrations of 3.63 mg L-1 of NO3- and 34.5 µg L-1 of PO43-. Algal population change was measured with flow cytometry using the method outlined in Section 12 of the main report. The complete list of tests is shown in Appendix H and associated quality control checks are in Appendices I and J. Nominal nutrient concentrations were used for analyses as the measured concentrations we received from an external analysis company were inaccurate (see Appendix J, Table J2). This has been an ongoing problem in the Supervising Scientist ecotoxicology lab for some time, a solution for measuring nitrate and phosphate accurately is needed.

Table B1 pH variation and percent coefficient of variation (% CV) in control treatments with and without 1mM of the buffer HEPES added to the test system after 72h (n=3). pH samples were measured from a pooled sample taken from the three test flasks.

| pH variation |  | % CV |  |
| --- | --- | --- | --- |
| Test | 1mM HEPES | No HEPES | 1mM HEPES | No HEPES |
| 1455G | 0.31 | 0.48 | 0.72 | 13.9 |
| 1466G | 0.07 | 0.59 | 0.19 | 1.63 |
| 1468G | 0.09 | 0.34 | 0.93 | 21.5 |

Figure B1 Chlorella sp. population growth rate in SSW with and without the inclusion of 1mM 
HEPES (n = 3).

## 3  Results and discussion

Reducing the initial starting density notably improved pH control with, at most, a 0.2 unit drift in pH in the nutrient trials and 0.3 of a unit in reference toxicity testing. Even in the absence of the HEPES buffer, which is used to control pH in algal tests, a change of up to 0.59 units was observed. While pH control was within the ‘acceptability criterion of 1 pH unit’ in the absence of HEPES (Table B1) the population growth rates were lower (Figure B1) and more variable between replicates (%CV HEPES x = 0.61, No HEPES x = 12.35). Moreover, pH changes of around 0.5 of a unit can still result in marked changes in metal speciation and bioavailability. Consequently, it was considered appropriate to retain HEPES in the test medium. In two of the tests performed abnormally high concentrations of iron, aluminium and manganese were detected; this indicates a source of contamination. Results for these tests fit within our expectations and as such these tests were included in analyses.

# Appendix C 
Reference toxicity testing with uranium in Synthetic Soft Water

## 1  Aims

On finalisation of the new methodology reference toxicity tests were performed to determine how the new method would affect the sensitivity of Chlorella sp. to uranium.

## 2  Method

Uranium (U) toxicity tests were performed using the reduced nutrient concentrations and starting cell density of 3 x 103 cells ml-1 in SSW. Initially, 7 U concentrations were tested ranging between 5 and 160 µg L-1 to establish definitive concentration–response curves. After 5 repeatable tests, this was reduced to 5 concentrations ranging between 20 and 160 µg L-1.

Control growth rates for each test were higher than with the past protocol of 1.4 ± 0.3 doublings d-1 (Reithmuller et al., 2003, Table C1) and a new acceptability criterion for population growth rate was calculated. After six tests, a growth rate acceptability criterion was derived by calculating the average and standard deviation of the growth rate of the control treatment for each test. The average was made the acceptability criterion and the standard deviation was used as the acceptable variance surrounding the criterion. Based on the control treatment growth rates in the reference toxicity tests a new acceptability criterion of 1.90 ± 0.15 doublings day-1 was calculated. All measured U concentrations and quality control checks associated with this data are shown in Appendices I, J and K.

## 3  Results

Nine valid U toxicity tests were performed in synthetic soft water (Appendix D) using the nutrient concentrations (¼ of the original nutrients of Reithmuller et al (2003)) of 3.63 mg L-1 NO3- and 34.5 µg L-1 PO43-. The 50% Effect Concentrations (EC50) for the nine tests ranged between 9 and 53 µg L-1. The current running mean (± 2 Standard Deviations, SD) for the new method is 
35 (6, 63) µg L-1 compared to 42 (7, 76) µg L-1 of U in the original method (Figure C1).

Table C1 Control population growth rates and toxicity estimates for reference toxicity tests using a starting algal density of 3 x 103 cells ml-1. Values in parentheses in the ‘Control population growth rate’ column represent % CV and in the ‘EC50’ column represent 95% confidence limits.

| Test Code | Date | Control population growth rate (Doublings d-1) | EC50 (µg L-1 U) |
| --- | --- | --- | --- |
| 1383G | 03/02/14 | 1.84 (8.36) | 40 (35, 45) |
| 1389G | 18/02/14 | 1.88 (2.39) | 41 (37, 45) |
| 1404G | 13/05/14 | 1.86 (0.42) | 39 (37, 40) |
| 1415G | 05/08/14 | 1.74 (3.44) | 49 (46, 55) |
| 1431G | 09/12/14 | 1.78 (1.58) | 53 (41, 86) |
| 1455G | 17/03/15 | 2.11 (0.72) | 37 (28, 48) |
| 1466G | 09/06/15 | 2.17 (0.19) | 9 (8, 10) |
| 1468G | 22/06/15 | 1.91 (0.93) | 19 (18,19) |
| 1479G | 07/09/15 | 2.05 (2.78) | 25 (23, 27) |

Figure C1 Reference toxicant control chart for Chlorella sp. using population growth rate as an endpoint. Data points represent EC50 µg L-1 U toxicity estimates and their 95% confidence limits (CLs). Reference lines represent the following: broken lines – upper and lower 99% confidence limits ( 3 standard deviations) of the whole data set; dotted lines – upper and lower warning limits ( 2 standard deviations); unbroken line – running mean

## 4  Discussion

At a reduced cell density and nutrient level, Chlorella sp. sensitivity to U increased slightly (17% reduction – 42 to 35 µg L-1), albeit non-significantly. This result was driven by the last three tests, which were more sensitive than the previous six tests (Figure C1).  An increase in sensitivity would be expected, as ligands such as phosphate cause speciation changes when complexation occurs with U in solution  (Markich 2002). This complexation is known to reduce U toxicity to freshwater organisms  (Markich 2002, Trenfield et al 2011).The lower starting algal density would also contribute to the reduction in sensitivity as less metals have been found to bind to algal cells at higher algae densities  (Franklin et al 2002). Seeing as only nine tests have been performed so far, it is difficult to determine whether the more sensitive results in the last three tests were due to a change in the Chlorella sp. culture or due to natural variation in U sensitivity. When a comparison is made between the variation in sensitivity of the current and previous method we see that it is similar, if not a little more variable in the old method (current test method EC50 range = 8.96 – 53.17 µg L-1, old test method EC50 range = 10.12 – 74.91 µg L-1). Further reference toxicity testing will allow for this to be better characterised. In one of the reference toxicity tests abnormally high concentrations zinc were detected; this indicates a source of contamination (Table J3). The growth rate and toxicity estimate for this test fit within our expectations and as such this test was included in analysis. Statistical analyses associated with this data are found in Appendix K.

# Appendix D
Synthetic Soft water preparation

## 1  Safety

Check the SDSs for any chemicals you are about to use prior to making up solutions to ensure you are aware of any WH&S issues and the appropriate PPE. There are no risks associated with SSW due to the low concentrations of all chemicals added. Gloves, eye protection and a lab coat must be worn while preparing Synthetic Soft Water.

## 2  Preparation of solution

Fill a 5 L volumetric flask with ultra-pure water and pour this into a clean 25 L plastic barrel designated for synthetic water preparation.

Partially refill the 5L flask with ultra-pure water and add the appropriate amount of the 7 stock solutions (see Table 1) to the flask. Make the flask up to volume with ultra-pure water and pour into the barrel.

Fill the 5 L flask twice more to make the volume in the barrel equal 20 L.

Aerate overnight to allow mixing and gaseous exchange.

Check pH after a minimum of 12 h aeration and adjust to 6.0  0.15 using 0.05 M H2SO4 or 0.05 M NaOH.

The water can be stored at 4C for up to two weeks if necessary. The pH needs to be checked before use to ensure it remains within range.

## 3  Making stocks

Stock solutions are made up every 18 - 24 months.

Add the appropriate amount of chemical in column 2 of Table D1.

Make up 1 L of each stock solution at a time.

Table D1 Stock solutions used to prepare synthetic soft water.

|  | Ingredient | Stock Solution (g L-1) | Volume of stock per 20 L | Nominal conc. of element in SSW |
| --- | --- | --- | --- | --- |
| 1 | NaHCO3 | 72.34 | 1 ml | 0.99 mg L-1 |
| 2 | Al2(SO4)3.18H2O* | 17.26 | 1 ml | 0.075 mg L-1 |
| 3 | MgSO4.7H2O | 121.52 | 1 ml | 0.599 mg L-1 |
| 4 | CaCl2.2H2O | 32.96 | 1 ml | 0.449 mg L-1 |
| 5 | KCl | 14.09 | 1 ml | 0.3107 mg L-1 |
| 6 | FeCl3.6H2O | 10 | 1 ml | 0.1285 mg L-1 |
| 7 | Trace Element Solution CuSO4.5H2O ZnSO4.7H2O Pb(NO3)2 (from EnRad ) MnSO4.H2O UO2SO4.3H2O (use 5gL-1 U stock in fridge 2) | In 1 L add: 0.11 0.123 0.008 1.188 0.007 | 0.5 ml | 0.975 µg L-1 0.699 µg L-1 0.125 µg L-1 9.654 µg L-1 0.1125 µg L-1 |

*Requires heating to dissolve

# Appendix E
Analysis of algal densities using the Accuri C6 flow cytometer

Prior to commencing algal density measurements the Accuri C6 must be run through a start-up process and performance check. The methods are detailed below.

## 1  Start-up clean

Open the ‘H2O Start-up’ file in the CFlow workspace.

Select the back flush button in the workspace and wait for this process to run.

Ensure there is a tube of at least 2 ml of ultra-pure water on the SIP.

Select the next free data cell, label it with the date and run the ultra-pure water for 10 minutes.

Save and close file.

Place a tube of Extended Flow Cell Clean solution on the SIP.

Open the ‘Clean’ file in the Maintenance folder.

Select the next free data cell and run for 2 minutes.

Save and close file. Reopen ‘H2O start-up’ file.

Select the back flush button in the workspace and wait for this process to run.

Replace the tube of ultra-pure water on the SIP.

Select the next free data cell and run the ultra-pure water again for another 10 minutes.

Save and close file.

## 2  Validation beads

### 2.1  The need for validation

Prior to analysis of experimental samples, the performance of the Accuri C6 should be checked using validation beads. The validation should verify whether the lasers are correctly positioned.

The Accuri C6 should be able to distinguish a defined number of peaks within each fluorescence channel i.e. the 8-peak beads should show 8 peaks when a sample is run and the 6-peak beads should show 6 peaks. If these peaks are not discernible, there may be a problem with either the beads or the instrument. If it is a case of the latter, any data collected may be incorrect. Thus, each data collection event should be accompanied by a file that shows the validation bead data. It is important not to proceed with the analysis of any experimental samples until the validation process has been completed successfully.

### 2.2  Setup

A template for bead validation has been created which has pre-defined settings. The file name is 8 and 6 peak template.c6t. A workspace has been created for routine bead validations. Each workspace file has the capacity to hold 48 x 8-peak validation data points and 48 x 6-peak data points. The workspace should be saved with the date that corresponds with the first entry (e.g., 8 and 6 peak 200313). Once all 96 wells are filled, then a new workspace can be created, using the pre-saved template. If for some reason, a new template needs to be created, ensure the following settings are entered:

Type 50000 in the events edit box and select Ungated Sample from the associated drop-down list (Figure E1). Set the fluidics to Slow rate (Under Fluidics Control in the Control Panel).

Figure E1 Run Limit: 50000 Events.

#### Run 8-Peak Validation Beads

- Open the current 8- and 6- peak validation workspace.

Prepare a sample of suspended 8-peak validation beads by adding 4 drops of the beads to 1 ml Ultra-pure water. A new set of validation beads is made up at the beginning of each test.

Vortex and place the tube on the Sample Introduction Platform (SIP).

Select the next empty data cell following the previous 8- peak validation. If this is the first run in a new template, select A1. Make sure the data cell in the CFlow template is empty before starting the run. If the button displays ADD TO, the cell already contains data.

Check that the fluidics is set to Slow

Click on the RUN button to start acquisition. Acquisition automatically stops after 50,000 total events are acquired.

Name the sample by typing a name in the text box just above the Sample Grid. Include the date in the sample name to differentiate it from samples collected on other dates (Figure E2). Samples can be named before, during or after collection.

When the collection is finished, remove the sample tube

Back flush the SIP. This can be achieved by either selecting the back flush icon or Instrument > Run Back flush. Ensure a blotter or empty sample tube is under the SIP to catch the dripping fluid. Wipe excess fluid from the SIP with a lint-free Kimwipe.

NOTE: The R1 region may not encompass the main population of bead events on the FSC-H vs. SSC-H plot. This is common and acceptable at this stage.

Figure E2 Sample Name: 8-Peak Beads.

#### Run 6-Peak Validation Beads

- Prepare a sample of suspended 6-peak validation beads by adding 3 drops each of Peak 1 and Peak 2 beads to 1 ml of Ultra-pure water.

Vortex and place the tube on the SIP.

Select the next empty data cell following the previous 6- peak validation and name the sample. If this is the first run in a new template, select E1 (Figure E3).

Again, verify that Events is still enabled and set at 50,000 in Ungated Sample and the fluidics is set to Slow.

Click on the RUN button.

Name the sample including the date in which it was processed.

NOTE: The R2 region may not encompass the main population of bead events on the FSC-H vs. SSC-H plot. This is common and acceptable at this stage.

Figure E3 Sample Name: 6-Peak Beads.

#### Saving validation bead data

By default, CFlow automatically saves validation bead data at the end of each run.

Sample and prompts you to save when moving between different workspaces. You can also save data manually at any time by selecting File > Save.

#### Analysing and recording validation bead data

After you collect the bead data, analyse the data using the Collect tab of CFlow to ensure that the C6 is functioning properly. The template has already predefined appropriate gate settings for the 8-peak and 6-peak validations.

To analyse the bead data:

- Click on the well that contains the most recent 8-peak bead data.

On the first FSC-H vs. SSC-H plot (scatter plot) in the bead file, adjust the pre-drawn region (R1) to encompass the main population by dragging the border of the region (Figure E4). R1 should contain more than 75% of all events.

NOTE: There is usually a “shadow” population (called bead doublets or clumps) that is slightly higher in FSC-H than the main cluster of beads; this is normal for these beads. Do not include the shadow group in R1.

Figure E4 Plot with Bead Doublets.

Verify that the next three plots (FL1-H, FL2-H, and FL3-H) are gated on scatter region R1 and that the plots display the message R1 in all next to the GATE button (Figure E5). If it is not displayed, click on the GATE button and select R1in all events from the pop-up dialog box.

NOTE: Gates may need to be moved slightly when a new packet of beads are opened as different batches of beads have slightly different characteristics. Move the gate to encompass the main population of beads but not the “shadow” population.

Figure E5 Example of data for 8-peak beads. The number of expected peaks is: FL1-H (8), FL2-H (8) and FL3-H (6). The peaks in FL4-H are not relevant.

#### Monitoring the Validation Bead data

A convenient way to monitor the Validation Bead data, and thus the C6 performance, is to store the 8- and 6- peak validation bead data in a single CFlow file, saving the data from each day in its own cell. Then, using the Statistics tab, a table can be created of the Mean channel numbers and Coefficient of Variation (CVs) for the top peaks, and for the forward scatter of the beads to make it easy to determine if the C6 performance is stable.

However, on a daily basis, it is sufficient to record the R1 and R2 percentages along with the CV for each of the M1, M2 and M4 peaks in the lab record book. This allows the user to quickly compare the results with previous entries. The R1 and R2 percentages should be over 75% and the FSC-H, FL1-H and FL2-H CVs should be less than 6%.

## 3  Final clean

- When the collection is finished, remove the sample tube from the SIP and wipe off the end of the SIP with a lint-free tissue.

Back flush the SIP.

Open the workspace “H2O start-up” in the maintenance folder and advance to the next empty data cell.

Check that the Time check box (Min Sec) in the Instrument Control Panel is set to ten minutes and that the fluidics are set to Fast

Click on the RUN button.

When the run is finished, leave the tube on the SIP.

The C6 is ready for experimental work after successful completion of the validation beads analyses.

### 4  Analysis of test samples

A template specifically for Chlorella sp., has been created and the file is named Chlorella TEMPLATE. Templates will have file name ending with .c6t (e.g. Chlorella TEMPLATE.c6t). The Chlorella TEMPLATE has predefined settings in place specifically tailored for Chlorella sp., algal cells. Figure E6 shows healthy algal counts at 48h in the template workspace.

To open a file template select File > Open CFlow File or Template >Templates> Chlorella TEMPLATE.c6t. Automatically, a blank workspace will open with all required parameters set. The C6 will prompt you to save this file before the first analysis can proceed.

Figure E6 Typical control Chlorella sp., count setting at 48 h.

#### 4.1  Preparing a sample

- Thoroughly mix the sample so that a representative sub-sample can be taken. Scrape the bottom of the flask using a Teflon scraper before sub-sampling.

Pipette a sub-sample (min. 500µL) into a tissue grinder and homogenise the sample. Pour the sample into sample tube. Suitable tubes for algal samples are polystyrene falcon tubes with round bottoms. Algal cells can stick to the surface of the container so it is recommended that samples are prepared just before being analysed. I.e. do not allow the sample to sit for longer than a few minutes.

#### 4.2  Analysing a sample

- Shake or agitate the sample in the tube before placing it onto the SIP.

If you are working from a predefined template, all data cells should have been pre-labelled with the sample ID. If you are not working from a template, type your sample ID in the text box. Click on the data cell.

Check that run limits and fluidics settings are correct. If you are working from a predefined template, these parameters should already be set. Chlorella sp., fluidics should be set to Medium speed and 25 µL sample volume.

Ensure the traffic light is green.

If green, then click the Run button.

Repeat steps 2-5 twice more for triplicate readings. Several readings can be done on the same sample to achieve higher replication.

Back flush the fluidics line before moving on to a new treatment.

Repeat the above steps for the remainder of the test.

For the Chlorella sp., template, the region being counted is gated to the P2 region (Figure E7).

- Figure E7 Chlorella sp. gated region counts vs. full plot counts

## 5  Shutdown procedure

When you finish collecting samples, rinse out the SIP to ensure cells or other particles are not left in the SIP.

- Open the ‘H2O shutdown’ file in the Maintenance folder.

Place a tube with 2 ml of filtered, de-ionized water on the SIP and advance to any empty data cell.

It should be set to run for 10 minutes.

Click on the RUN button.

Save this file and open the ‘Clean’ file in the Maintenance folder. Place a tube with 2 ml of extended clean solution on the SIP.

Select an empty data cell.

The time should be set to two minutes and the fluidics speed to fast.

Click on the RUN button.

Once the run is finished, remove the tube of cleaning solution from the SIP.

Select the back flush button in the workspace and wait for this process to run.

Re-open the ‘H2O shutdown’ file.

Place a tube with 2 ml of ultra-pure water on the SIP.

Select an empty data cell.

Click on the RUN button.

Once this is complete turn the C6 off.

# Appendix F
Calculation of algal growth constant

At any time (t), the number of cells (N) is expressed as:

N=N0eμt	 (1)

Where µ is the specific rate constant.

The actual rate is  dNdt

The specific growth rate - µ = 1t   lnNN0 (Take logarithm of eq. 1.)	 (2)

Convert to natural log

μ= 1tlogNN0 x 2.303	(3)

Meanwhile, using eq. 3 to take the time for the number of cells to double yield.

μ= 1t2x log2N0N0 2.303= 1t2xln2	(4)

Rate of doubling

1t2x= μln2	(5)

Rate of doubling per day

μln2 x 24h/day	(6)

Since μ= 1tlogNN02.303

Then substituting into eq. 6.

Rate of doubling per day

1tlogNN0 (2.303 ÷ln2) x 24

∴ 1tlogNN0 3.32 x 24

Where µ = slope of log N x t in hrs

#### Specific growth rate (µ)

μ= 1tlnNN0 	 (1)

Convert to logs

μ= 1tlogNN0 2.303 	(2)

Meanwhile, using eq. 2, calculate the doublings, i.e. time for the number of cells to double yield

μ=1t2xlog2N0N0 2.303	 (3)

Convert to natural log

μ= 1t2xln2 (Note: log2 N0N0 = log 2) 	 (4)

Now, “rate of doubling” is 1t2x therefore using equation (4)

“Rate of doubling” = 1t2x= μ ÷ln2 	 (5)

And since μ= 1tlogNN0 2.303 	(eq. 2.)

Then “rate of doubling” = 1tlogNN0 2.303 ÷ln2 	 (6)

Then rate of doubling per day = 1tlogNN0 3.32 x 24h/day

And say 1tlogNN0= μ, µ is the slope of log N vs. t(hrs)

Therefore rate of doubling per day = µ x 3.32 x 24

t

N

N0

At any time t, the number of cells (N) is expressed as

N= N0eμt 	 (1)

where µ is the specific rate constant for the cell division

The actual rate is given by

Rate =dNdt 	 (2)

i.e. the change in number of cells, dN, over a period of time, dt

Taking the log of eq. 1 and further simplifying gives:

μ= 1tlnNN0= 1tlogNN0 2.303 	 (3)

i.e. 2.303 comes about due to a change from natural log (ln) to log10

#### Doublings per day

Using equation 3 -  μ= 1t logNN02.303

i.e. t= 1μ logNN02.303 	 (4)

the time for the number of cells to double yield is (t2x)

t2x= 1μlog2N0N02.303

= 1μlog22.303 	 (5)

Convert back to natural logs (ln) then;

t2x= 1μln2 	 (6)

Now, “Rate of doubling” = 1t2x therefore reciprocal of (6)

Rate of doubling = μ1ln2 	 (7)

And from equation 3

μ= 1tlogNN02.303

∴ rate of doubling = 1tlogNN0 x 2.303ln2 24h/day

= μ x 3.32 x 24h/day

μ= 1tlog NN0

#### Specific growth rate (µ)

μ= 1tlnNN0 	 (1)

Convert to log;

μ= 1tlogNN02.303 	 (2)

Meanwhile, using eq. 2, calculate doublings.

i.e. time for the number of cells to double yield (2N0)

μ= 1t2xlog2N0N0 2.303 	 (3)

Convert to natural log - μ=1t2xln2	 (4)

Since log2N0N0=log2

Now, “rate of doubling” is 1t2x therefore using equation 4

‘Rate of doubling’ = 1t2x= μ ÷ln2	 (5)

And since μ= 1tlogNN02.303 (eq. 2)

Then “rate of doubling” = 1tlogNN02.303 ÷ln2 	 (6)

And say; 1tlogNN0=μ, then µ is the slope of log N vs. t(h)

μt=logN-logN0

logN= μt+logN0

Therefore ‘rate of doubling’ = μ x 2.303ln2

Therefore doublings per day = μ x 2.303ln2x 24h/day

and hence doublings/day = μ x 3.32 x 24 where µ = slope of log N v t(h).

# Appendix G
Example end of test worksheet

Figure G1 Algal population growth rate calculation table

# Appendix H
Lists of toxicity tests performed to determine nutrient concentrations and subsequent reference toxicity tests

Table H1 Details of each of the tests undertaken to determine the minimum amount of nutrients that can be added to a test and result in an acceptable population growth rate

| Test code | Test start date | SSW production date | Concentrations tested (µg L-1) |  | Valid test? (Y/N) |
| --- | --- | --- | --- | --- | --- |
| 1366G | 05/11/13 | 16/10/13 | N | 453, 907, 1209, 1814, 3628, 7255, 14510 | Y |
|  |  |  | P | 4.31, 8.63, 11.5, 17.3, 34.5, 69, 138 |  |
| 1373G | 25/11/13 | 15/11/13 | N | 907, 1209, 1814, 2418, 3628 | Y |
|  |  |  | P | 8.63, 11.5, 17.3, 23, 34.5 |  |
| 1376G | 09/12/13 | 6/12/14 | N | 227, 453, 605, 907, 1209, 1814 | Y |
|  |  |  | P | 2.16, 4.31, 5.75, 8.63, 11.5, 17.3 |  |
| 1378G | 13/01/14 | 10/1/14 | N | 907, 1209, 1451, 1814, 2418 | Y |
|  |  |  | P | 8.63, 11.5, 13.8, 17.3, 23 |  |
| 1380G | 20/01/14 | 10/1/14 | N | 1209, 1451, 1814, 2418, 2902 | Y |
|  |  |  | P | 11.5, 13.8, 17.3, 23, 27.6 |  |

Table H2 Details of each of the reference toxicity tests performed with the reduced starting algal density and reduced nutrients

| Test Code | Test start date | SSW production date | U concentrations tested (µg L-1) | Valid test? (Y/N) |
| --- | --- | --- | --- | --- |
| 1383G | 03/02/14 | 31/01/14 | 0, 5, 10, 20, 40, 80, 160 | Y |
| 1389G | 18/02/14 | 17/02/14 | 0, 5, 10, 20, 40, 80, 160 | Y |
| 1404G | 13/05/14 | 11/05/14 | 0, 5, 10, 20, 40, 80, 160 | Y but temp problems |
| 1415G | 05/08/14 | 22/07/14 | 0, 5, 10, 20, 40, 80, 160 | Y |
| 1431G | 09/12/14 | 04/12/14 | 0, 5, 10, 20, 40, 80, 160 | Y |
| 1455G | 17/03/15 | 07/03/15 | 0, 0 no HEPES, 20, 40, 80, 160 | Y |
| 1466G | 09/06/15 | 28/05/15 | 0, 0 no HEPES, 20, 40, 80, 160 | Y |
| 1468G | 22/06/15 | 12/06/15 | 0, 0 no HEPES, 20, 40, 80, 160 | Y |
| 1479G | 07/09/15 | 04/09/15 | 0, 20, 40, 80, 160 | Y |

# Appendix I  Water Quality measurements

Table I1  Nutrient reduction tests

1366G Alg_@3x103_03

| Treatment (%) | 0.031 |  | 0.063 |  | 0.125 |  | 0.250 |  | 0.5 |  | 1 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h |
| pH | 6.1 | 6.3 | 6.1 | 6.3 | 6.1 | 6.3 | 6.2 | 6.3 | 6.1 | 6.3 | 6.2 | 6.4 |
| EC (µS cm-1) | 22 | 22 | 22 | 22 | 24 | 24 | 27 | 27 | 34 | 34 | 48 | 48 |
| DO (%) | 112 | 94 | 110 | 93 | 109 | 95 | 110 | 95 | 107.5 | 95 | 108.1 | 94 |
| Temp (°C) | 22.6 | 24 | 22.4 | 24.3 | 22.1 | 24.4 | 22.1 | 24.6 | 22 | 24.9 | 21.8 | 24.9 |

1373G Alg_@3x103_04

| Treatment (%) | 0.063 |  | 0.083 |  | 0.125 |  | 0.167 |  | 0.25 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h |
| pH | 6.1 | 6.2 | 6.1 | 6.2 | 6.1 | 6.2 | 6.1 | 6.2 | 6.1 | 6.2 |
| EC (µS cm-1) | 21 | 21 | 22 | 22 | 23 | 22 | 24 | 23 | 26 | 26 |
| DO (%) | 103 | 91 | 109 | 93 | 108 | 93 | 106 | 93 | 106.7 | 91.5 |
| Temp (°C) | 22.4 | 24 | 22.5 | 24.3 | 22.4 | 24 | 22.1 | 24.2 | 21.9 | 24.7 |

1376G Alg_@3x103_05

| Treatment (%) | 0.016 |  | 0.031 |  | 0.042 |  | 0.063 |  | 0.083 |  | 0.125 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h |
| pH | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 | 6.1 | 6.2 |
| EC (µS cm-1) | 20 | 20 | 20 | 20 | 20 | 20 | 21 | 21 | 22 | 22 | 23 | 22 |
| DO (%) | 110 | 90 | 111 | 92 | 108 | 89 | 104 | 91 | 104.5 | 89.7 | 104.9 | 91.3 |
| Temp (°C) | 24 | 24.3 | 24 | 24.9 | 23.7 | 25.1 | 23.1 | 25.3 | 22.5 | 24.3 | 22 | 24.6 |

1378G Alg_@3x103_06

| Treatment (%) | 0.063 |  | 0.083 |  | 0.100 |  | 0.125 |  | 0.167 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h |
| pH | 6.0 | 6.0 | 6.0 | 6.1 | 6.0 | 6.1 | 6.0 | 6.0 | 6.0 | 6.0 |
| EC (µS cm-1) | 21 | 21 | 23 | 23 | 22 | 22 | 23 | 22 | 24 | 24 |
| DO (%) | 109 | 96 | 114 | 92 | 110 | 88 | 103 | 93 | 107.8 | 92.5 |
| Temp (°C) | 25 | 22.9 | 25.2 | 23.4 | 24.7 | 23.5 | 23.1 | 25 | 22.8 | 24.4 |

1380G Alg_@3x103_07

| Treatment (%) | 0.083 |  | 0.100 |  | 0.125 |  | 0.167 |  | 0.2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h |
| pH | 6.0 | 6.1 | 6.0 | 6.1 | 5.9 | 6.1 | 5.9 | 6.1 | 5.9 | 6.0 |
| EC (µS cm-1) | 22 | 24 | 22 | 22 | 23 | 23 | 24 | 24 | 25 | 24 |
| DO (%) | 104 | 85 | 108 | 85 | 105 | 91 | 103 | 89 | 102.5 | 89 |
| Temp (°C) | 26.4 | 25.4 | 25.9 | 25.4 | 25.5 | 25.7 | 25.5 | 24.9 | 24.2 | 24.9 |

Table I2 Reference toxicity tests

1383G Alg_Reftox_01

| Treatment (µg L-1) | 0 |  | 5 |  | 10 |  | 20 |  | 40 |  | 80 |  | 160 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h |
| pH | 6.1 | 6.3 | 6.0 | 6.2 | 6.1 | 6.2 | 6.0 | 6.2 | 6.0 | 6.1 | 6.0 | 6.1 | 6.0 | 6.1 |
| EC (µS cm-1) | 27 | 26 | 27 | 26 | 26 | 26 | 26 | 25 | 26 | 26 | 26 | 26 | 27 | 26 |
| DO (%) | 113.1 | 94.6 | 115.6 | 90.8 | 109.1 | 92.8 | 110 | 93.5 | 106.3 | 90.1 | 107.4 | 93 | 104.9 | 90.7 |
| Temp (°C) | 25.5 | 24.4 | 25.1 | 24.8 | 24.8 | 23.7 | 23.8 | 25.1 | 23.6 | 24.1 | 23.2 | 25.5 | 22.9 | 24.9 |

1389G Alg_Reftox_02

| Treatment (µg L-1) | 0 |  | 5 |  | 10 |  | 20 |  | 40 |  | 80 |  | 160 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h |
| pH | 6.0 | 6.2 | 6.0 | 6.2 | 6.0 | 6.2 | 6.0 | 6.1 | 6.0 | 6.1 | 5.9 | 6.0 | 6.0 | 6.0 |
| EC (µS cm-1) | 26 | 25 | 27 | 26 | 26 | 25 | 26 | 26 | 26 | 26 | 27 | 27 | 26 | 27 |
| DO (%) | 104.8 | 94.5 | 105.5 | 91.4 | 105.6 | 92.1 | 98.7 | 93 | 106.0 | 95.2 | 105.1 | 91.1 | 103.8 | 93.3 |
| Temp (°C) | 25.4 | 25.4 | 25.6 | 25.7 | 25.3 | 25.8 | 20.7 | 25.7 | 24.7 | 25.8 | 25.2 | 24.7 | 24.8 | 25.8 |

1404G Alg_Reftox_03

| Treatment (µg L-1) | 0 |  | 5 |  | 10 |  | 20 |  | 40 |  | 80 |  | 160 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h |
| pH | 6.1 | 6.2 | 6.1 | 6.2 | 6.1 | 6.2 | 6.1 | 6.2 | 6.1 | 6.2 | 6.1 | 6.1 | 6.1 | 6.2 |
| EC (µS cm-1) | 29 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 27 |
| DO (%) | 95.2 | 88.8 | 101.1 | 91.4 | 100.7 | 91.9 | 97.3 | 92.3 | 100.5 | 90.3 | 99.8 | 90.3 | 93.9 | 92.2 |
| Temp (°C) | 27 | 24.9 | 25 | 26.1 | 25 | 25.3 | 24.1 | 26.5 | 24.1 | 26.2 | 23.8 | 25.2 | 23.7 | 25.8 |

1415G Alg_Reftox_04

| Treatment (µg L-1) | 0 |  | 5 |  | 10 |  | 20 |  | 40 |  | 80 |  | 160 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h |
| pH | 5.8 | 6.0 | 5.8 | 6.0 | 5.8 | 6.0 | 5.9 | 6.1 | 5.9 | 5.9 | 5.9 | 5.9 | 5.9 | 5.8 |
| EC (µS cm-1) | 24 | 24 | 24 | 24 | 24 | 24 | 24 | 25 | 24 | 24 | 24 | 25 | 24 | 25 |
| DO (%) | 111.7 | 91.7 | 114.3 | 91.8 | 112.5 | 92.6 | 110.7 | 92.3 | 102.9 | 88.4 | 114.6 | 88.2 | 111.8 | 83.1 |
| Temp (°C) | 26.6 | 25.5 | 26.6 | 25.5 | 26.4 | 25.4 | 26.2 | 23.8 | 25.8 | 24.8 | 25.5 | 25.4 | 25.1 | 25.1 |

1431G Alg_Reftox_05

| Treatment (µg L-1) | 0 |  | 5 |  | 10 |  | 20 |  | 40 |  | 80 |  | 160 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h |
| pH | 6.0 | 6.1 | 6.0 | 6.1 | 5.9 | 6.1 | 6.0 | 6.1 | 6.0 | 6.1 | 6.1 | 6.0 | 6.0 | 6.0 |
| EC (µS cm-1) | 28 | 27 | 28 | 28 | 28 | 27 | 28 | 26 | 27 | 26 | 27 | 27 | 27 | 28 |
| DO (%) | 104.9 | 92.2 | 106.9 | 92.4 | 108.8 | 95.9 | 102.9 | 94.4 | 103.3 | 95.3 | 102.3 | 92.4 | 101.1 | 94.9 |
| Temp (°C) | 26.9 | 23 | 24.6 | 24.8 | 24.3 | 26.1 | 23.8 | 25.5 | 23.8 | 25.4 | 22.4 | 25.7 | 22 | 26.2 |

1455G Alg_Reftox_06

| Treatment (µg L-1) | 0 |  | 0 – No HEPES |  | 20 |  | 40 |  | 80 |  | 160 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h |
| pH | 6.0 | 6.3 | 5.9 | 6.4 | 6.0 | 6.2 | 5.9 | 6.1 | 5.9 | 6.1 | 5.9 | 6.1 |
| EC (µS cm-1) | 26 | 27 | 23 | 23 | 26 | 26 | 26 | 26 | 26 | 27 | 26 | 27 |
| DO (%) | 106.6 | 93.2 | 113.9 | 93.7 | 107.2 | 92.5 | 116.2 | 93.5 | 116.9 | 92.5 | 109.9 | 93.2 |
| Temp (°C) | 25.2 | 24.8 | 26.2 | 25.4 | 25 | 25.8 | 26.5 | 25.4 | 26.3 | 25.1 | 25.6 | 25.7 |

1466G Alg_reftox_07

| Treatment (µg L-1) | 0 |  | 0 – No HEPES |  | 20 |  | 40 |  | 80 |  | 160 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h |
| pH | 6.0 | 6.1 | 6.1 | 6.6 | 5.9 | 6.0 | 6.0 | 6.0 | 5.9 | 6.0 | 6.0 | 6.0 |
| EC (µS cm-1) | 26 | 25 | 23 | 22 | 26 | 26 | 26 | 26 | 26 | 26 | 26 | 26 |
| DO (%) | 114.6 | 95.1 | 119.5 | 95.8 | 115.9 | 94.2 | 115.9 | 93.6 | 113.3 | 93.4 | 106.1 | 23.3 |
| Temp (°C) | 25.7 | 26.2 | 26 | 26 | 26.2 | 26 | 26.2 | 26.1 | 26.1 | 25.8 | 25.7 | 26.1 |

1468G Alg_reftox_08

| Treatment (µg L-1) | 0 |  | 0 – No HEPES |  | 20 |  | 40 |  | 80 |  | 160 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h |
| pH | 6.2 | 6.6 | 6.2 | 6.3 | 6.1 | 6.2 | 6.1 | 6.2 | 6.1 | 6.1 | 6.1 | 6.1 |
| EC (µS cm-1) | 24 | 23 | 27 | 26 | 27 | 27 | 27 | 27 | 27 | 27 | 27 | 27 |
| DO (%) | 105 | 95.5 | 106.7 | 95.7 | 105.8 | 95.3 | 103.4 | 93.4 | 104.7 | 94.4 | 104.9 | 94 |
| Temp (°C) | 26.3 | 24.9 | 26.3 | 25.5 | 26.7 | 25.7 | 26.7 | 25.7 | 26.1 | 26 | 26.3 | 26.2 |

1479G Alg_reftox_09

| Treatment (µg L-1) | 0 |  | 20 |  | 40 |  | 80 |  | 160 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Parameter | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h | 0h | 72h |
| pH | 6.1 | 6.2 | 6.1 | 6.1 | 6.1 | 6.0 | 6.1 | 6.0 | 6.1 | 6.0 |
| EC (µS cm-1) | 27 | 26 | 27 | 26 | 27 | 27 | 27 | 27 | 27 | 27 |
| DO (%) | 100.4 | 93.5 | 106.4 | 95.2 | 107.6 | 92.9 | 94.8 | 91.9 | 103.1 | 91.6 |
| Temp (°C) | 25.7 | 25.1 | 27.6 | 25.5 | 27.7 | 25.3 | 27.7 | 25.5 | 26.9 | 25.6 |

# Appendix J  Chemical analyses

Table J1 Metal and major ion analyses of Quality Control (QC) waters for nutrient reduction trials.

| Analyte | Al | Cd | Co | Cr | Cu | Fe | Mn | Ni | Pb | Se | U | Zn | Ca | Mg | Na | SO4 |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Units | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | mg L-1 | mg L-1 | mg L-1 |
| PQL | 0.1 | 0.02 | 0.01 | 0.1 | 0.01 | 1 | 0.01 | 0.01 | 0.01 | 0.2 | 0.001 | 0.1 | 0.1 | 0.1 | 0.1 | 0.5 |
| 1366G SSW | 50 | <0.02 | 0.01 | <0.1 | 0.8 | 350 | 10 | 0.04 | 0.2 | <0.2 | 0.2 | 0.8 | 0.5 | 0.6 | 1.3 | 3 |
| 1373G SSW | 10 | <0.02 | 0.02 | <0.1 | 0.7 | 77 | 11 | 0.06 | 0.04 | <0.2 | 0.06 | 0.7 | 0.4 | 0.6 | 2.4 | 96 |
| 1376G SSW | 20 | <0.02 | 0.02 | <0.1 | 0.4 | 120 | 10 | 0.05 | 0.2 | <0.2 | 0.05 | 0.8 | 0.5 | 0.7 | 2.4 | 110 |
| 1378G SSW | 20 | <0.02 | 0.02 | <0.1 | 0.4 | 120 | 9 | 0.06 | 0.08 | <0.2 | 0.06 | 0.7 | 0.4 | 0.6 | 2.6 | 110 |
| 1380G SSW | 16 | <0.02 | 0.02 | <0.1 | 0.59 | 130 | 9.5 | 0.05 | 0.08 | <0.2 | 0.051 | 0.7 | 0.4 | 0.6 | 2.7 | 110 |
| 1366G Blank | <0.1 | <0.02 | <0.01 | <0.1 | 0.02 | <1 | <0.01 | 0.03 | <0.01 | <0.2 | <0.001 | <0.1 | <0.1 | <0.1 | <0.1 | <0.5 |
| 1373G Blank | <0.1 | <0.02 | <0.01 | <0.1 | 0.09 | <1 | <0.01 | <0.01 | <0.01 | <0.2 | <0.001 | <0.1 | <0.1 | <0.1 | <0.1 | 0.5 |
| 1376G Blank | 0.2 | <0.02 | <0.01 | <0.1 | 0.05 | <1 | <0.01 | <0.01 | 0.01 | <0.2 | <0.001 | <0.2 | <0.1 | <0.1 | <0.1 | <0.5 |
| 1378G Blank | 1 | <0.02 | <0.01 | <0.1 | 0.02 | <1 | <0.01 | <0.01 | <0.01 | <0.2 | 0.009 | 0.2 | <0.1 | <0.1 | <0.1 | <0.5 |
| 1380G Blank | 0.1 | <0.02 | <0.01 | <0.1 | 0.03 | <1 | <0.01 | <0.01 | <0.01 | <0.2 | <0.001 | <0.1 | <0.1 | <0.1 | <0.1 | <0.5 |
| 1366G Pro Blank | 0.1 | <0.02 | <0.01 | <0.1 | 0.03 | <1 | <0.01 | 0.05 | 0.01 | <0.2 | 0.005 | <0.1 | <0.1 | <0.1 | <0.1 | <0.5 |
| 1373G Pro Blank | 0.1 | <0.02 | <0.01 | <0.1 | 0.04 | <1 | <0.01 | 0.01 | <0.01 | <0.2 | 0.02 | <0.1 | <0.1 | <0.1 | <0.1 | 0.5 |
| 1376G Pro Blank | 0.2 | <0.02 | <0.01 | <0.1 | 0.05 | <1 | <0.01 | <0.01 | 0.01 | <0.2 | 0.02 | <0.2 | <0.1 | <0.1 | 0.1 | <0.5 |
| 1378G Pro Blank | 1 | <0.02 | <0.01 | <0.1 | 0.02 | <1 | <0.01 | <0.01 | <0.01 | <0.2 | 0.009 | <0.1 | <0.1 | <0.1 | <0.1 | <0.5 |
| 1380G Pro Blank | 1.6 | <0.02 | <0.01 | <0.1 | 0.03 | <1 | 2.7 | <0.01 | 0.02 | <0.2 | 0.020 | <0.1 | <0.1 | <0.1 | <0.1 | <0.5 |

*Highlighted numbers are higher than would be expected in normal synthetic soft water

Table J2 Measured nitrate and phosphate concentrations in QC waters in nutrient reduction trials.

| Test code | Proportion of current nutrients | Measured nutrient concentrations |  |
| --- | --- | --- | --- |
|  |  | Nitrate as N (mg L-1) | Phosphate as P (mg L-1) |
| 1366G | 1/32 | 0.070 | <0.005 |
|  | 1/16 | 0.30 | <0.005 |
|  | 1/8 | 0.25 | <0.005 |
|  | 1/4 | 0.53 | <0.005 |
|  | 1/2 | 0.70 | 0.006 |
|  | 1 | 1.3 | 0.016 |
| 1373G | 1/16 | 0.11 | <0.005 |
|  | 1/12 | 0.17 | <0.005 |
|  | 1/8 | 0.20 | <0.005 |
|  | 1/6 | 0.22 | <0.005 |
|  | 1/4 | 0.47 | 0.005 |
| 1376G | 1/64 | 0.18 | <0.005 |
|  | 1/32 | 0.16 | <0.005 |
|  | 1/24 | 0.23 | <0.005 |
|  | 1/16 | 0.22 | <0.005 |
|  | 1/12 | 0.27 | <0.005 |
|  | 1/8 | 0.32 | <0.005 |
| 1378G | 1/16 | 0.16 | <0.005 |
|  | 1/12 | 0.12 | <0.005 |
|  | 1/10 | 0.098 | <0.005 |
|  | 1/8 | 0.20 | <0.005 |
|  | 1/6 | 0.31 | <0.005 |
| 1380G | 1/12 | 0.12 | <0.005 |
|  | 1/10 | 0.21 | <0.005 |
|  | 1/8 | 0.21 | <0.005 |
|  | 1/6 | 0.14 | <0.005 |
|  | 1/5 | 0.34 | <0.005 |

Table J3 Metal and major ion analyses of QC waters for reference toxicity tests

| Analyte | Al | Cd | Co | Cr | Cu | Fe | Mn | Ni | Pb | Se | U | Zn | SO4 | Ca | Na | Mg |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Units | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | mg L-1 | mg L-1 | mg L-1 | mg L-1 |
| PQL | 0.1 | 0.02 | 0.01 | 0.1 | 0.01 | 1 | 0.01 | 0.01 | 0.01 | 0.2 | 0.001 | 0.1 | 0.5 | 0.1 | 0.1 | 0.1 |
| 1383G Blk | 0.3 | <0.02 | <0.01 | <0.1 | 0.02 | <1 | <0.01 | <0.01 | <0.01 | <0.2 | <0.001 | <0.1 | <0.5 | <0.1 | <0.1 | <0.1 |
| 1389G Blk | 0.3 | <0.02 | <0.01 | <0.1 | 0.03 | <1 | <0.01 | <0.01 | <0.01 | <0.2 | 0.006 | 1.6 | <0.5 | <0.1 | <0.1 | <0.1 |
| 1404G Blk | 0.1 | <0.02 | <0.01 | <0.1 | 0.02 | <1 | <0.01 | <0.01 | <0.01 | <0.2 | <0.001 | <0.1 | <0.5 | <0.1 | <0.1 | <0.1 |
| 1451G Blk | 0.4 | <0.02 | <0.01 | <0.1 | <0.01 | <1 | <0.01 | <0.01 | <0.01 | <0.2 | <0.001 | <0.1 | <0.5 | <0.1 | <0.1 | <0.1 |
| 1431G Blk | 0.2 | <0.02 | <0.01 | <0.1 | 0.07 | <1 | <0.01 | <0.01 | <0.01 | <0.2 | <0.001 | <0.1 | <1 | <0.1 | <0.1 | <0.1 |
| 1455G Blk | 0.5 | <0.02 | <0.01 | <0.1 | 0.08 | <1 | <0.01 | <0.01 | <0.01 | <0.2 | <0.001 | <0.1 | <0.5 | <0.1 | <0.1 | <0.1 |
| 1466G Blk | 0.2 | <0.02 | <0.01 | <0.1 | 0.06 | <1 | <0.01 | <0.01 | <0.01 | <0.2 | <0.001 | <0.1 | <0.5 | <0.1 | <0.1 | <0.1 |
| 1468G Blk | 0.3 | <0.02 | <0.01 | <0.1 | 0.06 | <1 | <0.01 | <0.01 | 0.01 | <0.2 | <0.001 | <0.1 | <0.5 | <0.1 | <0.1 | <0.1 |
| 1479G Blk | <0.1 | <0.02 | <0.01 | <0.1 | <0.01 | <1 | <0.01 | <0.01 | 0.01 | <0.2 | <0.001 | <0.1 | <0.5 | <0.1 | <0.1 | <0.1 |
| 1383G Pro Blk | 0.3 | <0.02 | <0.01 | <0.1 | 0.02 | <1 | <0.01 | <0.01 | <0.01 | <0.2 | <0.001 | <0.1 | <0.5 | <0.1 | <0.1 | <0.1 |
| 1389G Pro Blk | 0.4 | <0.02 | <0.01 | <0.1 | 0.03 | <1 | 0.01 | <0.01 | <0.01 | <0.2 | 0.009 | 3.1 | <0.5 | <0.1 | <0.1 | <0.1 |
| 1404G Pro Blk | 0.9 | <0.02 | <0.01 | <0.1 | 0.03 | <1 | <0.01 | <0.01 | 0.02 | <0.2 | 0.001 | <0.1 | <0.5 | <0.1 | <0.1 | <0.1 |
| 1415G Pro Blk | 9 | <0.02 | <0.01 | <0.1 | <0.01 | <1 | <0.01 | <0.01 | <0.01 | <0.2 | 0.008 | <0.1 | <0.5 | <0.1 | <0.1 | <0.1 |
| 1431G Pro Blk | 0.2 | <0.02 | <0.01 | <0.1 | 0.08 | <1 | <0.01 | <0.01 | <0.01 | <0.2 | <0.001 | 0.3 | <1 | <0.1 | <0.1 | <0.1 |
| 1455G Pro Blk | 1 | <0.02 | <0.01 | <0.1 | 0.07 | <1 | <0.01 | 0.01 | <0.01 | <0.2 | <0.001 | 0.4 | <0.5 | <0.1 | <0.1 | <0.1 |
| 1466G Pro Blk | 0.7 | <0.02 | 0.01 | <0.1 | 0.07 | <1 | <0.01 | 0.01 | 0.03 | <0.2 | 0.2 | 0.2 | <0.5 | <0.1 | <0.1 | <0.1 |
| 1468G Pro Blk | 0.9 | <0.02 | <0.01 | <0.1 | 0.08 | <1 | <0.01 | <0.01 | 0.04 | <0.2 | 0.03 | <0.1 | <0.5 | <0.1 | <0.1 | <0.1 |
| 1479G Pro Blk | 2 | <0.02 | <0.01 | <0.1 | 0.07 | 2 | 0.1 | 0.3 | 0.05 | <0.2 | 0.003 | 0.4 | <0.5 | <0.1 | <0.1 | <0.1 |
| 1383G A | 19 | <0.01 | 0.02 | <0.1 | 0.35 | 130 | 8.9 | 0.05 | 0.08 | <0.2 | 0.04 | 0.9 | 110 | 0.4 | 3.6 | 0.6 |
| Analyte | Al | Cd | Co | Cr | Cu | Fe | Mn | Ni | Pb | Se | U | Zn | SO4 | Ca | Na | Mg |
| Units | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | µg L-1 | mg L-1 | mg L-1 | mg L-1 | mg L-1 |
| PQL | 0.1 | 0.02 | 0.01 | 0.1 | 0.01 | 1 | 0.01 | 0.01 | 0.01 | 0.2 | 0.001 | 0.1 | 0.5 | 0.1 | 0.1 | 0.1 |
| 1389G A | 18 | <0.01 | 0.02 | <0.1 | 0.35 | 150 | 11 | 0.04 | 0.09 | <0.2 | 0.052 | 4.2 | 93 | 0.4 | 6.8 | 0.5 |
| 1404G A | 22 | <0.02 | 0.02 | <0.1 | 0.44 | 160 | 10 | 0.05 | 0.09 | <0.2 | 0.072 | 0.7 | 100 | 0.4 | 3.5 | 0.6 |
| 1415G A | 19 | <.0.02 | 0.02 | <0.1 | 0.6 | 120 | 10 | 0.04 | 0.08 | <0.2 | 0.06 | 0.7 | 100 | 0.5 | 1.1 | 0.6 |
| 1431G A | 10 | <0.02 | <0.01 | <0.1 | 0.4 | 89 | 9 | 0.04 | 0.09 | <0.2 | 0.03 | 0.8 | 84 | 0.4 | 3.6 | 0.5 |
| 1455G A | 10 | <0.02 | <0.01 | 0.1 | 0.5 | 78 | 9 | 0.1 | 0.08 | <0.2 | 0.02 | 0.8 | 110 | 0.5 | 3.6 | 0.6 |
| 1466G A | 8 | <0.02 | <0.01 | <0.1 | 0.5 | 50 | 9 | 0.1 | 0.2 | <0.2 | 0.8 | 1 | 110 | 0.4 | 3.7 | 0.6 |
| 1468G A | 12 | <0.02 | <0.01 | <0.1 | 0.6 | 74 | 9 | 0.02 | 0.1 | <0.2 | 0.07 | 2 | 3 | 0.5 | 2.5 | 0.6 |
| 1479G A | 12 | <0.02 | <0.01 | <0.1 | 0.7 | 68 | 9 | 0.2 | 0.1 | <0.2 | 0.05 | 1 | 110 | 0.5 | 0.6 | 3.4 |

*Highlighted numbers are higher than would be expected in normal synthetic soft water

Table J4 Measured uranium concentrations in reference toxicity tests.

| Test & treatment code | Nominal U concentration (µg L-1) | Measured U concentration (µg L-1) |
| --- | --- | --- |
| 1383G BLK | - | <0.001 |
| 1383G A | Bgd* | 0.044 |
| 1383G B | 5 | 3.8 |
| 1383G C | 10 | 7.3 |
| 1383G D | 20 | 16 |
| 1383G E | 40 | 28 |
| 1383G F | 80 | 56 |
| 1383G G | 160 | 130 |
| 1389G BLK | - | 0.006 |
| 1389G A | Bgd | 0.052 |
| 1389G B | 5 | 3.9 |
| 1389G C | 10 | 7.4 |
| 1389G D | 20 | 17 |
| 1389G E | 40 | 31 |
| 1389G F | 80 | 64 |
| 1389G G | 160 | 130 |
| 1404G BLK | - | <0.001 |
| 1404G A | Bgd | 0.072 |
| 1404G B | 5 | 3.8 |
| 1404G C | 10 | 7 |
| 1404G D | 20 | 14 |
| 1404G E | 40 | 27 |
| 1404G F | 80 | 56 |
| 1404G G | 160 | 110 |
| 1415G BLK | - | <0.001 |
| 1415G A | Bgd | 0.06 |
| 1415G B | 5 | 4.3 |
| 1415G C | 10 | 8.6 |
| 1415G D | 20 | 17 |
| 1415G E | 40 | 34 |
| 1415G F | 80 | 66 |
| 1415G G | 160 | 140 |
| 1431G BLK | - | <0.001 |
| 1431G A | Bgd | 0.03 |
| 1431G B | 5 | 4.2 |
| 1431G C | 10 | 8.4 |
| 1431G D | 20 | 18 |
| 1431G E | 40 | 35 |
| 1431G F | 80 | 70 |
| 1431G G | 160 | 140 |
| 1455G BLK | - | <0.001 |
| 1455G A | Bgd | 0.02 |
| 1455G B | Bgd – no HEPES | 0.02 |
| 1455G C | 20 | 15 |
| 1455G D | 40 | 31 |
| 1455G E | 80 | 62 |
| 1455G F | 160 | 130 |
| 1466G BLK | - | <0.001 |
| 1466G A | Bgd | 0.8 |
| 1466G B | Bgd – no HEPES | 0.03 |
| 1466G C | 20 | 15 |
| 1466G D | 40 | 26 |
| 1466G E | 80 | 54 |
| 1466G F | 160 | 110 |
| 1468G BLK | - | <0.001 |
| 1468G A | Bgd | 0.07 |
| 1468G B | Bgd – no HEPES | 0.1 |
| 1468G C | 20 | 14 |
| 1468G D | 40 | 25 |
| 1468G E | 80 | 48 |
| 1468G F | 160 | 97 |
| 1479G BLK | - | <0.001 |
| 1479G A | 0 | 0.05 |
| 1479G B | 20 | 16 |
| 1479G C | 40 | 30 |
| 1479G D | 80 | 63 |
| 1479G E | 160 | 140 |

*Bgd – indicates background uranium concentration in diluent waters

Table J5 Measured nitrate and phosphate concentrations in quality check waters in reference toxicity tests.

| Test code and treatment | Measured nutrient concentrations |  |
| --- | --- | --- |
|  | Nitrate as N (mg L-1) | Phosphate as P (mg L-1) |
| 1383G BLK | <0.005 | <0.005 |
| 1383G A | 0.5 | <0.005 |
| 1389G BLK | 0.015 | <0.005 |
| 1389G A | 0.44 | <0.005 |
| 1404G BLK | 0.054 | <0.005 |
| 1404G A | 0.69 | <0.005 |
| 1415G BLK | nma | nma |
| 1415G A | nma | nma |
| 1431G BLK | nma | nma |
| 1431G A | nma | nma |
| 1455G BLK | <0.005 | <0.005 |
| 1455G A | 0.77 | 0.008 |
| 1466G BLK | <0.005 | <0.005 |
| 1466G A | 0.32 | 0.006 |
| 1468G BLK | <0.005 | <0.005 |
| 1468G A | 0.4 | 0.005 |
| 1479G BLK | nm | nm |
| 1479G A | nm | nm |

a Samples lost due to power outage and defrosting of samples

# Appendix K  Analytical reports

Figure K1 Test raw data and analysis report for test 1366G

Figure K1 continued

Figure K2 Test raw data and analysis report for test 1373G

Figure K2 continued

Figure K3 Test raw data and analysis report for test 1376G

Figure K3 continued

Figure K4 Test raw data and analysis report for test 1378G

Figure K4 continued

Figure K5 Test raw data and analysis report for test 1380G

Figure K6 Test raw data and analysis report for test 1383G

Figure K6 continued

Figure K7 Test raw data and analysis report for test 1389G

Figure K7 continued

Figure K8 Test raw data and analysis report for test 1404G

Figure K8 continued

Figure K9 Test raw data and analysis report for test 1415G

Figure K9 continued

Figure K10 Test raw data and analysis report for test 1431G

Figure K10 continued

Figure K11 Test raw data and analysis report for test 1455G

Figure K11 continued

Figure K12 Test raw data and analysis report for test 1466G

Figure K12 continued

Figure K13 Test raw data and analysis report for test 1468G

Figure K13 continued

Figure K14 Test raw data and analysis report for test 1479G