Overview

The National assessment of chemicals associated with coal seam gas extraction in Australia was commissioned by the Department of the Environment and Energy and prepared in collaboration with NICNAS and CSIRO.

Assessment overview

It is important to understand what coal seam gas extraction means for human health and the environment. Coal seam gas extraction is closely regulated by state and the Commonwealth governments, which legally require protective measures to be in place to safeguard health and the environment. While the exact requirements vary by jurisdiction, this regulatory framework applies to all aspects of the industry, including the handling and use of chemicals throughout the extraction process.

To increase the knowledge base about chemicals used in the industry and in recognition of the scientific and community interest in the risks of this chemical use, the Australian Government commissioned the National assessment of chemicals associated with coal seam gas extraction in Australia in June 2012.

The Assessment examined 113 chemicals used by companies in Australia between 2010 and 2012 in drilling and hydraulic fracturing for coal seam gas, to develop a stronger understanding of the risks these chemicals could pose to the health of workers, the public and the environment. Industry reports that 59 of the 113 chemicals that were being used in coal seam gas extraction in 2010-12 were still being used in 2015-17.

The focus of the Assessment was solely on the above-ground (surface) handling of chemicals – it did not consider potential risks from chemicals entering deeper groundwater through drilling or fracturing operations. The Australian government has since commissioned additional research into deeper groundwater that found the risks to be very low. This is consistent with international studies that had shown that the greatest risk to human health or the environment from chemicals used in coal seam gas extraction is from spills or releases of chemicals during surface activities such as transport, handling, storage and mixing of chemicals. It is important to note that not all wells require fracturing and not all chemicals are used at all sites. Most of the chemicals are also commonly used in other industries. Some are used in homes.

The Assessment looked at scenarios in the coal seam gas extraction process where workers, the public and the environment could come into contact with the chemicals. The scenarios considered all parts of the extraction process including the activities listed above. Spills, leaks and accidents were the main release events identified in the scenarios. Worker scenarios also examined direct handling of the chemicals.

The Assessment took a very conservative approach, consistent with best practice, to ensure any pre-mitigation risks are not overlooked. It examined worst case scenarios and did not take into account all the safety and handling precautions that are taken to protect people and the environment from industrial chemical use. In reality, these precautions are required by law and significantly reduce any likelihood of potential harm occurring.

The Assessment found that the greatest pre-mitigation risk of harm to public health or the environment was in the event of a large-scale transport spill. Because they work with chemicals in more concentrated forms, the main pre-mitigation risks to coal seam gas workers is from industrial accidents and handling chemicals while maintaining equipment or mixing and blending. Even in this case, applying the required safety and handling precautions such as wearing protective equipment and promptly notifying and cleaning up spills, reduces the risk significantly.

Australia has a strict regulatory regime for coal seam gas operations that requires safety and handling precautions to prevent spills and promptly control, report and remediate them if they occur. Strict work, health and safety regulations are in place to protect workers. Comprehensive national standards apply to the handling and transport of chemicals for all industries, including the coal seam gas industry.

The Assessment has provided regulators and companies with an additional level of information directly relevant to the coal seam gas industry. This is new knowledge and information that will enable more targeted risk management actions and practices for the safe management of chemicals in coal seam gas operations.

What did the Assessment look at?

The Assessment looked at 113 chemicals used for drilling and hydraulic fracturing for coal seam gas in Australia within a sample time period from 2010 to 2012. At the time of the research, all coal seam gas operations were in NSW and Queensland. Each chemical was assessed for its potential risks to workers, the public and the environment in scenarios specific to the coal seam gas extraction process. Chemicals used in drilling and hydraulic fracturing can change over time, so not all of the 113 chemicals assessed would necessarily still be in use. For example, industry reports that 59 of the 113 chemicals that were being used in coal seam gas extraction in 2010-12 were still being used in 2015-17.

The focus of the Assessment was solely on the above-ground handling of chemicals. This was identified as a priority for research as international studies had shown that the greatest risk to human health or the environment is from spills or releases of chemicals during surface activities such as transport, handling, storage and mixing of chemicals.

As its focus was on surface handling, the Assessment did not assess potential risks from chemicals entering deeper groundwater, mixtures of chemicals, geogenics (chemicals in the coal seam or rock that are mobilised by the fracturing process), fugitive emissions and ambient air, shale or tight gas extraction, potential effects on agriculture or the food chain, or the individual risks of chemicals used at particular sites.

The Department of the Environment and Energy commissioned CSIRO to conduct further research on how chemicals used in coal seam gas extraction might move from deeper groundwater to other parts of the environment, how long this movement might take, and what the concentrations of chemicals might be at receiving environments such as water bores and streams. The research found that chemicals remaining underground after hydraulic fracturing are unlikely to reach people or ecosystems in concentrations that would cause concern. This conclusion is based on natural dilution and degradation that reduce concentrations to negligible levels. Risks are therefore likely to be very low. The results of this research are available on the Department’s website.

How was the Assessment done?

The Assessment was a complex, multi-agency Australian Government project. It is made up of 14 reports and reviews.

# Who was involved?

The Assessment was a collaborative effort of technical experts from the National Industrial Chemicals Notification and Assessment Scheme (NICNAS), the Commonwealth Scientific and Industrial Research Organisation (CSIRO), and the Department of the Environment and Energy.

The Assessment drew on technical expertise in risk assessment, chemistry, toxicology, ecotoxicology, hydrogeology, hydrology, geology and natural resource management.

The Independent Expert Scientific Committee on Coal Seam Gas and Large Coal Mining Development (IESC) provided independent advice and critical review at key points in the development of the Assessment.

Experts from Australia, the United States Environmental Protection Agency and Health Canada reviewed the Assessment and found it and its methods were consistent with international practice and unlikely to 
underestimate risks.

# What was involved?

The Assessment involved reviewing the scientific and technical literature and knowledge base, identifying the chemicals in use, developing new models and methodologies, and assessing the human health and environment risks of using the chemicals in coal seam gas extraction. Some of the methods had to be developed specifically for the Assessment. These methods can be applied in future risk assessments of chemicals proposed for use in coal seam gas extraction.

## Preparatory steps

Several preparatory steps were needed before the human health and environmental risks of chemicals used in coal seam gas extraction could be assessed.

The first step in the Assessment was to review the scientific and technical literature about coal seam gas extraction to collate what was already known about how chemicals are used in the process and the potential risks to human health and the environment.

NICNAS, CSIRO and the Department of the Environment and Energy conducted the literature reviews.

The reviews covered five topics:

how people could come into contact with chemicals during coal seam gas extraction, human health risk assessment methods, and regulation of chemicals—see the Human health implications review for details

how chemicals could enter the environment during coal seam gas production, how they could move through the environment and how they might affect the environment—see the Environmental risks review for details

integrity of coal seam gas wells and how fractures grow in coal seams after hydraulic fracturing—see the Hydraulic fracture growth and well integrity review for details

what chemicals occur naturally in coal seams and how they may be mobilised following hydraulic fracturing—see the Geogenic contaminants review for details

potential contamination of shallow groundwater by chemicals used in drilling and hydraulic fracturing—see the Leakage to shallow groundwater review for details.

See the Summary literature review for an overview of the literature reviews.

The second step in the Assessment was to identify 113 chemicals used in coal seam gas extraction in Australia between 2010 and 2012 from a survey of industry and publicly reported information on chemicals used in coal seam gas extraction.

During this phase of the Assessment, researchers also gathered information about how the chemicals were used during the coal seam gas extraction process and in what quantities.

See the

Identification of chemicals report for more details.

The third preparatory step was to develop models to show how chemicals might move from where they are released (most likely during an accidental leak or spill) to where humans and the environment could come into contact with them.

To do this, scientists identified the physical pathways the chemicals could travel along—for example, being washed off a surface into a stream or soaking into the soil and travelling through shallow groundwater.

The modelling stage also involved predicting the concentrations of chemicals that people or the environment could come into contact with, if the chemicals were released. These predictions were designed to overestimate the concentrations. For example, as some chemicals move through shallow groundwater they can ‘stick’ to soil particles, considerably reducing the concentration of chemical that humans or the environment might come into contact with, but the models did not take this into account. This ‘conservative’ approach is standard international best practice and ensures that any risks are not overlooked.

The models are described in detail in the:

- Human and environmental exposure conceptualisation report

- Environmental exposure conceptualisation report

- Predicted environmental concentrations report.

Before the human health risk assessment was performed, NICNAS assessed whether each of the chemicals was harmful (hazardous) to human health in its pure, concentrated form. This hazard assessment involved reviewing international studies and laboratory results in international and national databases.

Fifty-seven of the 113 chemicals were identified as harmful in their pure, concentrated form. This is because a risk arises only if humans come into contact with a chemical and at a high enough concentration to cause harm. For example, concentrated hydrochloric acid can be harmful if swallowed, spilled on the skin, splashed into the eye, or if people breathe the vapours. A chemical that is harmful in its pure, concentrated form may not necessarily cause harm when used in coal seam gas extraction at lower concentrations. State regulatory controls and industry practices on the use of drilling and hydraulic fracking chemicals are used to ensure that these risks are minimised. The Assessment went on to look at these chemicals in specific scenarios in the coal seam gas extraction process to identify the risks to human health. A risk arises only if people could come into contact with a high enough amount to cause harm. For example, the public is unlikely to come into contact with concentrated hydrochloric acid used for coal seam gas extraction and is therefore not at risk of harm. Coal seam gas workers, on the other hand, may handle concentrated hydrochloric acid and companies are required by law to take precautions against their workers coming into contact with it.

Results of the human health hazard assessment are detailed in the Chemicals of low concern and the Human health hazards of chemicals reports.

NICNAS also identified what the largest dose of a chemical could be before it would cause harm to human health. This dose (the ‘largest harmless dose’) was used later in the risk assessment.

## Human health risk assessment

NICNAS undertook the human health risk assessment. The first step in the risk assessment was to develop scenarios informed by the scientific literature and on-ground experience (through the industry survey), which defined how workers or the public might come into contact with chemicals used in coal seam gas extraction.

For workers, these scenarios involved skin contact, or breathing dusts or chemical vapours or contact with spills during the transport, storage and handling of chemicals. The Assessment looked at risks to workers when involved in:

transporting and storing chemicals

mixing or blending chemicals to produce formulations

injecting fluid formulations into the well

cleaning and conducting other maintenance activities

transporting and storing waste water (otherwise known as flowback and produced water)

an industrial accident resulting in contact with a chemical.

The general public are very unlikely to come into direct contact with chemicals as they are not involved in the day-to-day coal seam gas extraction process. It is generally only through large-scale accidental spills or leaks that are not detected and cleaned up that people could come into contact with the chemicals.

The Assessment looked at risks to the public in the following events:

a bulk spill occurs during transport of a chemical to a well site and it is not notified to the authorities, closed off to the public, or cleaned up, then a chemical enters surface water (such as a river) that is used for drinking, washing or swimming

a bulk spill occurs at a work site from a surface storage tank or pond and it is not detected, notified or cleaned up, then a chemical enters shallow groundwater or surface water that is used for drinking, washing or swimming

a long-term underground leak occurs from a waste water storage pond and it is not detected, notified or cleaned up, then a chemical enters shallow groundwater or surface water that is used for drinking, washing or swimming.

These scenarios were formed using very conservative assumptions. Importantly, these scenarios did not assume that legislated and standard precautions designed to protect people from chemicals were in place. These kinds of conservative assumptions are standard practice for this type of pre-mitigation assessment. For example, the scenarios did not consider that workers would be wearing personal protective equipment when handling potentially harmful chemicals though workplace health and safety laws require the use of such equipment. Similarly, the distances from the source of the chemical to where the chemical could enter the environment were considered to be shorter than they are likely to be in reality.

The second step in the risk assessment was to calculate the dose of a chemical a person might take in under each scenario. These doses are likely to be overestimated because the predicted concentration models overestimate the concentrations of chemicals and the scenarios do not take into account standard precautions which are applied to protect people from chemicals.

The final step in the risk assessment was to compare the dose a person might receive in the different scenarios to the largest harmless dose. If the dose a person might receive was much smaller than the largest harmless dose, the chemical was considered unlikely to be harmful to human health. If this wasn’t the case, the chemical was considered to be potentially harmful to human health under the scenario.

For some chemicals there was not enough information to calculate the largest harmless dose. For these chemicals, risk was assessed by analysing the scientific literature and applying expert judgement on the chemical’s potential to harm human health.

The risk assessment methods and findings are described in detail in the

Human health risks report.

## Environmental risk assessment

The Department of the Environment and Energy undertook the environmental risk assessment using two methods. The method used was based on the level of information that was available for the chemical.

For chemicals with only basic information available, the risk assessment was done by compiling evidence from the international scientific literature to support an expert judgement on the chemical’s potential to harm the environment. This approach considered the behaviour of the chemicals in the environment and what harm they could cause to water- and land-based animals and plants. It also considered how chemicals were used in coal seam gas extraction, factoring in the most likely ways for chemicals to be released during the extraction process and protective measures to prevent and limit spills and leaks.

Chemicals with more information available were assessed using a different method. Scenarios were developed to show how chemicals might enter a water body such as a river, pond or lake. Potential harm to the environment was assessed by determining what effect the chemicals could have on aquatic organisms in the water body, including algae, invertebrates and fish. As with the human health risk assessment, the scenarios were informed by the scientific literature and the industry survey in the preparatory steps.

The risks to the environment were assessed for the following scenarios:

a chemical spills during transport from a storage warehouse to the well site and the spill is not detected, notified or cleaned up

a chemical spills from storage at a storage facility and the spill is not detected, notified or cleaned up

a chemical spills from storage at the well site and the spill is not detected, notified or cleaned up

a chemical spills during use and handling at the well site and the spill is not detected, notified or cleaned up

waste water containing a chemical spills during use or management of the waste water and the spill is not detected, notified or cleaned up

waste water containing a chemical leaks from storage pond or tank and the leak is unreported

waste water containing a chemical is reused for dust suppression

waste water containing a chemical is reused for irrigation.

The second step in this method of risk assessment used the predicted concentration models and the above scenarios to calculate the concentrations of a chemical that the aquatic animals and plants might come into contact with. In keeping with the very conservative approach taken in this assessment, the models were designed to overestimate the concentration of a chemical that animals and plants might come into contact with.

The final step in this method of risk assessment was to compare the concentrations of chemicals that animals and plants might come into contact with under different scenarios to the largest harmless concentration. If the concentration animals and plants might come into contact with was smaller than the largest harmless concentration, the chemical was considered unlikely to cause harm to the environment. Otherwise, the chemical was considered to be potentially harmful to the environment.

This method used a staged approach with increasingly detailed consideration applied to those chemicals that were more likely to be a potential concern for the environment. Chemicals found to be safe were screened out at an early stage. The first stage used generic assumptions about extraction sites and worst case scenarios for the quantities of chemicals potentially entering the environment. Two more stages of testing were more specific to the conditions of actual extraction sites and the quantities of chemicals likely to be in use.

For some of the chemicals assessed using this method there was not enough information about the ways and quantities in which the chemicals were used to perform more detailed assessments. These chemicals were assessed only at the earliest, most conservative tier of testing.

Both methods of assessing risks to the environment, and their findings, are described in detail in the

Environmental risks report.

What did the Assessment find?

The Assessment identified 113 chemicals used in drilling and hydraulic fracturing for coal seam gas in Australia between 2010 and 2012. Not all of these chemicals were used at any one site and hydraulic fracturing is not necessary at many coal seam gas wells.

A results table can be found at the end of this document.

Most of the risk assessments did not take into account the safety and handling precautions that are taken to protect people and the environment from industrial chemical use. The chemicals identified as risks are potentially harmful in the specific conditions of the scenarios, which looked at what could happen if no protections were in place. The Assessment examined worst case scenarios with no protections in place because if a chemical is found to be unlikely to cause harm in these extreme situations, we can be confident that it can be used safely.

In reality, these precautions are required by law and significantly reduce any remaining likelihood of any potential harm occurring. The stringent protective measures imposed by state and territory and Commonwealth governments for industry are described in more detail on page 16. These measures significantly reduce the likelihood of potential harm.

# What do the results mean for public health?

As members of the public are not involved in the day-to-day coal seam gas extraction process, they are unlikely to come into direct contact with chemicals. It is generally only through large-scale accidental spills and leaks that people could come into contact with the chemicals. This is true of chemicals used in many other activities and sectors.

These scenarios did not take into account all standard precautionary measures that are taken to prevent, contain, clean up or report a spill or leak.

In these pre-mitigation scenarios, the majority of chemicals (73 of the 113 tested) were found to be unlikely to cause harm to public health when used in coal seam gas extraction, even if they were to spill or leak in large volumes without the leak or spill being detected or cleaned up.

There were two scenarios where some chemicals could be a risk to the health of a member of the public in the absence of standard risk management measures: a large transport spill and a long-term below ground leak from a storage pond. Both scenarios require the person to drink, wash with or swim in water containing the chemical on a regular basis over an extended period for harm to occur.

## What do the results mean for workers?

Workers may be involved in any or all parts of the coal seam gas extraction process, including storing and transporting chemicals to the well site, preparing and using chemicals in drilling or fracturing, cleaning and maintaining equipment, or managing waste water extracted from the well. During these activities, workers may be working directly with chemicals, sometimes in concentrated forms.

These scenarios did not take into account standard safety and handling precautions such as personal protective equipment or other workplace health and safety strategies, which are required by law.

In these scenarios, the majority of chemicals (65 of the 113 tested) were found to be unlikely to cause harm to workers’ health during coal seam gas extraction, even if standard protections weren’t in place.

Some chemicals were found to be potentially harmful to workers if one-off contact occurs in the event of an industrial accident, or if repeated contact occurs during long-term mixing and blending or cleaning and maintenance.

What do the results mean for the environment?

The Assessment looked at what could happen if a chemical entered the environment through accidental spills and leaks, or intentional reuse of untreated coal seam gas waste water for other purposes.

The majority of chemicals (61 of the 113 chemicals tested) were found to be unlikely to cause harm to the environment when used in coal seam gas extraction, even if they were to spill or leak in high volumes.

It is in the event of a transport spill or where untreated waste water containing chemicals is reused for irrigation or dust suppression, that certain chemicals have the potential to cause harm to the environment.

Where detailed information was unavailable the chemicals could only be assessed at the earliest, most conservative level of testing which was designed to overestimate risk. The chemicals were classed as potentially harmful at this level, but further information and testing would be required to determine the actual level of risk.

Protecting human health and the environment

Industrial chemical use and coal seam gas operations are closely regulated by state, territory and Commonwealth governments, which legally require protective measures to be in place to safeguard human health and the environment.

Legislation, regulations, standards and codes of practice cover the coal seam gas industry including workplace and public health and safety, environmental and water protection, managing and reusing waste water, and the transport, handling, storage and disposal of chemicals. Coal seam gas projects must be assessed and approved under state, territory and Commonwealth environmental laws and may be subject to site-specific conditions including how the companies manage chemical risk.

The risk assessments did not take all these regulatory controls into account.

The Assessment has provided regulators and companies with an additional level of information directly relevant to the coal seam gas industry. This is new knowledge and information that will enable more targeted risk management actions and practices for the safe management of chemicals in coal seam gas operations.

## Summary of protections in place

The research and methods established for the Assessment have also led to improved, targeted methods for assessing and managing the risks of chemicals used in coal seam gas operations. A guidance manual based on these improved methods has been peer-reviewed and released as an exposure draft and will be finalised after consultation. The guidance manual will provide a consistent and transparent approach to assessing risks. The guidance manual will provide industry and stakeholders more certainty because assessment requirements will be clear to everybody. Industry will know the standards that will apply and be able to plan early to meet them.

Results table

This table summarises the findings of the Assessment. For each of the 113 chemicals assessed, the table shows whether the chemical is a potential risk to workers, the public or the environment in the absence of all the usual precautions and legal requirements for handling chemicals. If it is a risk, the table also identifies the scenario in which it could cause harm.

| Chemical name | CAS number | Unlikely to cause harm to public | Potentially harmful to public health | Unlikely to cause harm to workers | Potentially harmful to workers’ health | Unlikely to cause harm to environment | Potentially harmful to environment |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Acetic acid | 64-19-7 |  | In the event of a transport spill |  | In the event of an industrial accident |  |  |
| Acetic acid, sodium salt (1:1) | 127-09-3 |  |  |  |  |  |  |
| Alcohols, C6-12, ethoxylated | 68439-45-2 |  | In the event of a transport spill or pond leak |  | In the event of an industrial accident |  | Limited assessment |
| Alkanes, C12-26 branched and linear | 90622-53-0 |  | In the event of a transport spill |  |  |  |  |
| Amine salt | Confidential Business Information |  |  |  |  |  | Limited assessment2 |
| Ammonium persulfate | 7727-54-0 |  | In the event of a transport spill or pond leak |  | When mixing and/or cleaning In the event of an industrial accident |  | In the event of a transport spill |
| Bentonite | 1302-78-9 |  |  |  |  |  |  |
| Benzisothiazolinone | 2634-33-5 |  |  |  | In the event of an industrial accident |  | Limited assessment2 |
| Borax | 1303-96-4 |  | In the event of a transport spill |  |  |  | When waste water is reused |
| Boric acid | 10043-35-3 |  | In the event of a transport spill |  |  |  | In the event of a transport spill When waste water is reused |
| Boric acid salt, monoethanolamine | 26038-87-9 |  | In the event of a transport spill |  |  |  | When waste water is reused |
| Bronopol | 52-51-7 |  |  |  | In the event of an industrial accident When mixing and/or cleaning |  | Limited assessment2 |
| Butoxyethanol | 111-76-2 |  | In the event of a transport spill or pond leak |  | In the event of an industrial accident |  |  |
| Calcined silica | 91053-39-3 |  |  |  | When mixing and/or cleaning |  |  |
| Calcium chloride | 10043-52-4 |  |  |  |  |  |  |
| Carbon dioxide | 124-38-9 |  |  |  |  |  |  |
| Carbonic acid | 463-79-6 |  |  |  |  |  |  |
| Carbonic acid sodium salt (1:1) | 144-55-8 |  |  |  |  |  |  |
| Carbonic acid sodium salt (2:3) | 533-96-0 |  |  |  |  |  |  |
| Caustic soda | 1310-73-2 |  |  |  | In the event of an industrial accident |  | In the event of a transport spill |
| Cellulase | 9012-54-8 |  |  |  | In the event of an industrial accident |  |  |
| Cellulose, 2-hydroxyethyl ether | 9004-62-0 |  |  |  |  |  |  |
| Cristobalite | 14464-46-1 |  |  |  | When mixing and/or cleaning |  |  |
| D-erythro-Hex-2-enonic acid, γ-lactone, sodium salt (1:1) | 6381-77-7 |  |  |  |  |  | Limited assessment2 |
| Deodorised kerosene | 64742-47-8 |  | In the event of a transport spill |  |  |  | Limited assessment2 |
| Diethylene glycol ethyl ether | 111-90-0 |  |  |  |  |  | Limited assessment2 |
| Diphosphoric acid, sodium salt (1:2) | 7758-16-9 |  |  |  |  |  | Limited assessment2 |
| Enzyme | Confidential Business Information3 |  |  |  | In the event of an industrial accident |  |  |
| Ester alcohol | Confidential Business Information3 |  | In the event of a transport spill |  |  |  | Limited assessment2 |
| Ethanaminium, 2-hydroxy-N,N N-trimethyl-, chloride (1:1) | 67-48-1 |  |  |  |  |  |  |
| Ethanedial | 107-22-2 |  |  |  |  |  |  |
| Ethanol | 64-17-5 |  | In the event of a transport spill |  | In the event of an industrial accident |  | Limited assessment2 |
| Ethanolamine | 141-43-5 |  |  |  | In the event of an industrial accident |  | Limited assessment2 |
| Ethoxylated fatty acid I | Confidential Business Information3 |  | In the event of a transport spill |  | In the event of an industrial accident |  | Limited assessment2 |
| Ethoxylated fatty acid II | Confidential Business Information3 |  |  |  |  |  | Limited assessment2 |
| Ethoxylated fatty acid III | Confidential Business Information3 |  | In the event of a transport spill |  | In the event of an industrial accident |  | Limited assessment2 |
| Ethylene glycol | 107-21-1 |  | In the event of a transport spill |  | In the event of an industrial accident |  |  |
| 2-Ethylhexanol heavies | Confidential Business Information3 |  | In the event of a transport spill |  |  |  | Limited assessment2 |
| Fatty acids ester | Confidential Business Information3 |  | In the event of a transport spill |  |  |  |  |
| Gelatins | 9000-70-8 |  |  |  |  |  |  |
| Glutaraldehyde | 111-30-8 |  | In the event of a transport spill |  | In the event of an industrial accident |  | In the event of a transport spill |
| Guar gum | 9000-30-0 |  |  |  |  |  | In the event of a transport spill |
| Guar gum, carboxymethyl 2-hydroxypropyl ether, sodium salt | 68130-15-4 |  |  |  |  |  |  |
| Hemicellulase | 9025-56-3 |  |  |  | In the event of an industrial accident |  |  |
| Hydrochloric acid | 7647-01-0 |  |  |  | In the event of an industrial accident |  | In the event of a transport spill |
| Hydrogen peroxide | 7722-84-1 |  | In the event of a transport spill or pond leak |  | In the event of an industrial accident |  |  |
| Inner salt of alkyl amines | Confidential Business Information3 |  | In the event of a pond leak |  | In the event of an industrial accident |  | Limited assessment2 |
| Isopropanol | 67-63-0 |  | In the event of a transport spill |  | In the event of an industrial accident |  | Limited assessment2 |
| Lime | 1305-78-8 |  |  |  | In the event of an industrial accident |  |  |
| Limestone | 1317-65-3 |  |  |  |  |  |  |
| Magnesium chloride | 7786-30-3 |  |  |  |  |  |  |
| Methanol | 67-56-1 |  | In the event of a transport spill or pond leak |  | When mixing and/or cleaning In the event of an industrial accident |  |  |
| Methyl isobutyl ketone | 108-10-1 |  |  |  |  |  | Limited assessment2 |
| Methylchloroisothiazolinone | 26172-55-4 |  | In the event of a transport spill |  | In the event of an industrial accident |  | In the event of a transport spill |
| Methylisothiazolone | 2682-20-4 |  | In the event of a transport spill |  | In the event of an industrial accident |  | In the event of a transport spill |
| Natural fibres I | Not specified |  |  |  |  |  |  |
| Natural fibres II | Not specified |  |  |  |  |  |  |
| Natural fibres III | Confidential Business Information3 |  |  |  |  |  |  |
| Nitric acid, magnesium salt (2:1) | 10377-60-3 |  |  |  |  |  |  |
| Nitrogen | 7727-37-9 |  |  |  |  |  |  |
| Nut hulls | Not specified |  |  |  |  |  |  |
| Organic acid salt | Confidential Business Information3 |  |  |  | In the event of an industrial accident |  | Limited assessment2 |
| Organic sulphate | Confidential Business Information3 |  | In the event of a transport spill |  | In the event of an industrial accident |  | Limited assessment2 |
| Pigment Red 5 | 6410-41-9 |  |  |  |  |  | Limited assessment2 |
| Polyamine | Confidential Business Information3 |  | In the event of a transport spill or pond leak |  | In the event of an industrial accident When mixing and/or cleaning |  |  |
| Polyacrylamide / polyacrylate copolymer | Confidential Business Information3 |  |  |  |  |  |  |
| Polyanionic cellulose PAC | Not specified |  |  |  |  |  |  |
| Polydimethyldiallyl-ammonium chloride | 26062-79-3 |  |  |  | In the event of an industrial accident |  |  |
| Polyesters | Not specified |  |  |  |  |  |  |
| Polymer I | Confidential Business Information3 |  |  |  |  |  |  |
| Polymer II | Confidential Business Information3 |  |  |  |  |  | Limited assessment2 |
| Polymer with substituted alkylacrylamide salt | Confidential Business Information3 |  | In the event of a transport spill |  |  |  | In the event of a transport spill |
| Polysaccharide | Confidential Business Information3 |  |  |  |  |  |  |
| Potassium carbonate | 584-08-7 |  |  |  | In the event of an industrial accident |  |  |
| Potassium chloride | 7447-40-7 |  |  |  |  |  | In the event of a transport spill |
| Precipitated silica | 112926-00-8 |  |  |  |  |  |  |
| 1,2,3-Propanetricarboxylic acid, 2-hydroxy- | 77-92-9 |  |  |  |  |  |  |
| 1,2,3-Propanetriol | 56-81-5 |  |  |  |  |  | Limited assessment2 |
| 2-Propenamide, homopolymer | 9003-05-8 |  |  |  |  |  |  |
| 2-Propenoic acid, methyl ester, polymer with 1,1-dichloroethene | 25038-72-6 |  |  |  |  |  |  |
| 2-Propenoic acid, polymer with 2-propenamide | 9003-06-9 |  |  |  |  |  |  |
| Quartz | 14808-60-7 |  |  |  | When mixing and/or cleaning |  |  |
| Quaternary amine | Confidential Business Information3 |  |  |  |  |  | Limited assessment2 |
| Silica | 7631-86-9 |  |  |  |  |  |  |
| Sintered bauxite | 144588-68-1 |  | In the event of a pond leak |  | In the event of an industrial accident |  |  |
| Slaked lime | 1305-62-0 |  |  |  | In the event of an industrial accident |  |  |
| Soda ash | 497-19-8 |  |  |  | In the event of an industrial accident |  | In the event of a transport spill |
| Sodium borate | 12008-41-2 |  | In the event of a transport spill or pond leak |  | In the event of an industrial accident |  | When waste water is reused |
| Sodium chloride | 7647-14-5 |  |  |  |  |  | In the event of a transport spill |
| Sodium chlorite | 7758-19-2 |  | In the event of a transport spill or pond leak |  |  |  | Limited assessment |
| Sodium hypochlorite | 7681-52-9 |  | In the event of a transport spill |  | In the event of an industrial accident When mixing and/or cleaning |  | Limited assessment2 |
| Sodium persulfate | 7775-27-1 |  | In the event of a transport spill |  | In the event of an industrial accident |  | Limited assessment2 |
| Sodium sulphite | 7757-83-7 |  | In the event of a transport spill |  | In the event of an industrial accident |  | Limited assessment2 |
| Sodium thiosulfate | 7772-98-7 |  | In the event of a transport spill |  |  |  |  |
| Sulfuric acid ammonium salt (1:2) | 7783-20-2 |  |  |  |  |  | Limited assessment2 |
| Sulfuric acid, barium salt (1:1) | 7727-43-7 |  |  |  |  |  |  |
| Sulfuric acid, mono-C6-10-alkyl esters, ammonium salts | 68187-17-7 |  | In the event of a transport spill |  | In the event of an industrial accident |  | Limited assessment2 |
| Sulfuric acid potassium salt (1:2) | 7778-80-5 |  |  |  |  |  | In the event of a transport spill |
| Sulfuric acid sodium salt (1:2) | 7757-82-6 |  |  |  |  |  | Limited assessment2 |
| Talc | 14807-96-6 |  |  |  |  |  |  |
| Terpenes and terpenoids | Confidential Business Information3 |  | In the event of a transport spill |  | In the event of an industrial accident |  |  |
| Terpenes and terpenoids, sweet orange-oil | 68647-72-3 |  | In the event of a transport spill |  | In the event of an industrial accident |  | Limited assessment2 |
| Tetramethylammonium chloride | 75-57-0 |  | In the event of a transport spill or pond leak |  | In the event of an industrial accident When mixing and/or cleaning |  | Limited assessment2 |
| Tetrasodium EDTA | 64-02-8 |  |  |  | In the event of an industrial accident |  |  |
| Tetrakis(Hydroxymethyl) Phosphonium Sulphate (THPS ) | 55566-30-8 |  | In the event of a transport spill or pond leak |  | When mixing and/or cleaning In the event of an industrial accident |  | In the event of a transport spill |
| Tributyltetradecyl phosphonium chloride | 81741-28-8 |  | In the event of a transport spill or pond leak |  | In the event of an industrial accident When mixing and/or cleaning |  | Limited assessment2 |
| Triethanolamine | 102-71-6 |  | In the event of a transport spill or pond leak |  | In the event of an industrial accident |  |  |
| Tridymite | 15468-32-3 |  |  |  | When mixing and/or cleaning |  |  |
| Walnut hulls | Not specified |  |  |  |  |  |  |
| Water | 7732-18-5 |  |  |  |  |  |  |
| Wood dust | Not specified |  |  |  |  |  |  |
| Wood fibre | Not specified |  |  |  |  |  |  |
| Xanthan gum | 11138-66-2 |  |  |  |  |  | In the event of a transport spill |

Where can I find out more about the Assessment?

The National assessment of chemicals associated with coal seam gas extraction webpage provides more detailed information about the Assessment, its findings and the measures in place to protect human health and the environment. The full literature reviews and technical reports are also available on this page.

List of Report Titles

This table lists the reports commissioned for the National assessment of chemicals associated with coal seam gas extraction in Australia.

|  | Number | Report Title |
| --- | --- | --- |
| Reviewing existing literature | 1 | ### Literature review: Summary report (NICNAS, 2017) This review summarises the results of the various literature reviews listed below. |
|  | 2 | ### Literature review: Human health Implications (NICNAS, 2017) This report reviews the literature about human health, regulation of chemicals and gas extraction available before 2013. |
|  | 3 | ### Literature review: Environmental risks posed by chemicals used in coal seam gas operations (DoEE, 2017) This report reviews the literature about the environment and gas extraction available up until 2013. |
|  | 4 | ### Literature review: Hydraulic fracture growth and well integrity (Jeffery et al., 2017) This report reviews the literature about hydraulic fracturing and coal seam gas wells available before 2013. |
|  | 5 | ### Literature review: Geogenic contaminants associated with coal seam operations (Apte et al., 2017) This report reviews the literature about naturally occurring chemicals and hydraulic fracturing available before 2013. |
|  | 6 | ### Literature review: Identification of potential pathways to shallow groundwater of fluids associated with hydraulic fracturing (Mallants et al., 2017) This report reviews the literature about leaks, accidents and spills associated with gas extraction available before 2013. |
| Identifying chemicals | 7 | ### Identification of chemicals associated with coal seam gas extraction in Australia (NICNAS, 2017) This report identifies 113 chemicals used in drilling and hydraulic fracturing for coal seam gas in Australia. |
| Modelling how chemicals could be released during coal seam gas extraction | 8 | ### Human and environmental exposure conceptualisation: Soil to shallow groundwater pathways (Mallants et al., 2017) This report describes how chemicals could be released during the coal seam gas extraction process into soil and shallow groundwater. |
| Modelling how chemicals could be released during coal seam gas extraction Assessing risks to workers and the public | 9 | ### Environmental exposure conceptualisation: Surface to surface water pathways (DoEE, 2017) This report describes how chemicals could be released during coal seam gas extraction processes into the environment and what aquatic plants and animals might be affected. |
|  | 10 | ### Human and environmental exposure assessment: Soil to shallow groundwater pathways – A study of predicted environmental concentrations (Mallants et al., 2017) This report calculates potential concentrations of chemicals in soils and shallow groundwater based on how chemicals could be released during the coal seam gas extraction process. |
|  | 11 | ### Chemicals of low concern for human health based on an initial assessment of hazards (NICNAS, 2017) This report identifies the chemicals used in coal seam gas extraction that are unlikely to cause harm to human health. |
|  | 12 | ### Human health hazards of chemicals associated with coal seam gas extraction in Australia (NICNAS, 2017) ### Appendix A – Hazard Assessment Sheets (NICNAS, 2017) This report assesses the potential harm that the chemicals used in coal seam gas extraction may cause. |
|  | 13 | ### Human health risks associated with surface handling of chemicals used in coal seam gas extraction (NICNAS, 2017) ### Appendix D – Human health chemical risk assessment sheets (NICNAS, 2017) This report assesses whether chemicals could be harmful to workers or the public in scenarios specific to coal seam gas extraction in Australia. |
| Assessing risks to the environment | 14 | ### Environmental risks associated with surface handling of chemicals used in coal seam gas extraction (DoEE, 2017) ### Appendices A, B, C, D, F, and G (DoEE, 2017) ### Appendix E (DoEE, 2017) This report assesses whether chemicals could be harmful to the environment in scenarios specific to coal seam gas extraction in Australia. |

Additional supporting research

Deeper groundwater hazard screening for chemicals used in coal seam gas extraction

The Department of the Environment and Energy commissioned CSIRO to conduct further research on how chemicals used in coal seam gas extraction might move from deeper groundwater to other parts of the environment, how long this movement might take, and what the concentrations of chemicals might be at receiving environments such as water bores and streams. The research found that chemicals remaining underground after hydraulic fracturing are unlikely to reach people or ecosystems in concentrations that would cause concern. This conclusion is based on natural dilution and degradation that reduce concentrations to negligible levels. Risks are therefore likely to be very low.

The research developed methods that can be used on a project-by-project basis to assess risks to human health and the environment from chemicals remaining deep underground as a result of hydraulic fracturing in coal seam gas operations. It did not assess the risks associated with any existing or proposed coal seam gas project. The results of this research are available on the Department’s website.

Risk Assessment Guidance Manual: for chemicals associated with coal seam gas extraction – Exposure draft

The Department is also developing a chemical risk assessment guidance manual based on the improved methods from the Assessment. This has been peer-reviewed and released as an exposure draft and will be finalised after consultation. The guidance manual will provide a consistent and transparent approach to assessing risks associated with these chemicals.

Regulations and Practices to keep People and the Environment Safe: Chemicals used in Coal Seam Gas

## Case Study: Ammonium Persulfate

Coal seam gas extraction is regulated by state and Commonwealth governments, which require protective measures to be in place to safeguard health and the environment. While the requirements vary from jurisdiction to jurisdiction, this regulatory framework applies to all aspects of the industry, including the handling and use of chemicals.

The National Assessment of Chemicals Associated with Coal Seam Gas Extraction in Australia (the Assessment) found that, in the absence of safety precautions required by law, some chemicals could be a risk to:

- workers in the event of an industrial accident or through short or long-term handling when mixing or cleaning

- the public through a large-scale transport spill or a long-term leak from a storage pond

- the environment through a large-scale transport accident or when waste water is reused.

The deeper groundwater hazard screening research found that chemicals remaining underground after hydraulic fracturing are unlikely to reach people or ecosystems in concentrations that would cause concern. This conclusion is based on natural attenuation processes that reduce concentrations (by chemicals being diluted (‘watered down’) or naturally degraded  (‘broken down’) to negligible levels. Risks are therefore likely to be very low.

Here, we use the example of one of the chemicals from the Assessment – ammonium persulfate – to illustrate how regulations and industry practices keep people and the environment safe. Ammonium persulfate was found to be potentially harmful, in the absence of safety precautions required by law, in all of the assessed scenarios except the reuse of waste water.

What is ammonium persulfate?

Ammonium persulfate is a white powder that is highly soluble in water. It is a strong oxidising agent, which means that it can speed up the development of a fire and make it burn more intensely. Ammonium persulfate can cause skin and respiratory allergies, irritate the skin and eyes and can be harmful if swallowed or inhaled. It is mainly used outside of the coal seam gas industry, e.g. to etch circuit boards, in making polymers, and in some bleaching products used by hairdressers.

“Breakers” such as ammonium persulfate are used in the coal seam gas industry to dissolve hydraulic fracturing gels so the fracturing chemicals can be pumped back up the well.

— The Australian Inventory of Chemical Substances

AMMONIUM

PERSULFATE

AMMONIUM

PERSULFATE

Industrial chemicals may only be imported or manufactured if they are listed on the Australian Inventory of Chemical Substances (AICS) and satisfy any conditions of use shown on the AICS listing. For a chemical not on AICS, the

National Industrial Chemicals Notification and Assessment Scheme (NICNAS) must assess risks to the environment and human health before adding a chemical to AICS or modifying its listed conditions of use. There are some exemptions to this requirement but they are unlikely to apply to chemicals used in coal seam gas extraction.

Ammonium persulfate (CAS RN: 7727-54-0) was listed in the ACIS as an industrial chemical when it was originally compiled. This means it can be manufactured, imported and used for any industrial purpose in Australia.

NICNAS was established by the Industrial Chemicals (Notification and Assessment) Act 1989 to assess the risks of industrial chemicals to health and the environment and make recommendations to promote their safe use. NICNAS has assessed ammonium persulfate as a Priority Existing Chemical for use in the hairdressing industry and at Tier 2 in the Inventory Multi-tiered Assessment and Prioritisation (IMAP) framework. The IMAP Tier 2 assessment found that ‘current risk management measures are considered adequate to protect public and workers’ health and safety, provided that all requirements are met under workplace health and safety, and poisons legislation as adopted by the relevant state or territory’. Neither of the NICNAS assessments considered the use of ammonium persulfate in coal seam gas extraction – this use was considered in the National Assessment.

### Protecting against transport spills

State and Federal guidelines set out the way in which a given chemical can be transported or stored. Ammonium persulfate is classified as a dangerous good under the Australian Dangerous Goods Code (ADG Code) for Transport by Road and Rail. The ADG Code is given legal force by the Transport Operations (Road Use Management)

Act 1995 in Queensland and the Dangerous Goods

(Road and Rail Transport) Act 2008 and Dangerous Goods (Road and Rail Transport) Regulation 2014 in NSW.

Under the ADG Code, drivers responsible for moving large quantities of ammonium persulfate must hold a dangerous goods driver’s licence. Licences are only granted to people who have completed appropriate training and have an acceptable criminal and traffic history. Vehicles used to transport large quantities of ammonium persulfate must have a dangerous goods vehicle licence. Because ammonium persulfate is an oxidising agent, there are also regulations about what else may be carried on a vehicle loaded with it, the warning symbols to be displayed on the outside of the vehicle, and packaging.

Companies transporting ammonium persulfate must use the Signal Word “Danger” and display the pictograms:

— (GHS03 Flame over Circle)

— and;

— (GHS08 Health Hazard).

Companies transporting ammonium persulfate must ensure the packaging and vehicle are marked to signal the dangerous good hazard.

This includes pictograms on the vehicle and marking of the package.

The packaging, storage and transport requirements are outlined in the Safety Data Sheet (SDS) provided by the manufacturer or supplier of the ammonium persulfate. These include precautions such as:

- using specially constructed containers

- keeping containers tightly closed in a dry, cool and well-ventilated place

- protecting the chemical from moisture, keeping away from open flames, hot surfaces and sources of ignition

- keeping ammonium persulfate away from flammable and combustible materials.

- keeping it away from direct sunlight and not storing with strong acids and bases.

Companies involved in coal seam gas extraction also have a general environmental duty of care under the Environmental Protection Act 1994 (Qld) and the Protection of the Environment Operations Act 1997 (NSW). These laws require companies to take precautions to prevent spills and leaks of chemicals that may harm the environment. In both NSW and Queensland there are specific offences relating to wilfully or negligently releasing a contaminant that harms, or is likely to harm, the environment. There are also requirements to report potentially harmful leaks and spills to state authorities who investigate emergency incidents 24 hours a day, 7 days a week.

Coal seam gas companies respond to these requirements with measures such as internal journey management systems that include expectations that transport workers will read and carry SDS sheets for all hazardous chemicals being transported. Companies also undertake vehicle checks, route planning, timing and tracking of vehicles, and monitoring of drivers for fatigue and speed with systems that can track vehicle movements in real time using GPS. Companies also use emergency response plans and emergency contacts.

### Protecting against leaks from storage ponds

Ponds are often used to store water produced during hydraulic fracturing that may contain naturally-occurring salts and minerals or trace quantities of industrial additives such as ammonium persulfate. Ponds must be constructed to minimise the risk that stored water poses to human health and the environment. Specific regulatory requirements apply to the management and storage of water produced during hydraulic fracturing. These regulations vary from state to state and cover the design, approval, construction, operation, and decommissioning of coal seam gas related storage ponds. If the extraction of coal seam gas is likely to have a significant impact on a matter of national environmental significance (which, for coal seam gas projects, includes water resources), the project is required by Environment Protection Biodiversity Conservation Act 1999 (the EPBC Act) to be referred to the Commonwealth Minister for the Environment and Energy for assessment and approval. As part of an approval under the EPBC Act, the Minister for the Environment and Energy may attach conditions to ensure any impact on protected matters, including water resources for coal seam gas projects and any water resource potentially impacted by the project, are acceptable.

In Queensland, the design of storage ponds must take into account legislative requirements under the Environmental Protection Act 1994 (Qld), information in guidelines (such as “Structures which are dams or levees constructed as part of environmentally relevant activities (EM634)” and manuals including Manual for Assessing Hazard Consequence and Hydraulic Performance of Structures [EM635]).

In NSW, the requirements are similar and the NSW Dam Safety Committee reviews all design and construction certification reports along with the NSW Government’s other regulating bodies. All chemicals used during drilling for exploration activities are required to be listed and the impact to the environment from their use assessed as part of the activity approval process. The Division of Resources and Geoscience assesses this, in accordance with Part 5 Environmental Planning and Assessment Act 1979

In addition, the NSW Code of Practice for Coal Seam Gas Fracture Stimulation also provides requirements for chemicals used during these types of processes.

Suitably qualified and experienced designers must approve and oversee the construction of storage ponds. In Queensland, the construction is typically certified by a Registered Professional Engineer of Queensland. Geotechnical investigation, design, construction and certification of the completed structure are subject to regulatory oversight and must meet specified standards. Ponds that hold water returned from wells that have been hydraulically fractured must be designed with a floor and sides of material that will ensure the contents are kept within the bounds of the containment system during the pond’s operational life, and in some cases, a double lining system may be used. Ponds are often required to have a system for detecting leaks through either the floor or sides of the dam, and this further reduces the risk that leaks will affect soil or groundwater.

Coal seam gas companies respond to these requirements with measures such as: • leak detection

- groundwater monitoring of multiple parameters

- seepage monitoring of potential contaminants to groundwater

- annual inspections and reporting (required for regulated ponds).

### Protecting workers

Ammonium persulfate can harm workers if they breathe air with ammonium persulfate dust in it, if it gets on their skin or in their eyes, or if they swallow it. Work, health and safety laws require companies to make sure measures are in place to stop any of these things happening. The measures required will depend on a number of conditions including the conditions at the site, handling methods used and how a worker may be exposed. The SDS, required under Commonwealth and state government workplace health and safety legislation, lists the protective measures that may be required (depending on site conditions). These may include ventilation that ensures the concentration of ammonium persulfate in the air stays as low as reasonably practicable and below the workplace exposure standard of 0.1 mg/ m3 (peak limitation); wearing overalls, chemical goggles, safety shoes, and impervious gloves when handling the chemical; and adequate training for workers. Respiratory protection may also be needed depending on the levels in air in the workplace.

Ammonium persulfate has been classified by Safe Work Australia as a hazardous substance with the following classification: • 	Acute toxicity – category 4

- Eye irritation – category 2A

- Specific target organ toxicity (single exposure)

– category 3

- Skin irritation – category 2

- Respiratory sensitisation – category 1

- Skin sensitisation – category 1

- Oxidising solid – category 3.

Danger

Businesses have specific duties under Work, Health and Safety regulations to manage the risks to health and safety associated with using, handling, generating and storing hazardous chemicals at a workplace. For more details see the Safe Work Australia’s website.

For more information on The National Assessment of Chemicals Associated with Coal Seam Gas Extraction in Australia visit http://www.

environment.gov.au/water/coal-and-coal-seamgas/national-assessment-chemicals

## Overview: Deeper groundwater hazard screening for chemicals used in coal seam gas extraction

In summary, the deeper groundwater hazard screening research found that chemicals remaining underground after hydraulic fracturing are unlikely to reach people or groundwater dependent terrestrial ecosystems in concentrations that would cause concern. Risks are therefore likely to be very low. Risks from naturally-occurring chemicals in the coal seam mobilised by hydraulic fracturing are likely to be very low for the same reasons. Note that strict regulatory controls apply to the handling and use of chemicals in coal seam gas extraction.

This research complements the National assessment of chemicals associated with coal seam gas extraction, which focussed on risks to human health and the environment from surface handling of chemicals used in coal seam gas extraction.

The research developed methods that can be used on a project-by-project basis to assess risks to human health and the environment from chemicals remaining deep underground as a result of hydraulic fracturing in coal seam gas operations. It did not assess the risks associated with any existing or proposed coal seam gas project, which are considered as part of the State and Federal government approval processes.

The movement of chemicals underground depends on the local geology and the properties of the chemicals. The project therefore used two case study areas, one in the Surat Basin and one in the Gunnedah Basin, to develop the methods. It used information about the chemical and physical properties of 13 chemicals chosen because they represent the range of chemicals used in coal seam gas extraction. Whenever the method made assumptions, care was taken they were always conservative (that is, likely to overestimate rather than underestimate risk factors) so that risks would not be underestimated.

### Chemicals in deeper groundwater

Hydraulic fracturing leaves a fraction of the chemicals injected deep underground, and can cause chemicals that occur naturally in the coal seam to move through the underground environment.

This research identified four potential pathways for chemicals to move from a hydraulic fracturing site to places where people or ecosystems might come into contact with them. The project found that all four pathways are either unlikely or extremely unlikely to exist in the study areas, which have similar geology to other areas where coal seam gas extraction is taking place or proposed in Australia.

Basement rock

### Chemicals become less concentrated as they move underground

Where a chemical requires more detailed assessment it will often be useful to calculate how much of it people or ecosystems might come into contact with. This research developed a method to do this.

This method involves calculating how far a chemical must travel to reach a place where people or ecosystems might come into contact with it, how long it will take to get there, and how much its concentration will be reduced when it arrives. The longer a chemical takes to reach a place where people or ecosystems might come into contact with it and the further it has to travel, the more its concentration will be reduced. For most locations investigated, chemicals would not reach people or ecosystems in detectable concentrations.

How long will it take for the chemical to reach a place where people and ecosystems might come into contact with it?

The project used a groundwater model to calculate how fast a chemical could travel along the pathways that might link it to places where people and ecosystems might come into contact with it. The project found that chemicals tend to move extremely slowly underground. In most cases the chemicals would take decades or centuries to reach a place where people and ecosystems might come into contact with it; by this time they are unlikely to do harm.

The time taken to reach a given location depends on the local geology and the properties of the chemical. The properties of the chemical are important because some chemicals ‘stick’ more strongly to surfaces and therefore move more slowly. Most organic chemicals will partially or completely degrade (break down) prior to reaching people or ecosystems.

How far does the chemical need to travel to reach a place where people and ecosystems might come into contact with it?

In each study area, the research identified coal seam gas wells and water-dependent assets such as water bores, surface water features, and vegetation. The project then calculated the distances between each well and each asset. This gave an indication, based on a straight distance, of how far a chemical would need to travel in order to reach an asset. For example, in the Surat Basin there were twenty groundwater bores within 500 metres of a CSG well; all these bores were used to water stock.

How much will the concentration of the chemical be reduced?

The concentration of a chemical can be reduced by dilution (‘watered down’) or degradation (‘broken down’).

All chemicals moving though deeper groundwater are diluted. The degree of dilution depends on the volume of hydraulic fracturing fluid injected and how far the chemical travels. The research found that dilution alone reduces the concentration of chemicals that travel 500 metres by a factor of about a thousand (when one million litres of fracturing fluid is injected). Dilution is greater for travel over longer distances.

Organic chemicals are degraded (broken down) by chemical reactions and biological processes. The degree of degradation depends on the chemical, the conditions underground, and how long it takes to reach the location of interest. The conditions underground are important because many chemicals break down faster when temperatures are higher or oxygen is present. Some micro-organisms, which may be found underground, can also speed up the breakdown of some chemicals.

The research calculated how fast sample chemicals break down under the range of conditions found at the study sites. It found that degradation reduces the concentration of many chemicals close to zero over relatively short travel distances. Other chemicals – mostly inorganics – do not degrade at all.

The combined effect of dilution and degradation reduces the concentration of chemicals that travel 500 metres by factors ranging from a thousand (for slowly degrading chemicals) to ten billion (for quickly degrading chemicals). The smaller values are for chemicals that do not degrade. The larger values are for chemicals that degrade quickly and travel through environments conducive to degradation.

### Using this research to improve risk assessments

The methods developed in this research can be used to estimate the concentration of a chemical when it reaches a water-dependent asset, such as a water bore or a groundwater-dependent ecosystem. These predicted environmental concentrations can be used to assess risks to human health and/or the environment. See the box for an example. More details about assessing risks of chemicals used in coal seam gas extraction can be found in the risk assessment guidance manual.

Dilution alone might reduce the concentration of chemicals reaching places where people and the environment may come into contact with them to an acceptable level. When dilution over short distances is considered, differences in regional geology do not make much difference to how much chemicals are diluted as they travel underground. This means risk assessments over short distances can use the information from the deeper groundwater hazard screening research without any further modelling.

If risk assessments also wish to include degradation for calculating concentrations, differences in regional geology can make a difference to how fast chemicals degrade, so it may be necessary to do some modelling using the approaches set out in the deeper groundwater hazard screening research.

### Using the research to calculate predicted environmental concentrations: an example

Suppose a CSG well was hydraulically fractured using one million litres of a fluid containing 525 milligrams per litre of acetic acid and 2600 milligrams per litre of guar gum. There is unlikely to be a pathway for the chemicals to move from the well site to a place where people or ecosystems might come into contact with them. But supposing there was a pathway, would the concentrations of acetic acid and guar gum at a site 1000 metres from the well be a risk to aquatic ecosystems?

Dilution alone would reduce the concentration of acetic acid to 0.2 milligrams a litre. This is well below the concentration of 15 milligrams a litre that has been found to have no effect on aquatic ecosystems. There is no need to consider degradation, which would reduce concentrations further. Note that the chemical would be further diluted if it entered a stream, wetland or other water body.

Dilution alone would reduce the concentration of guar gum to 1 milligram a litre. This is higher than the concentration of 0.22 milligrams a litre that has been found to have no effect on aquatic ecosystems. However, if we take degradation into account (based on an estimation of the time typically taken for a chemical to be carried 1000 metres) the concentration of guar gum falls to 0.001 milligrams a litre. This result holds in the two case study regions. Because local geology can affect degradation, additional modelling might be needed in other regions. Note that the chemical would be further diluted if it entered a stream, wetland or other water body.

The methods developed in this research can be also be used to calculate:

- the maximum concentration of a chemical in hydraulic fracturing fluid that will be reduced to an acceptable concentration at a given distance from the well, and

- the minimum distance between a hydraulically fractured well and a water body, water bore or groundwater-dependent ecosystem needed to reduce a given concentration of a chemical to an acceptable level.

Each chemical is identified by a unique numerical identifier assigned by Chemical Abstracts Service (CAS) to every chemical substance.

Where detailed information was unavailable the chemicals could only be assessed at the earliest, most conservative level of testing which was designed to overestimate risk. The chemicals were classed as potentially harmful at this level, but further information and testing would be required to determine the actual level of risk.

Confidential Business Information is not disclosed publically but all information was available to the scientists conducting the assessment.