# VIDAS Salmonella (SLM) Assay - AOAC 996.08

## SCOPE

This method is applicable to:

- Raw meats, meat products and carcass swabs.

## PRINCIPLES

Detection of Salmonella is based on enzyme-linked fluorescent immunoassay performed in an automated VIDAS instrument. Following enrichment (see below) Salmonella is detected in boiled broth by specific monoclonal antibodies. A fluorescent marker is then added and Salmonella detected automatically by the VIDS machine. The detection protocol can be broken down as follows:

### Pre-enrichment in non-selective liquid medium

Sample is diluted 1:10 in pre-warmed (room temperature or 37 ± 1C for large volumes) buffered peptone water and pre-enriched at 37 ± 1C for 18 h ± 2 h. For carcass sponges, buffered peptone water is added to the moistened sponge to bring the total volume to 60-100 mL and the sample incubated at 37 ± 1C for 18 ± 2 h. In the case of sponges BPW need not be warmed to room temperature before being used to re-hydrate the sponge, for all subsequent additions BPW should be warmed to room temperature.

### Selective Enrichment

Selenite cystine broth and tetrathionate broth are inoculated with pre-enrichment broth and incubated at 35 ± 1 and 42 ± 1C, respectively, for 18 ± 2 h.

### Post-enrichment

Selective enrichment broths are inoculated into M-broth and incubated for 6 h at 42 ± 1C.

### Enzyme immunoassay

Salmonella is assayed by testing a portion of boiled post-enrichment broth. The assay is automated and carried out in the VIDAS instrument following the manufacturer’s instructions. The assay should be finished in approximately 45 minutes.

### Cultural confirmation

All Salmonella positive samples should be confirmed using refrigerated M broth   following AS 5013.10.

## CHECKLIST

| Pre-enrichment | Is the buffered peptone water used for pre-enrichment? |  |
| --- | --- | --- |
|  | Is BPW warmed prior to use (room temperature or 37 ± 1C)? |  |
|  | Is the correct amount of enrichment broth used for the weight of sample analysed? |  |
|  | Is a positive control run with each batch of samples analysed? |  |
|  | Are reference cultures inoculated into primary enrichment broth at a level of 10 to 100 cells? |  |
|  | Is pre-enrichment done at 37 ± 1C for 18 h? |  |
| Selective-enrichment | Is selective enrichment carried out in SC and TT broth? |  |
|  | Is SC broth incubated at 35 ± 1C for 18 h? |  |
|  | Is TT broth incubated at 42 ± 1C for 18 h |  |
| Post-enrichment | Is M-broth incubated at 42 ± 1C for 6 h? |  |
| Enzyme immunoassay | Are the manufacturer’s instructions available? |  |
|  | Are kits stored at 2-8C when not in use? |  |
| Cultural confirmation | Is Salmonella confirmed from M-Broth or selective enrichment media? |  |
| (if applicable) | If an external laboratory is used is it department approved? |  |
|  | M broth should be supplied to off-site laboratories for confirmation following AS 5013.10 |  |
|  | Are Salmonella confirmed using AS 5013.10 (with appropriate selective agar plates)? |  |