# Solus Listeria ELISA – NF SOL 37/02 – 06/13

## SCOPE

This method is applicable to meat, environmental samples and other food samples. This method is not validated on food samples weighing more than 25 g.

## PRINCIPLES

Detection of Listeria is based on enzyme-linked immunosorbent assay (ELISA). Listeria specific antibodies are coated on the wall of microplate wells that attach with Listeria antigens when present in the enrichment. After washing an enzyme-labelled antibody is added to the well which then binds with the antigen-antibody complex. After a second wash, a substrate is added which in the presence of bound enzyme-antigen-antibody complex produces a blue colour. The substrate reaction is stopped after 30 seconds by acid and the intensity of the colour is measured. The detection protocol can be broken down as follows:

### Pre-enrichment

Sample (25 g) is enriched in 225 mL Half Fraser broth. Samples are homogenised and at incubated at 30 ± 1C for 24 ± 2h. A positive reference culture must be run through the enrichment daily or when testing is carried out.

### Selective enrichment

Subculture of 0.2 mL of the pre-enrichment is inoculated in 10 mL of RELM broth and incubated for 24 ± 2 h at 30 ± 1C.

### Enzyme immunoassay

Solus ELISA is performed using a portion of boiled selective enrichment broth. The assay is carried out in the Solus instrument following the manufacturer’s instructions.

### Cultural confirmation

All Listeria positive samples should be confirmed using selective enrichment broth following AS 5013.24.1. Confirmation carried out at an ‘off-site’ laboratory must be from retained selective enrichment.

## CHECKLIST

| Enrichment | Is correct amount of Half Fraser broth used for the weight of sample analysed? |  |
| --- | --- | --- |
|  | Is a positive control run with each batch of samples analysed? |  |
|  | Are reference cultures inoculated into primary enrichment broth at a level of 10 to 100 cells? |  |
|  | Are pre-enrichment and selective enrichment done at 30 ± 1C for 24 ± 2h? |  |
| Enzyme immunoassay | Are the manufacturer’s instructions available? |  |
|  | Are ELISA kits allowed one hour to warm up at room temperature before use? |  |
|  | Are positive & negative kit controls run with each batch of samples? |  |
|  | Are plates incubated at 37C for 30 min after adding samples and again after adding conjugate? |  |
|  | Are ELISA plates washed using a microplate washer? |  |
|  | Are OD readings taken within 10 min of adding stop solution? |  |
|  | Are samples with OD450 ≥0.200 considered presumptive positive? |  |
|  | Are ELISA kits stored as per manufacturer’s guidelines (2 -8C)? |  |
| Cultural confirmation | Is Listeria confirmed from selective enrichment broth? |  |
|  | Are Listeria confirmed at a department approved laboratory using AS 5013.24.1? |  |