# VeriflowTM Salmonella species (SS) - AOAC 011404

## SCOPE

This method is applicable to:

- Raw ground beef and other foods

## PRINCIPLES

VeriflowTM Salmonella species (SS) is a PCR assay combined with a rapid chromatographic vertical flowthrough system that provides specific and highly sensitive detection of Salmonella species. The system provides a rapid and easy-to-interpret results.

All samples identified as potentially positive for Salmonella using this test must be confirmed using AS 5013.10.

The detection of Salmonella spp. is broken down into following stages:

### Pre-enrichment in non-selective medium

As per AS 5013.10. For meat and meat products a 1:10 dilution of the sample is enriched in buffered peptone water at 37 ± 1C for 18 h ± 2 h. Buffered peptone water should be warmed to room temperature or to 37 ± 1C for large volumes (i.e. >225 mL). For carcass sponges, buffered peptone water is added to the moistened sponge to bring the total volume to 60-100 ml and the sample incubated at 37 ± 1C for 18 h ± 2h. In the case of sponges BPW need not be warmed to room temperature before being used to re-hydrate the sponge, for all subsequent additions BPW should be warmed to room temperature

A positive control culture must be run through all procedures daily or when testing is carried out.

### VeriflowTM SS screening

Salmonella species is screened in the sample following the manufacturer’s recommended protocol. VeriflowTM Salmonella species is a PCR based test for qualitative detection of Salmonella spp. in selected foods and environmental samples. The assay combines PCR with chromatographic flowthrough system that provides a rapid and highly sensitive detection of target organisms.

### Confirmation

All Salmonella positive samples should be confirmed using AS 5013.10. Confirmation must be carried out according to the manufacturer’s instructions using AS 5013.10 as appropriate, if confirmation is carried out off-site then the ‘pre-enrichment’ broth (i.e. BPW) must be used.

## CHECKLIST

| Pre-enrichment | Is the BPW warmed to room temperature (to 37± 1C for large quantities)? |  |
| --- | --- | --- |
|  | Is the correct amount of enrichment broth used for the weight of sample analysed? |  |
|  | Is a positive control run with each batch of samples analysed? |  |
|  | Are reference cultures inoculated into primary enrichment broth at a level of 10 to 100 cells? |  |
|  | Is enrichment carried out at 37± 1C for 18± 2h? |  |
| VeriflowTM screening | Are the manufacturer’s instructions available and are they reproduced in the laboratory manual? |  |
|  | Are kites stored at recommended conditions when not in use? |  |
| Confirmation | Are all suspect Salmonella sent to a reference Laboratory (see AS 5013.10) to be serotyped? |  |
|  | BPW should be supplied to off-site laboratories for confirmation following AS 5013.10 |  |
|  | If Salmonella confirmation is done in-house refer to AS 5013.10. |  |

See the original method for enrichment medium and food types cover by this method

The Invisible Sentinel, Inc, VeriflowTM Salmonella species (SS) protocol